# Chapter No. 1

# Salient Features of Rajanpur Canal Division

Rajanpur Canal Division was commenced in 1966. The Irrigation network of this Division comprises of two main canals i.e. Dajal Branch (RD: 99+000-190+000) & Link No. III (Rd: 23+000 – 227+820), which off-take from tail D.G. Khan Canal (Zero Head Regulator). The discharge of Link No. III is 3043 Cusecs at its head & 2739 Cusecs at RD: 23+000.  The Revised discharge of Dajal Branch at head is 1786 cusecs RD: 0+000 & 1582 cusec at RD: 99+000 (The Indent Point of Rajanpur Canal Division).

The other two major channels are Dhundi Kutab Canal (DKC) & Kadra Canal, which off-take from Link No. III and their respective discharges are 689 & 1340 cusecs respectively. A brief of the salient features is given in the table below:-

Table 1: Salient Features of Rajanpur Canal Division

| Total Number of Distributaries. | 40 |
| --- | --- |
| Total No. of Minors. | 33 |
| Total No. of Branch Canal. | 2 |
| Total No. of Channels | 75 |
| Total length of Channels | 590 Miles |
| Length of Link No. III, RD: 23000-227820 | 40 Miles |
| Length of Dajal Branch, RD: 99-190 | 18 Miles |
| Length of Distributaries. | 412 Miles |
| Length of Minors | 120 Miles |
| Number of Hill Torrent Crossing at Dajal Branch. | 6 |
| Number of X-ing at DKC, Raj Disty, Hamid Disty & Kadra Canal | 04 No. Crossings (1 for each) |
| Gross Command Area | GCA-503919 Acres |
| Culture-able Command Area | CCA-486378 Acres |
| Total Number of Outlets | 1720 |

## 1.1. Location

Rajanpur District is located in the extreme southwest part of Punjab, Pakistan (29:06N, 70:19E) with a geographical span of 12,319 km2. It is pertinent to mention here that District Rajanpur is one of the main district of Southern Punjab located at right side of the River Indus which is directly suffered by hill torrent floods and River Indus. The land of the District Rajanpur is sandwiched between River Indus and Suleman Mountains Range. According to the National Census Report 1998, it had a population of 1,103,618, and under the local administrative break up, whole district is divided into three Tehsils, Jampur (19 Union  Councils), Rajanpur (14 Union Councils), Rojhan Mazari (08 Union Councils) and a vast belt of provincially administered tribal area (PATA). Rajanpur Canals Division was commenced in 1966. The Irrigation network of this Division comprises of three main systems i.e. Dajal Branch, Link No.III and Kadra Canal. The Irrigation supply is received through Dera Ghazi Khan Canal, off taking from the right side of Tounsa Barrage D.G. Khan Canal, a non-perennial channel with an original design capacity of 8900 Cusec. The Capacity         D.G. Khan Canal was revised to 11549 Cusec, inclusive of 2 #. Silt ejector-having capacity of 2500 cusecs. The supply up to a maximum of 9500 cusecs is possible in D.G. Khan Canal with a partial supplementation through an inlet at Rd: 20 which is itself quite uncertain and fluctuates depending upon the rise or fall in the River discharge at this site.

## General Description

The entire irrigation network is under the direct hit of Hill Torrents of Koh Suleman Range. The main torrents are Kaha, Kala Bagga Khosra, Chachar, Pitokh, Zangi, Sori Shumali & Sori Janoobi. The quantum & velocity of the flood water cannot be ascertained by normal procedure but losses can be minimized by taking prompt action. Dajal Branch, Kadra Canal and D.K.C. along with Distys are under the direct hit of Kaha, Kala Bagga Khosra, Chacher, Pitok, Zangi, Sori Shumali and Sori Janoobi Hill Torrents. As earlier mentioned that the quantum and velocity of this flash flood water cannot be ascertained by normal procedure, therefore full protective and precautionary measures have to be adopted to cater these torrential floods during entire Monsoon Season so that minimum losses have to be faced.

As per previous record of Hill Torrents, flood water accumulates in shape of pond on the right side of the Dajal Branch. Hill Torrent structures have been provided at various active routes over Dajal Branch to drain off flood water into River Indus through their traditional routes. These Hill Torrent structures are the control point of this Division. These traditional routes have been shown on the attached index plan. The torrential flood also damages Zam Zam, Abe Hayat, Hajipur, Fatehpur, Sadiq Feeder, Sadiq Disty, NDC, Islampur, Taskeen, DKC, Mubarik Distys, Tariq Minor and their allied systems along with standing crops, private properties and government infrastructures on its way. It also attacks on D.K.C. at RD: 65+000 to 120+000/R just after crossing at RD: 94+750. Ultimately, this flood water proceeds to storm water drain namely Qutab Drain and out falls in River Indus.

Kachhi Canal has been constructed all along on the right side of Dajal Branch. Before the construction of Kachhi Canal, the right bank of Dajal Branch was the defense bank against Hill Torrent flood. After the construction of Kachhi Canal, the 1st defense is the right bank of Kachhi Canal. The flood water is supposed to pass through Hill Torrent crossing of Kachhi Canal and Dajal Canal. After passing through these crossing structures, this flood ultimately falls in River Indus by adopting traditional routes. Unfortunately, the incomplete banks and crossing structures on Kachhi Canal are behaving adversely.

The accumulated flood water along right bank of Kachhi Canal gushed through the gaps of incomplete / weak banks during flood 2022 and through the incomplete X-ing structures of Kachhi Canal. This uncontrolled flood water breached from various places and caused damage to these banks and X-ing was badly chocked. In flood 2022 hill torrent crossing Syphon-145 was collapsed. Tender has been floated for its rehabilitation / reconstruction. Other damages to canals have been repaired and tenders for repairing concrete panels has been floated. Dajal Branch Canal will remain under threat of Hill Torrent floods so long as the structures and banks of Kachhi Canal are not protected properly.

- Dajal Branch consists of Kousar Disty, 1-L Kousar Minor, Zam Zam Disty,    1-L Zam Zam,   1-R Zam Zam Minor, Noushera Disty, 1L Noushera Minor,   1-R Noushera Minor, Hajipur Disty, 2-R Hajipur, 1-R Hajipur, Islampur Disty, Abehayat Disty, Chang Minor, Channer Minor, Fatehpur Disty, 1-L Fatehpur Minor, Taskeen Disty, NDC, Jalalpur Disty, Jalalpur Minor, Sadiq Feeder, Sadiq Disty, Jahanpur Minor having canal command area.

- Link No. III consists of Lunda Disty, Mehray Wala Disty, Kauray Wala Disty, Hazrat Wah, Upper Talai Disty, Lower Talai Disty, Noorpur Disty, Dhundi Kutab Canal Pir Bux Minor, Murad Abad Minor, Bakhat Minor, Barera Disty, Fazil Disty, Bosan Minor, Kalu Disty, Hazoori Minor, Raj Disty, Mubarik Disty, Asni Minor having Canal Command Area.

- Kadra Canal consists of Tariq Minor, Bukhari Disty, Hamid Disty, Forest Disty, Hussain Disty, Jadu Minor, Nazik Disty, Wang Disty, Fareed Disty, Fakhar Minor, Bhagsari Disty, Bhagsari Minor, Umerkot Disty, Umerkot Minor, Hidayat Disty, Saleemabad Disty, Safdar Abad Disty, Rojhan Disty, Matwah Disty, Ghous Bux Minor, Machka Minor having canal command area.

## Recommendation & Proposals:

Following recommendations and proposals are rendered for better flood fighting and planning: -

- A number of sites at Dajal Branch are recommend as relief cut sites during flood emergency RD: 123+000/L, RD: 145+000/L, RD: 186+000/L.

- Properly designed flood carrying channels should be constructed from Dajal Branch to River Indus edge.

- The capacity of Bukhari drain should be revised from 900 cusec to 3000 cusecs.

- 1 No. escape channel of about 1000 cusec is needed at RD: 145+000/L Dajal Branch to make its way through the old route of Kaha Hill Torrent.

- Raising & strengthening of Hajipur Disty from RD: 0-tail.

- Proposed new flood bund from tail of Fatehpur Disty to tail of Sadiq Disty. (Estimate submitted for this proposal by this division).

- Maintaining and repairing flood embankment of Dajal Branch Right side from RD: 99 to 180.

- The banks of Distributaries / Minors, where hill torrent flood water hits and cause severe damage should be raised as per FFC criteria to save the abadies and precious human lives.

## Administration Setup of Rajanpur Canal Division, Rajanpur

EXECUTIVE ENGINEER

HEAD VERNICULAR

CLERK

HEAD DRAFTSMAN

HEAD CLERK

ACCOUNTS OFFICER

S.D.O.

ROJHAN

DEPUTY COLLECTOR

S.D.O.

HAJIPUR

S.D.O.

TALAI

S.D.O.

KADRA

SUB ENGINEER

KOUSAR

SUB ENGINEER

KOTLA HASSAN SHAH

SUB ENGINEER

REIKH

SUB ENGINEER

DHUNDI

SUB ENGINEER

ISLAMPUR

SUB ENGINEER

KOTLA HIDAYAT

SUB ENGINEER

GABOOL

SUB ENGINEER

FAZILPUR

SUB ENGINEER

HAJIPUR

ZILLADAR

KOTLA HASSAN SHAH

ZILLADAR

KOTLA HIDAYAT

SUB ENGINEER

DHINGAN

SUB ENGINEER

RAJANPUR

SUB ENGINEER

FATEHPUR

ZILLADAR

KADRA

SUB ENGINEER

MUHAMMADPUR

SUB ENGINEER

JALALPUR

ZILLADAR

FAZILPUR

ZILLADAR

KOUSAR

ZILLADAR

RAJANPUR

ZILLADAR

HAJIPUR

ZILLADAR

JALALPUR

ZILLADAR

FATEHPUR

# Chapter No. 2

# Detail of River Training Works

## 2.1. Design Parameters of River Training Works

Four River Training Structure exist in Rajanpur Canal Division. All these river training structures exist at newly proposed under construction Flood Bund from Hairo to Raikh Bagh Wala Flood Bund. Instant flood bund comprises of 73+000 RDs. Total length of flood bund is 14.6 miles. Detail of River Training Works is as under: -

Table 2: River Training Works

| Sr. # | Description | Earthen Shank (Length ft) | J-Head Portion (Length ft) | Top Width (ft) | Side Slope | Design Top Level | Apron Top Level | H.F.L. 2022 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | J-Head Spur No. 1 (RD:27+000 of H-R FB) | 2850 | 680 | 30 | 2:1/2:1 | 355.11 | 338.32 | 348.11 |
| 2 | J-Head Spur No. 2 (RD: 43+000 of H-R FB) | 1000 | 680 | 30 | 2:1/2:1 | 351.43 | 334.64 | 344.43 |
| 3 | J-Head Spur No. 3 (RD: 54+000 of H-R FB) | 3000 | 680 | 30 | 2:1/2:1 | 348.9 | 332.11 | 341.9 |
| 4 | Sloping Spur - 4 (RD: 7+000 of H-R FB) | 3000 | - | 30 | 3:1/2:1 | 359.71 | - | 352.71 |

## 2.2. Location Map

Location Map of Rajanpur Canal Division and its river training works is attached herewith.

# Chapter No. 3

# Past Flood Events

## Brief History of Past Flood Events

Monsoon current over Koh-e-Suleman Range causes heavy stream flows from water shed areas of Kaha, Chacher, Pitok & Zangi Hill Torrents. This enormous surface run off, rushes towards the command area with furious velocity and momentum because of steep terrain slope. The heavy rains occur during the period from June to August. Floods of very high magnitude happened during the monsoon season 2008, 2010, 2012, 2013 & 2022. The flood observed during July-August-2022 was unprecedented historical which caused colossal damages to the public / private properties and infrastructures. There are 10 numbers of cross drainage structures on Dajal Branch i.e. 2 for Mithawan H/T, 6 for Kaha & 2 for Chachar Hill Torrents. The first four Hill Torrent x-ings structures come under the supervision of D.G. Khan Canal Division. The history of flood water peak discharges for the last 6 years is given below:-

Table 3: Data of Past Flood Events

| Sr. No | Name of Hill Torrent | Name of Channel | Crossing Point RD: | Des. Capacity Cs | Peak Discharges (In Cs.) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aug 2019 | March 2020 | July 2021 | July-Aug 2022 | July 2023 | July 2024 |
| 1 | Kaha | Dajal Br. | 109+760 | 1000 | 234 | -- | -- | -- | -- | -- |
| 2 | Kaha | Dajal Br. | 123+850 | 5000 | 3089 | 1584 | -- | 1296 | -- | 2214 |
| 3 | Kaha | Dajal Br. | 145+760 | 7000 | 3026 | 2376 | -- | >10000 | -- | -- |
| 4 | Kaha | Dajal Br. | 165+760 | 2000 | 93 | 744 | 297 | 744 | -- | 888 |
| 5 | Chachar | Dajal Br. | 177+300 | 2000 | 436 | 888 | -- | 1600 | -- | 1600 |
| 6 | Chachar | Dajal Br. | 186+100 | 10000 | 5468 | 6747 | -- | 11580 | 2540 | 7790 |
| 7 | Kaha & Chachar | D.K.C. | 94+750 | 2000 | 1890 | 2079 | -- | 1890 | 1620 | -- |
| 8 | Kaha & Chachar | Raj Disty | 9+500 | 2000 | -- | -- | -- | -- | -- | -- |
| 9 | Kaha & Chachar | Hamid Disty | 18+350 | 2000 | -- | -- | -- | -- | -- | -- |
| 10 | Kaha & Chachar | Kadra Canal | 64+400 | 2000 | -- | -- | -- | -- | -- | -- |

The excessive flood discharge caused a number of breaches in Dajal Branch and other irrigation channels during July-August-2022 Hill Torrent Flood. The detail of breaches and inundation caused due to these breaches has been depicted in the plan attached in Part ‘B’ of Flood Fighting Plan.

## Detail of Hill Torrent Crossings

### 1. Kaha Hill Torrent

The flow of this Hill Torrent comes at Dajal Branch at the following x-drainage structure.

  - Crossing RD. 109+760

Probably, the design discharge may pass once a time in the period of 3 years. The peak discharge recorded at this Crossing was 720 Cs in year 2015. Its design capacity is 1000 Cs.

  - Crossing RD. 123+850

This crossing drainage structure is the main Crossing which remains under the direct hit of flood water throughout flood season, the peak discharge recorded at this x-ing was 7989 Cs in 1994. The flood water caused damages to the upstream pitching. The design capacity of this crossing is 5000 Cs. This crossing remains under the hit of floodwater throughout flood season every year. The flood water by passed through breaches occurred on u/s & d/s of the   x-ing during flood 2022. The excessive flood water caused the breaches in the left bank of Dajal Branch. The muddy flood water also chocked the barrels of x-ing and canal bed, which have been cleared recently.

  - Crossing RD. 145+760

This cross drainage structure is important structure which receives storm water from Kaha Hill Torrent. Its designed capacity to pass flood water is 7000 Cs. but during the year 2022, the discharge more than of 10000 Cs passed through this Crossing and structure was collapsed. Accordingly work for restoration of hill torrent crossing was commenced back in April 2023. Till date, 50% of the work has been completed.

- Crossing RD. 165+760.

The design capacity of this Crossing is 2000 Cs and 1367 Cs was received at this crossing during the flood 2022.

### 2. Chachar Hill Torrent

- Crossing RD. 177+300.

This cross drainage structure is important structure which receives storm water from Chachar Hill Torrent. Its designed capacity to pass flood water is 2000 Cs. and more than 3200 Cs was received and it over topped the syphon crossing structure during the flood 2022. An additional syphon downstream crossing RD: 177+300 of Dajal Branch has also been constructed.

- Crossing RD. 186+100

This crossing is important structure which receives storm water from Chachar Hill Torrent. Its designed capacity to pass flood water is 10,000 Cs and 11580 Cs was received at this crossing during the flood 2022. The rushing water out flanked its right side and forcefully entered into Dajal Canal. The breaches occurred have been repaired immediately to pass the irrigation supply. The muddy flood water also chocked the barrel x-ing and canal bed, which have been cleared.

### 3. Pitok & Zangi Hill Torrent

The floodwater of this Torrent directly hit Kadra Canal in between RD.131+000 to 223+500 & Matwah Disty from head to tail and serious damages occurred to the Kadra Canal during flood 2022. In Kadra Canal and Matwah Disty many breaches occurred as huge quantum of floodwater attacked certain reaches during the flood 2022. Maximum discharge received from Pitok Hill Torrent was 5000 Cs, Sori Shumali Hill Torrent was 7000 Cs, Zangi Hill Torrent was 9000 and Sori Janubi 17000 Cs. The storm water of Kaha and Chacher joined together between RD.65-103/R of Dhundi Kutab Canal, which has already been converted into flood embankment. Just after crossing through Crossing at RD. 94+750 of Dhundi Kutab Canal, it hit the left bank of D.K.C. between RD. 95-104. The flood water then passed through crossing at RD. 95+00 Raj Disty where it damaged Raj Disty between RD.7000-9500/R. By Breaching D.K.C between RD. 95-104/L some water enters in the D.K.C so long as floodwater entry remains within the Canal capacity it is utilized to feed tail system of D.K.C but uncontrolled entry causes Breaches in the Canal. The right bank of D.K.C.   RD.104-142 was shattered badly, during flood 2022. Therefore to save Rajanpur city and Dhundi Kutab Canal, some relief cuts were made in the right bank of Mubarik Disty. The breaches in D.K.C and Raj Disty were closed to restore Canal water supply. The banks of DKC from RD: 15+760 to 33+500, RD: 33+500 to 60000/L&R and RD: 60+000 to 91+000/L have been raised to protect canal from damages during flood.

# Chapter No. 4

# Flood Limits

## 4.1. Flood Limits

Table 4: Flood Limits of Chenab River

| Sr. No. | Discharge Lac (Cusec) Limit | Flood Stage |
| --- | --- | --- |
| 1 | Below 1.50 | Normal |
| 2 | 1.50 or above but below 2.00 | Low |
| 3 | 2.00 or above but below 3.00 | Medium |
| 4 | 3.00 or above but below 4.50 | High |
| 5 | 4.50 or above but below 6.00 | Very High |
| 6 | 6.00 or above | Exceptionally High |

Table 5: Flood Limits of River Indus

| Sr. No. | Discharge Lac (Cusec) Limit | Flood Stage |
| --- | --- | --- |
| 1 | Below 2.50 | Normal |
| 2 | 2.50 or above but below 3.75 | Low |
| 3 | 3.75 or above but below 5.00 | Medium |
| 4 | 5.00 or above but below 6.50 | High |
| 5 | 6.50 or above but below 8.00 | Very High |
| 6 | 8.00 or above | Exceptionally High |

## 4.2. Time Lags of Floods

Table 6: Time lag of flood events of river Chenab

| Sr. No. | Name of Head Work | Distance in KM | Time in Hour |
| --- | --- | --- | --- |
| 1 | Marala to Khanki | 56 | 12 |
| 2 | Khanki to Qadirabad Head Works | 30 | 7 |
| 3 | Qadirabad Head Works to Trimmu | 214 | 72 |
| 4 | Trimmu Barrage to Tail T.P Link | 162 | 48 |
| 5 | Tail T.P.Link to Sher Shah Bridge | 32 | 10 |
| 6 | Sher Shah to Punjnad Head works | 63 | 20 |
| 7 | Punjnad Head works to Mithan Kot | 78 | 24 |

Table 7: Time Lag of Flood Events in River Indus

| Sr. No. | Name of Head Work | Distance in KM | Time in Hour |
| --- | --- | --- | --- |
| 1 | Attock to Kala Bagh Head works | 106 | 36 |
| 2 | Kala Bagh H/works to Chashma Barrage | 61 | 24 |
| 3 | Chashma Barrage to Taunsa Barrage | 152 | 48 |
| 4 | Taunsa Barrage to Ghazi Ghat | 40 | 12 |
| 5 | Ghazi Ghat to Mithan Kot | 89 | 28 |
| 6 | Mithan Kot to Guddu Barrage | 59 | 24 |

## 4.3. Highest Floods

CHENAB RIVER

The maximum discharge received down stream

Trimmu Head works on 08.07.1959 				= 943212 Cusecs

INDUS RIVER

The maximum discharge received down stream

Taunsa Barrage on 02.08.2010 				= 959177 Cusecs

## 4.4. Peak Discharges

Table 8:  Peak Discharges D/S of Trimmu Barrage

| Year | Date | Discharge (in Cs) |
| --- | --- | --- |
| 2011 | 20-09-2011 | 127790 |
| 2012 | 07-08-2012 | 67694 |
| 2013 | 20-08-2013 | 234487 |
| 2014 | 14.09.2014 | 767849 |
| 2015 | 31.07.2015 | 138000 |
| 2016 | 11.8.2016 | 132525 |
| 2017 | 26.07.2017 | 26979 |
| 2018 | 17.08.2018 | 266123 |
| 2019 | 22.08.2019 | 93021 |
| 2020 | 01.09.2020 | 196067 |
| 2021 | 31.07.2021 | 123167 |
| 2022 | 19.08.2022 | 122115 |
| 2023 | 26.07.2023 | 116786 |
| 2024 | 19.08.2024 | 87030 |

Table 9: Peak Discharges on Indus River D/S Taunsa Barrage

| YEAR | DATE | Discharge (in Cs) |
| --- | --- | --- |
| 2011 | 01-09-2011 | 227689 |
| 2012 | 10-08-2012 | 235389 |
| 2013 | 17-08-2013 | 468048 |
| 2014 | 18-07-2014 | 247412 |
| 2015 | 05.08.2015 | 597730 |
| 2016 | 08.07.2016 | 343024 |
| 2017 | 07.08.2017 | 418388 |
| 2018 | 28.08.2018 | 252725 |
| 2019 | 17.08.2019 | 373000 |
| 2020 | 09.02.2020 | 464560 |
| 2021 | 04.08.2021 | 304121 |
| 2022 | 31.08.2022 | 624159 |
| 2023 | 27.07.2023 | 439908 |
| 2024 | 11.08.2024 | 435170 |

## Time Lags of Floods

Lag time between Taunsa to Kot Mithan is 	45 hours

Lag time between Panjnad to Kot Mithan is 	24 hours

### Data of Hill Torrents (Highest Floods)

Table 10: Highest Floods at Hill Torrents X-ings from 2017 to 2023

| Sr. No | Name of Hill Torrent | Name of Channel | Crossing Point RD | Des. Capacity | Peak Discharges (In Cs.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cusec | July 2018 | Aug 2019 | March 2020 | July 2021 | Jly-Ag: 2022 | Jly- 2023 | Jly-2024 |
| 1 | Kaha | Dajal Br. | 109+760 | 1000 | -- | 234 | -- | -- | -- | -- | -- |
| 2 | Kaha | Dajal Br. | 123+850 | 5000 | -- | 3089 | 1584 | -- | 1296 | -- | 2214 |
| 3 | Kaha | Dajal Br. | 145+760 | 7000 | 1543 | 3026 | 2376 | -- | 6100 | -- | -- |
| 4 | Kaha | Dajal Br. | 165+760 | 2000 | -- | 93 | 744 | 297 | 744 | -- | 888 |
| 5 | Chachar | Dajal Br. | 177+300 | 2000 | -- | 436 | 888 | -- | 1600 | -- | 1600 |
| 6 | Chachar | Dajal Br. | 186+100 | 10000 | -- | 5468 | 6747 | -- | 11580 | 2540 | 7790 |

Table 11: Peak Discharges of Hill Torrents Observed at Darrahs

| Sr. | Name of Hill Torrent | Design Capacity | Peak Discharges (In Cusecs) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No |  | (Cs) | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 |
| 1 | Kaha at Darra | 118500 | 46855 | 40905 | 31032 | 31032 | 84100 | 41193 | 21158 | 108941 | 35968 | 105276 |
|  | (Kala Khosra + Bagga Khosra) |  |  |  |  |  |  |  |  |  | 5500 | 27640 |
| 2 | Chachar at Darra | 85500 | 30800 | 24300 | 5250 | 3250 | 14500 | 21158 | 3250 | 75900 | 15000 | 63940 |
| 3 | Pitok at Darra | 23900 | 700 | 2560 | 0 | 0 | 1150 | 0 | 0 | 5000 | 1200 | 14600 |
| 4 | Sori Shumali at Darra | 7713 | 1300 | 2900 | 0 | 0 | 1800 | 2500 | 0 | 7000 | 1000 | 5845 |
| 5 | Zangi at Darra | 3500 | 600 | 4750 | 2400 | 5850 | 2800 | 3500 | 0 | 9000 | 3500 | 33600 |
| 6 | Sori Janobi at Darra | 41600 | 3000 | 3300 | 5000 | 2170 | 4047 | 9200 | 7000 | 17000 | 3500 | 16560 |

Table 12: Peak Discharges of Hill Torrents during Flood 2024

| Sr. No. | Name of Torrents | Discharge | Time | Dated |
| --- | --- | --- | --- | --- |
| 1 | Kaha + Kala Bagga | 132916 | 12:00 Hours | 18.08.2024 |
| 2 | Chachar | 63940 | 14.50 Hours | 18.08.2024 |

Table 13: Historical Peaks of River Indus, Chenab and Kaha Hill Torrent

| Sr. No. | Taunsa Barrage 02.08.2010 21:00 Hrs | Punjnad 25.08.2013 06:00 Hrs | Kaha Hill Torrent 14.08.2022 17:00 Hrs |
| --- | --- | --- | --- |
| 1 | 959177 Cs | 650000 Cs | 108941 Cs |

# Chapter No. 5

# Flood Fighting Strategy

The Executive Engineer, Rajanpur Canal Division, Rajanpur will be over all in-charge and will stay at Jampur Canal Rest House during flood days in River Indus. He will take appropriate action according to the emergency.

By 1st of June, the strength of the existing establishment will be doubled by employing work charge establishment to clear the jungle on slopes and toe of the embankments shanks etc of the spurs. The extra establishment will be deployed for the detection and removal of holes of burrowing animals. These holes will be removed before 30th June-2024.

In emergency, the flood information is received from upper region. The Sub Divisional Officer and Sub Engineer in-charge will shift their headquarters at site. The establishment will be deployed according to the flood stage and as prescribed, round the clock vigilant watching will be arranged along the entire embankment to take care of eventualities. The staff will be equipped with chargeable torches, cell torches, gunny bags, kassies and any other relevant material to control seepage / leakage in the flood bund / embankment. The information is passed on quickly to the concerned quarter and simultaneous action will be taken to close the same. In the event of exceptionally high flood stage the heavy earth moving machinery (Bull Dozers) private and Agricultural Engineering Department / Machinery Circle will also be kept ready and placed on alert at vulnerable sites of Dajal Branch RD: 140-145/L+R, RD: 166-167/L+R, Patwali Minor RD: 3-4/LR, Zam Zam Disty RD: 7148-15240/R, Islampur Disty RD: 23000/LR, Hajipur Disty RD: 11-34/LR, RD: 44-45/LR, Taskeen Disty RD: 13-15/LR, Fatehpur Disty RD: 7-8/LR, Link No. III RD: 168-170/L, RD: 193-194/L, 223-224/R, DKC RD: 60000-123000/R, Mubark Disty RD: 4000-5000/R, Kadra Mubarik Flood Bund RD: 6000 - 70000, RD: 18000 – 200000, Kadra Canal RD: 151-152/R, RD: 193-195/R, RD: 207-220/R, Tariq Mr RD: 17-18/R, Matwah Disty RD: 32-33/LR, RD: 34-59/LR and Fakhar Minor RD: 6500-11300/LR flood embankments for the safety of the public private e infrastructures, land and abadies of District Rajanpur. In emergency, relief cuts will be made whenever required according to the site situation.

STAGE-1

- Normal Floods (Below 2.5 Lac Cs)

When river discharge ranges between these limits, flood water remains within the lips of River, it does not touches any flood bund or embankment, watching establishment shall be deployed at the rate of one man per mile/per shift for two shifts in a day. Sub Engineers concerned will supervise the entire watching operation along Spurs and Flood embankments. Sub Divisional Officer concerned will act as the officer in-charge of watching operation.

STAGE-II

ii-   Low Floods (2.50 Lac Cs: to 3.75 Lac Cs)

When the discharge ranges between these limits, flood water remains within the lips of River, it does not touches any flood bund or embankment, watching establishment shall be deployed at the rate of one man per mile/per shift for two shifts in a day. Supervisory staff shall be the same as in stage-I.

STAGE-III

iii-  Medium Floods (3.75 Lac Cs: to 5.00 Lac Cs)

At this stage flood water starts touching the flood bunds / embankments. Watching establishment shall be deployed at the rate of three men per mile/per shift for three shifts in a day. Supervisory staff shall be the same as in stage-II.

STAGE-IV

iv-  High Floods (5.00 Lac Cs: to 6.50 Lac Cs)

Watching establishment shall be deployed at the rate of four men per mile/per shift for three shifts in a day. Supervisory staff shall be the same as in stage 1, their headquarters will be shifted at most critical points.

STAGE-V

v-   Very High Floods (6.50 Lac Cs: to 8.00 Lac Cs)

Watching establishment shall be deployed at the rate of five men per mile/per shift for three shifts in a day. Supervisory staff shall be the same as in stage 1, their headquarters will be shifted at most critical points.

STAGE-VI

vi-   Exceptionally High Floods (Above 8.00 Lac Cs)

At this stage, establishment at rate of five men per mile along with volunteer labours brought by Civil Administration and Army will help the watching operation. Police Guards will be deputed on vulnerable points to guard against any mishap/sabotage. Overall supervision of all operations will be done by the Executive Engineer.

All the flood gauges from upper region are received through the wireless sets already installed at various Police Stations in the District. To control the flood and to meet with the emergencies wireless sets and V.II.F. Telephones have to be provided in the area for quick information and proper monitoring. The key points and vehicles in which wireless sets and V.H.F have to be installed are given below: -

## Key Points for V.H.F Telephone/Wireless.

- Rajanpur Canals Rest Houses.

- Jampur Canal Rest Houses.

- Dajal Canal Rest Houses.

- Kotla Hidayat Rest Houses.

- Miranpur Canal Rest Houses.

- Fazilpur Canal Rest Houses.

- Tail of Dajal Branch.

## Wireless Sets for Officer’s Vehicle During Flood Emergency

- Vehicles of Superintending Engineer, Derajat Canal Circle, (Irrigation).

- Vehicles of Executive Engineer, Rajanpur Canal Division, Rajanpur.

- No. Vehicles being used by SDO Talai, Kadra, Rojhan & Hajipur Sub Division.

In case of emergency, the Telephone Nos. given in this plan may please be contacted during flood days. During the critical situation, flood gauges are to be supplied twice a day, by each Sub Engineer and SDOs to the nearest communication.

- Gauges of Hill Torrents at Darrahs provided by Sub Divisional Officer Hill Torrent Sub Division.

- Gauges of Hill Torrents at x-ings provided by Sub Divisional Officer Hajipur Sub Division.

- Gauges of Hill Torrents at x-ings at RD: 95+000 DKC provided by Sub Divisional Officer Talai Sub Division.

## Conveyance

To meet with alarming situation and flood emergencies, each Sub Divisional Officer in Rajanpur Canals Division will be by equipped with Government vehicle. About 5 No. Dozer will also be arranged to control the situation at site.

## Mobile Dispensaries

4 Nos. mobile dispensaries are needed during flood emergencies in Rajanpur District at various places in the effected areas. District Health Officer, Rajanpur has to arrange all the Medical Facilities regarding dysentery Malaria, Cholera etc. during the flood days in Rajanpur District.

## Excavators / Tractor Trolleys

The concerned SDO will be responsible for managing / providing 02× Excavator & 04× Tractor Trolley with front blade in their respective Sub Divisions to be maintained at critical reaches to protect the flood embankment, Irrigation Channels and to meet with emergent situation during ensuring hill torrent flood.

## Boats

Relates to Administrative Department.

# Chapter No. 6

# Flood Damages Restoration Works

## Flood Damages Occurred in Flood 2024

During hill torrent flood 2024 and riverine flood 2024, following damages occurred to embankment structures and river training structures in Rajanpur Canal Division, Rajanpur.

Table 14: Flood Damages occurred in Flood 2024

| Sr. No. | Name of Flood Embankment | Damaged Occurred |
| --- | --- | --- |
| 1. | Flood Bund from Hairo to Raikh Bagh Wala Flood Bund | Erosion of Flood Embankment due to direct hit of Zookin Creek at RD 60+000 to 65+000 in a length of 5000 feet. |
| 2. |  | Restoration of Apron J-Head Spur No. 01 at RD: 27+000 of Raikh Bagh Wala Flood Bund during flood season 2024. |
| 3. |  | Restoration of Apron J-Head Spur No. 03 RD: 54+000 of Raikh Bagh Wala Flood Bund during flood season 2024. |
| 4. | Right Flood Embankment of Kadra Canal from RD 129+000 to 219+500. | Slopes of Flood Embankment Damaged during the Hill Torrent Flood 2024. |

## Flood Restoration Works Post Flood 2024

Accordingly, this division has taken up following flood restoration works for the year 2024-25. These works are liable to be completed before flood season 2024.

Table 15: Flood Restoration Works taken up after Flood 2024.

| Sr. No. | Name of Work | T.S. Cost (Rs. M) | Status |
| --- | --- | --- | --- |
| 1. | Restoration of Stone Apron of Zokin Creek Damaged during flood 2024. | 4.340 | Tender Under Process |
| 2 | Restoration of stone Apron of J-head Super No.1 Damaged During Flood 2023. | 21.346 | Tender Process |
| 3 | Restoration of stone apron of JHS-3 Damaged during flood 2024. | 23.667 | Tender Under Process |
| 4 | Strengthening Right Flood Embankment of Kadra Canal from RD:129+000 to 219+500 & its protection by providing stone pitching at selective reaches in order to protect tehsil Rojhan from Torrential Flood water. | 450.860 | Tender Process |

## Flood Fighting Watching Arrangements

This canal system of Rajanpur Canal Division is in the direct Hit of Hill Torrent Flood which often causes heavy breaches and ruins main crops, adjoining abadies and roads net-work of this area. During flood days, the supply is almost doubled from the assumed supply, which runs through channels due to passage of flood water from the channels of Rajanpur Canal Division. Flood fighting is ensured by day and night watching at the critical reaches of all the channels through work charge and regular establishment.

# Chapter No. 7

# Pre-Flood Arrangements of Rajanpur Canal Division

## 7.1. Pre-Flood Arrangements

The Executive Engineer, Irrigation, Canal Division and Executive Engineer, Jampur Construction Division Rajanpur will establish pre-flood information emergency centers equipped with Telephone, Wireless Set and Fax at District Headquarters. The Centre will function round the clock. They will be responsible to inform from time to time about the intensity of flood and areas which may be affected. The Officer In-charge would be responsible to submit information to the Deputy Commissioner, Rajanpur. Executive Engineer, Jampur Construction Division will be responsible to make the arrangement all the time to let flood water go on its traditional routes to fall in the Indus River.

Consolidated status of reserve stock.

| SR No. | Name of Division | Reserve Stock Limit (as per yard stick) | Minimum Requirement 70% | Quantity Available | Balance Quantity Required | Tentative Cost of Stone (Rs. In Million) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Rajanpur Canal Division | 3.000 | 2.100 | 1.300 | 0.800 | 12.10 | Tenders Called |

## 7.2. Watching Establishment

During the flood season, heavy rainfall occurs in the catchment area of hill torrents. This rainy water passes through the X-ings of Dajal Branch and attacks the adjoining abadies and crops. It also causes breaches in channels due to over flow which is watched by watching establishment. All of this is possible by watching day and night at the critical reaches of Rajanpur Canal Division, Rajanpur through work charge and regular establishment during period 15.06.2025 to 15.10.2025. Detail of watching establishment and flood fighting material as mentioned in 7.3 and 7.4.

## 7.3. Arrangements at Sensitive Sites

All establishments will be employed on daily wages through government contractors.

  - Before Flood: Flood preparation will be done during the month of June/July to prepare all flood embankments / structures by employing establishment and essential machinery as per site requirements.

  - During floods: During floods essential establishment will be employed with the following scale.

Table 16: Detail of Employing Field Establishment during Flood 2024

| Sr. No | Name of Sub Div: | Name of Section | Description | Length In mile | Strength of establishment required |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mistry | Mate | R-Beldar | G/R | Beldar During Flood |
| 1 | Hajipur | Kousar | Dajal Br. RD: 100-120 Patwali Mr. RD: 0-11 Zam Zam Dy. RD: 7-15 | 7.80 | 1 | 1 | 4 | 1 | 20 |
| 2 |  | Islampur | Dajal Br. RD: 120-150 Islampur Dy RD: 16-24 | 7.60 | 1 | 1 | 4 | 1 | 15 |
| 3 |  | Hajipur | Dajal Br. RD: 150-180 Hajipur Dy. RD: 0-49 | 15.80 | 1 | 1 | 10 | 1 | 20 |
| 4 |  | Fatehpur | Dajal Br. RD: 180-190 Taskeen Dy. RD: 4-10 Fatehpur Dy. RD: 0-26 | 8.40 | 1 | 1 | 8 | 1 | 20 |
| 5 |  | Syphon Sites | Syphons in Hajipur Canal Sub Division | -- | 1 | 1 | -- | 6 | 120 |
|  |  |  | Total : | 39.60 | 5 | 5 | 26 | 10 | 195 |
| 6 | Talai | Rajanpur | DKC RD: 90-103 Mubarik Disty RD: 0-56500 | 16.50 | 1 | 1 | 6 | 1 | 19 |
| 7 |  | Rajanpur | Kadra Mubarik Flood Embankment RD: 26 | 4.92 | 1 | 1 | 1 | 1 | 13 |
| 8 |  | Fazilpur | DKC RD: 55-90 | 7.00 | 1 | 1 | 4 | 1 | 8 |
|  |  |  | Total : | 28.42 | 3 | 3 | 11 | 3 | 40 |
| 9 | Kadra | Gabool | Hairo to Raikh Bagh Wala Flood Bund | 14.60 | 1 | 1 | -- | 1 | 20 |
|  |  |  | Total | 14.60 | 1 | 1 | -- | 1 | 20 |
| 10 | Rojhan | Kotla Hassan Shah | Kadra Canal RD:131-192 | 11.55 | 1 | 1 | 9 | 1 | 20 |
| 11 |  | Kotla Hedayat | Kadra Canal RD: 192-223 Matwah Dy RD:0-59450 | 18.09 | 1 | 1 | 7 | 1 | 20 |
|  |  |  | Total | 29.64 | 2 | 2 | 16 | 2 | 40 |

Note: Payment for deployment of machinery will be made subject to verification by TPM / Consultants.

## 7.4. Watching Material

As informed by the Sub Engineers, in the stock of Rajanpur Canal Division, quantity of Flood Fighting Material is not enough to cope with the serious flood condition. So, material listed below will be purchased locally as per site conditions. Estimates have been submitted in this regard. Following material is required in coming flood season 2024 for Rajanpur Canal Division, Rajanpur.

Table 17: Detail of Flood Fighting Material for Flood 2024

| Sr. No. | Flood Material | D/S, U/S RD: 123 | RD: 145 | RD: 177300 | RD: 190 186-187 | 9-10 Raj, Mubarik | DKC 103-104 | H-R FB with Spurs | Kadra 142-143 | Gabool Section & Kotla Hedayat Section | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Kousar Section | Islampur Section | Hajipur Section | Fatehpur Section | Rajanpur Section | Fazilpur Section | Gabool Section (K.C.) | KHS Sec: |  |  |
| 1 | Led Light Chargeable | 10 No | 10 No | 10 No | 10 No | 10 No | 10 No | 20 No | 10 No | 10 No | 100 No. |
| 2 | Electric Cable7/029 | 100 Yrd | 100 Yrd | 100 Yrd | 100 Yrd | 100 Yrd | 100 Yrd | 300 Yrd | 100 Yrd | 100 Yrd | 1000 Yrd |
| 3 | Energy Saver | 15 No | 15 No | 15 No | 15 No | 15 No | 15 No | 45 No | 15 No | 15 No | 165 No. |
| 4 | Swiss Cottage | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 4 No. | 1 No. | 1 No. | 12 No. |
| 5 | Chair Table Set | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 4 No. | 1 No. | 1 No. | 12 No. |
| 6 | Water Cooler Set | 3 No. | 3 No. | 3 No. | 3 No. | 3 No. | 3 No. | 4 No. | 3 No. | 3 No. | 28 No. |
| 7 | Gunny Bags | 600 No. | 600 No. | 1200 No. | 600 No. | 1200 No. | 600 No. | 2000 No. | 600 No. | 600 No. | 8000 No. |
| 8 | Khaji Mats. | 100 No. | 100 No. | 100 No. | 100 No. | 100 No. | 100 No. | 400 No. | 100 No. | 100 No. | 1200 No. |
| 9 | Larmal 50 | 100 ft | 100 ft | 150 ft | 100 ft | 150 ft | 100 ft | 500 ft | 100 ft | 100 ft | 1400 ft |
| 10 | Basket | 100 No. | 100 No. | 150 No. | 100 No. | 150 No. | 100 No. | 200 No. | 100 No. | 100 No. | 1100 No. |
| 11 | Munj Ban. | 50 Kg | 50 Kg | 50 Kg | 50 Kg | 50 Kg | 50 Kg | 300 Kg | 50 Kg | 50 Kg | 700 No. |
| 12 | Sutli | 20 Kg | 20 Kg | 20 Kg | 20 Kg | 20 Kg | 20 Kg | 80 Kg | 20 Kg | 20 Kg | 240 No. |
| 13 | Kassi Handle | 50 No. | 50 No. | 50 No. | 50 No. | 50 No. | 50 No. | 150 No. | 50 No. | 50 No. | 550 No. |
| 14 | Axes with Handle | 50 No. | 50 No. | 50 No. | 50 No. | 50 No. | 50 No. | 100 No. | 50 No. | 50 No. | 500 No. |
| 15 | Jute Bags. | 70 No. | 70 No. | 100 No. | 70 No. | 100 No. | 70 No. | 500 No. | 70 No. | 70 No. | 1120 No. |
| 16 | Monj Patha Trungar | 1000 No. | 1000 No. | 1000 No. | 1000 No. | 1000 No. | 1000 No. | 2000 No. | 1000 No. | 1000 No. | 10000 No. |
| 17 | Generator | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 4 No. | 1 No. | 1 No. | 12 No. |
| Sr. No. | Flood Material | D/S, U/S RD: 123 | RD: 145 | RD: 177300 | RD: 190 186-187 | 9-10 Raj, Mubarik | DKC 103-104 | H-R FB with Spurs | Kadra 142-143 | Gabool Section & Kotla Hedayat Section | Total |
|  |  | Kousar Section | Islampur Section | Hajipur Section | Fatehpur Section | Rajanpur Section | Fazilpur Section | Gabool Section (K.C.) | KHS Sec: |  |  |
| 18 | Petrol | 300 Liter | 300 Liter | 300 Liter | 300 Liter | 300 Liter | 300 Liter | 1500 Liter | 300 Liter | 300 Liter | 3900 Liter |
| 19 | Bamboo | 1000 No. | 1000 No. | 1000 No. | 1000 No. | 1000 No. | 1000 No. | 4000 No. | 1000 No. | 1000 No. | 12000 No. |
| 20 | Choldari | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 1 No. | 4 No. | 1 No. | 1 No. | 12 No. |
| 21 | Motor Boat | 1 No. | 1 No. | - | - | - | - | - | - | - | 2 No. |
| 22 | Tube Light | 4 No. | 4 No. | 4 No. | 4 No. | 4 No. | 4 No. | 8 No. | 4 No. | 4 No. | 40 No. |
| 23 | Tea Set | 1 Set | 1 Set | 1 Set | 1 Set | 1 Set | 1 Set | 4 Set | 1 Set | 1 Set | 12 Set |
| 24 | First Aid Box | 1 Set | 1 Set | 1 Set | 1 Set | 1 Set | 1 Set | 4 Set | 1 Set | 1 Set | 12 Set |
| 25 | Umbrella | 3 No. | 3 No. | 3 No. | 3 No. | 3 No. | 3 No. | 4 No. | 3 No. | 3 No. | 28 No. |
| 26 | Life Jackets | 9 No. | 9 No. | 9 No. | 9 No. | 9 No. | 9 No. | 36 No. | 9 No. | 9 No. | 108 No. |
| 27 | Plastic Bags | 600 No. | 600 No. | 600 No. | 600 No. | 600 No. | 600 No. | 1800 No. | 600 No. | 600 No. | 6600 No. |

Table 18: Comparison of Available versus Required Flood Fighting Material

| Sr. No. | Required |  |  | Available |  | Balance Required |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Item | No. | Unit | No. | Unit | No. | Unit |
| 1 | Chargeable Flood Lights | 120 | No. | 10 | No. | 110 | No. |
| 2 | Chargeable Mini Lights | 165 | No. | 20 | No. | 145 | No. |
| 3 | Torches 3 Cells | 42 | No. | 15 | No. | 27 | No. |
| 4 | Cell For Torches. | 560 | No. | 0 | No. | 560 | No. |
| 5 | Gunny Bags | 8000 | No. | 0 | No. | 8000 | No. |
| 6 | Khaji Mates. | 1200 | No. | 12 | No. | 1188 | No. |
| 7 | Life Jackets | 108 | Kg | 0 | Kg. | 108 | Kg. |
| 8 | Basket | 1100 | No. | 50 | No. | 1050 | No. |
| 9 | Munj Ban. | 700 | Kg | 15 | Kg. | 685 | Kg. |
| 10 | Sutli | 240 | Kg | 0 | Kg. | 240 | Kg. |
| 11 | Kassi with Handle | 550 | No. | 0 | No. | 550 | No. |
| 12 | Axes with Handle | 500 | No. | 0 | No. | 500 | No. |
| 13 | Jute Bags. | 1120 | No. | 10 | No. | 1110 | No. |
| 14 | Trungar | 10000 | No. | 0 | No. | 10000 | No. |
| 15 | Generator | 12 | No. | 0 | No. | 12 | No. |
| 16 | Petrol | 3900 | Liter | 0 | Ltr | 3900 | Ltr |
| 17 | Bamboo | 12000 | No. | 12 | No. | 11988 | No. |
| 18 | Choldari | 12 | No. | 1 | No. | 11 | No. |
| 19 | Motor Boat | 2 | No. | 0 | No. | 2 | No. |
| 20 | Energy Saver (Large Size) | 40 | No. | 10 | No. | 30 | No. |
| 21 | Tea Set | 12 | Set | 0 | Set | 12 | Set |

## 7.5. Arrangement for Sounding / Probing

Not applicable in Rajanpur Canal Division, Rajanpur

## 7.63. Lighting Arrangements

During flood days, the complete system for lightening arrangement will be available in each camp site of concerned Sub Engineer.

## 7.7. Ration Arrangements

Not applicable in Canal Division, Rajanpur, it relates to Administration Department.

7.8. POL Arrangements for Vehicles

POL will be purchased for use of any kind of emergency during coming flood 2024 from the Petrol Pump by Rajanpur Canal Division, Rajanpur.

## 7.9. Transportation

To meet with alarming situation and flood emergencies, each Sub Divisional Officer in Rajanpur Canals Division has the facility of Government vehicles.

## 7.10. Law & Order

In any flood emergency, help of local police and rangers will be required to maintain law and order in district.

## 7.11. Medical Arrangements for Labor

Not applicable in Canal Division, Rajanpur, it relates to Administration Department.

## 7.12. Liaison with other Department

Executive Engineer will remain fully contacted with District Administration, District Police Officer, Flood Control Centre, all the Civil Departments and Army to combat any mishap occurring due to floods well in time and will also make liaison with IRSA authority, ministry of water and power as well as flood forecasting centers Lahore for collection of flood related information.

## 7.13. Role of Army

In case of exceptional flood, the help of Army will be required for safety of Rajanpur, Fazilpur & Kot Mithan Towns.

## 7.14. Duties of Telephone Attendant

All the flood gauges from upper region are received through the wireless sets already installed at various Police Stations in the District. To control the flood and to meet with the emergencies, cellular phones available with the filed staff of area will be used  for quick information and proper monitoring. Duties of telephone attendant as under:-

Table 19: Duty Roster of Signalers

| Sr. No. | Name of Official | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Abdul Ghafoor | 15.06.2025 to 15.10.2025 | 6:00 am | 6:00 pm |
| 2 | Allah Bakhsh |  | 6:00 pm | 6:00 am |
| 3 | Muhammad Sadique |  | 6:00 am | 6:00 pm |

The key points and vehicles in which wireless sets and V.H.F. have to be installed are given below: -

## Key Points for V.H.F Telephone/Wireless

- Rajanpur Canals Rest Houses.

- Jampur Canal Rest Houses.

- Dajal Canal Rest Houses.

- Kotla Hidayat Rest Houses.

- Miranpur Canal Rest Houses.

- Fazilpur Canal Rest Houses.

- Tail of Dajal Branch.

## 7.15. Wireless Arrangement (Installed by Police Department)

Wireless sets will be installed in the officer’s vehicles during flood emergency, as detail below:-

- Vehicles of Superintending Engineer, Derajat Circle.

- Vehicles of Executive Engineer Rajanpur Canal Division.

- 04 No. Vehicles being used by SDO Talai, Kadra, Rojhan & Hajipur Sub Division.

In case of emergency, during the critical situation, flood gauges are to be supplied twice a day, by each Sub Engineer, to the nearest communication.

# Chapter No. 8

# Detail of Encroachment

There are no critical encroachments in Rajanpur Canal Division, Rajanpur.

# Chapter No. 9

# Duty Roster / Flood Fighting Program

## Battle Stations of Key Personnel During High Flood Flow

All Officer’s / Official’s up to the level of Executive Engineer, will be vigilant and watch-full during flood days. The entire dangerous / weak sites will be frequently inspected. All the field staff will be deputed on their relevant duties during the flood days.

### Executive Engineer

The Executive Engineer is in-charge of the flood fighting programming in Rajanpur Canals Division. He will inspect all the flood bunds in flood area and issue necessary instruction to the lower staff time to time.

### Sub Divisional Officer

The Sub Divisional Officer is personally responsible for all the matters which occur in the Sub Divisions. He will be very watch full and vigilant regarding regulation flood gauges, flood embankment etc. and relief cuts. He will mobilize and activate his Sub-Ordinate staff for any required assignment/task during flood emergencies days. He is also duly bound to inform any type of mishap occurring in his sub Division to The Executive Engineer Rajanpur Canal Division, so that further necessary step at site in this regard may be taken.

### Sub Engineer

The Sub Engineer, in-charge will not leave his station head quarter without prior permission of higher officers during the flood emergency days. It is his responsibility to convey all the flood gauges to the higher authorities or nearest communication station during the flood days. He is duly bound to assign all important duties to his sub-ordinate staff prior to the flood season. He will watch and maintain all the flood bunds before the expected flood day so that he may be able to control the alarming situation. He will have very good relations with the concerned Zamindars/public & administration force, which are usually beneficial and help full in the flood days. All the data regarding flood will be submitted by him to up keep the record in Sub Divisional Officer. He will have to take immediate steps at site if required without waiting orders from the higher authorities to minimize the public and Government loss.

### Work Mistri/Mate Guage Reader/ Dak Runners/Signaler

During the flood days all the field staff including Mistri, Mate, Gauge Reader, Dak Runner, Signaler, Regulation Beldar and Beldar etc. are duly bound perform their duties day and night. They are not allowed to leave their respective place during flood emergency days. They will have to obey the orders issued by their seniors to watch the Government and public interest. The field staff regularly in their own assigned position will perform night watching and patrolling. Sub Engineer in-charge is bound to equip and furnish his field staff with all T&P articles prior to the flood season.

# Chapter No. 10

# Emergency Telephone Numbers

Following important telephone number of concerned officers/ officials of District Administration, Police Force, Irrigation Department and other allied departments will be contacted in case of any emergency.

Table 20: Telephone Directory for Rajanpur District

| Name of Officer | Designation | Office No. |  | Residence No. | Fax No | Official Mobile No. |
| --- | --- | --- | --- | --- | --- | --- |
| IRRIGATION DEPARTMENT |  |  |  |  |  |  |
| Wasif Khurshid | Secretary Irrigation |  | 04299212117-18 | 04299203037 | 04299212116 | 0304-0920033 |
| Rashid Aziz | Chief Engineer, D&F Zone, Lahore |  | 042-99733551 | 042-99230602 | 04299230731 | 0309-9970999 |
| Rana Zulfiqar Ali | Chief Engineer, Irrigation DGK Zone |  | 064-9260245 | -- | 064-9260246 | 0322-2228637 |
| Munir Ahmed Abbasi | Superintending Engineer, DJT Circle. |  | 064-9260247 | -- | -- | 0300-6920400 |
| Zeshan Ali | S.E., Project Circle D.G. Khan. |  | 064-9260248 | -- | 0649260426 | 0333-0508781 |
| Ameer Taimour | XEN (OP) Irrigation Zone, D.G. Khan. |  | 064-9260251 | -- | 0429260252 | 0332-6206393 |
| Nouman Baig | Executive Engineer, D.G.Khan Canal Div: I |  | 064-9260249 | -- | -- | 0300-6306140 |
| Mohsin Shabir | Executive Engineer, D.G.Khan Canal Div: II |  | -- | -- | -- | 0311-7861909 |
| Mujahid Kalim | XEN, Dera Const; Division D.G. Khan |  | -- | -- | -- | 0304-2616750 |
| Muhammad Munawer Iqbal | XEN, Jampur Const: Division, Rajanpur |  | 0604-690690 | -- | -- | 0300-0425111 |
| Umer Abdul Rehman | XEN, Rajanpur Canal Division Rajanpur |  | 0604-689064 | -- | 064689064 | 0333-1871816 |
| Nouman Baig | XEN, Taunsa Barrage Division, Kot Adu. |  | 066-2242796 | -- | 0662240296 | 0300-6306140 |
| -- | Flood Emergency Cell, Lahore |  | 042-99212116 | 042-5900049 | 0429212132 | 042-99212166 |
| -- | Flood Emergency Cell, Lahore |  | 042-7572091-93 | -- | 0427572092 | 042-99212166 |
|  | Director Flood, Lahore |  | 042-99231614 | 042-6653943 | -- | 0323-4864378 |
| Abid Hussain | Telegraph Office, DGK |  | 064-9260253 | -- | -- | 0336-6615223 |
| Muhammad Nasrullah | SDO-Mechanical |  | -- | -- | -- | 0333-3625555 |
| Qazi Faiz-ul-Hassan | SDO-HT-I |  |  | -- | -- | 0333-6431973 |
| Muhammad Mueed | SDO-HT-II |  |  | -- | -- | 0333-8821632 |
| Abdul Ghafoor Allah Bakhsh | Signaler |  | 0604-690690 | -- | -- | 0333-6439136 0333-6097657 |
| CIVIL ADMINISTRATION |  |  |  |  |  |  |
| Shafqat Ullah Mustaq | D.C. Rajanpur | 0604-920001 |  | -- | 0604-689131 | 0304-0920093 |
| Imran Mumtaz | ADCR Rajanpur | 0604-689495 |  | -- | 0604-920008 | 0300-4228878 |
| Farooq Amjad | D.P.O. Rajanpur | 0604-689055 |  | -- | 0604-920055 | 0305-699200 |
| Muhammad Siffat Ullah | ADCG Rajanpur | 0604-689288 |  | -- | 0604-00008 | 0333-8595003 0345-7077606 |
| Jamil Ahmad | A.C.RJP | 0604-689904 |  | -- | 0604-920020 | 0301-4057181 |
| Dr. Zulqarnain Saleem | A.C. Jampur | 0604-567509 |  | -- | -- | 0324-4229996 |
| Amir Nawaz | A.C. Rojhan | 0604-610001 |  | -- | -- | 0300-7729799 |
| Dilawer Hafeez | C.O. Rajanpur | 0604-689320 |  | -- | 0604-920032 | 0345-8134256 |
|  | SDPO Rajanpur H/Q | 0604-689033 |  | -- | 0604-920129 |  |
| ARMY AUTHORITY |  |  |  |  |  |  |
| Military Officer Head Quarter Engineers 2 Corps Multan Cantt. |  |  |  | -- | -- | 061/5313388 |
| Military Officer, Head Quarter Engineers 31-Corps Bahawalpur Cantt. |  |  |  | -- | -- | 062-89132511 |

# Chapter No. 11

# Standard Operating Procedure (SOP) for Breaching Sections.

## 11.1. History of Breaching Section

There is no breaching section in Rajanpur Canal Division, Rajanpur but during the flood season, the floodwater comes out and along the Canals thereby passing threat to life and property. To avert more serious damage it was advice-able to make relief cuts for more the public safety. Relief cuts are normally needed on the following Canals.

Table 21: Detail of Proposed Relief Cuts and Areas likely to be affected

| Sr. No. | Name of Tehsil | Name of Channels | Proposed sites of Relief Cuts | Rout of Water Coming from Relief Cut | Village Likely to be Affected from Relief Cut |
| --- | --- | --- | --- | --- | --- |
| 1 | Jampur | Dajal Branch | 500 ft D/S of Structure D/S of H/T Crossing RD: 123850 | In the rout of HT x-ing RD: 123850 | Raqba Nushera, Patwali, Channer, Islampur, Shahni, Nwan Shehar, Chak Ahmedpur, Bakharpur, Miranpur |
| 2 |  | Dajal Branch | 500 ft D/S of Structure D/S of HT x-ing RD: 145760 | In the rout of HT x-ing RD: 145760 | Noorpur, Muhammad Hora Kanwala, Rakh Gouspur and Patti Ghulam Muhammad, Tufki, Esra, Hoti |
| 3 |  | Dajal Branch | 500 ft D/S of Structure D/S HT x-ing RD: 177300 | In the rout of HT x-ing RD: 177300 | Miranpur, Hajipur, Patti Mian Saboo Khan and Kalam Sahib, Chak Talab, Patwali |
| 4 | Rajanpur | Dajal Branch | 500 ft D/S of Structure D/S RD: 186/L | In the rout of HT x-ing RD: 186100 | Soonwah,Patti Jumma Arrain, Kotla Rubait Chak Hamoon wala, Chak Patyat, Chak Mehisar, Kotla Esin, Chak Shaheed, Chak Hulwani, Chak Dat. |
| 5 |  | Dajal Branch | 500 ft D/S of Structure D/S RD: 190/R at tail of Dajal Branch | In the rout of abounded upper Asni Minor. | Bakharpur, Chak Burera, Chak Babul, Chak Dreshak, Chak Talook, Jahanpur |
| 6 | Jampur | Link No. III | 500 ft D/S of Structure D/S RD: 194-95/L | Left Bank 100 ft | Jhook Mahar, Naseerpur, Kotla Mughlan, Jampur City and surrounded rural area |
| 7 |  | Link No. III | 500 ft D/S of Structure D/S RD: 224-25/R and 226-27 of Noushera H/R of Kadra Canal at RD: 227. | Flood water Fazilpur Town remained under threat | Qasimpur, Aqilpur, Raqba Dad, Rakh Mari, Raqba Dad, Kotla Dad, Loyalpur, Ranpur, Qasimpur. |
| 8 | Rajanpur | DKC Mubarik Disty | 500 ft D/S of Structure D/S RD: 40-41, 50-60/R, 93-95, 102-121, 122 to 123 RD: 14-15, 18-19, 20-21 | In the rout of HT x-ing entered in the DKC | Kotla Khan Muhammad, Kotla Esin, Kotla Kabir, Hanmoowala, Chak Sohrani, Chak Seri, Rakh Diama Gherbi, Kotla Jinda, Jagir Gabool, Patti Khurd Miran, Rajanpur No. 2, Aqilpur |
| 9 | Rojhan | Kadra Canal | 500 ft D/S of Structure D/S 41-82/L, 151-152/R, 170-172/L, 192-193/L, 207-219/R | In the route of HT x-ing / River Indus | Gianmal, Rakh Qadra, Umer Kot, Kot Mithan, Murghai, Rakh Qadra, Noorpur, Wang, Wang II, Tariq Abad, Chak Dilber, Kot Zubairi |

In case of any emergency, the relief cuts should be made at site according to situation with the coordination of District Administration and Police.

## Procedure

Deputy Commissioner, Rajanpur will frame the relief Committee of each of the two Channels mentioned above. This Committee comprises the following: -

  - Deputy Commissioner, Rajanpur 				(Chairman)

  - Executive Engineer, Rajanpur Canal Division 		(Member)

  - A.D.C. (Revenue) Rajanpur 					(Member)

  - A.C. Rajanpur 						(Member)

This Committee should be framed by 15.06.2024 and start functioning as soon as flood Warning is received. The Exact timing and location of each relief cuts will be fixed by the Committee, keeping in view the intensity of the flood. The relief cut will be made at site under supervision of at least 2 Members of relief cut Committee and suitable Police Force. A gang of at least 50 No. strongmen with kassies or one No. excavator will be needed to make each relief cut in the embankment. After making and developing the relief cut at the site, Police guard, Sub Engineer and Sub Divisional Officer will be deputed to watch the grieved public resistance and interference in the interest of smooth flow. The relief cuts must be closed at site after the decision of Relief Cut Committee under the Supervision of Sub Divisional Officer & Sub Engineer.

## 11.2. Location, Design, Quantity & Variety of explosive required for detonation

Nil

## 11.3. Arrangement of explosive and security of explosive stores

Nil

## 11.4. List of the security staff along-with detail of their training etc.

Nil

## 11.5. Detail of mechanical means as standby arrangements in case of detonation failure

Nil

## 11.6. Duty roster in case of critical situation

Nil

## 11.7. Breaching committee with their action plan

No breaching section in Rajanpur Canal Division, Rajanpur.

## 11.8. 	List of Villages Likely to be Inundated in case of breach

Table 22: List of Villages to be inundated during Flood 2024

| Sr. No. | List of Villages Likely to be Inundation during various floods Limits. | List of Villages to be marooned |
| --- | --- | --- |
| 1 | Shani | Chak Jalalpur |
| 2 | Raqba Noushera | Chak Darishak |
| 3 | Patwali | Chak Dumra |
| 4 | Nawan Shaher | Kotla Rubait |
| 5 | Channer | Kotla Essan |
| 6 | Islampur | Patti Khurd Miran |
| 7 | Bukhara | Kotla Jindu |
| 8 | Chak Ahmad Pur | Chak Lakh |
| 9 | Hajipur | Chak Jindu Shah |
| 10 | Noor Pur | Jahan Pur |
| 11 | Muhammad Hora | Theer Joia |
| 12 | Bakhar Pur | Chak Talook |
| 13 | Soonwah | Rakh Diama Gherbi |
| 14 | Miranpur | Wong II |
| 15 | Jahanpur | Chak Dilber |
| 16 | Rakh Kotla Essan | Chak Mat No. 1 |
| 17 | Jaggier Gabool | Wah Machka |
| 18 | Chak Mishear | Chak Mosani |
| 19 | Chak Burrara | Chak Sohrani |
| 20 | Chak Shahid | Chak Shaheed |
| 21 | Chak Fareed Abad | Chak Burera |
| 22 | Chak Ahamdani | Chak Dat |
| 23 | Hamoonwala | Kotla Esin |
| 24 | Kotla Rubait | Rakh Kotla Esin |
| 25 | Chak Pattyat | Kotla Khan Muhammad |
| 26 | Chak Shah Pur | Wah Bohar |
| 27 | Fazil Pur | Shahani |
| 28 | Chak Shorani | Danwar |
| 29 | Chak Jalalpur | Esran |
| 30 | Chak Darishak | Tufki |
| 31 | Patti Khurd Miran | Hoti |
| 32 | Chak Lakh | Chak Talab |
| 33 | Kotla Jindu | Miranpur |
| 34 | Hajipur | Nwan Shehar |
| 35 | Kanwala | Bakharpur |

## 11.9	Announcement and detail of Evacuation Arrangement

The essential announcements during flood days will be made through revenue staff (Patwari / Zilladari) of Rajanpur Canal Division, Rajanpur.

## 11.10	Detail of Coordination with Civil / Army Authorities.

Regular meetings are being conducted with civil / army authorities as well as Rescue 1122 Officers in the chairman ship of Deputy Commissioner Rajanpur in connection with civil defence keeping in view of exceptional floods in Rajanpur District.

## 11.11	Parallel communication arrangement.

In addition to complete system of communication of administration of Rajanpur a complete wireless system is also maintained in Rajanpur Canal Division, Rajanpur.

## 11.12	Index Plan

Not related.

# Chapter No. 12

# Vulnerable Sites on Flood Bunds / Structures.

## Detail of Vulnerable Flood Bunds/ Structures

Rajanpur Canal Division, Rajanpur flood embankments are present, list of flood embankments with design criteria is given below:-

Table 23:  List of Vulnerable Flood Embankments

| Sr. No. | Name of Embankment / Bund | Reach |  |  | Length (ft) | Width (ft) | Canal Mile | Free Board | Side Slope |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From RD | To RD | Side |  |  |  |  |  |
| 1 | Dajal Branch | 100000 | 190000 | R | 90000 | 15 | 18.00 | 5 ft | 3:1/1:5:1 |
| 2 | Patwali Minor | 0 | 11000 | R | 11000 | 15 | 2.20 | 2.50 ft | 3:1/1:5:1 |
| 3 | Zam Zam Disty | 7148 | 15240 | R | 8092 | 15 | 1.62 | 5 ft | 3:1/1:5:1 |
| 4 | Islampur Disty | 16000 | 24000 | L | 8000 | 15 | 1.60 | 5 ft | 3:1/1:5:1 |
| 5 | Hajipur Disty | 0 | 45000 | L | 45000 | 15 | 9.00 | 5 ft | 3:1/1:5:1 |
| 6 | Taskeen Disty | 14000 | 20000 | R | 6000 | 15 | 1.20 | 5 ft | 3:1/1:5:1 |
| 7 | Fatehpur Disty | 0 | 60000 | R | 60000 | 20 | 12.00 | 5 ft | 3:1/1:5:1 |
| 8 | Link No. III | 112000 | 227820 | L | 115820 | 20 | 23.16 | 7 ft | 3:1/1:5:1 |
| 9 | D.K.C. | 60000 | 130000 | R | 70000 | 20 | 14.00 | 3 ft | 3:1/1:5:1 |
| 10 | Mubarik Disty | 0 | 55000 | R | 55000 | 15 | 10.57 | 5 ft | 3:1/1:5:1 |
| 11 | Kadra Mubarik Flood Embankment | 0 | 30600 | R | 26000 | 15 | 5.88 | 5 ft | 3:1/1:5:1 |
| 12 | Kadra Canal | 130000 | 223500 | R | 93500 | 20 | 18.70 | 5 ft | 3:1/1:5:1 |
| 13 | Tariq Minor | 0 | 28160 | R | 28160 | 15 | 5.63 | 5 ft | 3:1/1:5:1 |
| 14 | Matwah Disty | 0 | 59500 | R | 59500 | 15 | 11.90 | 4 ft | 3:1/1:5:1 |
| 15 | Fakhar Minor | 0 | 11300 | L | 11300 | 15 | 2.26 | Bank Top RL 305.71 | 3:1/1:5:1 |
| 16 | Hairo to Raikh Bagh Wala Flood Bund | 0 | 73000 | R | 73000 | 25 | 14.6 | 7 ft | 3:1/2:1 |
|  |  |  |  |  |  | Total | 152.32 |  |  |

Table 24: Detail of Vulnerable Flood Structures

| Sr. No. | ##### Name of Embankment | RD: of Vulnerable Sites | Village effected |
| --- | --- | --- | --- |
| 1. | Dajal Branch RD:100-190/R | RD: 140000 – 145000/L+R RD: 166000 – 167000/L+R | Mouza Islampur, Basti Babbar, Basti Lund |
| 2. | Patwali Minor RD.0-11/R | RD: 3000 - 4000/L+R | Mouza Patwah, Basti Channer |
| 3. | Zam Zam Disty RD.7-15/R | RD: 7148 - 15240/R | Raqba Noushera |
| 4. | Islampur Disty RD.16-24/L | RD: 23000/L+R | Raqba Noushera |
| 5. | Hajipur Disty RD: 0-45/L | RD: 11000 – 34000/L+R RD: 44000 - 45000/L+R | Mouza Hajipur, Miranpur, Chak Hoora |
| 6. | Taskeen Disty RD.14-20/R | RD: 13000 – 15000/L+R | Basti Mehh |
| 7. | Fatehpur Disty RD.0-26/R | RD: 7000 – 8000/L+R | Basti Mehh |
| 8. | Link No. III RD: 112-227 | 168-170/L, 193-194/L, 223-224/R | Mouza Sahinwala, Qasimpur, Rangpur, Raqba Daat |
| 9. | D.K.C RD.60-130/R | RD: 60000 to 61000/R RD: 93000 to 95000/R RD: 95000 to 102000/R RD: 102000 to 111000/R RD: 122000 to 123000/R | Chak Khas, Rakh Chak Khas, Kotla Rubait, Fazilpur, Kotla Esin |
| 10. | Mubarik Disty RD.0-55000/R | RD: 4000 to 5000/R | Rajanpur No. 1 Rajanpur No. 2 |
| 11. | Kadra Mubarik Flood Bund RD.0-30600/R | RD: 6000 – 70000 RD: 18000 – 200000 | Asni, Patti Kalan Miran Khan |
| 12. | Kadra Canal RD: 130-223500/R | RD: 151000 to 152000/R RD: 193000 to 195000/R RD: 207000 to 220000/R | Mouza Gianmal, Umer Kot, Kotla Hidayat |
| 13 | Tariq Minor Flood Embankment | RD:17000 to 18000/R | Chak Dilber |
| 14 | Matwah Disty RD.0-59500/R | RD: 32000 to 33000/ R+L RD: 34000 to 59500 tail/ R+L | Chak Mat 1, Chak Mat 2, Chak Mat 3 |
| 15 | Fakhar Minor RD: 0-11300. | RD: 6500 – 11300/L+R | Kot Mithan, Basti Phali, Basti Mohib Ali, Mouza Shahpur |
| 16 | Hairo to Raikh Bagh Wala Flood Bund | Sloping Spur S-4 | Basti Talbani, Ahmdani, Hairo, Pitafi Chowk and Kotla Mughlan |
|  |  | RD 62-63+000 | Basti Burra, Basti Gopang, Thul Muhtam |
|  |  | J-Head Spur No.1 at RD 27+000/L | Basti Burra, Basti Gopang, Thul Muhtam |
|  |  | J-Head Spur No. 2 at RD 43+000/L | Mad Lal Khan Gopang, Basti Ikram Khan Gopang, Fidao Chowk, Sherein Wala, Basti Yakeen Gopang |
|  |  | J-Head Spur No. 3 at RD 54+000/L | Basti Sarfraz Gopang, Basti Ahmdani, Basti Bosan, Mouza Raikh Bagh Wala |

## 12.1. Apprehended breaches in Flood Bunds / Structures

Breach of S-4 Sloping Spur (earthen shank) is apprehended at RD 3+000, however, this division already floated proposal for armoring of sloping spur S-4 on river side which was also approved by Departmental Expert Committee in its meeting dated 19.02.2024 vide Minutes of Meeting issued by PRO (Hydraulics) No. 1578/PRO (H)/83-M dated 29.02.2024.

## 12.2. Operation of Breaching Sections

There is no breaching section.

## 12.3. Breaches due to rise of flood water, deterioration of flood bunds.

No flood bund breached during past flood events.

# Chapter No. 13

# Emergency Contingency Plan for Vulnerable Sites

## Emergency Contingency Plan for Vulnerable Sites

All the flood embankments are vulnerable due to flash floods of hill torrents. These sites affect the canal command area of this Division, which are all combined together. Similarly Sloping Spur S-4 at RD 7+000 of Hairo to Raikh Bagh Wala Flood Bund is also vulnerable site.

### 13.1    Plan showing route of flood water coming out of the breach supported with levels

Plan is attached.

### 13.2   Detail of Villages Abadies likely to be affected and this should also be shown on the plan

Table 25: List of Abadies likely to be affected in District Rajanpur

TEHSIL RAJANPUR

| Sr. No. | Name of Villages |  | Sr. No. | Name of Villages |
| --- | --- | --- | --- | --- |
|  | Rajanpur No.2 |  |  | Raqba Ahmad |
|  | Basti Burani |  |  | Raqba Gulan |
|  | Kotla Jinda |  |  | Rakh Raqba Naseer |
|  | Jageer Gabol |  |  | Noshehra Sharqi |
|  | Muhammadpur Gam Wala |  |  | Rakh Noshehra |
|  | Thul Machi |  |  | Bait Arain |
|  | Darkhast Haji Muhammad |  |  | Pati Mutnaza |
|  | Pati Makhdoom Sahib |  |  | Kohar Markand |
|  | Pati Zaman Shah |  |  | Bait Sountra |
|  | Pati Khurd Meeran Khan |  |  | Rakh Bait Sountra |
|  | Pati Jindu Marral |  |  | Aizad Abad |
|  | Peer Bakhsh Sharqi |  |  | Bait Ghazlani |
|  | Ratan Therr |  |  | Katcha Drigh Khas |
|  | Kallanpur |  |  | Katcha Drigh Pati Daha |
|  | Rakh Hazrat Wala |  |  | Katcha Drigh Pati Murad |
|  | Hazrat Wala |  |  | Katcha Drigh Katchi Meeran |
|  | Rakh Reikh |  |  | Katcha Drigh Pati Ali |
|  | Reikh |  |  | Katcha Drigh Amda Riyasat Bahawalpur |
|  | Mehray Wala |  |  | Katcha Drigh Kaath Parr |
|  | Mushtarqa Mehray Wala |  |  | Katcha Drigh Awairra |
|  | Thehrri |  |  | Katcha Drigh Chak Keehal |
|  | Bait Gull Muhammad |  |  | Katcha Drigh Bait Waghwarr |
|  | Kotla Androon |  |  | Katcha Drigh Pati Behram Khan |
|  | Rakh Thul Menghraj |  |  | Katcha Drigh Mushtarqa Gambhir |
|  | Mad Gasoora |  |  | Katcha Drigh Katchi Gambhir |
|  | Rangpur |  |  | Katcha Drigh Kotla Muhammad Khan |
|  | Rakh Sahan Wala |  |  | Katcha Drigh Amda Bait Mughal |
|  | Rakh Kotla Sher Muhammad |  |  | Noorpur |
|  | Kotli Khudai |  |  | Rakh Noorpur Gharbi |
|  | Said Pur |  |  | Rakh Noorpur Sharqi |
|  | Kotla Sher Muhammad Pacca |  |  | Raqba Naseer |
|  | Kotla Sher Muhammad Katcha |  |  | Basti Khawaja |
|  | Bagh Wala |  |  | Wong Gulu Wali |
|  | Darkhawast Meeran Khan |  |  | Wong Dom |
|  | Rakh Bait Bagh Shah |  |  | Toung |
|  | Qasimpur |  |  | Rakh Toung |
|  | Basti Rindan |  |  | Baghoon |
|  | Shikarpur |  |  | Rakh Baghoon |
|  | Rakh Shikarpur |  |  | Wong Awal |
|  | Marri |  |  | Wong Chak Malanhas |
|  | Lunda Baraham |  |  | Wong Daha |
|  | Rakh Lunda Braham |  |  | Thul Karam Chak Khas |
|  | Jahanpur |  |  | Pati Kalan Meeran Khan |
|  | Sindh Gabol |  |  | Asni |
|  | Kotla Hazoori |  |  | Kot Mithan Pacca |
|  | Rakh Marri |  |  | Kot Mithan Jadeed |
|  | Raqba Dad |  |  | Kot Mithan Katcha |
|  | Aqilpur |  |  | Bait Mughal |
|  | Kotla Hussain Chak Dom |  |  | Kotla Hussain Chak Awal |
|  | Basti Mohib Ali Chak Awal |  |  | Chak Lakha |
|  | Shahpur |  |  | Chak Bangla |
|  | Basti Phalli |  |  | Chak Shaheed |
|  | Rakh Kot Mithan |  |  | Chak Haji |
|  | Kotla Said Khan |  |  | Chak Kot Zubairi |
|  | Mad Gola |  |  | Chak Hulwani |
|  | Kotla Meeran |  |  | Chak Domrah |
|  | Mad Moulvi |  |  | Chak Azam Abad |
|  | Rakh Mad Moulvi |  |  | Chak Pandh |
|  | Mad Sain Dad |  |  | Chak Sundha |
|  | Bangala |  |  | Chak Musani |
|  | Rakh Bangala |  |  | Chak Chowki |
|  | Rakh Machka Pacca |  |  | Chak Allah Abad |
|  | Raqba Khalti Katcha |  |  | Pati Ahmad Khan |
|  | Raqba Khalti Pacca |  |  | Chak Lashari |
|  | Katcha Sher Mahar |  |  | Chak Dalail |
|  | Murghai |  |  | Chak Chach |
|  | Rakh Murghai |  |  | Pati Seith Molchand |
|  | Kotla Ghulam Murtaza Shah |  |  | Chak Jindo Shah |
|  | Abdullah Shaheed |  |  | Chak Dreshak |
|  | Shah Garh |  |  | Rakh Dama Gharbi |
|  | Rakh Shah Garh |  |  | Rakh Dama Sharqi |
|  | Kotla Imam Bakhsh |  |  | Baroos Abad |
|  | Katchi Khan Muhammad |  |  | Paharpur |
|  | Mad Meeran |  |  | Hanmoon Wala |
|  | Machka |  |  | Chak Fareed Abad |
|  | Rakh Machka Katcha |  |  | Chak Kotla Rubait |
|  | Pati Sarkari |  |  | Chak Ahmadani |
|  | Chak Sher Purrah |  |  | Chak Patiyat |
|  | Chak Hafiz Abad |  |  | Paharpur |
|  | Chak Lundan |  |  | Hanmoon Wala |
|  | Chak Gabol |  |  | Chak Fareed Abad |
|  | Chak Baranda |  |  | Chak Kotla Rubait |
|  | Chak Patiyat |  |  | Chak Ahmadani |
| TEHSIL JAMPUR |  |  |  |  |
|  |  | Jampur |  | Basti Meeran |
|  |  | Kot Tahir |  | Kaloi Wala |
|  |  | Tibba Chandia Kalan |  | Ganeehar |
|  |  | Tibba Chandia Khurd |  | Basti Jindu Khan |
|  |  | Basti Rindan |  | Thul Hairo |
|  |  | Kotla Daha |  | Thul Janan |
|  |  | Bhanu Wala |  | Thul Seerak |
|  |  | Sahoo Wala |  | Raqba Dodiyani |
|  |  | Nawan Begraj |  | Basti Shah Ali Bohar |
|  |  | Basti Gaddan |  | Thul Gaman |
|  |  | Tatar Wala |  | Thul Ali Muhammad |
|  |  | Rakh Azmat Wala |  | Ali Abad |
|  |  | Muhammadpur Hanbhi |  | Beta |
|  |  | Cheitr Faqeer |  | Tibbi Lundan |
|  |  | Leek Wala |  | Thul Alam Khan |
|  |  | Khosa |  | Raqba Lundan |
|  |  | Rakh Registan Shumali |  | Kotli Jamal |
|  |  | Basti Cheena |  | Dajal |
|  |  | Pati Machi |  | Dinpur |
|  |  | Pati Qazi |  | Barkhurdar Purah |
|  |  | Allah Abad Gharbi |  | Eesran |
|  |  | Rasoolpur |  | Qasab |
|  |  | Azmat Wala |  | Onarr |
|  |  | Raqba Dhingana |  | Darkhan Mahtam |
|  |  | Tal Shumali |  | Shahani |
|  |  | Basti Dhandla |  | Wah Bohar |
|  |  | Tufki |  | Noshehra |
|  |  | Hothi |  | Danwar |
|  |  | Tal Janubi |  | Wah Khokhar |
|  |  | Mahal Mahtam |  | Raqba Noshehra |
|  |  | Basti Meo |  | Wah Lashari |
|  |  | Raqba Dhandla |  | Jhok Makwal |
|  |  | Kotla Gujar |  | Meeranpur |
|  |  | Wah Kandhil |  | Alipur |
|  |  | Basti Hanbhi |  | Lundi Saidan |
|  |  | Bochrra |  | Kalhot Pur |
|  |  | Chak Lucky |  | Rakh Registran Janubi |
|  |  | Hajo |  | Darkhast Ghulam Haidar |
|  |  | Gandha Lucky |  | Chatool |
|  |  | Binda Bur’rah |  | Bambli |
|  |  | Wah Alkani |  | Soma |
|  |  | Wah Saidan |  | Garkna Wazeeri |
|  |  | Harrand |  | Barkato |
|  |  | Basti Panah Ali |  | Thul Said Khan |
|  |  | Basti Nawab |  | Shaloba |
|  |  | Sorri |  | Thul Chang |
|  |  | Bilhar |  | Thul Baqir |
|  |  | Bhambha |  | Thul Wazeer |
|  |  | Raqba Aqil Muhammad |  | Rakh Registan Gharbi |
|  |  | Kotla Mughlan |  | Raqba Lundi Saidan |
|  |  | Dhora Hajana |  | Muhammadpur No.1 |
|  |  | Khan Wah |  | Muhammadpur No.2 |
|  |  | Makwal Wala |  | Bukhara |
|  |  | Kotla Sial |  | Islampur |
|  |  | Dingora |  | Channar |
|  |  | Shahwala |  | Pat Wali |
|  |  | Mad Ahmad Khan |  | Bulaywala |
|  |  | Malani Wala |  | Qazi Wala |
|  |  | Rakh Khan Wah |  | Patni |
|  |  | Basti Bakhsh |  | Dhora Rindan |
|  |  | Adam Garh Nou |  | Hairo |
|  |  | Kotla Deewan |  | Thul Mahtam |
|  |  | Pati Gadi |  | Katchi Bakhsh |
|  |  | Babul Wali |  | Basti Sikhani |
|  |  | Rustam Leghari |  | Gulab Shah |
|  |  | Shah Nawaz Purah |  | Chak Korewala |
|  |  | Seelra |  | Chak Ahmadpur |
|  |  | Jhok Mahar |  | Chak Shahwala |
|  |  | Naseerpur |  | Rakh Noorpur |
|  |  | Bur’rah |  | Chak Peerowala |
|  |  | Qasim Wala |  | Chak Lunda |
|  |  | Dukarr |  | Chak Mungeri |
|  |  | Basti Hafiz |  | Chak Nau Abad |
|  |  | Adam Garh Kohna |  | Raqba Nawan Shahar |
|  |  | Rakh Muhammadpur |  | Gulabi |
|  |  | Hajipur |  | Wah Meo |
|  |  | Son Wah |  | Gerro Lashari |
|  |  | Noorpur |  | Bulhi |
|  |  | Kanwala |  | Bahar Hussain |
|  |  | Malkani |  | Mirza Wah |
|  |  | Rakh Charroha |  | Ellahi Wah |
|  |  | Drighri |  | Golay Wah |
|  |  | Bakhirpur |  | Khumbi Garang |
|  |  | Nawan Shehar |  | Thehr Basti Joiya |
|  |  | Muhammad Horra |  |  |
| TEHSIL ROJHAN |  |  |  |  |
|  | Chak Dim |  |  | Mahal Thul Nawab |
|  | Chak Karrya |  |  | Peer Bakhsh |
|  | Chak Chandka |  |  | Dera Bhai |
|  | Pacca Mian Wali |  |  | Kareema Khor |
|  | Mat No.1 |  |  | Karriya Jat |
|  | Mat No.2 |  |  | Chak Jadeed |
|  | Mat No.3 |  |  | Meeranpur |
|  | Wah Machka |  |  | Safdar Abad |
|  | Makeehan |  |  | Kotla Gulsher |
|  | Dera Dildar |  |  | Mushtarqa Mazari |
|  | Mandhri |  |  | Mad Muhammad Shah |
|  | Dera Jeewan |  |  | Thul Naseer |
|  | Kin Khas |  |  | Badli Khas |
|  | Bambli Khairpur |  |  | Rakh Meeranpur |
|  | Dilbar |  |  | Thoyani |
|  | Dingri Sori |  |  | Lat |
|  | Sheikhwala |  |  | Mohri |
|  | Tariq Abad |  |  | Katcha Chohan |
|  | Giamal |  |  | Rakh Katcha Chohan |
|  | Rakh Qadra |  |  | Chakir Wali |
|  | Qadra Khas |  |  | Katcha Mian Wali No.1 |
|  | Dhora |  |  | Katcha Mianwali No.2 |
|  | Luteri |  |  | Shahwali |
|  | Lala |  |  | Charagh Shah |
|  | Purewar |  |  | Gari Sori |
|  | Rakh Purewar |  |  | Gadda Narr |
|  | Dahir Wali |  |  | Jongo Thandi |
|  | Rakh Sabzani |  |  | Patti Shahwali |
|  | Kaheeri |  |  | Rakh Shahwali |
|  | Sabzani Tukrra No.1 |  |  | Barra |
|  | Sabzani Tukrra No.2 |  |  | Leeni |
|  | Sabzani Katchi Islam |  |  | Sorri |
|  | Sabzani Garkan |  |  | Hithar |
|  | Sabzani Panjwani |  |  | Katcha Razi |
|  | Sabzani Bolani |  |  | Katcha Bhot |
|  | Sabzani Pacca |  |  | Chakar Bhot |
|  | Bhagsar |  |  | Danwani |

## 13.3. Strategy and action taken, explained in details

### 13.3.1	Arrangements

The entire flood fighting material along with watching establishment will be arranged at site before the commencement of flood period.

### 13.3.2. Establishment of Flood Fighting Campus

Relief camps will be established at elevated places near affected areas. Following Departments will establish their Relief Camps.

    - Revenue.

    - Health.

    - Live Stock.

Revenue Department will establish their relief camps near flood area at suitable places to provide food and tent to the flood affecters to accommodate them.

Health Department will establish their health camps and provide medical facilities of all kinds to flood affectees.

District Live Stock Officer will ensure the supply of medicine and fodders for animals in affected areas.

13.3.3 Flood Fighting Campus Name at sites

| Sr.No | Name at Site | Sr.No | Name at Site |
| --- | --- | --- | --- |
| 1 | Dajal Branch Syphon RD:145 | 4 | Kadra Canal RD:192 |
| 2 | DKC Syphon RD:95 | 5 | J-1 |
| 3 | DKC Syphon RD:55 | 6 | J-3 |

13.3.4.    Duties of Officers / Officials & Their Camp at Sites

All concerned Sub Divisional Officer & their Staff will remain in the area round the clock. Staff of Division will be set on high alert during day and night from 15 June up-to 15 September. Consequently flood watching camps will be installed at vulnerable points. Adequate arrangements to fight against the flood according to its magnitude will be made by all Sub Divisions. Any unwanted happenings will be reported by Sub Divisional officer to the Executive Engineer Rajanpur Canal Division, Superintending Engineer Derajat Canal Circle, Chief Engineer DG Khan Irrigation Zone and Secretary, Irrigation Department. It will also be duty of the Sub Divisional Officer to deliver messages well in time to the local civil authorities in addition to military personnel,

## 13.3.5. Departmental Machinery available

There is no departmental machinery available in Rajanpur Canal Division, Rajanpur for Flood Fighting.

## 13.3.6. Machinery available from Private Source

Essential Machinery will be arranged from the registered government contractors according to flood situation.

### Excavators / Tractor Trolley Required

The concerned SDO will be responsible for managing / providing 02 Nos. Excavator & 04 Nos. Tractor Trollies with front blade in their respective Sub Divisions to be maintained at critical reaches to protect the flood embankment, Irrigation Channels and to meet with emergent situation during ensuring hill torrent flood.

About 12 Nos. Motor Boats are required in Rajanpur District to bring out the effected people from the deep water. District management Rajanpur will arrange these Boats.

### 13.3.7	Flood Fighting Material Required

Detail of Flood Fighting Material Required given in table below;

Table 26: Detail of Flood Fighting Material Required

| Sr. No. | Required |  |  | Available |  | Balance Required |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Item | No. | Unit | No. | Unit | No. | Unit |
| 1 | Chargeable Flood Lights | 120 | No. | 10 | No. | 110 | No. |
| 2 | Chargeable Mini Lights | 165 | No. | 20 | No. | 145 | No. |
| 3 | Torches 3 Cells | 42 | No. | 15 | No. | 27 | No. |
| 4 | Cell For Torches. | 560 | No. | 0 | No. | 560 | No. |
| 5 | Gunny Bags | 8000 | No. | 0 | No. | 8000 | No. |
| 6 | Khaji Mates. | 1200 | No. | 12 | No. | 1188 | No. |
| 7 | Life Jackets | 108 | Kg | 0 | Kg. | 108 | Kg. |
| 8 | Basket | 1100 | No. | 50 | No. | 1050 | No. |
| 9 | Munj Ban. | 700 | Kg | 15 | Kg. | 685 | Kg. |
| 10 | Sutli | 240 | Kg | 0 | Kg. | 240 | Kg. |
| 11 | Kassi with Handle | 550 | No. | 0 | No. | 550 | No. |
| 12 | Axes with Handle | 500 | No. | 0 | No. | 500 | No. |
| 13 | Jute Bags. | 1120 | No. | 10 | No. | 1110 | No. |
| 14 | Trungar | 10000 | No. | 0 | No. | 10000 | No. |
| 15 | Generator | 12 | No. | 0 | No. | 12 | No. |
| 16 | Petrol | 3900 | Liter | 0 | Ltr | 3900 | Ltr |
| 17 | Bamboo | 12000 | No. | 12 | No. | 11988 | No. |
| 18 | Choldari | 12 | No. | 1 | No. | 11 | No. |
| 19 | Motor Boat | 2 | No. | 0 | No. | 2 | No. |
| 20 | Energy Saver (Large Size) | 40 | No. | 10 | No. | 30 | No. |
| 21 | Tea Set | 12 | Set | 0 | Set | 12 | Set |

## 13.3.8.	  Flood Fighting Material Available

## Given in Table No. 25

## 13.4     Detail of other infrastructure like electric, Sui Gas, Telephone installations, road network, other buildings, Canals & drainage network

Canal

Dajal Canal, Link No. III and Kadra Canal along with their infrastructure explained supra.

Road

Indus Road (N-55) and other village road network

Hospital

District Head Quarter Hospital, Rajanpur.

Sui Gas

Related to SNGPL.

Telephone installation

Related to PTCL

Drainage Network

Tayyab Drain, New Manka Drain, Qutab Drain, Bukhari Drain.

# Chapter No. 14

# Action Plan

## 14.1. Reshuffling/recouping plan of reserve stone departmentally

Reserve Stock of Stone is available at J-Head Spur No. 2 and 3 at RD 43+000 and 51+000 of Hairo to Raikh Bagh Wala Flood Bund and will be transported at any critical location on the flood bund as and when required by hiring machinery from resourceful contractor.

## 14.2. Detail of inlet/outlet crossing along with closing methodology

Inlet / outlet will be plugged well in time by sand filled plastic bags by work charge establishment.

## 14.3. Deployment Machinery (Medium to High Flood)

Following Machinery will be deployed at critical sites of flood embankments during flood season 2025 at medium to high flood.

Table 27: Detail of Machinery to be deployed during Medium to High Flood

| Sub Division | Detail of Structure | Excavator | Dozer | Trolley / Dumper | Tractor with front Blade | Labor WC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Beldar + Mate |
| Hajipur Sub Division | Head Bakharpur | 1 | 1 | 6 | 8 | 12+1 |
| Talai Sub Division | Pull Pathan | 1 | - | 6 | 8 | 12+1 |
| Kadra Sub Division | Fakhar Flood Bund | 2 | 2 | 10 | 12 | 24+1 |
|  | Hairo to Raikh Bagh Wala Flood Bund | 2 | 2 | 10 | 12 | 24+1 |
|  | Total | 6 | 5 | 32 | 40 | 76 |

Payment for deployment of machinery and work charge establishment subject to verification by TPM / Consultants.

## 14.4. Deployment of Machinery (High to very high flood)

Following Machinery will be deployed at critical sites of flood embankments during flood season 2025 at high to very high flood.

Table 28: Detail of Machinery to be deployed at High to very High Flood

| Sub Division | Detail of Structure | Excavator | Dozer | Trolley / Dumper | Tractor front Blade | Labour WC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Beldar + Mate |
| Hajipur Sub Division | Head Bakharpur | 2 | 2 | 12 | 10 | 24+2 |
| Talai Sub Division | Pull Pathan | 2 | 1 | 12 | 10 | 24+2 |
| Kadra Sub Division | Fakhar Flood Bund | 2 | 4 | 12 | 10 | 60+2 |
|  | H-R Flood Bund | 2 | 4 | 12 | 10 | 60+2 |
|  | Total | 8 | 11 | 48 | 40 | 176 |

## Payment for deployment of machinery and work charge establishment subject to verification by TPM / Consultants.

## 14.5. Deployment of Machinery (high to exceptionally high flood)

Following Machinery will be deployed at critical sites of flood embankments during flood season 2025 at high to exceptionally high flood.

Table 29: Detail of Machinery to be deployed at very high to exceptionally high flood

| Sub Division | Detail of Structure | Excavator | Dozer | Trolley / Dumper | Tractor with front Blade | Labour WC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Beldar + Mate |
| Hajipur Sub Division | Head Bakharpur | 10 | 2 | 20 | 15 | 84+7 |
| Talai Sub Division | Pull Pathan | 5 | 2 | 20 | 10 | 84+7 |
| Kadra Sub Division | Hairo to Raikh Bagh Wala Flood Bund | 10 | 5 | 20 | 14 | 84+7 |
| Rojhan Sub Division | Fakhar Flood Bund | 10 | 5 | 20 | 14 | 84+7 |
|  | Total | 35 | 14 | 80 | 53 | 364 |

Payment for deployment of machinery and work charge establishment subject to verification by TPM / Consultants.

## 14.6.  Police Deployment Plan

### Breaching Section

| Sr. No. | Site breaching section | Concerned Canal Division | Concerned Police Station & District | Police Personal to be deployed |  | Remarks (if Any) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Inspector / SI / ASI | Constables |  |
| 1 | There is No Breaching Section in this Division. |  |  |  |  |  |

### Vulnerable Sites

| Sr. No. | Vulnerable sites used for illegal cuts during floods | Concerned Canal Division | Concerned Police Station and District | Police Personnel to be deployed |  | Remarks (If any) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Inspection /SI/ASI | Constables |  |
| 1 | Dajal Branch RD:100-190/R | Rajanpur Canal Division, Rajanpur | PS Dajal | SI =1 No. ASI =2 No. | 5 No. | - |
| 2 | Patwali Minor RD.0-11/R |  | PS Dajal | SI =1 No. ASI =1 No. | 3 No. | - |
| 3 | Zam Zam Disty RD.7-15/R |  | PS Dajal | SI =1 No. ASI =1 No. | 3 No. | - |
| 4 | Islampur Disty RD.16-24/L |  | PS Dajal | SI =1 No. ASI =1 No. | 3 No. | - |
| 5 | Hajipur Disty RD: 0-45/L |  | PS Hajipur | SI =1 No. ASI =1 No. | 3 No. | - |
| 6 | Taskeen Disty RD.14-20/R |  | PS Hajipur | SI =1 No. ASI =2 No. | 5 No. | - |
| 7 | Fatehpur Disty RD.0-26/R |  | PS Rajanpur | SI =1 No. ASI =2 No. | 5 No. | - |
| 8 | Link No. III RD: 112-227 |  | PS Fazilpur | SI =1 No. ASI =4 No. | 10 No. | - |
| 9 | D.K.C RD.60-130/R |  | PS Fazilpur | SI =1 No. ASI =3 No. | 10 No. | - |
| 10 | Mubarik Disty RD.0-55000/R |  | PS Rajanpur | SI =1 No. ASI =2 No. | 5 No. | - |
| 11 | Kadra Mubarik Flood Bund RD.0-30600/R |  | PS Umerkot | SI =1 No. ASI =2 No. | 5 No. | - |
| 12 | Kadra Canal RD: 130-223500/R |  | PS Umerkot | SI =1 No. ASI =3 No. | 15 No. | - |
| 13 | Hairo to Raikh Bagh Wala Flood Bund (S-4) |  | PS Jampur Saddar | SI =1 No. ASI =3 No. | 15 No. | - |
| 14 | Tariq Minor Flood Embankment RD: 0- 28160 |  | PS Rojhan | SI =1 No. ASI =3 No. | 8 No. | - |
| 15 | Matwah Disty RD: 0-59500/R |  | PS Rojhan | SI =1 No. ASI =2 No. | 5 No. | - |
| 16 | Fakhar Minor RD: 0-11300. |  | PS Kotmithan | SI =1 No. ASI =2 No. | 5 No. | - |

## 14.7. Details of synthetic bags with capacity of 50 Kg and 100 Kg

Detail is mentioned supra. These will be filled with manual and mechanical means and placed wherever required.

| Sr. No | Required (Qty) | Available (Qty) | Balance (Qty) | Requirement |
| --- | --- | --- | --- | --- |
| 1 | Hajipur Sub Division | 0 | 0 | 8000 |
| 2 | Talai Sub Division | 0 | 0 | 6000 |
| 3 | Kadra Sub Division | 0 | 0 | 8000 |
| 4 | Rojhan Sub Division | 0 | 0 | 8000 |
|  | Total |  |  | 30000 |

## 14.8. Details of polythene sheets of black color to protect upstream slope against wave action and to control seepage through embankments

Apparently no strong wave action is expected near flood embankments. Only S-4 is under direct threat of wave action. For its protection, this division has already taken up work of armoring it. Same will be executed after fulfilling all codal formalities. Similarly in case of any untoward situation, polythene sheets of black color to protect upstream slope against wave action to control seepage through embankments will be required at RD 7+000 to 73+000 of Hairo to Raikh Bagh Wala Flood Bund and will be purchased through resourceful contractor as and when needed.

Right flood embankment of Kadra canal is vulnerable to wave action due to pondage of water in Aasni lake from RD 191 to 216+000. In order to protect the slopes from erosive wace action, polythene sheets will be provided on slopes as per lesson learnt from last year flood of 2024.

14.8.1 Detail of Polythen sheets sumrise in the table given below up.

| Sr. No | Required (Qty) | Available (Qty) | Balance (Qty) | Remakes |
| --- | --- | --- | --- | --- |
| 1 | 1000 | Nil | 1000 | Procurement will be done on the availability of funds |

# Chapter No. 15

# Backup Divisions (In case of breach)

Jampur Construction Division, Rajanpur will be the ‘Backup Division’ with fully support and backup to pass the flood safely.

| ###### Executive Engineer Rajanpur Canal Division Rajanpur | ###### Superintending Engineer Derajat Canal Circle D.G. Khan | ###### Chief Engineer D.G. Khan Irrigation Zone D.G. Khan |
| --- | --- | --- |