CHAPTER-1

SALIENT FEATURES OF JHELUM CANAL DIVISION

1.1	LOCATION

Upper Jhelum Canal off takes from Mangla Dam at 33 ͦ 07’ 16” N Latitude & 73 ͦ 38’ 24” Longitude and passes through the Districts Mirpur (RD 0+000 to 77+106), Bhimber (RD 77+106 to 90+000), Gujrat (RD 90+000 to 240+779) and RD 252+840 to 440+000) and Mandi Bahauddin (240+779 to 252+840) before it out falls in the River Chenab U/S Khanki Barrage at Latitude 32 ͦ  27’ 45” N and Longitude 74 ͦ 01’ 26” E.

1.2	GENERAL DESCRIPTION

Jhelum Canal Division has been entrusted with the maintenance of Upper Jhelum Canal from RD 27+500 to 440+000 (134 KM). The length of Flood embankment in the jurisdiction is 56.61 KM. Jhelum Canal Division is affected by the Floods in River Jhelum, River Kahan, River Chenab and Bhimber Nullah.

- RIVER JHELUM

Jhelum River originates from occupied territories of Jammu and Kashmir and after traversing a long distance in the hilly areas of Gilgit and Azad Kashmir it enters Mangla Dam Reservoir.

Mangla Head works was constructed on River Jhelum in the year 1915 at a distance of about 23 miles in the Northern direction of Jhelum City. It remained in operation till the construction of Mangla Dam. River Jhelum flows parallel to the UJC in reach RD. 27+500 to 243+000 and poses a threat to the safety of Canal in the reach RD 27+500 to 41+000. Following structures / river training works have become abandoned due to reasons mentioned against each structure.

| Sr. No. | Name of Bund or Stud | Top width | Length | Side Slope | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | C.D. Bund | 20 Ft | 2260 Ft | 3:1 inner & 2:1 outer | Structures from Sr. No. 1 to 8 are useless due to construction of Jhelum city flood bund in front of them. |
| 2. | G.H. Bund | 20 Ft | 840 Ft | 3:1 inner & 2:1 outer |  |
| 3. | Stud No.1 | 24 Ft | 1365 Ft | 2:1 inner & 2:1 outer |  |
| 4. | Stud No.2 | 24 Ft | 790 Ft | 2:1 inner & 2:1 outer |  |
| 5. | Stud No.3 | 24 Ft | 725 Ft | 2:1 inner & 2:1 outer |  |
| 6. | Stud No. ’X’ | 20 Ft | 130 Ft | 2:1 on either side |  |
| 7. | Stud No. ’Y’ | 30 Ft | 165 Ft | 2:1 on either side |  |
| 8. | Stud No. ’Z’ | 20 Ft | 185 Ft | 2:1 on either side |  |

- KAHAN RIVER

It is a hill torrent. Maximum discharge of 1,20,000 Cs was noticed during the flood of 1992. It outfalls into River Jhelum, D/S of Jhelum Cantonment near Village Naugran, after traversing a long distance in District Jhelum. As it is a hill torrent and carries flashy discharge due to very steep slope, it causes erosion to the lands and abadies of District Jhelum.

- BHIMBER NULLAH

Bhimber Nullah is a tributary of River Chenab and flows along the left bank of Upper Jhelum Canal from RD. 414+000 to 425+000 and then both merge before it outfalls into River Chenab, upstream of Khanki Headwork’s. This reach of the channel, where Nullah is in closed vicinity has been protected by properly designed stone pitching, however, the channel is kept under watch during flood season.

- NULLAH BUNAHAN

Nullah Bunahan is tributary of River Jhelum which originates from boundary of District Chakwal and falls in River Jhelum U/S Rasul Barrage. The Discharge of Nullah Bunahan is 75000 Cs which causes inundation of crop and abadies of Village Bhambar and water spreads over Jhelum-PD Khan Road as well. Now Bhamber Flood Bund has been constructed along left bank of Nullah Bunahan upto 7473 Ft to safeguard crops and village abadi.

XEN/JLM

  - ADMINISTATIVE SETUP

| Mangla Canal Sub Division RD 27+500 to 150+000 |  |  |  |
| --- | --- | --- | --- |
| Sr. No | Name of Section | Jurisdiction | Structures |
| - MANGLA SUB DIVISION FROM RD 27+500 TO RD 149+880 |  |  |  |
| 1 | Bong Section | 27+500 to 40+875 | Bong level crossing (RD 27+500) |
| 2 | Jatli Section | 40+876 to 80+000 | Jatli level crossing (RD 78+695) |
| 3 | Rehmanpur Section | 80+000 to 107+000 | Rehmanpur level crossing (RD 89+149) |
| 4 | Jaggu Section | 107+000 to 150+000 | Jaggu level crossing (RD 123+050) |
| 5 | Jhelum Section | River Jhelum from Mangla to Nullah Bunhan | Bunds, Studs, spurs etc |
| 6 | Mechanical Section | Mechanical workshop |  |
| - KHOKHRA CANAL SUB DIVISION RD 150+000 to 440+000 |  |  |  |
| 7 | Rajor Section | 150+000 to 216+000 |  |
| 8 | Khokhra Section | 216+000 to 293+000 | Khokhra Cross Regulator (RD 255+080) |
| 9 | Puranwala Section | 293+000 to 366+000 |  |
| 10 | Shadiwal Section | 366+000 to 440+000 |  |

- The Head Quarter of Jhelum Canal Division is at 46-A Sarwar Road Jhelum Cantt.

- Mangla Sub Division with Office of the SDO stationed at WAPDA Colony Mangla is responsible for maintaining Upper Jhelum Canal from RD. 27+500 to 150+000, flood bunds along River Jhelum from Mangla to G.T Road bridge and flood bunds along River Kahan from Malot Bridge to Jhelum Cantt and Nallah Bunhan.

- Khokhra Sub Division at Rasul is responsible to maintain Upper Jhelum Canal from            RD. 150+000 to RD. 440+000 (tail) and flood bunds/ guide walls along River Jhelum from G.T Road Bridge to Rasul Head Works.

XEN/JLM

CHAPTER-2

FLOOD PROTECTION AND RIVER TRAINING WORKS

2.1	  DESIGN PARAMETERS OF TRAINING WORKS

| S.No. | Name of Bund or Stud | Top width | Length | Side Slope | Remarks |
| --- | --- | --- | --- | --- | --- |
| i | RIVER JHELUM (Mangla Canal Sub Division) |  |  |  |  |
| 1 | Protection Bund near RD.33 UJC. | 25 Ft | 1500 Ft | 2:1 on either side | Mirpur |
| 2 | Jabbar Bund | 20 Ft | 4200 Ft | 3:1 on either side | Mirpur |
| 3. | Stud No. 1 near village Sheikhupur . | 15 Ft | 75 Ft | 2:1 on either side | 1994-95 |
| 4. | Stud No. 2 near village Loharian | 15 Ft | 75 Ft | 2:1 on either side | Stud No.1 to 7 in Dina |
| 5. | Stud No. 3 near village Khatana. | 15 Ft | 75 Ft | 2:1 on either side |  |
| 6. | Stud No. 4 near village Dulyal | 15 Ft | 75 Ft | 2:1 on either side |  |
| 7. | Stud No. 5, near village Dulyal | 15 Ft | 75 Ft | 2:1 on either side |  |
| 8. | Stud No. 6, near village Dulyal | 15 Ft | 75 Ft | 2:1 on either side |  |
| 9. | Stud No. 7 near village Budhar | 15 Ft | 75 Ft | 2:1 on either side | Washed away during Flood 2014. Reconstructed in 2023 |
| 10. | Stud No.8 near Pind Rajwal | 15 Ft | 75 Ft | 2:1 on either side | Stud No.8 to 17 in Jhelum |
| 11. | Stud No.9 near village Chak Balian | 15 Ft | 75 Ft | 2:1 on either side |  |
| 12. | Stud No.10 U/s village Bhoundnan Gakhran | 15 Ft | 75 Ft | 2:1 on either side |  |
| 13. | Stud No.11 near village Pindi Khaira | 15 Ft | 75 Ft | 2:1 on either side | Washed away during flood 2014 and estimate amounting to Rs.29 million was submitted |
| 14. | Stud No.12 near village Pindi More | 15 Ft | 75 Ft | 2:1 on either side |  |
| 15. | Stud No.13 near village Chak Daulat | 15 Ft | 75 Ft | 2:1 on either side | Stud No.8 to 17 in Jhelum |
| 16. | Stud No.14, near village Chak Daulat | 15 Ft | 75 Ft | 2:1 on either side |  |
| 17. | Stud No.15, near village Chak Daulat | 15 Ft | 75 Ft | 2:1 on either side |  |
| 18. | Stud No.16, near village Chak Daulat | 15 Ft | 75 Ft | 2:1 on either side |  |

XEN/JLM

| S.No. |  | Name of Bund or Stud | Top width | Length | Side Slope | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 19. |  | Stud No.17 near village Masumpur | 15 Ft | 205 Ft | 2:1 on either side | 150+55 Ft |
| 20. |  | Solid Stone Spur Pakhwal & Tawakal Pakhwal | 20 FT | 410 Ft | 1½:1 on either side |  |
| 21. |  | Jhelum Cantt Protection Bund | 25 Ft | 2500 Ft | 2:1 on either side | "Extending cantonment Flood Bund downstream G.T Road Bridge along right bank of Jhelum River" in length of 29500 ft has been framed amounting to Rs. 14000/- million for extension of Jhelum Cantt bund. |
| 22. |  | Jhelum City Bund | 25 Ft | 20000 Ft | 2:1 River side & 3:1 Country side | 2018 |
| 23. |  | Protecting Sarai Alamgir Abadies between Railway and G.T Road Bridge on Left Bank of River Jhelum. | 25 Ft | 1800 Ft | 2:1 River side & 3:1 Country side | Sarai Alamgir |
| 24. |  | Shah Madar guide wall | 25 Ft | 1847 Ft | 2:1 on either side | M Sarai Alamgir |
|  |  | Khokhra Canal Sub Division |  |  |  |  |
| 25. |  | Paharwal guide wall | 25 Ft | 2299 Ft | 2:1 on either side | Sarai Alamgir |
| 26. |  | Khaderiala guide wall | 25 Ft | 2182 Ft | 2:1 on either side | Sarai Alamgir |
| 27. |  | New Paharwal Guide wall | 25 Ft | 1980 Ft | 2:1 on either side | Sarai Alamgir |
| 28. |  | Kharka Guide wall | 25 Ft | 1444 Ft | 2:1 on either side | Sarai Alamgir |
| 29. |  | Gurha Jattan Guide wall | 25 Ft | 2105 Ft | 2:1 on either side | Sarai Alamgir |
| KAHAN RIVER (Mangla Canal Sub Division) |  |  |  |  |  |  |
| 1. | Stud No.1 |  | 15 Ft | 250 Ft | 2:1 on either side | 1986-90 |
| 2. | Stud No.2 |  | 15 Ft | 330 Ft | 2:1 on either side | 1986-90 |
| 3. | Stud No.3 |  | 15 Ft | 200 Ft | 2:1 on either side | 1986-90 |
| 4. | Stud No.4 |  | 15 Ft | 350 Ft | 3:1 on either side | 1986-90 |
| 5. | Stud No.5 |  | 15 Ft | 300 Ft | 2:1 on either side | 1986-90 |
| 6. | Stud No.6 |  | 15 Ft | 700 Ft | 2:1 on either side | 1986-90 |
| 7. | Gurha Ahmed Bund (R) |  | 25 Ft | 10900 Ft | 2:1 on either side | An ADP scheme of extension of Gurha Ahmed flood bund to length of 1850 ft amounting to Rs. about 85 million must be included in ADP. |
| 8. | Naugran Flood Bund (L) |  | 20 Ft. | 19500 Ft | 2:1 on either side | An ADP scheme of "Rehabilitation and extension of Naugran Flood Bund" has been prepared including pitching and apron amounting to Rs.1300/- million which must be included in ADP for proper repair of Bund. |
| BHIMBER NULLAH (Khokhra Canal Sub Division) |  |  |  |  |  |  |
| 1. | UJC Protection Bund RD.293-299/L |  | 25 Ft | 6000 Ft | 1½:1 on either side |  |
| 2. | UJC Protection Bund RD.305-320/L |  | 25 Ft | 15000 Ft | 1½:1 on either side |  |
| 3. | UJC Protection Bund RD.418-438/L |  | 25 Ft | 20000 Ft | 1½:1 on either side |  |
| 4. | UJC Protection Bund RD.427-440/R |  | 25 Ft | 13000 Ft | 1.½:1 on either side |  |

XEN/JLM

| Sr. No. | Name of Bund or Stud | Top width | Length | Side Slopes (River Side / Country Side) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Nullah Bunahan (Mangla Canal Sub Division) |  |  |  |  |
| 1 | Bhamber Flood Bund on Nullah Bunahan (L) | Village Bhambar | 7400 Ft | 20 Ft | 2:1 on either side |

BUNDS ALONG LEVEL CROSSINGS.

| 1. | BONG LEVEL CROSSING (Mirpur) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| i | U/S Right Bund | 20 Ft | 300 Ft | Pitched 1.5:1/2:1 |  |
| ii. | U/S Left Bund | 20Ft | 2500Ft | Pitched 1.5:1/2:1 |  |
| iii. | D/S Right Bund | 20 Ft | 150 Ft | Pitched 1.5:1/2:1 |  |
| iv | D/S Left Bund | 20 Ft | 2900 Ft | Pitched 1.5:1/2:1 |  |
| 2. | JATLI LEVEL CROSSING (Mirpur) |  |  |  |  |
| i. | U/S Right Bund | 30Ft | 925 Ft | Pitched 1.5:1/2:1 |  |
| ii. | U/S Left Bund | 30 Ft | 4550 Ft | Pitched 1.5:1/2:1 |  |
| iii. | D/S Right Bund | 30 Ft | 1250 Ft | Pitched 1.5:1/2:1 |  |
| iv. | D/S Left Bund | 30 Ft | 1250 Ft | Pitched 1.5:1/2:1 |  |
| 3. | REHMANPUR LEVEL CROSSING (Mirpur) |  |  |  |  |
| i. | U/S Right Bund | 30 Ft | 6000 Ft | Pitched 1.5:1/2:1 |  |
| ii. | U/S Left Bund | 30 Ft | 1000 Ft | Pitched 1.5:1/2:1 |  |
| iii. | D/S Right Bund | 30 Ft | 1000 Ft | Pitched 1.5:1/2: |  |
| iv. | D/S Left Bund | 30 Ft | 1000 Ft | Pitched 1.5:1/2: |  |
| - JAGGU LEVEL CROSSING (Sarai Alamgir) |  |  |  |  |  |
| i. | U/S Right Bund | 25 Ft | 4750 Ft | Pitched1.5:1/2:1 |  |
| ii. | U/S Left Bund | 25 Ft | 2705 Ft | Pitched 1.5:1/2:1 |  |
| iii. | D/S Right Bund | 20 Ft | 1000 Ft | Pitched 1.5:1/2:1 |  |
| iv. | D/S Left Bund | 25 Ft | 1000 Ft | Pitched 1.5:1/2:1 |  |
|  | TOTAL |  | 176415Ft. |  |  |
|  | G.TOTAL |  | 56.61 KM |  |  |

XEN/JLM

2.2	LOCATION MAP

CHAPTER-3

BREIF HISTORY OF PAST FLOOD EVENTS

RIVER JHELUM.

Since construction of Mangla Dam, flood discharges in River Jhelum D/S Mangla are very low, however, in 1992, an exceptionally high flood of 10,50,000 Cs. was experienced in River Jhelum. The abadies along River Jhelum on both banks were affected. Jhelum City was inundated with heavy loss to lives and properties. Vast lands of villages U/S Jhelum city on right bank were eroded. Consequently 17 No. Studs were constructed in 1994-95 for the protection of these villages against erosion.

The tendency of River flow D/S G.T. Road Bridge was towards left Bank and eroded away the abadies and agricultural land of villages Chak Sikandar, Kharka, Gurha Jattan, Bhalwal Gharbi, Paharwal. 6 No. Guide walls were constructed to safeguard these villages.

KAHAN RIVER

Normally 60,000 to 70,000 Cs. discharge is experienced in River Kahan, but in 1992 about 1,20,000 Cs. was recorded. The velocities are very high due to steeper slope. Six No. spurs for protection of village Chak Almas, Kalwantpur, Shahmadpur, Bagh, Rajupindi and Chah Gunja were constructed during 1986-87 and 1989-90 to save these abadies against erosion. A flood embankment for protection of village Gurha Ahmed has been constructed in a length of 10900 ft. in the year 1994-95 and 2016-17. Another flood embankment namely, “Naugran Flood Protection Bund” was constructed in the year 1929 by District Council Jhelum and has been rehabilitated under ADP up to length of 19500 Feet without pitching and apron for which ADP scheme amounting to Rs. 1300/- million has been framed.

BHIMBER NULLAH

Bhimber Nullah is a tributary of river Chenab and directly hits the left bank of UJC at RD 414-425.

XEN/JLM

CHAPTER-4

# DESIGN DATA HISTORIC PEAK FLOOD DATA AND PREVIOUS  YEARS FLOOD DATA OF HEAD WORKS/ BARRAGES AND / OR OTHER CONTROL POINTS.

Mangla Dam

Crest Level  		             = 1264 ft amsl

Normal Pond Level  (NPL)      = 1242 ft amsl

Critical Level for flood              = 1232 ft amsl

Critical Level for preparation   = 1220 ft amsl

Dead Storage Level  (DSL)     = 1050 ft amsl

Maximum storage at 1242	 = 7.277 MAF

4.1	FLOOD LIMITS

River Jhelum at Railway Bridge

| 1. Low flood. | 0.75 lac to 1.10 lac Cs. |
| --- | --- |
| 2. Medium Flood | 1.10 lac to 1.50 lac Cs. |
| 3. High Flood | 1.50 lac to 2.25 lac Cs. |
| 4. Very High Flood | 2.25 lac and above. |

4.2	TIME LAGS OF FLOODS

The flood water from Mangla to Railway Bridge Jhelum reaches in 7 hours after traversing a distance of 20 miles whereas from Railway Bridge to Rasul Barrage also takes 7 hours.

4.3	HIGHEST FLOODS

| YEAR | Max. Discharge (in Cs.) |
| --- | --- |
| 1988 | 2,53,000 |
| 1992 | 10,50,000 |
| 1995 | 2,80,000 |
| 1997 | 4,64,000 |
| 2010 | 2,26,000 |
| 2014 | 4,80,000 |

XEN/JLM

4.4	PEAK  DISCHARGES JHELUM RIVER

| YEAR | Max. Discharge (in Cs.) | YEAR | Max. Discharge (in Cs.) |
| --- | --- | --- | --- |
| 1980 | 58,146 | 2015 | 129000 |
| 1981 | - | 2016 | 56700 |
| 1982 | - | 2017 | 55000 |
| 1983 | 88,445 | 2018 | 73792 |
| 1984 | 49,993 | 2019 | 105000 |
| 1985 | - | 2020 | 125000 |
| 1986 | 1,03,085 | 2021 | 43000 |
| 1987 | 1,35,000 | 2022 | 55000 |
| 1988 | 2,53,000 | 2023 | 50000 |
| 1989 | 1,53,174 | 2024 | 70000 |
| 1990 | 1,25,247 |  |  |
| 1991 | 1,04,692 |  |  |
| 1992 | 10,50,000 |  |  |
| 1993 | 1,00,000 |  |  |
| 1994 | 1,03,430 |  |  |
| 1995 | 2,80,000 |  |  |
| 1996 | 1,45,336 |  |  |
| 1997 | 4,64,000 |  |  |
| 1998 | 79,500 |  |  |
| 1999 | 45,000 |  |  |
| 2000 | 31,000 |  |  |
| 2001 | 30,000 |  |  |
| 2002 | 40,000 |  |  |
| 2003 | 78,000 |  |  |
| 2004 | 42,000 |  |  |
| 2005 | 92,000 |  |  |
| 2006 | 86,000 |  |  |
| 2007 | 41,000 |  |  |
| 2008 | 37,000 |  |  |
| 2009 | 72,000 |  |  |
| 2010 | 2,26,000 |  |  |
| 2011 | 1,20,000 |  |  |
| 2012 | 82,950 |  |  |
| 2013 | 41,000 |  |  |
| 2014 | 480,000 |  |  |

XEN/JLM

CHAPTER-5

FLOOD FIGHTING STRATEGY

In order to combat the flood, following plan is to be put in operation for each River or Nullah and protective works as discussed under: -

RIVER JHELUM

Due to construction of Mangla Dam there is very rare flood trouble in Jhelum River downstream Mangla. However, any flood warning will be tackled as under.

- Low Flood (0.75 lac to 1.10 lac Cs.)

The Sub Engineer Mangla will remain present at Head quarter during the low floods. Sub Engineer Jhelum will remain present at Jhelum Spurs with watching establishment. Sub Engineer Rajar will remain present at Rajar and shall keep the guide wall along left bank of River Jhelum under watch.

- Medium Flood  (1.10 lac to 1.50 lac Cs.)

The Sub Engineer Mangla will remain present at Head quarter during the low and medium floods. Sub Engineer Jhelum will remain present at Jhelum Spurs with watching establishment. Sub Engineer Rajar will remain present at Rajar and shall keep the guide wall along left bank of River Jhelum under watch.

- HIGH FLOOD (1.50 lac to 2.25 lac Cs)

On receipt of high flood warning, Sub Divisional Officer, Mangla will be stationed at office of Jhelum Canal Division at Jhelum and Sub Engineer Bong will patrol right bank of UJC from RD. 27+500 to 41+000. He will be responsible to watch and report any unusual behavior of River Jhelum as far as its tendency to move closer to UJC in reach RD. 27+000-41+000. Sub Engineer Jhelum with watching establishment, shall watch all the flood works around Jhelum City.

- VERY HIGH FLOOD (2.25 lac and above)

On receipt of very high flood warning, downstream Mangla, the Sub Divisional Officer Mangla will keep Executive Engineer, Jhelum Canal Division and Project Director, Small dams Circle-II, Jhelum informed through telephone regarding the situation of Jhelum protection works. He will supervise flood works on River Jhelum. All the flood works will be kept under day & night watch with flood fighting material to meet any emergency. Sub Divisional Officer Khokhra will supervise day and night watching of guide bunds and will keep Executive Engineer, Jhelum Canal Division and Project Director, Small dams Circle-II, Jhelum informed about the latest situation.

XEN/JLM

RIVER CHENAB AT KHANKI BARRAGE

Flood warnings from Marala Headworks and Khanki Headworks will be transmitted through telephone to flood information center, Jhelum as well as Sub Divisional Officer, Khokhra Canal Sub Division.

i).	Low  Floods (1 to 1.50 Lac Cs.)

Sub Engineer, Shadiwal section, will shift to Shahdiwal Rest house and will inform Sub Divisional Officer, Khokhra Canal Sub Division about flood situation in Chenab River.

ii)	Medium Floods (1.50 to 2 Lac Cs.)

Sub Engineer, Shadiwal section, will shift to Shahdiwal Rest house and will inform Sub Divisional Officer, Khokhra Canal Sub Division about flood situation in Chenab River.

iii)	HIGH FLOOD (2 to 4 Lac Cs)

Sub Divisional Officer, Khokhra Canal Sub Division will supervise watching operations of banks of Upper Jhelum Canal from RD 150 to Tail.

iv)	Very High Flood (4 to 6.0 Lac Cs)

Sub Divisional Officer, Khokhra Canal Sub Division and will supervise watching operations of banks of Upper Jhelum Canal from RD 150 to Tail at RD 440+000. Deputy Commissioner and Superintendent of Police, Gujrat will be approached to assist the Department in case of any emergency.

v)	Exceptionally High Floods (6.0 Lac Cs and above)

Full mobilization of all resources shall be done. All the flood works will be kept under day and night watch.

Bhimber Nullah along Tail reach UJC, RD. 414+000 to 425+000.

Sub Engineer Shadiwal will remain present, watch and transmit gauges of Bhimber to all concerned round the clock during floods. Sub Divisional Officer Khokhra shall also supervise the operation.

XEN/JLM

CHAPTER-6

FLOOD DAMAGES RESTORATION WORKS

Due to constant heavy rainfall with high intensity throughout the vast area as well as in District Jhelum to the tune 104 mm on 30.08.2024, breaching of 5 No. local mini dams and receiving of run off from vast catchment area starting from Gujar khan, a heavy freshet entered into Bakrala Nullah the tributary of Kahan Nullah. Furthermore, breaching of 05 No. local mini Dams situated in vicinity of Domeli and Sohawa, besides excessive spilled out water of Tain pura Dam No.I & II, Domeli Dam, Gurha Uttam Singh Dam and Salial Dam caused devastating flooding.  Resultantly, all excessive rain / flood water entered into Kahan River and directly hit the flood bund. Ultimately, the Kahan Nullah could not bear the huge pressure of rain / flood water and the Naugran flood bund could not sustain such pressure due to burrowing animals like rats / porcupine holes and breached between RD.14-15, and also directly hit the bund between RD.9-10 and RD.13-14 which was timely controlled by tree launching. Due to continuous monitoring and timely interventions, the situation was brought under control, and the potential widespread destruction was mitigated.

The breached site between RD 14-15 was closed as per design section. Strengthening of reach at RD 13-14 and 9-10 was also done at site. The bund is still in dangerous condition as it is just an earthen bund, and flow has become parallel to flood bund causing erosion which necessitates provision of pitching and apron stone. An ADP scheme of "Rehabilitation and extension of Naugran Flood Bund" has been prepared including pitching and apron amounting to Rs.1300/- million which must be included in ADP for proper repair of Bund.

XEN/JLM

CHAPTER-7

FLOOD FIGHTING WATCHING ARRANGEMENT

7.1	PRE FLOOD ARRANGEMENT

The material as detail attached shall be stored at the following sites.

STATUS OF RESERVE STOCK

| Sr. No. | Name of Division | Reserve Stock Limit (As per Yard Stick) | Quantity available | Balance Required | Tentative Cost of Stone (Rs. In Million) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | Jhelum | 5.20 | 1.258 | 3.942 | 66.58 | Procurement of reserve stock against minimum requirement has been tried by this division, but no success has so far been achieved |

Detail is as under:-

| Sr. No. | Name of Structure | Reserve Stock (Lac Cft) |  |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  | Limit | Available | Required |  |
| 1. | Old Paharwal Guide Wall on Left Bank of River Jhelum | 1.00 | 0.20 | 0.80 | Will be procured before the Flood Season on receipt of funds. |
| 2. | Kharka Guide Wall on Left Bank of River Jhelum | 1.00 | 0.00 | 1.00 |  |
| 3. | Gurha Jattan Guide Wall on Left Bank of River Jhelum | 1.00 | 0.00 | 1.00 |  |
| 4. | Malo Kas on Left Bank of River Jhelum. | 0.20 | 0.00 | 0.20 |  |
| 5. | New Paharwal Guide Wall on left Bank of River Jhelum. | 0.40 | 0.00 | 0.40 |  |
| 6. | Khaderiala Guide wall on Left Bank of River Jhelum. | 0.40 | 0.20 | 0.20 |  |
| 7. | Gurha Ahmed Bund on Kahan River. | 0.30 | 0.324 | 0.00 |  |
| 8. | Jhelum Cantt: Bund on Right Bank of River Jhelum. | 0.20 | 0.174 | 0.08 |  |
| 9. | Jhelum Protection works & studs on Right Bank of River Jhelum. | 0.50 | 0.36 | 0.14 |  |
| 10. | Spurs on Kahan River. | 0.20 | 0.00 | 0.20 |  |
|  | Total | 5.20 | 1.258 | 3.942 |  |

RIVER JHELUM

| 1. Jhelum city flood bund. | For watching Protection works around Jhelum city |
| --- | --- |
| 2. Jhelum Cantonment flood bund | For watching Cantt Bund on River Jhelum |

XEN/JLM

KAHAN RIVER

| 3. Naugran Flood Bund | For watching Naugran flood bund on Kahan River. |
| --- | --- |
| 4. Gurha Ahmed Bund. | For watching Gurha Ahmed Bund on Kahan River |
| 5. Rajar Rest House. | For watching/Flood fighting on Guide Bunds at Paharwal, Khaderiala and Shah Madar etc |

BHIMBER NULLAH

| 6. Shadiwal Rest House | For watching of left bank of UJC from RD.414+000 to 425+000 and back flow through River Chenab U/S Khanki Barrage |
| --- | --- |

7.2	WATCHING ESTABLISHMENT

Following watching establishment will be engaged during the flood season for watching and general maintenance of flood works on monthly basis. However, in case of very high flood or in real emergency, the strength of watching establishment will be increased with the express permission of Executive Engineer, Jhelum Canal Division.

| Item | Flood Status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low |  | Medium |  | High |  | Very High |  | Exceptionally High |  |
|  | Men | Shift | Men | Shift | Men | Shift | Men | Shift | Men | Shift |
| - Jabbar Bund. | 1 | - | 1 | 2 | 3 | 2 | 4 | 2 | 5 | 3 |
| - UJC RD. 27500-41000. | 1 | - | 1 | 1 | 2 | 1 | 3 | 1 | 5 | 2 |
| - Jhelum Protection Works (Studs & Bunds). | 10 | - | 30 | 2 | 40 | 2 | 45 | 2 | 50 | 3 |
| - Guide bunds on left bank of River Jhelum. | 8 | - | 1 | - | 4 | 2 | 25 | 1 | 35 | 2 |
| - Kahan River. (Gurha Ahmed Bund & Studs).+ Naugran Flood Bund | 3 | - | 16 | 1 | 1 | 1 | 22 | 1 | 30 | 2 |
| - UJC RD. 414+000-425+000. | 1 | - | 4 | 1 | 2 | 1 | 6 | 1 | 10 | 1 |

Note:- 	Payment of Machinery and work charge Establishment subject to verification by TPM Consultants.

XEN/JLM

7.3	ARRANGEMENT AT SENSITIVE SITE

1.	Jhelum city flood bund.

2.	Jhelum Cantonment flood bund.

3.	Naugran Flood Bund.

4.	Gurha Ahmed Bund

5.	Rajar Rest House for Guide bunds on left bank of River Jhelum

6.	Shadiwal Rest House  for UJC RD. 414+000-425+000.

7.4	WATCHING MATERIAL

| S.No. | ITEM | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Torches Chargeable (Medium) | 10 No. | 10 No. | NIL | Sufficient quantity is available |
| 2. | New Gunny Bags (4-6 Cft) with Sulti and sewing needles | 100 No. | 100 No. | NIL |  |
| 3. | Axes large | 5 No. | 5 No. | NIL |  |
| 4. | Kassies with Handles | 10 No. | 10 No. | NIL |  |
| 5. | Long Shoes | 6 No. | 6 No. | NIL |  |
| 6. | Rain Coat | 6 No. | 6 No. | NIL |  |
| 7. | Umbrella | 6 No. | 6 No. | NIL |  |
| 8. | Jug Glass (1 Jug +6 Glass) | 1 Set. | 1 Set. | NIL |  |
| 9. | Shouldari Medium | 1 No. | 1 No. | NIL |  |
| 10. | Generator set. | 1 No. | 1 No. | NIL |  |
| 11. | Service Cable | 4 Coils | 4 Coils | NIL |  |
| 12. | Search light (Large) | 2 No. | 2 No. | NIL |  |
| 13. | Wooden Mallet | 2 No. | 2 No. | NIL |  |
| 14. | Petrol | 500 Liter | Will be procured as per situation |  |  |
| 15. | M. Oil | 10 Liter |  |  |  |
| 16. | Water Cooler (Medium Size) | 2 No | 2 No | NIL |  |
| 17. | First aid Box | 1 No. | 1 No. | NIL |  |

The Flood Fighting Material has already been purchased.

7.5	ARRANGEMENT FOR SOUNDING AND PROBING

Sub Divisional Officer concerned will be responsible for making necessary arrangement for observing soundings/ probing of structures in his jurisdiction with the help of Sub Engineers concerned.

7.6	LIGHTING ARRANGEMENT

During peak flood hours, flood fighting operations are carried out round the clock. Torches and rechargeable emergency lights will be arranged, which will facilitate the field formation for successful flood fighting operation. Regular field staff is required to be trained properly and thoroughly to carry out flood watching operations during odd hours. Material will be issued through proper indents.

XEN/JLM

7.7	RATION ARRANGEMENT

Necessary arrangements will be made for ration for the flood watching/ fighting establishment.

7.8	POL ARRANGEMENT FOR VEHICLE

In case the petrol, Oil and lubricants are in short supply on account of disruption for communication elsewhere in the country, the Executive Engineer Incharge will make timely arrangements for having adequate P.O.L reserve at conveniently places at one or more petrol pumps at Dina, Sarai Alamgir, Jhelum and Mandi Bahauddin in consultation with concerned Deputy Commissioner/ Deputy District Revenue Officers.

7.9	TRANSPORTATION

Already allocated vehicles to the Division will be used during flood season. In case of an exceptionally high flood in River Jhelum, Deputy Commissioner, will be requested to supply additional vehicles for the Supervisory staff and for transportation of labour as well as flood fighting material to the sites of work. A Tractor along with Front & rear blade and trolley has allotted to Jhelum Canal Division for transportation of material etc. Sand Bag filling Machine are also available for any emergency situation on Flood protection works in Jhelum Canal Division.

7.10	LAW AND ORDER

Experience shows that whenever any flood emergency arises, inhabitants of the area try to cut the protection bund at upstream to divert the flood threat from their locality. In such a situation it would be necessary for District Administration to provide.

- Day and night patrolling to guard against any attempt to cut a bund or structure.

- Provision of Police Force for any open and defiant attempt of breaching the bund at any place.

In these situations, the Deputy Commissioner and District Police Officer of the respective District would be expected to promptly provide adequate police force accompanied with a Magistrate on being apprised of the situation by Executive Engineer In-charge.

7.11	MEDICAL ARRANGEMENT FOR LABOUR

Medical arrangements will be made with the help of District Government / District Health Officer.

XEN/JLM

7.12	LIASON WITH OTHER DEPARTMENTS

The Deputy Commissioner and District Police Officer will be contacted in advance and a list of volunteers will be prepared who can be contacted during emergency. The list containing the names of such volunteers and the village Lumbardars made responsible by the Civil Authorities will also be maintained.

7.13	ROLE OF ARMY

Role of Army Authorities during pre flood and in the flood season is very important. Army Authorities have visited all the flood protection works along with the departmental inspection team. During flood and in emergency, Army is always available to assist the civil departments fully equipped with all kinds of machinery and man power.

7.14	DUTIES OF TELEPHONE ATTENDANT

| Sr. # | Name of Official | Period | Time |  | Ph. # | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | From | To |  |  |
| 1 | Muhammad Riaz Head Signaller | Flood Season | Overall Incharge |  | 0304-8633098 | Telegraph Office will be opened 24 Hours in Flood season. |
| 2. | Hafiz Ghulam Murtaza Signaller | Flood Season | 6.00 | 14.00 | 0331-9606258 |  |
| 3. | Saqlain Haider operator Signaller | Flood Season | 6.00 | 14.00 | 0315-3685100 |  |
| 4. | Hafiz Waqar Naib Qasid | Flood Season | 6.00 | 14.00 | 0345-5699889 |  |
| 5. | Asjad Mehmood Telephone Attendent | Flood Season | 14.00 | 22.00 | 0322-6745997 |  |
| 6. | Adeel Ali Naib Qasid | Flood Season | 14.00 | 22.00 | 0348-5524092 |  |
| 7. | Sajid Mehmood | Flood Season | 22.00 | 6.00 | 0302-5621164 |  |

Flood Control Centre will be setup in Canal Colony Jhelum from 1st July 2025 to 15th October 2025, which will remain open round the clock and is well equipped with telephone, whatsapp etc. to receive / pass important messages from all flood related quarters and will be responsible for supplying flood information to all concerned as well as public (Telephone numbers 0544-9270330).

7.15	WIRELESS ARRANGEMENTS

The wireless communication network does not exist with the Jhelum Canal Division and important flood messages are received through Mobile phone / WhatsApp etc.

XEN/JLM

CHAPTER-8

DETAIL OF ENCROACHMENT

River Jhelum

- Normal

| Total No. Encroachment | No. of Encroachment removed during last Year | Balance Encroachment | Remarks |
| --- | --- | --- | --- |
| NIL | NIL | NIL | NIL |

- Critical Encroachment

| Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- |
| NIL | NIL | NIL | NIL |

XEN/JLM

| 9. DUTY ROSTER FLOOD FIGHTING PROGRAM. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SITE | LENGTH | STAGE-I DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-II DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-III DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-III DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-III DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | Remarks |
|  |  |  |  | MATE | BELDAR |  |  | MATE | BELDAR |  |  | MATE | BELDAR |  |  | MATE | BELDAR |  |  | MATE | BELDAR |  |
| Jabba Band | - | 0.75 lac to 1.10 lac cs. | 2 | 1 | 1 | 1.10 lac to 1.50 lac cs. | 3 | 1 | 2 | 1.50 lac to 2.25 lac cs. | 4 | 1 | 3 | 2.25 lac to 6.00 lac cs. | 4 | 1 | 3 | 6.0 lac and above | 7 | 2 | 5 |  |
| UJC RD 27+500 to 41+000 | 13500 |  | 4 | 1 | 2 |  | 3 | 1 | 2 |  | 4 | 1 | 3 |  | 4 | 1 | 3 |  | 7 | 2 | 5 |  |
| JHELUM prptection Works (Stud & Bund) | - |  | 25 | 2 | 23 |  | 30 | 3 | 27 |  | 40 | 4 | 36 |  | 45 | 6 | 39 |  | 50 | 8 | 43 |  |
| Guide Bunds on Left Bank of River Jhelum | - |  | 18 | 1 | 17 |  | 20 | 1 | 19 |  | 4 | 1 | 3 |  | 25 | 2 | 23 |  | 35 | 3 | 32 |  |
| Kahan River (Gurah Ahmed Bund & Naugran Flood bund) | - |  | 8 | 1 | 7 |  | 16 | 1 | 15 |  | 2 | 1 | 1 |  | 22 | 2 | 20 |  | 30 | 3 | 27 |  |
| UJC RD 414+500 to 425+000 | 11000 |  | 2 | - | 2 |  | 4 | 1 | 3 |  | 2 |  | 2 |  | 6 | 1 | 5 |  | 10 | 1 | 9 |  |

XEN/JLM

CHAPTER-10

EMERGENCY TELEPHONE NUMBERS

| Sr. No. | Name | Designation | Office | Residence / Mobile |
| --- | --- | --- | --- | --- |
| 1. | Muhammad Kazim Pirzada | Minister Irrigation | 042-99212809-10 | 0300-8454445 |
| 2. | Dr. Wasif Khurshid | Secretary, Irrigation Department | 042-99212117 042-99212118 | 042-35731116 |
| 3. | Khuram Amin | Additional Secretary (Tech:) | 042-99212128 | 0301-8466321 |
| 4. | Kashif Shamim | Additional Secretary (OP) | 042-99213612 | 0300-8416196 |
| 5. | Vaccant | Deputy Secretary (Admn) | 042-99212126 |  |
| 6. | Hamad Ali | Deputy Secretary (OP) | 042-99212127 |  |
| 7. | Muhammad Usman | Deputy Secretary (Dev) | 042-99212129 | 0333-4136600 |
| 8. | Ammar Fasheen | Section Officer Flood | 042-99212134 | 0306-6201450 |

POTOHAR IRRIGATION ZONE RAWALPINDI/ISLAMABAD

| 1. | Muhammad Nawaz batti | Chief Engineer, Potohar Zone Islamabad | 051-9255749 | 0300-6763112 |
| --- | --- | --- | --- | --- |
| 2. | Tabraiz Ahmed | Project Director-II Jhelum | 0300-7624714 | 0300-7624714 |
| 3. | Ali Haider | Executive Engineer (OP), Islamabad | 0333-8880654 |  |
| 4. | Mubashir Majeed | Executive Engineer, Jhelum Canal Division, Jhelum | 0544-9270331 Fax-9270311 | 05449270331 03325449212 |
| 5. | Riasat Ali Malik | Sub Divisional Officer, Mangla |  | 0321-7756211 |
| 6. | Muhammad Tahir | Sub Divisional Officer, Khokhra (Rasul) |  | 03238008007 |
| 7. | Adeel Shah | Flood Control Centre Irrigation Jhelum | 0544-9270330 |  |

DISTRICT ADMINISTRATION JHELUM

| 1. | Mesam Abbas Shah | Deputy Commissioner Jhelum | 0544-9270081 | 0323-5000012 |
| --- | --- | --- | --- | --- |
| 2. | Tariq Aziz | District Police Officer Jhelum | 0544-9270042 0544-9270258 | 0544-9270043 |
| 3. | Saba Sher | Assistant Deputy Commissioner (G) Jhelum | 0544-9270109 | 0301-4423004 |
| 4. | Shamsur Rehman | Assistant Commissioner Jhelum | 0544-9270012 | 0544-9270012 |
| 5. | Madam Safa Abid | Assistant Commissioner Dina | 0544-280371 | 03337536977 |
| 6. | Afzaal Choudary | Assistant Commissioner Sohawa | 0544-224036 | 0324-9747930 |

XEN/JLM

DISTRICT ADMINISTRATION GUJRAT

| 1. | Safdar Hussain Virk | Deputy Commissioner Gujrat | 053-9260010 | 053-9260011 0304-0920066 |
| --- | --- | --- | --- | --- |
| 2. | Mastansar Atta | District Police Office Gujrat | 053-9260026 | 053-9260026 |
| 3. | Khizer Hayat bhatti | Assistant Deputy Commissioner (General) Gujrat | 053-9260300 | 053-9260061 |
| 4. | Bilal Zubair | Assistant Commissioner Gujrat | 053-9260063 | 0331-4964919 |
| 5. | Ahmad Shair Gondal | Assistant Commissioner Kharian | 053-9240212 | 0343-7546426 |
| 6. | Farooq Azam | Assistant Commissioner Sarai Alamgir | 0544-284530 | 0345-4917495 |

DISTRICT ADMINISTRATION M.B.DIN

| 1. | Dr.Muhammad Faisal Saleem | Deputy Commissioner M.B Din | 0546-504220 | 0546-504220 0304-0920068 |
| --- | --- | --- | --- | --- |
| 2. | Waseem Riaz | District Police Office M.B Din | 0546-502324 | 0546-502324 0323-595999 |
| 3. | LT.(R) Muhammad Ahmed Zaheer | Assistant Deputy Commissioner (General) M.B Din | 0546-506488 | 0335-3733034 |
| 4. | Nadeen Blooch | Assistant Commissioner M.B Din | 0546-650224 | 0546-650224 0314-2615656 |
| 5. | Riaz Ahmad | Assistant Commissioner Malikwal | 0546-591276 | 0300-4487417 |
| 6. | Fazail Mudasser | Assistant Commissioner Phalia | 0546-596460 | 0316-6805291 |

FLOOD CONTROL CENTRE PUNJAB, LAHORE

| 1. | Flood Warning Centre 46-Jail Road Lahore. | 042-99205157 |  |
| --- | --- | --- | --- |
| 2. | Flood Cell I & P Secretariat Lahore. | 042-99212134 | 042-9212116 |
| 3. | Police Wireless Control, Jail Road Lahore. | 042-99205156 | - |
| 4. | Flood Emergency Board of Revenue | 042-9204408-9 | 042-9204405 |
| 5. | National Flood Forecasting Bureau Lahore. | 042-99200139 |  |
| 6. | Senior Meteorologist. | 042-99200208 | 042-99200144 |
| 7. | Chief Engineer (D&F) Zone Lahore. | 042-35291304 | 042-5900049 |
| 8. | Director Flood D & F Zone Lahore | 042-99231614 0300-4777120 | 042-6653943 |
| 9. | Flood Cell I & P Secretariat Lahore. | 042-99212134 | - |

MILITARY ENGINEER’S TELEPHONE NUMBERS

| 1. | Headquarter Engineer 30-Corps Gujranwala Cantt. | 0553-269/2353 | - |
| --- | --- | --- | --- |
| 2. | 11-Engineer Battalion Sialkot Cantt. | 052-571/5186 | - |
| 3. | Headquarter Engineer 1-Corps Kharian Cantt | 053-59/2364 | - |

XEN/JLM

CHAPTER-11

STANDARD OPERATING PROCEDURE (SOP) FOR BREACHING SECTION.

11.1	HISTORY OF BREACHING SECTION

There is no breaching section in this Division.

11.2	LOCATION, DESIGN, QUALITY AND VARIETY OF THE 	EXPLOSIVE REQUIRED FOR DETONATION

Not applicable.

11.3	ARRANGEMENT OF EXPLOSIVE STORES

Not applicable.

11.4	LIST OF THE SECURITY STAFF ALONGWITH DETAIL OF THEIR 	TRAINING ETC.

Not applicable.

11.5	DETAIL OF MACHENICAL MEANS AS A STANDBY ARRANGEMENT IN CASE OF   DETONATION FAILURE.

Not applicable.

11.6	DUTY ROSTER IN CASE OF CRITICAL SITUATION

Please see Ch-9.

11.7	BREACHING COMMITTEE WITH THEIR ACTION PLAN

Not applicable.

11.8	LIST OF THE VILLAGES LIKELY TO BE INUNDATED IN CASE 	OF 	BREACH

Not applicable.

11.9	ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENT

Not applicable.

11.10	DETAIL OF COORDINATION WITH CIVIL/ ARMY AUTHORITIES

Please see Ch-10.

11.11	PARALLEL COMMUNICATION ARRANGEMENTS

Not applicable.

11.12	INDEX PLAN

Not applicable.

Project Director			      Chief Engineer		             Executive Engineer

Small Dam Circle-II                          	Irrigation Photohar Zone    		Jhelum Canal Division

Jhelum		                          	        Islamabad			     	 Jhelum

PART-B

CHAPTER-12

VULNERABLE SITES ON FLOOD BUNDS / STRUCTURES

  - APPREHENDED BREACHES IN FLOOD BUNDS / STRUCTURES

RIVER JHELUM

- Jhelum City

- Jhelum Cantonment Bund may breach at RD 1+200

RIVER KAHAN

- Naugran flood bund can breach between RD. 11+000 to 13+000.

- Gurha Ahmed Flood Bund on River Kahan can breach from RD.3+000- 3+500.

RIVER CHENAB

- Shadiwal Flood Bund

  - OPERATION OF BREACHING SECTIONS.

NIL

12.3   	BREACHES DUE TO RISE OF FLOOD WATER, DETERIORATION OF      FLOOD BUNDS.

NIL

XEN/JLM

CHAPTER-13

CONTINGENCY PLAN 2025

(COMING MONSOON SEASON)

- EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE NO.1: JHELUM CITY FLOOD BUND

- 13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

29-a.

- 13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

Jhelum City.

- 13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE:

Back flow through sewerage system will occur when discharge in River Jhelum increases upto 2 lac Cs. and may inundate Jhelum City. However, 8 No. disposal gates have recently been constructed/ Installed beneath the Jhelum City Flood Bund. Back watering will be controlled through these installed Gates.

- 13.3.1	ARRANGEMENTS

- Necessary arrangements have been made to meet with any emergency during Flood.

- 13.3.2	ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established Near Railway Bridge.

- 13.3.3	DUTIES OF OFFICERS/ OFFICIALS AND THEIR CAMP SITES

| Camp No. | Location | Officer Name Designation Mobile # | Official Name Designation Mobile # |
| --- | --- | --- | --- |
| 1 | Near Railway Bridge. | Malik Riasat Ali Sub Divisional Officer, Mangla 0321-7756211 | Shahzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 |

The Sub Divisional Officer, Mangla will shift his Head Quarter to Jhelum near Railway Bridge and will be responsible for flood fighting / restoration activities. The Sub Engineer Mangla and his establishment will assist the SDO Mangla in flood fighting activities. Mr. Amjad Mate will Act as Storekeeper.

XEN/JM

13.3.4		DEPARTMENTAL MACHINERY AVAILABLE

1 No tractor trolley is available with this Division.

13.3.5		MACHINERY AVAILABLE FROM PRIVATE SOURCE

Private machinery including tractor trolleys, excavators and dumpers are available near village Thune and Simbly Bridge opposite RD. 153-155 UJC and will be hired as and when required.

- 13.3.6		FLOOD FIGHTING MATERIAL REQUIRED

| S.No. | ITEM | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Torches Chargeable (Medium) | 6 No. | 6 No. | NIL | Sufficient quantity is available |
| 2. | New Gunny Bags (4-6 Cft) with Sulti and sewing needles | 100 No. | 100 No. | NIL |  |
| 3. | Axes large | 5 No. | 5 No. | NIL |  |
| 4. | Kassies with Handles | 10 No. | 10 No. | NIL |  |
| 5. | Long Shoes | 6 No. | 6 No. | NIL |  |
| 6. | Rain Coat | 6 No. | 6 No. | NIL |  |
| 7. | Umbrella | 6 No. | 6 No. | NIL |  |
| 8. | Jug Glass (1 Jug +6 Glass) | 1 Set. | 1 Set. | NIL |  |
| 9. | Shouldari Medium (camp) | 1 No. | 1 No. | NIL |  |
| 10. | Generator set. | 1 No. | 1 No. | NIL |  |
| 11. | Service Cable | 1 Coils | 1 Coils | NIL |  |
| 12. | Search light (Large) | 2 No. | 2 No. | NIL |  |
| 13. | Wooden Mallet | 2 No. | 2 No. | NIL |  |
| 14. | Petrol | 500 Liter | 500 Liter | NIL |  |
| 15. | M. Oil | 10 Liter | 10 Liter | NIL |  |
| 16. | Water Cooler (Medium Size) | 1 No | 1 No | NIL |  |
| 17. | First aid Box | 1 No. | 1 No. | NIL |  |

- 13.3.7		FLOOD FIGHTING MATERIAL AVAILABLE

Flood Fighting Material has already been purchased.

- 13.4	DETAIL OF OTHER INFRASTRUCTURES LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORKS, OTHER BUILDINGS, CANAL AND DRAINAGE WORKS.

Electric / Telephone lines installations are there across the River Jhelum besides important Government / public infrastructure in Jhelum City Area.

XEN/JM

- EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE NO.2: JHELUM CANTONMENT BUND RD 1+200

- 13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

31-a.

- 13.2		DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED.

Jhelum Cantt, Bagga, Saeela, Chottala, Khurrd and Sangoi.

- 13.3		STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL.

Water may spill over across the Jhelum Cantonment and adjoining villages Area when discharge exceeds limits of very high flood in River Jhelum (2.25 lac Cs.). Flood fighting will be done using all the available resources if and when required.

- 13.3.1		ARRANGEMENTS

- Necessary arrangements have been made to meet with any emergency during Flood.

- 13.3.2		ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD 0+500 of flood bund.

- 13.3.3		DUTIES OF OFFICERS/ OFFICIALS AND THEIR CAMP SITES

| Camp No. | Location | Officer Name | Official Name |
| --- | --- | --- | --- |
| 2 | Cantonment Bund RD 1+200. | Malik Riasat Ali Sub Divisional Officer, Mangla 0321-7756211 | Shahzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 |

The Sub Divisional Officer, Mangla will shift his Head Quarter to Jhelum Cantonment and will be responsible for flood fighting / restoration activities. The Sub Engineer Rahmanpur and his establishment will assist the SDO Mangla in flood fighting activities. Mr. Afzal Ahmed beldar will act as Storekeeper.

- 13.3.4		DEPARTMENTAL MACHINERY AVAILABLE

1 No tractor trolley is available with this Division.

XEN/JM

- 13.3.5		MACHINERY AVAILABLE FROM PRIVATE SOURCE

Private machinery including tractor trolleys, excavators and dumpers are available near village Thune and Simbly Bridge opposite RD. 153-155 UJC and can be hired as and when required.

- 13.3.6		 FLOOD FIGHTING MATERIAL REQUIRED

| S.No. | ITEM | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Torches Chargeable (Medium) | 5 No. | 5 No. | NIL | Sufficient quantity is available |
| 2. | New Gunny Bags (4-6 Cft) with Sulti and sewing needles | 100 No. | 100 No. | NIL |  |
| 3. | Axes large | 5 No. | 5 No. | NIL |  |
| 4. | Kassies with Handles | 10 No. | 10 No. | NIL |  |
| 5. | Long Shoes | 6 No. | 6 No. | NIL |  |
| 6. | Rain Coat | 6 No. | 6 No. | NIL |  |
| 7. | Umbrella | 6 No. | 6 No. | NIL |  |
| 8. | Jug Glass (1 Jug +6 Glass) | 1 Set. | 1 Set. | NIL |  |
| 9. | Shouldari Medium | 1 No. | 1 No. | NIL |  |
| 10. | Search light (Large) | 2 No. | 2 No. | NIL |  |
| 11. | Wooden Mallet | 2 No. | 2 No. | NIL |  |
| 12. | Water Cooler (Medium Size) | 1 No | 1 No | NIL |  |
| 13. | First aid Box | 1 No. | 1 No. | NIL |  |

- 13.3.7		FLOOD FIGHTING MATERIAL AVAILABLE

Flood Fighting Material has already been purchased.

- 13.4	DETAIL OF OTHER INFRASTRUCTURES LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORKS, OTHER BUILDINGS, CANAL AND DRAINAGE WORKS.

Electric / Telephone lines installations are there across the River Jhelum besides important infrastructure like Golf Club, Military dairy farm etc in Jhelum Cantonment Area.

XEN/JM

- EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE NO.3:

- RD 11+000 TO 13+000 NAUGRAN FLOOD BUND

- 13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

33-a.

- 13.2		DETAIL OF VILLAGE ABADIES LIKELY TO BE AFFECTED.

Village Naugran, Kotla Faqir and CMH Jhelum

13.3		STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL.

Day & Night watching will be done through Field Establishment especially when discharge exceeds limits of high flood in River Kahan. Back Water effects from River Jhelum may cause threats to Village Abadi of Naugran & Kotla Faqir, thus, Flood fighting will be done using all the available resources if and when required.

- 13.3.1		ARRANGEMENTS

- Necessary arrangements have been made to meet with any emergency during Flood.

- 13.3.2		ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD 11+000 of flood bund.

- 13.3.3		DUTIES OF OFFICERS/ OFFICIALS AND THEIR CAMP SITES

| Camp No. | Location | Officer Name Designation Mobile # | Official Name Designation Mobile # |
| --- | --- | --- | --- |
| 3 | NAUGRAN FLOOD BUND. RD 11+000 to 13+000 | Malik Riasat Ali Sub Divisional Officer, Mangla 0321-7756211 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 |

The Sub Divisional Officer, Mangla will shift his Head Quarter to Naugran Flood Bund and will be responsible for flood fighting/ restoration activities. Sub Engineer Jaggu, and his establishment will assist the SDO Mangla in flood fighting activities. Mr. Muhammad Amjad Mate will act as Storekeeper.

13.3.4		DEPARTMENTAL MACHINERY AVAILABLE

1 No tractor trolley is available with this Division.

- 13.3.5		MACHINERY AVAILABLE FROM PRIVATE SOURCE

XEN/JM

Private machinery including tractor trolleys, excavators and dumpers are available near village Thune and Simbly Bridge opposite RD. 153-155 UJC and can be hired as and when required.

- 13.3.6		FLOOD FIGHTING MATERIAL REQUIRED

| S.No. | ITEM | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Torches Chargeable (Medium) | 5 No. | 5 No. | NIL | Sufficient quantity is available |
| 2. | New Gunny Bags (4-6 Cft) with Sulti and sewing needles | 100 No. | 100 No. | NIL |  |
| 3. | Axes large | 5 No. | 5 No. | NIL |  |
| 4. | Kassies with Handles | 10 No. | 10 No. | NIL |  |
| 5. | Long Shoes | 6 No. | 6 No. | NIL |  |
| 6. | Rain Coat | 6 No. | 6 No. | NIL |  |
| 7. | Umbrella | 6 No. | 6 No. | NIL |  |
| 8. | Jug Glass (1 Jug +6 Glass) | 1 Set. | 1 Set. | NIL |  |
| 9. | Shouldari Medium | 1 No. | 1 No. | NIL |  |
| 10. | Search light (Large) | 2 No. | 2 No. | NIL |  |
| 11. | Wooden Mallet | 2 No. | 2 No. | NIL |  |
| 12. | Water Cooler (Medium Size) | 1 No | 1 No | NIL |  |
| 13. | First aid Box | 1 No. | 1 No. | NIL |  |

- 13.37		FLOOD FIGHTING MATERIAL AVAILABLE

Flood Fighting Material has already been purchased.

- DETAIL OF OTHER INFRASTRUCTURES LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORKS, OTHER BUILDINGS, CANAL AND DRAINAGE WORKS.

Electric / Telephone lines installations are there besides important infrastructure like CMH and other Army installations.

XEN/JM

- EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE NO.4: GURHA AHMED FLOOD BUND RD 3+000 TO 3+500

- 13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE  BREACH SUPPORTED WITH LEVELS.

35-a.

- 13.2		DETAIL OF VILLAGE ABADIES LIKELY TO BE AFFECTED.

Village Gurha Ahmed

- 13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL.

Day & Night watching will be done through Field Establishment especially when discharge exceeds limits of high flood in River Kahan. Back Water effects from River Jhelum may cause threats to Village Abadi of Gurha Ahmed, thus, Flood fighting will be done using all the available resources if and when required.

- 13.3.1		ARRANGEMENTS

- Necessary arrangements have been made to meet with any emergency during Flood.

- 13.3.2		ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD 3+500 of flood bund.

- 13.3.3		DUTIES OF OFFICERS/ OFFICIALS AND THEIR CAMP SITES

| Camp No. | Location | Officer Name Designation Mobile # | Official Name Designation Mobile # |
| --- | --- | --- | --- |
| 4 | GURHA AHMED FLOOD BUND RD 3+000 TO 3+500 | Malik Riasat Ali Sub Divisional Officer, Mangla 0321-7756211 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 |

The Sub Divisional Officer, Khokhra will shift his Head Quarter to Gurha Ahmed Flood Bund and will be responsible for flood fighting / restoration activities. Sub Engineer Pauranwala and his establishment will assist the SDO Khokhra in flood fighting activities. Mr. Sufian Ahmad will act as Storekeeper.

- 13.3.4		DEPARTMENTAL MACHINERY AVAILABLE

1 No tractor trolley is available with this Division.

XEN/JM

- 13.3.5		MACHINERY AVAILABLE FROM PRIVATE SOURCE

Private machinery including tractor trolleys, excavators and dumpers are 	available near village Thune and Simbly Bridge opposite RD. 153-155 UJC and can be hired as and when required.

- 13.3.6		FLOOD FIGHTING MATERIAL REQUIRED

| S.No. | ITEM | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Torches Chargeable (Medium) | 5 No. | 5 No. | NIL | Sufficient quantity is available |
| 2. | New Gunny Bags (4-6 Cft) with Sulti and sewing needles | 100 No. | 100 No. | NIL |  |
| 3. | Axes large | 5 No. | 5 No. | NIL |  |
| 4. | Kassies with Handles | 10 No. | 10 No. | NIL |  |
| 5. | Long Shoes | 6 No. | 6 No. | NIL |  |
| 6. | Rain Coat | 6 No. | 6 No. | NIL |  |
| 7. | Umbrella | 6 No. | 6 No. | NIL |  |
| 8. | Jug Glass (1 Jug +6 Glass) | 1 Set. | 1 Set. | NIL |  |
| 9. | Shouldari Medium | 1 No. | 1 No. | NIL |  |
| 10. | Search light (Large) | 2 No. | 2 No. | NIL |  |
| 11. | Wooden Mallet | 2 No. | 2 No. | NIL |  |
| 12. | Water Cooler (Medium Size) | 1 No | 1 No | NIL |  |
| 13. | First aid Box | 1 No. | 1 No. | NIL |  |

- 13.3.7		 FLOOD FIGHTING MATERIAL AVAILABLE

Flood Fighting Material has already been purchased.

- 13.4	DETAIL OF OTHER INFRASTRUCTURES LIKE ELECTRIC, SUI   GAS, TELEPHONE INSTALLATIONS, ROAD NETWORKS, OTHER BUILDINGS, CANAL AND DRAINAGE WORKS.

Electric lines and Roads.

XEN/JM

- EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE NO.5: SHADIWAL FLOOD BUND RD 31+000

- 13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

36-a.

- 13.2		DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED.

Village Qasim abad, Jassoki, Sadoki, Tarowal and Golaki.

- 13.3		STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL.

Back flow through River Chenab and U/S Head Khanki due to heading of water exceeding River discharge 3 Lac Cs. And also, it is effected by flash flooding in Nullah Bhimber joining Upper Jhelum Canal at RD 426.

- 13.3.1		ARRANGEMENTS

- Necessary arrangements have been made to meet with any emergency during Flood.

- 13.3.2		ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD 31000 of flood bund.

- 13.3.3		DUTIES OF OFFICERS/ OFFICIALS AND THEIR CAMP SITES

| Camp No. | Location | Officer Name Designation Mobile # | Official Name Designation Mobile # |
| --- | --- | --- | --- |
| 5 | SHADIWAL FLOOD BUND RD 31+000 | Muhammad Tahir Sub Divisional Officer, Khokhra 0321-7756211 | Imtiaz Sub Engineer, Shahdiwal 0347- 4607008 |

The Sub Divisional Officer, Khokhra will shift his Head Quarter to Shahdiwal rest house and will be responsible for flood fighting / restoration activities. The Sub Engineer Shahdiwal and his establishment will assist the SDO Khokhra in flood fighting activities. Mr. Shafqat Ullah Gauge Reader will act as Storekeeper.

- 13.3.4		DEPARTMENTAL MACHINERY AVAILABLE

1 No tractor trolley is available with this Division.

XEN/JM

13.3.5		MACHINERY AVAILABLE FROM PRIVATE SOURCE

Private machinery including tractor trolleys, excavators and dumpers are available near village Saroki Bridge opposite RD. 414385/L UJC and can be hired as and when required.

- 13.3.6		FLOOD FIGHTING MATERIAL REQUIRED

| S.No. | ITEM | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1. | Torches Chargeable (Medium) | 10 No. | NIL | 10 No. | Sufficient quantity is available, however missing items will be procured on availability of funds. |
| 2. | New Gunny Bags (4-6 Cft) with Sulti and sewing needles | 100 No. | NIL | 100 No. |  |
| 3. | Axes large | 5 No. | 5 No. | NIL |  |
| 4. | Kassies with Handles | 5 No. | 5 No. | NIL |  |
| 5. | Long Shoes | 6 No. | 6 No. | 6 No. |  |
| 6. | RainCoat | 6 No. | 6 No. | 6 No. |  |
| 7. | Umbrella | 6 No. | NIL | 6 No. |  |
| 8. | Jug Glass (1 Jug + 6 Glass) | 1 Set. | 1 Set. | NIL |  |
| 9. | Shouldari Medium | 1 No. | 1 No. | NIL |  |
| 10. | Search light (Large) | 2 No. | NIL | 2 No. |  |
| 11. | Wooden Mallet | 1 No. | 1 No. | NIL |  |
| 12. | Water Cooler (Medium Size) | 1 No | 1 No | NIL |  |
| 13. | First aid Box | 1 No. | 1 No. | NIL |  |

13.3.7		FLOOD FIGHTING MATERIAL AVAILABLE

Flood Fighting Material has already been purchased.

- 13.4	DETAIL OF OTHER INFRASTRUCTURES LIKE ELECTRIC, SUI GAS,    TELEPHONE INSTALLATIONS, ROAD NETWORKS, OTHER BUILDINGS, CANAL AND DRAINAGE WORKS.

Electric lines and Roads.

Executive Engineer 		       Chief Engineer 			     Project Director		       Jhelum Canal Division	 Irrigation Potohar Zone	              Small Dam Circle-II		                      	Jhelum			           Islamabad                                            Jhelum

ACTION PLAN

14.1   RE-SHUFFLING/ RECOUPING PLAN OF RESERVE STONE DEPARTMENTALLY.

Reserve stock of stone will be reshuffled through Govt. contractor in case of emergency.

14.2  	DETAIL OF INLET/ OUTLET CROSSING ALONG WITH CLOSING METHODOLOGY

NIL

14.3  	DEPLOYMENT MACHINERY (MEDIUM TO HIGH FLOOD)

| Sr.# | Name of Structure | Length in Mile | Vulnerable reach | Camp Location | Site incharge by Name & Cell # | Machinery Deployed |  |  |  | Labour Deployed | Availability of Stone Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/ Dumpers | Tractor with front blade | Beldar + Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Paharwal Guide Wall | 0.46 | 0 | Guide Wall Spur | Usman Ali Sub Engineer, Rajor 0300-7270475 | 1 | 0 | 2 | 1 | 1 | 0.20 |
| 2 | Kharka Guide Wall Jhelum | 0.29 | 0 | Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 1 | 0 |
| 3 | Gurah Jattan Guide Wall | 0.42 | 0 | Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 1 | 0 |
| 4 | New Paharwal Guide Wall | 0.396 | 0 | New Paharwal Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 1 | 0 |
| 5 | Khadirala Guide Wall | 0.436 | 0 | Khadirala Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 1 | 0.20 |
| 6 | Shah Madar Guide Wall | 0.369 | 0 | Guide Wall | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 0 | 1 | 1 | 1 | 0 |
| 7 | City Flood Bund | 4 | 0 | Near Railway Bridge | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 0 | 2 | 1 | 4 | 0.005 |
| 8 | Sara-I-Alamgir Flood Bund | 0.36 | 0 | Near Railway Bridge | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 0 | 1 | 1 | 1 | 0.185 |
| 9 | Gurah Ahmed Flood bund | 1.4 | RD 3-4 | RD 3-4 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 0 | 1 | 1 | 1 | 0.295 |
| 10 | Jhelum Cantt Flood bund | 0.5 | RD 12 | RD 12 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 0 | 1 | 1 | 1 | 0.036 |
|  | Total | 8.631 |  |  |  | 6 | 0 | 8 | 6 | 13 | 0.623 |

Note:- 	Payment of Machinery and work charge Establishment subject to verification by TPM Consultants.

Executive Engineer

Jhelum Canal Division

Jhelum

14.4   DEPLOYMENT MACHINERY (HIGH TO VERY HIGH FLOOD)

| Sr.# | Name of Structure | Length in Mile | Vulnerable reach | Camp Location | Site incharge by Name & Cell # | Machinery Deployed |  |  |  | Labour Deployed | Availability of Stone Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/ Dumpers | Tractor with front blade | Beldar + Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Paharwal Guide Wall | 0.46 | 0 | Guide Wall Spur | Usman Ali Sub Engineer, Rajor 0300-7270475 | 2 | 1 | 6 | 2 | 3 | 0.20 |
| 2 | Kharka Guide Wall Jhelum | 0.29 | 0 | Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 3 | 0 |
| 3 | Gurah Jattan Guide Wall | 0.42 | 0 | Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 3 | 0 |
| 4 | New Paharwal Guide Wall | 0.396 | 0 | New Paharwal Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 3 | 0 |
| 5 | Khadirala Guide Wall | 0.436 | 0 | Khadirala Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 3 | 0.20 |
| 6 | Shah Madar Guide Wall | 0.369 | 0 | Guide Wall | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 1 | 1 | 1 | 3 | 0 |
| 7 | Gurah Ahmed Flood bund | 1.4 | RD 3-4 | RD 3-4 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 1 | 2 | 1 | 3 | 0.005 |
| 8 | City Flood Bund | 4 | 0 | Near Railway Bridge | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 2 | 1 | 4 | 1 | 9 | 0.185 |
| 9 | Sara-I-Alamgir Flood Bund | 0.36 | 0 | Near Railway Bridge | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 1 | 2 | 1 | 3 | 0.295 |
| 10 | Jhelum Cantt Flood bund | 0.5 | RD 12 | RD 12 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 1 | 1 | 1 | 3 | 0.036 |
|  | Total | 8.631 |  |  |  | 8 | 6 | 16 | 7 | 36 | 0.623 |

Note:- 	Payment of Machinery and work charge Establishment subject to verification by TPM Consultants.

Executive Engineer

Jhelum  Canal Division

Jhelum

14.5   DEPLOYMENT MACHINERY (VERY HIGH TO EXCEPTIONALLY HIGH FLOOD)

| Sr.# | Name of Structure | Length in Mile | Vulnerable reach | Camp Location | Site incharge by Name & Cell # | Machinery Deployed |  |  |  | Labour Deployed | Availability of Stone Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/ Dumpers | Tractor with front blade | Beldar + Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Paharwal Guide Wall | 0.46 | 0 | Guide Wall Spur | Usman Ali Sub Engineer, Rajor 0300-7270475 | 3 | 1 | 8 | 3 | 4 | 0.20 |
| 2 | Kharka Guide Wall Jhelum | 0.29 | 0 | Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 4 | 0 |
| 3 | Gurah Jattan Guide Wall | 0.42 | 0 | Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 4 | 0 |
| 4 | New Paharwal Guide Wall | 0.396 | 0 | New Paharwal Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 4 | 0 |
| 5 | Khadirala Guide Wall | 0.436 | 0 | Khadirala Guide Wall | Usman Ali Sub Engineer, Rajor 0300-7270475 |  |  |  |  | 4 | 0.20 |
| 6 | Shah Madar Guide Wall | 0.369 | 0 | Guide Wall | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 1 | 3 | 1 | 4 | 0 |
| 7 | Gurah Ahmed Flood bund | 1.40 | RD 3-4 | RD 3-4 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 2 | 1 | 4 | 1 | 4 | 0.005 |
| 8 | City Flood Bund | 4.00 | 0 | Near Railway Bridge | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 3 | 1 | 9 | 2 | 12 | 0.185 |
| 9 | Sara-I-Alamgir Flood Bund | 0.36 | 0 | Near Railway Bridge | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 1 | 2 | 1 | 4 | 0.295 |
| 10 | Jhelum Cantt Flood bund | 0.50 | RD 12 | RD 12 | Shehzad Hassan Sub Engineer H/Q Jhelum 0345-5559274 | 1 | 1 | 3 | 1 | 4 | 0.036 |
|  | Total | 8.631 |  |  |  | 11 | 6 | 29 | 9 | 48 | 0.623 |

Note:- 	Payment of Machinery and work charge Establishment subject to verification by TPM Consultants.

Executive Engineer

Jhelum Canal Division

Jhelum

14.6		DEPLOYMENT PLAN OF POLICE.

| Sr. # | River Reach/ Embankment | Tehsil | Concerned Police Station |
| --- | --- | --- | --- |
| 1 | River Chenab Mangla Dam to G.T Road bridge Right side | Jhelum, Dina | Mangla, Jhelum Civil Line, Jhelum City |
| 2 | River Jhelum , Mangla Dam to G.T Road bridge left side | Mirpur, Bhaimber, Sarai Alamgir | Mangla, Afzalpur, Sarai Alamgir |
| 3 | G.T Road bridge to Rasul H/W right side | Jhelum PD Khan | Jhelum Sadar, Chotala |
| 4 | G.T Road bridge to Rasul H/W left side | Sarai Alamgir, M.B Din | Sarai Alamgir,M.B Din Sadar |
| 5 | Kahan River | Jhelum, Dina | Jhelum Sadar, Dina |
| 6 | Bunahan Nallah | Jhelum | Chotala |

14.7 DETAIL OF SYNTHIC BAGS WITH CAPACITY OF 50 KG AND 100 KG

| Sr. No. | Items | Required Qty | Available Qty | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Synthetic bags (50 KG Capacity) | 500 No. | --- |  |
| 2 | Synthetic bags (100 KG Capacity) | 300 No. | --- |  |

14.8 DETAIL OF POLYTHENE SHEETS OF BLACK COLOR TO PROTECT    U/S SLOPS AGAINST WAVE ACTION AND TO CONTROL SEEPAGE   THROUGH EMBANKMENT

| Sr. No. | Items | Required Qty. | Available Qty. | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Black Polythene Sheet | 15,000 Sft | Nil |  |

Executive Engineer

Jhelum Canal Division

Jhelum

CHAPER-15

BACK UP DIVISIONS (IN CASE OF BREACH)

Small Dam Division Jhelum and Chakwal Small Dams Division, Chakwal will back-up in case of emergency / Breach.

Executive Engineer

Jhelum Canal Division

Jhelum