CHAPTER-1

SALIENT FEATURES OF the

upper WESTERN BAR DIVISION  THINGI

1.1       LOCATION.

Upper Western Bar Division Thingi is situated in Tehsil and District Vehari. It is 21 K.M short of Vehari city on Multan – Vehari Road. The Division falls in two Districts whose split up of area is given below:-

| S# | DISTRICT. | G.A. | C.C.A | Sweet | Brackish. | %age of Brackish |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Vehari. | 410489 | 369912 | 351981 | 17931 | 05% |
| 2. | Sahiwal. | 321 | 321 | 321 | 0 | 0% |
|  | Total:- | 410810 | 370233 | 352302 | 17931 | 05% |

Upper Western Bar Division is one out of three Divisions of Nili Bar Circle at Sahiwal of Irrigation Zone Sahiwal It comprises of two Sub Divisions:-

- Joya Sub Division. (Non-perennial) Khadir Branch & its System.

- Sechanwala Sub Division (perennial) (Pakpattan Canal Upper & its System).

# IRRIGATION SYSTEM OF UPPER WESTERN BAR DIVISION.

- NON PERENNIAL SYSTEM

Khadir Branch RD 241+000 – RD  331+620 tail along with its System, off take at RD.112+000 Pakpattan Canal Upper at Ghmariwala Tehsil and District Pakpattan. Its full supply discharge at head is 2300 Cs. It falls in Suleimanki Division, Pakpattan Sub Division from   RD 0+000 – RD 241+000. From RD: 241+000 – RD 331+620 Tail, It falls in Upper Western Bar Division. Its designed discharge at RD: 241 is 1143 Cs. This is also indent point of Upper Western Bar Division. A meter flume at this site is   available. This Non perennial system is operated and maintained by Sub Divisional Officer Joya.

### PAKPATTAN CANAL (UPPER) PRENNIAL SYSTEM

Authorized Discharge of Pakpattan Canal (Upper) is 6594 Cs at its head and the indent point of Western Bar Division Upper is RD.407+000 while the jurisdiction of Upper Western Bar Division starts from R.D. 435+000. Its tails end is at R.D. 546+830. At indent point, the designed discharge of Pakpattan Canal Upper is 575 Cs. 3R Disty is 236 Cs which off takes at R.D.406+400/R of Pakpattan Canal Upper. Its total system is to be operated and maintained by Sub Divisional Officer Sechanwala.

LENGTH OF CHANNELS.

| Sr. No | KIND OF CANAL | LENGTH (IN MILE) |  |
| --- | --- | --- | --- |
|  |  | VEHARI | SAHIWAL |
| 1 | Main Canal | 22.37 | 0 |
| 2 | Branches | 18.12 | 0 |
| 3 | Disty | 199.65 | 0 |
| 4 | Minor. | 139.78 | 0 |
|  | Total:- | 379.92 | 0 |

LINED CHANNEL

| Sr. No | LENGTH (IN MILE) |  |  |
| --- | --- | --- | --- |
|  | VEHARI | SAHIWAL | TOTAL |
| A | LINED CHANNEL |  |  |
| 1 | 157.94 | 0 | 157.94 |
| B | EARTHEN CHANNEL |  |  |
| 2 | 221.98 | 0 | 221.98 |
| TOTAL | 379.92 | 0 | 379.92 |

FLOOD BUND.

There is One No. Flood Bund named Ghulam Wah Flood Embankment along Right bank of River Sutlej, situated on left bank of Ghulam Wah Disty from RD: 0+000 – RD 45+689. Total length of the bund is 9.14 Miles.

DISCHARGE.

| Sr. No | SYSTEM | DISCHARGE (IN CUSECS) |  |
| --- | --- | --- | --- |
|  |  | VEHARI | VEHARI |
| 1 | PC. Upper System. | 819 | 819 |
| 2 | Khadir Branch System | 1143 | 1143 |
| 3 | SMB Link Canal System | 46 | 46 |
|  | Total:- | 2008 | 2008 |

\

TOTAL INCOME OF DIVISION.                                      ABIANA + TAWAN

(FIGURE RS. IN MILLION)

| Sr. # | District | Rabi 2023-2024 |  |  | Kharif 2024 |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Abiana | Tawan | Total | Abiana | Tawan | Total |  |
| 1. | Vehari. | 61.123 | -- | 61.123 | 324.579 | -- | 324.579 | 385.702 |
|  | G.Total:- |  |  | 61.123 |  |  | 324.579 | 385.702 (M) |

1.2					GENERAL DESCRIPTION

Pakistan inherited on its Independence Day, two basic disputes Kashmir and the distribution of river waters. The Red Cliff boundary award was handed out in such a way that the main rivers which flow through Pakistan had been left their fountain heads in Indian control. Consequently, the Irrigation system of Bari Doab, Chaj Doab and the Indus valley remained dependent on the flow of water from the Indian- held territory. The size and frequency of a flood at any point depends on the volume and timing of runoff from upstream catchments. Physical characteristic of the catchments it self is also important in determining the size of flood which can result from a particular rainfall. These include geographical features such as catchment area and topography and hydrological features such as interception by vegetation, evaporation and infiltration. Artificial structures can also affect the nature of flooding.

Cloud bursts and monsoon changes to precipitate on the white tops of Himalaya through their torrential creeks inundating river resulting in the floods of 1976, 1988, 1992 and 1995 which caused havoc and devastations in major portions of Tehsil Burewala & Joya Sub Division. Vast tract of land were submerged in the flood and rain water also caused colossal damage to various crops, live stock, houses, socio-physical infrastructure and other properties.

In order to provide an effective relief to flood affected persons, it is essential to work out of a plan for this area which may cater for all aspects of a flood situation, so that we may be able to combine all our resources in a systematic manner for providing successful relief to flood effected peoples. Adequate and timely flood preventive measures can save the population from the onslaughts of water. This flood fighting plan has been evolved to effectively meet the flood situation and it would be put into operation immediately whenever the need arises. The year 2024 normal flood was received.                     12,578 Cs discharge was passed on 15/08/2024.

However, honest and zealous efforts are required from all those who are entrusted with the sacred duty of protecting and providing relief to flood affected people. The duty is indeed a moral obligation which is to be performed with missionary zeal.

Executive Engineer,                             Upper Western Bar Division Thingi.

CHAPTER-2

Flood Protection

AND River Training works

2.1			design PARAMETER OF TRAINING WORKS

| S.No | Name of flood bund | Length |  |
| --- | --- | --- | --- |
|  |  | RD | Miles |
| 1 | gHULAM wAH Flood EMBANKMENT | 45+689 | 9.14 |
|  | Total |  | 9.14 |

Upper Western Bar Division Thingi is affected by River Sutlej. The River Sutlej runs in its    East-South. During the flood season, the Katcha area along river is usually in the principal hit of the flood. Thus a constant vigilance and supervision is also necessary in this Division.

Flood warnings are received from Suleimanki Head Works specifying discharge in River Sutlej. This Division is responsible for safety of Canals System running along river and flood protection bunds. The details of which are as follow:-

Canal System.

The Ghulam Wah Disty & its systems run parallel to River Sutlej on its Right side. This system is protected from the spill of river by River training work i.e. Ghulam Wah Flood Embankment but following canal system requires vigilance and watching during flood.

| S.No | Canals | Reach |
| --- | --- | --- |
| 1 | Ghulam Wah Disty | RD 0+000-RD 45+689 |

Design Parameter.

A. Canal

| Canal | Design Parameter |  |  |  |
| --- | --- | --- | --- | --- |
|  | Length (mile) | Discharge (Cs) | Bed width (feet) | Depth (feet) |
| Ghulam Wah Disty | 9.14 | 361.50 | 42 | 4.30 |

B.	Flood Protection AND RIVER training works.

| S. No. | Type of Work | Structure | Design Parameters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Length (mile) | Top Width | Free board | Side slope |  |
|  |  |  |  |  |  | River | Country |
| A. | Flood Protection works | Ghulam Wah Flood Embankment | 9.14 | 25 | 5 | 3:1 | 2:1 |
|  |  | Total | 9.14 | -- | -- | -- | -- |

2.2   Location Map

attached  page. (10)

chapter-3

Brief History

of

Past Flood Events

Brief History OF PAST Flood Events.

Flood damages in this area have an old history which starts from 1955 when this area happened to come in the grip of unprecedented disaster, there was a discharge of 5.00 Lacs Cusecs of water in river Sutlej at Suleimanki during 1988 and 3.00 Lacs at Head Islam. In order to divert the flow, Pakpattan Canal was breached and washed away in upper reach RD 0+000 – RD 100+000, thus the pressure was reduced to some extent.

Similarly in 1995, the River Sutlej with its discharge of 3.00 lac, Cusec at Suleimanki Head Works badly affected villages in District Okara, Pakpattan & Vehari and caused heavy damage to human & animal life, crops and public property.

During year 2024, Normal flood was received in River Sutlej at Suleimanki Head Works and flood water no touch Ghulam Wah Flood Embankment from RD 0+000 to RD 45+689. No major infrastructure

was                   damaged / collapsed but river side slope of Ghulamwah Flood Embankment was partially damaged.

chapter-4

design data, historic peak flood data and previous five years flood data of head works / barrage and or other control points.

  - Flood Limits.

(discharge of river sutlej at suleimanki)

| Point | Normal (Cs) | Low Flood (Cs) | Medium Flood (Cs) | High Flood (Cs) | Very high Flood (Cs) | Exceptionally High Flood (Cs) |
| --- | --- | --- | --- | --- | --- | --- |
| Suleimanki to Ghulam Wah Flood Embankment | 000 to 50,000 | 50,000 to 80,000 | 80,000 to 1,20,000 | 1,20,000 to 1,75,000 | 1,75,000 to 3,00,000 | Above 3,00,000 |

4.2		time lagS of floodS.

| Location |  | Distance in Miles | Time Lag ( Hours) |
| --- | --- | --- | --- |
| From | To |  |  |
| Suleimanki | Ghulam Wah Flood Bund | 121 Canal Miles | 72/96 Hours |

Note: - 	Time lags vary due to artificial poundage resulting from Zimindars Cross Bund in between Suleimanki Head Work and Islam Head Works.

4.3	HIGHEST FLOODS.

| Sr. No. | Year | Peak Discharge | Classification |
| --- | --- | --- | --- |
| 1 | 1988 | 5,00,000 Cs | Highest Flood |
| 2 | 1995 | 3,02,000 Cs | Highest Flood |
| 3 | 2023 | 1,51,904 Cs | High Flood |
| 4 | 2024 | 12,578 Cs | Normal Flood |

4.4		PEAK DISCHARGES.

| Sr. No | Year | Peak Discharge | Classification |
| --- | --- | --- | --- |
| 01 | 1988 | 5,00,000 Cs | Exceptionally High Flood |
| 02 | 1995 | 3,02,000 Cs | Exceptionally High Flood |
| 03 | 1996 | 84,000 Cs | Medium Flood |
| 04 | 1997 | 44,800 Cs | Normal |
| 05 | 1998 | 91,000 Cs | Medium Flood |
| 06 | 1999 | 23,000 Cs | Normal |
| 07 | 2000 | 000 Cs | No Flood |
| 08 | 2001 | 000 Cs | No Flood |
| 09 | 2002 | 000 Cs | No Flood |
| 10 | 2003 | 000 Cs | No Flood |
| 11 | 2004 | 000 Cs | No Flood |
| 12 | 2005 | 000 Cs | No Flood |
| 13 | 2006 | 000 Cs | No Flood |
| 14 | 2007 | 000 Cs | No Flood |
| 15 | 2008 | 1,02,739 Cs | Medium Flood. |
| 16 | 2009 | 000 Cs | No Flood |
| 17 | 2010 | 30,000 Cs | Normal |
| 18 | 2011 | 70,125 Cs | Medium Flood |
| 19 | 2012 | 21,457 Cs | Normal |
| 20 | 2013 | 78,846 Cs | Low Flood |
| 21 | 2014 | 26,103 Cs | Normal |
| 22 | 2015 | 49,582 Cs | Normal |
| 23 | 2016 | 22,953 Cs | Normal |
| 24 | 2017 | 20,893 Cs | Normal |
| 25 | 2018 | 34722 Cs | Normal |
| 26 | 2019 | 66459 Cs | Low Flood |
| 27 | 2020 | 11136 Cs | Normal |
| 28 | 2021 | 7379 Cs | No Flood |
| 29 | 2022 | 17876 Cs | No Flood |
| 30 | 2023 | 151904 Cs | High Flood |
| 31 | 2024 | 12578 Cs | Normal |

Executive Engineer,

Upper Western Bar Division,

Thingi.

chapter-5

Flood fighting strategy

FLOOD FIGHTING STRATEGY

Following flood fighting strategy will be adopted at Joya Sub Division Joya.

STAGE-1

DISCHARGE  FROM 0.50 LAC CS to  0.80 LAC. CS (LOW FLOOD).

When river discharge ranges between these limits, watching establishment shall be deployed at the rate of one man per thousand feet.  Sub Engineer, of Joya section will supervise the entire watching operation of the Ghulam Wah Flood Embankment Right bank of river Sutlej Sub Divisional Officer Joya will act as the officer incharge of watching operation.

STAGE-  II

DISCHARGE  FROM 0.80 LAC Cs. TO  1.20 LAC Cs. (MEDIUM FLOOD).

When the discharge ranges between these limits, watching establishment will be increased to two times of that provided in stage-I. Supervisory staff shall be the same as in stage I.

STAGE- III

DISCHARGE FROM 1.20 LAC Cs. TO 1.75 LAC Cs. (HIGH FLOOD).

Watching establishment shall be increased to three times than that of stage-I. All establishment of Joya Sub Division and equal work charge establishment shall be engaged at site. Additionally, Sub Engineer, Ratta Tibba Section will shift his headquarter at site.

Sub Divisional Officer Joya Sub Division will be the officer in-charge of watching operation. Overall supervision of all operations will be done by the Executive Engineer western Bar Division.  The camp at Joya canal Rest House during flood days.

STAGE -IV

DISCHARGE FROM 1.75 LAC Cs. TO 2.25 LAC Cs. ( VERY HIGH FLOOD).

Watching establishment shall be increased to four times than that of stage-I. All establishment of Joya Sub Division and Sechanwala Sub Division and equal work charge establishment shall be engaged at site.

Sub Divisional Officer Joya Sub Division and Sechanwala Sub Divisions will be the officer                    in-charge of watching operation. Overall supervision of all operations will be done by the Executive Engineer Upper Western Bar Division. The camp at Joya canal Rest House during flood days.

STAGE- V

DISCHARGE  EXCEEDING  2.25 LAC Cs.

(EXCEPTIONALLY HIGH FLOOD).

At this stage, volunteer labors brought by civil administration and Army will help the watching operation. Police guards will be deputed on vulnerable points to guard against any mishap/sabotage. Overall supervision of all operations will be done by Executive Engineer Upper Western Bar Division The camp at Joya canal Rest House during flood days.

Battle station

The Department proposed to install battle station for protection of irrigation canals and river training works.

A.	Canals

| Canal | Reach | Length | Battle station |
| --- | --- | --- | --- |
| Ghulam Wah Disty | RD 0+000-RD 45+689 | 9.14 | At RD 10+000 and 35+000. |

Canal System.

The Ghulam Wah Disty & its system run parallel to River Sutlej on its Right side. This system is protected from the spill of river by large network of River training works but the following canals of this system require vigilance and watching during flood.

B.     Flood protection & River Training works.

| Structure | Reach | Length in mile | Battle station | Incharge Batttle Station |
| --- | --- | --- | --- | --- |
| Ghulam Wah flood Embankment | RD 0+000 - RD 20+000 | 5.0 | At RD 10+000 | Sub Divisional Officer Joya with 3 Sub Engineers. |
| Ghulam Wah flood Embankment | RD 20+000 - RD 45+986. | 4.14 | At RD 35+000 | Sub Divisional Officer Sechanwala with 4 Sub Engineers. |

- The Sub Divisional officer Joya Sub Division will be over all in charge of all operation on river training works in his jurisdiction.

- The battle station will be installed for controlling the protection activities for controlling at all spots, sufficient articles of stock item will be available in all these camps.

- There will be the provision for engagement of work charge establishment to supplement the efforts through regular establishments.

- All S.D.O’s will remain in touch with flood control officer at Divisional Office Upper Western Bar Division Thingi during flood period connected with Phone Nos. 067-3788115.

- The Executive Engineer Upper Western Bar Division Thingi is fully competent to assign duty to any officer/officials.

The Executive Engineer Upper Western Bar Division, Thingi will be over all incharge and will stay at Joya Rest House during flood days. He will take appropriate action according to the severances of the emergency.

chapter-6

Flood damages restoration works

Flood damages restoration works.

No flood   damage work is in hand.  The following 01 No work has been executed under A.D.P. during 2017-18.

| S.No | Name of work | Cost (Rs. In Million) |
| --- | --- | --- |
| 1. | Remodeling/Restoration of Ghulam Wah Flood Embankment from RD. 3+240 - RD. 45+689. | 131.551 |
|  | Total | 131.551 |

Note:- 100% work has been completed

chapter-7

Flood fighting

watching arrangement

7.1		PRE-FLOOD ARRANGEMENT.

Pre flood inspection has been carried out of Ghulam Wah Flood Bund. RD Marks, Data Boards and Flood Gauges have been repainted. (No reserve stock stone.)

7.2		watching establishment.

| Site | Flood Limits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low |  | Medium |  | High |  | Very High |  | Exceptionally high |  |
|  | Men | Shift | Men | Shift | Men | Shift | Men | Shift | Men | Shift |
| Ghulam Wah flood Embankment. | 20 | 1 | 30 | 2 | 40 | 3 | 50 | 3 | 60 | 3 |

Note:

- The Executive Engineer Upper Western Bar Division Thingi may employ work charge establishment as per site situation after obtaining approval form Superintending Engineer Nili Bar Circle Sahiwal.

- S.D.O Joya will employ watching establishment as per strength mentioned above subjected to approval form competent authority.

- In case of very high flood, the strength of establishment may be increased with express permission of Executive Engineer.

7.3		ARRNAGEMENT AT SENSITIVE SITES.

Reach RD.10+000-RD.12+000 of Ghulam Wah Flood Embankment is noted sensitive sites in case of exceptionally high flood in river Sutlej &  in this reach all arrangement i.e. camp site ,watching arrangements /machinery and  tool &  plan are already planed.

There are no reserve stock of stone is sanction at Ghulam Wah Flood Embankment

7.4		WATCHING MATERIAL.

(Quantity required at Camp No.1 at RD 10+000 and Camp No.2 at RD 35+000).

| Sr. No | Item | Unit | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Kassies | No | 30 | 10 | 20 | The Balance quantity of required material will be purchased before flood 2025 on receipt of funds, the then taken in stock/ T&P register. |
| 2 | Buckets | No | 47 | 10 | 37 |  |
| 3 | Handle for Kassises | No | 150 | 10 | 140 |  |
| 4 | Lanterns | No | 55 | 10 | 45 |  |
| 5 | Spare chimney for lanterns | No | 25 | 10 | 15 |  |
| 6 | Torches | No | 30 | 10 | 20 |  |
| 7 | Mulberry Basket | No | 90 | 10 | 80 |  |
| 8 | Hammers with Handle | No | 50 | 10 | 40 |  |
| 9 | Wick | Ft | 50 | 10 | 40 |  |
| 10 | Match Boxes | Gurse | 05 | 05 | 0 |  |
| 11 | Gunny Bags | No | 1370 | 59 | 1311 |  |
| 12 | Sotli | Kg. | 02 | 02 | 0 |  |
| 13 | Chouldaries | No. | 04 | 0 | 4 |  |
| 14 | Synthetic bags capacity 500 kg | No. | 500 | 75 | 425 |  |
| 15 | Synthetic bags capacity 1000 kg | No. | 350 | 35 | 315 |  |
| 16 | Polythene sheets of black color | Sft | 685335 OR 2741 Kg | 0 | 685335 OR 2741 Kg |  |

  - ARRNAGEMENT  FOR SOUNDING &   PROBING.

Not related.

  - LIGHTING ARRNAGEMENT.

Lightening material is readily available

  - RATION ARRANGEMENT

Ration will be provided as per requirement by Sub Divisional Officer Joya.

  - P.O.L ARRANGEMETN FOR VEHICLE.

P.O.L will be provided as per requirement by Sub Divisional Officer Joya.

  - TRANSPORTATION

Transportation will be provided as per requirement by the Civil Administration.

  - LAW AND ORDER

The Law and order situation will be controlled with the help of Civil Administration.

  - MEDICAL ARRANGEMENT FOR LABOUR

Medical facilities will be provided by the Health Department.

  - LIAISON WITH OTHER DEPARTMENT.

Liaison and coordination with all other concerned Departments will be made through PTCL and mobile connection.

  - Role of THE army.

The help of Pak Army will be requested by the

Civil Department as per flood situation.

7.14	DUTIES OF TELEPHONE ATTENDANT

The duty Roaster has been prepared as under.

| Sr No. | Name of official | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Muhammad Temoor Akram Camp Clerk & Zahid Iqbal Naib Qasid | 8 Hours | 0900 | 1600 |
| 2 | Ayub Shah Signaller & Rasool Bakhsh Chowkidar | 8 Hours | 1600 | 2400 |
| 3 | Muhammad Arshad Zaki Signaller & Muhammad Ramzan Naib Qasid. | 8 Hours | 0100 | 0900 |

  - WIRELESS  ARRANGEMENTS

During flood season wireless set installed by the

Police Department in Vehicle.

chapter-8

Detail of encroachment

DETAIL OF ENCROACHMENT.

- Encroachment

| Sr No. | Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| 1 | NiL | NiL | NiL |  |

B.          Critical Encroachment

| Sr No. | Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| 1 | NiL | NiL | NiL |  |

chapter-9

Duty roster /

flood fighting program.

Duty roster FLOOD FIGHTING PROGRAM.

The flood area under the control of this Division is divided into two sectors. Each sector is assigned to one Sub Divisional officer who will shift his head quarter in field. Detail of sectors given below:-

| Sector | Officer incharge | Staff | Reach to be controled |
| --- | --- | --- | --- |
| 1 | At RD 10+000 of Ghulam Wah Flood Embankment. | Muhammad Arshad Nadeem Sub Divisional Officer Joya 0300-6870599 | i) Asghar Ali SBE Joya Section. 0300-8699231 ii) Muhammad Ali Akbar SBE Rta. Section. 0304-6988421 iii) Tahir Magsi SBE Sgn. Section. 0306-0174727 |
| 2 | At RD 35+000 of Ghulam Wah Flood Embankment . | Muhammad Arshad Nadeem Sub Divisional Officer Sechanwala 0300-6870599 | i) Muhammad Jafar SBE Scw Section. 0313-7240329 ii) Rao Muhammad Abid SBE Mcw Section. 0302-5033182 iii) Abdul Rehman SBE Pakhi Section. 0300-7726762 iv) Amir Sohail SBE Mari Section 0315-8703073 |

The Sub Divisional officer Joya Sub Division, Sub Divisional Officer Sechanwala Sub Division, Sub Engineer Joya Section, Sub Engineer Rata Tibba Section, Sub Engineer Sargana Section, Sub Engineer Sechanwala Section, Sub Engineer Machianwala Section, Sub Engineer Mian Pakhi Section and Sub Engineer Mari Section will be the representative of Department in the field and are entrusted with the job of supervision of flood works. The Executive Engineer Upper Western Bar Division Thingi will be responsible to implement the instructions received from high authorities for flood protection measures during flood. He will give the information form telegraph office Thingi and by police wireless at Thingi to following officers stationed at Vehari, Sahiwal.

- Superintending Engineer Nili Bar Circle Sahiwal.

- Deputy Commissioner Vehari.

- Flood emergency officer, office of the Chief Engineer Irrigation Sahiwal Zone Sahiwal

OFFICE OF THE

EXECUTIVE ENGINEER,

UPPER WESTERN BAR DIVISION

THINGI.

OFFICE ORDER.

A Divisional Flood Cell has been constituted which will start functioning in the office of Executive Engineer, Upper Western Bar Division Thingi with effect from 15-06-2025 under the overall control and supervision of Muhammad Arshad Nadeem  Sub Divisional Officer Joya, who is hereby nominated as Flood Emergency liaison Officer for flood season 2025. The contact phone Nos. are as under: -

Phone No. 067-3788115. Fax No. 067-3788115. Flood Emergency Officer                     No. 0300-6870599

The flood cell will perform the duty 24 Hours and following Camp Clerk / Signalers will remain present round the clock.

Shift No: - 1	0900 to 1600 Hours.

- Temoor Akram Camp Clerk.

- Zahid Iqbal  Naib Qasid.

Shift No: - 2	1600 to 2400 Hours.

- Muhammad Ayub Shah Signaller.

- Rasool Bakhsh Chowkidar.

Shift No. 3: -	0100 to 0900 Hours.

- Muhammad Arshad Zaki Signaller.

- Muhammad Ramzan Naib Qasid.

Note: - The Shift of the Officials will be changed on every week turn by turn.

Executive Engineer,

Upper Western Bar Division,

Thingi.

CHAPTER-10

Emergency Telephone NOs

Emergency Telephone Numbers

A.     irrigation department.

| S. No | Designation | Office | Telephone |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Office | Residence | Mobile |
| 1 | Secretary Irrigation | Lahore | 042-99212117 042-99212118 | 042-99200954 |  |
| 2 | Deputy Secretary Operation | Lahore | 042-99112127 |  |  |
| 3 | Director Flood | Lahore | 042-99233552 |  |  |
| 4 | Flood Warning Centre | Lahore | 042-99205157 | 042-99205158 |  |
| 5 | Chief Monitoring PMIU | Lahore | 042-99213595 042-99213597 |  | Toll Free No. 0800-11333 |
| 6 | Chief Engineer | Sahiwal Zone | 040-9200221 |  | 0300-4810150 |
| 7 | Superintending Engineer | Nili Bar Circle Sahiwal | 040-9200217 | 040-9200216 | 0304-7888143 |
| 8 | Executive Engineer | Upper Western Bar Division Thingi | 067-3788115 | 067-3788114 | 0300-7687686 |
| 9 | Flood Emergency Officer | Executive Engineer (OP) Sahiwal | 040-9200225 |  |  |
| 10 | Chief Engineer | D&F Lahore | 042-35291304 | 042-99233551 | 0303-7713303 |
| 11 | Sub Divisional Officer | Joya Sub Division |  |  | 0300-6870599 |
| 12 | Sub Divisional Officer | Sechanwala Sub Division | 067-3780113 |  | 0300-6870599 |
| 13 | Sub Engineer | Joya Section |  |  | 0300-8699231 |
| 14 | Sub Engineer | Rata Tibba Section |  |  | 0304-6988421 |
| 15 | Sub Engineer | Sargana Section |  |  | 0306-0174727 |
| 16 | Sub Engineer | Sechanwala Section |  |  | 0313-7240329 |
| 17 | Sub Engineer | Machianwala Section |  |  | 0302-5033182 |
| 18 | Sub Engineer | Mian Pakhi Section |  |  | 0300-7726762 |
| 19 | Sub Engineer | Mari Section |  |  | 0315-8703073 |

B.        Civil Administration

| S.No | Designation | Telephone |  |  |
| --- | --- | --- | --- | --- |
|  |  | Office | Residence | Mobile |
| 1 | D.C. Vehari | 067-3362122 067-3363488 | 067-3363477 067-3363478 |  |
| 2 | District Officer Revenue | 067-3364063 | 067-3360347 |  |
| 3 | Assistant Commissioner Vehari | 067-3361250 |  |  |
| 4 | Assistant Commissioner Burewala | 067-9200108 |  |  |
| 5 | D.P.O Vehari. | 067-3363246 | 067-3363246 |  |
| 6 | D.H.O Vehari | 067-3362195 | 067-3362195 |  |
| 7 | District Officer Agriculture Vehari | 067-3362603 | 067-3363570 |  |

chapter-11

standard operating procedure (sop)

for breaching sectionS

11.1		HISTORY OF THE BREACHING SECTION

Not related.

11.2	LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION.

Not related.

11.3			ARRANGEMENT OF EXPLOSIVES AND SECURITY OF EXPLOSIVE STORES.

Not related.

11.4			LIST OF SECURITY STAFF ALONGWITH DETAIL OF THEIR TRAINING ETC.

Not related.

11.5			DETAIL OF MECHANICAL MEANS AS A STAND BY ARRANGEMENTS IN CASE OF DETONATION FAILURE.

Not related.

11.6	     	DUTY ROSTER IN CASE OF CRITICAL SITUATION.

Not related.

11.7	    	BREACHING COMMITTEE WITH THEIR ACTION PLAN.

Not related.

11.8		LIST OF THE VILLAGES  LIKELY TO BE INUNDATED IN CASE OF BREACH.

Not related.

11.9		ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENTS.

Not related.

11.10		DETAILS OF COORDINATION WITH CIVIL / ARMY AUTHORITIES.

Not related.

11.11	PARALLEL COMMUNICATION ARRANGEMENTS.

Not related.

  - INDEX PLAN

Attached

Executive Engineer,		    Superintending Engineer,	   Chief  Engineer,      Upper Western Bar Division,	    Nili Bar Circle,                            Sahiwal Irrigation Zone,

Thingi.			              Sahiwal.			             Sahiwal.

chapter-12

vulnerable sites on

flood bund / structures

  - APPREHENDED BREACHES IN FLOOD BUNDS / STRUCTURES.

RD. 10+000 – 12+000 of Ghulam Wah Flood Embankment is noted as critical in case of exceptionally high flood in River Sutlej.

  - OPERATION OF BREACHING SECTION.

Not applicable.

  - BREACHES DUE TO RISING OF FLOOD WATER DETERIORATION OF FLOOD BUND ETC.

No Such Case.

chapter-13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE (SITES) LISTED ABOVE, EMERGENCY CONTIGENCY PLAN MUST INCLUDE.

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF

THE BREACH SUPPORTED WITH LEVELS.

RD. 10+000 – 12+000 of Ghulam Wah Flood Embankment is noted as critical in case of exceptionally high flood in River Sutlej.

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

| S.No | Name of Villages | Name of River |
| --- | --- | --- |
| 1 | 42/KB | Sutlej |
| 2 | 44/KB | Sutlej |
| 3 | 45/KB | Sutlej |
| 4 | 46/KB | Sutlej |
| 5 | 50/KB | Sutlej |
| 6 | 51/KB | Sutlej |
| 7 | 52/KB | Sutlej |
| 8 | Mal pur | Sutlej |
| 9 | Khokhra | Sutlej |
| 10 | Muhammad Shah | Sutlej |
| 11 | Mani Shah | Sutlej |
| 12 | Kot Qadir Bakhsh | Sutlej |
| 13 | Rahim Shah | Sutlej |
| 14 | Saldera | Sutlej |

  - STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL.

THIS MAY INCLUDE.

In case of exceptionally high flood expected in River Sutlej, District Administration Vehari will be informed by Irrigation Department for evacuation from the area lying in pocket (Ghulam Wah Disty, Khadir Branch and P.I. Link) the accumulated water will be ultimately diverted towards River Sutlej.

13.3.1	 	ARRANGEMENTS.

Flood fighting material has been purchased and watching will be applied as per flood situation, As explained in chapter No. 7.

    - ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

The Camp of S.D.O Joya Sub Division and his team will be at RD. 10+000 of Ghulam Wah Flood Embankment and SDO Sechanwala Sub Division and his team will be at RD. 35+000 Ghulam Wah Flood Embankment. They will employ watching establishment as per requirement. In case of very high flood, the strength of establishment may be increased with express permission of Executive Engineer.

    - DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

| Camp No. | Location | officer Name and Designation (Mobile No) | official Name and Designation (Mobile No) |
| --- | --- | --- | --- |
| 1 | At RD 10+000 of Ghulam Wah Flood Embankment. | Muhammad Arshad Nadeem Sub Divisional Officer Joya 0300-6870599 | i) Ali Asghar SBE Joya Section. 0300-8699231 ii) Ali Akbar SBE Rta. Section. 0304-6988421 iii) Tahir Magsi SBE Sgn. Section. 0306-0174727 |
| 2 | At RD 35+000 of Ghulam Wah Flood Embankment . | Muhammad Arshad Nadeem Sub Divisional Officer Sechanwala 0300-6870599 | i) Muhammad Jafar SBE Scw Section. 0313-7240329 ii) Rao Muhammad Abid SBE Mcw Section. 0302-5033182 iii) Abdul Rehman SBE Pakhi Section. 0300-7726762 iv) Amir Sohail SBE Mari Section 0315-8703073 |

The Sub Divisional officer Joya Sub Division, Sub Divisional officer Sechanwala Sub Division, Sub Engineer Joya Section , Sub Engineer Ratta Tibba Section, Sub Engineer Sargana Section , Sub Engineer Sechanwala Section , Sub Engineer Machianwala Section , Sub Engineer Mian Pakhi Section and Sub Engineer Mari Section will be the representative of Department in the field and are instructed with the job of supervision of flood works. The Executive Engineer Upper Western Bar Division  Thingi will be responsible to implement the instructions received from high authorities for flood protection measures during flood. He will give the information from telegraph office Thingi and by police wireless at Thingi to following officers stationed at Vehari and Sahiwal.

- Superintending Engineer Nili Bar Circle Sahiwal.

- District Coordination officer Vehari.

- Flood emergency Officer, Office of the Chief Engineer Irrigation Sahiwal Zone, Sahiwal.

13.3.4	          DEPARTMENTAL MACHINERY AVAILABLE.

No such machinery is available in Upper Western Bar Division, Thingi.

13.3.5	MACHINERY AVAILABLE FROM PRIVATE SOURCE.

Machinery will be hired by the private source as per requirement, if required.

13.3.6.           FLOOD FIGHTING MATERIAL REQUIRED.

13.3.7.	FLOOD FIGHTING MATERIAL AVAILABLE.

| Sr. No | Item | Unit | Total Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Kassies | No | 30 | 10 | 20 | The Balance quantity of required material will be purchased before flood 2025 on receipt of funds, the then taken in stock/ T&P register. |
| 2 | Buckets | No | 47 | 10 | 37 |  |
| 3 | Handle for Kassises | No | 150 | 10 | 140 |  |
| 4 | Lanterns | No | 55 | 10 | 45 |  |
| 5 | Spare chimney for lanterns | No | 25 | 10 | 15 |  |
| 6 | Torches | No | 30 | 10 | 20 |  |
| 7 | Mulberry Basket | No | 90 | 10 | 80 |  |
| 8 | Hammers with Handle | No | 50 | 10 | 40 |  |
| 9 | Wick | Ft | 50 | 10 | 40 |  |
| 10 | Match Boxes | Gurse | 05 | 05 | 0 |  |
| 11 | Gunny Bags | No | 1370 | 59 | 1311 |  |
| 12 | Sotli | Kg. | 02 | 02 | 0 |  |
| 13 | Chouldaries | No. | 04 | 0 | 4 |  |
| 14 | Synthetic bags capacity 500 kg | No. | 500 | 75 | 425 |  |
| 15 | Synthetic bags capacity 1000 kg | No. | 350 | 35 | 315 |  |
| 16 | Polythene sheets of black color | Sft | 685335 | 0 | 685335 | (OR) 2741= Kg |

13.4.	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDINGS, CANALS AND DRAINAGE NETWORK.

- Irrigation network i.e. Koura Disty, Saldera Minor, Joya Disty, Tajwana Minor,    Lal Deh Minor, Hussain Wah Minor and Karim Wah Minor.

- Irrigation Rest Houses i.e. Joya and Lakha Rest Houses.

- Burewala to Ludden Road and other village link Roads etc.

- Schools and basic health units.

- Electric installation etc.

Executive Engineer,

Upper Western Bar Division,

Thingi.

chapter-14

ACTION PLAN.

14.1       RESHUFFLING/RECOUPING PLAN OF RESERVE  STONE

DEPARTMENTALLY.

| Sr. No. | Name of Structure | Sanctioned Limit of Reserve Stock of Stone (Cft) | Stone available at site (Cft) | Balance stone required (Cft) |
| --- | --- | --- | --- | --- |
| 1 | Ghulam Wah Flood Embankment from RD 0+000-RD 45+689. | Nil | Nil | Nil |

Executive Engineer,

Upper Western Bar Division,		Thingi

14.2     DETAIL OF INTLET AND OUTLET CROSSING ALONG WITH CLOSING

METHODOLOGY.

| Sr. No. | Name of Bund | Spur | Name of Structure with location | Location of Crossing | Problem | Bottlenecks occurred during flood 2022 | Remedial measures proposals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ghulam Wah Flood Embankment | - | Ghulam Wah Disty RD 22505/L | RD 22-23 | - | - | The barrel structure should be leak proof. |
| 2 |  |  | Ghulam Wah Disty RD 29519/L | RD 29-30 | - | - |  |
| 3 |  |  | Ghulam Wah Disty RD 32355/L | RD 32-33 | - | - |  |
| 4 |  |  | Ghulam Wah Disty RD 37500/L | RD 37-38 | - | - |  |
| 5 |  |  | Ghulam Wah Disty RD 41760/L | RD 41-42 | - | - |  |

METHODOLOGY.

On receipt of flood warning each outlet throat will be closed by Pacca masonry work and ring bund will be made around the structure with earth fill jute bags or any other alternating method which consider to be corrected and certificate will be taken from Sub Engineer / Sub Divisional Officer concerned.

Executive Engineer,

Upper Western Bar Division,		Thingi

14.3      DEPLOYMENT MACHINERY ( MEDIUM TO HIGH FLOOD).

| Sr. No. | Name of Structure | Length in mile | Vulnerable Reach | Camp Location | Site Incharge By Name & Cell No. | Machinery Deployed |  |  |  |  | Availability of stone Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys /Dumpers | Tractor with front blade | Labour Beldar+Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Ghulam Wah Flood Embankment RD 0+000-RD 45+689. | 9.14 | RD 10+000-RD 12+000 | RD 10+000 & RD 35+000 | Sub Engineer Joya Section Asghar Ali (0300-8699231 ) | - | - | - | 1 | 15+5 | - |
|  | Total | 9.14 |  |  |  | - | - | - | 1 | 15+5 | - |

Executive Engineer,

Upper Western Bar Division,		Thingi

14.4     DEPLOYMENT MACHINERY (HIGH TO VERY HIGH FLOOD ).

| Sr. No. | Name of Structure | Length in mile | Vulnerable Reach | Camp Location | Site Incharge By Name & Cell No. | Machinery Deployed |  |  |  |  | Availability of stone Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys /Dumpers | Tractor with front blade | Labour Beldar+Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Ghulam Wah Flood Embankment RD 0+000-RD 45+689. | 9.14 | RD 10+000-RD 12+000 | RD 10+000 & RD 35+000 | Sub Engineer Joya Section Asghar Ali (0300-8699231 ) | 1 | 2 | 2 | 1 | 20+5 | - |
|  | Total | 9.14 |  |  |  | 1 | 2 | 2 | 1 | 20+5 | - |

Executive Engineer,

Upper Western Bar Division,		Thingi

14.5    DEPLOYMENT MACHINERY (HIGH TO EXCEPTIONALLY HIGH FLOOD).

| Sr. No. | Name of Structure | Length in mile | Vulnerable Reach | Camp Location | Site Incharge By Name & Cell No | Machinery Deployed |  |  |  |  | Availability of stone Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys /Dumpers | Tractor with front blade | Labour Beldar+Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Ghulam Wah Flood Embankment RD 0+000-RD 45+689. | 9.14 | RD 10+000-RD 12+000 | RD 10+000 & RD 35+000 | Sub Engineer Joya Section Asghar Ali (0300-8699231 ) | 1 | 2 | 4 | 2 | 25+5 | - |
|  | Total | 9.14 |  |  |  | 1 | 2 | 4 | 2 | 25+5 | - |

Note: - Payment for Machinery and Work Charge Employs subject to verification by M/S NES PAK / Departmental Committee.

Executive Engineer,

Upper Western Bar Division,		Thingi

14.6      POLICE DEPLOYMENT PLAN

| Sr. No. | Vulnerable sites used for illegal cuts during floods | Concerned Canal Division | Concerned Police Station & District | Police personnel to be deployed |  | Remarks (If any) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Inspector/ SI/ASI | Constables |  |
| 1 | RD:10+000-RD 12+000 Ghulam Wah Flood Embankment | Western Bar Division | Sahuka, Fateh Shah & Ludden District Vehari | ASI 1 No. | 4 No. | In case of high flood in River Sutlej |

14.7   DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG AND 1000 KG.

| ITEM | UNIT | REQUIRED QUANTITY | AVAILABLE | BALANCE | REMARKS |
| --- | --- | --- | --- | --- | --- |
| Synthetic bags capacity 500 kg | No. | 500 | 75 | 425 | The Balance quantity of required material will be purchased before flood 2025 on receipt of funds. |
| Synthetic bags capacity 1000 kg | No. | 350 | 35 | 315 |  |

  - DETAILS OF POLYTHENE SHEET OF BLACK COLOUR TO PROTECT UP STREAM   SLOPE AGAINST WAVE ACTION AND TO CONTROL SEEPAGE THROUGH   EMBANKMENTS.

| ITEM | UNIT | REQUIRED QUANTITY | AVAILABLE | BALANCE | REMARKS |
| --- | --- | --- | --- | --- | --- |
| Polythene sheets of black color | Sft | 685335 OR = 2741 Kg | 0 | 685335 OR = 2741 Kg | The Balance quantity of required material will be purchased before flood 2025 on receipt of funds. |

Executive Engineer,

Upper Western Bar Division,		Thingi

chapter-15

BACKUP DIVISIONS

In case of breach Eastern Bar Division of Nili Bar Circle Sahiwal

Will work as backup of this Division

Executive Engineer,		  Superintending Engineer,	Chief Engineer,

Upper Western Bar Division,        Nili Bar Circle, 			Sahiwal Irrigation Zone,

Thingi.			   	  Sahiwal.				Sahiwal.