CHAPTER -1

SALIENT FEATURE OF THE SHUJABAD CANAL DIVISION MULTAN.

  - LOCATION

The Divisional Office Shujabad Canal Division is Located at Abdali Road Opposite to Parvaiz Ilahi Cardiology Centre Multan. The jurisdiction of Shujabad Canal Division starts from Tehsil Kabirwala District Khanewal and runs through Tehsil Multan, Shujabad & extends upto Tehsil Jalalpur Pir Wala of District Multan.

  - GENERAL DESCRIPTION

Shujabad Canal Division covers the area of Tehsil Shujabad, part of   Tehsil Jalalpur Pirwala, Tehsil Multan of District Multan and Tehsil Kabirwala of District Khanewal Shujabad Canal Division, Multan also has a Minor part of Muzaffargarh Tehsil of Muzaffargarh District, lies within the jurisdiction of this Division. These areas lie on the left bank of River Chenab D/S confluence point of River Ravi in a narrow belt to confluence point of Sutlej and Chenab River within a width varying from two to ten miles and covering a length of about 90 miles. The floods in River Chenab therefore threaten extensive damage to life and property. The existing flood bunds system which was developed gradually over the last half century has been converted in to a full-fledged network of first and 2nd defence lines after flood of 1973. All the bunds in the first line of defence were originally designed for flood heights which now correspond to medium flood discharges of the order of 4.0 lac cusec. These bunds are very close to the river as explained in the succeeding paragraphs and are prone to mishaps during high floods and cannot be expected to bear the strain of exceptionally high floods. Exceptionally high floods of 1973 and 1976 caused colossal damages to these bunds. The further status of these bunds was discussed in the 50th meeting of Punjab Flood Commission on 19.12.1973 and it was decided not to raise or strengthen these bunds but repair them to their original designed data so as to make them safe for original flood heights, and 2nd defence line was approved to face the river spills in the case of any mishap in the first line at high discharges. These bunds came under severe action during flood 1992 and were heavily damaged. The matter was referred to higher authorities which in view of heavy extent of damages proposed their restoration with 6.0 ft free board above H.F.L of 1992. The 2nd defence line is 20.0 ft top width and side slope of 3:1/2:1 and has free board of 5.0 ft above H.F.L of 1973. The location of bunds and different river training structures have been shown on the attached index plan and page No.49. The total length of channels in Shujabad Canal Division is 368.10 Miles and Flood embankment/bunds is 173.75 miles alongwith 10 Nos J-Head Spurs.

EXECUTIVE ENGINEER, SHUJABAD CANAL DIVISION, MULTAN.

MUHAMMAD BIN KHURSHID

Binda Sub Division

Ilampur Sub Division

Shujabad Sub Division

Sub Divisional Officer

Ch Muhammad Ahmad

Sub Engineers

Iftikhar Ahmad           (Sardarpur)

Rai Nasir Ali             (Binda)

Muhammad Muzamil (Bosan)

Zilladar

Musharaf Shah    (Ilampur)

Sub Divisional Officer

Rasheed Baig Baber

Sub Engineers

Rana Sikandar Bakht      (Khairpur)

Muhamamd Iqbal           (Muzaffarabad)

Muhammad Zafar Iqbal (Nawabpur)

Waqas Yaseen                (River Survey)

Zilladar

Musharaf Shah          (Ilampur)

Haji Sikandar           (Shujabad)

Sub Divisional Officer

Rasheed Baig Baber

Sub Engineers

Shoaib Ul Hassan         (Sikandarabad)

Abdul Hameed Bhatti   (Gajju Hatta)

Sajid Hussain                (Shujabad)

Muhammad Sajid          (Panjani)

Zilladar

Haji Sikandar             (Shujabad)

1.3       ADMINISTRATIVE SETUP

CHAPTER -2

FLOOD PROTECTION AND RIVER TRAINING WORKS

  - DESIGN PARAMETER OF TRAINING WORKS

| Sr. No. | Name of Bund |  | Length Canal Miles |  | Top Width | Side Slopes |  |  | Free Board F/levels |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | River Face | Land Face |  |  |  |  |  |
| 1 | 2 |  | 3 |  | 4 | 5 | 6 |  | 7 |  |  |  |
| ## 1ST DEFENCE LINE / BUNDS (DESIGN DATA) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Fazil Shah flood bund RD.0+000 to RD10+500 |  | 2.10 |  | 25.00ft | 3:1 | 2:1 | 6.0ft (1992) |  |  |  |  |
| 2-i | Chenab flood bund RD.0+000-94+000 |  | 18.80 |  | 25.0ft | 3:1 | 3:1 | 6.0ft (1992) |  |  |  |  |
| 2-ii | Chenab flood bund RD.94+000-157+950 Tail |  | 12.79 |  | 25.00 | 3:1 | 2:1 | 6.00ft(1992) |  |  |  |  |
| 3 | Nawabpur escape bund L&R RD.0+000-8+000x2 =16+000 |  | 3.20 |  | 16.0ft | 3:1 | 2:1 | 5.0ft (1973) |  |  |  |  |
| 4 | Nawabpur flood bund RD.0+000-17+950 |  | 3.60 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1992) |  |  |  |  |
| 5 | Muzaffarabad flood bund RD.0+000-12+710 |  | 2.50 |  | 12.0ft | 3:1 | 2:1 | 4.0ft (1959) |  |  |  |  |
| 6 | Akbar flood bund RD.0+000-21+710 |  | 4.30 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1992) |  |  |  |  |
| 7 | Sher Shah Flood Bund RD.0+000-20+437 |  | 4.08 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973 |  |  |  |  |
| 8 | Sher Shah Railway Flood Bund. RD.0+000-15+100 |  | 3.02 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973 |  |  |  |  |
| 9 | Right bank of Shujabad BranchRD.231+000-279+000 |  | 9.6 |  | 20.0ft | 3:1 | 2:1 | 6.0ft (1976) |  |  |  |  |
| 10 | Right bank of Gajju Hatta Branch RD.17+000-35+000 |  | 3.60 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1976) |  |  |  |  |
| 11 | Nagni flood bund RD.0+000-51+000 |  | 10.20 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973) |  |  |  |  |
| 12 | Dhundoo flood bund RD.0+000-26+500 |  | 5.30 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973) |  |  |  |  |
| 13 | Gardez flood bund RD.0+000-15+750 |  | 3.28 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973) |  |  |  |  |
| 14 | Jalalpur Khaki flood bund RD.0+000-60+000 |  | 12.0 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973) |  |  |  |  |
| 15 | Satburji Flood Bund RD.54+000-72+500 along Satburji Disty |  | 3.70 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973) |  |  |  |  |
| 16 | Tie Flood Bund RD.0+000-8+600 |  | 1.72 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973) |  |  |  |  |
| 17 | Baloch wah flood bund RD.0+000-62+230. |  | 12.44 |  | 25.0ft | 3:1 | 2:1 | 6.0ft (1973) |  |  |  |  |
| Total: |  |  | 116.23 |  |  |  |  |  |  |  |  |  |
| ## 2ND DEFENSE LINE / BUNDS (DESIGN DATA) |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No. | Name of Bund |  | Length Canal Miles |  | Top Width | Side Slopes |  |  | Free Board F/levels |  |  |  |
|  |  |  |  |  |  | River Face | Land Face |  |  |  |  |  |
| 1 | 2 |  | 3 |  | 4 | 5 | 6 |  | 7 |  |  |  |
| 1 | Right bank of Shujabad Branch RD.54000-231000 |  | 35.4 |  | 20.0ft | 3:1 | 2:1 |  | 5.0ft (1973) |  |  |  |
| 2 | Right bank of Gajju Hatta Branch RD: 35+000-67+000 and Sutburji Disty RD: 0+000-54+000 |  | 17.02 |  | 20.0ft | 3:1 | 2:1 |  | 5.0ft (1973) |  |  |  |
| 3 | Right bank of Panjani Disty RD.87+000-120+000 |  | 6.60 |  | 20.0ft | 3:1 | 2:1 |  | 5.0ft (1973) |  |  |  |
| 4 | Right bank of Panjani Disty RD.123+000-131+000 |  | 1.60 |  | 20.0ft | 3:1 | 2:1 |  | 5.0ft (1973) |  |  |  |
|  | Total:- |  | 60.62 |  |  |  |  |  |  |  |  |  |
| ## STRUCTURE AT FLOOD BUNDS |  |  |  |  |  |  |  |  |  |  |  |  |
| ## J-HEAD SPUR |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No. | Name of Bund | Length Canal Miles |  | Top Width |  | Side Slopes |  |  | Free Board F/levels |  |  |  |
|  |  |  |  |  |  | River Face | Land Face |  |  |  |  |  |
| 1 | 2 | 3 |  | 4 |  | 5 | 6 |  | 7 |  |  |  |
| 1 | J-Head Spur RD: 4+000 Fazil Shah Flood Bund | 1.50 |  | 30.00 |  | 3:1 | 3:1 |  | 06 ft(1992) |  |  |  |
| 2 | J-Head Spur RD: 10+000 Chenab Flood Bund | 1.04 |  | 30.00 |  | 3:1 | 2:1 |  | 06 ft(1992) |  |  |  |
| 3 | J-Head Spur RD: 151+800 Chenab Flood Bund | 1.15 |  | 30.00 |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 4 | Extended Guided Spur No.1 RD.0+920 of Nawabpur Flood Bund by converting into J-Head Spur. | 0.92 |  | 30.00 |  | 3:1 2:1 | 3:1 2:1 |  | 6.0ft above H.F.L (1973) |  |  |  |
| 5 | J-head spur No.2 oppt:RD.14+400 Akbar flood bund | 1.21 |  | 25.0ft |  | 3:1 2:1 | 3:1 2:1 |  | 6.0ft above H.F.L (1973) |  |  |  |
| 6 | J-head spur No.3 oppt:RD.248+000 Shujabad | 0.878 |  | 25.0ft |  | 3:1 2:1 | 3:1 2:1 |  | 6.0ft above H.F.L (1973) |  |  |  |
| 7 | Extended Guided Spur No.3-B RD.252+000 of Shujabad Branch by converting to J-Head Spur. | 0.60 |  | 25.0ft |  | 3:1 2:1 | 3:1 2:1 |  | 6.0ft above H.F.L (1973) |  |  |  |
| 8 | J-head spur No.4 oppt: RD.263+000 Shujabad Branch. | 1.08 |  | 25.0ft |  | 3:1 2:1 | 3:1 2:1 |  | 6.0ft above H.F.L (1973) |  |  |  |
| 9 | J-head spur RD.42+200 Nagni flood bund (2950’) | 0.59 |  | 25/ 30.0ft |  | 3:1 | 3:1 |  | 6.0ft (1976) |  |  |  |
|  | Total:- | 8.968 |  |  |  |  |  |  |  |  |  |  |
| ## MOLE HEAD SPUR |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No. | Name of Bund | Length Canal Miles |  | Top Width |  | Side Slopes |  |  | Free Board F/levels |  |  |  |
|  |  |  |  |  |  | River Face | Land Face |  |  |  |  |  |
| 1 | 2 | 3 |  | 4 |  | 5 | 6 |  | 7 |  |  |  |
| 1 | Mole Head Spur RD: 55+000 Chenab Flood Bund | 0.10 |  | 30.00 |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 2 | Mole Head Spur at RD:45+700 Nagni Flood Bund (1250ft) | 0.25 |  | 30.0ft |  | 3:1 | 3:1 |  | 6.0ft (1976) |  |  |  |
| 3 | Sloping Spur at RD.10+000 Baloch Wah Flood Bund | 0.93 |  | 25.0ft |  | 3:1 | 2:1 |  | 6.0ft (1992) |  |  |  |
|  | Total:- | 1.28 |  |  |  |  |  |  |  |  |  |  |
| ## DATA OF CROSS SPURS ALONG CHENAB FLOOD BUND |  |  |  |  |  |  |  |  |  |  |  |  |
| Sr. No. | Name of Bund | Length Canal Miles |  | Top Width |  | Side Slopes |  |  | Free Board F/levels |  |  |  |
|  |  |  |  |  |  | River Face | Land Face |  |  |  |  |  |
| 1 | 2 | 3 |  | 4 |  | 5 | 6 |  | 7 |  |  |  |
| 1 | RD.6+600 | 0.04 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 2 | RD.10+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 3 | RD. 11+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 4 | RD. 16+200 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 5 | RD. 22+300 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 6 | RD. 25+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 7 | RD. 30+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 8 | RD. 33+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 9 | RD. 48+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 10 | RD.53+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 11 | RD. 56+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 12 | RD. 57+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 13 | RD. 61+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 14 | RD. 66+200 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 15 | RD. 70+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 16 | RD. 75+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 17 | RD. 93+000 | 0.13 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 18 | RD. 94+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 19 | RD. 95+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 20 | RD.96+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 21 | RD.97+500 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 22 | RD. 103+500 | 0.055 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 23 | RD.106+500 | 0.075 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 24 | RD. 107+500 | 0.065 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 25 | RD. 110+500 | 0.143 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 26 | RD. 111+200 | 0.050 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 27 | RD111+900 | 0.06 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 28 | RD. 113+000 | 0.095 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 29 | RD. 114+000 | 0.085 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 30 | RD. 115+000 | 0.08 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 31 | RD.115+500 | 0.095 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 32 | RD.131+000 | 0.07 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 33 | RD.132+000 | 0.113 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 34 | RD.138+500 | 0.09 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 35 | RD.140+000 | 0.09 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 36 | RD.141+400 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 37 | RD.141+700 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 38 | RD. 142+000-143+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 39 | RD. 143+000-144+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 40 | RD. 144+000-145+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 41 | RD. 145+000-146+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 42 | RD. 147+000-148+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 43 | RD. 148+000-149+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 44 | RD. 149+000-150+000 | 0.10 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
|  | CROSS SPURS ALONG DUHNDOO FLOOD BUND |  |  |  |  |  |  |  |  |  |  |  |
| 1 | RD.0+000 | 0.036 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 2 | RD. 3+500 | 0.12 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 3 | RD. 23+300 | 0.138 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
|  | CROSS SPURS ALONG BALOCHWAH FLOOD BUND |  |  |  |  |  |  |  |  |  |  |  |
| 1 | RD. 9+750 | 0.095 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 2 | RD. 17+100 | 0.038 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 3 | RD. 17+500 | 0.042 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 4 | RD. 18+000 | 0.038 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 5 | RD. 19+000 | 0.005 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
|  | CROSS SPURS ALONG JALALPUR KHAKI FLOOD BUND |  |  |  |  |  |  |  |  |  |  |  |
| 1 | RD. 10+000 | 0.08 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 2 | RD. 15+000 | 0.05 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 3 | RD. 24+500 | 0.043 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 4 | RD. 30+000 | 0.043 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 5 | RD. 40+000 | 0.043 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 6 | RD. 42+500 | 0.043 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 7 | RD. 45+000 | 0.054 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 8 | RD. 45+500 | 0.044 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 9 | RD. 50+000 | 0.08 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 10 | RD.52+200 | 0.044 |  | 25 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
| 11 | RD.55+000 | 0.1 |  | 20 ft |  | 3:1 | 3:1 |  | 06ft (1992) |  |  |  |
|  | Total | 4.978 |  |  |  |  |  |  |  |  |  |  |
| ## SOLID STONE STUDS ALONG CHENAB FLOOD BUND |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RD. 150+000-151+000 (3 No.) 3 x .04 = 0.12 | 0.12 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
|  | RD. 151+000-152+000 (3 No) 3 x .04 = 0.12 | 0.12 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
|  | RD. 152+000-153+000 (2 No) 2 x .04 = 0.08 | 0.08 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
|  | RD. 153+000-154+000 (2 No) 2 x .04 = 0.08 | 0.08 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
|  | RD. 154+000-155+000 (3 No) 3 x .04 = 0.12 | 0.12 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
|  | Total | 0.52 |  |  |  |  |  |  |  |  |  |  |
|  | SOLID STONE SLOPING SPUR CHENAB FLOOD BUND |  |  |  |  |  |  |  |  |  |  |  |
| 1 | RD. 151+000 | 0.035 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
| 2 | RD. 152+000 | 0.035 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
| 3 | RD. 153+000 | 0.035 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
| 4 | RD. 154+000 | 0.035 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
| 5 | RD.155+000 | 0.035 |  | 15 ft |  | 2:1 | 2:1 |  | 06ft (1992) |  |  |  |
|  | Total | 0.175 |  |  |  |  |  |  |  |  |  |  |

  - LOCATION PLAN

CHAPTER -3

BRIEF HISTORY OF PAST FLOOD EVENTS

- In the year 1973, Baloch wah flood bund existed in very weak condition and it was breached at many places due to unprecedented high floods of 1973.

- Flood water damaged Chenab Flood Bund during super flood 1992 between RD.48+000-50+000 and RD.79+000-98+000,110+000-136+000 and 150+000-Tail.

- Dhundhoo Flood Bund was breached between RD:21+000-22+000 during flood 2006.

- Due to construction of Head Muhammad Wala bridge by National Highway Authority, serious effect of back water by heading up was observed during flood of 2010.To protect Nawabpur Flood bund from RD.3+950-17+950, work for the raising of bund was completed.

- The Muhammad Wala bridge was constructed during 2009-10 in front of RD.11+350 of Akbar Flood Bund by National Highway Authority. As per Model Study, the level will be attained 417.50 U/S of bridge at a discharge of 10.0 lac Cs and back water effect will produced upto RD.3+950 of Nawabpur Flood Bund. A scheme for raising Akbar Flood Bund RD.6+050 to 11+350 and Nawabpur Flood Bund RD.3+950 to 17+950 have been completed.

- During flood 2014, Severe leakages were observed on various flood bunds i.e. RD.270+000-272+000 1st Defence line of Shujabad Branch, RD.87+000-88+000 of Chenab Flood Bund which were controlled by adopting effective and timely protective measures.

- During flood 2014, Erosive actions were observed at RD.24+000-56+000 of Jalalpur Khakhi Flood Bund, RD.37+000-38+000 Baloach wah Flood Bund and                      RD.47+000-53+000 of Chenab Flood Bund during flood 2014 which were protected with tree launching, bamboo bushing and flood fighting material etc.

# BRIEF HISTORY OF PAST FLOOD EVENTS

1.	FIRST DEFENCE LINE.

a)	FAZIL SHAH FLOOD BUND RD.0+000 to 10+500.

This bund is the upper most in the first line of defence and has a length of 10500ft. It serves the purpose of stopping the river spills from entering the pocket behind Chenab flood bund around its upstream anchor. This bund comes under severe action of fast flow and large spill depth at discharge above 3 lac Cs. The confluence of River Ravi and Chenab falls just opposite the head reach of this bund and the bund is therefore, effected more by Ravi floods than by Chenab spills. This bund not only protects the fertile agricultural area with rich produce, but also serves as the only flood defence for Sardarpur Town.

With reference to the classification of bunds given in Sind Bund Manual it is high height Bund but there is no cause of concern upto a discharge of 4.00 lac Cs. Vigilant watching is needed as the whole bund becomes vulnerable beyond this discharge. This bund was badly damaged in extra ordinary super high flood of 1992. It was restored in 1996 under flood damaged restoration programme. The design data of the bund is as under:-

Top width		=	25ft

Side slopes		=	3:1 / 2:1

Free board		=	6.0ft above H.F.L.1992

During 2010, it was experienced that back flow of water beyond this point threatened the local abadies and it was suggested that start of this flood Bund should be connected to some higher edge for the proper protection of area in this regard,

b)	CHENAB FLOOD BUND RD.0+000 to 157+950.

The Chenab flood bund was constructed along the left bank of river Chenab after the flood of 1959 with the purpose to save the abadies of District Multan, Tehsil Kabirwala of District Khanewal. Thickly populated area of Multan City, valuable irrigation system and other Government installations are situated behind this bund. In the first instance, this bund was designed for low to medium floods capacities with a free board of 4.0ft above H.F.L of 1973.

This bund has a total length of 31.59 miles. It protects as 1st Defence Line in Binda and Ilampur Sub Division of Shujabad Canals Division. With reference to the classification of bunds given in Sind Bund Manual major part of this bund is high height Bund. This bund is close to river. During flood season large spill depths / occurr towards River side. Due to large spill depths, strong wave action is also commonly generated causing substantial damages to river side slopes which have to be watched and protected.  In some reaches, this bund follows a sinuous alignment. There are several depressions on the River side. These depressions have been silted upto a large extent on the River side but still creates parallel flows.

This bund is safe upto a discharge of 4.0 lac Cs. Being close to river and beyond 4. Lac Cs, it is prone to damage due to parallel flow intensive watching is absolutely necessary to with-stand high water levels. To stabilize these parallel flow condition generated due to a deep creek running all along the bund, earthen and stone pitched x-spurs were constructed at vulnerable sites prior to flood 1997. This bund remained stable during peak flood of 2013 and 2014 and no abnormal damages were reported. The design data of the bund is as under:-

RD.			=	0+000 to 94+000

Top width		=	25ft

Side slopes		=	3:1 / 3:1

Free board		=	6.0ft above H.F.L.1992

RD.			=	94+000 to 157+950

Top width		=	25ft

Side slopes		=	3:1 / 2:1

Free board		=	6.0ft above H.F.L.1992

c)	NAWABPUR FLOOD BUND RD.0+000 to 17+950

Nawabpur flood bund is 3.6miles long with an average height of about 10ft except at RD.13300 and 14100 where two old creeks crosses the bund. The height at these two point increases upto 16ft making it a very high bund. This flood bund forms one side of Nawabpur Pocket. In order to check serious river erosion opposite Nawabpur bund  guide head spur was converted into J head Spur at RD.0920. National Highway Authority constructed Head Muhammad Wala Bridge in year 2010. Due to construction of Muhammad Wala Bridge serious back water effect was observed and Nawabpur flood bund was raised from RD.3950-17950. To ensure the safety of Multan city, sheet piling were also installed from 3950 to 17950 for controlling seepage under the flood bund

Top width				=	25ft

Side slopes				=	3:1/ 2:1

Free board				=	6.0ft above H.F.L 1992.

d)	AKBAR FLOOD BUND RD.0+000 to 21+710.

Akbar Flood Bund has a total length of 21710 Ft and starts from tail of Muzaffarabad Flood bund to protect Multan city and Multan Cantonment, Nawabpur flood bund joins Akbar Flood Bund at  its RD. 6050. This bund is anchored to some what high patch of ground at its tail at RD. 21710.The spill flows around this enter into the pocket at the back. This spill has been checked to some extant by construction of a NHA pacca road. National Highway Authority Constructed Head Muhammad Wala Bridge in year 2010. Due to construction of Muhammad Wala Bridge serious back water effect was observed. On the recommendation of Model study conducted by IRI and physically observed water level raising of Akbar Flood Bund RD.6+050 to 11+350 was completed.

Top width		=25ft

Side slopes		=3:1/ 2:1

Free board		=6.0ft above H.F.L 2010 as per Hydraulic Model Study Report 						   No. 1317 dated March 2016 (417.50+6’)

In order to check river tendency to shift toward the left flank, a J-head spur was also constructed at     RD. 14400 of Akbar flood bund. To ensure the safety of Multan city, sheet piling were also installed from 6+050 to 11+350 for controlling seepage under the flood bund

The spur remained safe during flood 2014.

e)	MUZAFFARABAD FLOOD BUND RD.0+000 to 12+710.

This bund forms one side of the Nawabpur pocket and has the second defence along right bank of Shujabad Branch for Multan City and cantonment. The bund also serves as the right bank of upper Muzaffarabad Disty. It comes under action only when flood water enters into the pocket on account of breach in escape channel link bund, on Nawabpur flood bund. During the high flood of previous years, there was no serious damage to this bund and it remained almost safe.

Top width				=	12ft

Side slopes				=	3:1/ 2:1

Free board				=	4.0ft above H.F.L 1959.

f)	SHUJABAD BRANCH RD:231+500-279+000 IST AND SECOND DEFENCE LINE.

The right bank of Shujabad Branch was restored from RD.231-263 under flood damages restoration programme with the following design data after 1992 flood.

Top width		=20.0ft

Side slope		=3:1/ 2:1

Free board		=6.0ft above H.F.L 1976.

In order to protect Multan city and the cantonment area, 2 No. J-head spurs at RD.248+000 and 263+000 of Shujabad Branch have been constructed under flood protection sector. Both these spurs remained almost safe during exceptional high flood of 1997 with minor damage to stone apron and slopes of earthen shanks. No serious damage occurred during flood 2013. During flood 2014 severe leakages were observed on flood bund RD.270-272 of Shujabad Branch and were controlled by adopting affective and timely protective measures.

g)	SPUR 3-B RD.252+000

During flood of 1994, the river changed its course in reach RD.248+000-263+000 of Shujabad Branch. It engaged spur 3-B and erosive action towards Shujabad Branch continued. A scheme was prepared for checking erosion against river action opposite RD. 252+000 Shujabad Branch for the protection of Multan City and irrigation system. This spur was constructed prior to flood 1997. Spur is in satisfactory condition and intact. To ensure the safety of Multan city, sheet piling were also installed from 231+500 to 279+000 for controlling seepage under the flood bund

Top width		=25.0ft

Side slope		=3:1/ 2:1

Nose Portion		=2:1 / 2:1

Free board		=6.0ft above H.F.L 1973.

h)	SHER SHAH FLOOD BUND RD.0+000 to 20+437

This bund is an extension of the left Marginal bund of Sher Shah Railway Bridge. It is located on the left bank of Sher Shah disty. During flood in 1973, the bund shattered and flooding occur in Multan –Muzaffargarh arterial road and area between Muzaffargarh and Sher Shah. After this incident and in order to protect this road from such damage in future, the Highway Department constructed its own bund on the left side of Sher Shah Disty. This bund was handed over to Irrigation Department in 1982 vide relief Commissioner Punjab letter No.54-85/91 C.R.C dated 18-2-82. The bund is not according to the specifications of Irrigation Department. During the unprecedented floods of 2014, the bund suffered some damages to its river side slopes.

Top width		=25.0ft

Side slope		=3:1/ 2:1

Free board		=6.0ft above H.F.L 1973.

i)	SHER SHAH RAILWAY FLOOD BUND RD.0+000 to 15+100.

Sher Shah Railway Flood Bund starts from tail of Sher Shah Flood Bund. This bund was constructed by Pakistan Railway Multan. Now, it has been transferred to Irrigation Department. Its length is 3.02 mile (15100ft) and runs parallel to Multan-Muzaffergarh railway track. The Breaching Section exist between RD:11000 to RD:12000 with maximum discharge capacity of 55000 cs. The Breaching Section has 4 No. of sets. The length of each set is 100ft and width 30ft. The Breaching Section is operated on attaining flood level 393.50 ft, The guage for operationing of Breaching Section of right pier of Railway Sher Shah Bridge.

Top width		=25.0ft

Side slope		=3:1/ 2:1

Free board		=6.0ft above H.F.L 1973.

g)	RIGHT BANK OF GAJJU HATTA BRANCH RD: 17000-35000.

Right bank of Gajju Hatta Branch in reach RD.17-35 was converted in to full fledged flood bund with the following parameter and act as the 1st Line of Defence in Shujabad Canal Division. It falls in Shujabad Canal sub division. The basic purpose is to protect main Railway Line, Basti Khokran, Sikandarabad Town canal network in Tehsil Shujabad.

Top width 		=25.0ft

Side slopes		=3:1/ 2:1

Free board		=6.0ft above H.F.L1976.

h)	NAGNI FLOOD BUND RD 0+000 TO 51+000.

Nagni Flood Bund has a total length of 51000 Ft and constructed for the safety of Shujabad Town. Bakhtuwa disty runs parallel to this flood bund. The design parameter of this flood bund is as under.

Top width		=25.0ft

Side slopes		=3:1/ 2:1

Free board		=6.0ft above H.F.L of 1973

i)	J-HEAD SPUR RD.42+200 NAGNI FLOOD BUND RD.0+000 to 2+950.

J-head spur was constructed by extending the shank of Mole Head Groyne at RD.42+200 of Nagni Flood bund during the year 1987-88. Due to super floods of 1988, the complex flow conditions generated in the proximity of the shank of the J-head due to deep scour. The shank gave way and about 1700 ft length was washed away.The departmental expert committee decided not to restore it in full length, but to pitch the same with 500ft in the existing condition. The restoration work was completed. The part of apron of j-head was launched / settled which was replenished accordingly. This structure is located at most sensitive point, as it remains under the direct hit of main Active River. The structure remained safe during flood 2014.

Length 				=RD.0+000 to 2+000

Top width				=25.0  ft

Side slopes				=3:1/ 3:1

Free board				=6.0ft above H.F.L of 1976

Length 				=RD.2+000 to 2+950

Top width				=30.0 ft

Side slopes				=3:1/ 2:1

Free board				=6.0ft above H.F.L of 1976

j)	MOLE HEAD SPUR AT RD.45+700 NAGNI FLOOD BUND RD.0+000 to 1250.

The apron of Mole Head Spur was replenished during 1987-88 and remained intact in super flood of 1988. Also the structure remained heavily engaged during flood 1996, but was quite safe during flood 1997.

Top width				=30.0 ft

Side slopes				=3:1/ 2:1

Free board				=6.0ft above H.F.L of 1976

k)	DHUNDU FLOOD BUND RD 0+000 TO 26+500.

This bund is in continuation of Nagni flood bund and has a total length of 26500 Ft. The tail of this bund connects with Gardez Flood Bund. The river has approached very close to this flood bund between RD. 3+000 to 6+000 due to erosion. This bund has to face strong parallel flow generated due to proximity of the left arm main River Chenab and formation of deep creek. It protects vast fertile area irrigated through Bakhtuwah Disty and Gardez Minor.

The designed data of the bund is as under:-

Length			=5.1 Mile

Top width		=25.0ft

Side slopes		=3:1/ 2:1

Free board		=5.0ft above H.F.L 1973

Dhundoo Flood Bund tails off in open area, the river spill flows around its tail end and inundates the area (about 4000 acres) on the country side by reverse flow (back water effects). To save these irrigated areas from spill, Dhundoo.

f)	GARDEZ FLOOD BUND RD.0+000 to 15+750.

Gradez Flood bund has a total length of 3.02 miles and terminates at RD 54+000 of Satburji Disty. Hence it link tail of Dhundoo Flood Bund with 2nd flood Defence Line at RD 54+000 of  Satburji Disty.  Leakage has been occurred during flood 2014. The designed data of the bund is given as below.

Top width		=25.0ft

Side slopes		=3:1/ 2:1

Free board		=6.0ft above H.F.L. 1973

g)	RIGHT BANK OF SATBURJI DISTY RD.54+000-72+000.

The Right bank of Satburji Disty from RD 54+000 TO 72+000 is converted into Flood Embankment to protect the mouza Bangala, khara, satburji and bagre town. The designed data of the bund is given as below.

Length			=3.46 Miles

Top width		=25.0ft

Side slopes		=3:1/ 2:1

Free board		=5.0ft above H.F.L 1973

.

h)	JALALPUR KHAKI FLOOD BUND RD 0+000 TO 60+000

Jalalpur Khaki flood Bund has a total length of 60000Ft.This bund protects the areas fed by Panjani Disty, system in Jalalpur Pirwala Tehsil. This bund is very close to the River Chenab and at some places the river arm is about 500ft from the bund. The nearby creeks remain dormant during winter. With the flood season strong parallel flow start specially at RD.55. The designed data of the bund is as under.

Length			=11.53Miles

Top width		=25.0ft

Side slopes		=3:1/ 2:1

Free board		=6.0ft above H.F.L 1973

During flood of 2014 erosive actions were observed between RD.24-56 of this flood bund which were protected with tree launching, bamboo bushing and flood fighting material etc.

h)	TIE FLOOD BUND RD 0+000 TO 8+400

This bund has a total length of 8400 ft and connects the tail RD.60+000 Jalalpur Khaki Flood Bund with RD.139+000 of Panjani Disty. It is very helpful in protecting the Jalalpur Pirwala Town, Behli Canal Rest House and Irrigation system at the tail of Panjani Disty.

The existing designed data of the bund is as under:-

Length			=1.6 Mile

Top width		=20.0ft

Side slopes		=3:1/ 2:1

Free board		=6.0ft above H.F.L 1973

i)	BALOACHWAH FLOOD BUND.RD 0+000 TO 62+230

Balochwah flood bund has a total length of 62230 Ft and is the last component of series of flood bunds in Shujabad Canal Division. The bund is very close to the river and thus suffers damages due to action of river spill and direct hits through active creek especially in reach RD.16+000-26+000. Flood level along this bund is also affected by the afflux created by the U/S training works of Punjnad Headworks on left marginal bund. Importance of this bund is however moderated with levels rising towards country side and spill water flowing round the tail end on the country side of bund.

Length			=11.96Mile

Top width		=25.0ft

Side slopes		=3:1/ 2:1

Free board		=5.0ft above H.F.L 1973

There are two villages Shahpur Lamma and Wachha Sandila which are under severe threat of River Chenab and ordinate near these villages is 3+600-4+200ft respectively opposite RD. 10+000 to 12+000 of Baloachwah Flood Bund. To save these villages proposal for constructions, 2Nos sloping spurs were submitted for approval but only one spur opposite RD.10+000 was agreed by the Federal Flood Commissioner. After construction of this spur, the above two villages are now safe from the direct river action.

OUTLET WATER COURSE AND TUBEWELL CROSSING.

It is experienced during the previous floods that the outlet/water course and tube well crossings under the flood bunds, created serious problems at some sites. Some outlets were also settled. Therefore sufficient flood fighting material is required to be available at each effective point for closing all these crossing before start of the flood seasons. In this regard, necessary certificates may be exercised by the Sub Divisional Officer, that all these crossing have been closed. Moreover special day and night watching establishment is also necessary to be appointed at each point to avoid any possibility of mishap during flood period.

2ND DEFENSE LINE IN SHUJABAD DIVISION.

#### Shujabad Branch (RD-54+000-231+000)

The right bank of Shujabad Branch from RD 54+000 to 231+00 has been converted into 2nd defence line flood embankment. The sub division wise differentiation of 2nd defence line Shujabad Branch RD 54+000 to 231+000 is as under.

      - Binda Canal Sub Division 	RD:54+000-163+000

      - Ilampur Canal Sub Division 	RD:163+000-231+000

The design parameter is as under.

Top width 		=20.0 ft

Side slopes		=3:1/2:1

Free board 		=5.0 ft above H.F.L 1973

- Right Bank of Gajju Hatta Branch Flood Bund RD:35+000-67+000

The right bank of Gajju Hatta Branch from RD 35+000 to 67+00 has been converted into                  2nd defence line flood embankment.

The design parameter is as under.

Top width		=20.0ft

Side slopes		=3:1/2:1

Free board		=5.0ft above H.F.L. 1973

- Right Bank of Satburji Disty Flood Bund RD:0+000-54+000

The right bank of Satburji Disty RD 0+000 to 54+000 has been converted into 2nd defence line flood embankment.

The design parameter is as under.

Top width		=20.0ft

Side slopes		=3:1/2:1

Free board		=5.0ft above H.F.L. 1973

- Right Bank of Panjani Disty Flood embankment RD:87+000-131+000

The right bank of Panjani Disty RD 87+000 to 131+000 has been converted into 2nd defence line flood embankment.

The design parameter is as under.

Length			=8.46mile

Top width		=20.0ft

Side slopes		=3:1/2:1

Free board		=5.0ft above H.F.L. 1973

CHAPTER -4

DESIGNED DATA, HISTORIC PEAK FLOOD DATA AND PREVIOUS FIVE YEARS FLOOD DATA OF HEAD WORKS/ BARRAGES AND OR OTHER CONTROL POINTS.

  - FLOOD LIMITS IN SHUJABAD CANAL DIVISION

| Description | Flood Limits (Lac Cusec) |
| --- | --- |
| Low Flood | 1.5 to 3.0 |
| Medium Flood | 3.0 to 4.0 |
| High Flood | 4.0 to 4.5 |
| Very High Flood | 4.5 to 6.0 |
| Exceptionally High Flood | Above 6.0 |

Flood limits upstream Gauging Station of Shujabad Canal Division

| Description | Trimmu Barrage River Chenab Flood Limits (Lac Cusec) | Sidhnai Barrage River Chenab Flood Limits (Lac Cusec) |
| --- | --- | --- |
| Low Flood | 1.5 to 3.0 | 0.3 to 0.45 |
| Medium Flood | 3.0 to 4.0 | 0.45 to 0.6 |
| High Flood | 4.0 to 4.5 | 0.6 to 0.9 |
| Very High Flood | 4.5 to 6.0 | 0.9 to 1.20 |
| Exceptionally High Flood | Above 6.0 | Above 1.20 |

  - TIME LAGS OF FLOODS

TABLE OF DISTANCES AND TIME LAGS FOR FLOOD IN RIVER CHENAB AND RAVI IN SHUJABAD DIVISION.

| Sr No | Stages | River Chenab |  |  |  |  | River Ravi |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Marala to Khanki 39 -miles | Marala to Qadarabad 57-miles | Qadarabad to Trimmu 163-miles | Trimmu to Shershah 100-miles | Shershah to Punjnad 60-miles | Jassar to Shahdara 72-miles | Shahdara to Balloki 54-miles | Balloki to Sidhnai Barrage 240miles | Sidhnai Barrage to Shershah 80-miles | Shershah to Punjnad 60-miles |
|  |  | Hours | Hours | Hours | Hours | Hours | Hours | Hours | Hours | Hours | Hours |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Low flood | 12 | 15 | 48 | 48 | 30 | 45 | 25 | 110 | 40 | 35 |
| 2 | Medium flood | 12 | 14 | 45 | 48 | 30 | 42 | 24 | 100 | 36 | 32 |
| 3 | High flood | 10 | 12 | 40 | 44 | 25 | 36 | 19 | 90 | 30 | 30 |
| 4 | Exceptionally high flood | 10 | 12 | 40 | 38 | 22 | 35 | 18 | 88 | 29 | 29 |

TABLE SHOWING DISTANCES AND TIME LAGS FOR FLOOD IN RIVER CHENAB IN SHUJABAD DIVISION.

##### River Chenab

| S.No | Stages | TRIMMU BARRAGE to |  |  | RIVER RAVI-CHENAB CONFLUENCE POINT TO | Nawabpur Flood Bund RD: 0+920 at Spur No.1 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Fazil Shah Flood Bund (44.0 miles) | River Ravi Confluence point/ Opposite RD 8+000 of Fazil Shah Flood Bund (45.5 miles) | Nawabpur Flood Bund RD: 0+920 at Spur No.1 (76.0 miles) | Nawabpur Flood Bund RD: 0+920 at Spur No.1 (30.6 miles) | Sher Shah Bridge (24.0 miles) |
|  |  | Hours | Hours | Hours | Hours | Hours |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | Low Flood | 21 | 22 | 36 | 15 | 12 |
| 2 | Medium Flood | 21 | 22 | 36 | 15 | 12 |
| 3 | High Flood | 19 | 20 | 33 | 13 | 11 |
| 4 | Exceptionally High Flood | 17 | 17 | 29 | 12 | 09 |

TABLE OF SHOWING DISTANCES AND TIME LAGS FOR FLOOD IN RIVER RAVI IN SHUJABAD DIVISION.

| S.No | Stages | SIDHNAI BARRAGE TO |  |  | Nawabpur Flood Bund RD: 0+920 at Spur No.1 |
| --- | --- | --- | --- | --- | --- |
|  |  | Fazil Shah Flood Bund (24.0 miles) | Nawabpur Flood Bund RD: 0+920 at Spur No.1 (30.6 miles) | Nawabpur Flood Bund RD: 0+920 at Spur No.1 (56.0 miles) | Sher Shah Bridge (24 Miles) |
|  |  | Hours | Hours | Hours | Hours |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | Low Flood | 12 | 15 | 27 | 12 |
| 2 | Medium Flood | 12 | 15 | 27 | 12 |
| 3 | High Flood | 11 | 13 | 25 | 11 |
| 4 | Exceptionally High Flood | 09 | 12 | 21 | 09 |

##### River  Ravi

NOTE:-

Time lags mentioned above have been calculated for stages which correspond to low floods. High discharges make more transit time due to heavy spilling enroot.

  - HIGHEST FLOODS

| AT SIDHNAI HEAD WORKS |  |  | AT TRIMMU HEAD WORKS |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Date | Discharge (Cs) | Year | Date | Discharge (Cs) |
| 1972 | 16.07.1972 | 22916 | 1972 | 12.07.1972 | 566984 |
| 1973 | 16.08.1975 | 210339 | 1973 | 12.08.1973 | 963249 |
| 1992 | 16.09.1992 | 93910 | 1992 | 14.09.1992 | 982027 |
| 1995 | 12.09.1995 | 210320 | 1995 | 01.08.1995 | 859881 |
| 2014 | 12.09.2014 | 71112 | 2014 | 10.09.2014 | 626006 |

- Highest Flood Discharge observed during 1992 as 982027 Cs. in Shujabad Canal Division.

  - PEAK DISCHARGES

| AT SIDHNAI HEAD WORKS |  |  | AT TRIMMU HEAD WORKS |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Date | Discharge (Cs) | Year | Date | Discharge (Cs) |
| 2001 | 07.08.2001 | 30585 | 2001 | 18.08.2001 | 85831 |
| 2002 | 18.08.2002 | 16086 | 2002 | 17.08.2002 | 125427 |
| 2003 | - | - | 2003 | 22.02.2003 | 165223 |
| 2003 | 09.08.2003 | 25486 | 2003 | 08.08.2003 | 137705 |
| 2004 | 23.08.2004 | 12773 | 2004 | 20.08.2004 | 48739 |
| 2005 | 23.07.2005 | 12078 | 2005 | 11.07.2005 | 171137 |
| 2006 | 02.08.2006 | 25595 | 2006 | 07.09.2006 | 270270 |

Peak Discharges

| AT SIDHNAI HEAD WORKS |  |  | AT TRIMMU HEAD WORKS |  |  |
| --- | --- | --- | --- | --- | --- |
| Year | Date | Discharge (Cs) | Year | Date | Discharge (Cs) |
| 2007 | 19.08.2007 | 14405 | 2007 | 17.08.2007 | 70093 |
| 2008 | 24.08.2008 | 38691 | 2008 | 07.08.2008 | 67836 |
| 2009 | 25.08.2009 | 23009 | 2009 | 21.08.2009 | 62265 |
| 2010 | 11.08.2010 | 10378 | 2010 | 11.08.2010 | 333404 |
| 2011 | 18.09.2011 | 17676 | 2011 | 20.09.2011 | 133090 |
| 2012 | 10.09.2012 | 26086 | 2012 | 07.08.2012 | 67694 |
| 2013 | 25.08.2013 | 77630 | 2013 | 20.08.2013 | 267609 |
| 2014 | 12.09.2014 | 71112 | 2014 | 10.09.2014 | 626006 |
| 2015 | 28.07.2015 | 38539 | 2015 | 31.07.2015 | 134715 |
| 2016 | 10.08.2016 | 9806 | 2016 | 11.08.2016 | 153334 |
| 2017 | 08.08.2017 | 16667 | 2017 | 05.08.2017 | 89345 |
| 2018 | 08.08.2018 | 15570 | 2018 | 05.08.2018 | 87204 |
| 2019 | 05.09.2019 | 15384 | 2019 | 22.08.2019 | 93021 |
| 2020 | 01.09.2020 | 28214 | 2020 | 01.09.2020 | 196077 |
| 2021 | 04.08.2021 | 32002 | 2021 | 31.07.2021 | 123167 |
| 2022 | 29.07.2022 | 22843 | 2022 | 18.08.2022 | 122891 |
| 2023 | 03.08.2023 | 43758 | 2023 | 26.07.2023 | 116786 |
| 2024 | 30.08.2024 | 24711 | 2024 | 19.08.2024 | 87030 |

CHAPTER -5

FLOOD FIGHTING STRATEGY

The following precautionary measures are adopted before flood season to face any alarming situation is Shujabad Canal Division Multan even then arrangements are necessary to avoid heavy damages and flood fighting strategy is adopted in such a way that flood water passes safely through River Chenab upto its limit.

- LOW FLOOD

In low flood, river water remains within the deep channel and do not cause any problem if regulation of gates is done as per rules. Clearance of Jungle , filling gharas and porcupine holes if any will be completed through contractors.

- MEDIUM FLOOD

When River water enters into Medium Flood i.e. 3 to 4 Lacs, it will start spreading on adjoining area and also touching the toe of bunds in some reaches. Then it becomes necessary to be more vigilant. The watching of flood bunds will start to avoid any leakage from bunds. Sub Engineers will shift their camps to sites as per duty roaster. Hourly gauges and discharge will be conveyed to Civil Administration and flood warning centers to inform public accordingly.

- HIGH FLOOD

When river water enters into high flood range i.e. 4 to 4.5 Lacs, Sub Divisional Officers, concerned will shift there camps to supervise staff working on Flood Bunds.

- VERY HIGH FLOOD

When River water enters into very High Flood range i.e. 4.5 to 6 Lacs, Executive Engineer alongwith Sub Divisional Officers will shift there camps to supervise staff working on said camps and to check any leakage if any. Critical gauge will be checked on hourly basis round the clock. If discharge is in rising position then breaching section is activated in consultation with Civil Administration and Pak Army.

- EXCEPTIONALLY HIGH FLOOD

Gauges of different sites especially gauge at RD.11350 of Akbar Flood Bund will be noted regularly and recorded. Executive Engineer will inform all agencies especially Civil Administration as well as Pak Army authorities about flood situation in rising position. In case of very high flood or exceptionally high flood, meeting will be held in field office with Pak Army and Civil Administration to face the situation.

Two dozers will be kept ready at RD.6000 to 11350 Akbar Flood Bund for making cuts and closing breaches as and when required according to circumstances. When gauges RD.11350 Akbar Flood Bund touches level 417.50 and is found in rising position it will be the duty of Executive Engineer, to request Pak Army for operation of breach section for the safety of Flood protection bund. When water subsides, the breaching sections will be closed (if operated) as soon as possible.

REACHES PRONE TO PARALLEL FLOW / DAMAGES

The reaches on various flood bunds where parallel flows are likely to occur to damage the bunds and the spill conditions, have been shown in a statement appearing as Appendix-I. (Page No.27)

VULNERABLE SITES.

The sites/reaches where the bunds become vulnerable and require special attention have been shown in a statement appearing Appendix-II. (Page No.29)

DANGER GAUGES.

The general population along flood bunds do not fully understand the flood conditions expressed in terms of discharges as gravity of flood gauges. It is therefore already shown at gauge site weather the flood situation is safe are not. For this purpose, special gauges named as danger/emergency gauges have been selected on the bunds. The bund will be vulnerable when flood mark touches the danger level and evacuation of general public protected by the bund becomes necessary. The danger level will be marked on the gauge in a red band 3 inches wide. A statement depicting the location at each gauge and the R.L. of danger marks has been attached as Appendix III. (Page No.30) When these attain the danger level with river rising situation the population on the country side will be warned to evacuate.

EXTENT OF FLOODING.

The flooded area depends upon the magnitude of river discharge and water levels attained in the flooded plain. Normally the area lying on river side will be submerged at medium flood level while some areas on the back of first defence line at the tail of Akbar Flood Bund will also be submerged due to spill flow around the tail end. Increase in the river discharge will result in the increase of submerged area and cannot be predicted readily for the time being. However to check this problem and to make area safe. Due to above facts a scheme has been prepared namely connecting tail Akbar Flood Bund with Shujabad Branch, which will make some area safe against the spill out from the tail of 1st Defence line.

RELIEF CUTS.

In the event of a mishap in the first defence, the escapage spreads in the compartment between first and second defence bund starts accumulating. After the compartment has been filled, the water starts heading up at the tail end due to pocketing effect which will vary with the actual distance of the site of mishap from the tail end. This excessive depth of water submerges almost everything in the pocket with the result that numerous cuts are made by the public or by the staff incharge in complete panic which in addition to spoiling the bund sometime also proves to be in-effective.

It is, therefore, necessary that some definite sites be selected before hand for making relief cuts to release the accumulated spill effectively without spoiling the bunds through unnecessary gaps. There are three such pockets in Shujabad Division.

It is therefore, proposed that in the case of need, relief cuts will be made at the following location.

1.	Chenab flood bund.			=RD.151+000-157+000(Near tail)

2	Escape link bund

(R&L)					=Between RD.5+500+7+300

3	Nawabpur flood bund.		=Between RD.16+000-17+000

MACHINERY FOR RELIEF CUTS AND EMERGENCIES.

a)	FOR RELIEF CUTS.

As the relief cuts will involve quite a substantial quantity of earthwork, it will be necessary to use either dozers and excavators or blasting method. For this purpose, arrangement of at least 3 dozers and 4 No. of excavators would be made available either from Agricultural Engineering Department or some other source and these would be stationed at Nawabpur Regulator & Satburji Rest House. If dozers and excavators are not available, matter would be taken up with Army Engineers for blasting the bund at relief cut sites when required. If necessary, Trimmu and Sidhnai Barrage Division may be contacted for requirement of liners, because they have such arrangement ready at all times.

b)	FOR EMERGENT REPAIR.

Some machinery is also required for emergent repairs at vulnerable/dangerous points/reaches, detail of which appear below:-

1.	Two dozers at RD.28+000-30+000 and two dozers at RD.30+000-86+000 Chenab flood bund.

2.	One dozer and one excavator at RD.252+000 of Shujabad Branch.

3.	One dozer at RD.14+000-17+000 of Baloachwah Flood Bund and one dozer at RD.49+000 of Nagni Flood Bund.

This will make a total of eight bulldozers which will be obtained from Agricultural Engineering Department directly, or through Deputy Commissioner, if necessary Army dozers can also be requisitioned.

.

COUNTER SABOTAGE MEASURES.

During floods of very  high   and  exceptionally  high  magnitude,  the  spill   depth  on  river side sometimes becomes so large   that  people living   in spill area cut the  bunds to   release pressure on their dwellings. Similar cuts can also be expected from Anti-National   Forces. It will, therefore, be necessary in such a case that Civil Authorities or police officers who are on duty at the bunds or in relief

centers may be contacted by the Sub Divisional Officer, incharge under intimation to higher Officers

for timely provision of police patrol and taking other necessary counter sabotage measures.

MOBILE/FIELD DISPENSARY AND SERA.

Floods bring diseases and misery. Poisonous insects and reptiles like scorpions, snakes etc, are picked up by flood waters and are thrown on to dry bunds. Thus snake and insect bite cases are not uncommon. It therefore, becomes necessary that emergency drugs and serum be made available at the bund. For this purpose, the District Officer Health would be requested to provide Field/Mobile Dispensary with sufficient quantity of emergency drugs and Anti snake venom serum, Anti Tetanus serum and necessary vaccines to control disease and epidemics which  come in the wake of floods.

FLOOD INUNDATIONS.

The flood heights have been increasing persistently due to :-

1.	Aggradations in river Channel.

2.	Reduction in effective water way. This reduction can be attributed to :-

a)	Construction of pacca houses, Tube well structures and other permanent obstructions like gardens etc.

b)	Cultivation of tall crops like sugarcane and juntar etc.

The construction of structures and sowing of tall crops will have to be discouraged to reduce further encroachment of river width.

FLOOD STAGES AND LIMITING DISCHARGES.

There is no discharge observation site opposite flood bunds in Shujabad Division and thus flood categories could not be defined uptill now. The only reliable way is to fix the discharge limits for various flood categories opposite flood bunds in Shujabad Division with reference to the total discharges passed downstream Trimmu and Sidhnai Barrages.

It is an accepted fact that substantial aggradation took place in all the rivers during the low discharge cycle in the decade prior to 1973. This resulted in early spills and spills depths for all discharge stages increased. It thus became necessary to revise the existing flood limits on Barrages.	The Punjab Flood Commissioner, therefore, considered this aspect in the 55th meeting held on 17-06-1974 and approved revised discharge limit for various categories of floods, These limits are in force since then and appear in the statement attached  as  Chapter-4 (4.1)page-16. The distance between various barrages on river Jhelum, Chenab & Ravi and main strategic points on flood bunds in this Division along with respective time lags are attached at Chapter-4 (4.2)Page 17-18. The limiting discharges for river Chenab at Sardarpur (Chowki Mojhan) and Nawabpur near Multan are proposed to be reckoned with reference to the effective totals of discharges below Trimmu limits are:-

1.	Discharge above 1.50 lac Cs

but less than 3.00 lac Cs.		            =	Low flood

2.	Discharge above 3.00 lac

but less than 4.00 lac.				=	Medium flood

3.	Discharge above 4.00 lac

but below 4.5 lac Cs.			            =	High flood

4.	Discharge above 4.5 lac

but below 6.00  lac Cs.			=	Very high flood

5.	Discharge of 6.00 lac Cs

and above.				           =	Exceptionally high flood

The word discharge as used above means the total effective discharge downstream Trimmu and Sidhnai Barrages. The categories of flood in this text is in accordance with the above limits.

STATEMENT SHOWING THE REACHES OF FLOOD BUNDS & DISCHARGES AT WHICH SPILL TOUCHES AND POSITION OF PARALLEL FLOW.		APPENDIX-I

| Sr No. | Name of Flood Bund | Discharge at Which Spill Touches | Reaches of the Bund Touched | Position of Parallel Flow | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | Fazal Shah flood bund | 2.0 Lac Cs | 6+000-10+500 | Tail Mild | The parallel flow intensities beyond 4.0 lac Cs. Earthen and No. of pitched x-spurs have been constructed to check parallel flow in addition.7 No. sloping spur and 14 No. stone studs have also been constructed. |
|  |  | 3.0 Lac Cs | 3+000- Tail | Moderate |  |
|  |  | 4.0 Lac Cs | 3+000- Tail | Severe |  |
|  |  | Above |  |  |  |
|  |  | 4.0 Lac Cs | 0+000- Tail | Severe to intense |  |
| 2 | Chenab flood bund | 2.0 Lac Cs | 40+000-65+000 | Mild |  |
|  |  |  | 120+000- Tail | Mild |  |
|  |  | 3.0 Lac Cs | 0+000- Tail | 0+000-40+000 Moderate |  |
|  |  |  |  | 40-55 Severe |  |
|  |  |  |  | 4859 Severe |  |
|  |  |  |  | 90-91 Severe |  |
|  |  |  |  | 103-104 Severe |  |
|  |  |  |  | 120 Tail |  |
|  |  | 4.0 Lac Cs | 0+000- Tail | Intensive |  |
|  |  | Above |  |  |  |
| 3 | Nawabpur flood bund | 2.0 Lac Cs | 0+000- Tail | Mild |  |
|  |  | 3.0 Lac Cs | 0+000- Tail | Moderate |  |
|  |  | 4.0 Lac Cs | 0+000- Tail | Severe |  |
|  |  | above4.0 Lac Cs | 0+000- Tail | Intensive5-6,10-11 &17-Tail |  |
| 4 | Akbar flood bund | 2.0 Lac Cs | 6+000- Tail | Mild |  |
|  |  | 3.0 Lac Cs | 6+000- Tail | Severe |  |
|  |  | 4.0 Lac Cs | 6+000- Tail | Severe intensive |  |
|  |  | above4.0 Lac Cs | 6+000-Tail | Intensive RD.12-14 |  |
| 5 | RD.17-35 Gajju Hatta Branch Nagni flood bund | 2.0 Lac Cs | 25+000-40+000,48+000-52+000 | Normal | The portion of flood bund comes under action when Sher Shah Railway flood bund is breached. |
|  |  | 3.0 Lac Cs | 33+000-40+000 | Mild |  |
|  |  | 4.0 Las Cs | 33+000-51+000 | Moderate |  |
|  |  | above 4.0 Lac Cs | 2+000-51+000 | Severe intensive |  |
|  |  | 3.0 Lac Cs | 21+000-24+000 | Mild |  |
|  |  | 4.0 Lac Cs | 20+000-26+000 | Moderate |  |
|  |  | above4.0 Lac Cs | 0+000-26+000 | Severe intensive |  |

| 6 | Dhundoo flood bund | 2.0 Lac Cs | 0+000-23+000 | Normal | Spill water touches. Five earthen cross spurs have also been constructed to check parallel flow. |
| --- | --- | --- | --- | --- | --- |
|  |  | 3.0 Lac Cs | 21+000-24+000 | Mild |  |
|  |  | 4.0 Lac Cs | 20+000-26+000 | Moderate |  |
|  |  | above4.0 Lac Cs | 0+000-26+000 | Severe intensive |  |
| 7 | Jalalpur Khaki flood bund | 3..0 Lac Cs | 40+000-45+000 | Moderate | Spill water and also creates emergency in reach RD.16+000-20+000 due to parallel flow |
|  |  | 4.0 Lac Cs | 30+000-Tail | Severe |  |
|  |  | 4.5 Lac Cs | 10+000-Tail | Severe intensive RD.45-Tail |  |
| 8 | Gardezpur flood bund Tie bund | 2.25 Lac Cs | 0+000-2+000 | Normal | Four earthen cross spurs have already been constructed to check this parallel flow |
|  |  | 2.0 Lac Cs | 0+000-13+000 | Moderate |  |
|  |  | 3.0 Lac Cs | 0+000-13+000 | Severe |  |
|  |  | 4.0 Lac Cs | 0+000-13+000 | Intensive |  |
| 9 | Balochwah flood bund | 2.0 Lac Cs | 5+000-14+000 | Moderate | Spill water touches, Reach RD.16+000-20+000 is very sensitive and usually creates alarming situation during flood. |
|  |  | 3.0 Lac Cs | 0+000-18+000 | Severe |  |
|  |  | 4.0 Lac Cs | 0+000-18+000 | Severe |  |
|  |  | above4.0 Lac Cs | 0+000-18+000 | Intensive |  |

(Appendix-II)

STATEMENT SHOWING VULNERABLE REACHES/SITES ON FLOOD BUNDS IN SHUJABAD CANAL DIVISION MULTAN

WHICH REQUIRE SPECIAL ATTENTION.

| Sr. No. | Name of Flood Bund | Vulnerable Reach/Sites |
| --- | --- | --- |
| BINDA SUB DIVISION |  |  |
| 1 | Chenab flood bund. | RD. 3+000, 6+000, 50+000-52+000, 90+000-98+000 |
| ILAMPUR SUB DIVISION |  |  |
| 4 | Akbar flood bund | RD. 6+000-11+000 |
| 5 | Shujabad Branch 1st Defence Line | RD:252+000 |
| SHUJABAD SUB DIVISION |  |  |
| 6 | Nagni flood bund. | RD. 40+000-45+000 |
| 7 | Dhundoo flood bund. | RD. 20+000-25+000, |
| 8 | Bloachwah flood bund. | RD. 5+000-10+000, |
| 9 | Jalal Pur Khaki flood bund. | RD. 7+000-11+000, 45+000-55+000 |

NOTE:-

A site becomes vulnerable when it is expected to face.

Wave wash

Direct attack by a creek.

Intense parallel flow or contains bed soil with large depth and head across.

(APPENDIX-III)

TABLE SHOWING THE DANGER GAUGES FOR FLOOD BUNDS IN SHUJABAD CANAL DIVISION MULTAN.

| Sr No. | Name of Flood Bund | Danger Flood Mark River Rising | Site | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Chenab flood bund. | 431.60 | RD.50+000 |  |
| 2 | Chenab flood bund. | 418.50 | RD.130+000 |  |
| 3 | Chenab flood bund., | 413.30 | RD.157+000 |  |
| 4 | Nawabpur flood bund. | 410.30 | RD.10+000 |  |
| 5 | Akbar flood bund. | 417.50 | RD.11+350 |  |
| 6 | Nagni flood bund. | 378.50 | RD.45+000 |  |
| 7 | Dhundoo flood bund | 373.50 | RD.20+000 |  |
| 8 | Jalalpur Khaki flood bund. | 359.80 | RD.55+000 |  |
| 9 | Baloachwah flood bund. | 355.70 | RD.10+000 |  |

The above danger gauges have been proposed in consideration of the site conditions contained during high flood( when the danger levels on these gauges are touched). The people will automatically know that they have to evacuate immediately.

CHAPTER -6

FLOOD DAMAGES RESTORATION WORKS

THERE IS NO FLOOD DAMAGES RESTORATION WORKS CARRIED OUT IN THIS DIVISION DURING THE YEAR 2024.

f

CHAPTER -7

FLOOD FIGHTING / WATCHING ARRANGEMENT

  - PRE-FLOOD ARRANGEMENT

The pre-flood period will be counted as the period starting from the start of flood season upto the time the flood water rises upto 1.00ft, above general N.S.L. along bunds. During this period two watch men per mile per shift will be employed for two shifts during 24 hours. This will apply to Marginal Bund and other flood bunds. Now as decided during Zonal Office meeting dated.03/04/2000that work charge establishment be allowed at the rate of one man per two miles for Pre-flood preparation.

| Sr. No. | Name of Intervention | Sanctioned Reserve Stock of Stone Limit | Reserve Stock Available at Site | Balance Reserve Stock Required | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | Guide Bank Spur No. 1 RD: 0+920 Nawabpur Flood Bund | 2.00 Lac (Cft) | 1.96 Lac (Cft) | 0.04 Lac (Cft) | 63.25 % reserve stock of stone is available at site. |
| 2 | J-Head Spur No. 2 RD: 14+400 Akbar Flood Bund | 1.00 Lac (Cft) | 0.72 Lac (Cft) | 0.28 Lac (Cft) |  |
| 3 | J-Head Spur No. 3 RD: 248+000 Shujabad Branch | 1.00 Lac (Cft) | 0.96 Lac (Cft) | 0.04 Lac (Cft) |  |
| 4 | J-Head Spur No. 4 RD: 263+000 Shujabad Branch | 1.00 Lac (Cft) | 0.62 Lac (Cft) | 0.38 Lac (Cft) |  |
| 5 | Sher Shah Railway flood bund | 1.00 Lac (Cft) | 0.11 Lac (Cft) | 0.89 Lac (Cft) |  |
| 6 | J- Head Spur Chenab flood bund RD.151+800 | 0.50 Lac (Cft) | 0.30 Lac (Cft) | 0.20 Lac (Cft) |  |
| 7 | Chenab Flood Bund RD.151+000 to 157+000 | 0.50 Lac (Cft) | 0.20 Lac (Cft) | 0.30 Lac (Cft) |  |
| 8 | Spur No. 3 B RD 252+000 Shujabad Branch | 1.00 Lac (Cft) | 0.37 Lac (Cft) | 0.63 Lac (Cft) |  |
| 9 | J-Head Spur at RD: 42+200 Nagni Flood Bund | 1.00 Lac (Cft) | 1.00 Lac (Cft) | - |  |
| 10 | Mole Head Spur at RD: 45+700 Nagni Flood Bund | 1.00 Lac (Cft) | 0.82 Lac Cft | 0.18 Lac (Cft) |  |
| 11 | Sloping Spur RD 10000 at Baloch wah Flood Bund | 0.50 Lac (Cft) | - | 0.50 Lac (Cft) |  |
| 12 | Chenab Flood Bund RD: 21+000-26+000 | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 13 | Chenab Flood Bund RD: 28+000-33+000 Sardarpur Section | 0.15 Lac (Cft) | 0.046 Lac (Cft) | 0.104 Lac (Cft) |  |
| 14 | Chenab Flood Bund RD: 47+000-50+000 Sardarpur Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 15 | Chenab Flood Bund RD: 61+000-63+000 Binda Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 16 | Chenab Flood Bund RD: 80+000-96+000 Binda Section | 0.15 Lac (Cft) | 0.39 Lac (Cft) | 0.06 Lac (Cft) |  |
| 17 | Chenab Flood Bund RD: 96+000-110+000 Binda Section | 0.15 Lac (Cft) |  |  |  |
| 18 | Chenab Flood Bund RD: 113+000-117+000 Bosan Section | 0.15 Lac (Cft) |  |  |  |
| 19 | Chenab Flood Bund RD: 118+000-129+000 Bosan Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 20 | Chenab Flood Bund RD: 155+000-157+000 Bosan Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
|  | Total | 11.85 Lac (Cft) | 7.496 Lac (Cft) | 4.354 Lac (Cft ) |  |
|  | Note:- Percentage of available Reserve Stock of Stone = 63.25% |  |  |  |  |

  - Watching Establishment

The watching establishment on marginal bunds is required to be employed as per following scale in accordance with the approval given by Chief Engineer Irrigation Multan vide his letter No.16746-49 dated 31-05-79.

| Site | FLOOD LIMITS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low |  | Medium |  | High |  | Very High |  | Exceptionally High |  |
|  | Men | Shift | Men | Shift | Men | Shift | Men | Shift | Men | Shift |
| Camp at RD.10+000 of Chenab Flood Bund (4.10 Miles) | - | - | 43 | 3 | 44 | 3 | 60 | 3 | 80 | 3 |
| Camp at RD.87+000 of Chenab Flood Bund (15.40 Miles) | - | - | 55 | 3 | 55 | 3 | 60 | 3 | 90 | 3 |
| Camp at RD.151+800 Chenab Flood Bund and Akbar Flood Bund RD.11+350 (12.96 Miles) | - | - | 60 | 3 | 85 | 3 | 90 | 3 | 106 | 3 |
| Camp at RD.252+000 Shujabad Branch, RD6-7 Sher Shah Flood Bund, Sher Shah Flood Bund RD.19-20 & Breaching Section Railway Flood Bund (4.20 Miles) | - | - | 105 | 3 | 115 | 3 | 120 | 3 | 132 | 3 |
| Nagni Flood Bund RD.42+200 (8.44 Miles) | - | - | 28 | 3 | 44 | 3 | 53 | 3 | 66 | 3 |
| Camp RD.21+000 Dhundoo Flood Bund (4.20 Miles) | - | - | 24 | 3 | 35 | 3 | 40 | 3 | 44 | 3 |
| Camp Jalalpur Khakhi Flood Bund RD.45+000 & Baloch wah Flood Bund RD.23+000 (13.60 Miles) | - | - | 35 | 1 | 57 | 3 | 66 | 3 | 78 | 3 |

Note :- Payment for work charge labour subject to verification by TPM Consultants.

7.3	Arrangement at sensitive sites

- The normal strength for watching at sensitive sites along flood protection and River training works will be doubled during high flood / exceptionally high flood.

(ii)	The establishment can be increased after consultation with Executive Engineer, Shujabad Canal Division, Multan / Superintending Engineer, Haveli Canal Circle Multan in emergency & in accordance with site requirements.

The establishment will be responsible for maintenance of flood bunds and Spurs Shanks. It will arrange Minor accomplish to bank top, side slopes, removal of Sarkanda etc as shanks and on pitched part of spurs. Establishment as well as Machinery would also be responsible for filling gharas / rain cuts etc as occurred due to rains during the monsoon season.

Note:- Detail given in Chapter No.14.

7.4	Watching Material

Watching material will be placed at different hut sites, an additional material will also be required in order to meet with extraordinary worst situation and to recoup the site situation. Hence the following material will be kept ready in the Workshop for this purpose.

| Sr. No. | Name of articles | Unit | Quantity Required | Quantity Available | Balance Quantity Required | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  | Re-chargeable Torches | No. | 10 | 2 | 8 | Balance quantity of watching Material would be purchased before the onset of flood 2025 subject to approval of competent authority. |
|  | Munj Ban | Kg | 856 | 856 | -- |  |
|  | Gunny bags | No. | 5540 | 5540 | -- |  |
|  | Chain Saw | No. | 10 | 0 | 10- |  |
|  | Lar Mal 50’ | No. | 2500 | 2500 | -- |  |
|  | Kassi with handle | No. | 496 | 496 | -- |  |
|  | Hammer | No. | 10 | 10 | -- |  |
|  | Wire Crate | KG | 1000 | 1000 | -- |  |
|  | Buckets 12” size | No. | 265 | 265 | -- |  |
|  | Pump With generator (Petrol Engine 100 CC, Centrifugal Pump, Flexible Pipe Frame for shifting) | No. | 10 | 10 | 0 |  |
|  | Plastic Sheet | RFT | 2830 | 2830 | -- |  |
|  | Sand | Cft. | 15000 | 15000 | -- |  |
|  | Baskets | No. | 5632 | 5632 | -- |  |
|  | Sticks 6’-8’ long | No. | 8255 | 8255 | -- |  |
|  | Flood Relief Jackets | No. | 1500 | 1500 | -- |  |
|  | Generator | No. | 10 | 10 | -- |  |
|  | Manila Rope ½” to 1" | Kg | 100 | 100 | -- |  |
|  | Water Cooler | No. | 10 | 10 | -- |  |
|  | Water Set | No. | 10 | 10 | -- |  |
|  | Chair Set | Set | 10 | 0 | 10 |  |
|  | Steel wire 10 No. | Kg | 500 | 500 | -- |  |
|  | Bamboo 8'to 12' | No. | 2000 | 2000 | -- |  |
|  | Charpai | No. | 40 | 20 | 20 |  |
|  | DC Fan Pedestal | No. | 10 | 0 | 10 |  |
|  | Bulb, Light Holder | No. | 120 | 120 | -- |  |
|  | Mat Plastic | No. | 10 | 0 | 10 |  |
|  | Crockery | Set | 10 | 0 | 10 |  |

Note :- Material will be purchased before onset of flood season 2025 subject to approval of competent authority.

7.5	Arrangement for Sounding & probing

The arrangement for sounding and probing will be taken up with the motor boat

7.6	Lighting Arrangement

On very dangerous locations like depressions or old breach sites, electric lighting would be provided with portable generating sets, if possible, low tension electric supply line would be got installed in dangerous reaches of flood bunds, as generating sets and regular transmission line facilities may not be normally possible; available resources may be utilized. However, efforts to obtain portable electric generating sets of about 2.5 K.V. capacities will be continued 5 No, such generating sets are be required.

7.7	Ration Arrangement

Ration will be arranged by the local agencies and through NGO,s etc,

7.8	P.O.L Arrangement for vehicle

P.O.L facilities / arrangement will be made by the Irrigation Department & chargeable to flood embankments during flood.

  - Transportation

Provision for at least one jeep for each Sub Divisional Officer plus one for the Executive Engineer, will be aimed at. If these cannot be provided from departmental resources, requisition will be placed with civil authorities well in time for arrangement in terms of District Flood Protection Scheme.

One motor truck or pick up will also be provided in each Sub Division to handle carriage of materials. The truck may be hired direct or requested through Civil Authorities. Special vehicles would be arranged in each Sub Division through private sources in order to combat any vigorous situation. This vehicles comprises of special flood fighting arrangements along with establishment.

  - Law and order

Law and order situation will be tackled by the Civil Administration with the help of Police Department

  - Medical Arrangement for Labour

Health Department will be responsible for mobilizing their sources to carry out necessary operation for providing medical facilities to the labour.

7.12	  Liaison with other Department

Executive Engineer Shujabad Canal Division Multan will keep liaison with the Civil and Army Authorities and will also keep the Superintending Engineer Haveli Canal Circle Multan and Chief Engineer, Irrigation Multan Zone Multan fully apprised of the flood situation in Shujabad Canal Division Multan.

7.13	Role of Army

Commander, Engineer 2-Corps, Multan will perform activities to fight flood emergency During Flood 2025.

7.14 Duties of telephone Attendant

| Sr.No. | Name of Person | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Muhammad Aslam | 15.06.2025 to 15.10.2025 | 06:00 | 14:00 |
| 2 | Muhammad Umair | 15.06.2025 to 15.10.2025 | 06:00 | 14:00 |
| 3 | Muhammad Amnan | 15.06.2025 to 15.10.2025 | 06:00 | 14:00 |
| 4 | Muhammad Tariq | 15.06.2024 to 15.10.2024 | 14:00 | 22:00 |
| 5 | Muhammad Akram | 15.06.2025 to 15.10.2025 | 14:00 | 22:00 |
| 6 | Muhammad Waqas | 15.06.2025 to 15.10.2025 | 14:00 | 22:00 |
| 7 | Muhammad Asim | 15.06.2025 to 15.10.2025 | 22:00 | 6:00 |
| 8 | Muhammad Amnan | 15.06.2025 to 15.10.2025 | 22:00 | 6:00 |
| 9 | Muhammad Zeshan | 15.06.2025 to 15.10.2025 | 22:00 | 6:00 |

The Telephone attendants will be present 24 hours turn wise .They will receive gauges from all related stations and pass on to concerned quarter for necessary action

7.15 Wireless arrangements

The wireless sets will be provided by the Deputy Commissioner concerned before the coming flood

DETAIL OF VEHICLES WHERE WIRELESS SET INSTALLED

| Sr.No. | Vehicle Name | Designation | Remarks |
| --- | --- | --- | --- |
| 1 | GBA-547 | Executive Engineer, Shujabad Canal Division, Multan | Wireless set will be installed before flood season. |
| 2 | LEG-3440 | Sub Divisional Officer, Ilampur Canal Sub Division, Multan | ----do---- |
| 3 | BRB-7794 | Sub Divisional Officer, Binda Canal Sub Division, Binda | ----do---- |
| 4 | MNM-3200 | Sub Divisional Officer, Shujabad Canal Sub Division, Shujabad | ----do---- |

Note:- Telegraph office will act as base station.

CHAPTER -8

DETAIL OF ENCROACHMENTS

- LIST OF UNAUTHORIZED ENCROACHES ON FLOOD BUNDS IN SHUJABAD CANAL DIVISION.

| Sr. No. | Name of Flood Bund | Total No. Encroachments | No of Encroachments Removed | No of Encroachments to be Removed | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | TEHSIL KABIRWALA & DISTRICT KHANEWAL |  |  |  |  |
| 1 | Fazil Shah Flood Bund | 13 | 8 | 5 | Operation for removal of encroachments is under process. |
| 2 | Chenab Flood Bund RD:0100 | 100 | 84 | 16 |  |
|  | TOTAL | 113 | 92 | 21 |  |
|  | TEHSIL SADAR MULTAN & DISTRICT MULTAN |  |  |  |  |
| 1 | Chenab Flood Bund RD:100-tail | 221 | 221 | 0 |  |
| 2 | Nawabpur Flood Bund | 103 | 103 | 0 |  |
| 3 | Akbar Flood Bund | 7 | 7 | 0 |  |
| 4 | Sher Shah Flood Bund | 42 | 38 | 4 |  |
| 5 | 1st Defence Shujabad Branch | 44 | 44 | 0 |  |
|  | TOTAL | 417 | 413 | 04 |  |
|  | TEHSIL SHUJABAD & DISTRICT MULTAN |  |  |  |  |
| 6 | Gajju Hatta Flood Bund | 18 | 18 | 0 |  |
| 7 | Nagni Flood Bund | 99 | 99 | 0 |  |
| 8 | Dhundu Flood Bund | 124 | 124 | 0 |  |
| 9 | Gardez Flood Bund | 30 | 30 | 0 |  |
| 10 | Jalalpur Khakhi Flood Bund | 112 | 112 | 0 |  |
|  | TOTAL | 383 | 383 | 0 |  |
|  | TEHSIL JALALPUR PIR WALA & DISTRICT MULTAN |  |  |  |  |
| 10 | Jalalpur Khakhi Flood Bund | 112 | 112 | 0 |  |
| 11 | Balochwah Flood Bund | 78 | 78 | 0 |  |
| 12 | Sat Burji Flood Bund | 38 | 38 | 0 |  |
| 13 | Tie Flood Bund | 57 | 56 | 1 |  |
|  | TOTAL | 285 | 284 | 1 |  |
|  | G-TOTAL | 1198 | 1172 | 26 |  |

| Sr. No. | Name of Division | Type / Name of Encroachment | Location | Name of encroacher | Status Whether Removed or not |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Category |  | Status of Encroachments |  |  |
|  |  |  |  |  | Commercial | Residential | Removed | Not Removed |  |
|  |  | FAZIL SHAH FLOOD BUND |  |  |  |  |  |  |  |
| 1 | Shujabad Canal Division | House | RD.1-2 | Muhamamd Iqbal S/o Ahmed Nawaz | - | Residential | - | To be Removed | Removal of encroachments is being under process. |
| 2 |  | House | RD.3-4 | Ghulam S/o Qasim | - | Residential | - | To be Removed | " |
| 3 |  | House | RD.6-7 | Iqbal S/o Shafi | - | Residential | - | To be Removed | " |
| 4 |  | House | RD.8-9 | bakir S/o Ziarat | - | Residential | - | To be Removed | " |
| 5 |  | House | RD.9-10 | Hayat S/o Sher | - | Residential | - | To be Removed | " |
|  |  | CHENAB FLOOD BUND |  |  |  |  |  |  |  |
| 1 |  | Shop | RD.0-1 | Haq Nawaz S/o Ghulam Muhammad | - | Residential | - | To be Removed | " |
| 2 |  | House | RD.3-4 | Fazil S/o ALLAH Bakhsh | - | Residential | - | To be Removed | " |
| 3 |  | House | RD.4-5 | Falak Sher S/o Nawaz | - | Residential | - | To be Removed | " |
| 4 |  | House | RD.5-6 | Jamshaid S/o Khuda Bakhsh | - | Residential | - | To be Removed | " |
| 5 |  | House | RD.10-11 | Haq Nawaz S/o Sardar | - | Residential | - | To be Removed | " |
| 6 |  | House | RD.11-12 | Noor S/o Ramzan | - | Residential | - | To be Removed | " |
| 7 |  | House | RD.12-13 | ALLAH Ditta S/o Rajab | - | Residential | - | To be Removed | " |
| 8 |  | House | RD.13-14 | Haji S/o Muhammad Nawaz | - | Residential | - | To be Removed | " |
| 9 |  | House | RD.15-16 | Sarfaraz S/o Slabat | - | Residential | - | To be Removed | " |
| 10 |  | House | RD.16-17 | Ghulam Shabir S/o Salabat | - | Residential | - | To be Removed | " |
| 11 |  | House | RD.19-20 | Ghulam S/o ALLAH Wadhaya | - | Residential | - | To be Removed | " |
| 12 |  | House | RD.20-21 | Riaz S/o Haq Nawaz | - | Residential | - | To be Removed | " |
| 13 |  | House | RD.24-25 | Haq Nawaz S/o Ghulam | - | Residential | - | To be Removed | " |
| 14 |  | House | RD.26-27 | Mukhtiar S/o Shameer | - | Residential | - | To be Removed | " |
| 15 |  | House | RD.27-28 | Zafar S/o ALLAH Ditta | - | Residential | - | To be Removed | " |
| 16 |  | House | RD.28-29 | Nazar S/o Ameer | - | Residential | - | To be Removed | " |
|  |  | SHER SHAH FLOOD BUND |  |  |  |  |  |  |  |
| 1 |  | 2 Houses | RD.2-3/L | Manzoor S/o Ghulam Muhammad | - | Residential | - | To be Removed | " |
| 2 |  | 1 House Kacha | RD.2-3/L | Nazim S/o Muhamamd Shareef | - | Residential | - | To be Removed | " |
| 3 |  | 3 Shops 1 House | RD.4-5/L | Shoukat Shah S/o Yazdan Shah | - | Residential | - | To be Removed | " |
| 4 |  | Boundary Wall | RD.4-5/L | Fazal Ahmed S/o Muhammad Bakhsh | - | Residential | - | To be Removed | " |
|  |  | TIE FLOOD BUND |  |  |  |  |  |  |  |
| 1 |  | House | RD.1-2/R | Haji Mulazim S/o Khuda Bakhsh | - | Residential | - | To be Removed | " |

B- CURRENT STATUS OF ENCROACHMENTS

| Sr.No. | Total Encroachment | Encroachment Removed | Balance Encroachment | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 1198 | 1172 | 26 | Removal of encroachments is under process. |

Note:	List of Encroachments have already been reported to Civil Administration for vacation of unauthorized encroachments vide this office letter No.821-22/7-L dated 04.04.2020, No. 389-90/7-L dated 04.03.2021,  No.1464-65/7-L dated 19.03.2023, No.234-35/7-L dated 28.02.2024 & No.221-22/7-L dated 13.02.2025 Civil Department has vigorously approached for removal of encroachments.

Note:- There is no critical encroachment in Shujabad Canal Division.

CHAPTER -9

DUTY ROSTER / FLOOD FIGHTING PROGRAM

All Sub Engineers incharge of flood bunds will setup their flood emergency headquarter at assigned points on flood bunds as given in duty Roster(Appendix IV) and will stay there from Ist of July or when the river rises to flood discharge whichever occurs earlier till the end of flood season. They will be responsible for watching operation in low medium and high flood discharges. When a very high flood is expected, the Sub Engineer, and other staff deputed from other Divisions will also move for help according to the programme detailed in the following paragraphs where in each bund has been treated individually. As the canals are generally closed during high floods, all Sub Engineers deputed for watching duties will also bring their sectional establishment with them unless directed otherwise.

Zilladar and other revenue establishment will help in perusing labour and material as required by Engineering staff and will also participate in active watching whenever called upon to do so. They will however remain available at short notice throughout the flood season by keeping their movements known to their Sub Divisional Officers and Deputy Collector.

The Sub Divisional Officer in charge and other those who are deputed will shift to their assigned flood emergency headquarters at bunds when warning of a high flood is received and will supervise the watching arrangement till subsidence of high floods.

A duty roster showing the assigned duties of different staff during very high flood period appears on page No.43-47.

FAZAL SHAH FLOOD BUND.

When a very high flood is anticipated Sub Engineer Sahu Section of Multan Canals Division will move to RD.4000 of Fazal Shah Flood Bund to supervise the watching of the bund.

CHENAB FLOOD BUND.

The Chenab Flood Bund being 32 miles long will be divided into seven reaches with a Sub Engineer incharge of watching arrangement in the reach of his charge.

These  reaches are:-

Sub Engineer, Sardarpur		RD.0+000-38+000 with H.Q.	  	at RD.28+000

Sub Engineer, Kukarhata Hatta	RD.38+000-60+000 with H.Q.	at RD.57+000

Sub Engineer, Rashida		RD.60+000-80+000 with H.Q.	at RD.78+000

Sub Engineer, Binda   		RD.80000-94+000 with H.Q.	 	at RD.100+000

Sub Engineer, Bosan   		RD.100+000-135+000 with H.Q.	at RD.117+000

Sub Engineer, Chawan		RD.135+000-150+000 with H.Q. 	at RD.145+000

Sub Engineer, Nawabpur		RD.150+000-157+950 with H.Q.	at RD. Chenab F/B

Sub Engineer, Nawabpur F/B		RD.0+000-Tail with H.Q.		at RD.157+950

AKBAR FLOOD BUND.

Sub Engineer river survey with H.Q at RD.6+000 Akbar flood bund.

Sub Engineer Mubarik Pur right bank of Shujabad Branch RD.210+000-245+000 at RD.231+000 Sub

Engineer Muzaffarabad RD.245+000-279+000 with H.Q at RD.252+000 Sub Engineer Binda and Rashida will be located at RD.94+000 OF Chenab flood bund RD.152+000 of Chenab flood bund and RD.4+000 of Fazal Shah flood bund respectively.

NAGNI FLOOD BUND AND RIGHT BANK OF GAJJU HATT BRANCH RD.17+000-35+000

Sub Engineer Qasba will supervise RD.17+000 to 35+000 of Gajju Hatta Branch and 0+000 to 33+000 Nagni Flood Bund with H.Q RD.20+000 of Nagni Flood Bund.

Sub Engineer Gajju Hatta will supervise RD.33+000 to 51+000 of Nagni Flood Bund with H.Q at RD.42+200 of Nagni Flood Bund.

DHUNDOO FLOOD BUND.

Sub Engineer Shujabad will be incharge of these bunds and will keep his headquarter at RD.15+000 of this bund.

JALALPUR KHAKI FLOOD BUND.

Sub Engineer Rawan will supervise RD.0+000-60+000 Tail with H.Q.at RD.45+000.

BALOACH WAH FLOOD BUND RD.0+000 TO 20+000 AND CONNECTING BUND ON RIGHT BANK OF PANJANI DISTY RD.131+000 TO 139+000.

Sub Engineer Panjani will be incharge of these bunds during high flood and will keep his H.Q at RD.17+000 of Baloach wah flood bund & Sub Engineer Sikandarabad will be the incharge of Baloachwah Flood Bund RD.20+000 to Tail 62+230 keeping his H.Q at RD.34+000.

Sub Divisional Officer Shujabad Sub Division of Shujabad Division will be incharge of watching arrangement on Nagni and Dhundoo Flood Bund while Sub Divisional Officer Multan will supervise Jalalpur Khaki Flood Bund connecting bund and Baloachwah Flood Bund and both these officers will assist Executive Engineer Multan who will be staying at Shujabad and will act as overall Flood Control Officer of Shujabad Sub Division .

Officers from revenue and police department will be assigned duties on flood bunds. They will help in procurement of labour, materials and transport etc, and maintenance of law and order together with security/counter sabotage arrangements, as and when such needs arise. After flood, they will make relief measures with the help of Army.

The reaches assigned to them as given in Deputy Commissioner Multan letter No.766-H.C/FR dated 28-6-78 have been shown in the duty Roster for regular staff.

GENERAL.

During high floods, storage of rations (food stuffs) at bunds, machinery spares and P.O.L item are commonly required as communication links are severed by flood spills. It will be advisable to collect sufficient quantities of such items before high flood arrives, so as to avoid inconvenience and frustration at the time of passage of floods.

This scheme touches all important points. However, if any item of actions are necessitated due to site conditions, the officer incharge will find out some solution.

During exceptionally high floods, army help is also normally available, and officers incharge should not hesitate in asking for it whenever it becomes necessary. This help can be in the form of men, boats, dozers, demolishing material (for making relief cuts) and relief work through helicopters. According to latest decision of the Government, the Army Engineers and other branches will associate themselves with flood watching from the beginning of the flood season. The civil authorities will also depute Revenue Officers for each bund to help in watching and procurement of labour and material during emergency.

SHUJABAD CANAL DIVISION MULTAN.

DUTY ROSTER FOR REGULAR STAFF OF FLOOD BUND IN SHUJABAD CANALS DIVISION MULTAN DURING HIGH FLOODS.

| Sr. No. | Name of Officer Incharge & His Head Quarter | Name of Bund | Sub Engineer Incharge | Reach & Head or | Revenue Officer On Duty | Police Officer On Duty | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | S.D.O Rashida Sub Division Multan | Fazal Shah F/B RD.0+000-Tail | Sub Engineer Sahu Sec: | RD.0+000-10+500 H.Q.@ RD.4+000 | Tehsil Municipal officer Kabirwala | SDPO Kabirwala |  |
|  | H.Q @ RD.55000 Chenab F/B | Chenab F/B RD.0+000-79+000 | Sub Engineer Sardarpur | RD.0+000-38+000 H.Q.RD.28+000 of C.F.B | -do- | -do- |  |
| 2 | S.D.O Binda H.Q. RD.94000 Chenab F/B | Chenab F/B RD.79+000-141+000 | Sub Engineer Kukarhatta | RD.38-60 H.Q.@ RD.57 | -do- | -do- |  |
|  |  |  | Sub Engineer Rashida | RD.60+000-80+000 H.Q RD.78+000 |  |  |  |
|  |  |  | Sub Engineer Binda | RD.80+000-100+000 H.Q@ RD.94+000 | Tehsil Municipal officer Sadar/ Multan | SDPO Multan |  |
|  |  |  | Sub Engineer Bosan | RD.100+000-135+000 H.Q.@ RD.117+000 | -do- | -do- |  |
| 3 | S.D.O Ilampur H.Q. RD.152+000 Chenab F/B | Chenab F/B RD. 141+000-157+950 | Sub Engineer Chawen | RD.135+000-150+000 H.Q.@ RD.145+000 | -do- | -do- |  |
|  |  | Escape channel spur RD.0+920 Nawabpur F/B | Sub Engineer Nawabpur | RD. 150+000-157+950 Chenab F/B RD.0+000-Tail H.Q.@ RD.157+950 Chenab F/B | -do- | -do- |  |
| 4 | S.D.O Tubewell Multan H.Q.Spur No.3-B | Nawabpur F/B & Akbar F/B | Sub Engineer River survey | RD.0-Tail Akbar F/B H.Q.@ RD.6+000 | Tehsil Municipal officer Multan | SDPO Sadar Mutlan |  |
|  |  | 2nd defence line | Sub Engineer Muzaffar abad | RD.245+000-279+000 H.Q. RD.252+000 Shujabd Branch | E.A.C Multan | -do- |  |
|  |  | -do- | Sub Engineer Mubarakpur | RD.210+000-245+000 H.Q.@ RD.231+000 Shujabad branch | -do- | -do- |  |
|  |  | Right bank of GHT branch and Nagni F/B | Sub Engineer Qasba | RD.17+000-35+000 Gajju Hatta branch RD.0+000-33+000 Nagni F/B H.Q.@ RD.20+000 | -do- | -do- |  |
| 5 | Shujabd Sub Division S.D.O Shujabad Sub Division H.Q.@ RD.45+000 Nagni F/B | Right bank of GHT branch and F/B Mole head groyne and j-head. | Sub Engineer Gajju Hatta Branch | RD.33+000-51+000 Nagni F/B H.Q. RD.42+200 Nagni F/B. | Tehsildar Shujabad | SHO Shujabad |  |
|  |  | Dhundoo and Gardez F/B | SEE: Shujabad | Dhundoo and Gardez F/B H.Q RD.15+000 Dhundoo F/B | -do- | -do- |  |
|  | S.D.O Multan H.Q.@ RD.45+000 Jalalpur Khaki F/B | Jalalpur Khaki F/B | Sub Engineer Rawan | RD.0-60+000 Jalalpur Khaki H.Q. RD.45+000 | -do- | SHO Jalalpur Pirwala |  |
|  |  | Balochwah F/B | SEE:SKD | RD. 2062+230 Tail Baloachwah F/B H.Q. RD.17 | -do- | -do- |  |
|  |  | Balochwah F/Band right bank of Panjani Panjani disty | RD.0+000-20+000 Bloachwah RD.131+000-139+000 Panjani Disty H.Q. RD.10+000 Bloachwah F/B | S.B.E: Panjani | -do- | -do- |  |

##### NOTE:-

When the total discharge D/S Trimmu and Sidhnai Barrage exceeds 4.5 lac Cs. The Executive Engineer Multan Division will shift to Shujabad Canal Rest House and control all flood fighting operations of Shujabad Sub Division; as Executive Engineer Shujabad will be busy on other important flood defence related activities / arrangement in the vicinity of Multan City and keeping contact with seniors and other Department.

CHAPTER -10

EMERGENCY TELEPHONE NUMBERS

| Sr. No | Designation of Officer | H/Q | Office Phone | Residence / Mobile Phone | Addressed |
| --- | --- | --- | --- | --- | --- |
|  | Executive Engineer Shujabad Canal Division Multan Abdali Road Multan. | Multan | 061-9200699 | 0300-7323748 | Residence No.14 in officer canal colony old Bwp road, Multan. |
|  | Sub Divisional Officer Ilampur Sub Division Multan. | Multan |  | 0300-6387148 | Residence No.1 in officer canal colony old Bwp road, Multan. |
|  | Sub Divisional Officer Binda Canal Sub Division Binda. | Binda | - | 0301-8683610 | Residence No.12 in officer canal colony old Bwp road, Multan. |
|  | Sub Divisional Officer Shujabad Canal Sub Division Shujabad. | Shujabad | 061-4398434 | 0301-8683610 | Canal Colony, Shujabad. |
|  | Executive Engineer Operation Multan Irrigation Zone Multan. | Multan | 061-9200139 Fax061-9200133 | 0304-7032344 | Residence No.09 in officer canal colony, old BWP road Multan |
|  | Chief Engineer Irrigation Multan Zone Multan. | Multan | 061-9200144 | 0303-7713303 | Residence No.04 in officer canal colony old Bwp road, Multan |
|  | Superintending Engineer Haveli Canal Circle Multan. | Multan | 061-9200136 | 0333-7638416 0300-1238418 | Residence No.03 in officer canal colony old Bwp road, Multan |
|  | Deputy Commissioner | Multan | 061-200042/43 Fax061-200725 | 061-9200045 | District Court Multan. |
|  | Deputy Commissioner | Khanewal | 065- 9200032 Fax065-200033 | 065-9200031 | District Courts Khanewal |
|  | Flood center 531-2353- Major Qazi Saleem | Multan | 061-5312353 | 061-5312354 | Multan. |
|  | H/Q Engineer 2-Crops Multan Cantt Multan. | Multan | 061-5312360 | 061-5312362 | Multan. |
|  | 313 Assault Engineer Battalion Multan Cantt. | Multan | 061-5315188 | 061-5315189 | Multan. |
|  | Haveli Telegraph Office/Emergency flood center. | Multan | 061-9200922 | - | Old BWP road Multan near Chief Engineer Office. |
|  | Town Administrator | Kabirwala | 065-2411192 | 065-585500 | Municipal Committee Kabirwala |
|  | Town Administrator | Bosan Town | 061-9201030 | 061-4595111 | Near M.D.A. Chwok Multan |
|  | Town Administrator | Sher Shah Town | 061-9200344 | - | Near M.D.A. Chwok Multan |
|  | Town Administrator | Musa Pak Town | 061-9201448 | - | Mumtazabad Multan |
|  | Town Administrator | Shah Rukne Alam Town | 061-9200345 | - | Near M.D.A. Chwok Multan |
|  | Tehsil Municipal officer | Kabirwala | 065-2411587 | - | Municipal Committee Kabirwala |
|  | Tehsil Municipal officer | Khanewal | 065-2556001 | - | Municipal Committee Khanewal |
|  | Tehsil Municipal officer | Shujabad | 061-4396670/ 396715 | - | Municipal Committee Shujabad |
|  | Tehsil Municipal officer | Jalal pur Pirwala | 061-4210334 | - | Municipal Committee Jalalpur Pirwala |
|  | Tehsil Municipal officer | Sher Shah Town | 061-9200777 | - | Near M.D.A. Chwok Multan |
|  | Tehsil Municipal officer | Musa Pak Town | 061-9201446 | - | Mumtazabad Multan |
|  | Tehsil Municipal officer | Bosan Town | 061-9200056 | - | Near M.D.A. Chwok Multan |
|  | Tehsil Municipal officer | Shah Rukne Alam Town | 061-9200685 | - | Near M.D.A. Chwok Multan |
|  | District Police officer | Multan | 061-9200437 | - | Near S.P. Chowk Multan |
|  | District Police officer | Khanewal | 065-9200175 | - | District Court, Khanewal |
|  | Deputy Superintendent Police | Shujabad | 0614-396503 | - | Thana Chowk Shujabad |
|  | Deputy Superintendent Police | Jalalpur Pirwala | 0614-210235 0614-210020 | - | Khan Bela Road, Jalalpur Pirwala |
|  | Chief Engineer Drainage & Flood Zone, Lahore | Flood Emergency cell | 0429-9230602 Fax042-35862081 | 0309-9970999 | Government Engineering Academy Thokhar Niaz Baig Lahore. |
|  | General Staff Officer Grade-2 (Operation & Planning) Lahore Cantt. | Lahore | 042-66992360 | - | - |
|  | Rescue Operation (Internal Security) (Flood Relief Operation) Responsibility 5-Brigade 51-Lancers Duty Clerk | Shujabad Jalalpur Pir Wala | NR 531-5120 NR 531-4304 NC 531.4306 | 531.4305 | - |
|  | Additional Deputy Commissioner (Revenue) | Khanewal. | 065-9200069 | - | - |
|  | Assistant Commissioner | Khanewal | 065-9200242 | - | - |
|  | Assistant Commissioner | Kabirwala | 065-2410611 | - | Tehsil Office, Kabirwala |
|  | Additional Deputy Commissioner Revenue, | Multan | 061-9200073 | - | District Court, Multan |
|  | Additional Deputy Commissioner / Revenue, City Multan. | Multan | 061-9200089 | - | District Court, Multan |
|  | Additional Deputy Commissioner ( Revenue) Sadar Multan | Mutltan | 061-9200784 | - | District Court, Multan |
|  | Assistant Commissioner | Jalalpur Pir Wala | 0614-210232 | - | Tehsil Office, Jalalpur Pirwala |
|  | Assistant Commissioner | Shujabad | 061-4396480 | - | Thana Chowk Shujabad |
|  | Flood Section Irrigation Secretariat, Lahore | Lahore | 0429-9212134 0429-6857141 Fax 0429-9012132 | - | Old Anarkali, Lahore |
|  | Incharge Police Wireless Set Multan. Police Control Call Sign | Multan | 15 | - | Police Line, Multan |
|  | Forecasting Deptt: |  | 061-9200763 531-2016 | - | Cant: Multan |
|  | Radio Pakistan Security Incharge |  | 061-9200795 061-9200221 | - | Near S.P. Chowk, Multan |
|  | PTV. News |  | 061-9220227 | - | Old Shujabad Road, Multan |
|  | Executive Engineer, WAPDA, Cantt. |  | 061-551226 061-9200520 | - | Sher Shah Road, Multan |
|  | Executive Engineer, WAPDA, City. |  | 061-553348 061-9200518 | 065-932-552065 | Khanewal Road, Multan |
|  | Chief Engineer D&F Lahore. |  | 042-99230602 Fax 042-35862081 | 0309-9970999 | Government Engineering Academy Thokar Niaz Baig Lahore. |
|  | Director Flood & Flood warning Centre |  | 042-99231614 | 0334-4669308 |  |
|  | Major 51- Lancers Multan Cantt incharge Flood Monitoring Unit. |  | 061-531-5120 | - | Multan Cantt: |
|  | Major 53-Cavellary Multan. |  | 061-531-5112 | - | Multan Cantt: |

CHAPTER -11

STANDARD OPERATING PROCEDURE (SOP) FOR

BREACHING SECTIONS

  - History of the Breaching section

There are two breaching sections:

i-	Railway Flood Bund near Sher Shah Bridge which is being maintained by Irrigation Department.

ii-	Head Muhammad Wala approach road maintained by National Highway Authority.

SHER SHAH RAILWAY BREACHING SECTION

The Breaching Section of Railway Flood Bund is exist between RD:11000 to RD:12000 with maximum discharge capacity of 55000 Cs. The Breaching Section has 4 No. of sets. The length of each set is 100ft and width 30ft. The spacing between two consecutive sets is 100ft. Making total length of Breaching Section as 700ft. There are 11 Rows of liners in each sets. The depth of each liner is 11ft and dia 6 inches. The total No of liners in 4 No. set of Breaching Section is 55*4=220Nos. The Breaching Section is operated on attaining flood level 393.50 ft, The gauge for operating of Breaching Section of right pier of Railway Sher Shah Bridge.  The Breaching Section is operated on 12/9/2014 with discharge 55000cs.

The Designed detail of set of Breaching Section is as under between RD:11000 to RD:12000 on Sher Shah Railway Flood bund.

The Statistic data of Breaching Section is as under:-

Upon operation of breaching Section, the following village/abadies will be affected.

1.	Gagra kachoor				2.    Mirza Pur

3. Jambhour  					4.     Khan pur Qazi

Plan showing route of flood water coming out of the breach supported with levels

PLAN ATTACHED

BREACHING SECTION AT SHER SHAH RAILWAY FLOOD BUND BETWEEN RD.11+000 TO 12+000

RD.11+000-12+000

  - Location ,Design ,Quantity, and Variety of the explosive required for detonation

STATUS OF EXPLOSIVE /ACCESSORIES

| Sr.No. | Name of Explosive | A/U | Quantity Required | Quantity Available | Deficient Quantity | Outstanding Dues if any |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Wabox / Eulite (PE-3A) | Kgs | 14740 | 14740 | - | - |
| 2 | Wabocord | M | 1000 | 1000 | - | - |
| 3 | Safety Fuze No.11 | “ | 150 | 250 | - | - |
| 4 | Det No.8,27,&33 (Elec/ Non Elect/Plain) | Nos | 60 | 60 | - | - |
| 5 | Det No.8 Elec | Nos | 23 | 23 | - | - |
| 6 | Elect Det No.6 (MS 25 Delay) / Det 25 Micro Sec | “ | 16 | 16 | - | - |
| 7 | Det MS Elect Delay | “ | 6 | 6 | - | - |
| 8 | Firing Cable | M | 1000 | 1000 | - | - |
| 9 | Primer CE | Nos | 500 | 560 | - | - |

Pak Army is responsible for explosive material.

11.3	Arrangement of explosives and security of explosive stores

Kept under safe custody of Pakistan Army.

  - List of Security staff alongwith detail of their training etc.

Security staff done through Pakistan Army.

  - Detail of mechanical means as a standby arrangements in case of detonation failure

Machinery including excavator 3 Nos. dozer 1 No. would be placed at site of Breaching Section along with 15 No. of establishment during operation of Breaching Section.

11.6	Duty Roster in case of critical situation

As above chapter No.9

11.7	 Breaching committee with their action plan

(A) BREACHING OPERATION COMMITTEE. (Sher Shah Breaching Section)

Breaching section shall be operated on the orders of the following committee constituted by Government of the Punjab Irrigation Department vide Notification No.SO(FLOODS) VI-33-97 dated 15.07.2015.

- Deputy Commissioner , Multan 						= Convener

- Deputy Commissioner, Muzaffar Garh					= Member

- Executive Engineer, Shujabad Canal Division, Multan			= Member

- Executive Engineer, Highway Department (C&W), Multan			= Member

- Representative from Pak Army not below the rank of a Lt. Colonol		= Member

11.8	List of the villages likely to be inundated in case of 	breach.

i)	VILLAGES LIKELY TO BE IN-UNDATED DURING FLOOD.

| S.NO. | DISTRICT MULTAN. | S.NO. | DISTRICT KHANEWAL. |
| --- | --- | --- | --- |
| 1 | Nawabpur | 1 | Binda Sargna. |
| 2 | Bakir Abad | 2 | Chiragh Bela |
| 3 | Jalalabad. | 3 | Choki Mohan |
| 4 | Rana Wahin | 4 | Salar Wahin Nau |
| 5 | Akbar Pur | 5 | Chak Mani |
| 6 | Jalalpur Khakhi | 6 | Chiragh Wali |

ii)	VILLAGES LIKELY TO BE IN-UNDATED DURING VERY HIGH FLOOD.

| S.NO. | DISTRICT MULTAN. | S.NO. | DISTRICT KHANEWAL. |
| --- | --- | --- | --- |
| 1 | Khor Sajra Wahin | 1 | Gagra Qalandar |
| 2 | Bargain | 2 | Ahar Bela |
| 3 | Bait Qaiser | 3 | Binda Sargana. |
| 4 | Karmon Wali. | 4 | Chiragh Wali. |
| 5 | Almadi sura | 5 | Salar Wahin Kona |
| 6 | Jhangi | 6 | Jamal-ke-charbi |
| 7 | Wacha Sandila | 7 | Mumdal Gharbi |
| 8 | Lakshmi Narain | 8 | Muhammad Pur Nashaib. |

iii)	VILLAGES LIKELY TO BE IN-UNDATED DURING EXCEPTIONALLY HIGH FLOOD.

| S.NO. | DISTRICT MULTAN. | S.NO. | DISTRICT KHANEWAL. |
| --- | --- | --- | --- |
| 1 | Nawabpur | 1 | Binda Sargana |
| 2 | Al Madi Sura | 2 | Chiragh Bela |
| 3 | Akbar Pur | 3 | Salar Wahin |
| 4 | Khanpur | 4 | Jamal-ke-Garbi |
| 5 | Sohman | 5 | Chiragh Wali |
| 6 | Wains | 6 | Muhammad Pur Nashaib |
| 7 | Choti Sharif | 7 | Salar Wahin Nau |
| 8 | Baqir Abad. | 8 | Ahar Bela |
| 9 | Lutaf Abad. | 9 | Mumdal Gharbi |
| 10 | Binda Sandila |  |  |
| 11 | Lakshmi Narain |  |  |
| 12 | Chak Mani |  |  |
| 13 | Mochi Pura |  |  |
| 14 | Mahra |  |  |
| 15 | Agar Afani. |  |  |
| 16 | Tahir Pur |  |  |
| 17 | Rana Wahin. |  |  |

LIST OF VILLAGES TO BE MAROONED AS A CONSEQUENCE OF ACTIVATION OF BREACHING SECTION.

| S.NO. | DISTRICT MULTAN. |
| --- | --- |
| 1 | Sultan Pur Hammar |
| 2 | Kachoor |
| 3 | Jai |
| 4 | Gagra |
| 5 | Khan Pur Qazi. |

11.9		Announcement and detail of evacuation arrangements

Deputy Commissioner, Multan performs the announcement and he is responsible for evacuation arrangements.

11.10	Details of coordination with Civil/Army authorities.

During flood, several Civil Departments and agencies as well as Army authorities Coordinate with Irrigation Department in the jurisdiction of Shujabad Canal Division. Pre-Flood Joint visits of flood infrastructures are arranged with Army authorities. To Combat with flood situations, meeting are arranged with the following authorities.

- The Divisional Superintendent Railway, Multan.

- The Chief Engineer, Irrigation Multan Zone, Multan.

- The Deputy Commissioner, Multan.

- The Chief Engineer, Irrigation D.G.Khan Zone, D.G.Khan.

- The Deputy Commissioner, Muzaffargarh.

- Representative of WAPDA.

- Superintending Engineer, Highway Multan.

- Representative of National Highway Authority.

- The Commander, Engineer 2-Crops, Multan.

- The Executive District Officer, Health Multan.

(B)  BREACHING OPERATION COMMITTEE.(Head Muhammad Wala Breaching Section)

- Deputy Commissioner Representative of DC Multan			= Convener

- CO 313 Assault Engineering Battalion Multan				= Member

- Deputy Director (Maintenance) NHA Multan				= Member

- Executive Engineer, Shujabad Canal Division, Multan			= Member

- Executive Engineer, Highway C&W Department Multan			= Member

11.11	Parallel communication arrangements

In Shujabad Canal Division communication arrangement about flood run parallel at different forums / departments.  Pakistan Meteorological Department Displays the forecast flood situation on its website. Punjab Disaster Management Authority, (PDMA) warns about flood in anticipation. Signaling system of Irrigation Department Army authorities, Police Department as well as print and electronic media play important role in commination arrangements.

Executive Engineer,

Shujabad Canal Division,

Multan

Superintending Engineer,

Haveli Canal Circle,

Multan.

11.12. Plan of Breaching Section

The Designed detail of set of Breaching Section is as under between RD:11000 to RD:12000 on 	Sher Shah Railway Flood bund.

The Statistic data of Breaching Section is as under:-

Upon operation of breaching Section, the following village/abadies will be affected.

1.	Gagra kachoor				2.    Mirza Pur

3. Jambhour  					4.     Khan pur Qazi

Plan showing route of flood water coming out of the breach supported with levels

PLAN ATTACHED

BREACHING SECTION AT SHER SHAH RAILWAY FLOOD BUND BETWEEN RD.11+000 TO 12+000

RD.11+000-12+000

CHAPTER-12

Vulnerable sites on flood Bund /Structures:

12.1 Apprehended breaches in flood bunds/structures:

Following Flood Bunds at following reaches can be vulnerable.

| Sr. No. | Description | Vulnerable Reach |
| --- | --- | --- |
| 1 | Akbar Flood Bund | RD. 6+000-11+000 |
| 2 | Sher Shah Railway Flood Bund | RD.11+000-12+000 |

12.2 Operation of Breaching sections:

The Breaching section of  Head Muhammad Wala was approved vide Secretary Irrigation Department letter No.SO(Flood)vi-33/97 dated 15.07.2015 on left approaching road of the bridge in front of the deep river creek plugged by the road embankment when the Critical gauge achieved as  RL 417.50.

| Year | Head Muhammad Wala | Through Relief Cut | Sher Shah Railway | Through Relief Cut |
| --- | --- | --- | --- | --- |
| 1992 | - | - | 982027 | 205000 |
| 2014 | 626006 | 85000 | 626006 | 55000 |

12.3 Breaches due to rise of flood water, deterioration of flood bunds etc:

Sher Shah Railway Route plans attached at Page-61.

| Sr. No. | Name of Flood Bund | Reach |  | Page No. |
| --- | --- | --- | --- | --- |
|  |  | From RD | To RD |  |
| 1 | Sher Shah Railway flood bund | 11+000 | 12+000 | 61 |

CHAPTER-13

Emergency contingency Plan for vulnerable site (sites)

listed above. Emergency Contingency Plan Must include:-

Emergency contingency plan No.1

SITE NO.1

APPROACH ROAD OF MUHAMMAD WALA BRIDGE

13.1  Plan showing route of flood water coming out of  the breach supported with levels

PLAN ATTACHED

RD.9+000

RD.17+000

13.2  Detail of villages abbadeies  likely to be affected and this should also be  shown on the plan.

- Basti Mangana

- Purana Grey Wal

- Basti Mala karloo

- Basti Akbar pur

13.3	Strategy and action taken be explained in detail. This may include:

Special watching will be exercised during flood season and strengthening of the site will be ensured before coming flood.

    - Arrangement

Special watching of flood infrastructure will be exercised during flood season. Strengthening / Repairing of flood bunds will be made before flood. All flood restoration works will be completed well before flood. Flood camps will be established at Significant sites. Flood fighting material will be made ready to fight with the flood. Machinery will be made available through private sources. The reserve Stock of Stone to the minimum required limit is available at site. Any other arrangement to meet the urgency of flood situation.

13.3.2	Establishment of flood fighting camps

Detail of Flood fighting camp attached Chapter No.14 at page No.72-74.

13.3.3	Duties of officers/ officials and their camp site.

Detail of Flood fighting camp attached Chapter No.14.

| Sr. No. | Camp No. | Location | Officer name designation / Mobile No. | Official Name designation Mobile No. |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| 1 | Camp No.1 | RD:10+000 of Chenab Flood Bund | Muhammad Ahmad / Sub Divisional Officer/ 0301-8683610 | Iftikhar Ahmad/ Sub Engineer/ (0300-6383404) |
| 2 | Camp No.2 | RD:87+000 at Chenab Flood Bund | ----do---- | Rai Nasir Ali/ Sub Engineer/ (0306-7144126) |
| 3 | Camp No.3 | Chenab Flood Bund RD:151+800 | Rasheed Baig Sub Divisional Officer/ 03006387148 | Zafar Iqbal/ Sub Engineer/ (0301-6821818) |
|  |  | RD:11+350 Akbar flood bund | ----do---- | ----do---- |
| 4 | Camp No.4 | RD:252+000 Shujabad Branch | ----do---- | Rana Muhammad Iqbal Sub Engineer / (03007389494) |
|  |  | RD:6 to 7 Sher Shah Flood Bund | ----do---- | ----do---- |
|  |  | RD:19-20 Sher Shah Flood Bund & RD.11-12 Breaching Section of Sher Railway flood bund | ----do---- | ----do---- |
| 5 | Camp No.5 | RD.42200 Nagni Flood Bund | Rasheed Baig Sub Divisional Officer/ 03006387148 | Abdul Hameed Bhatti/ Sub Engineer Gajju Hatta/ (0300-7372539) |
| 6 | Camp No.6 | RD.21+000 of Dhundoo Flood Bund | ----do---- | Sajid Hussain Bhatti/ Sub Engineer Shujabad/ (0300-7818374) |
| 7 | Camp No.7 | RD.45 of Jalalpur Khakhi Flood Bund RD.23 Baloch Wah Flood Bund | ----do---- | Muhammad Sajid / Sub Engineer Panjani/ (0300-7362786) |

13.3.4	Departmental Machinery available.

1-	Placement of sand filling machine.

2-	Front Blade Tractor

3-	Tractor Trolley

13.3.5	Machinery available from private source

To be hired from Government Contractors.

Following Government contractors undertaking for provide Machinery in emergency during flood 2025.

1-	Al-Siddique Enterprise.

2-	Syed Nasir Hassan Shah

3-	Muhammad Iqbal

4-	Suleman Services.

5-	Fareed Builders and Restoration.

6-	Saddat Engineers.

7-	Ali Ali Engineers.

13.3.6   Flood fighting material Required /13.3.7  Flood fighting material Available

| Sr. No | Item | Unit | Total Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Gunny bags | No. | 520 | 520 | - | Balance quantity of watching Material would be purchased before the onset of flood 2025 subject to approval of competent authority. |
|  | Pump With generator (Petrol Engine 100 CC, Centrifugal Pump, Flexible Pipe Frame for shifting) | No. | 1 | 0 | 1 |  |
|  | Kassis with handles | No. | 100 | 100 | - |  |
|  | Chain Saw | No. | 1 | 0 | 1 |  |
|  | Re-chargeable Torch | No | 2 | 0 | 2 |  |
|  | Wire crate | Kg | 100 | 100 | - |  |
|  | Manila Rope ½`` to 1`` | Kg | 150 | 150 | - |  |
|  | Lar Mal | No. | 490 | 490 | - |  |
|  | Water Cooler | No | 2 | 2 | - |  |
|  | Water Set | No | 1 | 1 | - |  |
|  | Chair set (one for each camp) | Set | 2 | 2 | - |  |
|  | Steel Wire 10 No. | Kg | 50 | 50 | - |  |
|  | Bamboos 8` to 12` | No | 200 | 200 | - |  |
|  | Plastic sheet | Rft | 100 | 100 |  |  |
|  | Plastic sand bags with plastic strips | No. | 100 | 100 | - |  |
|  | Charpai (4 No. for each camp) | No. | 4 | 4 | - |  |
|  | D.C Fan Pedestal | No. | 2 | 0 | 2 | Balance quantity of watching Material would be purchased before the onset of flood 2025 subject to approval of competent authority. |
|  | Bulb, light Holder | No. | 12 | 12 | - |  |
|  | Generator | No. | 1 | 0 | 1 |  |
|  | Mat Plastic | No. | 1 | 0 | 1 |  |
|  | Crockery | Set | 1 | 0 | 1 |  |

Note:-

It is certified that any other required material will be purchased before start of flood season 2025 subject to approval of competent authority.

13.4.  Detail of other information like electric sui gas telephone installation, road network, other building , canal and drainage network.

- Approach Road to Bridge Muhammad Wala.

- Electric Supply Lines.

- Sui gas = Nil.

CHAPTER-13

Emergency contingency Plan for vulnerable sites.

Emergency contingency plan No.2

SITE NO.2

RD. 11+000 - 12000 OF SHER SHAH RAILWAY FLOOD BUND

13.1  Plan showing route of flood water coming out of  the breach supported with levels

PLAN ATTACHED

BREACHING SECTION AT SHER SHAH RAILWAY FLOOD BUND BETWEEN RD.11+000 TO 12+000

RD.11+000-12+000

13.2  Detail of villages abbadeis  likely to be affected and this should also be  shown on the plan.

1. Gagra kachoor

2. Mirza Pur

3. Jambhour

4. Khan pur Qazi

13.3	Strategy and action taken be explained in detail. This may include:

The following arrangements /strategy will be adopted.

13.3.1     Arrangement

Special watching of flood infrastructure will be exercised during flood season. Strengthening / Repairing of flood bunds will be made before flood. All flood restoration works will be completed well before flood. Flood camps will be established at Significant sites. Flood fighting material will be made ready to fight with the flood. Machinery will be made available through private sources. The reserve Stock of Stone to the minimum required limit is available at site. Any other arrangement to meet the urgency of flood situation.

13.3.2	Establishment of flood fighting camps

Flood fighting camps would be placed at designated vulnerable sites.

13.3.3	Duties of officers/officials and their camp sites

Sub Engineer			=	01 No.	 Incharge

Beldar				=	32 -50Nos 	(Depend on relevancy)

Chowkidar				=	04 Nos.	(in 2 Shifts)

13.3.4	Departmental Machinery available.

1-	Placement of sand filling machine.

2-	Front Blade Tractor

3-	Tractor Trolley

13.3.5	Machinery available from private source

To be hired from Government Contractors.

Following Government contractors undertaking for provide Machinery in emergency during flood 2025.

1-	Al-Siddique Enterprise.

2-	Syed Nasir Hassan Shah

3-	Muhammad Iqbal

4-	Suleman Services.

13.3.6    Flood fighting material required /13.3.7  Flood fighting material Available

| Sr. No | Item | Unit | Total Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Gunny bags | No. | 520 | 520 | - | Balance quantity of watching Material would be purchased before the onset of flood 2025 subject to approval of competent authority. |
|  | Pump With generator (Petrol Engine 100 CC, Centrifugal Pump, Flexible Pipe Frame for shifting) | No. | 1 | 0 | 1 |  |
|  | Kassis with handles | No. | 100 | 100 | - |  |
|  | Chain saw | No. | 1 | 0 | 1 |  |
|  | Re-charge able torch | No | 2 | 0 | 2 |  |
|  | Plastic sheet | Rft | 100 | 0 | 100 |  |
|  | Wire crate | Kg. | 200 | 0 | 200 |  |
|  | Lar Mal | No. | 490 | 490 | - |  |
|  | Water Cooler | No. | 1 | 1 | - |  |
|  | Water Set | No. | 1 | 1 | - |  |
|  | Manila Rope ½” To 1” | Kg | 150 | 150 | - |  |
|  | Chair set | set | 2 | 0 | 2 | Balance quantity of watching Material would be purchased before the onset of flood 2025 subject to approval of competent authority. |
|  | Bamboo | No. | 200 | 200 | - |  |
|  | Charpai (4 No. for each camp) | No. | 4 | 4 | - |  |

Note: -

It is certified that any other required material will be purchased before start of flood season 2025 subject to approval of competent authority.

13.4.  Detail of other information like electric sui gas telephone installation, road network, other building , canal and drainage network.

1.        Approach Multan to Muzaffargarh road

2.        Electric transmission line.

3.        Pipe line of Sui gas = Nil

CHAPTER-14

Action Plan.

14.1  Re-shuffling/ Recouping plan of reserve stone departmentally

STATUS OF RESERVE STOCK OF STONE IN THE VULNERABLE LOCATION/FLOOD PROTECTION STRUCTURE

| Sr. No. | Name of Intervention | Sanctioned Reserve Stock of Stone Limit | Reserve Stock Available at Site | Balance Reserve Stock Required | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | Guide Bank Spur No. 1 RD: 0+920 Nawabpur Flood Bund | 2.00 Lac (Cft) | 1.96 Lac (Cft) | 0.04 Lac (Cft) | 63.25 % reserve stock of stone is available at site. |
| 2 | J-Head Spur No. 2 RD: 14+400 Akbar Flood Bund | 1.00 Lac (Cft) | 0.72 Lac (Cft) | 0.28 Lac (Cft) |  |
| 3 | J-Head Spur No. 3 RD: 248+000 Shujabad Branch | 1.00 Lac (Cft) | 0.96 Lac (Cft) | 0.04 Lac (Cft) |  |
| 4 | J-Head Spur No. 4 RD: 263+000 Shujabad Branch | 1.00 Lac (Cft) | 0.62 Lac (Cft) | 0.38 Lac (Cft) |  |
| 5 | Sher Shah Railway flood bund | 1.00 Lac (Cft) | 0.11 Lac (Cft) | 0.89 Lac (Cft) |  |
| 6 | J- Head Spur Chenab flood bund RD.151+800 | 0.50 Lac (Cft) | 0.30 Lac (Cft) | 0.20 Lac (Cft) |  |
| 7 | Chenab Flood Bund RD.151+000 to 157+000 | 0.50 Lac (Cft) | 0.20 Lac (Cft) | 0.30 Lac (Cft) |  |
| 8 | Spur No. 3 B RD 252+000 Shujabad Branch | 1.00 Lac (Cft) | 0.37 Lac (Cft) | 0.63 Lac (Cft) |  |
| 9 | J-Head Spur at RD: 42+200 Nagni Flood Bund | 1.00 Lac (Cft) | 1.00 Lac (Cft) | - |  |
| 10 | Mole Head Spur at RD: 45+700 Nagni Flood Bund | 1.00 Lac (Cft) | 0.82 Lac Cft | 0.18 Lac (Cft) |  |
| 11 | Sloping Spur RD 10000 at Baloch wah Flood Bund | 0.50 Lac (Cft) | - | 0.50 Lac (Cft) |  |
| 12 | Chenab Flood Bund RD: 21+000-26+000 | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 13 | Chenab Flood Bund RD: 28+000-33+000 Sardarpur Section | 0.15 Lac (Cft) | 0.046 Lac (Cft) | 0.104 Lac (Cft) |  |
| 14 | Chenab Flood Bund RD: 47+000-50+000 Sardarpur Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 15 | Chenab Flood Bund RD: 61+000-63+000 Binda Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 16 | Chenab Flood Bund RD: 80+000-96+000 Binda Section | 0.15 Lac (Cft) | 0.39 Lac (Cft) | 0.06 Lac (Cft) |  |
| 17 | Chenab Flood Bund RD: 96+000-110+000 Binda Section | 0.15 Lac (Cft) |  |  |  |
| 18 | Chenab Flood Bund RD: 113+000-117+000 Bosan Section | 0.15 Lac (Cft) |  |  |  |
| 19 | Chenab Flood Bund RD: 118+000-129+000 Bosan Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
| 20 | Chenab Flood Bund RD: 155+000-157+000 Bosan Section | 0.15 Lac (Cft) | - | 0.15 Lac (Cft) |  |
|  | Total | 11.85 Lac (Cft) | 7.496 Lac (Cft) | 4.354 Lac (Cft ) |  |
|  | Note:- Percentage of available Reserve Stock of Stone = 63.25% |  |  |  |  |

14.2  Detail of inlet/ outlet crossing along with closing methodology

| Sr. No. | Name of Bund | Spur | Name of Structure with Location RD: | Location of Crossing RD: |
| --- | --- | --- | --- | --- |
|  | Binda Canal Sub Division |  |  |  |
| 1 | Chenab Flood Bund | - | Shah Din Disty 7470/R | 14-15 |
| 2 | " | - | Shah Din Disty 9183/R | 16-17 |
| 3 | " | - | Shah Din Disty 14461/R | 21-22 |
| 4 | " | - | Shah Din Disty 21040/R | 24-25 |
| 5 | " | - | Shah Din Disty 26338/R | 32-33 |
| 6 | " | - | Shah Din Disty 29200/R | 35-36 |
| 7 | " | - | Tail Shah Din Disty 63015 | 79000 |
| 8 | " | - | " | 82500 |
| 9 | " | - | 6938/R Inayatpur Disty | 86500 |
|  | Ilampur Canal Sub Division |  |  |  |
| 10 | Akbar Flood Bund | - | Akbar Minor 6080/R | 6080 |
| 11 | " | - | 6600/R | 6600 |
| 12 | " | - | 8350/R | 8350 |
| 13 | " | - | 12410/R | 12410 |
| 14 | " | - | 17000/R | 17000 |
| 15 | Sher Shah Flood Bund | - | 1090/L | 1090 |
| 16 |  | - | 6850/L | 6850 |
| 17 |  | - | 11828/L | 11828 |
| 18 |  | - | 15780/L | 15780 |
|  | Shujabad Canal Sub Division |  |  |  |
| 19 | Nagni Flood Bund | - | Bakhtu Wah Disty 1000/R | 1000 |
| 20 | " | - | 11142/R | 11143 |
| 21 | " | - | 14238/R | 14238 |
| 22 | " | - | 33350/R | 33350 |
| 23 | " | - | 40000/R | 40000 |
| 24 | " | - | 50943/R | 50943 |
| 25 | Gardez Flood Bund | - | Gardez Minor 16338/R | 1-2 |
| 26 | " | - | Sat burji Disty RD.52377/R | 13-14 |
| 27 | Sat Burji Flood Bund | - | 54000/R | 0 |
| 28 | " | - | 56400/R | 3-4 |
| 29 | " | - | Tubewell Water Course crossing RD.57-58/R | 7-8 |
| 30 | " | - | 60000/R | 5-6 |
| 31 | " | - | 66300/R | 11-12 |
| 32 | " | - | 57-58/R | 7-8 |
| 33 | Baloch Wah Flood Bund | - | Baloch Wah Minor 4260/R | 4260 |
| 34 | " | - | Baloch Wah Minor 16200/R | 16200 |
| 35 | " | - | Baloch Wah Minor 19090/R | 19090 |
| 36 | " | - | Baloch Wah Minor 21930/R | 21930 |
| 37 | " | - | Baloch Wah Minor 24100/R | 24100 |
| 38 | " | - | Baloch Wah Minor 28440/R | 28440 |
| 39 | " | - | Baloch Wah Minor 34300/T.R | 34300 |
| 40 | Tie Flood Bund | - | Panjani Disty 134500/R | 3500 |
| 41 | " | - | Panjani Disty 13700/R | 5300 |

Methodology:-

After receipt of warning of flood, each crossing will be closed through gunny bags or any alteration  method which consider to necessary well in time and certificate of closing will be taken from Sub Engineer and Sub Divisional Officer concerned.

14.3  Deployment Machinery (Medium to high flood)

Payment for Machinery & work change employees subject to verification by Third Party Monitoring Consultants.

14.4  Deployment Machinery (High to very high flood)

Payment for Machinery & work change employees subject to verification by Third Party Monitoring Consultants.

14.5  Deployment Machinery (high to exceptionally high flood)

Payment for Machinery & work change employees subject to verification by Third Party Monitoring Consultants.

14.6  Police deployment plan

| Sr. No. | Site of Breaching Section | Concerned Canal Division | Concerned Police Station District | Police Persons Deployed Inspector |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sub Inspector | A.S.I. | Constable |  |
| 1 | Head Muhammad Wala | Shujabad | Sadar District Multan | 1 | 1 | 4 | - |
| 2 | Sher Shah | Shujabad | Sadar District Multan | 1 | 1 | 4 | - |

14.7  Details of synthetic bags with capacity  of 500 kg and 1000kg

| SR. NO. | NAME OF EMBANKMENT | BAGS CAPACITY 500 KG |  | BAGS CAPACITY 1000 KG |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Required | Available | Required | Available |  |
| 1 | Fazil Shah flood bund | 100 No. | 100 No. | 50 No. | 50 No. | Required Quantity available in each site, so no material required. |
| 2 | Chenab flood bund | 700 No. | 700 No. | 500 No. | 500 No. |  |
| 3 | Nawabpur flood bund | 200 No. | 200 No. | 100 No. | 100 No. |  |
| 4 | Akbar flood bund | 200 No. | 200 No. | 200 No. | 200 No. |  |
| 5 | 1st Defence Line shujabad branch | 500 No. | 500 No. | 250 No. | 250 No. |  |
| 6 | Sher Shah flood bund | 100 No. | 100 No. | 100 No. | 100 No. |  |
| 7 | Sher Shah Railway flood bund | 100 No. | 100 No. | 100 No. | 100 No. |  |
| 8 | Nagni flood bund | 150 No. | 150 No. | 150 No. | 150 No. |  |
| 9 | Right Bank of Gajju Hatta Branch | 150 No. | 150 No. | 50 No. | 50 No. |  |
| 10 | Dhundoo flood bund | 100 No. | 100 No. | 50 No. | 50 No. |  |
| 11 | Gardez flood bund | 100 No. | 100 No. | 50 No. | 50 No. |  |
| 12 | Sat Burji flood bund | 100 No. | 100 No. | 50 No. | 50 No. |  |
| 13 | Jalalpur Khakhi flood bund | 200 No. | 200 No. | 150 No. | 150 No. |  |
| 14 | Tie flood bund | 100 No. | 100 No. | 50 No. | 50 No. |  |
| 15 | Baloch wah flood bund | 200 No. | 200 No. | 150 No. | 150 No. |  |
|  | TOTAL | 3000 No. | 3000 No. | 2000 No. | 2000 No. |  |

14.8  Details of polythene sheet of black colour to protect upstream slope against wave action and to control seepage through embankments

| SR. NO. | NAME OF EMBANKMENT | LENGTH |
| --- | --- | --- |
| 1 | Fazil Shah Flood Bund RD:0+000 TO 10+500 | 10500 |
| 2 | Chenab Flood Bund RD:0+000 TO 157+950 | 157950 |
|  | Chenab Flood Bund RD:109+294 TO 157+950 | 48656 |
| 3 | Nawabpur Flood Bund RD:0+000 TO 17+500 x 2 | 35000 |
| 4 | Akbar Flood Bund RD:0+000 TO 21+500 x 2 | 43000 |
| 5 | 1st Defence line Shujabad Branch RD:231+000 TO 279+000 x 2 | 96000 |
| 6 | Sher Shah flood Bund RD:0+000 TO 20+437 x 2 | 40874 |
| 7 | Sher Shah Railway Flood Bund RD:0+000 TO 15+100 x 2 | 30200 |
| 8 | Nagni Flood Bund RD:0+000 TO 51+000 | 51000 |
| 9 | Right Bank of Gajju Hatta Branch RD:0+000 TO 18+000 | 18000 |
| 10 | Dhundoo Flood Bund RD:0+000 TO 26+500 | 26500 |
| 11 | Gardez Flood Bund RD:0+000 TO 16+400 | 16400 |
| 12 | Sat Burji Flood Bund RD:54+000 TO 72+000 | 18000 |
| 13 | Jalalpur Khakhi Flood Bund RD:0+000 TO 60+000 | 60000 |
| 14 | Tie Flood Bund RD:0+000 TO 8+600 | 8600 |
| 15 | Baloch Wah Flood Bund RD:0+000 TO 62+200 | 62200 |
|  | TOTAL | 722880 |

CHAPTER-15

Back up Divisions (In case of breach)

Executive Engineer, Multan Canal Division will act as backup Division in case of emergency.

Executive Engineer,

Shujabad Canal Division,

Multan

Superintending Engineer,

Haveli Canal Circle,

Multan.

Chief Engineer,

Irrigation Multan Zone,

Multan.