PART----A

CHAPTER-1

SALIENT FEATURE OF THE TRIMMU HEADWORKS DIVISION TRIMMU

1.1	LOCATION

Trimmu Headworks is located at a distance of about 25 Kilometres from Jhang Sadar. Its precise location is 72.14650 Latitude and 31.14480 Longitude. It falls in semi arid zone with desert on its right and Rechna Doab on its left.

1.2	GENERAL DESCRIPTION

Trimmu Headworks was constructed in 1937–1939 as a part of the Haveli Project. It is located just below the confluence point of the river Jhelum and Chenab. The main features of the barrage are as under:

| Length between the abutments | = | 3937 ft |
| --- | --- | --- |
| Total No. of bays / gates with LCPs (51+13) | = | 64 Nos. |
| No. of bays / gates with LCPs Left under sluice | = | 08 No. (30’ clear width each |
| Total No. of bays / gates with LCPs Main Weir | = | 37 (60’ clear width each) |
| No. of bays/gates with LCPs Right Under sluice | = | 6 No. (30’ clear width each) |
| Additional Bays / gates with LCPs (right side) | = | 13 No. (11 No. 60’ and 2 No. 30’ width) |
| Pond level (Summer) | = | 491.50 Ft |
| (Winter) | = | 490.00 Ft |
| RL of the crest of main weir | = | 477.50 Ft |
| RL of the crest of Left under sluice | = | 472.00 Ft |
| RL of Additional Bays | = | 471.00 Ft |
| ## DESIGNED CAPACITY |  |  |
| ## Main Weir | = | 491,500 Cusecs |
| ## Left under sluice | = | 70,500 Cusecs |
| ## Right under sluice | = | 65,000 Cusecs |
| ## Discharge Through Additional Bays | = | 248,000 Cusecs |
| ## Total |  | 875,000 Cusecs |

Three canals – Haveli Main Line, Trimmu Sidhnai Link (TS Link) and Rangpur Canal are off taking from the Headworks. Haveli main Line and TS Link off take from the left flank whereas the Rangpur off-takes from the right flank of the Headworks. Brief description of the canals is as under:

## Haveli Main Line

## Designed discharge		=5170 Cusecs

Crest RL			= 481.00 Ft

No. of bays 		= 5 Nos bays / gates with LCPs with width of 24 feet

each (524=120 Feet)

Design F.S level		= 489.00

Silt Ejector RD 1200

Maximum Discharge	= 1200 Cusecs

Crest RL			= 477.40

Waterway			= 24 ft

Silt Ejector RD 2000

Maximum discharge		= 1000 Cusecs

Crest RL			= 477.35

Waterway			= 20 ft

## Trimmu Sidhnai Link

## Designed discharge		=12500 Cusecs

Crest RL			= 481.00 Ft

No. of bays			= 10 Nos bays / gates with LCPs with width of 24 feet

each (1024=240 Feet)

## Rang Pur Canal (New Head Regulator)

## Designed discharge		=2710 Cusecs

Crest RL			= 483.50 Ft

No. of bays			= 3 Nos bays / gates with LCPs with width of 24 feet

each ( 324 = 72 Feet )

1.3	ADMINISTRATIVE SETUP

CHAPTER-2

FLOOD PROTECTION AND RIVER TRAINING WORKS

2.1		DESIGN PARAMETERS OF TRAINING WORKS

| Sr. No | Name of Flood Bund | Length |  | Top Width | Side Slope |  | Free board | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Feet | Miles | Feet | River Side | Country Side | Feet |  |
| HEAD WORKS SUB DIVISION |  |  |  |  |  |  |  |  |
| 1.-A | Left Marginal Bund | 46000 | 9.2 | 30 | 1:3 | 1:2 | 7 | Free board with respect to anticipated HFL for 875,000 Cs. |
|  | Wetting channel along LMB | 46000 | 9.2 | 6 | 3:1 | 2:1 | 7 |  |
| B | Length of x-spurs at RD: 8900,10000,13000,11900, 16900,17900,18600,23000,28500. 29500 & 34350 Length = 950+950+950+1100 +700+860+355+360+290+ 400+400 | 7315 | 1.68 | 25 | 3:1 | 2:1 | 6 |  |
| C | Guide Wall No. I (Connected with LMB RD 20+000) |  |  |  |  |  |  |  |
|  | Length of Shank | 3772 | 0.75 | 30 | 3:1 | 3:1 | 6 |  |
|  | Length of Head | 1150 | 0.23 | 30 | 2:1 | 3:1 | 6 |  |
|  | Total | 4922 | 0.98 |  |  |  |  |  |
| D | Guide Wall No. II (Connected with LMB RD 15+000) |  |  |  |  |  |  |  |
|  | Length of Shank | 576 | 0.12 | 30 | 3:1 | 3:1 | 6 |  |
|  | Length of Head | 1100 | 0.22 | 30 | 2:1 | 2:1 | 6 |  |
|  | Total | 1676 | 0.34 |  |  |  |  |  |
| E | Inverted Hockey spur in between 17+900 & 20+400 connected with Guide wall No. I RD 20+440 |  |  |  |  |  |  |  |
|  | Length of Shank | 600 | 0.12 | 25 | 3:1 | 3:1 | 6 |  |
|  | Length of Curve portion | 200 | 0.04 | 25 | 2:1 | 2:1 | 6 |  |
|  | Total | 800 | 0.16 |  |  |  |  |  |
| F | Guide wall at RD 17+900 of LMB |  |  |  |  |  |  | Free board with respect to anticipated HFL for 875,000 Cs. |
|  | Length of Shank | 1186 | 0.23 | 30 | 3:1 | 3:1 | 7 |  |
|  | Length of Head | 1295 | 0.25 | 30 | 2:1 | 2:1 | 7 |  |
|  | Total | 2481 | 0.48 |  |  |  |  |  |
|  | TOTAL | 64894 | 13.18 |  |  |  |  |  |
| 2-A | Right Marginal Bund | 57765 | 11.55 | 25 | 3:1 | 2:1 | 7 | Free board with respect to anticipated HFL for 875,000 Cs. |
| A | Wetting Channel along R.M.B | 16000 | 3.20 | 6 | 3:1 | 2:1 | 7 |  |
| B | J-Head Spur at RD: 4597 RMB | 9250 | 1.85 | 25 | 3:1 | 2:1 | 6 |  |
| C | X-spurs at RD: 23590 & 28815, Length= 900+1200 | 2100 | 0.42 | 25 | 3:1 | 2:1 | 6 |  |
|  | TOTAL | 69115 | 13.82 |  |  |  |  |  |
| 3 | Left Guide Bank U/S | 4500 | 0.90 | 30 | 3:1 | 2:1 | 6 |  |
|  | TOTAL | 4500 | 0.90 | 30 | 3:1 | 2:1 | 6 |  |

| Sr. No | Name of Flood Bund |  | Length |  |  |  | Top Width |  | Side Slope |  |  | Free board | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Feet |  | Feet |  | Feet |  | River Side |  | Country Side | Feet |  |
| 4 | Right Guide Bank U/S |  | 4520 |  | 0.90 |  | 30/60 |  | 2:1 |  | 3:1 | 7 | Free board with respect to anticipated HFL for 875,000 Cs. |
| 4-A | Right Guide Bank D/S |  | 810 |  | 0.16 |  | 30/60 |  | 2:1 |  | 3:1 | 7 |  |
|  | TOTAL |  | 4520 |  | 0.90 |  | 30/60 |  | 2:1 |  | 3:1 | 7 |  |
| 1 | Colony Ring Bund |  | 9600 |  | 1.93 |  | 25 |  | 3:1 |  | 2:1 | 6 |  |
| A | T-Headed Spur No. 1 |  | 4704 |  | 0.94 |  | 25 |  | 3:1 |  | 2:1 | 6 |  |
| B | X-spur |  | 700 |  | 0.14 |  | 25 |  | 3:1 |  | 2:1 | 6 |  |
| C | T-Headed Spur No. 2 |  | 6582 |  | 1.32 |  | 25 |  | 3:1 |  | 2:1 | 6 |  |
| D | X-spur |  | 1400 |  | 0.28 |  | 25 |  | 3:1 |  | 2:1 | 6 |  |
|  | TOTAL |  | 22986 |  | 4.61 |  | 125 |  | 3:1 |  | 2:1 | 6 |  |
| SHORKOT SUB DIVISION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Hassuwali Flood Bund | 72000 |  | 14.4 |  | 25 |  | 3:1 |  | 1.5:1 |  | 6 |  |
| 8 | Rajbana Flood Bund | 22750 |  | 4.55 |  | 25 |  | 3:1 |  | 2:1 |  | 6 |  |
| 9 | Connecting Tail Rajbana Bund with Lakhi Flood Bund RD: (0-11400) | 11400 |  | 2.28 |  | 25 |  | 3:1 |  | 2:1 |  | 6 |  |
| 10-A | Lakhi Flood Bund RD(25-70000) | 45000 |  | 9 |  | 20 |  | 3:1 |  | 1.5:1 |  | 6 |  |
| B | Mole Head | 100 |  |  |  |  |  |  |  |  |  |  |  |
| C | Stone studs 1,2&3 | 400 |  | 0.6 |  |  |  |  |  |  |  |  |  |
| D | Protection Bund | 18500 |  |  |  | 10 |  | 1.5:1 |  | 1.5:1 |  |  |  |
| E | Defence Bund Dabkalan | 650 |  |  |  | 10 |  | 1.5:1 |  | 1.5:1 |  |  |  |
|  | Total | 170800 |  | 30.83 |  |  |  |  |  |  |  |  |  |
| 18-HAZARI SUB DIVISION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11-A | Dadal Flood Bund |  |  |  |  |  |  |  |  |  |  |  |  |
| B | RD 0+000 to 11+000 | 11000 |  | 2.20 |  | 20 |  | 3:1 |  | 1½:1 |  | 6 |  |
| C | RD 11+000 to 69+629 | 58629 |  | 11.72 |  | 25 |  | 3:1 |  | 2:1 |  | 6 |  |
| D | X –spur at RD: 53,56,60 & 66 Length=540+800+250+1000+450 | 3040 |  | 0.60 |  | 10 |  | 2:1 |  | 2:1 |  | 6 |  |
| E | J-Head Spur at RD 54+500 | 2300 |  | 0.46 |  | 30 |  | 3:1 |  | 2:1 |  | 6 |  |
|  | Total | 74969 |  | 14.98 |  |  |  |  |  |  |  |  |  |
| 12-A | Rangpur Flood embankment along left bank of Rangpur Canal |  |  |  |  |  |  |  |  |  |  |  |  |
| B | RD 260+000 to 308+000 | 48000 |  | 9.6 |  | 20 |  | 3:1 |  | 2:1 |  | 6 |  |
| C | RD 308+000 to 318+000 | 10000 |  | 2.0 |  | 25 |  | 3:1 |  | 2:1 |  | 6 |  |
| D | Like inverted hockey spur RD 292+500 | 2654 |  | 0.53 |  | 30 |  | 3:1 |  | 2:1 |  | 6 |  |
| E | J-Head Spur at RD 302+000 | 6455 |  | 1.2 |  | 30 |  | 3:1 |  | 2:1 |  | 6 |  |
|  | TOTAL | 70959 |  | 14.10 |  |  |  |  |  |  |  |  |  |
|  | GRAND TOTAL | 145928 |  | 29.08 |  |  |  |  |  |  |  |  |  |

2.2	LOCATION MAP

CHAPTER-3

BRIEF HISTORY OF PAST FLOOD EVENTS

| YEAR | DISCHARGE |  | BREACHED REACHES |
| --- | --- | --- | --- |
| 1959 | Through Barrage Through Breach Total | 718,255 Cs 225,000 Cs 943,255 Cs | 6 Nos Breaches in Right Marginal Bund RD: 16-18 |
| 1973 | Through Barrage Through Breach Total | 666,910 Cs 66,000 Cs 732,910 Cs | 4 Nos Breaches in Right Marginal Bund RD: 16-18 |
| 1976 | Through Weir | 704,633 Cs | - |
| 1992 | Through Barrage Through Breach Total | 683,117 Cs 205,000 Cs 888,117 Cs | 6 Nos Breaches in Right Marginal Bund RD: 16-18 |
| 2014 | Through Barrage Through Breach Total | 626,006 Cs 77,000 Cs 703,006 Cs | 6 Nos Breaches in Right Marginal Bund RD: 16-18 |

Events Flood 2014

| Sr No. | Date of occurrence | Time | Nature of incidents | Place of incidents | Efforts made for controlling incidents | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 08-09-2014 | 5 :00 PM | Right pocket become non-modular | Barrage | By effective watching and exercising timely protective measure with the help of deployed machinery and flood fighting material, leakages, even of dangerous nature were controlled | Public property saved |
| 2 | 09-09-2014 | 7 :20 PM | Leakage of dangerous nature | RD:44+360 LMB |  |  |
| 3 | 10-09-2014 | 1 :00 AM | Main Weir + Right Pocket Gates become free | Barrage |  |  |
| 4 | 10-09-2014 | 2:25 AM | Seepage of dangerous nature | RD:44+450 LMB |  |  |
| 5 | 10-09-2014 | 4:10 AM | Very serious leakage | RD:20+000 LMB |  |  |
| 6 | 10-09-2014 | 5:00 AM | All gates of barrage become free | Barrage |  |  |
| 7 | 10-09-2014 | 6:40 AM | Serious Seepage | RD:44+650 LMB |  |  |
| 8 | 10-09-2014 | 7:30 AM | Seepage of dangerous nature occurred | RD:44+500 LMB |  |  |
| 9 | 10-09-2014 | 8:15 AM | Seepage of dangerous nature was occurred | RD:25+940 LMB |  |  |
| 10 | 10-09-2014 | 10:30 PM | Seepage of dangerous nature was occurred | RD:44+450 LMB |  |  |

CHAPTER-4

DESIGN DATA, HISTORIC PEAK FLOOD DATA AND PREVIOUS FIVE YEARS FLOOD DATA OF HEAD WORKS / BARRAGES AND OR OTHER CONTROL POINTS

4.1	FLOOD LIMITS IN TRIMMU HEADWORKS DIVISION, TRIMMU

| DESCRIPTION | FLOOD LIMITS (LAC CUSECS) |
| --- | --- |
| Low Flood | 1.50 to 2.00 |
| Medium Flood | 2.00 to 3.00 |
| High Flood | 3.00 to 4.50 |
| Very High Flood | 4.50 to 6.00 |
| Exceptionally High Flood | Above 6.00 |

4.2	TIME LAG OF FLOODS

The time lag of the peaks for river Jhelum and Chenab based upon study of actual Hydrograph for data of many years is tabulated as under:-

| Sr. No. | Stage | River Chenab |  |  | River Jhelum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Marala to Khanki 39 Miles | Marala to Qadirabad 57 Miles | Qadirabad to Trimmu 163 Miles | Mangla to Rasool 43 Miles | Rasool to Trimmu 178 Miles |
|  |  | Hours | Hours | Hours | Hours | Hours |
| 1 | 0.5 to 1 lac Cusecs | 10 | 12 | 40 | 16-20 | 48 |
| 2 | Low Flood | 12 | 15 | 48 | 15 | 54 |
| 3 | Medium Flood | 14 | 16 | 60 | 12 | 60 |
| 4 | High Flood | 15 | 18 | 66-72 | 10 | 66-72 |
| 5 | Exceptionally High Flood | 15 | 18 | 78 | 8 | 72-78 |
| 6 | As approved by the federal Flood Commission for the months of July and August | - | 48 | 48 | 24 | 48 |

4.3	HIGHEST FLOODS

The history of highest floods is as under:-

| Year | Flood Level |  | Through Head Works | Through Breaches in embankments |  | Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | U/S | D/S |  | Left Side | Right Side |  |
| 1959 | 492.10 | 489.50 | 71,8255 | - | 225,000 | 943,255 |
| 1960 | 490.00 | 488.50 | 605,132 | - | - | 605,132 |
| 1973 | 494.30 | 491.15 | 666,910 | - | 66,000 | 732,910 |
| 1976 | 493.6 | 489.7 | 70,4633 | - | - | 704,633 |
| 1992 | 494.40 | 490.80 | 683,117 | - | 205,000 | 888,117 |
| 1995 | 494.30 | 493.50 | 629,561 | - | - | 629,561 |
| 1997 | 494.50 | 491.95 | 677,417 | - | - | 677,417 |
| 2014 | 494.60 | 494.20 | 626,003 | - | 77,000 | 703,006 |

4.4	PEAK DISCHARGES

The history of peak discharges of last 30-years is as under: -

| Year | Date | Discharge |
| --- | --- | --- |
| 1995 | 01-08-1995 | 629,561 |
| 1996 | 27-08-1996 | 543,708 |
| 1997 | 01-09-1997 | 677,417 |
| 1998 | 13-07-1998 | 148,668 |
| 1999 | 22-07-1999 | 61,476 |
| 2000 | 25-07-2000 | 116,179 |
| 2001 | 18-08-2001 | 72,431 |
| 2002 | 17-08-2002 | 108,627 |
| 2003 | 08-08-2003 | 122,892 |
| 2004 | 21-08-2004 | 42,756 |
| 2005 | 11-07-2005 | 162,097 |
| 2006 | 07-09-2006 | 266,270 |
| 2007 | 15-03-2007 | 173,532 |
| 2008 | 07-08-2008 | 54,234 |
| 2009 | 21-08-2009 | 46,765 |
| 2010 | 11-08-2010 | 323,026 |
| 2011 | 26-09-2011 | 127,790 |
| 2012 | 07-08-2012 | 67,694 |
| 2013 | 21-08-2013 | 267,609 |
| 2014 | 10-09-2014 | 703,006 |
| 2015 | 29-07-2015 | 150,865 |
| 2016 | 10.08.2016 | 153,339 |
| 2017 | 05.08.2017 | 89,345 |
| 2018 | 17.08.2018 | 81,680 |
| 2019 | 22.08.2019 | 93,021 |
| 2020 | 01.09.2020 | 196,077 |
| 2021 | 31.07.2021 | 106,867 |
| 2022 | 18.08.2022 | 122,115 |
| 2023 | 25.07.2023 | 116,786 |
| 2024 | 19.08.2024 | 87,030 |

CHAPTER-5

FLOOD FIGHTING STRATEGY

The following precautionary measures are adopted before flood season to face any alarming situation at Trimmu Headworks but even then, arrangements are necessary to avoid heavy damages and flood fighting strategy is adopted in such a way that flood water passes safely through Barrage, up to its limit.

In low flood, river water remains within the deep channel and does not cause any problem if regulation of gates is done as per rules. Watching establishment employed for this purpose will remain busy in removing jungle, filling gharas and percupine holes          if any.

- MEDIUM FLOOD

When river water enters into Medium Flood i.e 2 to 3 Lacs, it will start spreading on adjoining area and also touching the toe of bunds. Then it becomes necessary to be more vigilant. The Flood bunds will be fully watched to avoid any leakage from bunds. Sub Engineers will shift their camps to sites as per duty roaster. Hourly gauges and discharge will be conveyed to Civil Administration and flood warning centers to inform public accordingly.

- HIGH FLOOD

When river water enters into high flood range i.e 3 to 4.5 Lacs, Sub Divisional Officer Head Works will shift his camp at Trimmu Headworks to supervise staff working on L.M.B and D/S Spurs. In whole period, regulation will be done in such a way that silt deposit in front of canals may be washed D/S of the river to minimize silt entry in to canals. Silt contents in water will be checked thrice a day and if it exceeds, the permissible limit, canals will be closed accordingly.

- VERY HIGH FLOOD

When river water enters into very high flood range i.e 4.5 to 6 Lacs, Executive Engineer alongwith Sub Divisional Officer Head Works will shift his camp at Trimmu Barrage to supervise staff working on L.M.B, RMB and D/S Spurs and to check any leakage if any. Critical gauge will be checked on hourly basis round the clock. If discharge is in rising position then breaching section is activated in consultation with Civil Administration and Pak Army.

- EXCEPTIONALLY HIGH FLOOD

Gauges of different sites especially gauge at RD: 15000 of L.M.B will be noted regularly and recorded. Executive Engineer will inform all agencies especially civil administration as well as Army about flood situation in rising position. In case of very high flood or exceptionally high flood, meeting will be held in field office with Army and Civil Administration to face the situation.

Two dozers will be kept ready at RD: 16000 to 18000 R.M.B for making cuts and closing breaches as and when required according to circumstances. When gauge at RD: 15000 L.M.B touches level 500.00 Ft and is found in rising position it will be the duty of Executive Engineer to request Army for operation of breach section for the safety of Barrage. When water subsides, the breaching sections will be closed (if operated) as soon as possible.

CHAPTER-6

FLOOD RESTORATION WORKS LIKELY TO BE EXECUTED

BEFORE FLOOD 2025

| Sr No. | Name of work | Amount (Rs. In Million) |
| --- | --- | --- |
| 1 | Repairing and maintaining Rangpur Flood Embankment from RD 0+000 to 50+000 | 33.20 |
| 2 | Repairing and maintaining dadal flood bund from RD 0+000 to 69+500. | 39.00 |
| 3 | Installation of new well pumps along newly constructed wetting channel of RMB RD 0+000 to 15+000. | 8.000 |
| 4 | Repairing apron of Guide Wall No.2 RD 14+900 of LMB of Trimmu | 7.460 |
| 5 | Repairing apron of inverted hockey spur connected with Guide Wall No.1 RD 20+440 of LMB Trimmu | 4.000 |
| 6 | Strengthening wetting channel at RD 16+900 to RD 21+000 of LMB. | 2.960 |
| 7 | Repairing and maintaining Makori flood bund Trimmu Headworks Sub Division, Trimmu | 6.000 |
| 8 | Maintaining of Service track of Rangpur Flood Bund from RD 260+000 to 318+000 | 1.500 |
| 9 | Rehabilitation of approach road & Structure of like Hockey Spur at RD 292+500. | 45.000 |
| 10 | Filling Rain Cuts on Dadal Flood Bund Between RD 0+000, RD 45+000 in Pir Abdul Rehman Section | 0.956 |
| 11 | Filling Rain Cuts on J Head Spur at Rd 54+500 in Pir Abdul Rehman Section | 0.500 |
| 12 | Maintaining / replacing Breaching Section for the capacity of 875000 Cs. | 20.00 |

CHAPTER-7

FLOOD FIGHTING WATCHING ARRANGEMENTS

7.1	PRE-FLOOD ARRANGEMENTS

The watching establishment on marginal bunds is required to be employed as per following scale in accordance with the approval given by Superintending Engineer Haveli Canal Circle Multan vide his letter No. 6272-73/170-W dated 31.05.1974 (Duty Roaster attached). In addition to previous mentioned material, which will be placed at different camp sites, an additional material will be also be required in order to meet with extraordinary worst situation and to recoup the site situation.

As a guide the following scale is assumed to be sufficient. The Executive Engineer with the consultation of the Superintending Engineer, Haveli Canal Circle, Multan / Director Technical / DPD PMO Punjab Barrages, Lahore can increase the scale:

Flood Management Strategy for Flood Season 2022

- Weak sites of flood bund / river training works have been attended / being attended.

- Data boards / gauges, RD Marks have been updated.

- Deficient reserve stock of stone has been recouped. 17.24 Lac cft reserve stock of stone is available to face any emergent situation during flood.

| Sr. No. | Name of structure | Sanctioned Quantity | Available Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | LGB U/S AND D/S | 5.00 | 4.87 | The available quantity is more than minimum requirement. |
| 2 | RGB U/S AND D/S | 5.00 | 4.76 |  |
| 3 | J-HEAD SPUR RD 5500 OF RMB | 1.50 | 1.57 |  |
| 4 | SLOPING SPUR ALONG COLONY RING BUND | 1.00 | 0.32 |  |
| 5 | T-1 SPUR ALONG COLONY RING BUND | 1.50 | 0.81 |  |
| 6 | T-2 SPUR D/S TRIMMU BARRAGE | 1.50 | 0.79 |  |
| 7 | J-HEAD SPUR MAKORI | 1.50 | 0.90 |  |
| 8 | DAB KALAN (Relates to SDO Shorkot) | 1.25 | 0.89 |  |
| 9 | GUIDE WALL NO.1 ALONG LMB | 1.50 | 0.76 |  |
| 10 | GUIDE WALL NO.2 ALONG LMB | 1.50 | 0.76 |  |
| 11 | GUIDE WALL NO.3 ALONG COLONY RING BUND | 1.50 | 0.59 |  |
| 12 | X-SPUR RD 23000 OF RMB | 0.25 | 0.21 |  |
|  | TOTAL (LACS CFT) | 23.00 | 17.24 |  |

- Flood Fighting material is available to fight and pass the flood safely.

- Gates and gearing system of the Barrage have been checked and is functioning properly.

- Critical reaches of flood bunds are being attended to avoid any mishap.

- Breaching section has been restored.

- On receipt of flood warning, watching establishment will immediately be deployed on flood bunds as per flood fighting plan. Critical reaches of flood bund will be focused with much more care. The scale of watching material approved in Flood Fighting plan prepared after Flood of 1975 will be followed which is given below:-

| Sr. No. | Item | # Scale |
| --- | --- | --- |
| 1 | Lamps Hurricane | 1 per beldar, 1 per mate and 20% extra of total only for the night shift |
| 2 | Wicks | 9 inch per lamp |
| 3 | Chimneys | 1 spare for each lamp |
| 4 | Burners & caps | 1 spare for each lamp |
| 5 | Torches – 3 cell | 1 for Executive Engineer, 1 for Sub Divisional Officer, 1 for Sub Engineer 1 one spare |
| 6 | Cells for Torches | 1 full and six spare sets |
| 7 | Petromax lamp | At depressions, old breach sites and dangerous sites each lamp with 2 spare chimneys, 2 nozzles, 2 funnels fitted with wire gauze |
| 8 | Fuel for lighting fire | To be collected by beldar establishment |
| 9 | Kerosene Oil | As per practice 1/3 bottle per lantern per night and one bottle per patromax per night (22 nights per month) |
| 10 | Match Boxes | 1 match box per month |
| 11 | Spirit | 1 bottle per patromax per month |
| 12 | Funnels | 2 No. per watching hut |
| 13 | Oil Extractor (Ordinary) | 2 No. per watching hut |
| 14 | Spirit cans | 1 per petromax |
| 15 | Larmal 50 feet long | 50 per watching hut |
| 16 | Mallets wooden | 5 mallets per watching hut |
| 17 | Tranger wire | 1 roll of 14 BSW gauge at each bund |
| 18 | Gunny bags | 100 bags per mile |
| 19 | Axes | 2 No per mile |
| 20 | Kassies with handles | 1 for each beldar + 10% (for one shift only) |
| 21 | Hammer 2.5 lbs | 1 per mile |
| 22 | Hammer 10 lb | 2 per mile |
| 23 | Buckets (12 inch size) | 5 No. per mile |
| 24 | Sledge hammer 14 lbs | 1 per mile |
| 25 | Bhan (Cow Dung) in EC bags | 20 bags per mile for dangerous portions |
| 26 | Sand | Collection of 200 cft. Every mile for dangerous portions |
| 27 | Stacks (6-8 feet) long | 100 stacks per mile |
| 28 | Khajji mats 3’*4’ | 50 Nos per mile |
| 29 | Baskets | 1 toot/2 Pilchi baskets per beldar of shift twice a month |
| 30 | Sutli | 1 lb per 100 bags |
| 31 | Needles | 4 Nos per beldar huts |
| 32 | Kassi Handles | 50% of the no. of kassies as spare |
| 33 | Munj/Patha/Pees/ Taranger 3 cft capacity of Stone. | 500 Nos for each structure. |
| 34 | Tooth basket | 100 Nos each Sub Engineer |

Flood camps will be established during the flood season at vulnerable sites of embankment for the storage of the watching material and shelter for staff on or off duty. The watching material will be shifted into these camps at the start of the flood season.

Officer in-charge will issue the material as and when emergency arises. Some balance material will be obtained / received back as issued during last year during Flood season only balance will be procured to meet the requirement of this year.

7.2	WATCHING ESTABLISHMENT

| Site | Flood Limit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low |  | Medium |  | High |  | Very High |  | Excep: High |  |
|  | Men | Shift | Men | Shift | Men | Shift | Men | Shift | Men | Shift |
| Left Marginal Bund | -- | -- | 67 | 2 | 132 | 3 | 199 | 3 | 264 | 3 |
| Right Marginal Bund | -- | -- | 72 | 2 | 144 | 3 | 216 | 3 | 288 | 3 |
| Colony Ring Bund | -- | -- | 13 | 2 | 24 | 3 | 35 | 3 | 48 | 3 |
| Hassuwali Flood Bund | -- | -- | 130 | 2 | 202 | 3 | 274 | 3 | 346 | 3 |
| Rajbana Flood Bund | -- | -- | 41 | 2 | 82 | 3 | 123 | 3 | 164 | 3 |
| Rangpur Flood Bund | -- | -- | 102 | 2 | 167 | 3 | 232 | 3 | 297 | 3 |

The watching establishment is deployed according to Flood stage. The following statement depicts strength of establishment required during flood season in addition to regular establishment.

7.3	ARRANGEMENT AT SENSITIVE SITES.

ARRANGEMENT AT SENSITIVE SITES

Extra machinery and watching labour will be deployed at the sensitive sites. In addition to extra machinery & watching labour, a sufficient quantity of reserve stock of stone is present to meet with any emergency. The limit of reserve stock of stone was approved by Chief Engineer, Irrigation Multan Zone, Multan vide his No. 301/ B&A/ 2015/ 5885-89/ A/292/68 dated 06.04.2015 at site. Detail is given below:-

CONSOLIDATED STATUS OF RESERVE STOCK OF STONE

| Sr. No. | Sanctioned Reserve stock Limit (Lac Cft) | Available Quantity (Lac Cft) | Balance required with respect to Sanctioned limit (Lac Cft) | Minimum Quantity Available (Lac Cft) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 23 | 17.24 | 5.76 | 16.10 | However, as per minimum requirement, there is no deficiency of stone. |

DETAIL OF SANCTIONED RESERVE STOCK

| SR NO. | NAME OF SITE | SANCTIONED LIMIT (LACS CFT) | AVAILABLE AT SITE (LACS CFT) | BALANCE QUANTITY (LAC CFT) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | LGB U/S AND D/S | 5.0 | 4.87 | 0.13 | 74% quantity against minimum threshold of 70% is available which is sufficient to combat any untoward situation. |
| 2 | RGB U/S AND D/S | 5.0 | 4.76 | 0.24 |  |
| 3 | J-HEAD SPUR RD 5500 OF RMB | 1.5 | 1.57 | 0 |  |
| 4 | SLOPING SPUR ALONG COLONY RING BUND | 1.0 | 0.32 | 0.68 |  |
| 5 | T-1 SPUR ALONG COLONY RING BUND | 1.5 | 0.81 | 0.69 |  |
| 6 | T-2 SPUR D/S TRIMMU BARRAGE | 1.5 | 0.79 | 0.71 |  |
| 7 | J-HEAD SPUR MAKORI | 1.5 | 0.90 | 0.6 |  |
| 8 | DAB KALAN (Relates to SDO Shorkot) | 1.25 | 0.89 | 0.36 |  |
| 9 | GUIDE WALL NO.1 ALONG LMB | 1.5 | 0.76 | 0.74 |  |
| 10 | GUIDE WALL NO.2 ALONG LMB | 1.5 | 0.76 | 0.74 |  |
| 11 | GUIDE WALL NO.3 ALONG COLONY RING BUND | 1.5 | 0.60 | 0.9 |  |
| 12 | X-SPUR RD 23000 OF RMB | 0.25 | 0.21 | 0.04 |  |
|  | TOTAL (LACS CFT) | 23.0 | 17.24 | 5.76 |  |

7.4	WATCHING MATERIAL

It was agreed in the meeting held in the office of the Chief Engineer, Irrigation Zone Multan at Multan on 18-06-1978 that the scale of watching material approved in the Flood Fighting Plan prepared after the floods of 1975 will be followed. The scale was approved by the Superintending Engineer, Haveli Canal Circle, Multan No 6272-73/170-W/ Floods/1973-74 dated 31-05-1974.

Watching material will be provided in ample quantity as the experience shows that presence of watching material is very necessary. The particular care would be taken to ensure that there is adequate quantity of the required material shown as to be readily available everywhere particularly at vulnerable / critical sites. (Watching material)

| Sr# | Description | Unit | Quantity Required | Quantity Available | Balance Quantity required | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes | No. | 200 | 47 | 153 | Balance quantity will be recouped during flood season 2025 |
| 2 | Baskets (Water uses) | No. | 5082 | 1677 | 3405 |  |
| 3 | Buckets (earth filling) | No. | 726 | 26 | 700 |  |
| 4 | Cell for Torches | No. | 723 | 33 | 690 |  |
| 5 | Cow Dung in EC | No. | 837 | 50 | 787 |  |
| 6 | Funnel (Ordinary) | No. | 138 | 32 | 106 |  |
| 7 | Funnel with W.G | No. | 58 | 10 | 48 |  |
| 8 | Gas Lamp | No. | 29 | 10 | 19 |  |
| 9 | Gunny Bags | No. | 10503 | 2434 | 8069 |  |
| 10 | Hammer 2.5 lb | No. | 76 | 16 | 60 |  |
| 11 | Hammer 10 lb | No. | 137 | 35 | 102 |  |
| 12 | Kassi with handles | No. | 957 | 100 | 857 |  |
| 13 | Kerosene Oil | Liter | 2324 | 250 | 2074 |  |
| 14 | Khajji Mate | No. | 5792 | 300 | 5492 |  |
| 15 | Lanterns | No. | 864 | 40 | 824 |  |
| 16 | Larmal 50 Ft | No. | 3374 | 171 | 3203 |  |
| 17 | Mallet wooden (use in Killa pushing) | No. | 322 | 74 | 248 |  |
| 18 | Match Box | No. | 796 | 100 | 696 |  |
| 19 | Mental | No. | 796 | 100 | 696 |  |
| 20 | Needles | No. | 66 | 10 | 56 |  |
| 21 | Needles for sewing bags | No. | 259 | 15 | 244 |  |
| 22 | Nozzles | No. | 61 | 10 | 51 |  |
| 23 | Oil Extractor | No. | 133 | 10 | 123 |  |

| 24 | Sledge Hammer 14 lb | No. | 150 | 10 | 140 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 25 | Spare handle for kassies | No. | 489 | 25 | 464 |  |
| 26 | Siprit | No. | 75 | 10 | 65 |  |
| 27 | Spirit Can | No. | 30 | 10 | 20 |  |
| 28 | Sticks (6-8) | No. | 3635 | 574 | 3061 |  |
| 29 | Sutli | No. | 181 | 38 | 143 |  |
| 30 | Torch 3 Cell | No. | 52 | 10 | 42 |  |
| 31 | Tranger Wire | No. | 25 | 10 | 15 |  |
| 32 | Washers | No. | 61 | 10 | 51 |  |
| 33 | Wicks | No. | 429 | 145 | 284 |  |
| 34 | Munj Patha Taranger | No. | 1600 | 1600 | 0 |  |
| 35 | Petrol for generator | Liter. | 3960 | 100 | 3860 |  |
| 36 | Diesel for Petroling | Liter | 300 | 300 | 0 |  |
| 37 | Empty Drum | No. | 3 | 3 | 0 |  |
| 38 | Manila rope ½” to 1” | Ft | 500 | - | 500 |  |
| 39 | Water Supply | No. | 10 | - | 10 |  |
| 40 | Water set | No. | 8 | - | 8 |  |
| 41 | Plastic Chair Set | No. | 8 | - | 8 |  |
| 42 | Steel wire 10 No. | Ft | 100 | - | 100 |  |
| 43 | Bamboos 8’ to 12’ | No. | 500 | - | 500 |  |

Note:	Full quantum of material will be purchased before flood season on receipt of funds. The balance / deficient watching material will be purchase of before flood season.

7.5	ARRANGEMENT FOR SOUNDING AND PROBING

The arrangement for sounding and probing will be taken up with the boat. An extra boat equipped with bathymetric arrangements is also available for drawing of sounding and probing by using software.

7.6	LIGHTING ARRANGEMENT

All the gates of Trimmu Barrage, additional bays and off-taking canals have electrically operated local control panels in addition to provision of mechanically operation which are provided under the ongoing Rehabilitation and Up-gradation project.

Independent 11-Kv transmission line have been laid to the energize the installations provided at Barrage. FESCO – WAPDA will ensure proper power supply to the head works. A well-equipped squad of electricians will be directed to be present at Head works for maintaining the proper lighting arrangement and to deal with any emergency related to the electric supply at the barrage.

2 Nos. generators of 100 KVA capacity & 1 No. portable 50 KVA capacity generator are available at Trimmu Barrage and will be used in case of electricity failure at Barrage and at the most vulnerable point under the river attack. Whereas at other vulnerable points, small generators as well as torches and lamps will be used for the lighting / watching arrangement.

7.7	RATION ARRANGEMENT

Ration is provided by the Irrigation Department to the watching establishment deployed along flood bunds during flood days. However ration for displaced persons from the flood area is provided by District Administration and NGOs.

7.8	P.O.L ARRANGEMENT FOR VEHICLE.

## (Attention D.C Jhang)

Efforts will be made to stock some POL before the start of the flood season. The Deputy Commissioner, Jhang will order to reserve 5000 liters of Petrol and 10000 liters of Diesel at each of the following Petrol Pumps.

- M/S M B Ahmad and Sons At Athara Hazari

2. 	P.S.O Patrolling Mahloana More

3. 	M/S Ateeq Shorkot city

4.	P.S.O Garh More

7.9	TRANSPORTATION

# (Attention – Superintending Engineer, Trimmu Canal Circle, Trimmu

(Attention – Executive Engineer, Trimmu Headworks Division, Trimmu)

(Attention – D.C. Jhang)

There is one Toyota Hilux available with the Executive Engineer and one Pickup double cabin case with the Sub Divisional Officer, Trimmu Headworks which will be used for the shifting of material, human resources and patrolling. There are One No. Pickup Double Cabin and One No. Suzuki Jeep Photohar under the use of the SDO Athara Hazari and Shorkot Sub division located on each side of the River Chenab on D/S of Trimmu Barrage. For further preparation in this regard, the SE Trimmu Canal Circle Trimmu and Deputy Commissioner Jhang will be requested to keep one Jeep on the standby basis for dealing with the situation if the emergency arises.

Executive Engineer, Sidhnai Headworks Division, Sidhnai / Superintending Engineer, Trimmu Canal Circle, Trimmu will be requested to provide Trucks/ Pickups in tip top condition to deal with the Emergency if required. The supervisory staff will be directed to arrange for the transport themselves as far as possible.

7.10	LAW AND ORDER

# (Attention: Deputy Commissioner & District Police Officer Jhang)

Police guard at right and left side of Trimmu Headworks would be available round the clock and the D.C Jhang will be requested to increase the strength so that the flood emergency measures are not interrupted. Police will be particularly requested to quell any effort made by the public to cut or breach any bund or embankment. In case miscreants try to cut any flood bund, the officer incharge will immediately call for the help of police. In case of any other type of law and order situation, the call for police will be made to deal with the situation, so that the flood emergency measure may go un-interrupted. The police will also be responsible for the protection of the installations at the Headworks. Furthermore, security guards are also hired for protection of Trimmu Headworks. The canal guards equipped with weapons will also available with the Executive Engineer and Sub Divisional Officers of Trimmu Headworks Division.

7.11	MEDICAL ARRANGEMENT FOR LABOUR

# (Attention: Chief Executive Officer Health, Jhang)

There is one Dispensary available at Trimmu Headworks to deal any emergency labour working at Rehabilitation Project of Trimmu Headworks. The Chief Executive Officer, Jhang will be requested to set up a medical facility at Trimmu Headworks having trained and experienced staff along with the sufficient medicine during the flood season. The staff will remain present for 24 hours to meet with the emergency during the extreme flood at Trimmu.

7.12	LIAISON WITH ANOTHER DEPARTMENT

# (Attention: Irrigation/Civil/Police department)

Executive Engineer Trimmu and Sub Divisional Officer, Trimmu Headworks will have close liaison with their counterparts of other departments and will call for any support they deem fit from them. Close contact will be maintained with civil authorities and police departments for all emergency measures. They will be constantly kept informed of the flood situation at Trimmu Headworks.

7.13	ROLE OF THE ARMY

The army authorities do the pre-flood inspection of the explosive store and breaching section. The breaching section is activated with the help of Pakistan Army by using the explosive material. In case this method fails, the breaches are made by using the dozers / excavator / labour. The Pakistan Army keeps on visiting the site from time to time.

7.14	DUTIES OF TELEPHONE ATTENDANT

| Sr No. | Name of Official | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Muhammad Usman | 15.06.2025 To 15.10.2025 | 06:00 A.M | 02:00 P.M |
| 2 | Amir Iqbal |  | 02:00 P.M | 10:00 P.M |
| 3 | Qayoum Khan |  | 10:00 P.M | 06:00 A.M |

7.15	WIRELESS ARRANGEMENTS

There are two wireless sets working in Trimmu Headworks Division. One is working in Telegraph office Trimmu, and second with police is working at the Headworks. The wireless sets in vehicles will be provided by the DPO concerned before the coming flood.

Chapter - 08

| S. No. | Name of Canal Division | Name of District | Total No. of Encroachments | No. of Encroachments removed so far | No. of Encroachments to be removed |
| --- | --- | --- | --- | --- | --- |
| 1 | Trimmu Headworks Division, Trimmu | Jhang | 54 | 4 | 50 |

DETAIL OF ENCROACHMENT

CONSOLIDATED UPDATED STATUS OF REMOVAL OF ENCROACHMENTS

Status of encroachment of Trimmu Barrage division

| Sr.No. | Name of infrastructure | Nature of encroachments | Total No of encroachments | No. of encroachments removed | Tehsil | District | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 4 | 5 |  | 6 | 7 | 8 |
| 1 | Colony Ring Bund | Katcha Houses | 4 No. | 1 | Jhang | Jhang | Few encroachments where there was not much resistance have been removed. District Administration has been requested to help in removal of encroachments. |
| 2 | Right Marginal Bund |  | 28 No. | 2 | 18-Hazari | Jhang |  |
| 3 | Left Marginal Bund |  | 20 No. | 1 | Jhang | Jhang |  |

Detail of Encroachment

| Sr No. | Name of Bund / Spur | RD | Name of Encroachee |
| --- | --- | --- | --- |
| 1 | Right Marginal Bund | 12000-13000 | Mr, Muhammad Aslam Caste Muslim Sheikh |
| 2 | Right Marginal Bund | 12000-13000 | Mst: Nooran caste Muslim Sheikh |
| 3 | Right Marginal Bund | 12000-13000 | Mr, Muhammad Nawaz Sial |
| 4 | Right Marginal Bund | 12000-13000 | Mr, Farid S/O Pehlwan |
| 5 | Right Marginal Bund | 12000-13000 | Mr, Bilal Sindhi |
| 6 | Right Marginal Bund | 12000-13000 | Mr, Suleman Sindhi |
| 7 | Right Marginal Bund | 12000-13000 | Mr, Ghulam Akbar Sindhi |
| 8 | Right Marginal Bund | 13000-14000 | Mr, Ghulam Akbar Sindhi |
| 9 | Right Marginal Bund | 13000-14000 | Mr, Muhammad Shafi Sindhi |
| 10 | Right Marginal Bund | 13000-14000 | Mr, Mastoo Sindhi |
| 11 | Right Marginal Bund | 13000-14000 | Mr, Hameed Sindhi |
| 12 | Right Marginal Bund | 20000-21000 | Mr, Muhammad Sharif Jhabail |
| 18 | Right Marginal Bund | 20000-21000 | Mr, Allah Bakhsh Piraien |
| 19 | Right Marginal Bund | 24000-25000 | Mr, Muhammad Hayat S/O Sher Muhammad Umrana |
| 20 | Right Marginal Bund | 24000-25000 | Mr, Lal S/O Saluh Umrana |
| 21 | Right Marginal Bund | 27000-28000 | Mr, Zulifqar S/O Muhammad ramzan Umrana |
| 22 | Right Marginal Bund | 28000-29000 | Mr, Muhammad Riaz S/O Allah Bukhsh Umrana |
| 23 | Left Marginal Bund | 15000 | Mr, Alam Sher S/O Shameer Caste Diraj |
| 24 | Left Marginal Bund | 19000 | Mr, Farid S/O Haji |
| 25 | Left Marginal Bund | 20000 | Mr, Hakim Ranjha |
| 26 | Left Marginal Bund | 32000 | Mr, Akbar S/O Allah Yar |
| 27 | Left Marginal Bund | 35000 | Mr, Sarfaraz S/O Muhammad |
| 28 | Left Marginal Bund | 35000 | Mr, Ijaz S/O Muhammad |
| 29 | Left Marginal Bund | 35000 | Mr, Sarfaraz S/O Dalmir |
| 30 | Left Marginal Bund | 35000 | Mr, Islam Mochi |
| 31 | Left Marginal Bund | 38000 | Mr, Dalmir S/O Gahran |
| 32 | Left Marginal Bund | 38000 | Mr, Riaz S/O Noor Muhammad |
| 33 | Left Marginal Bund | 38000 | Mr,Sarfaraz |
| 34 | Left Marginal Bund | 41000-42000 | Mr, Muhammad Aslam S/O Nawab |
| 35 | Left Marginal Bund | 41000-42000 | Mr, Bashir Ahmed S/O Allah Ditta |
| 36 | Left Marginal Bund | 41000-42000 | Mr, shammas Dhirkhan |
| 37 | Left Marginal Bund | 41000-42000 | Mr,Ghulam Haider Noon |
| 38 | Left Marginal Bund | 41000-42000 | Mr, Noor Khan Noon |
| 39 | Left Marginal Bund | 41000-42000 | Mr, Ghulam Farid Noon |
| 40 | Left Marginal Bund | 41000-42000 | Mr, Nazeer S/O Abdul Ghani |
| 41 | Left Marginal Bund | 41000-42000 | Mr, Bilal S/O Abdul Ghani |
| 42 | Left Marginal Bund | 41000-42000 | Mr, Muhammad Aslam Caste Oad |
| 43 | Left Marginal Bund | 41000-42000 | Mr, Munir Ahmed Oad |
| 44 | Colony Ring Bund | 2-3/R | Mr, M. Rafique S/O Bahadar Caste Dingrach R/O Mouza Dhoin |
| 45 | Colony Ring Bund | 2-3/R | Muammad Rafique S/O Bahadar Qaum Dingrach |
| 46 | Colony Ring Bund | 2-3/R | Ghulam Muhammad S/O Sultan Qaum Dingrach |
| 47 | Colony Ring Bund | 2-3/R | Zafar S/O Sultan Qaum Dingrach |
| 48 | Colony Ring Bund | 2-3/R | Muzafar S/O Sultan Qaum Dingrach |
| 49 | Colony Ring Bund | 3-4/R | Begam Zouja Sultan Dingrach Mouza |
| 50 | Colony Ring Bund | 3-4/R | Khuda Baksh S/O Muhammad Baksh Malang |
| 51 | Colony Ring Bund | 3-4/R | Mushtaq S/O Ghulam Fareed |
| 52 | Colony Ring Bund | 3-4/R | Arif S/O Nawaz Mouza Dhuin Muhammad |
| 53 | Colony Ring Bund | 5-6/R | Abdul Rasheed /Ramzan Muslim Sheikh |
| 54 | Colony Ring Bund | 5-6/R | Ilyas So/ Mummad Waryam Qaum Changar |
| 55 | Colony Ring Bund | 5-6/R | Waris S/O Allah Bakhsh Qaum Changar |
| 56 | Colony Ring Bund | 5-6/R | Manzoor S/O Maji Muhammad Qaum Changar |
| 57 | Colony Ring Bund | 5-6/R | Palwan S/O Ghulam Muhammad Qaum Rabani |
| 58 | Colony Ring Bund | 6-7/R | Haji Dalmeer S/O Maji Muhammad Qaum Changar |
| 59 | Colony Ring Bund | 7-8/R | Muhammad Riaz / Muhammad Nawaz |
| 60 | Colony Ring Bund | 7-8/R | Saeed S/O Muhammad Bukhsh Cast Jhatta |
| 61 | Colony Ring Bund | 7-8/R | Shabbir Caste Jhatta |
| 62 | Colony Ring Bund | 7-8/R | Ghazanfer Abbas S/O Jewan Raja |
| 63 | Colony Ring Bund | 7-8/R | Liaqat Raja S/O Talib Hussain |
| 64 | Colony Ring Bund | 7-8/R | Muhammad Ilyas Caste Khokha |
| 65 | Colony Ring Bund | 2-3/R | Iqbal S/O Allah Yar |
| 66 | Rangpur Flood Bund | 268500/L | Mr, Khizer Kumhar, Ahmed Nawaz, Pappu Machi & Naeem Kharal |
| 67 | Rangpur Flood Bund | 283-284/L | Mr, Iqbal S/O Haider caste Jhatial 3-L Ali Sarwani |
| 68 | Rangpur Flood Bund | 299-300/L | Mr, Sher Muhammad Mulana Mouza Mulana |
| 69 | Rangpur Flood Bund | 307-308 | Mr, M.Ismail S/O Allah Yar Kheera Mouza Fatu Halkana |
| 70 | Dadal Flood Bund | 0-1000/L | Mr, Bashir Ahmed S/O Pehlwan Near Adda Ghayalpur |
| 71 | Dadal Flood Bund | 3000-4000/L | Mr, Mustafa S/O Manzoor Hussain Cast Baloach |
| 72 | Dadal Flood Bund | 9-10/L | Mr, Ramzan S/O Ameer Caste Khokhar |
| 73 | Dadal Flood Bund | 14-15/L | Mr, Mustafa S/O Shafi Caste Khokhar Mouza Daraz |
| 74 | Dadal Flood Bund | 14-15/R | Mr, Riaz S/O Allah Bukhsh Caste Thiraj Mouza Daraz |
| 75 | Dadal Flood Bund | 16-17/L | Mr, Akram S/O Allah Ditta Caste Arayein Mouza PAR |
| 76 | Dadal Flood Bund | 19-20/L | Sharif S/O Daim Caste Choan Pir Abdur Rehman |
| 77 | Dadal Flood Bund | 20-21/R | Mr, Haq Nawaz S/O Makal Caste Bharwana PAR |
| 78 | Dadal Flood Bund | 21-22/L&R | Mr, Maznoor Hussain S/O Sher Caste Luck Pir Abdur Rehman |
| 79 | Dadal Flood Bund | 21-22/L&R | Mr, Muhammad Hussain S/O Ramzan Caste Luck PAR |
| 80 | Dadal Flood Bund | 21-22/L&R | Mr, Allah Ditta S/O Shamma Caste Luck Pir Abdur Rehman |
| 81 | Dadal Flood Bund | 22-23/L&R | Mr, Iqbal S/O Ramzan Caste Luck Pir Abdur Rehman |
| 82 | Dadal Flood Bund | 22-23/L&R | Mr, Khan S/O Gul Muhammad Caste Luck Pir Abdur Rehman |
| 83 | Dadal Flood Bund | 22-23/L&R | Mr, Mushtaq Ahmed S/O Pathan Caste Luck PAR |
| 84 | Dadal Flood Bund | 22-23/L&R | Mr, Bahali S/O Nawaz Caste Luck Pir Abdur Rehman |
| 85 | Dadal Flood Bund | 27-28/R | Mr, Irshad S/O Ameer Caste Maitla Pir Abdur Rehman |
| 86 | Dadal Flood Bund | 28-29/R | Mr, Ramzan S/O Rajab Caste Choan Pir Abdur Rehman |
| 87 | Dadal Flood Bund | 28-29/R | Mr, Manzoor Hussain S/O Sher Caste Luck Pir Abdur Rehman |
| 88 | Dadal Flood Bund | 29-30/L&R | Mr, Kazim S/O Ahmed caste Choan Pir Abdur Rehman |
| 89 | Dadal Flood Bund | 29-30/L&R | Mr, Qaiser S/O Nawaz caste Choan Pir Abdur Rehman |
| 90 | Dadal Flood Bund | 29-30/L&R | Mr, Ghulam,Ramzan S/O Baig Caste Sanmpal |
| 91 | Dadal Flood Bund | 29-30/L&R | Mr, Hayat S/O Allah Bukhsh Pir Abdur Rehman |
| 92 | Rajbana Flood Bund | 1-2/R | Mumtaz S/O Nawab |
| 93 | Rajbana Flood Bund | 2-3/R | Tahir Mehdi S/O Rab Nawaz |
| 94 | Rajbana Flood Bund | 2-3/R | Iqbal S/O Talib Hussain |
| 95 | Rajbana Flood Bund | 2-3/R | Tasawar S/O Mulazim Hussain |
| 96 | Rajbana Flood Bund | 2-3/R | Mumtaz Hussain S/O Rab Nawaz |
| 97 | Rajbana Flood Bund | 2-3/R | M.Murad S/O Hasta |
| 98 | Rajbana Flood Bund | 12-13/R | Mumtaz S/O Ghulam |
| 99 | Link Flood Bund | 6-7/L | M.Imran S/O Khadim Hussain |
| 100 | Link Flood Bund | 6-7/L | Bashir Ahmad S/O Sher |
| 101 | Link Flood Bund | 6-7/R | Ahmad S/O Sultan |
| 102 | Link Flood Bund | 7-8/R | Mazhar Abbas S/O Allah Bakhsh |
| 103 | Link Flood Bund | 7-8/R | Safdar Abbas S/O Muhammad Zafar |
| 104 | Link Flood Bund | 7-8/R | Ramzan S/O Allah Ditta |
| 105 | Link Flood Bund | 7-8/L | Safdar Abbas S/O Muhammad Zafar |
| 106 | Link Flood Bund | 7-8/L | Bashir Ahmad S/o Muhammad Fazal |
| 107 | Link Flood Bund | 7-8/L | Allah Ditta S/O Haq Nawaz |
| 108 | Link Flood Bund | 7-8/L | Bashir S/O Abdul Rehman |
| 109 | Link Flood Bund | 8-9/L | Allah Ditta S/O Sultan |
| 110 | Link Flood Bund | 9-10/L | Mahar Noor S/O Mahar Allah Ditta |
| 111 | Link Flood Bund | 9-10/L | M.Nawaz S/O Mahar Nusrat |
| 112 | Link Flood Bund | 10-11/L | Bashir S/O Ghulam |
| 113 | Link Flood Bund | 11-400/R | Gull Muhammad S/O Allah Bakhsh |
| 114 | Link Flood Bund | 11-400/L | Mahar Ijaz S/O Pehlwan |
| 115 | Lakhi Flood Bund | 28-29/R | Noor S/O Ghulam Fareed |
| 116 | Lakhi Flood Bund | 40-41/R | Ghulam Shabir S/O Ghulam Fareed |
| 117 | Lakhi Flood Bund | 42-43/R | Sarang Ali S/O Haji Baig |
| 118 | Lakhi Flood Bund | 54-55/R | Riaz S/O M. Shafi |
| 119 | Hassuwali Flood Bund | 26-27/L | Ansar S/O Sultan |
| 120 | Hassuwali Flood Bund | 26-27/R | Malang s/O Bahadar Ali |
| 121 | Hassuwali Flood Bund | 26-27/R | Haji Bashir S/O Noor Muhammad |
| 122 | Hassuwali Flood Bund | 28-29/R | Sharafat Ali S/O Bashir |
| 123 | Hassuwali Flood Bund | 26-27/R | Mumtaz S/O Shahmand |
| 124 | Hassuwali Flood Bund | 32-33/R | Nasir S/O Hashim |
| 125 | Hassuwali Flood Bund | 38-39/R | Afzal S/O Jaffar Sial |
| 126 | Hassuwali Flood Bund | 63-64/R | Ghulam Abbas S/O Haji |

Chapter - 09

DUTY ROSTER / FLOOD FIGHTING PROGRAM

## Phase I - (Pre-Flood Period)

This phase covers the pre-flood period starting from start of the flood season upto the time the flood water raises upto 1 foot above general Natural Surface Level (NSL) along the flood bunds.

##### WATCHING:	 Flood watching establishment will be arranged according to flood scales / stages after consultation with the concerned authority and other essential need at site.

| ## Stage | Strength |
| --- | --- |
| Water level is 1 foot above NSL or below along the flood bund | 1 watchman per mile per shift in two shifts during 24 hours on Marginal and other flood bunds |
| Water level is more than1 foot above NSL along the flood bund | 1 watchman per mile per shift in three shifts during 24 hours on Marginal and other flood bunds |

During this phase the Sub Divisional officers of the bunds and respective Sub Engineers of the sections of the bunds will be the controlling officers under the overall supervision of the Executive Engineer, Trimmu Headworks Division, Trimmu.

Phase II – (Low Flood i.e. 1.50 to 2.00 Lac cusecs)

This phase starts when the discharge at the Trimmu Headworks is between 1.5 to 2 Lac cusecs. The details of the actions to be taken during this phase are as follows:

| ## Activity | Action |
| --- | --- |
| Shifting of watching & necessary flood material to watching huts | Respective Sub Engineers, SDO, XEN. |

| ## Stage | Strength |
| --- | --- |
| The discharge ranges between 1.5 to 2.0 lac cusecs | 2 persons per mile per shift in three shifts during 24 hours on Marginal and other flood bunds 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

The duty roster during this phase of the flood, officers/officials duties are tabulated below. The officials under the overall control of the sub-divisional officers concerned will perform the duties.

| Sr. No. | ## Location | Duty of Officer/Official |
| --- | --- | --- |
| 1 | Barrage and Headworks | Sub Engineer Headworks |
| 2 | Right Marginal Bund ( RD 0 – 54 ) J Head Spur ( RD 0 – 10 ) | Sub Engineer Headworks |
| 3 | Left Marginal Bund (RD 0 – 46) along with wetting Channel & Guide Wall No.1 & 2 | Sub Engineer River Survey |
| 4 | Colony Ring Bund along with - T- Cum Hockey Spur - T Head No I and II Spur - Makori Spur. - Guide Wall No.3 | Sub Engineer Station Area |
| 5 | Hassuwali Flood Bund (RD 0-24 ) Hassuwali Flood Bund (RD 24-48 ) Hassuwali Flood Bund (RD 48-72 Tail) | Zilladar, Shorkot Sub Engineer, Jarala Sub Engineer, Farid Mehmood |
| 6 | Rajbana Flood Bund (0 – tail) Link Bund (0 – tail) | Sub Divisional officer Shorkot Sub Engineer, Dauranpur Sub Engineer, Kakki |
| 7 | Lakhi Flood Bund (13 – 45) Lakhi Flood Bund (45 – 68 tail) | Sub Engineer Rajbana Zilladar Dauranpur |
| 8 | Dadal Flood Bund & Embankment of 1L/3L Dadal Flood Bund (RD 0 – 25 ) Dadal Flood Bund (RD 25 – tail ) Embankment of 1L/3L (RD 5 – 15380) | Sub Divisional Officer, 18-Hazari Sub Division (Overall Incharge) Sub Engineer Pir Abdur Rehman Zilladar Ahmad pur Sub Engineer Kapuri Section |
| 9 | Rangpur Flood Bund Rangpur Flood Bund ( RD 260 - 320) Rangpur Flood Bund ( RD 304 - 318) | Executive Engineer, Trimmu Headworks Division, Trimmu (Overall Incharge) Sub divisional Officer Athara Hazari Subdivision (Head quarter at RD 292) Sub Engineer Rangpur Sub Engineer Diana |
| 10 | Head Mohlanwala Creek | Sub Engineer River Survey |
| 11 | Mole Head Gryne with 3 No. Solid Stone Studs at Dab Kalan including protection bund and defence bund | Sub Engineer, Rajbana. |

Phase III – (Medium Flood i.e. 2.00  to 3.00 Lac cusecs)

This phase starts when the discharge at the Trimmu Barrage crosses the medium level and is upto high level flood i.e. between 2.00 to 3.00 Lac cusecs. The details of the actions to be taken during this phase are as follows:

| ## Stage | Strength |
| --- | --- |
| The discharge ranges between 2.0 to 3.0 lac cusecs | 4 persons per mile per shift in three shifts during 24 hours on Marginal and other flood bunds. 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

The duty roster during this phase of the flood, officers/officials duties are tabulated below. The officials under the overall control of the concerned Sub Divisional officers will perform the duties.

| Sr. No. | ## Location | Duty of Officer/Official |
| --- | --- | --- |
| 1 | Barrage and Headworks/Guide Bunds | Sub Divisional Officer, Headworks Sub Engineer, Headworks |
| 2 | Right Marginal Bund ( RD 0 – 54 ) J Head Spur ( RD 0 – 10 ) | Sub Engineer, Headworks. |
| 3 | Left Marginal Bund (RD 0 – 46) along with wetting Channel & Guide Wall No. 1 & 2 | Sub Engineer, River Survey |
| 4 | - Colony Ring Bund along with i) T- Cum Hockey Spur ii) T Head No I and II Spur iii) Guide Wall No.III - Makori Spur | Sub Engineer, Station Area Sub Engineer, Station Area Sub Engineer, Station Area Sub Engineer, Station Area Sub Engineer, River Survey |
| 5 | Hassuwali Flood Bund (RD 0-72 Tail) | Sub Enginerr Farid Mehmood |
| 6 | Rajbana Flood Bund (0 – tail) Link Bund (0 – tail) | Sub Engineer, Dauranpur |
| 7 | Lakhi Flood Bund (0 – tail) | Sub Engineer Rajbana Zilladar Shorkot |
| 8 | Dadal Flood Bund (RD 0 – 25 ) Dadal Flood Bund (RD 25 – tail ) Embankment of 1L/3L (RD 5 – 15380) | Sub Engineer Pir Abdur Rehman Sub Engineer Lashari Sub Engineer Lashari |
| 9 | Head Mohlanwala Creek | Sub Engineer River Survey |

| 10 | Rangpur Flood Bund ( RD 292) Rangpur Flood Bund ( RD 260 - 320) Rangpur Flood Bund ( RD 304 - 318) | Sub divisional Officer AtharaHazari Sub Engineer Rangpur Sub Engineer Diana |
| --- | --- | --- |
| 11 | Mole Head Gryne with 3 No. Solid Stone Studs at Dab Kalan including protection bund and defence bund | Sub Engineer, Rajbana |

Phase IV – ( High Flood i.e. 3.00  to 4.50 Lac cusecs)

This phase starts when the discharge at the Trimmu Barrage is between 3.00 to 4.50 Lac cusecs. The details of the actions to be taken during this phase are as follows:

| ## Activity | Action |
| --- | --- |
| Requisition for 100 Army Jawans | XEN |

| ## Stage | Strength |
| --- | --- |
| The discharge ranges between 3.0 to 4.5 lac cusecs | 6 persons per mile per shift in three shifts during 24 hours on Marginal and other flood bunds 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

The duty roster during this phase of the flood, officers/officials duties are tabulated below. The officials under the overall control of the Sub Divisional Officers concerned will perform the duties.

| Sr. No. | ## Location | Duty of Officer/Official |
| --- | --- | --- |
| 1 | Barrage and Headworks/ Guide Bunds | Executive Engineer Sub Divisional Officer, Headworks Sub Engineer, Headworks |
| 2 | Right Marginal Bund ( RD 0 – 30 ) - J Head Spur ( RD 0 – 10 ) Right Marginal Bund ( RD 30 – 54 ) | Sub Engineer, Leave Reserve (Head Quarter at RD 23) Sub Engineer, Kapuri (Head Quarter at RD 45) |
| 3 | Left Marginal Bund along with wetting Channel ( RD 0 - 20 ) Left Marginal Bund along with wetting Channel ( RD 20 - 46 ), Guide Wall No.I&II | Sub Engineer River Survey (Head quarter at RD 20) Sub Engineer, Hydraulic (Head quarter at RD 36) |
| 4 | 1. Colony Ring Bund along with - T- Cum Hockey Spur - T Head No I and II Spur - Guide Wall No.III 2. Makori Spur | Clerical Staff of Trimmu Division Deputy Collector, Trimmu Sub Engineer, Station Area Sub Engineer, River Survey |
| 5 | Hassuwali Flood Bund (RD 0-72 Tail) | Sub Enginerr Farid Mehmood |
| 6 | Rajbana Flood Bund (0 – tail) Link Bund (0 – tail) | Sub Engineer, Dauranpur |
| 7 | Lakhi Flood Bund (0 – tail) | Sub Engineer Rajbana Zilladar Shorkot |
| 8 | Dadal Flood Bund (RD 0 – 25 ) Dadal Flood Bund (RD 25 – tail ) Embankment of 1L/3L (RD 5 – 15380) | Sub Engineer Pir Abdur Rehman Sub Engineer Lashari Sub Engineer Lashari |
| 9 | Rangpur Flood Bund ( RD 292) Rangpur Flood Bund ( RD 260 - 320) Rangpur Flood Bund ( RD 304 - 318) | Subdivisional Officer AtharaHazari Sub Engineer Rangpur Sub Engineer Diana |
| 10 | Head Mohlanwala Creek | Sub Engineer River Survey |
| 11 | Mole Head Gryne with 3 No. Solid Stone Studs at Dab Kalan including protection bund and defence bund | Sub Engineer, Rajbana |

Phase V – (Very High Flood i.e. 4.50  to 6.00 Lac cusecs)

This phase starts when the discharge at the Trimmu Barrage is in very high flood stage i.e. between 4.50 to 6.00 Lac cusecs. During this phase special attention will be given to watch most vulnerable reaches of the bunds, concerned officer incharge is responsible that these points are especially watched. The details of the actions to be taken during this phase are as follows:

| ## Activity | Action |
| --- | --- |
| Requisition for 200 Army Jawans | XEN |

## Watching

| ## Stage | Strength |
| --- | --- |
| The discharge ranges between 4.50 to 6.00 lac cusecs | 8 persons per mile per shift in three shifts during 24 hours on Marginal and other flood bunds 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

The duty roster during this phase of the flood, officers/officials’ duties are tabulated below. The officials under the overall control of the Sub Divisional Officers concerned will shift their headquarters and will perform the duties.

| Sr. No. | ## Location | Duty of Officer/Official |
| --- | --- | --- |
| 1 | Barrage and Headworks/ Guide Bunds | Executive Engineer Sub Divisional Officer, Headworks Sub Engineer, Headworks |
| 2 | Right Marginal Bund Right Marginal Bund ( RD 0 – 54 ) - J Head Spur ( RD 0 – 10 ) | Sub Divisional Officer, T.S. Link Overall Incharge (Headquarter at RD 37) Sub Engineer, Leave Reserve Sub Engineer, Leave Reserve |
| 3 | Left Marginal Bund Left Marginal Bund along with wetting Channel ( RD 0 - 20 ), Guide Wall No.I&II Left Marginal Bund along with wetting Channel ( RD 20 - 46 ) | Sub Divisional Officer, Old Sidhnai, Overall Incharge (Headquarter at RD 42) Sub Engineer River Survey Sub Engineer, Hydraulic |
| 4 | 1. Colony Ring Bund along with - T- Cum Hockey Spur - T Head Spur No I iii) T Head Spur No II iv) Guide Wall No.III - Makori Spur | Clerical Staff of Trimmu Division Deputy Collector, Trimmu Silt Analyst Sub Engineer, Station Area Sub Engineer, Station Area Sub Engineer, River Survey |
| 5 | Hassuwali Flood Bund (RD 0-72 Tail) | Sub Enginerr Farid Mehmood |
| 6 | Rajbana Flood Bund (0 – tail) Link Bund (0 – tail) | Sub Engineer, Shorkot |
| 7 | Lakhi Flood Bund (0 – tail) | Sub Engineer Rajbana Zilladar Shorkot |
| 8 | Dadal Flood Bund (RD 0 – 25 ) Dadal Flood Bund (RD 25 – tail ) Embankment of 1L/3L (RD 5 – 15380) | Sub Engineer Pir Abdur Rehman Sub Engineer Lashari Sub Engineer Lashari |
| 9 | Rangpur Flood Bund ( RD 292) Rangpur Flood Bund ( RD 260 - 320) Rangpur Flood Bund ( RD 304 - 318) | Subdivisional Officer AtharaHazari Sub Engineer Rangpur Sub Engineer Diana |
| 10 | Head Mohlanwala Creek | Sub Engineer River Survey |
| 11 | Mole Head Gryne with 3 No. Solid Stone Studs at Dab Kalan including protection bund and defence bund | Sub Engineer, Rajbana |

Phase VI – (Exceptionally High Flood i.e. more than 6.00 Lac cusecs)

This phase commences when the discharge at the Trimmu Barrage is in exceptionally high flood stage i.e. more than 6.00 Lac cusecs. The details of the actions to be taken during this phase are as follows:

| ## Activity | Action to be taken by |
| --- | --- |
| Deputy Commissioner is informed to make arrangement for the evacuation of the marooned public within LMB and RMB Military Authorities to keep ready for emergent operation Police to watch that no un authorized cut is made by public especially all along LMB, RD 0 – 15 of RMB. Engineers 4 Corps for energizing the breaching section, when water level approaches R.L 500 mark at the emergency gauge installed at RD 15 of LMB. Arrangement for 200 Volunteers to cut RMB at Breaching section in case the blasting arrangement doesn’t work. | Executive Engineer Executive Engineer Executive Engineer Executive Engineer District Administration & Zilladar 18 Hazari Sub division. |

## Watching

| ## Stage | Strength |
| --- | --- |
| The discharge more than 6.0 Lac cusecs | 8 persons per mile per shift in three shifts during 24 hours on Marginal and other flood bunds 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

The duty roster during this phase of the flood, officers/officials’ duties are tabulated below. The officials under the overall control of the Sub Divisional Officers concerned will shift their headquarters and will perform the duties.

| Sr. No. | ## Location | Duty of Officer/Official |
| --- | --- | --- |
| 1 | Barrage and Headworks/ Guide Bunds | Executive Engineer, Trimmu Barrage Sub Divisional Officer, Headworks Sub Engineer, Headworks |
| 2 | Right Marginal Bund Right Marginal Bund ( RD 0 – 30 ) - J Head Spur ( RD 0 – 10 ) Right Marginal Bund ( RD 0 – 30 ) Right Marginal Bund ( RD 30 – 54 ) | Sub Divisional Officer, T.S. Link Overall Incharge (Headquarter at RD 37) Sub Engineer, Yasmin Sub Engineer, Yasmin Sub Engineer, Leave Reserve Sub Engineer, Kapuri |
| 3 | Left Marginal Bund along with wetting channel Left Marginal Bund along with wetting Channel ( RD 0 - 20 ) Left Marginal Bund along with wetting Channel ( RD 20 – 46 ) | Sub Divisional Officer, Old Sidhnai, Overall Incharge (Headquarter at RD 42) Sub Engineer River Survey Sub Engineer, Hydraulic |
| 4 | - Colony Ring Bund along with - T- Cum Hockey Spur - T Head Spur No I - T Head Spur No II - Guide Wall No.I&2 - Makori Spur | Clerical Staff of Trimmu Division Deputy Collector, Trimmu Silt Analyst Sub Engineer, Station Area Sub Engineer, River Survey |
| 5 | Hassuwali Flood Bund (RD 0-24 ) Hassuwali Flood Bund (RD24-48 ) Hassuwali Flood Bund (RD 48-72 Tail) | Zilladar, Shorkot Sub Engineer, Jarala Sub Engineer, Farid Mehmood |
| 6 | Rajbana Flood Bund (0 – tail) Link Bund (0 – tail) | Sub Divisional officer Shorkot Sub Engineer, Dauranpur Sub Engineer, Kakki |
| 7 | Lakhi Flood Bund (13 – 45) Lakhi Flood Bund (45 – 68 tail) | Sub Engineer Rajbana Zilladar Shorkot |
| 8 | Dadal Flood Bund & Embankment of 1L/3L Dadal Flood Bund (RD 0 – 25 ) Dadal Flood Bund (RD 25 – tail ) Embankment of 1L/3L (RD 5 -15380) | Sub Divisional Officer, 18-Hazari Sub Division Garh More (Overall Incharge) Sub Engineer Pir Abdur Rehman Zilladar Ahmad pur Sub Engineer Kapuri Section |
| 9 | Rangpur Flood Bund Rangpur Flood Bund ( RD 260 - 320) Rangpur Flood Bund ( RD 304 - 318) | Executive Engineer, Trimmu Headowkrs Division Trimmu (Overall Incharge) Sub divisional Officer Athara Hazari Sub Division (Head quarter at RD 292) Sub Engineer Rangpur Sub Engineer Diana |
| 10 | Head Mohlanwala Creek | Sub Engineer River Survey |
| 11 | Mole Head Gryne with 3 No. Solid Stone Studs at Dab Kalan including protection bund and defence bund | Sub Engineer, Rajbana |

In case of breach in the Right Marginal bund, Sub Engineer Lashari will look after the Rangpur Canal and manage cuts and cross bunds in Rangpur canal at RD 20-30, 40-46 and 70-80. Sub Engineer Dharkana will watch Haveli main Line from RD 184 – tail.

## Breach Closing Squad

When the discharge will exceed 4.00 Lac cusecs, the mobile units consisting of following items are to be raised in each sub division. The Tip Top condition of Pick up and Tractor Trolley will be ensured.

Stack ballies 16-foot-long			= 140 No.

Killas						= 200 No.

Munj Ban Course				= 140 Kg

Showels					= 50 No.

Gunny bags					= 700 No.

Sutli						= 3 Kg

Needles					= 28 No.

Baskets					= 350 No.

In Headworks Sub Division, one unit will be kept on Left Marginal Bund and one at Right marginal Bund. In Shorkot Sub division, one unit will be kept at Lakhi Flood Bund, one at Hassuwali Flood Bund along with Rajbana Flood Bund and connecting tail Rajbana Flood Bund.Whereas in 18 Hazari Sub division two units will be deployed – one at Dadal Flood Bund and one at Rangpur Flood Bund.

A gang of 10 persons and a mate will move along with the unit at a moment notice. At the same time the Army Authorities will also be requested to provide manpower for closing leakage/breach.

CHAPTER-10

EMERGENCY TELEPHONE NUMBERS

| # # | # DESIGNATION | # OFFICE |  | # RESIDENCE |
| --- | --- | --- | --- | --- |
| # 1. | # Secretary to Government of the Punjab, Irrigation Department, Lahore. | # 042-99212117 042-99212118 |  | 042-35731116 |
| # 2 | # Secretary South Punjab Irrigation Department Bahawal pur | # 062-9250544 |  | # - |
| # 3. | # P.S. to Secretary IRR: Lahore. | # 042-99212116 042-99212117 042-99212118 |  | # 042-5821839 |
| # 4. | # Additional Secretary (Tech:) IRR Department Lahore. | # 042-99212128 |  | # 042-37841123 |
| # 5. | # Additional Secretary (Admn) IRR department Lahore. | # 042-99212119 |  | # 042-7561576 |
| # 6. | # Deputy Secretary (Admn) | # 042-99212126 |  | # 042-7575766 |
| # 7. | # Deputy Secretary (General) | # 042-99212125 |  | # 042-5725896 |
| # 8. | # Deputy Secretary Development) | # 042-99212129 |  | # 042-7586597 |
| # 9. | # Deputy Secretary (Power) | # 042-99212129 |  | # 042-6826748 |
| # 10. | # Deputy Secretary (Operation) | # 042-99212127 |  | # 042-891579 |
| # 11. | # Officer on Special Deputy | # 042-99212130 |  | # 042-7573504 |
| # 12. | # U.S/S.O, (Floods) flood Emergency Officer | # 042-99212134 |  |  |
| # 13. | # Deputy Director, P & D under Secretary | # 042-99212131 |  | # 042-6853264 |
| # 14. | # S.O. General | # 042-99212133 |  |  |
| # 15. | # S.O. (V) | # 042-99214021 |  |  |
| # 16. | # Telegraph office Irrigation LHR | # 042-99212166 |  |  |
| # 17. | # Chief Engineer D & F Zone Lahore. | # 042-99230602 Fax: 042-35862081 |  | # 0300-4877257 |
| # 18 | # Director Flood D&F Zone LHR | # 042-99231614 |  |  |
| # 19. | # Executive Engineer, Irrigation Jhang. | # 047-9200300 |  | # 0321-6095101 |
| # 20. | # XEN Rasool Barrage | # 0546/553211 |  | # 0340-7159459 |
| # 21. | # XEN Qadirabad Barrage | # 054-7550190 |  | # 0300-1030530 |
| # 22. | # Chief Engineer Mangla Dam | # 054-4639161 Fax 639162 |  |  |
| # 23 | # XEN Marala | # 052-3502102 |  | # 0346-4132454 |
| # 24 | # XEN Khanki | # 055-6266239 |  | # 0331-8484303 |
| # 25. | # D.C. Jhang. | # 047-9200081 |  | # 047-7625881 047-7612437 |
| # 26. | # District Session Judge Jhang |  | # 047-9200092 |  |
| # 27. | # Addl Distt Session, Jhang |  | # 5372 |  |
| # 28. | # Senior Civil Judge, Jhang. |  | # 047-9200333 |  |
| # 29. | # Divisional Engineering phones |  | 622999 & 624299 |  |
| # 30. | # Superintending Engineer, WAPDA, Jhang |  | # 047-9200228 |  |
| # 31 | # XEN WAPDA Jhang. |  | # 047-9200224 | # 03701813310 |
| # 32 | # S.D.O. WAPDA Jhang |  | # 047-9200227 | # 03701813342 |

| # 33. | # Executive District Officer # (Revenue Jhang) | # 047-7612010 |  | # 047-7624420 |
| --- | --- | --- | --- | --- |
| # 34. | # District Officer Revenue Jhang. | # 047-920012 |  | # 047-7612010 |
| # 35. | # Deputy District Officer (Revenue Jhang) | # 047-9200433 |  | # 047-7612697 |
| # 36. | # 314-Assault Engineers Battalion Gujranwala Cantt: | # 055-269-35188 |  | # 0321-2200314 |
| # 37. | # 25 Engineers Battalion Gujranwala Cant. | # 055-2694146 |  | # 055-2694144 055-2695184 |
| # 38. | # Head Quarter Engineers 1 Crops Kharian Cantt (Tel: Miltary) | # 05771-592353 |  | # 05771-592353 |
| # 39. | # XEN Balloki Barrage Division | # 049-4610300 |  |  |
| # 40. | # Assistant Agricultural Engineering | # 625668 |  |  |
| # 41. | # Sub Divisional Officer T.S. Link Shorkot Cantt:. | # 047-5501954 |  | # 047-5501954 |
| # 42. | # XEN operation office of the Chief Engineer, Faisalabad Irrigation Zone Faisalabad | # 041-9200270 |  |  |
| # 43. | # Chief Engineer, Sargodha Zone Sargodha | # 048-9230446 |  |  |
| # 44. | # Chief Engineer, Bhalwalpur Zone Bahawalpur | # 062-9250265 Fax: 9250263 |  |  |
| # 45. | # Chief Engineer, Research Punjab Lahore. | 042-9212091 & 9212092 Fax 9212094 |  |  |
| # 46. | # Director Flood Forecasting Bearue | # 7586479 |  |  |
| # 47. | # Deputy Director Advertisement Lahore. | # 042-9201395 , 9201396 9201397 |  |  |
| # 48. | # Relief Commissioner Lahore | # 042-9210780 & 9210781 Fax 9210555 |  |  |
| # 49. | # Chairman, Federal Flood Commission, Lahore. | # 042-9206589 042-9221805 |  |  |
| # 50. | # Director Flood / Sec: Punjab Flood Commission | # 042-99231614 |  | # 0300-8442724 |
| # 51. | # Flood warning Center 46-Jail Road, Lahore. | # 042-7572091-93 |  |  |
| # 52 | # Assistant Commissioner Ahmed Pur Sial | # 047-534111 |  |  |
| # 53 | # Assistant Commissioner 18-Hazari | # 047-7645732 |  |  |
| # 54 | # Assistant Commissioner Jhang | # 047-9200733 |  |  |
| # 50 | # Assistant Commissioner SKT | # 047-5310171 |  |  |
| # CHIEF ENGINEER, IRRIGATION MULTAN ZONE MULTAN |  |  |  |  |
| # 1. | # Chief Engineer | # 061-9200144 |  | # - |
| # 2. | # Executive Engineer Operation | # 061-9200139 # Fax 9200133 |  | 061-584303 |
| # 3. | # Assistant Design Engineer | # 061-9200143 |  | # - |
| # 4. | # Administrative officer | # 061-9200141 |  | # - |
| # 5. | # Zonal Accounts Officer | # 061-9200140 |  | # - |
| # 6. | # Telegraph office | # 061-9200922 |  | # - |
| # TRIMMU CANAL CIRCLE, TRIMMU |  |  |  |  |
| # 1. | # Superintending Engineer Trimmu Canal Circle, Trimmu | 0300-8696527 |  | # 047-7647051 |
| # 2. | # XEN, Trimmu Headworks Division, Trimmu | # 047-7647200 |  | # 0300-0762038 |
| # 3. | # XEN, Sidhnai Headworks Division, Sidhnai | # 0312-7823014 |  | # - |
| # 4. | # XEN, Northern Tubewell Division (NTD),Multan | # 0333-7057024 |  |  |
| # 5. | # S.D.O, Head works Sub Division, Trimmu | # 0316-6224684 |  |  |
| # 6. | # S.D.O, Shorkot Sub Division, Shorkot | # 0318-4611878 |  |  |
| # 7. | # S.D.O, 18-Hazari Sub Division, Garh More | # 0304-7879774 |  |  |
| # HAVELI CANAL CIRCLE MULTAN |  |  |  |  |
| # 1. | # Superintending Engineer Haveli Canal Circle, Multan |  | 061-9200136 | # 061-9200300 |
| # 2. | # XEN, Multan Canal Division Multan |  | # 061-9200679 |  |
| # 3 | # XEN, Shujabad Canal Division Multan. |  | # 061-9200699 | # 061-580184 |
| # MAILSI CANAL CIRCLE MULTAN |  |  |  |  |
| # 1. | # Superintending Engineer |  | # 9200134 Fax: 9200934 | # 061-9200142 |
| # 2. | # XEN, Lodhran Division |  | # 06089200174 | # 0608-511549 |
| # 3. | # XEN, Lower Western Bar |  |  |  |
| # 4 | # XEN, Mailsi Syphon |  |  |  |
| # OTHERS |  |  |  |  |
| # 1 | # XEN ,C.J. Link Kundian |  | # 045202-41289 |  |
| # 2 | # District Attorney Jhang |  | # 047-9200076 |  |
| # 3 | # Deputy Commissioner Jhang |  | # 047-79200081 |  |
| # 4 | # District Police Officer, Jhang |  | # 047-9200444 |  |
| # 5 | # A.S.P. Jhang |  | # 047-9200446 |  |
| # 6 | # D.S.P Sadar Jhang |  | # 047-9200060 | # 047-7613402 |
| # 7 | # P.D.S.P Jhang |  | # 047-9200060 |  |
| # 8 | # Police Station (Saddar) Jhang |  | # 047-9200313 |  |
| # 9 | # D.S.P (H.Q) |  | # 047-9200082 |  |

CHAPTER-11

STANDARD OPERATION PROCEDURE (SOP) FOR BREACHING SECTIONS

11.1	HISTORY OF THE BREACHING SECTION

In order to protect the Trimmu Barrage and its allied training works from the           on slaught of the floods, a breaching section was proposed on Right Marginal Bund between RD: 16000-18000 in the first instance. However, keeping in view the breaches made during the previous floods, a working paper was prepared and submitted to the Punjab Flood Commission embodying therein information regarding stages for activation of the breaching section and other physical factors relevant to the flood. The Punjab Flood Commission in its 55th meeting held on 17.06.1974 deliberated on the issue in detail and decided to activate the breaching section with water level at RD: 15000 Left Marginal Bund R.L 501. Subsequently Punjab Flood Commission in its meeting held on 30.06.1977 revised the R.L at RD: 15000 of Left Marginal Bund from R.L 501 to R.L 500 authorizing the Executive Engineer, Trimmu Headworks Division Trimmu to order the activation of the breaching section in case of sound and valid reasons for the protection of the Barrage. However, the project of Rehabilitation and up-gradation of Trimmu Headworks is going underway since 1st June 2016 to enhance the design capacity of Trimmu Barrage from 645000 to 875000 Cs to minimize the chances of activation of Breaching section. The previous  history of operation of breaching section is tabulated below.

| Year | Discharge through Barrage (Cusecs) | Discharge through Relief Cut (Cusecs) | Total Discharge (Cusecs) |
| --- | --- | --- | --- |
| 1959 | 718,255 | 225,000 | 943’255 |
| 1973 | 666,910 | 66,000 | 732,910 |
| 1992 | 683,117 | 205,000 | 888,117 |
| 2014 | 626,006 | 77,000 | 703,006 |

11.2	LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION.

The breaching section of Trimmu Barrage is located at RD 16 to 18 of R.M.B. The detail of breaching section is shown in figure below.

Breaching section At Trimmu Barrage

11.3	ARRANGEMENT OF EXPLOSIVE & SECURITY OF EXPLOSIVE

Explosive store has been shifted under the supervision of army authority at 1-Core Engineering Batalian Kharian Cantt, which will be shifted at site during the flood season as per schedule. The detail of explosive requirement as well as available is tabulated below.

CURRENT STATUS OF EXPLOSIVE / ACCESSORIES

| S. No. | Name of explosive / accessories | Quantity required | Quantity available | Balance quantity required | When balance quantity will be arranged | Any previous Pending / outstanding payment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PE3A | 6886 Kg | 6886 Kg | - | - | No outstanding payment |
| 2 | Vabo Card | 9100 Mtr | 9100 Mtr | - | - |  |
| 3 | Safety Fuze No.11 | 750 Mtr | 750 Mtr | - | - |  |
| 4 | Detonating card | 6000 Mtr | 6000 Mtr | - | - |  |
| 5 | Primer CE | 2940 No. | 2940 No. | - | - |  |
| 6 | Ceiling compound | 60 Kgs | 60 Kgs | - | - |  |
| 7 | Liner 12 Ft | 156 Nos | 156 Nos | - | - |  |
| 8 | Jute | 150 Mtr | 150 Mtr | - | - |  |
| 9 | Liner 9 Ft | 108 Nos | 108 Nos | - | - |  |

11.4	LIST OF SECURITY STAFF ALONG WITH DETAIL OF THEIR TRAINING ETC

The explosive material for operation of Breaching section of Trimmu Barrage lies with Engineering Corps-I Stationed at Gujranwala Cantt. Army personal has been deputed for the security with the specialized training.

11.5	DETAIL OF MECHANICAL MEANS AS STANDBY ARRANGEMENTS IN CASE OF DETONATION FAILURE

The breaching section is activated with the help of the Army personal with explosive materials available at site. In case of the failure of this method, two No. Bulldozers and one No. excavator will be deployed to cause cuts in the Right Marginal Bund. For this purpose standby labour is also deployed. Furthermore six bulldozers have been requested to be arranged by the Deputy Commissioner, Jhang.

In the event of the activation of the breaching section, the flood waters spread surround 18-Hazari Town and travels in 3 directions as enunciated below:-

1.  	Main current hits Rangpur Canal between RD: 21000-23000.

2. 	Another current flows in creek cuts through 1-R Disty between RD: 3000 and 4000 then hits to Rangpur Canal between RD: 44000-45000.

3.	3rd current flows in depression along sand dunes and 2-R Disty up to RD: 13000 and then hits to Rangpur Canal between RD: 74000-75000 being a natural depression.

11.6	DUTY ROSTER IN CASE OF CRITICAL SITUATION

In case of critical situation, the Executive Engineer Trimmu Barrage Division and Sub Divisional Officer Head Works Sub Division will be available at the Barrage site for the coordination and Sub Divisional Officer, T.S. Link Sub Division will be available at R.M.B site as overall incharge of the site. However, the Secretary to Government of the Punjab Irrigation Department Lahore will depute officers on the flood bunds of Trimmu Barrage Division as per SOP’s during Flood days.

11.7	BREACHING COMMITTEE WITH THEIR ACTION PLAN

Breaching section shall be operated on the orders of the following committee constituted by Government of the Punjab Irrigation Department vide notification No. SO(FLOODS)VI-33-97 dated 15-07-2015.

i) Deputy Commissioner Jhang (Convener).

ii) Deputy Commissioner, Muzaffar Garh (Member).

iii) Executive Engineer, Trimmu Headworks Division, Trimmu (Member).

iv) Representative from C & W Department not below the rank of as Executive Engineer.

v) Executive Engineer concerned of the Irrigation Department.

vi) Representative from Pakistan Army not below the rank of a Lt. Colonol.

(c) 	The decision to operate a breaching section shall be taken by the committee keeping in view the technical advice of the Executive Engineer Trimmu Headworks Division, Trimmu. In case of any difference of opinion among the members, the final decision shall rest with the Deputy Commissioner Jhang.

(d)	In case of unavailability of the Deputy Commissioner, Jhang or his inability to take decision, or in case of a difference of opinion between the Deputy Commissioner, Jhang in which the breaching section is located and the district which is likely to be affected by the operation of the breaching section, the decision to operate a breaching section shall be taken by the respective Commissioner.

(e)	Breaching section shall be operated when one or more of the following conditions are met:-

(i)	The discharging capacity of the structure for which breaching section is located is exceeded and the safety of structure is threatened.

(ii)	The critical gauge i.e R.L 500 Ft as indicated in the relevant Flood Fighting Plan is achieved or exceeded and is rising.

(iii)	The safety of major population center-(s) is threatened due to high flood levels and in view of the Executive Engineer concerned of the Irrigation Department, operation of breaching section can help in reduction of flood levels.

(f)	Where, in view of the Executive Engineer concerned of the Irrigation Department or the representative of the department owning the structure at which the breaching section is located, the aforesaid criteria is expected to be met after due consideration of all available technical and hydrological conditions, the aforesaid committee may operate the breaching section before the critical gauge is achieved or exceeded. In every such case, the view of the Executive Engineer concerned of Irrigation Department or the representative of the department owing the structure shall be expressed in writing and shall be based on flood forecast by Pakistan Meteorological Department or other reliable sources, the trend of discharges in the relevant river at upstream locations, the lag times, estimated attenuation and expected inflows from tributaries, nullahs and streams en-route. Where advance breaching is planned, the committee may also consult professional experts and seek advice from the Superintending Engineer or the Chief Engineer concerned of the Irrigation Department.

(g)	Breaching section shall be operated by the contingent plan by the Chief Engineer and Pakistan Army as indicated in the Flood Fighting Plan. Explosive method shall be the preferred method for operation of breaching section. However, the Department owning the structures on which the breaching section is located, and in case of their failure to do so, Irrigation Department shall make available, in advance, necessary earth moving machinery and other equipment for affecting the breach through mechanical means in the shortest possible time.

(h)	The Deputy Commissioner concerned and all other relevant agencies shall ensure all necessary arrangements for the safety and if required evacuation of population likely to be effected by the operation of breaching section.

11.8	LIST OF VILLAGES LIKELY TO BE INUNDATED IN CASE OF BREACH / HIGH FLOOD

The water escaping through the activated breaching section will flow along Right Bank of Rangpur Canal crossing Jhang-Muzaffar Garh Road at 30 KM from Jhang. The flood water traveling along Rangpur Canal crosses through cuts at Rangpur Canal at RD: 22000, 44000 and 80000 and remaining water spreads between Right Bank of Rangpur Canal and sand dunes up to a distance of about 100 K.M from Trimmu Barrage.

The area likely to be affected / inundated by the spills. The villages / abadies and other infrastructures like road, canals etc inundated / damaged are detailed here under:-

| S# | Tehsil | Villages |
| --- | --- | --- |
| 1 | Tehsil Jhang | Dhoin Muhammad , Mansoor Sial , Diraj , Kot Khera, Mohal, Guniana, Wijhlana. Total Villages = 7 Nos. |
| 2 | Tehsil 18-Hazari | Mari Shakir, Kot Shakir, Machiwal, Sajhran , Kot Maldave, Munday Syed, Dosa, Dabi, Umran Janubi, Kamra, Baila Chotra , Lateef Shah, Basti Sanga, Loaharan Wala, Kharal Sipra, Kotla Naik Ahmed, Kotli Baqir Shah, Gari Fateh Ullah, Sheroana, Loha Bhir, Johiya, Ahmed Abas, Bela Shar, Bolla Patwana. Rashid Pur , Jbanoana, Dhan Miani, Pahar Pur, Kot Murad, Rodo Sultan, Taror, Kot Rustum, Basti Nusrat , Dhoin Muhammad, Lashari, Malkana, Total Villages = 36 |
| 3 | Tehsil Ahmed Pur Sial | Sultan Bahoo, Dull, Diraj, Pir Abdul Rehman , Samanduana. Naikokara Kotli, Mehmood Shah, Garh Maharaja , Chak 5/2-L, Tiba Gehli, Fateh Pur, Jusa, Fateh Pur Pariti, Islam Pur Nashaib, Islam Pur faraz, Chak Norang Shah, Kot Mupal, Inayat Shah. Saiwa Sadhana. Total Villages =23 |
| 4 | Tehsil Muzaffar Garh | Maqsoodpur , Dera Fazil ,Bahadaur, Amir Pur, Sargana, Kirari Ali Mardan , Kacha Nashaibi, Behram Pur ,Chbootra Mamil, Fatoo Fanaka, Dera Hayat, Bahli Shumali, Sadi Pur, Makoo Jamal Total Villages = 13 |
| TOTAL VILLAGES |  | 79 Nos |

11.9	ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENTS

The time lag between Qadirabad Barrage and Trimmu Barrage is 72 hours and Rasul Barrage to Trimmu Barrage is 72 hours. Thus, Irrigation Department has ample time to inform Civil Administration and other concerned agencies about the flood situation for warning the civil population, arranging their evacuation and adopting other protective measures. A critical gauge stands fixed at RD: 15000 of Left Marginal Bund depicting the trend of the flood heights and when the R.L. of the critical gauge touches the value as R.L. 500 if the river is still in rising position, the necessity operation of the Barrage is established on the basis of 3 hourly gauges of the upper Barrages.

The gauges received from Qadirabad Barrage and Rasul Barrage are calculated / assessed at the Barrage keeping in view the time lag and flood situation is brought to the notice of Army, Civil Administration and other relevant agencies for informing the public about the quantum of flood and its affect for making arrangement of evacuation. The operation of the breaching section will be intimated to the Army at least 12 hours in advance for making arrangements to shift the explosive to the breaching site.

Similar Civil Administration is to be informed about the operation of breaching section at least 12 hours in advance.

- The Deputy Commissioner Jhang to make arrangement for the evacuation of the public within LMB and RMB and the population coming on route of spill water.

- Military authorities to keep ready for emergency operation.

- Police to watch that no unauthorized cuts be made by public especially along LMB & RMB.

- Engineering 1-Corp for energizing the breaching section, when water level approaches 500.00 Mark at the emergency gauge installed at RD: 15000 Left Marginal Bund.

11.10	DETAIL OF COORDINATION WITH CIVIL / ARMY AUTHORITIES

There is breaching section located at RD: 16000-18000 Right Marginal Bund. A critical gauge fixed at RD: 15000 of Left Marginal Bund when its level reaches to 500.00 and is in rising position, the hourly flood gauges are passed to Civil Administration as well as Army authorities. These three Departments remain in touch through telephonic and wireless system. The breaching section will be operated with the help of Army as per standing instructions.

In the event of the activation of the breaching section, the flood waters spread surround 18-Hazari Town and travels in 3 directions as enunciated below:-

1.  	Main current hits Rangpur Canal between RD: 21000-23000.

2. 	Another current flows in creek cuts through 1-R Disty between RD: 3000 and 4000 then hits to Rangpur Canal between RD: 44000-45000.

3.	3rd current flows in depression along sand dunes and 2-R Disty up to RD: 13000 and then hits to Rangpur Canal between RD: 74000-75000 being a natural depression.

4.	Metalled Road linking Trimmu Barrage to Tehsil 18-Hazari head quarter will be disconnected. Jhang- Muzaffar Garh Road, Jhang-Bhakkar Road can be disconnected.

5.	Telephone exchanges 18-Hazari , Rodu Sultan and 2 Nos Suger Mills, Rice Mills , Petrol pumps, Government colleges, Middle and High Schools,  basic health units, grid station are likely to be disturbed.

The following officers will be informed before operating the breaching section and it is the duty of Executive Engineer, Trimmu Barrage to inform.

- The Secretary, Government of the Punjab, Irrigation Department, Lahore.

- The Chief Engineer, Irrigation Multan Zone, Multan.

- The Superintending Engineer, Trimmu Canal Circle, Trimmu.

- The Superintending Engineer, Haveli Canal Circle, Multan.

- The Chief Engineer, Drainage & Flood Zone, Lahore.

- The Flood Relief Commissioner, Lahore.

- The Chief Engineering Adviser,  Chairman Federal Flood Commission, 6-Ataturk Avenue G-5/1,Islamabad.

- The Commissioner, Faisalabad Division, Faisalabad.

- The Deputy Commissioner, District Muzaffargarh.

- The Deputy Commissioner, District Jhang.

- The Head Quarter 1-Corp Engineer, Battlion, Kharian Cantt:

- The Deputy District Officer (Revenue) Jhang/ Shorkot / 18-Hazari/Ahmedpur  Sial and Muzaffar Garh.

- The District Police offcer’s Jhang / Muzaffar Garh.

- The Sub Divisional Officer’s on watching.

- All Sub Engineer’s on duty.

- Concerned Executive Engineer of FESCO.

- The Chief Executive Officer’s Jhang / Muzaffar Garh

11.11	PARALLEL COMMUNICATION ARRANGEMENT

In addition to installation of Wireless set at Trimmu Barrage some temporary connections will be arranged at left side of the Barrage as well as field office where all the Officers / Officials will meet together to chalk out further programs. One supervisor of T & T Department will remain available at site for proper control and operation of all this system.

11.12	INDEX PLAN

Executive Engineer,		Superintending Engineer,	        Chief Engineer,

Trimmu Headworks Division	Trimmu Canal Circle, Trimmu	        Irrigation Multan Zone

Trimmu							        Multan.

Part – B

CHAPTER-12

VULNERABLE SITES ON FLOOD BUND / STRUCTURES

12.1	APPREHENDED BREACHES IN FLOOD BUNDS / STRUCTURES

| Sr. No. | Name of Flood Bund | Reach |  |
| --- | --- | --- | --- |
|  |  | From RD | To RD |
| 1 | Right Marginal Bund | 20+000 | 27+000 |
| 2 | Left Marginal Bund | 14+000 | 21+000 |
| 3 | Rangpur Flood Bund | 290+000 | 293+000 |
| 4 | Hassu Wali Flood Bund | 49+000 | 50+000 |
| 5 | Rajbana Flood Bund | 0+000 | 6+500 |

12.2	OPERATION OF BREACHING SECTION

The Breaching section of Trimmu Barrage was approved on Right Marginal Bund between RD 16000-18000. The Punjab Flood Commission in its 55th meeting held on 17.06.1974 deliberated on the issue in detail and decided to activate the breaching section with water level at RD: 15000 Left Marginal Bund R.L 501. Subsequently Punjab Flood Commission in its meeting held on 30.06.1977 revised the R.L at RD: 15000 of Left Marginal Bund from R.L 501 to R.L 500 authorizing the Executive Engineer, Trimmu Barrage Division Trimmu to order the activation of the breaching section in case of sound and valid reasons for the protection of the Barrage. They history of operation of breaching section is tabulated below.

| Year | Discharge through Barrage (Cusecs) | Discharge through Relief Cut (Cusecs) | Total Discharge (Cusecs) |
| --- | --- | --- | --- |
| 1959 | 718,255 | 225,000 | 943,255 |
| 1973 | 666,910 | 66,000 | 732,910 |
| 1992 | 683,117 | 205,000 | 888,117 |
| 2014 | 626,006 | 77,000 | 703,006 |

12.3	BREACHES DUE TO RISING OF FLOOD WATER, DETERIORATION FLOOD BUNDS ETC. 	R.R.E,	Route plans attached at Pages

No such occurrence.

CHAPTER-13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE (SITES) LISTED ABOVE EMERGENCY CONTINGENCY PLAN MUST INCLUDE

PLAN NO. 1

RD: 20+000 – 27+000 RIGHT MARGINAL BUND

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan Attached

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

- 18-Hazari

- Behllar.

- Chowkan Janpur

- Malkana

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

The employed labour, Flood Fighting Material and available machinery will be deployed on the breach for control of spilling water coming out from the breach. All efforts will be made to control the breach.

13.3.1	ARRANGEMENTS

The necessary arrangements will be made in case of breach at vulnerable sites.

13.3.2 ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD: 20+000 of R.M.B

13.3.3 DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

Sub Engineer Head Works Section will be incharge of the camp site. The following staff will be at the camp site.

Sub Engineer		=	1 No.

Mate				=	2 No.

Beldar				=	16  Nos

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

- Government Tractor Trolly

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCE

The following machinery can be made available through private source i.e Government contractors.

- Excavator.

- Showal.

- Tractor with Trolleys

- Dozers

- Tractor with front blade.

The numbers of machinery will increase or decrease as per flood stage.

MERGE TOGETHER 13.3.6 & 13.3.7 STATUS OF FLOOD FIGHTING MATERIAL FOR VULNERABLE SITE NO. I

| S.# | Description | Total Quantity Required |  | Total Quantity Available | Deficient Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes | 1 | No. | 1 | - | There is no deficient Flood fighting material. |
| 2 | Baskets | 15 | No. | 15 | - |  |
| 3 | Buckets | 2 | No. | 2 | - |  |
| 4 | Cell for Torches | 3 | No. | 3 | - |  |
| 5 | Cow Dung in EC | 5 | Bags | 5 | - |  |
| 6 | Funnel (Ordinary) | 1 | No. | 1 | - |  |
| 7 | Funnel with W.G | 1 | No. | 1 | - |  |
| 8 | Gas Lamp | 1 | No. | 1 | - |  |
| 9 | Gunny Bags | 44 | No. | 44 | - |  |
| 10 | Hammer 2.5 lb | 1 | No. | 1 | - |  |
| 11 | Hammer 10 lb | 1 | No. | 1 | - |  |
| 12 | Kassi with handles | 5 | No. | 5 | - |  |
| 13 | Kerosene Oil | 11 | Ltr | 11 | - |  |
| 14 | Khajji Mate | 22 | No. | 22 | - |  |
| 15 | Lanterns | 3 | No. | 3 | - |  |
| 16 | Larmal 50 Ft | 18 | No. | 18 | - |  |
| 17 | Mallet wooden | 2 | No. | 2 | - |  |
| 18 | Match Box | 9 | No. | 9 | - |  |
| 19 | Mental | 6 | No. | 6 | - |  |
| 20 | Needles | 1 | No. | 1 | - |  |
| 21 | Needles for sewing bags | 2 | No. | 2 | - |  |
| 22 | Nozzles | 1 | No. | 1 | - |  |
| 23 | Oil Extractor | 1 | No. | 1 | - |  |
| 24 | Sledge Hammer 14 lb | 1 | No. | 1 | - |  |
| 25 | Spare handle for kassies | 2 | No. | 2 | - |  |
| 26 | Spirit | 1 | Bottles | 1 | - |  |
| 27 | Spirit Can | 1 | No. | 1 | - |  |
| 28 | Sticks (6-8) | 44 | No. | 44 | - |  |
| 29 | Sutli | 1 | Lb | 1 | - |  |
| 30 | Torch 3 Cell | 1 | No | 1 | - |  |
| 31 | Tranger Wire | 1 | CWT | 1 | - |  |
| 32 | Washers | 1 | No | 1 | - |  |
| 33 | Wicks | 1 | Yards | 1 | - |  |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC SUI GAS TELEPHONE INSTALLATION, ROAD NETWORK, OTHER BUILDING, CANAL AND DRAINAGE NETWORK.

- Grid Station Wassu Astana

- Telephone Exchange 18-Hazari

- Basic Health Centers 18-Hazari

- Tehsil council 18-Hazari

- Jhang-Bhakkar Road

- Jhang-Muzaffar Garh Road

- Rangpur Canal & its system

PLAN NO. 2

RD: 14+000 – 21+000 LEFT MARGINAL BUND

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan Attached

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

- Sumandari Kalan

- Mansoor Sial

- Mahloana

- Kot Dewan

- Haveli Bahadar Shah

- Mahranwala

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

The employed labour, Flood Fighting Material and available machinery will be deployed on the breach for control of spilling water coming out from the breach. All efforts will be made to control the breach.

13.3.1	ARRANGEMENTS

The following arrangements will be made.

13.3.2 ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD: 16+000 of L.M.B

13.3.3 DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

Sub Engineer River Survey Section will be incharge of the camp site. The following staff will be at the camp site.

Sub Engineer		=	1 No.

Mate				=	2 No.

Beldar				=	16  Nos

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

- Government Tractor Trolly

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCE

The following machinery can be made available through private source i.e Government contractors.

- Excavator.

- Showal.

- Tractor with Trolleys

- Dozers

- Tractor with front blade.

The numbers of machinery will increase or decrease as per flood stage.

MERGE TOGETHER 13.3.6 & 13.3.7 STATUS OF FLOOD FIGHTING MATERIAL FOR VULNERABLE SITE NO. II

| S.# | Description | Total Quantity Required |  | Total Quantity Available | Deficient Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes | 2 | No. | 2 | - | There is no deficient Flood fighting material. |
| 2 | Baskets | 29 | No. | 29 | - |  |
| 3 | Buckets | 4 | No. | 4 | - |  |
| 4 | Cell for Torches | 6 | No. | 6 | - |  |
| 5 | Cow Dung in EC | 10 | Bags | 10 | - |  |
| 6 | Funnel (Ordinary) | 1 | No. | 1 | - |  |
| 7 | Funnel with W.G | 1 | No. | 1 | - |  |
| 8 | Gas Lamp | 1 | No. | 1 | - |  |
| 9 | Gunny Bags | 87 | No. | 87 | - |  |
| 10 | Hammer 2.5 lb | 1 | No. | 1 | - |  |
| 11 | Hammer 10 lb | 2 | No. | 2 | - |  |
| 12 | Kassi with handles | 9 | No. | 9 | - |  |
| 13 | Kerosene Oil | 23 | Ltr | 23 | - |  |
| 14 | Khajji Mate | 44 | No. | 44 | - |  |
| 15 | Lanterns | 6 | No. | 6 | - |  |
| 16 | Larmal 50 Ft | 36 | No. | 36 | - |  |
| 17 | Mallet wooden | 4 | No. | 4 | - |  |
| 18 | Match Box | 18 | No. | 18 | - |  |
| 19 | Mental | 12 | No. | 12 | - |  |
| 20 | Needles | 1 | No. | 1 | - |  |
| 21 | Needles for sewing bags | 3 | No. | 3 | - |  |
| 22 | Nozzles | 1 | No. | 1 | - |  |
| 23 | Oil Extractor | 1 | No. | 1 | - |  |
| 24 | Sledge Hammer 14 lb | 1 | No. | 1 | - |  |
| 25 | Spare handle for kassies | 5 | No. | 5 | - |  |
| 26 | Spirit | 1 | Bottles | 1 | - |  |
| 27 | Spirit Can | 1 | No. | 1 | - |  |
| 28 | Sticks (6-8) | 87 | No. | 87 | - |  |
| 29 | Sutli | 1 | Lb | 1 | - |  |
| 30 | Torch 3 Cell | 1 | No | 1 | - |  |
| 31 | Tranger Wire | 1 | CWT | 1 | - |  |
| 32 | Washers | 1 | No | 1 | - |  |
| 33 | Wicks | 1 | Yards | 1 | - |  |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC SUI GAS TELEPHONE INSTALLATION, ROAD NETWORK, OTHER BUILDING, CANAL AND DRAINAGE NETWORK.

- PARCO Pipe Line

- Telephone Exchanges

- Grid Station

- Petrol Pumps

- Jhang-Multan Road.

- Jhang-Bhakkar Road

- Rice Mills.

- Basic Health Centers

- T.S.Link Canal.

- Haveli Main Line.

RD 21

CHAPTER-13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE (SITES) LISTED ABOVE. EMERGENCY CONTINGENCY PLAN MUST INCLUDE

PLAN NO. 3

RD: 290+000 – 293+000 Rangpur Flood Bund

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan Attached

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

- Rangpur Town.

- Kaudi Wal

- Kari Ali Mardan

- Bahrampur

- Chabootra Kamal

- Dera Habit

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

The employed labour, Flood Fighting Material and available machinery will be deployed on the breach for control of spilling water coming out from the breach. All efforts will be made to control the breach.

13.3.1	ARRANGEMENTS

The following arrangements will be made.

13.3.2 ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD: 303+000 of Rangpur Flood Bund

13.3.3 DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

Sub Engineer Rangpur section will be incharge of the camp site. The following staff will be present at the camp site.

Sub Engineer		=	1 No.

Mate				=	4 No.

Beldar				=	36 Nos

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

Not available

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCE

The following machinery can be made available through private source i.e Government contractors.

- Excavator.

- Showal.

- Tractor with Trolleys

- Dozers

- Tractor with front blade.

The numbers of machinery will increase or decrease as per flood stage.

13.3.6 FLOOD FIGHTING MATERIAL REQUIRED

| S.# | Description | Quantity Required Site No. 4 |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Qty | Unit |  | Total Requirement |  |
| 1 | Axes | 2 |  | No. | 2 | Balance material will be recouped during flood season 2025 |
| 2 | Baskets | 38 |  | No. | 38 |  |
| 3 | Buckets | 6 |  | No. | 6 |  |
| 4 | Burner and Lamps | 1 |  | No. | 1 |  |
| 5 | Cell for Torches | 6 |  | No. | 6 |  |
| 6 | Chimnies | 10 |  | No. | 10 |  |
| 7 | Chimnies of Gas | 1 |  | No. | 1 |  |
| 8 | Cow Dung in EC | 6 |  | Bags | 6 |  |
| 9 | Funnel (Ordinary) | 2 |  | No. | 2 |  |
| 10 | Funnel with W.G | 1 |  | No. | 1 |  |
| 11 | Gas Lamp | 1 |  | No. | 1 |  |
| 12 | Gunny Bags | 160 |  | No. | 160 |  |
| 13 | Hammer 2.5 lb | 1 |  | No. | 1 |  |
| 14 | Hammer 10 lb | 2 |  | No. | 2 |  |
| 15 | Kassi with handles | 10 |  | No. | 10 |  |
| 16 | Kerosene Oil | 54 |  | Ltr | 54 |  |
| 17 | Khajji Mate | 60 |  | No. | 60 |  |
| 18 | Lanterns | 10 |  | No. | 10 |  |
| 19 | Larmal 50 Ft | 40 |  | No. | 40 |  |
| 20 | Mallet wooden | 4 |  | No. | 4 |  |
| 21 | Match Box | 17 |  | No. | 17 |  |
| 22 | Mental | 15 |  | No. | 15 |  |
| 23 | Needles | 1 |  | No. | 1 |  |
| 24 | Needles for sewing bags | 3 |  | No. | 3 |  |
| 25 | Nozzles | 1 |  | No. | 1 |  |
| 26 | Oil Extractor | 1 |  | No. | 1 | Balance material will be recouped during flood season 2025 |
| 27 | Sledge Hammer 14 lb | 1 |  | No. | 1 |  |
| 28 | Spare handle for kassies | 1 |  | No. | 1 |  |
| 29 | Spirit | 1 |  | Bottles | 1 |  |
| 30 | Spirit Can | 1 |  | No. | 1 |  |
| 31 | Sticks (6-8) | 170 |  | No. | 170 |  |
| 32 | Sutli | 1 |  | Lb | 1 |  |
| 33 | Torch 3 Cell | 1 |  | No | 1 |  |
| 34 | Tranger Wire | 1 |  | CWT | 1 |  |
| 35 | Washers | 1 |  | No | 1 |  |
| 36 | Wicks | 8 |  | Yards | 8 |  |
| 37 | Petrol for generator | 10 |  | Liter | 10 |  |
| 38 | Diesel for Petroling | 30 |  | Liter | 30 |  |
| 39 | Empty Drum | 0 |  | Each | 0 |  |
| 40 | Polythene Bag | 400 |  | Each | 400 |  |
| 41 | Bamboos | 100 |  | Each | 100 |  |

13.6.7 FLOOD FIGHTING MATERIAL AVAILABLE AT SITE NO.4

| S.# | Description | Quantity Available |
| --- | --- | --- |
| 1 | Axes | 2 |
| 2 | Baskets | 38 |
| 3 | Buckets | 6 |
| 4 | Burner and Lamps | 1 |
| 5 | Cell for Torches | 6 |
| 6 | Chimnies | 10 |
| 7 | Chimnies of Gas | 1 |
| 8 | Cow Dung in EC | 6 |
| 9 | Funnel (Ordinary) | 2 |
| 10 | Funnel with W.G | 1 |
| 11 | Gas Lamp | 1 |
| 12 | Gunny Bags | 160 |
| 13 | Hammer 2.5 lb | 1 |
| 14 | Hammer 10 lb | 2 |
| 15 | Kassi with handles | 10 |
| 16 | Kerosene Oil | 54 |
| 17 | Khajji Mate | 60 |
| 18 | Lanterns | 10 |
| 19 | Larmal 50 Ft | 40 |
| 20 | Mallet wooden | 4 |
| 21 | Match Box | 17 |
| 22 | Mental | 15 |
| 23 | Needles | 1 |
| 24 | Needles for sewing bags | 3 |
| 25 | Nozzles | 1 |
| 26 | Oil Extractor | 1 |
| 27 | Sledge Hammer 14 lb | 1 |
| 28 | Spare handle for kassies | 1 |
| 29 | Spirit | 1 |
| 30 | Spirit Can | 1 |
| 31 | Sticks (6-8) | 170 |
| 32 | Sutli | 1 |
| 33 | Torch 3 Cell | 1 |
| 34 | Tranger Wire | 1 |
| 35 | Washers | 1 |
| 36 | Wicks | 8 |
| 37 | Petrol for generator | 10 |
| 38 | Diesel for Petroling | 30 |
| 39 | Empty Drum | 0 |
| 40 | Polythene Bag | 400 |
| 41 | Bamboos | 100 |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC SUI GAS TELEPHONE INSTALLATION, ROAD NETWORK, OTHER BUILDING, CANAL AND DRAINAGE NETWORK.

- PARCO Pipe Line

- Grid Station

- Telephone Exchange

- Rural Health Center

- Government Boys Degree College

- Vocational Institute.

- Government Schools

- Muzaffar Garh to Jhang Road.

- Rangpur to Chowk Sarwar Shaheed Road.

- 5-R Disty

- Rangpur Canal

PLAN NO.4

RD.292+000 TO 318+000 RANGPUR FLOOD BUND

RD

318

RD

292

PLAN NO. 4

RD: 49+000 TO 50+000 OF HASSU WALI FLOOD BUND

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan Attached

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

- Budhuana.

- Qaim Bharwana

- Kakku Wala

- Shah Sadiq Niang

- Fouja Dhara

- Allah Yar Joota

- Meerak Sial

- Mouza Mangan

- Shorkot City

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

The employed labour, Flood Fighting Material and available machinery will be deployed on the breach for control of spilling water coming out from the breach. All efforts will be made to control the breach.

13.3.1	ARRANGEMENTS

The following arrangements will be made.

13.3.2 ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD: 22000 of Hassu Wali Flood Bund

13.3.3 DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

Sub Engineer Farid Mehmoood Section will be incharge of the camp site. The following staff will be at the camp site.

Sub Engineer		=	1 No.

Mate				=	1 No.

Beldar				=	10  Nos

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

Not Available

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCE

The following machinery can be made available through private source i.e Government contractors.

- Excavator.

- Showal.

- Tractor with Trolleys

- Dozers

- Tractor with front blade.

The numbers of machinery will increase or decrease as per flood stage.

13.3.6 FLOOD FIGHTING MATERIAL REQUIRED

| S.# | Description | Quantity Required Site No.5 |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Qty | Unit |  | Total Requirement |  |
| 1 | Axes | 4 |  | No. | 4 | Balance material will be recouped during flood season 2025 |
| 2 | Baskets | 25 |  | No. | 25 |  |
| 3 | Buckets | 4 |  | No. | 4 |  |
| 4 | Burner and Lamps | 1 |  | No. | 1 |  |
| 5 | Cell for Torches | 3 |  | No. | 3 |  |
| 6 | Chimnies | 6 |  | No. | 6 |  |
| 7 | Chimnies of Gas | 1 |  | No. | 1 |  |
| 8 | Cow Dung in EC | 3 |  | Bags | 3 |  |
| 9 | Funnel (Ordinary) | 1 |  | No. | 1 |  |
| 10 | Funnel with W.G | 1 |  | No. | 1 |  |
| 11 | Gas Lamp | 1 |  | No. | 1 |  |
| 12 | Gunny Bags | 72 |  | No. | 72 |  |
| 13 | Hammer 2.5 lb | 1 |  | No. | 1 |  |
| 14 | Hammer 10 lb | 4 |  | No. | 4 |  |
| 15 | Kassi with handles | 6 |  | No. | 6 |  |
| 16 | Kerosene Oil | 63 |  | Ltr | 63 |  |
| 17 | Khajji Mate | 36 |  | No. | 36 |  |
| 18 | Lanterns | 6 |  | No. | 6 |  |
| 19 | Larmal 50 Ft | 8 |  | No. | 8 |  |
| 20 | Mallet wooden | 1 |  | No. | 1 |  |
| 21 | Match Box | 12 |  | No. | 12 |  |
| 22 | Mental | 8 |  | No. | 8 |  |
| 23 | Needles | 1 |  | No. | 1 |  |
| 24 | Needles for sewing bags | 1 |  | No. | 1 |  |
| 25 | Nozzles | 1 |  | No. | 1 |  |
| 26 | Oil Extractor | 1 |  | No. | 1 |  |
| 27 | Sledge Hammer 14 lb | 1 |  | No. | 1 |  |
| 28 | Spare handle for kassies | 3 |  | No. | 3 | Balance material will be recouped during flood season 2025 |
| 29 | Spirit | 1 |  | Bottles | 1 |  |
| 30 | Spirit Can | 1 |  | No. | 1 |  |
| 31 | Sticks (6-8) | 72 |  | No. | 72 |  |
| 32 | Sutli | 1 |  | Lb | 1 |  |
| 33 | Torch 3 Cell | 1 |  | No | 1 |  |
| 34 | Tranger Wire | 1 |  | CWT | 1 |  |
| 35 | Washers | 1 |  | No | 1 |  |
| 36 | Wicks | 4 |  | Yards | 4 |  |

13.3.7 FLOOD FIGHTING MATERIAL AVAILABLE AT SITE NO.5

| S.# | Description | Quantity Available |
| --- | --- | --- |
| 1 | Axes | 4 |
| 2 | Baskets | 25 |
| 3 | Buckets | 4 |
| 4 | Burner and Lamps | 1 |
| 5 | Cell for Torches | 3 |
| 6 | Chimnies | 6 |
| 7 | Chimnies of Gas | 1 |
| 8 | Cow Dung in EC | 3 |
| 9 | Funnel (Ordinary) | 1 |
| 10 | Funnel with W.G | 1 |
| 11 | Gas Lamp | 1 |
| 12 | Gunny Bags | 72 |
| 13 | Hammer 2.5 lb | 1 |
| 14 | Hammer 10 lb | 4 |
| 15 | Kassi with handles | 6 |
| 16 | Kerosene Oil | 63 |
| 17 | Khajji Mate | 36 |
| 18 | Lanterns | 6 |
| 19 | Larmal 50 Ft | 8 |
| 20 | Mallet wooden | 1 |
| 21 | Match Box | 12 |
| 22 | Mental | 8 |
| 23 | Needles | 1 |
| 24 | Needles for sewing bags | 1 |
| 25 | Nozzles | 1 |
| 26 | Oil Extractor | 1 |
| 27 | Sledge Hammer 14 lb | 1 |
| 28 | Spare handle for kassies | 3 |
| 29 | Spirit | 1 |
| 30 | Spirit Can | 1 |
| 31 | Sticks (6-8) | 72 |
| 32 | Sutli | 1 |
| 33 | Torch 3 Cell | 1 |
| 34 | Tranger Wire | 1 |
| 35 | Washers | 1 |
| 36 | Wicks | 4 |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC SUI GAS TELEPHONE INSTALLATION, ROAD NETWORK, OTHER BUILDING, CANAL AND DRAINAGE NETWORK.

- Telephone Exchange

- Petrol Pumps

- Jhang-Multan Road.

- Rice Mills.

- Basic Health Centers

- Hassu Wali Disty.

- Shorkot Disty.

PLAN NO. 5

RD: 0+000 – 6+500 RAJBANA FLOOD BUND

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan Attached

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

- Babal wala

- Yousaf Wala.

- Binda Sarbana

- Lakhi

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

The following arrangements / strategy will be adopted

13.3.1	ARRANGEMENTS

The following arrangements will be made.

13.3.2 ESTABLISHMENT OF FLOOD FIGHTING CAMPS

Flood fighting camp will be established at RD: 6+000 of Rajbana Flood Bund

13.3.3 DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

Sub Engineer Rajbana Section will be incharge of the camp site. The following staff will be at the camp site.

Sub Engineer			=	1 No.

Mate				=	1 No.

Beldar				=	10  Nos

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

Not Available

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCE

The following machinery can be made available through private source i.e Government contractors.

- Excavator.

- Showal.

- Tractor with Trolleys

- Dozers

- Tractor with front blade.

The numbers of machinery will increase or decrease as per flood stage.

13.3.6 FLOOD FIGHTING MATERIAL REQUIRED

| S.# | Description | Quantity Required at Site No. 6 |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Qty. | Unit |  | Total Requirement |  |
| 1 | Axes | 1 |  | No. | 1 | Balance material will be recouped during flood season 2025 |
| 2 | Baskets | 22 |  | No. | 22 |  |
| 3 | Buckets | 4 |  | No. | 4 |  |
| 4 | Burner and Lamps | 1 |  | No. | 1 |  |
| 5 | Cell for Torches | 6 |  | No. | 6 |  |
| 6 | Chimnies | 6 |  | No. | 6 |  |
| 7 | Chimnies of Gas | 1 |  | No. | 1 |  |
| 8 | Cow Dung in EC | 6 |  | Bags | 6 |  |
| 9 | Funnel (Ordinary) | 1 |  | No. | 1 |  |
| 10 | Funnel with W.G | 1 |  | No. | 1 |  |
| 11 | Gas Lamp | 1 |  | No. | 1 |  |
| 12 | Gunny Bags | 70 |  | No. | 70 |  |
| 13 | Hammer 2.5 lb | 1 |  | No. | 1 |  |
| 14 | Hammer 10 lb | 1 |  | No. | 1 |  |
| 15 | Kassi with handles | 6 |  | No. | 6 |  |
| 16 | Kerosene Oil | 61 |  | Ltr | 61 |  |
| 17 | Khajji Mate | 35 |  | No. | 35 |  |
| 18 | Lanterns | 6 |  | No. | 6 |  |
| 19 | Larmal 50 Ft | 15 |  | No. | 15 |  |
| 20 | Mallet wooden | 2 |  | No. | 2 |  |
| 21 | Match Box | 12 |  | No. | 12 |  |
| 22 | Mental | 15 |  | No. | 15 |  |
| 23 | Needles | 1 |  | No. | 1 |  |
| 24 | Needles for sewing bags | 1 |  | No. | 1 |  |
| 25 | Nozzles | 1 |  | No. | 1 |  |
| 26 | Oil Extractor | 1 |  | No. | 1 |  |
| 27 | Sledge Hammer 14 lb | 1 |  | No. | 1 |  |
| 28 | Spare handle for kassies | 3 |  | No. | 3 |  |
| 29 | Spirit | 1 |  | Bottles | 1 |  |
| 30 | Spirit Can | 1 |  | No. | 1 | Balance material will be recouped during flood season 2025 |
| 31 | Sticks (6-8) | 70 |  | No. | 70 |  |
| 32 | Sutli | 1 |  | Lb | 1 |  |
| 33 | Torch 3 Cell | 1 |  | No | 1 |  |
| 34 | Tranger Wire | 1 |  | CWT | 1 |  |
| 35 | Washers | 1 |  | No | 1 |  |
| 36 | Wicks | 4 |  | Yards | 4 |  |

13.3.7 FLOOD FIGHTING MATERIAL AVAILABLE AT SITE NO.6

| S.# | Description | Quantity Available |
| --- | --- | --- |
| 1 | Axes | 1 |
| 2 | Baskets | 22 |
| 3 | Buckets | 4 |
| 4 | Burner and Lamps | 1 |
| 5 | Cell for Torches | 6 |
| 6 | Chimnies | 6 |
| 7 | Chimnies of Gas | 1 |
| 8 | Cow Dung in EC | 6 |
| 9 | Funnel (Ordinary) | 1 |
| 10 | Funnel with W.G | 1 |
| 11 | Gas Lamp | 1 |
| 12 | Gunny Bags | 70 |
| 13 | Hammer 2.5 lb | 1 |
| 14 | Hammer 10 lb | 1 |
| 15 | Kassi with handles | 6 |
| 16 | Kerosene Oil | 61 |
| 17 | Khajji Mate | 35 |
| 18 | Lanterns | 6 |
| 19 | Larmal 50 Ft | 15 |
| 20 | Mallet wooden | 2 |
| 21 | Match Box | 12 |
| 22 | Mental | 15 |
| 23 | Needles | 1 |
| 24 | Needles for sewing bags | 1 |
| 25 | Nozzles | 1 |
| 26 | Oil Extractor | 1 |
| 27 | Sledge Hammer 14 lb | 1 |
| 28 | Spare handle for kassies | 3 |
| 29 | Spirit | 1 |
| 30 | Spirit Can | 1 |
| 31 | Sticks (6-8) | 70 |
| 32 | Sutli | 1 |
| 33 | Torch 3 Cell | 1 |
| 34 | Tranger Wire | 1 |
| 35 | Washers | 1 |
| 36 | Wicks | 4 |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC SUI GAS TELEPHONE INSTALLATION, ROAD NETWORK, OTHER BUILDING, CANAL AND DRAINAGE NETWORK.

- Shorkot-Garh Maharaja Road

- Basic Health Centre

- Union Council

- Vetrinary Hospital.

- Basic Health Unit

- Rajbana Minor.

- Jalalpur Minor

CHAPTER – 14

ADDITIONAL INFORMATION

ACTION PLAN

CONSOLIDATED STATUS OF RESERVE STOCK OF STONE

| Sr. No. | Sanctioned Reserve stock Limit (Lac Cft) | Available Quantity (Lac Cft) | Balance required with respect to Sanctioned limit (Lac Cft) | Minimum Quantity Available (Lac Cft) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 23 | 17.24 | 5.76 | 16.10 | However, as per minimum requirement, there is no deficiency of stone. |

14.1 – Re-Shuffling / Recouping Plan Of Reserve Stone Departmentally

| Sr. No. | Name of Structure | Sanctioned Reserve stock Limit (Lac Cft) | Available Quantity (Lac Cft) | Balance required (Lac Cft) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | LGB U/S AND D/S | 5 | 4.87 | 0.13 | Minimum quantity i.e 16.10 Lac Cft is available. However, as per minimum requirement, there is no deficiency of stone. |
| 2 | RGB U/S AND D/S | 5 | 4.76 | 0.24 |  |
| 3 | J-HEAD SPUR RD 5500 OF RMB | 1.5 | 1.57 | 0 |  |
| 4 | SLOPING SPUR ALONG COLONY RING BUND | 1 | 0.32 | 0.68 |  |
| 5 | T-1 SPUR ALONG COLONY RING BUND | 1.5 | 0.81 | 0.69 |  |
| 6 | T-2 SPUR D/S TRIMMU BARRAGE | 1.5 | 0.79 | 0.71 |  |
| 7 | J-HEAD SPUR MAKORI | 1.5 | 0.90 | 0.6 |  |
| 8 | DAB KALAN (relates to SDO Shorkot) | 1.25 | 0.89 | 0.36 |  |
| 9 | GUID WALL NO.1 ALONG LMB | 1.5 | 0.76 | 0.74 |  |
| 10 | GUID WALL NO.2 ALONG LMB | 1.5 | 0.76 | 0.74 |  |
| 11 | GUID WALL NO.3 ALONG COLONY RING BUND | 1.5 | 0.60 | 0.9 |  |
| 12 | X-SPUR RD 23000 OF RMB | 0.25 | 0.21 | 0.04 |  |
| TOTAL |  | 23 | 17.24 | 5.76 |  |

Note:-	However, in case of emergency and as per necessity of the quantity of stone in accordance with the requirement will be fulfilled by re-shuffling plan at vulnerable site likely to be affected.

14.2 – Detail of inlet / outlet crossing along with closing methodology.

There is no inlet / outlet crossing in flood bunds i.e Right Marginal Bund, Left Marginal Bund and Colony Ring Bund in the jurisdiction of Trimmu Barrage Division. The Inlet / Outlet crossing are on Hassuwali, Rajbana, Lakhi, Dadal Flood Bund. These Flood Bunds are now falls in the jurisdiction of Jhang Canal Division and Multan Canal Division Multan due to detachment from Trimmu Barrage Division.

14.3 – DEPLOYMENT MACHINERY (MEDIUM TO HIGH)

| Sr. No. | Name of Structure |  | Length in Mile | Vulnerable Reach | Camp Location | Site Incharge by name & cell No. |  |  | Machinery Deployed |  |  |  | Availability of Stone Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Labour | Excavator |  | Dozer | Trolleys / Dumpers | Tractor with front blade |  |
| 1 | 2 |  | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 | 11 | 12 |
| 1 | Left Marginal Bund RD 0-46000, Including 12 No. X-Spurs, Makori Spur, Guide Wall No.I & II |  | 14.89 | RD 14-21, RD 42-46 | RD 17000, RD 43000 LMB | Akhtar Hamid Sub Engineer River Survey Cell No.0302-3787027 | 132 | 1 |  | 0 | 4 | 1 | 2.42 Lac Cft (Guide Wall No.I, II Makori Spur) |
| 2 | Right Marginal Bund RD 0-57765, Including X-Spur 2 No., Left Guide Bund, Right Guide Bund , J-Head Spur RD 4597 |  | 15.36 | RD 20-27 | R.M.B RD 25000, RGB RD 1200, LGB RD 1200 | Asif Rehman Sub Engineer Head Works Cell No.0333-6751933 | 144 | 1 |  | 0 | 4 | 1 | 11.38 Lac Cft (RGB, LGB, J-Head Spur 5500 & X-Spur RD 23000 along RMB |
| 3 | Colony Ring Bund RD 0-9600, T-Headed Spur No.1, T-Headed Spur No.2, T-Cum Hockey sloping Spur, Guide Wall No.III |  | 5.04 |  | Hockey Spur | Saif ur Rehman Sub Engineer Station Area Cell No. 0300-7590043 | 24 | 0 |  | 0 | 0 | 1 | 2.53 Lac Cft (T-1, T-2, sloping spur) D/S Trimmu |
| Total length |  |  |  |  |  |  | 300 | 2 |  | 0 | 8 | 3 |  |
|  |  | "payment for machinery & work charge employees subject to verification by TPM/Consultant" |  |  |  |  |  |  |  |  |  |  |  |

14.4 – DEPLOYMENT MACHINERY (HIGH TO VERY HGIH)

| Sr. No. | Name of Structure |  | Length in Mile | Vulnerable Reach | Camp Location | Site Incharge by name & cell No. |  |  | Machinery Deployed |  |  |  | Availability of Stone Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Labour | Excavator |  | Dozer | Trolleys / Dumpers | Tractor with front blade |  |
| 1 | 2 |  | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 | 11 | 12 |
| 1 | Left Marginal Bund RD 0-46000, Including 12 No. X-Spurs, Makori Spur, Guide Wall No.I & II |  | 14.89 | RD 14-21, RD 42-46 | RD 17000, RD 43000 LMB | Akhtar Hamid Sub Engineer River Survey Cell No.0302-3787027 | 199 | 2 |  | 0 | 8 | 2 | 2.42 Lac Cft (Guide Wall No.I, II Makori Spur) |
| 2 | Right Marginal Bund RD 0-57765, Including X-Spur 2 No., Left Guide Bund, Right Guide Bund , J-Head Spur RD 4597 |  | 15.36 | RD 20-27 | R.M.B RD 25000, RGB RD 1200, LGB RD 1200 | Asif Rehman Sub Engineer Head Works Cell No.0333-6751933 | 216 | 2 |  | 2 | 8 | 2 | 11.38 Lac Cft (RGB, LGB, J-Head Spur 5500 & X-Spur RD 23000 along RMB |
| 3 | Colony Ring Bund RD 0-9600, T-Headed Spur No.1, T-Headed Spur No.2, T-Cum Hockey sloping Spur, Guide Wall No.III |  | 5.04 |  | Hockey Spur | Saif ur Rehman Sub Engineer Station Area Cell No. 0300-7590043 | 35 | 1 |  | 0 | 0 | 1 | 2.53 Lac Cft (T-1, T-2, sloping spur) D/S Trimmu |
| Total length |  |  |  |  |  |  | 300 | 5 |  | 2 | 16 | 5 |  |
|  |  | "payment for machinery & work charge employees subject to verification by TPM/Consultant" |  |  |  |  |  |  |  |  |  |  |  |

14.5 – DEPLOYMENT MACHINERY (EXCEPTIONALLY HIGH)

| Sr. No. | Name of Structure |  | Length in Mile | Vulnerable Reach | Camp Location | Site Incharge by name & cell No. |  |  | Machinery Deployed |  |  |  | Availability of Stone Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Labour | Excavator |  | Dozer | Trolleys / Dumpers | Tractor with front blade |  |
| 1 | 2 |  | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 | 11 | 12 |
| 1 | Left Marginal Bund RD 0-46000, Including 12 No. X-Spurs, Makori Spur, Guide Wall No.I & II |  | 14.89 | RD 14-21, RD 42-46 | RD 17000, RD 43000 LMB | Akhtar Hamid Sub Engineer River Survey Cell No.0302-3787027 | 264 | 2 |  | 0 | 8 | 2 | 2.42 Lac Cft (Guide Wall No.I, II Makori Spur) |
| 2 | Right Marginal Bund RD 0-57765, Including X-Spur 2 No., Left Guide Bund, Right Guide Bund , J-Head Spur RD 4597 |  | 15.36 | RD 20-27 | R.M.B RD 25000, RGB RD 1200, LGB RD 1200 | Asif Rehman Sub Engineer Head Works Cell No.0333-6751933 | 288 | 2 |  | 2 | 8 | 2 | 11.38 Lac Cft (RGB, LGB, J-Head Spur 5500 & X-Spur RD 23000 along RMB |
| 3 | Colony Ring Bund RD 0-9600, T-Headed Spur No.1, T-Headed Spur No.2, T-Cum Hockey sloping Spur, Guide Wall No.III |  | 5.04 |  | Hockey Spur | Saif ur Rehman Sub Engineer Station Area Cell No. 0300-7590043 | 48 | 1 |  | 0 | 0 | 1 | 2.53 Lac Cft (T-1, T-2, sloping spur) D/S Trimmu |
| 4 | Hassuwali Flood Bund |  | 14.4 | RD 8 to 9 20 to 22 38 to 39 49 to 50 | Camp at RD 22000 of Hassu Wali Flood Bund | Muzafar Ali Sajid Farid Mehmood Section Cell No.0300-6847593 | 346 | 2 |  | 0 | 8 | 2 |  |
| 5 | Rajbana Flood Bund, Connecting tail Rajbana Flood Bund with lakhi Flood Bund RD 0-11400, |  | 6.83 | RD 0-6500 | Camp at RD 6+000 of Rajbana Flood Bund | Asghar Ali Shah Sub Engineer Rajbana Cell No.0301-7203509 | 164 | 1 |  | 0 | 4 | 1 |  |
| 6 | Rangpur Flood Embankment along left bank of Rangpur Canal RD 260-308 & 308-318 (Tiba) Range),left bank of escape channel RD 0-3850 |  | 12.88 | RD 292 to 318 | Camp at RD 303 of RGP Flood Bund | Muhammad Aslam Pervaiz Sub Engineer Rangpur Cell No.0324-7008563 | 297 | 1 |  | 0 | 4 | 2 |  |
| Total length |  |  |  |  |  |  | 1407 | 9 |  | 2 | 32 | 10 |  |
|  |  | "payment for machinery & work charge employees subject to verification by TPM/Consultant" |  |  |  |  |  |  |  |  |  |  |  |

14.6 – POLICE DEPLOYMENT PLAN

14.7 – Detail Of Synthetic Bags With Capacity Of 500kg And 1000 Kg

| Sr# | Name of Bund | Synthetic bags (500 Kg) | Synthetic bags (100 Kg) | Quantity Required | Quantity Available | Balance Quantity required |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Left Marginal Bund | 1000 | 500 | 1500 | - | 1500 |
| 2 | Right Marginal Bund | 1000 | 500 | 1500 | - | 1500 |
| TOTAL |  | 2000 | 1000 | 3000 | - | 3000 |

The deficient quantity of Synthetic Bags with capacity of 500 Kg and 1000 Kg will be purchased before the start of Flood season.

14.8 – Details of polythene sheets of black color to protect upstream slope against wave action and to control seepage through embankments.

| Sr# | Name of Bund | Polythene sheets | Quantity Required | Quantity Available | Balance Quantity required |
| --- | --- | --- | --- | --- | --- |
| 1 | Left Marginal Bund | 40000 Ft | 40000 Ft | - | 40000 Ft |
| 2 | Right Marginal Bund | 45000 Ft | 45000 Ft | - | 45000 Ft |
| TOTAL |  | 85000 Ft | 85000 Ft | - | 85000 Ft |

The deficient quantity of Polythene sheets will be purchased before the start of Flood season.

CHAPTER NO. 15

BACK UP DIVISION (In case of Breach)

In case of breach, the staff of Executive Engineer, Sidhnai Headworks Division will provide backup to Trimmu Headworks Division Trimmu and in case of any flood emergency, the local contractors may also be engaged.

Executive Engineer,		Superintending Engineer,	        Chief Engineer,

Trimmu Headworks Division	Trimmu Canal Circle, Trimmu	        Irrigation Multan Zone

Trimmu							        Multan.

PART – A

PART - B