Chapter 1

SALIENT FEATURE OF THE CONCERNED DIVISION

1.1	LOCATION

The Dallas Canal Division is located on the left bank of the Indus River. Its starting point is downstream of Chachran town in Tehsil Khanpur. The division is bordered by the Indus River to the north, the Guddu Barrage command area to the southwest, and the irrigation boundary of Rahimyarkhan Division to the southeast.

  - GENERAL DESCRIPTION

The Gross command area of Dallas Canal Division is 437,320 acres, with a culturable command area of 410,274 acres. The lands in this region are highly fertile. To safeguard the irrigation system and the fertile lands from potential river spills, three flood embankments, spanning a total length of 22 miles, have been established. These embankments include Minchin Bund (RD 243,000-309,000), Machka Dilawra Bund (RD 0-38,000), and Loop Bund (RD 0-6,070).

During the floods of 1995 to 1997, Minchin Bund experienced erosion from RD 246 to 259. In response, three defensive bunds were constructed in setback positions by the Rahimyarkhan Canal Division. The 4th defence bund is linked to Minchin Bund at RD 263 and to the 3rd defence bund at RD 5-6. In 1998, the government approved the construction of five spurs to combat river erosion and protect the Rahimyarkhan and Dallas Divisions. These spurs have been completed, with Spur No. 4 at RD 8 of the 4th defence bund and Guide wall spur No. 5 at RD 260 of the Minchin Bund being incorporated into the Dallas Canal Division.

The Minchin Bund, existing at the site, has been rehabilitated and raised to a 6 ft freeboard above the flood levels of 1992. An extension of the existing Minchin Flood bund from RD 309,000 to LMB Guddu Barrage has been constructed, featuring three J-Head Spurs and one Concave Convex Guide Wall. This extension is connected at RD 509+900 with the LMB Guddu Barrage, providing protection to the infrastructure and canal system in the area against river spills.

Length of Canal 		= 490   Mile

Length of Bund		= 67.34    Mile

No. of Spurs     		= 6 No.

Guide Wall			= 1 No

Inverted Hockey Spur		 = 3 No

Stone Stud			= 3 No

Mole Head			= 2 No

GCA / CCA			=437320 / 410274 Acres

  - ADMINISTRATIVE SETUP.

ORIGINATION CHART OF DALLAS CANAL DIVISION IS AS UNDER

Chapter 2

FLOOD PROTECTION & RIVER TRAINING WORKS

Flood protection works of the division are briefed as under.

  - DESIGN PARAMETERS OF FLOOD PROTECTION BUND & RIVER TRAINING WORKS.

1.	Minchin bund from (RD:260-310)       	                      = RD 50		           = 13.40   Miles

2.	Fourth Defence Bund (RD 0-15840) 	                      = RD 15.80	           = 3.17     Miles

3.	Extension Minchin Bund (RD: 309+500-509+900)   = RD 200.40	           = 40.08   Miles

4.	Machka Dilawara Bund (RD: 0-38000)                      = RD 38	                        = 7.60    Miles

5.	Loop Bund (RD: 0-6070)		                      = RD 6.07	            = 1.21    Miles

6.	2nd Defence Bund (RD:0-4000)		         = RD 4	  	            =  0.80    Miles

7.	3rd Defence Bund (RD:0-5375)  	                      = RD 5.37	            =  1.08    Miles

Total RD:			                                 =319.64                         = 67.34 Miles

BUNDS.

| Sr. No. | Name of bund | Top width in ft | Side slope |  | Free board | Remarks. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | City side | River side |  |  |
| 1 | Minchin Bund | 25 | 2:1 | 3:1 | 6' |  |
| 2 | Extension Minchin Bund | 25 | 2:1 | 3:1 | 7' |  |
| 3 | 4th Defence Bund | 25 | 2:1 | 3:1 | 7' |  |
| 4 | Machka Dilawra Bund | 25 | 2:1 | 3:1 | 6' |  |
| 5 | Loop bund | 25 | 2:1 | 3:1 | 6' |  |

SPURS.

| Sr.No | Structures | Length |  |  | Section |  |  |  |  |  |  |  | Pitching Ft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Shank Ft. | Head Ft | Total | Top width Ft | Side Slope Head |  | Side Slope (Shank) |  | Free board | Apron | Crush & spawl |  |
|  |  |  |  |  |  | U/S | D/S | U/S | D/S |  |  |  |  |
| 1 | Inverted Hockey Spur U/S of J-Head Spur No. 4 | 2020 | 249 | 2269 | 25 |  |  | 2:1 | 2:1 | 6 ft |  | 1’ | 1.0 |
| 2. | J Head spur no. 4 at RD 8 of 4th defence Bund. | 1400 | 1651 | 3051 | 30 | 2:1 | 3:1 | 3:1 | 3:1 | 6 ft | 70x7' | 1’ | 1.0 |
| 3. | Guide Wall spur no. 5 at RD 260 of Minchin Bund. | 637 | 941 | 1578 | 30 | 2:1 | 2:1 | 3:1 | 2:1 | 6 ft | 70x7' | 1’ | 1.0 |
| 4 | J-Head Spur No. 1-A at RD: 379+500 of Extension Minchin Bund | 2340 | 1950 | 4290 | 30 | 2:1 | 3:1 | 3:1 | 3:1 | 6 ft | 70x7' | 1’ | 1.0 |
| 5 | J-Head Spur No. 1 at RD: 388+746 of Extension Minchin Bund | 5196 | 1993 | 7189 | 30 | 2:1 | 3:1 | 3:1 | 3:1 | 6 ft | 70x7' | 1’ | 1.0 |
| 6 | Stone Stud at RD: 441+500 of Extension Minchin Bund | 300 | 80 | 380 |  | 2:1 | 2:1 | 3:1 | 2:1 |  |  |  |  |
| 7 | Guide Head Spur RD: 442+000 of Extension Minchin Bund | 190 | 745 | 935 |  | 2:1 | 3:1 |  |  |  | 20 ft |  |  |
| 8 | J-Head Spur No. 2 at RD: 447+380 of Extension Minchin Bund | 2340 | 1950 | 4290 | 30 | 2:1 | 3:1 | 3:1 | 3:1 | 6 ft | 70x7' | 1’ | 1.0 |
| 9 | Guide Wall Spur at RD: 453+735 of Extension Minchin Bund | 471 | 3319 | 3790 | 30 | 2:1 | 3:1 |  |  | 6 ft | 70x7' | 1’ | 1.0 |
| 10 | Solid Stone Studs at RD: 462-463 (3 Nos) | Solid stone stud with top width 25 ft length 90 to 100 |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Mole Head at RD 245+000 Minchin Flood Bund |  |  |  | 25 | 2:1 | 2:1 | 2:1 | 2:1 | 6 | 25x7’ | 1’ | 1.0 |
| 12 | Mole Head at RD 4+000 2nd Defence Bund. |  |  |  | 25 | 2:1 | 2:1 | 2:1 | 2:1 | 6 | 25x7’ | 1’ | 1.0 |
| 13 | Inverted Hockey at RD 3+000 of 3rd Defence Bund. |  |  |  | 25 | 1.5:1 | 1.5:1 | 2:1 | 2:1 | 6 | 25x7’ | 1’ | 1.0 |
| 14 | Inverted Hockey at RD 0+000 of 4th Defence Bund. |  |  |  | 25 | 1.5:1 | 1.5:1 | 2:1 | 2:1 | 6 | 25x7’ | 1’ | 1.0 |
| 15 | Slopping stud at RD 1+400 at 4th Defence Bund. | 550 |  | 550 | 25 | 2:1 | 2:1 | 2:1 | 2:1 | 6 | 25x7’ | 1’ | 1.0 |
| 16 | Solid Stone Studs between RD: 466+500-467+100 (3 Nos) (Under Construction) | Solid stone stud with top width 15 ft length 100 |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Solid Stone Studs between RD: 466+500-467+100 (4 Nos) (Under Construction) | Solid stone stud with top width 15 ft length 50 |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Inverted hockey at RD 468+300 | Solid stone inverted Hockey with top width 25 ft and 360 ft length |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Solid Stone Studs between RD 468+800 to 469+400 (4 Nos) | Solid Stone Stud with top width 50 ft length 50 ft |  |  |  |  |  |  |  |  |  |  |  |

## Location Map

Dallas Canal Division

Rahim Yar Khan

Chapter 3

BRIEF HISTORY OF PAST FLOOD EVENTS

Indus is the largest river in Pakistan having a length of about 3200 KM. It originates from Lake Manasavover in Hamaliyas. It flows in mountainous ranges upto Kala Bagh and then in plains upto Thatta and ultimately falls in Arabian Sea. Discharge of river varies from 0.50 Lac Cs in winter to 11.0 Lac Cs in summer.

River Chenab falls in River Indus upstream of Chachran Town in District Rahimyarkhan opposite Minchin Flood Bund RD: 105. River Indus Plane is about 10 miles wide having a network of small & large creaks.

Minchin Flood Bund was constructed downstream of Panjnad headworks along left bank of river Chenab and Indus for protection of Panjnad Main Line/Canal & its Canal System. It starts opposite to RD: 52 of Panjnad Main Line and ends near RD: 55 of Dallas Branch, with total length of about 315+500 thousand feet ending at RD: 7 of right bank of Ehsan Disty of Dallas Canal System.

The terrain & the contour of the area is such that if a spill of river Indus crosses the highest ridge on the left flank of the river in the said gap, the spill of water will never fall back into main river but will devastate everything on its way in Punjab & Sindh provinces. The same happened when the Left Marginal Bund (LMB) of Panjnad Head Works breached during flood 1973.

The course of river has gradually shifted from right bank to left bank, after 1994 & 1995 flood, the main current further shifted toward left bank. For the changed posture of the river and keeping in view the geophysical features, a scheme was proposed to extend the Minchin  flood bund to the Left Marginal Bund (LMB) of Guddu Barrage, in order to protect the abadies, precious agriculture lands, industry and infrastructure on left bank of the river all the way from Panjand Headworks to Guddu barrage.

The project of extension Minchin Bund was launched during the year 2008. The contractor completed the river training works under this project and major part of the embankments but construction of the embankments at 18 sites in varying lengths was not allowed by the landowners of the area by getting stay orders from the court. Initially, when the project was launched, no land acquisition was provided which created discomfort and feeling of unease especially among the small landowners losing their land to (under) the embankment. The project was re-launched for the completion of the Extension Minchin Flood Bund after flood 2010 and at present all the gaps have been filled and linked with LMB Guddu Barrage.

In 2010 exceptional flood came in River Indus. After subsidence of flood 2010, the course of River Indus has changed significantly. The main creek of River has shifted to left bank and is attacking the structures present on left flanks of river. No significant damage was observed after flood 2010 and structures were stable. However at low flow after 2011 the aprons of Spur No. 2, 4 and 5 and guide wall were observed with continued regular erosive behavior. In 2011-12 the apron of the concavo convex guide wall at RD 453+735 and river started eroding the flood bund at RD 442-43. The projects of replenishing the apron of concavo convex guide wall and checking erosion at RD 442-43 were taken up under para 2.89 PWD code. The stone stud at RD: 441+500 and Guide Spur at RD: 442+000 were constructed and completed in 2012.

During the flood 2013, the following sites under the control of Dallas Division became under attack of River Indus.

- Guide Spur No. 5 at RD: 260+000 of Minchin Flood Bund.

- Minchin Flood Bund at RD: 262+000 to RD: 263+000.

- Extension Minchin Flood Bund at RD: 450+000 to RD:453+000

These sites were checked and controlled by taking temporary measures as advised Expert Committee during its visit, at sites.

After the flood 2013, the detail survey was carried out and working paper was submitted to Chief Engineer Irrigation Research Institute Lahore for seeking advice for the permanent solution of these problems.

The Expert Committee recommended the following proposals.

- Guide Spur No. 5. At RD: 260 of Minchin Flood Bund, an Inverted Hockey Spur and prong U/S of the Guide Spur.

- RD: 262+000 to 264+000, Minchin Flood Bund, the proposal as per Model Study.

- RD: 450+000 to 453+000 Extension Minchin Flood Bund, a stone pitching with 10 ft wide apron.

But unfortunately due to paucity of funds the work could not be executed and again the emergent work under para 2.89 PWD was carried out during flood 2014 and site was temporarily protected. After the flood 2014 the PC-I for these proposal has been submitted to Govt. But work could not be executed at site.

During flood 2015, the high flood passed through Indus River the major part of flood embankment remain protected. At the rising stage, the shank of mole head truncated part at RD: 245 was became under erosive action and the site was checked by launching trees and providing killa bushing and the width of bank was restored in set-back position. And the sluffing due to the wind storm / wave action at the different location was checked by placing earth filled gunny plastic bags. A seepage phenomenon was observed at RD: 395-400 which checked by providing pushta to cover the hydraulic gradients. However Guide Spur No. 5 at RD: 260 U/S Nose in length 325 ft was damaged and concavo convex Guide Wall at RD: 453+735 D/s Solid Slope 90 ft in length was also damage. These sites were checked by temporarily by providing stone dumping.

After the flood, the detailed estimate of these structures has been technically sanctioned and restoration works were completed.

During flood season 2018, the main river attacks on Extension Minchin Flood Bund from RD: 458-465 and started erosion to the bund between RD: 462-463. The three No. solid stone studs were constructed in the said reach to protect the bund from erosive action of river on emergent basis. The PC-I of Rs. 136-906 (M) for the construction of two No. solid stone studs at RD: 461+000 and RD: 462+000 was approved by CDWP in Aril 2018 with the condition that feasibility report may get approved from competent forum which was submitted to the Government.

Meanwhile, river trend changed altogether and river started severe erosive action b/w RD 462+000 to 463+000. This alarming situation was discussed in Departmental Expert Committee in June, 2018 and on the direction of Departmental Expert Committee 03 Nos solid stone studs were constructed in July 2018 at RD 462+000, 462+500, and 463+000 by invoking Para 2.89 of PWD code and Para 59d(iii) of PPRA Rules 2014.

In January, 2019 River Indus started erosion b/w RD 460+000 to 463+000. Due to aggressive river attack, the matter was referred to Departmental Expert Committee after detailed river survey, In the light of direction of Departmental Expert Committee, the flood bund was successfully protected by dumping of stone under Para 2.89 of PWD code and Para 59d(iii) of PPRA Rules 2014.

After flood season 2019, working paper was submitted to Departmental Expert Committee after conducting detailed river survey. The Departmental Expert Committee proposed construction of 03 Nos Inverted Hockey at RD 462+000, 463+800 and 465+500. Further Raising & strengthening of Minchin Flood Bund from RD: 370+000 to 509+900 has been done during the year 2018-19. The work was executed under the supervision of Project Implementation Unit (PIU).

During flood 2024, there was a significant shift in the river trend, causing the river to flow dangerously close to the toe of the flood bund. This resulted in erosion along the riverbank in the stretch between RD 466+000 and 474+000. Subsequently, the matter was brought to the attention of the Departmental Expert Committee (DEC), which suggested the immediate flood fighting measures. The reach was protected from breach with flood fighting efforts.

Chapter 4

DESIGN DATA, HISTORIC PEAK FLOOD DATA AND PREVIOUS FIVE YEARS FLOOD DATA OF HEAD WORKS / BARRAGES AND OR OTHER CONTROL POINTS.

4.1	FLOOD LIMITS

Minchin Bund from RD: 122-315 is affected by the joint discharge of River Chanab D/S Punjnad Head Works and River Indus D/S Taunsa Barrage. Flood stages are as under.

| Sr. No | Site | Low (Cs) | Medium (Cs) | High (Cs) | Very High (Cs) | Exceptionally High (Cs) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | D/S Panjnad Head Works | 1,50,000 | 2,00,000 | 3,00,000 | 4,50,000 | 6,00,000 |
| 2. | D/S Tounsa Barrage | 2,50,000 | 3,75,000 | 5,00,000 | 6,50,000 | Above 8.0 Lac |
| 3. | U/S Guddu Barrage | 3,50,000 | 5,00,000 | 7,00,000 | 9,00,000 | Above 9.0 (Lac Cs) |

The approximate discharge at Chachran will be assessed by adding the 48 hours earlier discharge downstream Taunsa Barrage and 24 hours earlier discharge downstream Panjnad Headworks. However River stage will be the same as indicating the Guddu Barrage.

Chachran is not discharge measuring point. The gauge fixed at Chachran indicates only the level of water from which free board of existing bund will be calculated.

The strict monitoring of River gauges will observed and discharge of hill torrents falling in River Indus downstream Taunsa Barrage will be added so as to assess the discharge passing through Chachran.

4.2	TIME LAG OF FLOODS

| S.No. | Description |  | Reach |  |  |  | Time Login Days |  |  |  | Distance in Canal Miles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | RIVERS |  |  |  |  |  | NOV to APR | MAY JUN | JUL AUG | SEP OCT |  |
|  | 1 | Channab | I | Marala | - | Khanki | 1 | 1 | 1 | 1 | 40 |
|  |  |  | Ii | Khanki | - | Qadirabad | 1 | 0 | 0 | 0 | 29 |
|  |  |  | Iii | Qadirabad | - | Trimmu | 4 | 3 | 2 | 3 | 160 |
|  | 2 | Jehlum | I | Mangla | - | Rasool | 1 | 1 | 1 | 1 | 43 |
|  |  |  | Ii | Rasool | - | Rail C.J, Link | 3 | 2 | 2 | 2 | 118 |
|  |  |  | Iii | Rail C.J, Link | - | Trimmu | 1 | 1 | - | 1 | 80 |
|  | 3 | Channab Cum Jehlum | I | Trimmu | - | Tail T.P.Link | 4 | 3 | 2 | 2 | 119 |
|  |  |  | Ii | Tail T.P.Link | - | Punjnad | 1 | 1 | 1 | 1 | 70 |
|  | 4 | Punjnad | I | Punjnad | - | Mithankot/ Chanhran | 1 | 1 | 1 | 1 | 58 |
|  | 5 | Indus | I | Terbela | - | Attock | 1 | 0 | 0 | 0 | 25 |
|  |  |  | Ii | Attock | - | Kalabagh | 2 | 1 | 1 | 1 | 100 |
|  |  |  | Iii | Kalabagh | - | Chashma | 1 | 1 | 1 | 1 | 61 |
|  |  |  | Iv | Chashma | - | Taunsa | 4 | 3 | 2 | 2 | 152 |
|  |  |  | V | Taunsa | - | Mitan Kot | 3 | 2 | 2 | 2 | 129 |
|  |  |  | Vi | Mitan Kot | - | Guddu | 1 | 1 | 1 | 1 | 59 |
|  |  |  | Vii | Guddu | - | Sukkar | 2 | 1 | 1 | 1 | 130 |
|  |  |  | Viii | Sukkar | - | Kotri | 5 | 4 | 3 | 3 | 298 |

  - HIGHEST FLOODS.

No control point on River exists in the jurisdiction of Dallas Canal Division. However previous 15 year maximum discharge at Guddu Barrage is as under:-

| S. No | Year | Discharge at Guddu Barrage in Cusecs | Flood Stages |
| --- | --- | --- | --- |
| 1 | 2005 | 548148 | High |
| 2 | 2006 | 762344 | Very High |
| 3 | 2007 | 395110 | Medium |
| 4 | 2008 | 285135 | Low |
| 5 | 2009 | 376152 | Medium |
| 6 | 2010 | 1148738 | Exceptionally High |
| 7 | 2011 | 400313 | Medium |
| 8 | 2012 | 258262 | Low |
| 9 | 2013 | 443540 | Medium |
| 10 | 2014 | 595150 | High |
| 11 | 2015 | 727785 | Very High |
| 12 | 2016 | 362118 | Medium |
| 13 | 2017 | 432890 | Medium |
| 14 | 2018 | 344642 | Medium |
| 15 | 2019 | 422653 | Medium |
| 16 | 2020 | 570722 | High |
| 17 | 2021 | 423545 | Medium |
| 18 | 2022 | 717787 | Very High |
| 19 | 2023 | 481913 | Medium |
| 20 | 2024 | 439179 | Medium |

Maximum flood passed in August, 2010 Guddu Barrage discharge was observed 1148738

4. PEAK DISCHARGE

There is no Head works or other control points in Dallas Division. The flood protection works of the division are meant to save the irrigation system and the area from the River spill. River discharges are not observed.

Discharges of river Indus & Chenab are as under:-(Cusecs)

| Month | Punjnad |  | Taunsa |  | Chachran |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maximum | Minimum | Maximum | Minimum | Maximum | Minimum |

Year 2016

| Month | Punjnad |  | Tounsa |  | Chachran |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maximum | Minimum | Maximum | Minimum | Maximum | Minimum |
| April-16 | 8800 | 7807 | 86831 | 38430 | - | - |
| May-16 | 9250 | 8800 | 146114 | 81004 | 7.10 | 5.2 |
| Jun-16 | 9200 | 8900 | 253713 | 145617 | 8.2 | 7.0 |
| Jul-16 | 9300 | 8900 | 352818 | 181911 | 9.6 | 7.5 |
| Aug-16 | 9400 | 5393 | 277690 | 129355 | 9.6 | 6.6 |
| Sep-16 | 9400 | 1191 | 148138 | 85145 | 6.9 | 5.10 |
| Oct-16 | 6000 | 3000 | 91293 | 39404 | - | - |
| Nov-16 | 6000 | 3204 | 61398 | 37100 | - | - |
| Dec-16 | - | - | 55773 | 17087 | - | - |

Year 2017

| Month | Punjnad |  | Tounsa |  | Chachran |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maximum | Minimum | Maximum | Minimum | Maximum | Minimum |
| April-17 | 29050 | 1831 | 55968 | 31542 | - | - |
| May-17 | 32419 | 2026 | 152239 | 59465 | - | - |
| Jun-17 | 29089 | 833 | 215206 | 156044 | 10.20 | 7.10 |
| Jul-17 | 73071 | 2074 | 275849 | 205255 | 10.90 | 7.5 |
| Aug-17 | 78788 | 8208 | 424388 | 178329 | 13.10 | 3.6 |
| Sep-17 | 36132 | 1663 | 159837 | 91263 | 7.4 | 5.4 |
| Oct-17 | 13900 | 4919 | 91263 | 37569 | - | - |
| Nov-17 | 9127 | 1000 | 52434 | 36947 | - | - |
| Dec-17 | 2500 | 1000 | 41093 | 15346 | - | - |

Year 2018

| April-18 | 4304 | 800 | 45538 | 19666 | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| May-18 | 6082 | 2291 | 65868 | 43887 | - | - |
| Jun-18 | 12450 | 2774 | 214424 | 57311 | 9.6 | 8.8 |
| Jul-18 | 19159 | 6280 | 212004 | 165700 | 9.1 | 8.0 |
| Aug-18 | 57464 | 10013 | 287178 | 178946 | 10.4 | 7.9 |
| Sep-18 | 36764 | 7862 | 171885 | 115916 | 8.4 | 8.0 |
| Oct-18 | 14400 | 2165 | 111682 | 30734 | - | - |
| Nov-18 | 5893 | 2008 | 58410 | 36301 | - | - |
| Dec-18 | 3893 | 1907 | 65146 | 17711 | - | - |

Year 2019

| April-19 | 24271 | 8410 | 95348 | 27274 | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| May-19 | 29745 | 6402 | 10858 | 8503 | - | - |
| Jun-19 | 45180 | 8231 | 168698 | 94897 | - | - |
| Jul-19 | 47066 | 15700 | 282282 | 168698 | 10.6 | 8.40 |
| Aug-19 | 87336 | 21973 | 358358 | 137348 | 12.80 | 5.50 |
| Sep-19 | 45713 | 14926 | 158869 | 120202 | 6.60 | 5.40 |
| Oct-19 | 29974 | 5192 | 122088 | 34715 | - | - |
| Nov-19 | 12688 | 4500 | 68822 | 34522 | - | - |
| Dec-19 | 11530 | 4000 | 58628 | 15026 | - |  |

Year 2020

| April-20 | 105760 | 19746 | 85900 | 31324 | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| May-20 | 34044 | 11550 | 138055 | 68374 | - | - |
| Jun-20 | 41889 | 17784 | 196491 | 134632 | 7.50 | 7.20 |
| Jul-20 | 32741 | 14480 | 230597 | 188953 | 7.95 | 7.20 |
| Aug-20 | 60949 | 11653 | 321920 | 167508 | 9.00 | 7.40 |
| Sep-20 | 147296 | 23344 | 464560 | 95893 | 12.50 | 4.00 |
| Oct-20 | 21380 | 5400 | 107328 | 39536 | - | - |
| Nov-20 | 14955 | 7536 | 64773 | 38254 | - | - |
| Dec-20 | 12792 | 3274 | 63372 | 15023 | - | - |

Year 2021

| April-21 | 24681 | 12945 | 50137 | 28360 | - | - |
| --- | --- | --- | --- | --- | --- | --- |
| May-21 | 24681 | 10908 | 92556 | 42529 | 5.5 | 4.5 |
| Jun-21 | 18258 | 9683 | 189504 | 83390 | 7.75 | 5.7 |
| Jul-21 | 43530 | 8691 | 220170 | 15770 | 8.9 | 6.7 |
| Aug-21 | 72683 | 10956 | 327621 | 135469 | 10.01 | 5.3 |
| Sep-21 | 34053 | 9580 | 159289 | 108268 | 6.00 | 4.2 |
| Oct-21 | 19038 | 2067 | 108529 | 32335 | - | - |
| Nov-21 | 8594 | 2390 | 55982 | 32335 | - | - |
| Dec-21 | 6128 | 3547 | 49234 | 15238 | - | - |

Year 2022

| Month | Punjnad |  | Tounsa |  | Chachran |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maximum | Minimum | Maximum | Minimum | Maximum | Minimum |
| April-22 | 11528 | - | 49362 | 22334 | - | - |
| May-22 | - | - | 92306 | 34407 | - | - |
| Jun-22 | - | - | 104731 | 66321 | - | - |
| Jul-22 | 98758 | - | 292900 | 93478 | 8.90 | 8.5 |
| Aug-22 | 112564 | 26173 | 622095 | 124204 | 11.10 | 5.5 |
| Sep-22 | 34031 | - | 580056 | 61138 | 11.10 | 5.6 |
| Oct-22 | 3060 | - | 52042 | 41259 | - | - |
| Nov-22 | 3060 | - | 45308 | 36922 | - | - |
| Dec-22 | - | -` | 40905 | 11104 | - | - |

Year 2023.

| Month | Punjnad |  | Tounsa |  | Chachran |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maximum | Minimum | Maximum | Minimum | Maximum | Minimum |
| April-23 | 7005 | - | 45043 | - | - | - |
| May-23 | 0 | - | 81918 | - | - | - |
| Jun-23 | 0 | - | 173317 | - | 6.60 | - |
| Jul-23 | 97230 | - | 419895 | - | 9.90 | - |
| Aug-23 | 134504 | - | 357457 | - | 9.40 | - |
| Sep-23 | 86059 | - | 134018 | - | 7.60 | - |
| Oct-23 | 16727 | - | 90040 | - | 6.20 | - |
| Nov-23 | 9471 | - | 50856 | - | - | - |
| Dec-23 | 4013 | - | 44960 | - | - | - |

Year 2024

| Month | Punjnad |  | Tounsa |  | Chachran |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Maximum | Minimum | Maximum | Minimum | Maximum | Minimum |
| April-24 | 6102 | - | 116369 | - | - | - |
| May-24 | 11000 | - | 168925 | - | 7.60 | - |
| Jun-24 | 20380 | - | 179335 | - | 7.80 | - |
| Jul-24 | 21441 | - | 204749 | - | 8.00 | - |
| Aug-24 | 64320 | - | 439179 | - | 10.20 | - |
| Sep-24 | 51462 | - | 192251 | - | 7.70 | - |
| Oct-24 | 5388 | - | 85302 | - | 5.00 | - |
| Nov-24 | 0 | - | 50639 | - | - | - |
| Dec-24 | 0 | - | 46722 | - | - | - |

Chapter 5

FLOOD FIGHTING STRATEGY.

Minchin Flood Bund is affected by both the Rivers i.e. Indus and Chanab. The correct flood warnings may be assessed by watching the discharge downstream of Taunsa and Punjnad Head works. Proper watching arrangements are required.

Minchin Bund is affected by the joint discharge of River Chanab D/S Punjnad Head Works and River Indus D/S Taunsa Barrage.

| Sr. No | Site | Low (Cs) | Medium (Cs) | High (Cs) | Very High (Cs) | Exceptionally High (Cs) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | D/S Panjnad Head Works | 1,50,000 | 2,00,000 | 3,00,000 | 4,50,000 | 6,00,000 |
| 2. | D/S Tounsa Barrage | 2,50,000 | 3,75,000 | 5,00,000 | 6,50,000 | Above 8.0 Lac |
| 3. | U/S Guddu Barrage | 3,50,000 | 5,00,000 | 7,00,000 | 9,00,000 | Above 9.0 (Lac Cs) |

The Minchin Flood Bund and Extension Minchin Flood Bund in the jurisdiction of Dallas Canal Division is divided in three section. The Sub Divisional Officer Bund Sub Division Dallas is in-charge of whole Bund.

- RD: 243+000 – 325+000 Minchin Flood Bund, Extension Minchin Flood Bund and          4th Defence Bund along with all allied structures in Section 1st Bund Sub Division.

- RD: 325+000 – 445+000 Extension Minchin Flood Bund along with allied structures in Section 2nd of Bund Sub Division.

- RD: 445+000 – 511+000 Extension Minchin Flood Bund, Machka Dilawara Bund and Loop Bund in Section 3rd of Bund Sub Division.

The each section is divided in sub section and Sub Engineer is in charge of sub section.

The detail sub section is as under:-

SECTION I

| 1 | Sub Section RD:242-272 and all allied Structure. | Sub Engineer Rukanpur, Sub Engineer Bund Ist | Camp at RD:262, Minchin Flood Bund |
| --- | --- | --- | --- |
| 2 | Sub Section RD:272-325 Minchin Flood Bund and all allied Structure | Sub Engineer Adamwali, Sub Engineer Pallu Shah | Camp at RD:312, Minchin Flood Bund |

SECTION II

| 1 | Sub Section RD:325-382 Minchin Flood Bund. | Sub Engineer Degi, Sub Engineer Bund II | Camp at RD:353, Minchin Flood Bund |
| --- | --- | --- | --- |
| 2 | Sub Section RD:382-445 Minchin Flood Bund. | Sub Engineer Sharin, Sub Engineer Shahbaz Pur | Camp at RD:388, Minchin Flood Bund |

SECTION III

| 1 | Sub Section RD:445-470 Minchin Flood Bund and all allied structure. | Sub Engineer Bhong, Bund III | Camp at RD:460, Minchin Flood Bund |
| --- | --- | --- | --- |
| 2 | Sub Section RD:470-511 Minchin Flood Bund. | Sub Engineer Kasmani, Lamma | Camp at RD:511, Minchin Flood Bund |
| 3 | Sub Section RD:0-38 Machka dilawra Flood Bund and loop Bund, all allied structure | Sub Engineer Duawala, Sub Engineer Walhar | Camp at RD:16+000, Minchin Flood Bund |

The establishment and other requirements are arranged depending on flood stages

LOW FLOOD STAGE

When flood discharge assessed through Guddu Barrage is below 3,50,000 Cs there is no immediate danger and the river water remains within the pocket.

MEDIUM FLOOD STAGE

In case of medium flood discharge from 3,50,000 to 5,00,000 it is considered to be affecting the bund in Dallas Division and in some low lying areas, it touches the Minchin Bund. 140 No. men power required for watch & ward of bunds / River Training works.

HIGH FLOOD STAGE

From 5,00,000 to 7,00,000 Cs at Guddu Barrage a condition of High flood is to be tackled on Guddu Barrage discharge emergency is considered and maximum strength for watching is arranged. 210 No. men power required for watch & ward of bunds / River Training works.

VERY HIGH FLOOD STAGE

The discharge 7,00,000 to 9,00,000 at Guddu Barrage, the Executive Engineer Dallas Division will inform the District authorities and shift his headquarter to Degi Canal Rest House. As Minchin Flood Bund /Extension Minchin Flood Bund, Machka Dilawra Flood Bund & Loop Bund will be equally affected so the Executive Engineer Dallas Division will also have to move on all bunds. The Executive Engineer Dallas Division will inspect watching arrangement on all bunds and will camp at Rukanpur, Bangla Dilkusha or Dauwala and will inform to all concerned regarding the latest position of River. 350 No. men power required for watch & ward of bunds / River Training works.

Chapter 6

FLOOD DAMAGES RESTORATION WORKS.

The Extension Minchin Bund exists on Left Side of Indus River which was constructed for safety of canal network, agriculture lands and adjoining abadies from attack of river spill. The bund has been constructed to cover 40 miles gap between Minchin Flood Bund and Left Marginal Bund of Guddu Barrage which was required to save the road infrastructures, agriculture lands and adjoining adadies and Irrigation networks. Extension Minchin Bund was started in 2008 with 3 No. J-Head Spurs and Guide Wall. Whereas on Minchin Flood Bund,  3 No spurs (J head No 4, J Head No 5 and inverted Hockey spur) are constructed to deal with the floods. Inverted hockey spur was constructed upstream of spur No 4 to safeguard the spur no 4 and is now intact and has successfully served its purpose.

During the flood 2015, the following sites under the control of Dallas Division became under attack of River Indus and restoration work was taken in hand and completed before flood season.

- The Guide Wall Spur No. 5 at RD: 260+000 U/S Nose in length 325 ft  of Minchin Flood Bund.

- The concavo convex Guide Wall at RD: 453+735 D/S solid slop 90 ft in length of Extension Minchin Flood Bund.

In 2016 the flood range was remain as normal. However the apron of concavo convex Guide Wall Spur at RD: 453+735 Extension Minchin Flood Bund was launched which was restored with 20 ft width.

In January, 2019 River Indus started erosion b/w RD 460+000 to 463+000. Due to aggressive river attack, the matter was referred to Departmental Expert Committee after detailed river survey, In the light of direction of Departmental Expert Committee, the flood bund was successfully protected by dumping of stone under Para 2.89 of PWD code and Para 59d(iii) of PPRA Rules 2014.

However after flood season 2019, working paper was submitted to Departmental Expert Committee after conducting detailed river survey. The Departmental Expert Committee proposed construction of 03 Nos Inverted Hockey at RD 462+000, 463+800 and 465+500. The work for construction of 03 Nos Hockey has been completed. The Extension Minchin Flood Bund from RD 463+400 to 464+900 was damaged during falling condition of flood 2019. The restoration of subjected flood bund has been completed in protection work. Moreover, an invented Hockey has also been constructed at RD: 460+500 after recommendation of DEC during the year 2021.

At the onset of the 2024 flood season, the river exerted pressure on the flood bund between RD 468+000 and RD 474+000, causing erosion. The Expert Committee inspected the site on July 3, recommending immediate flood-fighting measures. Due to changing conditions, they revisited on July 10 and advised treating the bund as a setback and secondary defense line.

Local resistance to land use delayed efforts, prompting a meeting led by the Commissioner of Dera Ghazi Khan, where officials stressed protecting the existing bund. Continuous flood-fighting was carried out, particularly at the setback position. Damage at the J-Head spur RD 447+200 and stone stud RD 467+100 was locally managed.

Currently, the RD 468–474 reach is restored at set back condition and stable but remains vulnerable, requiring ongoing monitoring and precautions.

Figure No. 8.5

Chapter 7

FLOOD FIGHTING WATCHING ARRANGEMENTS.

7.1    PRE-FLOOD ARRANGEMENT

The detail survey was conducted to check the river condition and elevating of the bed. The critical / most essential work reported to authorities i.e. Chief Engineer, Superintending Engineer and Executive Engineer also visited the site and submitted their inspection notes. About the condition of flood bunds river training structure intimated to worthy Secretary Irrigation and restoration of most critical work of vulnerable reach RD: 453+000 to 475+000 are taken in hand and work will be completed prior to flood 2025.

For the watch & ward and maintaining the flood bund in Dallas Division necessary watching material and watching establishment will arranged as per approved yardstick.

FOLLOW UP ACTION ON THE FLOOD PREPAREDNESS ACTIVITIES FOR THE YEAR 2025

| S. No | Irrigation Zone | Flood Fighting Plans | Field Inspection (Certificate) | Encroachments | Gauges, RD Marks Data Boards | Reserve Stock of Stone | Inspection Notes | Status of Breaching Section | Status of Explosives Store | Security of Explosive Store | Notification of Flood Emergency Officer | Purchase of Flood Fighting Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 1 | BWP | Flood Fighting Plan for the year 2025 submitted to C.E D & F for approval. | NIL | List of encroachment submitted to DC Rahimyarkhan, Rajanpur, Ghotki for getting help for removing encroachments. Notices to all encroaches have been served. | All gauges exist but painting work will be done up to 31/5/2025 | As per approved temporary Reserve stock limit, total reserve stock stone required 1454000 cft. But only 597500 cft stone is available. | Pre-Flood inspection of all flood embankment / spur has been done. | No breaching section exist. | N.A | N.A | Sub Divisional Officer Bund Sub Division | Flood Fighting material as described in Flood Fighting Plan will be purchased. |

Consolidated status of Reserve Stack of Stone (in Lac CFT)

| Sanctioned Reserve Stock Qty | Available Reserve Stock Qty | 70% of sanctioned Reserve Stock | Difference in available and sanctioned Reserve Stock | Deficiency in Reserve Stock | Remarks |
| --- | --- | --- | --- | --- | --- |
| 14.540 | 8.557 | 10.178 | 5.975 | 1.621 | Balance required quantity upto 100% will be recouped upto 30.06.2025. |

Structure wise Detail of Reserve Stack of Stone

| S. No. | Name of Structure | Sanctioned Reserve Stock Limit (Lac Cft) | Quantity available (Lac Cft) | Balance Quantity Required (Lac Cft) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | Inverted Hockey Spur U/S Spur No. 4 | 1.150 | 1.150 | 0 |  |
| 2 | J-Head Spur No. 4 | 1.500 | 1.258 | 0.241 |  |
| 3 | Guide Wall Spur No. 5 | 1.500 | 0.6257 | 0.874 |  |
| 4 | J-Head Spur No. 1 | 1.500 | 1.446 | 0.053 |  |
| 5 | J-Head Spur No. 1-A | 1.500 | 0.4637 | 1.036 |  |
| 6 | J-Head Spur No. 2 | 1.500 | 0.7283 | 0.772 |  |
| 7 | Stone Stud at RD: 441+500 | 0.890 | 0.8716 | 0.018 |  |
| 8 | Guide Head Spur at RD: 442+000 | 1.500 | 0.9855 | 0.510 |  |
| 9 | Stone Studs at RD: 462-463 EMFB and inverted Hockey | 2.000 | - | 2.000 |  |
| 10 | Guide Wall at RD: 453+735 | 1.500 | 1.028 | 0.471 |  |
|  | Total:- | 14.540 | 8.557 | 5.975 |  |

7.2    WATCHING ESTABLISHMENT.

| Flood Stages | Discharge at U/S Guddu Barrage | Name of Bund. | Watching Establishment per miles. |
| --- | --- | --- | --- |
| Medium | 500000 Cs | Minchin bund & 4th defence bund. | 02 Person per mile |
| Medium | 500000 Cs | Spurs. | 2. No Each Spur |
| Medium | 500000 Cs | Machka Dilawra | 1 No. |
| Medium | 500000 Cs | Loop Bund. | 1 No. |
| High | 700000 Cs | Minchin bund & 4th defence bund. | 10 Person per mile |
| High | 700000 Cs | Spurs | 04. No Each Spur |
| High | 700000 Cs | Machka Dilawra | 2 No. |
| High | 700000 Cs | Loop Bund. | 2 No. |
| Very High | 900000 Cs | Minchin bund & 4th defence bund. | 20 Person per mile |
| Very High | 900000 Cs | Spurs. | 08. No Each Spur |
| Very High | 900000 Cs | Machka Dilawra | 05 No. |
| Very High | 900000 Cs | Loop Bund. | 3 No. |
| Exceptionally High | Above 900000 Cs | Minchin bund & 4th defence bund. | 40 Person per mile |
| Exceptionally High | Above 900000 Cs | Spurs. | 16. No Each Spur |
| Exceptionally High | Above 900000 Cs | Machka Dilawra | 08 No. |
| Exceptionally High | Above 900000 Cs | Loop Bund. | 04 No. |

Note:-Payment of machinery and work charged establishment subject to verification by the TPM / Consultant.

7.3 	ARRANGEMENT AT SENSITIVE SITES.

Special watching of Bunds, Spurs & monitoring of flood bund in eroded / sensitive sites of Bunds will be made against the spill of floods. Reasonable quantity of stone exists at site and flood fighting material will be provided at site as per requirements after shifting the camp at these sites.

  - WATCHING MATERIAL

Watching material will be arranged in ample quantities. Particular care will be taken to ensure that adequate quantity of required material is distributed at all the centers carefully and that it is available particularly at critical sites.

| S.No | Status of the Watching Material |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Name of Item | Total Required Quantity | Available Quantity | Balance Required Quantity |  |
| 1 | Supplying jute bags 4.0 to 5.0 Cft Capacity. | 8000 No. | 0 No. | 8000 No. | Balance required quantity will procured through contractor prior to flood 2025. |
| 2 | Supplying plastic bags. | 15000 No. | - | 15000 No. |  |
| 3 | Supply sutli for sewing jute bags | 12 Kg | 0 Kg | 12 Kg |  |
| 4 | Supply needle for sewing jute bags | 60 No. | 0 No. | 60 No. |  |
| 5 | Supply toote Basket | 300 No. | 0 No. | 300 No. |  |
| 6 | Supplying Search Light Torches Large. | 12 No. | 0 No. | 12 No. |  |
| 7 | Supplying Torches chargeable. | 180 No. | 0 No. | 180 No. |  |
| 8 | Supplying chairs & table set. | 6 Set | 0 Set | 6 Set |  |
| 9 | Supplying Kassi with handle | 180 No. | 0 No. | 180 No. |  |
| 10 | Supplying khajji mates | 4500 No. | - | 4500 No. |  |
| 11 | Supplying axes with handle. | 180 No. | 0 No. | 180 No. |  |
| 12 | Supplying wooden hammer | 12 No. | - | 12 No. |  |
| 13 | Supplying chowldari. | 12 No. | 0.No. | 12 No. |  |
| 14 | Supplying swiss cottage | 4 No. | - | 4 No. |  |
| 15 | Generator | 7 No. | 0.No. | 7 No. |  |
| 16 | Fuel per generator | As per situation | - | - |  |

7.5	ARRANGEMENT FOR SOUNDING & PROBING

Sounding observation instruments are available. However the boat will be hired

7.6	LIGHTING ARRANGEMENT.

As per requirement, proper light arrangement will be arranged on all flood bunds by hiring generator through contractor.

7.7	RATION ARRANGEMENT.

Ration arrangement will be done through local contractors.

  - P.O.L ARRANGEMENT FOR VEHICLE

Reserve Stock of P.O.L for vehicles will be kept at different sites on the following scale.

Petrol = 2000 Ltr, Diesel = 5000 Ltr,

7.9 	TRANSPORTATION

Following vehicles are available in Dallas Canal Division

GBA 481 (Double Cabin), Suzuki Jeep, LEG 1417, BNH 2211 (Double Cabin), RNT 3100 (Double Cabin) Transportation will be provided by Civil Administration. In case of emergency, transportation will be hired from local market

7.10 	LAW AND ORDER

Law and order will be maintained by cooperation of Civil Administration

7.11 	MEDICAL ARRANGEMENT FOR LABOUR

Medical facilities for Labour will be provided by cooperation of Health Department.

7.12 	LIAISON WITH OTHER DEPARTMENT

Executive Engineer Dallas Canal Division and Sub Divisional Officer, Bund Sub Division, will keep close liaison with their counter parts in other Departments. Higher authorities of other departments will be approached if needed through Director Technical / Chief Engineer. At site Sub Engineers will attend inspections of the visiting Army and Civil Officers if an Irrigation Officer is not available at site.

7.13 	ROLE OF THE ARMY

In case of emergency i.e. when discharge D/S Taunsa Barrage 3.75 Lac and D/S Panjnad exceeds 2.0 Lac cusecs, Army will be stationed at Jamal Din Wali to face any emergency. In case of occurrence of any breach, Pakistan Army will help Irrigation Department.

7.14 	DUTIES OF TELEPHONE ATTENDANT

An emergency / flood control center will be established by Superintending Engineer Rahim Yar Khan Canal Circle Telegraph Office and attendant would remain available round the clock during flood season. Their contact numbers as under.

| S.No | Name of official | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1. | Muhammad Nawaz Khan (Hd Signaller) 0302-9672301 | 15.06.2025 to 15.10.2025 | 6.00 AM | 02.00 PM |
| 2. | Sami ur Rehman Signaller (0303-310467) | 15.06.2025 to 15.10.2025 | 02.00 PM | 10.00 PM |
| 3. | Tariq Shahzad Signaller (0300-6702148) | 15.06.2025 to 15.10.2025 | 10.00 PM | 6.00 AM |

Telegraph office Rahim Yar Khan 068-9230097

7.15 	WIRELESS ARRANGEMENTS

Wireless are provided by (15 Rescue Police) Rahimyarkhan in all Officer’s vehicles as well as Telegraph Offices in Rahimyarkhan, Rukanpur & Ahmadpur Lamma

Chapter 8

DETAIL OF ENCROACHMENT.

| S.No | RD | Name of Bund | Name of Encroachers | Type of Encroachment | Location | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | Machka Dilawara Bund | Ghulab Mirani S/o Allah Rakhue Mirani | House | Countryside slope | To be Removed |
| 2. |  | -do- | Hazoor Bukhsh S/o Allah Rakhue Mirani | -do- | Countryside slope | To be Removed |
| 3. |  | -do- | Nawab Ali S/o Allah Rakhue Mirani | -do- | Countryside slope | Removed |
| 4. |  | -do- | Shorab Ali Mirani S/o Allah Rakhue Mirani | -do- | Countryside slope | To be Removed |
| 5. | 2 | -do- | Karam Ali Kundary S/o Jind Wada Kundary | -do- | Countryside slope | To be Removed |
| 6. |  | -do- | Noor Hassan S/o Lal Bukhsh Kundari | -do- | Countryside slope | To be Removed |
| 7. |  | -do- | Allah Rakha S/o Allah Dita | -do- | Countryside slope | To be Removed |
| 8. | 4 | -do- | Muhammad Hayyat S/o Choat Machi | -do- | Countryside slope | Removed |
| 9. |  | -do- | Muhammad Rehman S/o Habib Machi | -do- | Countryside slope | To be Removed |
| 10. | 19 | -do- | Dilber Ali S/o Muhammad Ramzan Mirani | -do- | Countryside slope | To be Removed |
| 11. |  | -do- | Ghulam Yasin S/o Wali Muhammad Solangi Machi | -do- | Countryside slope | To be Removed |
| 12. |  | -do- | Bacho S/o Shamshir Machi | -do- | Countryside slope | To be Removed |
| 13. |  | -do- | Muhammad Arab S/o Badlo Mirani | -do- | Countryside slope | To be Removed |
| 14. |  | -do- | Lal Bukhsh S/o Bubo Mirani | -do- | Countryside slope | To be Removed |
| 15. |  | -do- | Puno S/o Muhammad Ismail Daya | -do- | Countryside slope | To be Removed |
| 16. |  | -do- | Arbab Ali S/o Khandu Daya | -do- | Countryside slope | Removed |
| 17. |  | -do- | Bahadar S/o Muhammad Ismail Daya | -do- | Countryside slope | To be Removed |
| 18. |  | -do- | Muhammad Ramzan S/o Faqeer Bukhsh Daya | -do- | Countryside slope | To be Removed |
| 19. |  | -do- | Riaz Ahmad S/o Rahim Bukhsh Daya | -do- | Countryside slope | Removed |
| 20. |  | -do- | Khuda Bukhsh S/o Bukshoo Daya | -do- | Countryside slope | To be Removed |
| 21. |  | -do- | Nazir S/o Qadir Bukhsh Daya | -do- | Countryside slope | To be Removed |
| 22. |  | -do- | Jani S/o Din Muhammad Daya | -do- | Countryside slope | To be Removed |
| 23. |  | -do- | Mubak S/o Allah Jiwaya | -do- | Countryside slope | To be Removed |
| 24. |  | -do- | Manzoor Ahmad S/o Sulaman Machi | -do- | Countryside slope | To be Removed |
| 25. |  | -do- | Habib-Ul-Allah S/o Muhammad Ramzan | -do- | Countryside slope | Removed |
| 26. |  | -do- | Aziz S/o Sumwar Machi | -do- | Countryside slope | To be Removed |
| 27. |  | -do- | Shabbir Ahmad S/o Loqman Machi | -do- | Countryside slope | To be Removed |
| 28. |  | -do- | Faloo S/o Usman Machi | -do- | Countryside slope | To be Removed |
| 29. |  | -do- | Rasheed Ahmad S/o Shadoo | -do- | Countryside slope | Removed |
| 30. |  | -do- | Rahim Bukhsh S/o Muhammad Esa | -do- | Countryside slope | To be Removed |
| 31. |  | -do- | Nazir Ahmad S/o Elahi Bukhsh Machi | -do- | Countryside slope | To be Removed |
| 32. |  | -do- | Abdul Qadir S/o Allah Dad Machi | -do- |  | To be Removed |
| 33. | RD: 262 to 315+500 | Minchin Flood Bund | NIL | NIL |  | To be Removed |
| 34 | RD: 4-5 | 4th Defence Bund | Fida Hussain S/o Hanan | House (Kacha) | Countryside slope | To be Removed |
| 35 | RD: 4-5 | -do- | Elahi Bux S/o Rehmat-Ullah | House 2 No. (Kacha) | Countryside slope | To be Removed |
| 36 | RD: 5-6 | -do- | Qadir Bux S/o M. Bux Mohana | House 3 No. (Kacha) | Countryside slope | To be Removed |
| 37 | RD: 5-6 | -do- | Nahal S/o Khamisa Caste Mohana | Shop (Kacha) | Countryside slope | To be Removed |
| 38 | RD: 8-9 | -do- | Malik Akram S/o Noor Monik | -do- | River side Slope | Removed |
| 39 | RD:11-12 | -do- | Abdul Aziz S/o Fazul Din Monik | House (Pacca) | Countryside slope | To be Removed |
| 40 | RD:12-13 | -do- | Muhammad Iqbal S/o Imam Bux Monik | -do- | River / Country side Slope | To be Removed |
| 41 | RD:13-14 | -do- | Fareed S/o Haji Mushtaq Monik | -do- | Countryside slope | To be Removed |
| 42 | RD:14-15 | -do- | Sajja S/o Ghafoor Monik | Wall (Kacha) | Countryside slope | To be Removed |
| 43 | RD:15-16 | -do- | Sadiq S/o Mohib Caste Monik | House (Kacha) | Countryside slope | To be Removed |

CURRENT STATUS OF ENCROACHMENT

| Total No. of encroachment | No. of encroachment removed | Balance encroachment to be removed | Remarks |
| --- | --- | --- | --- |
| 43 | 7 | 36 | The list of encroaches has been submitted to the Civil Administration, which are under process for removal the encroachments. |

CRITICAL ENCROACHMENT

| S. No | RD | Name of Bund | Name of Encroachers | Type of Encroachment | Location |
| --- | --- | --- | --- | --- | --- |
| -------------------------------NIL------------------------------------ |  |  |  |  |  |

Chapter 9

DUTY ROSTER / FLOOD FIGHTING PROGRAMME.

On receipt of daily gauges from flood warning center as well as flood forecasting of metrological department, a comprehensive plan will be chalked out for flood management in Dallas Canal Division. Keeping in view lengthy system of Bunds along river Indus, the flood fighting area will be divided into three sectors. The detail of sector and their duties are as under:-

OVERALL INCHARGE.

Executive Engineer, Dallas Canal Division is overall Incharge of 4th Defence Bund, Minchin Flood Bund, Extension Minchin Flood Bund, Machak Dilawara Bund and Loop Bund and its allied structure s.

SECTION NO.I

Sub Divisional Officer Bund is overall Incharge in flood day & of 4th Defence Bund RD: 0 to 16269, Minchin Flood Bund RD: 243+000 to 315+000.

| (i) | Camp at RD: 262+000 of Minchin Bund | Sub Engineer Rukanpur & Bund Section I |
| --- | --- | --- |
| (ii) | Camp at RD: 312+000 Minchin Flood Bund | Sub Engineer Pallu Shah / Adamwali |

SECTION NO.II

Sub Divisional Officer Bund is overall Incharge from RD: 315+000 to 445+000 Extension Minchin Bund.

| (i) | Sub Section Degi camp at RD: 353+000 Extension Minchin Flood Bund | Sub Engineer Degi & Section II Bund |
| --- | --- | --- |
| (ii) | Sub Section Sharin camp at RD: 388+000 Extension Minchin Flood Bund. | Sub Engineer Sharin / Shahbazpur |

SECTION NO.III

Sub Divisional Officer Bund is overall Incharge along with staff from RD:445+000 to 511+000 Extension Minchin Bund, Machka Dilawara Bund from RD: 0+00 to 38+000 and Loop Bund from RD: 0+00 to 6+070.

| (i) | Camp at RD: 460+000 Extension Minchin Flood Bund. | Sub Engineer Bhong, Bund III |
| --- | --- | --- |
| (ii) | Camp at RD: 511+000 Extension Minchin Flood Bund. | Sub Engineer Kasmani, Lamma |
| (iii) | Camp at RD: 16+000 Machka Dilawara Bund. | Sub Engineer Duwala, Walhar |

| ### Supervisory Staff | ### S.D.O Rukanpur ### Section – I |  |  |  |  |  |  | ### S.D.O Lamma ### Section –II |  |  |  |  |  |  |  |  |  |  |  | ### SDO Bhong ### Section –III |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.D.O Bund S/Division | Dallas Canal Division Rahimyarkhan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sub Engineers | RKP |  | ADW | PLS |  | Bund I |  | Degi | SBP |  | Bund II |  |  | SRN |  | DEGI |  | BNG |  |  | KSM | DWA |  | Bund III |  |
| Zilladar & Patwaries |  | Rukanpur |  |  |  |  | Sharin |  |  | Ahmadpur Lamma |  |  |  |  |  |  | Bhong |  |  |  |  |  |  |  |  |
| Official & Revenue Department | A.C & Tehsildar with Patwari K.G.O(Khan Pur Rahimyarkhan) |  |  |  |  |  |  | A.C & Tehsildar with Patwari K.G.O (Rahimyarkhan & Sadiq Abad) |  |  |  |  |  |  |  |  |  | A.C & Tehsildar with Patwari K.G.O (Sadiq Abad |  |  |  |  |  |  |  |
| Police Department | (A.S.I With Three Constable (Khan Pur) |  |  |  |  |  |  | (A.S.I With Three Constable (Abad Pur) | A.S.I With Three Constable (Sadiq Abad) |  |  |  |  |  |  |  |  | D.S.P (Sadiq Abad) & S.H.O Machka With 5 Constable |  |  |  |  |  |  |  |
| Mates / Beldar Irri: Deptt High Flood | 1/12 |  | 1/10 |  | 1/10 | 1/10 |  | 1/12 | 1/10 |  |  | 1/10 | 1/12 |  | 1/10 |  |  | 1/10 | 1/10 |  |  |  | 1/10 | 1/10 |  |
| Mates / Beldar Irri: Deptt Very High Flood | 1/18 |  | 1/15 |  | 1/15 | 1/15 |  | 1/18 | 1/15 |  |  | 1/15 | 1/18 |  | 1/15 |  |  | 1/15 | 1/15 |  |  |  | 1/15 | 1/15 |  |
| Mates / Beldar Irri: Deptt exceptionally High Flood | 2/24 |  | 2/20 |  | 2/20 | 2/20 |  | 2/24 | 2/20 |  |  | 2/20 | 2/24 |  | 2/20 |  |  | 2/20 | 2/20 |  |  |  | 2/20 | 2/20 |  |

Chapter 10

EMERGENCY TELEPHONE NOS.

| Sr No. | ### Name of Officers | ### Telephone Numbers |  |
| --- | --- | --- | --- |
|  |  | Office | Cell & Residence. |
| Telephone No. of Irrigation Department |  |  |  |
| 1 | Secretary Irrigation Department Lahore. | 042-99212117 | 042-99212118 |
| 2 | Chief Engineer Drainage & Flood Lahore Zone. | 042-99250309 | - |
| 3 | Director Flood & Flood warning center 46-Jail Road Lahore. | 042-99231614 | - |
| 4 | Chief Engineer Irrigation Bahawalpur Zone. | 062-9850333 | 0301-6714174 0333-8581585 |
| 5 | Executive Engineer (Operation) Bahawalpur Zone. | 062-9250265 | 0345-6500439 |
| 6 | Superintending Engineer Canal Circle Rahim Yar Khan | 068-9230090 | 0300-6702694 |
| 7 | Executive Engineer Dallas Division Rahim Yar Khan. | 068-9230092 | 0333-7626359 |
| 8 | Executive Engineer Rahimyarkhan Canal Division. | 068-9230095 | 0304-9628449 |
| 9 | Sub Divisional Officer Rukan Pur. | 068-9230092 | 0303-4763256 |
| 10 | Sub Divisional Officer Lamma. | 068-9230092 | 0308-1891805 |
| 11 | Sub Divisional Officer Bhong. | 068-9230092 | 0300-9675465 |
| 12 | Sub Divisional Officer Bund Sub Division. | 068-9230092 | 0303-4711114 |
| 13 | Deputy Commissioner Rahim Yar Khan | 068-9230266 Fax:068-9230267 | 03040923820 |
| 14 | District Administrator Rahim Yar Khan | 068-9230222 | - |
| 15 | Tehsil Administrator Rahim Yar Khan | 068-9230240 | 0300-8678825 |
| 16 | Assistant Commissioner Khanpur | 068-5571383 | 0301-4188882 |
| 19 | Assistant Commissioner Rahim Yar Khan | 068-9230271 | 0300-1122991 |
| 20 | Assistant Commissioner Sadiq Abad. | 068-5705533 | 0333-7838858 |
| 21 | Assistant Commissioner Liaqutpur. | 068-5795625 | 0300-1984444 |
| 22 | Flood Emergency Center Rahim Yar Khan | 068-9230097 | - |
| 23 | Flood Emergency Center Bahawalpur. | 062-9250273 | - |
| Telephone No. of Police Department |  |  |  |
| 24 | Senior Superintendent Police Rahim Yar Khan. | 068-9230301 | 068-9230304 0301-8670464 |
| 25 | Superintendent of Police Head Quarter Rahim Yar Khan. | 068-9230309 | 068-9230304 |
| 26 | Deputy Superintendent Police Sadar Rahimyarkhan. | 068-5705445 | 0332-7225541 |
| 27 | ASP/Deputy Superintendent Police Sadar Sadiqabad. | 068-5705445 | 0332-7225541 |
| Telephone No. of Pakistan Military |  |  |  |
| 28 | Brigade Headquarter Rahim Yar Khan. | 561-6007 | 561-6007 |
| 29 | Brigade Major (Army co-ordination Unit) | 05614124 05614125 |  |

Chapter 11

STANDARD OPERATING PROCEDURE (SOP) FOR BREACHING SECTIONS.

11.1 	HISTORY OF THE BREACHING SECTION

Not Related.

11.2 	LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION

Not Related.

11.3     ARRANGEMENT OF EXPLOSIVES AND SECURITY OF EXPLOSIVE STORES

Not Related.

11.4 	LIST OF THE SECURITY STAFF ALONGWITH DETAIL OF THEIR TRAINING ETC

Not Related.

11.5 	DETAIL OF MECHANICAL MEANS AS A STANDBY ARRANGEMENTS IN CASE OF DETONATION FAILURE.

Not Related.

11.6 	DUTY ROSTER IN CASE OF CRITICAL SITUATION

Not Related.

11.7 	BREACHING COMMITTEE WITH THEIR ACTION PLAN

Not Related.

11.8 	LIST OF THE VILLAGES LIKELY TO BE INUNDATED IN CASE OF BREACH

Not Related.

11.9 	ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENTS.

Not Related.

11.10 	DETAIL OF COORDINATION WITH CIVIL / ARMY / AUTHORITIES

Not Related.

11.11 	PARALLEL COMMUNICATION ARRANGEMENTS.

Not Related.

11.12 	Index Plan

Not Related.

Superintending Engineer

Rahimyarkhan Canal Circle

Rahimyarkhan

Executive Engineer

Dallas Canal Division

Rahimyarkhan

Chief Engineer

Bahawalpur Irrigation Zone

Bahawalpur

Chapter 12

VULNERABLE SITES OF FLOOD BUND / STRUCTURE:

  - APPREHENDED BREACHES IN FLOOD BUNDS / STRUCTURES

The list of vulnerable sites are as follows

- RD: 453-473 of Extension Minchin Flood Bund a significant shift in the river trend. Site Plan and its position is attached herewith.

  - OPERATION OF BREACHIHNG SECTINOS

- No such breaching sections in the reach of Dallas Canal Division.

12.3	BREACHES DUE TO RISE OF FLOOD WATER, DETERIORATION OF FLOOD BUNDS.

- No such occurrence.

Chapter 13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE

- Vulnerable Site Rd: 453-473 of Extension Minchin Flood Bund

During falling condition of river in 2016, the main river changed its behavior and a deep creek developed just downstream concavo convex guide wall at RD: 453+735 of extension Minchin Flood Bund.

The main river started erosion to the bund from RD 462+000 to 463+000 during June, 2018. The three No. solid stone studs were constructed to protect the bund from aggressive behaviour of River Indus under the supervision of Third Party Monitoring (Consultant) during flood season of 2018. The main river was flowing near the bund from RD: 458+000 to 466+000. The River started erosion aggressively and was approaching to the bund. The offshore from river edge to the bund at RD: 464+000 was 12 feet and continuous erosion was in progress. The working paper was submitted to the office of PRO (Hydraulic) to carry out model study. Model Study report was printed in June, 2019 but proposed intervention cannot be executed due to flood season. In January, 2019 the river behavior was totally changed and new creak was developed hitting foreshore at RD 459+000 and developing sharp convex loop, threating the safety of adjoining abadies, valuable agricultural land and other infrastructures.

The Minch Flood bund was successfully protected / saved by hectic efforts during flood season, 2024.

After flood season 2019, working paper was submitted to Departmental Expert Committee after conducting detailed river survey. The Departmental Expert Committee proposed construction of 03 Nos Inverted Hockey at RD 462+000, 463+800 and 465+500. The execution work for construction of these Hockey has been completed and a new Hockey at RD: 460+500 has also been constructed after recommendations of DEC. In 2023, there was a significant shift in the river trend, causing the river to flow dangerously close to the toe of the flood bund. This resulted in erosion along the riverbank in the stretch between RD 466+000 and 473+000. The river is currently flowing near the toe of the flood bund in reach 468-473. Flood fighting measures were implemented in flood 2024, ensuring the stability of the site. A model study for this reach is in progress, and a dedicated scheme has been included in the Annual Development Program (ADP).

13.1 	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE 		BREACH SUPPORTED WITH LEVELS.

If breach occur at RD: 453-473 of Extension Minchin Flood Bund, the flood water will pass between the area of Extension Minchin Flood Bund and Bhong Disty which is running parallel.

13.2    DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

- RD 453 to 473of Extension Minchin Bund

The following abadies likely to be affected i.e. Jamal Din wali city, Mauza Sonmiani, Kot Miran Shah, Kacha Razi, Basti Jhulan, Basti Sona zimkhani and Bhong city situated in the area.

13.3 	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

- RD: 453-473 OF EXTENSION MINCHIN FLOOD BUND

During falling condition of river in 2016, the main river changed its behavior and a deep creek developed just downstream concavo convex guide wall at RD: 453+735 of extension Minchin Flood Bund.

The main river started erosion to the bund from RD 462+000 to 463+000 during June, 2018. The three No. solid stone studs were constructed to protect the bund from aggressive behaviour of River Indus under the supervision of Third Party Monitoring (Consultant) during flood season of 2018. The main river was flowing near the bund from RD: 458+000 to 466+000. The River started erosion aggressively and was approaching to the bund. The offshore from river edge to the bund at RD: 464+000 was 12 feet and continuous erosion was in progress. The working paper was submitted to the office of PRO (Hydraulic) to carry out model study. Model Study report was printed in June, 2019 but proposed intervention cannot be executed due to flood season. In January, 2019 the river behavior was totally changed and new creak was developed hitting foreshore at RD 459+000 and developing sharp convex loop, threating the safety of adjoining abadies, valuable agricultural land and other infrastructures.

After flood season 2019, working paper was submitted to Departmental Expert Committee after conducting detailed river survey. The Departmental Expert Committee proposed construction of 03 Nos Inverted Hockey at RD 462+000, 463+800 and 465+500. The execution work for construction of these Hockey has been completed and a new Hockey at RD: 460+500 has also been constructed after recommendations of DEC. In 2023, there was a significant shift in the river trend, causing the river to flow dangerously close to the toe of the flood bund. This resulted in erosion along the riverbank in the stretch between RD 466+000 and 473+000. The river is currently flowing near the toe of the flood bund in reach 468-473. Flood fighting measures were implemented in flood 2024, ensuring the stability of the site. A model study for this reach is in progress, and a dedicated scheme has been included in the Annual Development Program (ADP).

13.3.1 	ARRANGEMENTS

According to flood stages, this division has made the following arrangements for flood strategy at different stages and have explained above, however their plans are as under:-

- Day and night watching of Minchin Flood Bund and Spurs will be done by deputing the staff and establishing watching camps.

- Ample quantity of watching / flood fighting material will be arranged at each camp throughout the flood season.

- Additional machinery will be deployed:

- Lighting arrangements i.e. Generators and search lights will be arranged at each camp site in addition to arrangement already done on commencement of flood season.

- An Army contingent Jawans will also patrol along Minchin Flood Bund.

- All crossings on all the bunds will be closed thoroughly.

13.3.2 	ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

Camp will be established at following site

SECTION I (SUB DIVISIONAL OFFICER BUND)

| 1 | Sub Section RD:243-272 and all allied Structure. | SDO Bund Sub Engineer Rukanpur Bund I | Camp at RD:262, Minchin Flood Bund |
| --- | --- | --- | --- |
| 2 | Sub Section RD:272-325 Minchin Flood Bund and all allied Structure | SDO Bund Sub Engineer Adamwali Pallu Shah | Camp at RD:312, Minchin Flood Bund |

SECTION II (SUB DIVISIONAL OFFICER BUND)

| 1 | Sub Section RD:325-382 Minchin Flood Bund. | SDO Bund Sub Engineer Degi, Bund II | Camp at RD:353, Minchin Flood Bund |
| --- | --- | --- | --- |
| 2 | Sub Section RD:382-445 Minchin Flood Bund. | SDO Bund Sub Engineer Sharin, Shahbaz Pur | Camp at RD:388, Minchin Flood Bund |

SECTION III (SUB DIVISIONAL OFFICER BUND)

| 1 | Sub Section RD:445-470 Minchin Flood Bund and all allied structure. | SDO Bund Sub Engineer Bhong, Bund III | Camp at RD:460, Minchin Flood Bund |
| --- | --- | --- | --- |
| 2 | Sub Section RD:470-511 Minchin Flood Bund. | SDO Bund Sub Engineer Kasmani, Lamma | Camp at RD:511, Minchin Flood Bund |
| 3 | Sub Section RD:0-38 Minchin Flood Bund and loop Bund, all allied structure | SDO Bund Sub Engineer Duawala, Walhar | Camp at RD:16+000, Minchin Flood Bund |

    - DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES.

Camp at RD:262, Minchin Flood Bund Abaid Ullah Anwar SDO Bund Cell No. 0303-4711114

| 1 | Sub Section RD:243-272 and all allied Structure. | Tahir Masod Sub Engineer Rukanpur Cell # 0302-7703225 | Sub Engineer Bund I Vacant Post |
| --- | --- | --- | --- |

Camp at RD:312, Minchin Flood Bund

| 2 | Sub Section RD:272-325 Minchin Flood Bund and all allied Structure | Waseem Haider Sub Engineer Pallu Shah Cell Cell-0307-3921085 | Nasir Shah Sub Engineer Adam Wali 0300-6748180 |
| --- | --- | --- | --- |

Camp at RD:353, Minchin Flood Bund Abaid Ullah Anwar SDO Bund Cell No. 0303-4711114

| 1 | Sub Section RD:325-382 Minchin Flood Bund. | Khalid Rafiq Sub Engineer Degi Cell#0300-6802103 | Sub Engineer Bund II Vacant Post |
| --- | --- | --- | --- |

Camp at RD:388, Minchin Flood Bund

| 2 | Sub Section RD:382-445 Minchin Flood Bund. | Waseem Haider Sub Engineer Sharin, Cell No# 0307-3921085 | Muhammad Rehan Siraj Sub Engineer Shahbaz Pur Cell No# 03087452242 |
| --- | --- | --- | --- |

Camp at RD:460, Minchin Flood Bund Abaid Ullah Anwar SDO Bund Cell No. 0303-4711114

| 1 | Sub Section RD:445-470 Minchin Flood Bund and all allied structure. | Muhammad Tahir Sub Engineer Bhong Cell No# 0302-9225223 | Khalid Rafiq Bhatti Sub Engineer Bund III Cell No 03006802103 |
| --- | --- | --- | --- |

Camp at RD:511, Minchin Flood Bund

| 2 | Sub Section RD:470-511 Minchin Flood Bund. | Amanit Ali Sub Engineer Kasmani CellNo# 03006730338 | Khalid Rafiq Bhatti Sub Engineer Lamma Cell No. 03006802103 |
| --- | --- | --- | --- |

Camp at RD:16+000, Minchin Flood Bund

| 3 | Sub Section RD:0-38 Minchin Flood Bund and loop Bund, all allied structure | Sub Engineer Duawala Cell No. Vacant Post | Anjum Shaheen Sub Engineer Walhar Cell No 0301-7634244 |
| --- | --- | --- | --- |

    - DEPARTMENTAL MACHINERY AVAILABLE

Not Available

    - MACHINERY AVAILABLE FROM PRIVATE SOURCE

As per requirement machinery will be arranged through contractor i.e Dozer, Excavator, Tractor Trolley, Front blade Tractor. The requirement of machinery is as under in very high stage:

- RD: 453 – 466 Extension Minchin Flood Bund

Dozer				=	01 Nos.

Excavator			=	01 Nos.

Tractor with front blade	=	02 Nos.

Tractor with Trolley		=	04 Nos

However the Agriculture Department will have to supply the dozer as provided in the District Flood Plan to meet any eventuality. The District Coordination Officer Rahimyarkhan will be specially requested to direct the District Machinery pool officer to arrange the same well in time.

    - FLOOD MATERIAL REQUIRED

AND

    - FLOOD  MATERIAL AVAILABLE

| S.No | Status of the Watching Material |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Name of Item | Total Required Quantity | Available Quantity | Balance Required Quantity |  |
| 1 | Supplying jute bags 4.0 to 5.0 Cft Capacity. | 8000 No. | 0 No. | 8000 No. | Balance required quantity will procured through contractor prior to flood 2025. |
| 2 | Supplying plastic bags. | 15000 No. | - | 15000 No. |  |
| 3 | Supply sutli for sewing jute bags | 12 Kg | 0 Kg | 12 Kg |  |
| 4 | Supply needle for sewing jute bags | 60 No. | 0 No. | 60 No. |  |
| 5 | Supply toote Basket | 300 No. | 0 No. | 300 No. |  |
| 6 | Supplying Search Light Torches Large. | 12 No. | 0 No. | 12 No. |  |
| 7 | Supplying Torches chargeable. | 180 No. | 0 No. | 180 No. |  |
| 8 | Supplying chairs & table set. | 6 Set | 0 Set | 6 Set |  |
| 9 | Supplying Kassi with handle | 180 No. | 0 No. | 180 No. |  |
| 10 | Supplying khajji mates | 4500 No. | - | 4500 No. |  |
| 11 | Supplying axes with handle. | 180 No. | 0 No. | 180 No. |  |
| 12 | Supplying wooden hammer | 12 No. | - | 12 No. |  |
| 13 | Supplying chowldari. | 12 No. | 0.No. | 12 No. |  |
| 14 | Supplying swiss cottage | 4 No. | - | 4 No. |  |
| 15 | Generator | 7 No. | 0.No. | 7 No. |  |
| 16 | Fuel per generator | As per situation | - | - |  |

  - DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDINGS, CANALS AND DRAINAGE NETWORK.

NIL

  - Vulnerable site RD: 453-466 Extension Minchin Flood Bund.

Map showing vulnerable RD: 453-466 Extension Minchin Flood Bund

To be inserted separately.

Chapter 14

ACTION PLAN

14.1	RE-SHUFFLING/ RECOUPING PLAN OF RESERVE STONE DEPARTMENTALLY

As per yard stick balance quantity of reserve stock stone will recouped prior to 30.06.2024. Re-shuffling of stone will be done through contractor at site as per requirement.

14.2	DETAIL OF INLET/OUTLET CROSSING ALONG WITH CLOSING METHODOLOGY

All the inlets / outlets crossings will be closed before flood.

| Name of Flood Bund / Structure | RD. of Inlet / Outlet |
| --- | --- |
| 2nd Defense Bund 4th Defense Bund 4th Defense Bund Extension Minchin Flood Bund | RD:4+000 RD:0-1000 RD:4-5 RD:372,354-55,357-58,359-60,360-61,361-62, 364-65,366-67,371,405-6,415-16,428-29,441-42, 378,391,401,404,405 |

14.3	DEPLOYMENT MACHINERY (MEDIUM TO HIGH FLOOD)

| Sr. No | Name of Structure / Bund | Site In charge by name & cell No. | Camp Location | Machinery Deployed |  |  |  |  | Available Quantity of Stone (Lac Cft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Excavator | Dozar | Tractor With Trolley / Dumper | Tractor with Blade | Gang / Labour (Beldar) |  |
| 1 | Reach from RD:243-272 and all allied Structure. | Sub Engineer Rukanpur Bund I | Camp at RD:262, Minchin Flood Bund | 1 | - | 2 | 2 | 20 | 2.06 |
| 2 | Reach from RD:272-325 Minchin Flood Bund and all allied Structure | Sub Engineer Adamwali Pallu Shah | Camp at RD:312, Minchin Flood Bund | 1 | - | 2 | 2 | 20 | 0 |
| 3 | Reach from RD:325-382 Minchin Flood Bund. | Sub Engineer Degi, Bund II | Camp at RD:353, Minchin Flood Bund | 1 | - | 2 | 2 | 20 | 4.014 |
| 4 | Reach from RD:382-445 Minchin Flood Bund. | Sub Engineer Sharin, Shahbaz Pur | Camp at RD:388, Minchin Flood Bund | 1 | 1 | 2 | 2 | 20 |  |
| 5 | Sub Section RD:445-470 Minchin Flood Bund and all allied structure. | Sub Engineer Bhong, Bund III | Camp at RD:460, Minchin Flood Bund | 1 | - | 2 | 2 | 20 | - |
| 6 | Sub Section RD:470-511 Minchin Flood Bund. | Sub Engineer Kasmani, Lamma | Camp at RD:511, Minchin Flood Bund | 1 | - | 2 | 2 | 20 |  |
| 7 | Sub Section RD:0-38 Minchin Flood Bund and loop Bund, all allied structure | Sub Engineer Duawala, Walhar | Camp at RD:16+000, Minchin Flood Bund | 1 | - | 2 | 2 | 20 |  |
|  |  |  | Total | 7 | - | 14 | 14 | 140 |  |

Note:-Payment of machinery and work charged establishment subject to verification by the TPM / Consultant.

14.4	DEPLOYMENT OF MACHINERY (HIGH TO VERY HIGH FLOOD)

| Sr. No | Name of Structure / Bund | Site In charge by name & cell No. | Camp Location | Machinery Deployed |  |  |  |  | Available Quantity of Stone (Lac Cft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Excavator | Dozar | Tractor With Trolley / Dumper | Tractor with Blade | Gang / Labour (Beldar) |  |
| 1 | Reach from RD:243-272 and all allied Structure. | Sub Engineer Rukanpur Bund I | Camp at RD:262, Minchin Flood Bund | 2 | - | 4 | 4 | 30 | 2.06 |
| 2 | Reach from RD:272-325 Minchin Flood Bund and all allied Structure | Sub Engineer Adamwali Pallu Shah | Camp at RD:312, Minchin Flood Bund | 2 | - | 4 | 4 | 30 | 0 |
| 3 | Reach from RD:325-382 Minchin Flood Bund. | Sub Engineer Degi, Bund II | Camp at RD:353, Minchin Flood Bund | 2 | - | 4 | 4 | 30 | 4.014 |
| 4 | Reach from RD:382-445 Minchin Flood Bund. | Sub Engineer Sharin, Shahbaz Pur | Camp at RD:388, Minchin Flood Bund | 2 | 1 | 4 | 4 | 30 |  |
| 5 | Sub Section RD:445-470 Minchin Flood Bund and all allied structure. | Sub Engineer Bhong, Bund III | Camp at RD:460, Minchin Flood Bund | 2 | - | 4 | 4 | 30 | - |
| 6 | Sub Section RD:470-511 Minchin Flood Bund. | Sub Engineer Kasmani, Lamma | Camp at RD:511, Minchin Flood Bund | 2 | - | 4 | 4 | 30 |  |
| 7 | Sub Section RD:0-38 Minchin Flood Bund and loop Bund, all allied structure | Sub Engineer Duawala, Walhar | Camp at RD:16+000, Minchin Flood Bund | 2 | - | 4 | 4 | 30 |  |
|  |  |  | Total | 14 | - | 28 | 28 | 210 |  |

Note:-Payment of machinery and work charged establishment subject to verification by the TPM / Consultant.

14.5	DEPLOYMENT OF MACHINERY ( VERY HIGH TO EXCEPTIONALLY HIGH FLOOD)

| Sr. No | Name of Structure / Bund | Site In charge by name & cell No. | Camp Location | Machinery Deployed |  |  |  |  | Available Quantity of Stone (Lac Cft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Excavator | Dozar | Tractor With Trolley / Dumper | Tractor with Blade | Gang / Labour (Beldar) |  |
| 1 | Reach from RD:243-272 and all allied Structure. | Sub Engineer Rukanpur Bund I | Camp at RD:262, Minchin Flood Bund | 3 | 1 | 5 | 5 | 50 | 2.06 |
| 2 | Reach from RD:272-325 Minchin Flood Bund and all allied Structure | Sub Engineer Adamwali Pallu Shah | Camp at RD:312, Minchin Flood Bund | 3 | - | 5 | 5 | 50 | 0 |
| 3 | Reach from RD:325-382 Minchin Flood Bund. | Sub Engineer Degi, Bund II | Camp at RD:353, Minchin Flood Bund | 3 | - | 5 | 5 | 50 | 4.014 |
| 4 | Reach from RD:382-445 Minchin Flood Bund. | Sub Engineer Sharin, Shahbaz Pur | Camp at RD:388, Minchin Flood Bund | 3 | 1 | 5 | 5 | 50 |  |
| 5 | Sub Section RD:445-470 Minchin Flood Bund and all allied structure. | Sub Engineer Bhong, Bund III | Camp at RD:460, Minchin Flood Bund | 3 | 1 | 5 | 5 | 50 | - |
| 6 | Sub Section RD:470-511 Minchin Flood Bund. | Sub Engineer Kasmani, Lamma | Camp at RD:511, Minchin Flood Bund | 3 | - | 5 | 5 | 50 |  |
| 7 | Sub Section RD:0-38 Minchin Flood Bund and loop Bund, all allied structure | Sub Engineer Duawala, Walhar | Camp at RD:16+000, Minchin Flood Bund | 3 | - | 5 | 5 | 50 |  |
|  |  |  | Total | 21 | 3 | 35 | 35 | 350 |  |

Note:-Payment of machinery and work charged establishment subject to verification by the TPM / Consultant.

14.6	POLICE DEPLOYMENT PLAN

| Sr. No | Name of Structure / Bund | Camp Location | Site Incharge by name & Cell No. | Detail of Police |
| --- | --- | --- | --- | --- |
| 1 | Minchin Flood Bund RD: 243 to 310 i/c Guide Wall spur No. 5 | J- Head Spur No. 5 RD: 260 | Tahir Masood Sub Engineer Rukanpur 0302-7703225 | Inspector = 01 No. ASI = 02 No. Constable = 10 No. |
| 2 | 4th Defence Bund RD: 0 to 16 i/c J-Head Spur No. 4, Inverted Hockey Spur U/S of J-Head Spur No. 4 | J-Head Spur No. 4 RD: 8, 4th Defence Bund | Waseem Haider SBE Pallu Shah 0307-3921085 | ASI = 01 No. Constable = 05 No. |
| 3 | Ext: M Flood Bund RD: 309+500 to 509+900 i/c J-Head Spur 3 No, Guide Head Spur, Guide Wall Spur, Solid Stone Stud 15 No | J-Head Spur No. 1 RD: 388 Ext: M. Bund | Khalid Bhatti SBE Degi 0300-6802103 | ASI = 01 No. Constable = 05 No |
|  |  | Guide Wall Spur RD: 453 Ext: M.Bund | Waseem Haider SBE Sharin 0307-3921085 | Inspector = 01 No. ASI = 02 No. Constable = 10 No. |
|  |  | RD: 460 Ext: M.Bund | Tahir Maqsood SBE Bhong 0302-9225223 | ASI = 01 No. Constable = 05 No. |
| 4 | Machka Dilawara Bund RD: 0 to 34+000 | RD: 26 - 27 | Tahir Maqsood SBE Dauwala 0302-9225223 | ASI = 01 No. Constable = 05 No. |
| 5 | Loop Flood Bund RD: 0 to 60+70 | RD: 6 | Amanat Ali SBE Kasmani 0300-6730338 | ASI = 01 No. Constable = 03 No. |

14.7	DETAILS OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG AND 1000 KG

| Sr. No | Name of Structure / Bund | Camp Location |  | Site Incharge by name & Cell No. | Detail of Synthetic Bags with capacity 500 kg |  | Detail of Synthetic bags with capacity of 1000 kg |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Required | Available | Required | Available |  |
| 1 | Minchin Flood Bund RD: 243 to 310 i/c Guide Wall spur No. 5 |  | J- Head Spur No. 5 RD: 260 | Tahir Masood Sub Engineer Rukanpur 0302-7703225 | 3000 | 0 | 1500 | 0 | Synthetic bags will be recouped prior to flood 2025 |
| 2 | 4th Defence Bund RD: 0 to 16 i/c J-Head Spur No. 4, Inverted Hockey Spur U/S of J-Head Spur No. 4 |  | J-Head Spur No. 4 RD: 8, 4th Defence Bund | Waseem Haider SBE Pallu Shah 0307-3921085 | 2000 | 0 | 1000 | 0 |  |
| 3 | Ext: M Flood Bund RD: 309+500 to 509+900 i/c J-Head Spur 3 No, Guide Head Spur, Guide Wall Spur, Solid Stone Stud 15 No |  | J-Head Spur No. 1 RD: 388 Ext: M. Bund | Khalid Bhatti SBE Degi 0300-6802103 | 2000 | 0 | 1000 | 0 |  |
|  |  |  | Guide Wall Spur RD: 453 Ext: M.Bund | Waseem Haider SBE Sharin 0307-3921085 | 4000 | 0 | 2000 | 0 |  |
|  |  |  | RD: 460 Ext: M.Bund | Tahir Maqsood SBE Bhong 0302-9225223 | 2000 | 0 | 1000 | 0 |  |
| 4 | Machka Dilawara Bund RD: 0 to 34+000 |  | RD: 26 - 27 | Tahir Maqsood SBE Dauwala 0302-9225223 | 2000 | 0 | 1000 | 0 |  |
| 5 | Loop Flood Bund RD: 0 to 60+70 |  | RD: 6 | Amanat Ali SBE Kasmani 0300-6730338 | 1500 | 0 | 1000. | 0 |  |

14.8	DETAILS OF POLYTHENE SHEETS OF BLACK COLOR TO PROTECT UPSTREAM SLOPE AGAINST WAVE ACTION AND TO CONTROL SEEPAGE THROUGH EMBANKMENTS.

| Sr. No | Name of Structure / Bund | Camp Location | Site Incharge by name & Cell No. | Detail of Polythene required |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Required | Available |  |
| 1 | Minchin Flood Bund RD: 243 to 310 i/c Guide Wall spur No. 5 | J- Head Spur No. 5 RD: 260 | Tahir Masood Sub Engineer Rukanpur 0302-7703225 | 30000 Sft | 0 | Polythene sheets will be recouped prior to flood 2025 |
| 2 | 4th Defence Bund RD: 0 to 16 i/c J-Head Spur No. 4, Inverted Hockey Spur U/S of J-Head Spur No. 4 | J-Head Spur No. 4 RD: 8, 4th Defence Bund | Waseem Haider SBE Pallu Shah 0307-3921085 | 20000 Sft | 0 |  |
| 3 | Ext: M Flood Bund RD: 309+500 to 509+900 i/c J-Head Spur 3 No, Guide Head Spur, Guide Wall Spur, Solid Stone Stud 15 No | J-Head Spur No. 1 RD: 388 Ext: M. Bund | Khalid Bhatti SBE Degi 0300-6802103 | 30000 Sft | 0 |  |
|  |  | Guide Wall Spur RD: 453 Ext: M.Bund | Waseem Haider SBE Sharin 0307-3921085 | 40000 Sft | 0 |  |
|  |  | RD: 460 Ext: M.Bund | Tahir Maqsood SBE Bhong 0302-9225223 | 20000 Sft | 0 |  |
| 4 | Machka Dilawara Bund RD: 0 to 34+000 | RD: 26 - 27 | Tahir Maqsood SBE Dauwala 0302-9225223 | 20000 Sft | 0 |  |
| 5 | Loop Flood Bund RD: 0 to 60+70 | RD: 6 | Amanat Ali SBE Kasmani 0300-6730338 | 10000 Sft | 0 |  |

Chapter 15

BACKUP DIVISIONS (IN CASE OF BREACH)

- Khanpur Canal Division Khanpur.

It is certified that there is no breaching section exists in Dallas Canal Division. In case of emergency / breach Khanpur Division will be act as backup Division.

Chief Engineer

Bahawalpur Irrigation Zone

Bahawalpur

Executive Engineer

Dallas Canal Division

Rahimyarkhan

Superintending Engineer

Rahimyarkhan Canal Circle

Rahimyarkhan