Chapter No.1

SALIENT FEATURES JAMPUR CONSTRUCTION DIVISION, RAJANPUR

  - LOCATION

Rajanpur District is located in the extreme southwest part of Punjab, Pakistan (29:06N, 70:19E) with a geographical span of 12,319 km2. It is pertinent to mentioned here that District Rajanpur is one of the main district of Southern Punjab located at right side of the River Indus which is directly suffered by hill torrents flood and River Indus flood. The land of the District Rajanpur is sandwiched between River Indus and Suleman Mountains Range. According to the National Census Report 2017, it had a population of 1,996,000 and under the local administrative break up, whole the District is divided into three Tehsils, Jampur (19 Union  Councils), Rajanpur (14 Union Councils), Rojhan Mazari (08 Union Councils) and a vast belt of Provincially Administered Tribal Area (PATA).

The Headquarter of Jampur Construction Division; Rajanpur is situated at Rajanpur, which comprises of three Sub Divisions namely (i) Mechanical Sub Division at Jampur, (ii) Hill Torrent-I Sub Division at Jampur & (iii) Hill Torrent No. II Sub Division at Rajanpur.

Jurisdiction of Sub Divisions is as under:-

- Mechanical Sub Division at Jampur.

Flood Bunds with their systems & right side of River Indus from Jakhar Imam Shah to Kot Mithan.

- Hill Torrent-I Sub Division at Jampur.

Drains with their systems from Dajal Branch to River Indus.

- Hill Torrent-II Sub Division, Rajanpur.

Management and distribution of Hill Torrent Flood water and flood bunds along right edge of River Indus from Kot Mithan to Kashmoor.

1.2. General Description

Jampur Construction Division, Rajanpur is responsible for maintenance and development of allied training works along right side of River Indus & flood embankments, drains / flood carrying channels, as described following briefly:-

- River Indus

Right side of river Indus falls within the jurisdiction of Rajanpur District up-to Guddu Barrage in Sind Province and part of pond area of Guddu barrage is in the jurisdiction of District Rajanpur. The part of Jampur Flood Bund from RD. 0+000 – 26+000, Ganda Jakhar Imam Shah, J-Head Spur at RD: 10+000 Banuhan Disty, J-Head Spur upstream Basti Rindan, J-Head Spur Jakhar Imam Shah & 3 No. Studs are in the jurisdiction of D.G. Khan District.

- Hill Torrents

Similarly on the western side, we have to confront with hill torrent flood of major hill torrents Kaha, Chachar, Pitok, Sori Shumali, Zangi & Sori Janubi originating from Suleman Range. The watersheds of these torrents lie in Balochistan Province and the flood water from Balochistan to Punjab passes through the Pachad Area (western area) of Rajanpur District. A network of storm water and seepage drains in District Rajanpur is also maintained by Jampur Construction Division, Rajanpur.

1.3. 				   ADMINISTRATIVE SETUP

| Executive Engineer |  |  |  |
| --- | --- | --- | --- |
| SDO Mechanical JMP | SDO HT-1 JMP | SDO HT-II RJP | Office |
| Workshop | Mithawan – I | Kaha – I | Drawing Branch |
| Kotla Mughlan | Mithawan – II | Kaha – II | Account Branch |
| Jampur | Sori Lund | Chachar | C-Branch |
| Store Section | Sakhi Sarwar. | Pitok |  |
|  | Mithanwan III. | Sori Shumali |  |
|  | Vidore | / Zangi |  |
|  |  | Sori Janubi |  |

| Executive Engineer |  |  |
| --- | --- | --- |
| C- Branch | Account Branch | Drawing Branch |

***

| Sr. No. | Officer Name | Designation | Phone No. |
| --- | --- | --- | --- |
| 1 | Muhammad Munawar Iqbal | Executive Engineer | 0333-6296609 |
| 2 | Muhammad Abdul Mueed | SDO-HT-I | 0335-0502269 |
| 3 | Qazi Faiz ul Hassan | SDO-HT-II | 0334-6431973 |
| 4 | Muhammad Nasrullah | SDO-Mechanical | 0333-3625555 |

***

Mechanical Sub Division, Jampur

| Sr. No. | Name | Section | Phone No. |
| --- | --- | --- | --- |
| 1 | Sabir Hussain | Kotla Mughlan | 0300-3282020 |
| 2 | Ubaidullah | Jampur | 0331-7014141 |
| 3 | Tanvir Hussain | Store & Workshop | 0336-6212021 |
| 4 | Samee Ullah Khosa | Workshop | 0333-8560665 |

***

Hill Torrent No. I, Sub Division, Jampur.

| Sr. No. | Name | Section | Phone No. |
| --- | --- | --- | --- |
| 1 | Muhammad Rashid Ali | Mithawan – I | 0334-6684599 |
| 2 | Muhammad Azam Suleman | Mithawan – II | 0344-5493305 |
| 3 | Muhammad Orangzaib | Sori Lund | 0333-6455321 |
| 4 | Sabir Hussain (Additional) | Sakhi Sarwar | 0300-3282020 |
| 5 | Fiaz Baddar | Mithanwan – III | 0336-7069700 |
| 6 | Azhar Manzoor (Additional) | Vidore | 0333-5093694 |

***

Hill Torrent No. II, Sub Division, Rajanpur.

| Sr. No. | Name | Section | Phone No. |
| --- | --- | --- | --- |
| 1 | Azhar Manzoor | Kaha – I | 0333-5093694 |
| 2 | Muhammad Adnan Saleem | Kaha – II | 0334-1737777 |
| 3 | Muhammad Rafiq Gabool | Chachar | 0333-8822684 |
| 4 | Muhammad Adeeb | Pitok | 0334-6940819 |
| 5 | Muhammad Kashif | Sori Shumali / Zangi | 0336-6163964 |
| 6 | Muhammad Younis Sharif | Sori Janubi | 0333-7739413 |

Chapter No.2 	FLOOD PROTECTION & RIVER TRAINING WORKS

General Abstract

| Sr.# | Type of Structure | No. | Length in Mile |
| --- | --- | --- | --- |
| 1 | Flood Bund | 10 | 74.120 |
| 2 | Stud | 6 | 0.53 |
| 3 | Groyne | 8 | 12.070 |
| 4 | Spur | 6 | 3.710 |
| 5 | Drains / Flood Carrying Channels | 6 | 99.75 |

2.1 			i-Design Parameter of River Training Works

| Sr. No. | Description | Length (Ft) |  | In Mile | Top Width (ft) | Side Slopes | Design Top level | Apron Top Level | H.F.L 2010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Total | Shank |  |  |  |  |  |  |
| 1 | Gandha Jakhar Imam | 2900 | 2900 | 0.58 | 25 | 3:1/2:1 | 377.00 | 377.00 | 372.00 |
| i- | J-Head RD:10 Banuhan Dy. | 2875 | 1200 | 0.57 | 30 | 3:1/2:1 | 383.50 | 369.50 | 377.50 |
| ii- | J-Head Spur U/S Basti Rindan | 5485 | 3810 | 1.10 | 30 | 3:1/2:1 | 381.00 | 367.00 | 375.00 |
| iii- | J-Head Spur Jakhar Imam Shah | 4425 | 2650 | 0.88 | 30 | 3:1/2:1 | 378.00 | 364.00 | 372.00 |
| iv- | Stud No. 1 | 250 | 250 | 0.04 | 25’ | 2:1/2:1 | 386.50 | 375.00 | 380.50 |
| v- | Stud No. 2 | 250 | 250 | 0.04 | 15’ | 2:1/2:1 | 382.00 | 375.00 | 376.00 |
| vi- | Stud No. 3 | 200 | 200 | 0.03 | 25’ | 2:1/2:1 | 380.50 | 374.50 | 374.50 |

| Sr. No. | Description | RD | In Ft | In Mile | Top Width (ft) | Side Slopes | Design Top level | H.F.L 2010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1- | Jampur Flood Bund. | 0-18000 | 18000 | 3.60 | 16’ | 3:1/2:1 | 370.00 | 368.40 |
|  |  | 18000-71600 | 53600 | 10.72 | 20’ |  |  |  |
|  |  | 71600-103650 | 32050 | 6.41 | 20’ |  |  |  |
|  |  | 103650-170720 | 67070 | 13.41 | 15’ |  |  |  |
|  | (Wetting Channel) | 38000-52000 | 14000 | 2.80 | -- | -- | -- | -- |
| i- | Qallandar Shah Groyne | 0-4260 | 4260 | 0.85 | 20’ | 3:1/2:1 | 370.60 | 366.10 |
| ii- | Shah Jamal Groyne | 0-6000 | 6000 | 1.20 | 20’ | 3:1/2:1 | 365.00 | 361.85 |
| 2- | Chandia Flood bund. | 0-7750 | 7750 | 1.55 | 12’ | 3:1/2:1 | 374.00 | 369.00 |
| 3- | Gaddan Wala Bund | 0-19100 | 19100 | 3.82 | 15’ | 3:1/2:1 | 365.00 | 360.00 |
| i- | Ghafoor Groyne | 0-14000 | 14000 | 2.80 | 20’ | 3:1/2:1 | 362.00 | 360.00 |
| 4- | Laskani Flood Bund | 0-26365 | 26365 | 5.27 | 15’ | 3:1/2:1 | 374.40 | 370.40 |
| 5- | Sheru Flood Bund | 0-3116 | 3116 | 0.62 | 20’ | 2:1/2:1 | 71.45 | 65.45 |
| i- | Old Qalandar Shah Groyne | 0-18500 | 18500 | 3.70 | 16’ | 3:1/2:1 | 368.18 | 364.30 |
| ii- | Musa Groyne | 0-7000 | 7000 | 1.40 | 10’ | 3:1/2:1 | 358.49 | 354.49 |
| iii- | Kotla Mughlan Groyne | 0-1500 | 1500 | 0.30 | 12’ | 3:1/2:1 | 356.93 | 352.93 |
| iv- | L-Groyne | 0-3100 | 3100 | 0.62 | 12’ | 3:1/2:1 | 354.57 | 352.57 |
| v- | Boly Wala Groyne | 0-6000 | 6000 | 1.20 | 12’ | 3:1/2:1 | 346.57 | 344.57 |
| 6- | W.X. Bund. | 0-7000 | 7000 | 1.40 | 12’ | 3:1/2:1 | 351.20 | 346.20 |
| 7- | Harrow Flood bund. | 0-29500 | 15000 | 3.00 | 12’ | 3:1/2:1 | 360.30 | 354.30 |
| i- | Stud-1 | (25000) | 1140 | 0.23 | 20’ | 2:1/2:1 | 360.30 | 354.30 |
| ii- | Stud-2 | (26000) | 700 | 0.14 | 20’ | 2:1/2:1 | 360.30 | 354.30 |
| iii- | Stud-3 | (27000) | 270 | 0.05 | 20’ | 2:1/2:1 | 360.30 | 354.30 |
| 8- | Fakhar Flood Bund | 10-25300 | 15300 | 3.06 | 20’ | 3:1/2:1 | 306.35 | 301.35 |
| 9- | Rojhan Flood Bund | 0-61250 | 61250 | 12.25 | 20’ | 3:1/2:1 | 282.80 | 275.40 |
| i- | J-Head Spur Rojhan | 0-4660 | 4660 | 0.93 | 30’ | 3:1/2:1 | 281.71 | 276.71 |
| ii- | J-head Spur R-64 | 0-6280 | 6280 | 1.25 | 30’ | 3:1/2:1 | 288.19 | 283.19 |
| 10- | Umer Kot Flood Bund | 0-45000 | 45000 | 9.00 | 25’ | 3:1/2:1 | 300.00 | 293.00 |
| i- | J-Head Spur R-63 | 0-250 | 250 | 0.05 | 30’ | 3:1/2:1 | 289.95 | 284.95 |

ii-Design Parameter of Drains / Flood Carrying Channels

| Sr. No. | Name of Drain | Length |  | Outfall Capacity Cft. | Bed Width (Feet) |  | F.C. Depth (Feet) |  | Catchment Area (Sq. Mile) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | R.D. | Miles |  | At Out Fall | At Source | Out Fall | Source |  |
| 1 | New Manka | 126060 | 25.21 | RD: 37-outfall 2000 | 36 | 23.50 | 7.15 | 6.45 | 290.5 |
| 2 | Tayyab Drain | 131000 | 26.20 | 2000 | 80 | 80 | 8 | 8 | 700 |
| 3 | Drain / FCC 95 | 47000 | 9.40 | 1000 | 50 | 50 | 7 | 7 | 500 |
| 4 | Drain / FCC 109 | 63000 | 12.60 | 1000 | 50 | 50 | 7 | 7 | 500 |
| 5 | Darkhast Drain | 42700 | 8.54 | 135 | 20 | 8 | 5 | 3.5 | 67.5 |
| 6 | Bukhari Drain | 89000 | 17.80 | 1050 | 50 | 50 | 8 | 8 | 525 |
| 7 | FCC-145 | 91500 | 18.30 | 6000 | 200 | 150 | 9.95 | 6.8 | Flood water of Kaha H.T. |
| 8 | FCC-166 | 24500 | 4.90 | 1000 | 35 | 35 | 5.95 | 5.95 | -do- |
| 9 | FCC-177 | 64000 | 12.80 | 2000 | 80 | 35 | 7.81 | 5.95 | -do- |
| 10 | FCC-186 | 56000 | 11.20 | 5000 | 110 | 110 | 9.80 | 9.80 | Flood water of Chachar H.T. |
| 11 | Qutab Drain | 290000 | 58.00 | 9000 | 200 | 200 | 12.70 | 12.70 | Flood water received from FCC-123,145,166,177,&186. |

2.2.   LOCATION MAP

Chapter- 3.

BRIEF HISTORY OF PAST FLOOD EVENTS

The jurisdiction of Jampur Construction Division Rajanpur badly effects from floods originating from Suleman Range Hill torrents in West and from River Indus in East. Torrential and wide spread monsoon rains in the catchment area of Suleman Range and Indus basin generates unprecedented surface flows and plays havoc in the upper catchment areas.

Maximum flood passed through the jurisdiction of Jampur Construction Division, Rajanpur during the year 2010 (959991 Cs in River Indus, in the year 2013 (96000 Cs from Hill Torrents, in the year 2022 (108941 Cs and during the year 2024 (105276 Cs from Hill Torrents resultantly serious damages to river training / canal infrastructures were observed. During the year 2010, flood water of mighty River Indus also entered into the city of Jampur and Rajanpur which caused the serious damages of Govt./ public property, schools, graveyard etc.

Hill Torrent

During the Month July & August 2022, very high flood was received in all 07 major hill torrents in Jampur Construction Division and unprecedented floods were also received in sub hill torrents besides many “Churs” (minor torrents) which caused severe damages. The H/T- Source Chacher and Kaha flow a maximum discharge (75900 Cs, 108941 Cs) and caused a high flood discharge at Tayyab Drain, Bukhari Drain, Qutab Drain and its system i-e FCC-123, FCC-145, FCC-166, FCC-177 and FCC-186 while the flood carrying channels have the capacity of 50 % discharge against the capacity of full capacity of Hill Torrent Crossings. The torrential flash floods 2022 have severely damaged the banks of subject hill torrents at different places for which the area of Tehsil Jampur, Rajanpur, Rojhan and many others populated areas were endangered to disaster.

During flood season 2023, a discharge of 35968 Cs received from Kaha Hill Torrent on 30.07.2023 and more than 3500 Cs entered into flood carrying channel (FCC) at RD: 145+000 of Dajal Branch. As a result the channel was breached at RD: 41-42/R and the water inundated the nearby lands of Chak Ahmadani.

Moreover, a discharge of 15000 Cs received from Chachar Hill Torrent on 29.07.2023 and more than 4500 Cs entered into flood carrying channel (FCC) at RD: 186+000 of Dajal Branch. As a result, the channel was breached at RD: 32-33/R & RD: 28000/R. The flood water after passing through nearby lands of Mouza Soon Wah and Chak Shaheed reached at RD: 23-24/R of FCC-186 and was headed up. To provide relief to adjoining abadies and fertile area, a relief cut was made at RD: 23-24/R and all the flood water was drained out from the relief cut.

Keeping in view the critical / alarming site situation, the restoration of both works were taken and completed through resourceful contractors under clause 59 d(iii) of PPRA Rules to avoid further public loss.

During flood season 2024, heavy rains occurred in District Rajanpur and eastern Baluchistan during the month of August twice i.e. on 05.08.2024 and then 17.08.2024 to 21.08.2024. In District Rajanpur, the hill torrents received flood water due to these rains, which caused flood situation very high to exceptionally high range in past few days.

The accumulated floodwaters formed extensive ponds, particularly along the right embankments of Kadra Canal and Mushraf Flood Bund, with average water depths reaching 10 feet. These conditions led to severe damage to the flood bunds and associated infrastructure due to wave action, seepage, and sloughing of embankments.

The Mushraf Flood Bund, located in Tehsil Rojhan, spans a length of 61,000 feet, plays a critical role in protecting several settlements (Mouza Dilber, Mouza Safdar abad, Chak Mats #1 and #2, Mouza Gianmal, Basti Jatt) and vital infrastructure (Tariq Minor, Mawah Disty, Ghous Bux Minor, Machka Minor, Indus Highway, and Rojhan City). Despite its importance, the flood bund has suffered significant damage during recent floods 2024, particularly from RD 30+000 to 58+000.

On 19.08.2024, Head Noshera at tail of Link No. III received more than 4000 discharge against 1365 CS authorized discharge. During such critical conditions, right bank of Link III overtopped and severe leakage was observed at various places, hence making Fazilpur City at risk of flooding. Similarly all the FCCs namely FCC 123+000, FCC 145+000, FCC 165+000, FCC 177+000 and FCC 186+000 (of Dajal Branch Canal) besides Tayyab drain also overtopped due to capacity constraints, hence inundating the surrounding lands and abadies.

Indus River

After subsiding of flood 2022, it was observed that the river changed its morphology in the front of Hairo flood bund. The creek in front of bund RD.25+000 to 29+600 widened its width about 2500 ft which was 1200 to 1500 ft before start of flood season. A new creek developed from upstream in front of RD.20+000 after making right ward loop between RD.20+000 to 25+000 and advancing toward the bund rapidly even at low discharge. Daily advancement of creek erosion was 10-15 ft. As per recommendation of DEC, an estimate for construction of 02 No. Studs was framed and after competitive bidding process work was awarded to M/S Abdul Aziz Govt. Contractor. The work for construction of 02 No Studs started on 02/05/2023. Both Studs were completed up to shank lengths along with apron and pitching. Construction of Solid portion of both studs was in progress that Indus River suddenly started drifting right ward and towards the flood bund due to oblique flow at angle of 60 degree. The river edge rapidly started decreasing between RD 20+250 and RD 23+500 due to huge distance of 3250ft between both studs. Moreover, an aggressive swirl action with high velocity was observed on downstream shank of stud Rd 23+500 near solid portion. Meanwhile soundings of both studs continuously increased as checked jointly with NESPAK Consultants. The severe swirl action with high velocity was very aggressive which targeted earthen shank of stud Rd 23+500 on D/s side. Utmost efforts were made to control the action by constructing mini prongs, placing Munj Patha Tarangars, tree launching, bamboo bushing and constructing rip rap but no fruitful results achieved. Similarly distance between river edge and the flood bund started decreasing rapidly on upstream of the stud RD: 23+500.

During night of 06-07/08/2023, River started straight hitting on upstream side of stud 23+500 up to 22+900. All possible efforts were made to protect the flood bund but affected length was increased up to 900 ft and action was very severe and aggressive that flood bund bisected and newly constructed studs also washed away. A severe swirl action in combination with boiling action was also observed on upstream of stud RD: 25+000 which targeted the stud both on upstream and downstream sides. As per departmental policy, Zonal Technical Committee also inspected the site on 08.08.2023.

On the recommendations of Zonal Technical Committee work for holding both ends of the flood bund was immediately started under para 2.89 of PWD Code and under 59d(iii) of PPRA Rules 2014. Administrative Department was requested to constitute expert members to visit the site and propose remedial measures to control the aggressive behavior of the river. Expert members comprising Chief Engineer D.G. Khan, Superintending Engineer Rahimyar Khan Circle, Superintending Engineer, Project Circle (Irrigation), D.G. Khan, Principal Research Officer (H), IRI Lahore and Resident Engineer M/S NESPAK Consultants D.G.Khan inspected the site of Hairo flood bund on 11.08.2023 and decided to hold the existing structures at their position by flood fighting and the matter must be referred to Departmental Expert Committee for further guidance.

After subsiding flood season-2023, necessary river survey was carried out by the field formation & working paper thereof was prepared & submitted to Departmental Expert Committee with proposal of 04 No. Hockey Spurs in between RD. 20+000 to 25+000 for placing in the meeting held on 27.10.2023 but the said proposal was not considered by the members & it was suggested that model study may be carried out prior to proceeding ahead. In response, fresh river survey plan was framed & submitted to PRO on 06.11.2023 for the needful. Moreover, necessary charges amounting to Rs. 4.20 Million were deposited to the Executive Engineer, Irrigation Research Institute, Lahore vide Ordinary Cheque No. G295950/G02960, dated 23.12.2023 for carrying out the model study. The process of model study remained stuck off due to unavoidable reasons. Furtherance, the said working paper were again submitted to DEC on short term solution basis on 19.02.2024 wherein it was recommended that 02 No. Sloping Studs may be constructed whereas the Stud at RD. 25+000 may be converted into Inverted Hockey Spur after furnishing of design parameters by the PRO.

For the needful, Chief Engineer, Irrigation D.G.Khan Zone, D.G.Khan vide letter No.Woks/2024/3058-63/178/94, dated 06.03.2024 requested the Chief Engineer, Irrigation Research Zone, Lahore to direct the Principle Research Officer to visit the vulnerable site & finalize the design parameters & in response the Principle Research Officer along with Resident Engineer, NESPAK Consultants visited the site on 09.03.2024 & after due consultation with worthy Chief Engineer, Irrigation D.G.Khan Zone, D.G.Khan the requisite geometry and location was finalized.

The issue of Hairo flood bund was aggravated during flood 2023 as main creek of river drifted towards the bund and its training works. As discussed above, the matter was taken to Departmental Expert Committee forum various times but proposal was not implemented due to non-availability of funds and rapid change of river course. Lately, working paper to DEC was submitted which was discussed on 24.06.2024 due to direct River action on nose of stud RD. 25+000 and passing of flow with extremely high velocity in a narrow creek in front of stud RD. 25+000.

The issue was again aggravated on 25.06.2024 and matter was reported to high ups to take remedial measures against erosive action of Indus River on damaged part of stud RD.25+000. The Zonal Technical Committee was constituted who inspected the site on 27.06.2024 and recommended initiation of work on emergent basis to save the damaged Spur RD. 25+000.

Similarly, in pursuance to Administrative Department, Lahore canal wire No. 304, dated 03.07.2024, the team of experts inspected the critical site of Hairo Flood Bund on 04.07.2024. After visiting the problematic / vulnerable site, the team of experts recommended that existing part of damaged stud at RD. 25+000 of Hairo flood bund should be hold to its present position expeditiously as per recommendation of DEC dated 24.06.2024 to avoid any untoward situation.

In light of the decision of Zonal Technical Committee as well as member of experts, tentative cost of work was worked out to be Rs. 175.00 Million which could vary due to river dynamic condition. Keeping in view the sensitivity of matter, the case was initiated under Para 2.89 of PWD Code & Clause 59 (d) (iii) of PPRA Rules because no funds were available with the Division to take up the work. The Administrative Department accorded formal approval of emergent work up to maximum Rs.175.00 Million vide No. SO(FLOODS) VI-11/2008(DGK) Dated:18.07.2024. The work was awarded to M/s Sparco Construction Company after negotiated tendering under clause 59 (d) (iii) of PPRA Rules 2014 vide Executive Engineer, Jampur Construction Division, Rajanpur letter No. 535-38 / AB dated: 19.07.2024. The contractor mobilized at site and the work was started on 15.07.2024.The work remained continued upto 10.09.2024 under the resident supervision of M/s NESPAK Consultants (M&R) D.G.Khan.

Due to fluctuation in River Indus Discharge during month of May and June 2024, the sever swirl action started on U/S of stud RD.20+250 in between line 600-982 resultantly the apron started launching rapidly & situation was reported vide Canal Wire No. 680, dated 25.06.2024. The Superintending Engineer Project Circle visited the site & confirmed the situation to high ups for taking remedial measures for the safety of structure. However, the morphology of the river was rapidly changed, resultantly Chief Engineer, Irrigation D.G.Khan Zone, D.G.Khan constituted a Zonal Technical Committee vide No. Works/2024/9443-50/SA/1098/PC dated: 26.06.2024. The committee visited the site on 27.06.2024 and suggested that launched apron of stud at RD.20+250 should be immediately restored. The team of experts also visited the site on 04.07.2024 and suggested that in case of any action at stud RD.20+-250, necessary steps may be taken in light of recommendation of DEC as per necessity / requirement against the site prevailing conditions. The site condition was again aggravated and the next meeting of ZTC was called on 12.07.2024, wherein it was recommended by committee that the remedial measures regarding damaged apron of stud RD.20+250 should be immediately taken up under Clause 59 (d) (iii) of PPRA Rules to prevent further damage to the structure. Estimate of the work was immediately framed and got sanctioned vide Chief Engineer, Irrigation D.G.Khan letter No. Works/2024/10400-04/407/16-JMP dated: 18.07.2024 after clearance from M/s NESPAK Consultants vide Resident Engineer, NESPAK (M&R) D.G.Khan letter No.4688/13/MAB/02/438 dated: 15.07.2024. The work was awarded to M/s Rana Traders vide this office Letter No 539-42/ AB dated:- 19.07.2024 after negotiated tendering under clause 59 (d) (iii) 2014. The contractor started the work and completed on 30.07.2024.

After completion of the work, the River Indus again suddenly changed morphology and washed the upstream apron of stud RD. 20+250 from Line No.400 to 982. The critical situation was intimated to high up vide Executive Engineer, Jampur Construction Division, Rajanpur vide C.W. No. 318 dated: 25.08.2024. After inspection the site, Superintending Engineer, Project Circle (Irrigation) D.G.Khan and Chief Engineer, Irrigation D.G.Khan Zone, D.G.Khan directed to immediately restore the launched apron vide their C.W. No. 2500 dated: 25.08.2024 and C.W. No.2503 dated: 25.08.2024 respectively. As per direction, Resident Engineer, NESPAK (M&R) D.G.Khan was requested to immediately observe the baseline data vide C.W.No. 320 dated: 25.08.2024. After observing the base line data jointly with NESPAK, the work was started and with hectic efforts, the structure was saved.

The detail of floods discharges in River Indus and Hill Torrents is explained in Chapter No. 4.

At present, all the river training Infrastructure within the jurisdiction of Jampur Construction Division Rajanpur has designed as per old highest flood levels, which needs to be updated as per flood levels observed during the flood 2010 in River Indus and during year 2013 as well as 2022 & 2024 from Hill Torrents. It is also pertinent to mention here that the damages of Kachhi Canal have not yet been restored as per original design by WAPDA since 2022. Resultantly, the ponded water is accumulated in the area / abadies such as Nizam Abad, Kanwala, Mouza Gianmal, Safdarabad, Arbi Tibba, Basti Jatt and many other scattered populations of Tehsil Rajanpur and Rojhan. The detail of floods damages occurred due to River Indus and Hill Torrents flood water during the year 2024 is explained in Chapter No. 6.

CHAPTER- 4.

Design Data, Historic Peak Data & Previous Five Year Flood Data of Head Works / Barrages & or Other Control Points.

4.1 FLOOD LIMITS.

| FLOOD POSITION. | DISCHARGE RANGE. |
| --- | --- |
| Normal Floods | Below 2.5 Lac Cs: |
| Low Floods | 2.50 Lac Cs: to 3.75 Lac Cs: |
| Medium Floods | 3.75 Lac Cs: to 5.00 Lac Cs: |
| High Floods | 5.00 Lac Cs: to 6.50 Lac Cs: |
| Very High Floods | 6.50 Lac Cs: to 8.00 Lac Cs: |
| Exceptionally High Floods | Above 8.00 Lac Cs: |

4.2. TIME LAG OF FLOODS:-

| Sr. No. | Reach | Distance in Km: | Time in hours. |
| --- | --- | --- | --- |
| 1 | Attock – Kala Bagh | 169 | 24 |
| 2 | Kala Bagh – Chashma | 58 | 12 |
| 3 | Chashma – Taunsa | 237 | 72 |
| 4 | Taunsa – Ghazi Ghat | 52 | 13 |
| 5 | Ghazi Ghat – Jakhar Imam | 31 | 8 |
| 6 | Jakhar Imam – Kot Mithan | 108 | 27 |
| 7 | Kotmithan – Rojhan | 51 | 13 |
| 8 | Rojhan – Guddu Barrage | 38 | 10 |
|  | Total | 744 | 179 |

4.3. HIGHEST RIVER FLOODS:-

| Sr. No. | Year | Peak Discharge (Cs) | Date | Day | Flood Position |
| --- | --- | --- | --- | --- | --- |
| 1 | 1992 | 654579 | 14.09.1992 | 1 | Very High Flood |
| 2 | 2006 | 612269 | 09.08.2001 | 1 | High Flood |
| 3 | 2010 | 959991 | 02.08.2010 | 1 | Exceptionally High Flood |
| 4 | 2013 | 516017 | 17.08.2013 | 1 | High Flood |
| 5 | 2015 | 597730 | 05.08.2015 | 1 | High Flood |
| 6 | 2022 | 624159 | 31.08.2022 | 1 | High Flood |
| 7 | 2023 | 419895 | 28.07.2023 | 1 | Medium Flood |
| 8 | 2024 | 440296 | 08.08.2024 | 1 | Medium Flood |

4.4. Peak Discharges.

i)-	At Darras

| Sr. No | Name of Hill Torrent | Design Capacity (Cs) | Peak Discharges (In Cusecs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2013 | 2014 | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 |
| 1 | Kaha at Darra | 118500 | 96000 | 21158 | 46855 | 40905 | 31032 | 31032 | 84100 | 41193 | 21158 | 108941 | 35968 | 105276 28000 (Kala & Baga Khosra) |
|  | (Kala Khosra + Bagga Khosra) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Chachar at Darra | 85500 | 65000 | 3200 | 30800 | 24300 | 5250 | 3250 | 14500 | 21158 | 3250 | 75900 | 15000 | 27640 |
| 3 | Pitok at Darra | 23900 | 3400 | -- | 700 | 2560 | 0 | 0 | 1150 | 0 | 0 | 5000 | 1200 | 63940 |
| 4 | Sori Shumali at Darra | 7713 | 3280 | -- | 1300 | 2900 | 0 | 0 | 1800 | 2500 | 0 | 7000 | 1000 | 14600 |
| 5 | Zangi at Darra | 3500 | 2950 | -- | 600 | 4750 | 2400 | 5850 | 2800 | 3500 | 0 | 9000 | 3500 | 5845 |
| 6 | Sori Janobi at Darra | 41600 | 7800 | 3350 | 3000 | 3300 | 5000 | 2170 | 4047 | 9200 | 7000 | 17000 | 3500 | 33600 |

ii)-	At River

| YEAR | DATE | DISCHARGE AT TAUNSA BARRAGE (Cusecs) |
| --- | --- | --- |
| 2008 | 9th August | 263320 |
| 2009 | 21st August | 320292 |
| 2010 | 02nd August | 959991 |
| 2011 | 01st September | 227698 |
| 2012 | 10th September | 235389 |
| 2013 | 17th August | 516017 |
| 2014 | 18th July | 247412 |
| 2015 | 05th August | 597730 |
| 2016 | 08th July | 343024 |
| 2017 | 07th August | 418388 |
| 2018 | 28th August | 252725 |
| 2019 | 15th August | 378194 |
| 2020 | 06th September | 479860 |
| 2021 | 04th August | 304121 |
| 2022 | 31st August | 624159 |
| 2023 | 28th July | 419895 |
| 2024 | 08th August | 440296 |

Chapter No.5.

Flood Fighting Strategy along River Indus

The Executive Engineer, Jampur Construction Division, Rajanpur will be over all in-charge and will stay at Jampur Canal Rest House during flood days in River Indus. He will take appropriate action according to the emergency.

By 1st of June, the strength of the existing establishment will be doubled by employing work charge establishment to clear the jungle on slopes and toe of the embankments shanks etc of the spurs. The extra establishment will be deployed for the detection and removal of holes of burrowing animals. These holes will be removed before 30th June-2023.

In emergency, the flood information is received from upper region. The Sub Divisional Officer and Sub Engineer in-charge will shift their headquarters at site. The establishment will be deployed according to the flood stage and as prescribed, round the clock vigilant watching will be arranged along the entire embankment to take care of eventualities. The staff will be equipped with chargeable torches, cell torches, gunny bags, kassies and any other relevant material to control seepage / leakage in the flood bund / embankment. The information is passed on quickly to the concerned quarter and simultaneous action will be taken to close the same. In the event of exceptionally high flood stage the heavy earth moving machinery (Bull Dozers) private and Agricultural Engineering Department / Machinery Circle will also be kept ready and placed on alert at vulnerable sites of Jampur Flood Bund RD. 24+000, 54+000 and Rojhan Flood Bund RD. 12+000 – 14+000 and RD. 12+000 of Fakhar Flood Bund for the safety of the public private e infrastructures, land and abadies of District Rajanpur. In emergency, relief cuts will be made in 2nd defence bunds or whenever required according to the site situation.

STAGE-1

- Normal Floods (Below 2.5 Lac Cs)

When river discharge ranges between these limits, flood water remains within the lips of River, it does not touches any flood bund or embankment, watching establishment shall be deployed at the rate of one man per mile/per shift for two shifts in a day. Sub Engineers concerned will supervise the entire watching operation along Spurs, studs and Flood Bunds from Jampur to Rojhan. Sub Divisional Officer concerned will act as the officer in-charge of watching operation.

STAGE-II

ii-   Low Floods (2.50 Lac Cs: to 3.75 Lac Cs)

When the discharge ranges between these limits, flood water remains within the lips of River, it does not touches any flood bund or embankment, watching establishment shall be deployed at the rate of one man per mile/per shift for two shifts in a day. Supervisory staff shall be the same as in stage-I.

STAGE-III

iii-  Medium Floods (3.75 Lac Cs: to 5.00 Lac Cs)

At this stage flood water starts touching the flood bunds / embankments. Watching establishment shall be deployed at the rate of three men per mile/per shift for three shifts in a day. Supervisory staff shall be the same as in stage-II.

STAGE-IV

iv-  High Floods (5.00 Lac Cs: to 6.50 Lac Cs)

Watching establishment shall be deployed at the rate of four men per mile/per shift for three shifts in a day. Supervisory staff shall be the same as in stage 1, their headquarters will be shifted at most critical points.

STAGE-V

v-   Very High Floods (6.50 Lac Cs: to 8.00 Lac Cs)

Watching establishment shall be deployed at the rate of five men per mile/per shift for three shifts in a day. Supervisory staff shall be the same as in stage 1, their headquarters will be shifted at most critical points.

STAGE-VI

vi-   Exceptionally High Floods (Above 8.00 Lac Cs)

At this stage, establishment at rate of five men per mile along with volunteer labours brought by Civil Administration and Army will help the watching operation. Police Guards will be deputed on vulnerable points to guard against any mishap/sabotage. Overall supervision of all operations will be done by the Executive Engineer.

WATCHING HUTS

Temporary watching huts are proposed at vulnerable sites. Detail of the field supervisory staff deployed for the purpose at the start of flood season is as under:-

| Sr. No. | Description. | No. |
| --- | --- | --- |
| 1 | J-Head Spur at Stud No. 4 at RD: 10 Banuhan Dy. | 1 No. |
| 2 | J-Head Spur U/S Basti Rindan | 1 No. |
| 3 | J-Head Spur Jakhar Imam Shah | 1 No. |
| 4 | Jampur Flood Bund RD. 24+000 | 1 No. |
| 5 | Harrow Flood Bund | 1 No. |
| 6 | Bukhari Drain. | 1 No. |
| 7 | New Manka Drain. | 1 No. |
| 8 | Tayyab Drain. | 1 No. |
| 9 | Fakhar Flood Bund RD. 14+000. | 1 No. |
| 10 | At nose of J-Head spur R – 63. | 1 No. |
| 11 | At nose of J-Head spur R – 64. | 1 No. |
| 12 | At nose of J-head spur R-65 near Rojhan. | 1 No. |
| 13 | At X-ing of district council bund Miranpur road. | 1 No. |
| 14 | Near Rojhan town RD. 16+000 Rojhan flood bund. | 1 No. |
| 15 | At RD. 22+000 Rojhan flood bund. | 1 No. |
| 16 | At RD: 4+000 Umer Kot Flood Bund | 1 No. |

Flood Fighting Strategy along Hill Torrents

Flood Fighting Strategy along Embankments of Dispersion Structures

During flood season from 1st April to 30th September, flood watching establishment will be deployed on embankment along dispersion structure of Kasha Hill Torrent. In case of warning / alert issued by met office as well as pdma, machinery will also be deployed on embankments for caring of earthen embankments and dispersion structures. Labour will be deployed at the rate of one person / mile in 02 Shifts.

Flood Fighting Strategy along Flood Carrying Channels and Main Drains

During flood season for watching and other activities of repair / maintenance of banks of Flood Carrying Channel as well as main drains, work charge establishment at the rate of one person / mile in 02 Shifts will be deployed. Similarly machinery will also be deployed upon receiving alert of freshet.

Chapter- 6

FLOOD DAMAGES RESTORATION WORKS 2024

A-Hill Torrents.

During flood season August 2024, maximum discharge received from Chachar hill torrent was 27640 Cs. and the design discharge of FCC-186 is 5000 Cs but the channel received discharge of 6596 Cs and due to excessive discharge and high velocity of 12-15 ft/sec, channel could not sustain and breached at various places. It is pertinent to mention here that after unprecedented flood of 2022 & 2023, banks of FCC-186 could not be strengthened due to unavailability of funds and condition of the banks is very poor due to passage of more than doubled discharge from its designed capacity. As a result, the channel was breached in between reach RD: 19-39 at RD 19-20/1 23-24/R,28000/R,32-33/R.32-33/L & 38-39/1. It inundated the Mouza Soonwah, Hajipur, Chak Shaheed, Patti Jumma Arrain and Chak Hulwani. The situation was intimated vide Sdo Hill torrent No 1 C.W No.122 dated 05-08-2024, resultantly Xen Jampur Construction Division, Rajanpur requested to Superintending Engineer, Project Circle, D.G.Khan vide C.W. No. 170 dated 05.08.2024 to execute the work under Para 59 d(iii) of PPRA Rules 2014. In response, Superintending Engineer, Project Circle, D.G.Khan directed to immediately take up the work by invoking emergent clause of PPRA Rules vide C.W No. 544 dated 05-08-2024.

Keeping in view the critical / alarming site situation, the following emergent nature work was taken through resourceful contractor under clause 59d(iii) of PPRA Rules to avoid further public loss.

| Sr. No. | Name of Work | Estimated Cost (M) | Remarks |
| --- | --- | --- | --- |
| 1 | Plugging priority breaches of flood carrying channel at RD 145+000 & FCC-186 of Dajal branch occurred during flood 2024. | 22.002 | The work got completed physically 100% in the supervision of NESPAK (M&R) Consultants. |

During date 16-20 August 2024, Kala Bagga Khosra Hill torrent flow a maximum discharge 28000 Cs. and caused a high discharge at FCC-79 and Darkhast Drain. The drain structures overflowed and breaches occurred which damaged Mouza Darkhast Jamal Khan, Mouza Hanbhi, Mouza Tal Shumali, Mouza Kaloi wala, Mouza Raqba Dhandla, Mouza Basti Dhandla and Mouza Rakh Azmat wala. Similarly, Kaha Hill torrent flow maximum discharge 105276 Cs. and caused Lundi Saidan to Lal Garh flood bund, Hazoori wah and Muhammad wah breached at various reaches which damaged the Lundi Saidan Town, allied abadies and fertile lands.

Keeping in view the serious site situation, the following works were taken through resourceful contractor to avoid further public loss.

| Sr. No. | Name of Work | Estimated Cost (M) | Remarks |
| --- | --- | --- | --- |
| 1 | Filling breaches and cuts damaged during flood 2024 at Tayyab drainage system.(FCC-79 & Darkhast Drain) | 7.348 | The works got completed physically 100% in the supervision of NESPAK (M&R) Consultants. |
| 2 | Closing / plugging priority breaches of Lundi Saidan to Lal Garh flood bund, Hazoori Wah & Muhammad Wah damaged during flood 2024. | 10.512 |  |

The Mushraf Flood Bund, located in Tehsil Rojhan, spans a length of 100,000 feet, plays a critical role in protecting several settlements (Mouza Dilber, Mouza Safdarabad, Chak Mats #1 and #2, Mouza Gianmal, Basti Jatt) and vital infrastructure (Tariq Minor, Mawah Disty, Ghous Bux Minor, Machka Minor, Indus Highway, and Rojhan City).

Flood water of all the hill torrents of Rajanpur is ponded between right flood embankment of Kadra Canal from RD 129+000 to 219+000 and Mushraf flood bund from RD 0+000 to 27+000 hence leading towards a large storage pond. During the pondage, average depth observed for this ponded water was 7 feet. Hence a total of 375000 Acre feet water was stored between two flood bunds i.e. Right Flood Embankment of Kadra Canal and Mushraf Flood Bund. Only drainage network of ponded water from Asni Lake to River Indus is Musharaf Drain. In order to control and dispose of this large storage of water into river Indus through Musharraf Drain, there remains need to have constant flood fighting in the form of closing breaches of canals including minors and distributaries besides Kadra Canal, controlling seepage of flood water into canal at various reaches, protecting outlets on right side of Kadra Canal from seepage and controlling sloughing of both Kadra and Mushraf flood bund from wave action by providing polythene sheets and plastic bags with bamboos. Furthermore, continuous soil dumping on river side (ponded side) was also required in order to restore the slopes under water so that seepage of flood water into canal as well as through flood bund due to exposure of hydraulic grade line. It is also worth mentioning that canal network of Rajanpur Canal Division is also used for draining of flood water of Kala BagaKhosra, Chachar, Pitok, SuriShumali, SuriJanubi, Zangi and Kaha Hill Torrent through Kadra Creek, Link No. III, Inlets of Kadracanal at RD 148+000, 206+000 and 219+000/R. Due to insufficient design parameters, most often the Kadra and Mushraf flood bund breaches due to exposure of hydraulic grade line.

Hence, in order to avoid continuous sloughing of slopes of Mushraf and Kadra flood bund when becomes under water (due to pondage of torrential water) and damaging of flood bund due to splashing of ponded flood water as a result of wave action and avert damage of flood embankment during periodic torrential flooding, it is necessary to strengthen both Musharaf and right Kadra flood bund from 0+000 to 28+000 and 129+000 to 223+000 considering the maximum water levels attained during torrential flood of year 2024. Similarly, in order to safely pass the flood water from Asni Lake to river Indus, there is also a dire need of capacity enhancement from existing 1500 cusecs to 10000 cusecs as well as strengthening of Musharaf drain as well.

Based on above mentioned discussion, in order to avoid such breaching of flood embankment as well as drainage system due to exposure of hydraulic grade line and overtopping, following works are proposed:-

| Sr. No. | Name of Work | Estimate Cost (M) | Status |
| --- | --- | --- | --- |
| 1 | Strengthening & Raising of Mushraf Flood Bund from RD 0 to 61+500 | 643.789 | Estimate sanctioned vide No. W / 2025 / 1302-06 / 407 / 16-JMP dated: 11.02.2025 and Tender opened on 04.03.2025. Approval of rates is in process. |
| 2 | Raising & Strengthening of Left Bank of Musharaf Drain from RD 61+500 to 102+300 | 256.940 | Estimate sanctioned vide No. W/2025/1307-11 /407/16-JMP dated: 11.02.2025 and Tender opened on 04.03.2025. Approval of rates is in process. |
| 3 | Rehabilitation of New Manka Drain from RD 126 to 137+000 | 55.233 | Estimate cleared by NESPAK and under sanction. |
| 4 | Restoration of FCC 186+000 from Head to Tail | 180.329 | Estimate cleared by NESPAK and under sanction. |
| 5 | Plugging Breaches and bank strengthening of Qutab Drain from RD 0 to 290+000 | 31.820 | Estimate cleared by NESPAK and under sanction. |
| 6 | Plugging Priority Breaches of FCCs 145+000, 166+000 and 177+000 occurred during flood 2024 | 113.500 | Estimate cleared by NESPAK and under sanction. |
| 7 | Restoration of Rojhan and Umar Kot Flood Bund | 16.000 | Estimate cleared by NESPAK and under sanction. |
| 8 | Rehabilitation/ Restoration of Darkhast Drain along with Protection Bund on right side from RD 0+000 to 19+500 | 34.613 | Estimate cleared by NESPAK and under sanction. |
| 9 | De-silting & Strengthening of Weak Banks of Channels on Kaha & Chachar Hill Torrent | 65.000 | Estimate cleared by NESPAK and under sanction. |
| 10 | Restoration & Bed Clearance of Tayyab Drain and FCC 79+000 | 17.481 | Estimate cleared by NESPAK and under sanction. |
| 11 | Restoration of Lundi Saidan and Tibbi Lundan Flood Bund | 25.000 | Estimate cleared by NESPAK and under sanction. |
| 12 | Restoration & Bed Clearance of FCC 95+000 and 109+000 | 10.215 | Estimate cleared by NESPAK and under sanction. |
|  | Total | 1449.92 |  |

It is pertinent to mention here that the damages of Kachhi Canal have not yet been restored by WAPDA since 2022. Resultantly, the ponded water is presently standing in the area / abadies such as Nizam Abad, Kanwala, Mouza Gianmal, Safdarabad, Arbi Tibba, Basti Jatt and many other scattered populations of Tehsil Rajanpur and Rojhan. The flood water also damaged the cultivated lands of these mouzas and it may cause further diseases in case this stagnant water will not be drained off. From Arbi Tibba Road to Shahwali, there is no any infrastructure to drain off the water safely in to river. Standing water is damaging the slopes of Mushraf Flood Bund. Hence, a work namely, “Disposal of torrential water from Asni Lake & Mushraf Flood Bund to River Indus through Mushraf Drain” amounting to Rs.116.130 Million have been awarded to Machinery Circle, Multan to safely dispose the ponded water From Uzman road to Indus River.

B-Indus River.

The issue of Hairo flood bund is aggravated since flood 2022 & 2023 as main creek of river drifted towards the bund and its training works. As discussed above in Chapter No.3, the matter was taken to Departmental Expert Committee forum various times but proposal was not implemented due to non-availability of funds and rapid change of river course. Lately, working paper to DEC was submitted which was discussed on 24.06.2024 due to direct River action on nose of stud RD. 25+000 and passing of flow with extremely high velocity in a narrow creek in front of stud RD. 25+000.

The issue was again aggravated on 25.06.2024 and matter was reported to high ups to take remedial measures against erosive action of Indus River on damaged part of stud RD.25+000. The Zonal Technical Committee was constituted who inspected the site on 27.06.2024 and recommended initiation of work on emergent basis to save the damaged Spur RD. 25+000.

Similarly, in pursuance to Administrative Department, Lahore canal wire No. 304, dated 03.07.2024, the team of experts inspected the critical site of Hairo Flood Bund on 04.07.2024. After visiting the problematic / vulnerable site, the team of experts recommended that existing part of damaged stud at RD. 25+000 of Hairo flood bund should be hold to its present position expeditiously as per recommendation of DEC dated 24.06.2024 to avoid any untoward situation.

In light of the decision of Zonal Technical Committee as well as member of experts, tentative cost of work was worked out to be Rs. 175.00 Million which could vary due to river dynamic condition. Keeping in view the sensitivity of matter, the case was initiated under Para 2.89 of PWD Code & Clause 59 (d) (iii) of PPRA Rules because no funds were available with the Division to take up the work. The Administrative Department accorded formal approval of emergent work up to maximum Rs.175.00 Million vide No. SO(FLOODS) VI-11/2008(DGK) Dated:18.07.2024. The work was awarded to M/s Sparco Construction Company after negotiated tendering under clause 59 (d) (iii) of PPRA Rules 2014 vide Executive Engineer, Jampur Construction Division, Rajanpur letter No. 535-38 / AB dated: 19.07.2024. The contractor mobilized at site and the work was started on 15.07.2024.The work remained continued upto 10.09.2024 under the resident supervision of M/s NESPAK Consultants (M&R) D.G.Khan as detailed below:-

| Sr. No. | Name of Work | Estimated Cost (M) | Remarks |
| --- | --- | --- | --- |
| 1 | Emergent remedial measures to check the severe erosive action of River Indus at RD.25+000 Hairo Flood Bund. (under 2.89 of PWD Code) | 128.031 | The works got completed physically 100% in the supervision of NESPAK (M&R) Consultants. |

After completion of above narrated work, the River Indus again suddenly changed morphology and washed the upstream apron of stud RD. 20+250 from Line No.400 to 982. The critical situation was intimated to high up vide Executive Engineer, Jampur Construction Division, Rajanpur vide C.W. No. 318 dated: 25.08.2024. After inspection the site, Superintending Engineer, Project Circle (Irrigation) D.G.Khan and Chief Engineer, Irrigation D.G.Khan Zone, D.G.Khan directed to immediately restore the launched apron vide their C.W. No. 2500 dated: 25.08.2024 and C.W. No.2503 dated: 25.08.2024 respectively. As per direction, Resident Engineer, NESPAK (M&R) D.G.Khan was requested to immediately observe the baseline data vide C.W.No. 320 dated: 25.08.2024. After observing the base line data jointly with NESPAK, the work was started and with hectic efforts, the structure was saved. The detail is as under:-

| Sr. No. | Name of Work | Estimated Cost (M) | Remarks |
| --- | --- | --- | --- |
| 1 | Emergent restoration of Apron of stud at RD. 20+250 of Hairo Flood Bund. (under clause 59 (d) (iii). | 124.645 | The works got completed physically 100% in the supervision of NESPAK (M&R) Consultants. |

Chapter- 7.

FLOOD FIGHTING / WATCHING ARRANGEMENT

7.1	PRE-FLOOD ARRANGEMENTS

About 200 hill torrents of which 13 are major originate from Suleiman range and flow through D.G Khan and Rajanpur Districts towards the River Indus. The Catchments area of these hill torrents is 11881 Sq Miles. The area extends over a length of 360 Km (200 miles) from Ramak to Kashmore, while in width it varies from 25 Km to 40 Km. Kaha is the major hill torrents originating from Suleman Range with watershed area of 2207 Sq miles. Flood water of Kaha Hill Torrent, after passing through Pachad area and dams up along right bank of Kachhi Canal. After crossing Kachhi Canal and Dajal Branch this flood water enters in Canal Irrigated and properly developed area on eastern side of Dajal Branch. To protect these valuable assets, an ADP Scheme, Constructing Flood Carrying Channel Hill Torrent Crossing at RD. 123+000 of Dajal Branch,” has approved and work is in progress (Physical Progress 73%) by the Govt. Contractor in the supervision of NESPAK ADP Consultants.

Catchments area of Chachar Hill Torrent is 274 Sq miles and its 25 years return period discharge is about 81430 Cs and maximum discharge received during torrential flood 2022 is 75900 Cs. Flood water of this torrent without management is almost damaging the crops, infrastructure, villages like Drigri, Mad Babra, Mad Meh, Mad Machi and Bakharpur Kachi Canal, Dajal Branch and canal command area. The rough cost estimation for the purpose of PC-I, “Management of Chachar Hill Torrent Flood Water                                                in District Rajanpur” amounting to Rs.4101.852 Million has been submitted to Administrative Department, prepared on the basis of typical drawings which have already been tested and proven successful in torrential flood mitigation on Chachar hill torrent as well. However, each dispersion structures and off-takes will be designed based upon fresh geological position / actual site condition and hydrology of the area. The approval is still awaited.

Due to flood damages 2022, the management of Kaha, Kala and Bagga Khosra hill torrent for the benefits of Pachad area completely became non-functional. The following PC-I has been got cleared by NESPAK Consultants and arrangement of funds has been intimated to Administrative Department.

| Sr. No. | ADP No. | Name of Scheme. | Estimated Cost | Allocation 2024-25 | Tehsil & District | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2421 | Restoration / rehabilitation of damaged structures on Kaha, Kala Khosra & Bagga hill torrent. | 1859.666 | 150.00 | Jampur / Rajanpur | PC-I amounting to Rs. 1859.66 M is with Administrative Department vide C.E. office letter No. W/2024/15434-35 /668/24 dated:31.10.2024. Pre PDWP Meeting is awaited. |

After subsiding flood 2024, it was noticed that river Indus drifted towards right and existing infrastructure are under threat. The river Indus targeted the Hairo Flood Bund in between RD.15+000 to RD.26+000. The Physical model study of Hairo Flood Bund about 10 miles was constructed in Irrigation Research Institute, Nandipur based on river survey 2023. The fresh / updated river survey 2024 given by field formation was also incorporated in the physical model.  During based test for simulating the model the Executive Engineer, Jampur Construction Division, Rajnpur along with Superintending Engineer, Project Circle (Irrigation) D.G Khan also visited the HRS Nandipur to check the performance of the different interventions on the physical model study. After detail experimentations and deliberations with Concerned Engineers, the following proposals seem to be suitable for the execution at site:-

- Inverted Hockey Spur tied with the existing Zimidara Flood Bund.

- J-Head spur at RD. 23+000 tied with new Hairo Flood Bund.

After physical Model Study the above proposals was submitted to Departmental Expert Committee (DEC) for its recommendation. The Departmental Expert Committee(DEC)decision in its meeting held on 08.10.2024 that “ The Departmental Expert Committee members unanimously agreed with the proposals i.e Inverted Hockey Spur tied with the existing Zimidara Flood Bund & -Head spur at RD. 23+000 tied with new Hairo Flood Bund as tested on the physical model study”.

Model study report and design parameters / geometry of proposed structures has been furnished by Director / PRO (Hydraulics), Irrigation Research Zone, Lahore & delivered on 26.11.2024. Fresh site survey for proposed “Inverted Hockey Spur at RD.16+000 tied with Zimidara Flood Bund & J-Head Spur at RD.23+000 tied with new Hairo Flood bund” has been carried out by field formation based on geometrical recommendations in model study as well as Departmental Expert Committee (DEC). The status of the scheme is as under:-

| Sr. No. | ADP No. | Name of Scheme. | Estimated Cost | Allocation 2024-25 | Tehsil & District | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2420 | Checking rightward drifting of Indus River at Hairo Flood Bund RD. 20+000 to RD. 25+000 (Mouza Hairo) | 771.745 | 150.00 | Jampur / Rajanpur | Scheme has been cleared in DDSC meeting held on 04.03.2025 and Administratively Approved vide No.SO(Floods)VI-01/2025 dated; 07.03.2025. |

Moreover, Camp sites have been identified. The flood fighting material has been checked. Request for funds to replenishing the flood fighting material as well as procurement of reserve stock stone (detailed below) has been placed to the Administrative Department.

Structure Wise Detail of Reserve Stock Stone

| Sr. No. | Name of Structure | Reserve Stock Limit (L.Cft) | Minimum Requirement 70 % (L.Cft) | Quantity Available (L.Cft) | Balance Qty Required (L.Cft) | Tentative Cost of Stone (Rs.M) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | J-Head Spur Jakhar Imam Shah | 2.500 | 1.750 | 1.660 | 0.090 | 1.127 |
| 2 | J-Head Spur U/S Basti Rindan | 2.500 | 1.750 | 1.720 | 0.030 | 0.370 |
| 3 | J-Head Spur at Stud No. 4 at RD: 10 Banuhan Dy. | 1.500 | 1.050 | 0.916 | 0.134 | 1.606 |
| 4 | Qalandar Shah Groyne. | 0.500 | 0.350 | 0.166 | 0.184 | 2.483 |
| 5 | J-Head Spur R-63 | 1.000 | 0.700 | 0.917 | 0.000 | 0.000 |
| 6 | J-Head Spur R-64 | 1.000 | 0.700 | 0.497 | 0.203 (Not Required) | 0.000 |
| 7 | J-Head Spur Rojhan | 1.450 | 1.020 | 1.261 | 0.000 | 0.000 |
| 8 | 3 No. Studs Banuhan | 0.600 | 0.420 | 0.000 | 0.420 | 5.036 |
| 9 | Shah Jamal Groyne | 0.500 | 0.350 | 0.000 | 0.350 | 4.970 |
| 10 | Fakhar Flood bund | 1.500 | 1.050 | 1.500 | 0.000 | 0.000 |
| 11 | Stud No.1,2&3 of Harrow Flood bund | 1.500 | 1.050 | 0.000 | 1.050 | 14.445 |
|  | Total | 14.550 | 10.185 | 8.637 | 2.258 | 30.037 |

a.	Flood Warning: It is important that as soon as the Officer In-charge of Taunsa Barrage knows about any probable rise in the River, he will initiate the necessary watch over embankment and regulate the canal according to the rules sanctioned for each head works.

b.	During High Flood, when the River over flows the banks, expensive area along the River banks is liable to be inundated. It is therefore, necessary to warn civil officer to be on the alert and to be ready to evacuate villages likely to be inundated.

c.	For the reasons stated in paragraph above, flood warning are to be sent by higher officer to the Civil Administration & other Irrigation officers. Flood warning will be issued by telegraph office to concerned officers.

d.	Between the dated October 01 and March 31 warning should be issued immediately when a River rises to “Low” flood level. The Executive Engineer Muzaffargarh Canal Division and Executive Engineer Dera Ghazi Khan will be intimated whenever the gauge at Attock is 10’ ft or above.

e.	For the period April 01 to September 30 warning should be issued only when a river reaches the “Medium” stage. No warning is issued for low flood during flood season.

SPECIAL FLOOD WARNING TO THE CHIEF ENGINEER

In addition to issue of warning as described above, when the river rises to the high flood stage, warnings by postal / wireless will also be issued to Chief Engineer from Tarbella Dam, Chashma Barrage, Kala Bagh, Mangla, Marala, Trimmu and Taunsa Barrage.

7.2. WATCHING ESTABLISHMENT

River Training Works

During different stages of flood limits, sufficient strength of labour shall remain throughout the period of flood in vulnerable reaches on the following basis.

| Site | Flood Limits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low (One man per 3 mile) |  | Medium (Three man per mile) |  | High (Four man per mile) |  | Very High (Five man per mile Av |  | Excep: high (Six man per mile Av |  |
|  | Men | Shift | Men | Shift | Men | Shift | Men | Shift | Men | Shift |
| Jampur Flood Bund | 17 | 2 | 34 | 2 | 136 | 3 | 170 | 3 | 204 | 3 |
| Harrow Flood Bund | 3 | 2 | 3 | 2 | 12 | 3 | 15 | 3 | 18 | 3 |
| Tayyab Drain | 13 | 2 | 26 | 2 | 104 | 3 | 130 | 3 | 156 | 3 |
| Qutab Drain | 29 | 2 | 58 | 2 | 232 | 3 | 290 | 3 | 348 | 3 |
| Rojhan Flood Bund | 6 | 2 | 13 | 2 | 52 | 3 | 65 | 3 | 78 | 3 |
| Umer Kot Flood Bund | 4 | 2 | 9 | 2 | 36 | 3 | 45 | 3 | 54 | 3 |
| Fakhar Flood Bund | 1 | 2 | 3 | 2 | 12 | 3 | 15 | 3 | 18 | 3 |
| 03 No. J-Head Spur & 03 No. Stone Studs | 4 | 2 | 8 | 2 | 32 | 3 | 40 | 3 | 48 | 3 |
| Qalinder Shah Gryne | 1 | 2 | 1 | 2 | 4 | 3 | 5 | 3 | 6 | 3 |
| Total:- | 78 |  | 155 |  | 620 |  | 775 |  | 930 |  |

Note: -

Payment for work charge establishment will be subject to verification by the TPM / Consultants.

For reaches other than vulnerable reaches, the watching establishment shall be employed as under to maintain and up keep the banks to become motor able.

- During normal flood stage, one man per mile / shift for two shifts in a day.

- During low flood stage, the scale of establishment is one men / mile / shift for two shifts in a day.

- In medium floods the scale will be three men / mile / shift for three shifts in a day.

- During high floods, four men / mile / shift for three shifts per day will be employed.

- During very high flood stage of 6.5 Lac Cs; discharge is above 8.00 Lac the scale fixed for above will be doubled, the strength shall be as below:-

- For 1’–2’depth of water along bunds/spurs three men/mile/shift in three shift of 8 hours each.

- For 2’–3’ depth of water along bunds/spurs four men/mile/shift in three shift of 8 hours each.

- For 3’–4’ depth of water along bunds/spurs five men/mile/shift in three shift of 8 hours each.

- For 4’–5’ depth of water along bunds/spurs seven men/mile per shift in three shift of 8 hours each.

- For 6’–7’ depth of water along bunds/spurs eight men/mile/shift in three shift of 8 hours each.

- For 7’–8’ depth of water along bunds/spurs ten men/mile/shift in three shift of 8 hours each.

- For 8’–9’ depth of water along bunds/spurs twelve men/mile/shift in three shift of 8 hours each.

- For 9’–10’depth of water along bunds/spurs fifteen men/mile/shift in three shift of 8 hours each.

Note: -

After every three miles there will be a Mate and for every five miles there will be a Mistry in addition to Mate to supervise the work of Beldars.

EMERGENCY GANGS

At important or vulnerable sites, emergency gangs in strength of 20 Beldars with one or two Mates will be maintained when the River rises above the prescribed gauge till it finally drops below that gauge. This gang shall be usefully employed on bund and will shift to portions where leaks slip or wave-wash is giving trouble. In addition to the above, the Executive Engineer can employ additional labour as and when required in emergency under intimation to the higher office.

7.3. Arrangement at Sensitive Sites.

- Temporary watching huts will be installed at sensitive sites during flood season. The normal strength for watching along flood bunds and site of relief cuts during floods / rains (quoted above) will be made double during high floods / rain.

- The establishment can be increased after consultation with Executive Engineer / Superintending Engineer in emergency and in accordance with the site requirements.

- The establishment employed will carry out minor repairs to bund / drains banks top, side slopes, removal of weed growth, opening / filling rat holes, rain gharas etc.

7.4 Watching Material.

Flood Watching Material

Name of Sub Division				     Mechanical Sub Division

| Sr. No. | Description | Jampur Flood Bund 0- 71000 & Harrow Flood Bund RD 0-29500 | Jakhar Imam Gandha, Ghafoor Groyne, Qallandar Shah Groyne, Shah Jamal Groyne, Old Qallandar Shah Groyne | Total |
| --- | --- | --- | --- | --- |
| 1. | Length bunds & Groynes | 14.20 Mile 5.90 Mile | 9.13 Mile | 23.45 Mile |
| 2. | Temporary Head Quarter | RD 7+500 | RD: 36+000 |  |
| 3. | Sub Divisional Officer | Mechanical | Mechanical |  |
| 4. | Sub Engineer Section | Jampur Sec: |  |  |
| 5. | Regular Seasonal Beldar | 5 | --- | 5 |
| 7. | Mate/Mistri | 1 | 1 | 2 |
| 8. | Gauge Reader | 2 | --- | 2 |
| 9. | Addt: watching Mate/ Beldar | 49 | 36 | 85 |

Flood Fighting Material Required

| Sr. No. | Items | Unit | Jampur Flood Bund 0- 71000 & Harrow Flood Bund 0-29500 | Jakhar Imam Gandha, Ghafoor Groyne, Qallandar Shah Groyne, Shah Jamal Groyne, Old Qallandar Shah Groyne | Total Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Required Quantity |  |  |  |  |
| 1. | Search Lights | No. | 3 | 2 | 5 | 0 | 5 |
| 2. | Trangers / wire crate | No. | 75 |  | 100 | 0 | 100 |
| 3. | Gunny Bags | No. | 1000 | 1000 | 2000 | 0 | 2000 |
| 4. | Axes | No. | 10 | 10 | 20 | 1 | 19 |
| 5. | Kassies with handle | No. | 20 | 20 | 40 | 2 | 38 |
| 6. | Hammers 10 Lbs | No. | 10 | 10 | 20 | 0 | 20 |
| 7. | Bucket 12 Inch Size | No. | 50 | 30 | 80 | 2 | 78 |
| 8. | Pegs 6-8 Ft Long | No. | 1400 | 850 | 2250 | 150 | 2100 |
| 9. | Basket Toot | No. | 50 | 30 | 80 | 5 | 75 |
| 10. | Sutlli | Kg | 5 | 5 | 10 | 0 | 10 |
| 11. | Air Cooler 12 Watt | No. | 3 | 2 | 5 | 0 | 5 |
| 12. | Water Cooler & W.Set | No. | 3 | 2 | 5 | 0 | 5 |
| 13. | Chargeable Torches | No. | 20 | 10 | 30 | 0 | 30 |
| 14. | Petrol & M.oil | Ltr | 400 | 400 | 800 | 0 | 800 |
| 15. | Generator Engine | No. | 1 | 1 | 2 | 1 | 1 |
| 16. | Electric Wire | Mtr | 40 | 100 | 140 | 0 | 140 |
| 17. | Electric Blub | Dzn | 3 | 3 | 6 | 0 | 6 |
| 18. | Switch | No. | 20 | 30 | 50 | 0 | 50 |
| 19. | Boards | No. | 5 | 10 | 15 | 0 | 15 |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 | 0 | 1 |
| 21. | Bamboos | No. | 500 | 200 | 700 | 20 | 680 |
| 22. | Swiss cottage / Choldari | No. | 1 | 1 | 2 | 0 | 2 |
| 23. | Chair Set with Table | No. | 2 | 1 | 3 | 0 | 3 |

Note: The balance flood fighting material will be procured before 30.06.2025.

Name of Sub Division				Mechanical Sub Division

| Sr. No. | Description | Jampur Flood Bund 71000 - 170720 | Boly Wala, Musa & L-Groyne | Total |
| --- | --- | --- | --- | --- |
| 1. | Length bunds & Groynes | 19 Mile | 3.22 Mile | 22.22 Mile |
| 2. | Temporary Head Quarter | RD 73+500 | RD :10+2000 | --- |
| 3. | Sub Divisional Officer | Mechanical | Mechanical | --- |
| 4. | Sub Engineer Section | Kotla Mughlan Section | --- | -- |
| 5. | Regular seasonal Beldar | 7 No | --- | 7 No |
| 7. | Mate / Mistri | --- | --- | --- |
| 8. | Gauge Reader | 2 No | --- | 2 No |
| 9. | Ad: watching estb: Mate/Beldar | 75 No | 16 No | 91 No |

***

Flood Watching Material Required

| Sr. No. | Item | Unit | Jampur Flood Bund 71000 – 170720 | Boly Wala, Musa & L- Groyne | Total Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Required Quantity |  |  |  |  |
| 1. | Search Lights | No. | 1 | 1 | 2 | 0 | 2 |
| 2. | Trangers / wire crate | No. | 75 |  | 75 | 0 | 75 |
| 3. | Gunny Bags | No. | 500 | 500 | 1000 | 80 | 920 |
| 4. | Axes | No. | 10 | 5 | 15 | 0 | 15 |
| 5. | Kassies with handle | No. | 20 | 15 | 35 | 2 | 33 |
| 6. | Hammers 10 Lbs | No. | 20 | 10 | 30 | 1 | 29 |
| 7. | Bucket 12 Inch Size | No. | 50 | 20 | 70 | 0 | 70 |
| 8. | Pegs 6-8 Ft Long | No. | 1000 | 500 | 1500 | 0 | 1500 |
| 9. | Basket Toot | No. | 50 | 25 | 75 | 0 | 75 |
| 10. | Sutlli | Kg | 5 | 5 | 10 | 0 | 10 |
| 11. | Air Cooler 12 Watt | No. | 1 | 1 | 2 | 0 | 2 |
| 12. | Water Cooler & Water Set | No. | 1 | 1 | 2 | 0 | 2 |
| 13. | Chargeable Torches | No. | 30 | 10 | 40 | 0 | 40 |
| 14. | Petrol & M.oil | Ltr | 400 | 400 | 800 | 0 | 800 |
| 15. | Generator Engine | No. | 1 | 1 | 2 | 1 | 1 |
| 16. | Electric Wire | Mtr | 40 | 100 | 140 | 0 | 140 |
| 17. | Electric Blub | Dzn | 2 | 3 | 5 | 0 | 5 |
| 18. | Switch | No. | 20 | 30 | 50 | 0 | 50 |
| 19. | Boards | No. | 5 | 10 | 15 | 0 | 15 |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 | 0 | 1 |
| 21. | Bamboos | No. | 500 | 150 | 650 | 0 | 650 |
| 22. | Swiss cottage / Choldari | No. | 0 | 1 | 1 | 0 | 1 |
| 23. | Chair Set with Table | No. | 2 | 1 | 3 | 0 | 3 |

Note: The balance flood fighting material will be procured before 30.06.2025.

Name of Sub Division.				Hill Torrent II Sub Division

| Sr. No. | Description | Fakhar Flood Bund RD: 0- 25000 | Total |
| --- | --- | --- | --- |
| 1. | Length Bund | 25000 ft | 5.00 Mile |
| 2. | Temporary Head Quarter | Kot Mithan |  |
| 3. | Sub Divisional Officer | Hill Torrent II |  |
| 4. | Sub Engineer Section | Kotla Mughlan Section |  |
| 5. | Regular seasonal Beldar | 5 No | 5 No |
| 7. | Mate/Mistri | 1 | 1 |
| 8. | Gauge Reader | 1 No | 1 No |
| 9. | Additional Watching establishment Mate/ Beldar | 30 No | 30 No |

Flood Watching Material Required (Fakhar Flood Bund RD: 0- 25+000)

| Sr. No. | Item | Unit | Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 2 | 0 | 2 |
| 2. | Trangers / wire crate | No. | 75 | 0 | 75 |
| 3. | Gunny Bags | No. | 200 | 25 | 175 |
| 4. | Axes | No. | 20 | 1 | 19 |
| 5. | Kassies with handle | No. | 30 | 2 | 28 |
| 6. | Hammers 10 Lbs | No. | 5 | 0 | 5 |
| 7. | Bucket 12 Inch Size | No. | 20 | 2 | 18 |
| 8. | Pegs 6-8 Ft Long | No. | 300 | 35 | 265 |
| 9. | Basket Toot | No. | 150 | 0 | 150 |
| 10. | Sutlli | Kg | 2 | 0 | 2 |
| 11. | Air Cooler 12 Watt | No. | 2 | 0 | 2 |
| 12. | Water Cooler & Water Set | No. | 2 | 0 | 2 |
| 13. | Chargeable Torches | No. | 15 | 0 | 15 |
| 14. | Petrol & M.oil | Ltr | 100 | 0 | 100 |
| 15. | Generator Engine | No. | 1 | 0 | 1 |
| 16. | Electric Wire | Mtr | 2 | 0 | 2 |
| 17. | Electric Blub | Dzn | 100 | 0 | 100 |
| 18. | Switch | No. | 30 | 0 | 30 |
| 19. | Boards | No. | 20 | 0 | 20 |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 |
| 21. | Bamboos | No. | 200 | 6 | 194 |
| 22. | Swiss cottage / Choldari | No. | 1 | 0 | 1 |
| 23. | Chair Set with Table | No. | 2 | 0 | 2 |

Note: The balance flood fighting material will be procured before 30.06.2025.

Name of Sub Division.				Hill Torrent No. I Sub Division.

| Sr. No. | Description | Bukhari Drain. | Tayyab, Darkhast | Total |
| --- | --- | --- | --- | --- |
| 1. | Length Drain. | 17.80 mile | 34.74 mile | 51.60 mile |
| 2. | Temporary Head Quarter | RD. 75+000 | RD. 12+000 of Tayyab Drain |  |
| 3. | Sub Divisional Officer | Hill Torrent I |  |  |
| 4. | Sub Engineer Section | Mithawan | Mithawan I, Mithawan II, Mithawan III, Vidore, Sori Lund & Sakhi Sarwar |  |
| 5. | Regular Seasonal Beldar | ---- |  | --- |
| 7. | Mate/Mistri | 1 | 1 | 2 |
| 8. | Gauge Reader | 1 No | 2 No. | 3 No |
| 9. | Additional watching establishment Mate/ Beldar | 30 No | 30 No. | 60 No |

Flood Watching Material Required (Bukhari, Darkhast, Tayyab & Qutab Drain)

| Sr. No. | Item | Unit | Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 4 | 0 | 4 |
| 2. | Trangers / wire crate | No. | 50 | 20 | 30 |
| 3. | Gunny Bags | No. | 1000 | 0 | 1000 |
| 4. | Axes | No. | 30 | 0 | 30 |
| 5. | Kassies with handle | No. | 60 | 5 | 55 |
| 6. | Hammers 10 Lbs | No. | 10 | 0 | 10 |
| 7. | Bucket 12 Inch Size | No. | 100 | 0 | 100 |
| 8. | Pegs 6-8 Ft Long | No. | 100 | 12 | 88 |
| 9. | Basket Toot | No. | 300 | 12 | 288 |
| 10. | Sutlli | Kg | 5 | 0 | 5 |
| 11. | Air Cooler 12 Watt | No. | 5 | 0 | 5 |
| 12. | Water Cooler & Water Set | No. | 5 | 0 | 5 |
| 13. | Chargeable Torches | No. | 25 | 2 | 23 |
| 14. | Petrol & M.oil | Ltr | 100 | 0 | 100 |
| 15. | Generator Engine | No. | 1 | 1 | 0 |
| 16. | Electric Wire | Mtr | 2 | 0 | 2 |
| 17. | Electric Blub | Dzn | 100 | 15 | 85 |
| 18. | Switch | No. | 30 | 0 | 30 |
| 19. | Boards | No. | 20 | 0 | 20 |
| 20. | Water Proof Camps | No. | 3 | 0 | 3 |
| 21. | Bamboos | No. | 200 | 8 | 192 |
| 22. | Swiss cottage / Choldari | No. | 2 | 0 | 2 |
| 23. | Chair Set with Table | No. | 3 | 0 | 3 |

Note: The balance flood fighting material will be procured before 30.06.2025.

Name of Sub Division.				Hill Torrent # II Sub Division.

| Sr. No. | Description | Rojhan Flood Bund | J-Head Spur Rojhan | Total (Canal Mile) |
| --- | --- | --- | --- | --- |
| 1 | Length bund / Spur | 12.25 | 0.932 | 13.182 |
| 3 | Temporary Head Quarter | Rojhan | Rojhan |  |
| 4 | Sub Divisional Officer | Hill Torrent No-II | Hill Torrent No-II |  |
| 5 | Sub Engineer Section | Kaha - I | Kaha – I |  |
| 6 | Regular | --- | --- |  |
| 7 | Additional Seasonal Beldar | 1 No. | 2 No. | 3 No. |
| 8 | Mate/Mistri | 1 No. | 2 No. | 3 No. |
| 9 | Gauge Reader | 2 No. | 2 No. | 4 No. |
| 10 | Additional watching establishment Mate/ Beldar | 46 No. | 8 No. | 54 No. |
| 11 | Regular seasonal Beldar | --- | --- |  |

***

Flood Watching Material Required (Rojhan Flood Bund, J-Head Spur Rojhan)

| Sr. No. | Item | Unit | Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 2 | 0 | 2 |
| 2. | Trangers / wire crate | No. | 70 | 0 | 70 |
| 3. | Gunny Bags | No. | 1000 | 35 | 965 |
| 4. | Axes | No. | 30 | 2 | 28 |
| 5. | Kassies with handle | No. | 60 | 2 | 58 |
| 6. | Hammers 10 Lbs | No. | 10 | 0 | 10 |
| 7. | Bucket 12 Inch Size | No. | 20 | 0 | 20 |
| 8. | Pegs 6-8 Ft Long | No. | 60 | 10 | 50 |
| 9. | Basket Toot | No. | 200 | 0 | 200 |
| 10. | Sutlli | Kg | 5 | 0 | 5 |
| 11. | Air Cooler 12 Watt | No. | 2 | 0 | 2 |
| 12. | Water Cooler & Water Set | No. | 2 | 0 | 2 |
| 13. | Chargeable Torches | No. | 25 | 0 | 25 |
| 14. | Petrol & M.oil | Ltr | 100 | 0 | 100 |
| 15. | Generator Engine | No. | 1 | 0 | 1 |
| 16. | Electric Wire | Mtr | 2 | 1 | 1 |
| 17. | Electric Blub | Dzn | 100 | 0 | 100 |
| 18. | Switch | No. | 30 | 0 | 30 |
| 19. | Boards | No. | 20 | 0 | 20 |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 |
| 21. | Bamboos | No. | 200 | 7 | 193 |
| 22. | Swiss cottage / Choldari | No. | 1 | 0 | 1 |
| 23. | Chair Set with Table | No. | 2 | 0 | 2 |

Note: The balance flood fighting material will be procured before 30.06.2025.

Name of Sub Division.				Hill Torrent # II Sub Division.

| Sr No. | Description | J- Head Spur R-63 | J-Head Spur R-64 | Total (Canal Mile) |
| --- | --- | --- | --- | --- |
| 1 | Length of Spurs | 0.92 | 1.12 | 2.04 |
| 3 | Temporary Head Quarter | At Zero RD | At Zero RD |  |
| 4 | Sub Divisional Officer | Hill Torrent No-II | Hill Torrent No-II |  |
| 5 | Sub Engineer Section | Kaha - I | Kaha – I |  |
| 6 | Regular | -- | -- |  |
| 7 | Additional Seasonal Beldar | 1 No. | 1 No. | 2 No. |
| 8 | Mate/Mistri | 1/1 | 1/1 | 2/2 No. |
| 9 | Gauge Reader | 1 No. | 1 No. | 2 No. |
| 10 | Additional watching establishment Mate/ Beldar | 8 No. | 8 No. | 16 No. |
| 11 | Regular seasonal Beldar | --- | --- | -- |

Flood Fighting Material Required (J- Head Spur R-63,    J-Head Spur R-64)

| Sr. No. | Item | Unit | Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 2 | 0 | 2 |
| 2. | Trangers / wire crate | No. | 75 | 0 | 75 |
| 3. | Gunny Bags | No. | 200 | 12 | 188 |
| 4. | Axes | No. | 5 | 0 | 5 |
| 5. | Kassies with handle | No. | 10 | 2 | 8 |
| 6. | Hammers 10 Lbs | No. | 2 | 0 | 2 |
| 7. | Bucket 12 Inch Size | No. | 10 | 1 | 9 |
| 8. | Pegs 6-8 Ft Long | No. | 100 | 13 | 87 |
| 9. | Basket Toot | No. | 30 | 2 | 28 |
| 10. | Sutlli | Kg | 2 | 0 | 2 |
| 11. | Air Cooler 12 Watt | No. | 2 | 0 | 2 |
| 12. | Water Cooler & Water Set | No. | 2 | 0 | 2 |
| 13. | Chargeable Torches | No. | 10 | 0 | 10 |
| 14. | Petrol & M.oil | Ltr | 50 | 0 | 50 |
| 15. | Generator Engine | No. | 1 | 0 | 1 |
| 16. | Electric Wire | Mtr | 1 | 0 | 1 |
| 17. | Electric Blub | Dzn | 50 | 0 | 50 |
| 18. | Switch | No. | 10 | 0 | 10 |
| 19. | Boards | No. | 10 | 0 | 10 |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 |
| 21. | Bamboos | No. | 100 | 6 | 94 |
| 22. | Swiss cottage / Choldari | No. | 1 | 0 | 1 |
| 23. | Chair Set with Table | No. | 1 | 0 | 1 |

Note: The balance flood fighting material will be procured before 30.06.2025.

Design Data of Structures & Gandhas in Hill Torrent II, Sub Division

| Sr No. | Name of Structure | Length (Ft) | Stone pitching (Lac Cft) | Stone Spawl (Lac Cft) | Stone Apron (Lac Cft) | Stone in Wire Crate |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Sad Alif (Earthen) | 4000 | 0 | 0 | 0 | 5 |
| 2 | Muhammad Wah | 3000 | 0.75 | 0.75 | 2.61 | 2.67 |
| 3 | Hazoori Wah | 5000 | 0.80 | 0.80 | 1.78 | 2.41 |
| 4 | Khan Wah | 2000 | 1.35 | 1.35 | 3.05 | 1.74 |
| 5 | Upper Noor Wah | 8000 | 2.60 | 2.60 | 5.50 | 1.75 |
| 6 | Garmail wah | 25000 | 3 | 3 | 10.71 | 3.78 |
| 7 | Link Channel Wah | 18000 | 1.08 | 1.08 | 6.50 | 2.70 |
| 8 | Chatool X- Structure | 8000 | 2.50 | 2.50 | 7 | 2.50 |
| 9 | Soma wah | 6000 | 2 | 2 | 6.03 | 2.27 |
| 10 | Lashari wah | 7000 | 1.93 | 1.93 | 7.33 | 2.75 |
| 11 | Old Jindra wah | 1000 | 0.50 | 1.30 | 0.75 | 0.80 |
| 12 | New Jindra wah | 10000 | 1.93 | 1.93 | 3.23 | 3 |
| 13 | Bund Hamza Sultan | 28000 | 0 | 0 | 0 | 0 |
| 14 | Ganda Lashari | 5100 | 0 | 0 | 0 | 0 |
| 15 | Hajoo wah | 15000 | 1.20 | 1.20 | 3.70 | 1.55 |
| 16 | Dhora wah | 4000 | 1.07 | 1.07 | 3 | 0 |
| 17 | Noor Khona Wah | 4000 | 1.60 | 1.60 | 4 | 0.40 |
| 18 | But Pahore wah | 3000 | 0.80 | 0.50 | 1 | 0 |
| 19 | But wah & Gaman Wah | 2000 | 0 | 0 | 0 | 0 |
| 20 | Kala Khosra H/T | 12000 | 2.19 | 2.19 | 3.63 | 2.09 |
| 21 | Bagga Khosra H/T | 10000 | 5.74 | 5.74 | 4.59 | 9.46 |
| 22 | Gandha Rakhi | 1350.20 | 0.20 | 0.11 | 0.10 | 0 |
| 23 | Gandha Gaman | 1000 | 0.18 | 0.10 | 0.10 | 0 |
| 24 | Gandha Khooni | 1000 | 0.18 | 0.10 | 0.10 | 0 |
| 25 | Gandha Thul Harrow | 800 | 1.79 | 1.79 | 3.20 | 0 |
| 26 | Leghari Gandha Wah | 3000 | 1 | 0.70 | 1.30 | 0 |
| 27 | Chachar wah H/T | 1000 | 1.69 | 0.91 | 1.92 | 2.23 |
| 28 | Sori Shumali H/T | 1200 | 0.36 | 0.19 | 0.08 | 0 |
| 29 | Zangi H/T | 2000 | 0.40 | 0.21 | 0.10 | 0 |
| 30 | Soori Janubi H/T | 2000 | 0.60 | 0.32 | 0.12 | 0 |
| 31 | Muhammad pur Flood Bund | 16000 | 0 | 0 | 0 | 0 |
|  | Total | 209450 |  |  |  |  |

Name of Sub Division.				Hill Torrent # II Sub Division.

| Sr No. | Description | Sad Alif | 3 No. Salai at Muhammad Wah, Hazoori Wah & Khan Wah. | Upper Noor wah | Germail Wah. |
| --- | --- | --- | --- | --- | --- |
| 1 | Length of bunds | 4000 ft | 10000 ft | 8000 ft | 25000 ft |
| 2 | Temporary Head Quarter | Lal Garh |  |  |  |
| 3 | Watching establishment | -- | -- | -- | -- |
| 4 | Regular | -- | -- | -- | -- |
| 5 | Additional Beldar | 4 + 4 | 6 + 6 | 4 + 4 | 10 + 10 |
| 6 | Mate/Mistri | 1 + 1 | 1 + 1 | 1+1 | 1 + 1 |

Flood Watching Material Required (Sad Alif, 3 No. Salai at Muhammad Wah, Hazoori Wah & Khan Wah, Upper Noor wah, Germail Wah)

| Sr. No. | Item | Unit | Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 2 | 0 | 2 |
| 2. | Trangers / wire crate | No. | 100 | 23 | 77 |
| 3. | Gunny Bags | No. | 2000 | 0 | 2000 |
| 4. | Axes | No. | 20 | 1 | 19 |
| 5. | Kassies with handle | No. | 40 | 4 | 36 |
| 6. | Hammers 10 Lbs | No. | 10 | 1 | 9 |
| 7. | Bucket 12 Inch Size | No. | 20 | 4 | 16 |
| 8. | Pegs 6-8 Ft Long | No. | 1500 | 73 | 1427 |
| 9. | Basket Toot | No. | 100 | 3 | 97 |
| 10. | Sutlli | Kg | 10 | 0 | 10 |
| 11. | Air Cooler 12 Watt | No. | 2 | 0 | 2 |
| 12. | Water Cooler & Water Set | No. | 2 | 0 | 2 |
| 13. | Chargeable Torches | No. | 20 | 0 | 20 |
| 14. | Petrol & M.oil | Ltr | 200 | 0 | 200 |
| 15. | Generator Engine | No. | 2 | 0 | 2 |
| 16. | Electric Wire | Mtr | 2 | 0 | 2 |
| 17. | Electric Blub | Dzn | 300 | 0 | 300 |
| 18. | Switch | No. | 20 | 0 | 20 |
| 19. | Boards | No. | 20 | 0 | 20 |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 |
| 21. | Bamboos | No. | 1500 | 0 | 1500 |
| 22. | Swiss cottage / Choldari | No. | 1 | 0 | 1 |
| 23. | Chair Set with Table | No. | 2 | 0 | 2 |

Note: The balance flood fighting material will be procured before 30.06.2025.

Name of Sub Division.				Hill Torrent # II Sub Division.

| Sr. No. | Description | Link Channel Wah | Chatool X-Structure | Soma Wah | Lashari wah | Old+New Jindra Wah | Bund Hamza Sultan |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length of bunds | 18000 Ft | 8000 Ft | 6000 Ft | 7000 Ft | 11000 Ft | 28000 Ft |
| 2 | Temporary Head Quarter | Tibbi Solgi |  |  |  |  |  |
| 3 | Watching establishment | -- | -- | -- | -- | -- | -- |
| 4 | Regular | -- | -- | -- | -- | -- | -- |
| 5 | Additional Beldar | 8+8 | 5+5 | 5+5 | 6+6 | 6+6 | 5+5 |
| 6 | Mate/Mistri | 1+1 | 1+1 | 1+1 | 1+1 | 1+1 | 1+1 |

Flood Watching Material Required on Structures (Link Channel Wah, Chatool

X-Structure, Soma Wah, Lashari Wah, Old/ New Jindra Wah, Bund Hamza Sultan)

| Sr. No. | Items | Unit | Total Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1 | Search Lights | No. | 2 | 0 | 2 |
| 2 | Trangers / wire crate | No. | 50 | 0 | 50 |
| 3 | Gunny Bags | No. | 3500 | 0 | 3500 |
| 4 | Axes | No. | 60 | 0 | 60 |
| 5 | Kassies with handle | No. | 120 | 0 | 120 |
| 6 | Hammers 10 Lbs | No. | 30 | 0 | 30 |
| 7 | Bucket 12 Inch Size | No. | 100 | 0 | 100 |
| 8 | Pegs 6-8 Ft Long | No. | 6000 | 0 | 6000 |
| 9 | Basket Toot | No. | 550 | 0 | 550 |
| 10 | Sutlli | Kg | 140 | 0 | 140 |
| 11 | Air Cooler 12 Watt | No. | 2 | 0 | 2 |
| 12 | Water Cooler & Water Set | No. | 2 | 0 | 2 |
| 13 | Chargeable Torches | No. | 120 | 0 | 120 |
| 14 | Petrol & M.oil | Ltr | 2000 | 0 | 2000 |
| 15 | Generator Engine | No. | 3 | 0 | 3 |
| 16 | Electric Wire | Mtr | 150 | 0 | 150 |
| 17 | Electric Blub | Dzn | 14 | 0 | 14 |
| 18 | Switch | No. | 150 | 0 | 150 |
| 19 | Boards | No. | 38 | 0 | 38 |
| 20 | Water Proof Camps | No. | 1 | 0 | 1 |
| 21 | Bamboos | No. | 305 | 0 | 305 |
| 22 | Swiss cottage / Choldari | No. | 1 | 0 | 1 |
| 23 | Chair Set with Table | Each | 2 | 0 | 2 |

Note: The required flood fighting material will be procured before 30.06.2025.

Name of Sub Division			Hill Torrent # II Sub Division.

| Sr. No. | Description | Gandha Lashari | Hajoo Wah | Dhora wah | Noor Kohna | But Pahor + But & Gaman Wah | Kala Khosra |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Length of bunds | 5100 Ft | 15000 Ft | 4000 Ft | 4000 Ft | 3000 + 600 & 2000 Ft | 12000 Ft |
| 2 | Temporary Head Quarter | Dajal | Dajal | Dajal | Dajal | Dajal | Tibbi Lundan |
| 3 | watching establishment | -- | -- | -- | -- | -- | -- |
| 4 | Regular | -- | -- | -- | -- | -- | -- |
| 5 | Additional Beldar | 4+4 | 8+8 | 4+4 | 5+5 | 12+12 | 18+18 |
| 6 | Mate/Mistri | 1+1 | 1+1 | 1+1 | 1+1 | 1+3 | 6 |

Flood Watching Material Required (Gandha Lashari, Hajoo Wah, Dhora Wah, Noor Kohna, But Pahor + But & Gaman Wah, Kala Khosra)

| Sr. No. | Items | Unit | Total Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1 | Search Lights | No. | 2 | 0 | 2 |
| 2 | Trangers / wire crate | No. | 50 | 0 | 50 |
| 3 | Gunny Bags | No. | 3000 | 0 | 3000 |
| 4 | Axes | No. | 60 | 0 | 60 |
| 5 | Kassies with handle | No. | 120 | 0 | 120 |
| 6 | Hammers 10 Lbs | No. | 60 | 0 | 60 |
| 7 | Bucket 12 Inch Size | No. | 120 | 0 | 120 |
| 8 | Pegs 6-8 Ft Long | No. | 4500 | 0 | 4500 |
| 9 | Basket Toot | No. | 400 | 0 | 400 |
| 10 | Sutlli | Kg | 120 | 0 | 120 |
| 11 | Air Cooler 12 Watt | No. | 2 | 0 | 2 |
| 12 | Water Cooler & Water Set | No. | 2 | 0 | 2 |
| 13 | Chargeable Torches | No. | 120 | 0 | 120 |
| 14 | Petrol & M.oil | Ltr | 100 | 0 | 100 |
| 15 | Generator Engine | No. | 6 | 0 | 6 |
| 16 | Electric Wire | Mtr | 120 | 0 | 120 |
| 17 | Electric Blub | Dzn | 12 | 0 | 12 |
| 18 | Switch | No. | 60 | 0 | 60 |
| 19 | Boards | No. | 18 | 0 | 18 |
| 20 | Water Proof Camps | No. | 1 | 0 | 1 |
| 21 | Bamboos | No. | 600 | 0 | 600 |
| 22 | Swiss cottage / Choldari | No. | 1 | 0 | 1 |
| 23 | Chair Set with Table | Each | 2 | 0 | 2 |

Note: The required flood fighting material will be procured before 30.06.2025.

Name of Sub Division				Hill Torrent # II Sub Division

| Sr No. | Description | Bagga Khosra | Ganda Rakhi | Ganda Gaman | Ganda Khoni |
| --- | --- | --- | --- | --- | --- |
| 1 | Length of bunds | 10000 Ft | 1350 Ft | 1000 Ft | 1000 Ft |
| 2 | Temporary Head Quarter | Tibbi Lundan |  |  |  |
| 3 | Watching establishment | --- | ---- | --- | ---- |
| 4 | Regular Seasonal Beldar | 4 No. | 4 No. | 4 No. | 4 No. |
| 5 | Additional Beldar | 30+30 | 3+3 | 2+2 | 2+2 |
| 6 | Mate/Mistri | 3+3 | 1+1 | --- | --- |

Flood Watching Material Required

(Bagga Khosra, Ganda Rakhi, Ganda Gaman, Ganda Khoni)

| Sr. No. | Items | Unit | Total Required Quantity | Available Quantity | Balance |
| --- | --- | --- | --- | --- | --- |
| 1 | Search Lights | No. | 2 | 0 | 2 |
| 2 | Trangers / wire crate | No. | 50 | 0 | 50 |
| 3 | Gunny Bags | No. | 3000 | 0 | 3000 |
| 4 | Axes | No. | 40 | 0 | 40 |
| 5 | Kassies with handle | No. | 80 | 0 | 80 |
| 6 | Hammers 10 Lbs | No. | 20 | 0 | 20 |
| 7 | Bucket 12 Inch Size | No. | 40 | 0 | 40 |
| 8 | Pegs 6-8 Ft Long | No. | 5400 | 0 | 5400 |
| 9 | Basket Toot | No. | 400 | 0 | 400 |
| 10 | Sutlli | Kg | 100 | 0 | 100 |
| 11 | Air Cooler 12 Watt | No. | 2 | 0 | 2 |
| 12 | Water Cooler & Water Set | No. | 2 | 0 | 2 |
| 13 | Chargeable Torches | No. | 80 | 0 | 80 |
| 14 | Petrol & M.oil | Ltr | 300 | 0 | 300 |
| 15 | Generator Engine | No. | 2 | 0 | 2 |
| 16 | Electric Wire | Mtr | 80 | 0 | 80 |
| 17 | Electric Blub | Dzn | 4 | 0 | 4 |
| 18 | Switch | No. | 40 | 0 | 40 |
| 19 | Boards | No. | 10 | 0 | 10 |
| 20 | Water Proof Camps | No. | 1 | 0 | 1 |
| 21 | Bamboos | No. | 400 | 0 | 400 |
| 22 | Swiss cottage / Choldari | No. | 1 | 0 | 1 |
| 23 | Chair Set with Table | Each | 3 | 0 | 3 |

Note: The required flood fighting material will be procured before 30.06.2025.

7.5. Arrangement for Soundings & Probing.

Sub Divisional Officers, Sub Engineer concerned with establishment i.e. Beldars Mates, Mistries, Boat, Boatman and leveling instruments will remain present at their own site during flood season.

7.6. Lighting arrangement.

During high / exceptional high floods lighting arrangements at vulnerable reaches will be arranged by the department, provision for lighting articles has already been included in the flood fighting material detail as above in tabulated form.

7.7. Ration Arrangement.

Revenue Department & District Government will arrange the ration for flood affected persons.

7.8 P.O.L. Arrangement for Vehicles.

Provision of POL for vehicles has already been included in the estimate of running Govt. Jeeps.

7.9. Transportation.

Police, Army and Civil Authorities will arrange the transportation for shifting the flood affected persons to the safe places.

7.10. Law & Order.

Police officials deputed will maintain law and order situation.

7.11. Medical arrangement for labour.

The District Administration will provide medical facilities through concerned District Health officer.

7.12. Liaison with other Department.

Executive Engineer will remain fully contacted with all the Civil / Forces Departments to combat any mishap occurring due to floods and will also make liaison with IRSA authority, ministry of water and power as well as Flood Forecasting Centers Lahore for collection of flood related information.

7.13. Role of Army.

Pak Army will cooperate with the Department for protection measures against the forthcoming flood, during flood and after flood season i.e. provision and arrangement of explosive for breaching section and also help in the shifting of flood affected persons to the safe places.

7.14. Duties of Telephone Attendant

To collect the gauges / any kind of messages from field or any other sources and its proper delivery, the experienced officials will be deputed to perform their duties on telephone as telephone attendant during flood season w.e.f. 01.06.2025 to 15.10.2025 respectively as tabulated below:-

| Sr. No. | Name of Official | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Abdul Ghafoor | 15.06.2025 to 15.10.2025 | 05:00 pm | 01:00 am |
| 2 | Allah Bakhsh |  | 01:00 am | 09:00 am |
| 3 | All clerical staff of Division |  | 09:00 am | 05:00 pm |

7.15. Wireless Arrangement.

During flood season, wireless sets will be fixed in all the Govt. vehicles. All the officers / official will have their own mobile sets at sites.

Chapter -8.

- Detail of Encroachments

23 No. encroachments are present in Jampur Construction Division, Rajanpur on Flood Bunds. Notices have been delivered to the encroachers for removal of encroachment and also cases have been registered in the concerned Police Stations for removal of the same.

| Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- |
| 57 | 34 | 23 | List of encroachers has been submitted to Civil Administration Department and action is awaited. The Encroachments could not be removed due to stay orders from Civil / High Courts. Now the case is under prosecution. |

- Critical Encroachments

| Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- |
| There is no critical encroachment in Jampur Construction Division, Rajanpur. |  |  |  |

Chapter-9.

Duty Roaster / Flood Fighting Program

All the flood gauges from upper region are received through wireless sets already installed at various Police Stations in the District as well as though cell phones. To meet the latest flood stages and emergency very high flood, wireless sets / cell phones will be provided in the vehicles of Executive Engineer & Sub Divisional Officers besides at the following key points.

- Jampur Canal Rest House.

- Rajanpur Canal Rest House.

- Canal Telegraph Office, D.G. Khan

It is further added that, the establishment i.e. Beldars, Mates & Mistries and Canal Patwaries of Rajanpur Canal Division will also coordinate with Jampur Construction Division, Rajanpur. The field establishment of Kadra Sub Division of Rajanpur Canal Division, Rajanpur will also cooperate in watching the Rojhan flood Bund, Rojhan ring Bund, Spur R-63, Spur R-64 & J-Head Spur Rojhan. Similarly the establishment of Talai Sub Division of Rajanpur Canal Division would remain present at Syphon RD: 95+000 of DKC & Bukhari Drain for day & night watching. Rajanpur Canal Division will be the backup Division of Jampur Construction Division, Rajanpur during high floods.

DUTY ROASTER

| Hill Torrent-I Sub Division, D.G. Khan |  |  |
| --- | --- | --- |
| Drains-I Section. | Drains-II Section | Drains-III Section |
| New Manka RD. 0-92+000 and Muhammad Pur Drain. | New Manka RD. 92000 to 126+000. Tayyab Drain, Lashari Drain & Darkhast Drain. | Bukhari Drain |
| Sub Engineer concerned. | Sub Engineer concerned. | Sub Engineer concerned. |
| Note: Sub Divisional Officer, Hill Torrent-I will look after of Fakhar Flood Bund during flood current flood season in addition to his own duties. |  |  |
| Hill Torrent-II Sub Division, Rajanpur |  |  |
| Sori Shumali Section | Sori Janubi Section | Kaha-I Section |
| Umer Kot Flood Bund J-Head Spur R-63 | Rojhan Flood Bund J-Head Spur R-64&65 Rojhan Ring Bund | Muhammad Wah, Hazoori Wah, Khan Wah, Sad Alif, Upper Noor Wah, Germail Wah & Bagga Khosra Hill Torrent. |
| Sub Engineer concerned. | Sub Engineer concerned. | Sub Engineer concerned. |
| Kaha-II Section |  |  |
| Link Channel, Chatool X-Structure, Sona Wah, Leshari Wah, Old & New Jindra Wah & Bund Hamza Sultan, Hajoo Wah, Dhora Wah, Noor Kohna, Bagga Khosra Hill Torrent and Kala Khosra Hill Torrent. |  |  |
| Sub Engineer concerned. |  |  |
| Mechanical Sub Division, D.G. Khan |  |  |
| Jampur Section | Kotla Mughlan Section. |  |
| Jampur Flood Bund and system RD. 0-74+000 & Gandha Jakhar Imam. | Jampur Flood Bund and system RD. 74+000-170+720 Tail and Fakhar Flood Bund. |  |
| Sub Engineer concerned | Sub Engineer concerned |  |

Gauges of Hill Torrents sent by gauge readers appointed at Darra sites are received by the Sub Engineer Kaha & Chachar Section, who is responsible to inform well in time.

- Office the Chief Engineer, D.G. Khan Zone, D.G. Khan.

- Office of the Deputy Commissioner Office, Rajanpur.

- Office of the Assistant Commissioner Rajanpur, Jampur and Rojhan.

- Superintending Engineer, Derajat Canal Circle, D.G. Khan.

- Superintending Engineer, Project Circle (Irrigation), D.G. Khan.

- Executive Engineer, Canal Division Rajanpur.

- Executive Engineer, Jampur Construction Division, Rajanpur.

- Sub Divisional Officer, Kadra,Hajipur,Talai/Hill Torrent I,II & Mechanical Sub Div:.

Chapter-10.

Emergency Telephone Numbers

| Name of Officer | Designation | Office No. |  | Residence No. | Fax No | Official Mobile No. |
| --- | --- | --- | --- | --- | --- | --- |
| IRRIGATION DEPARTMENT |  |  |  |  |  |  |
| Wasif Khurshid | Secretary Irrigation |  | 042-99212117-18 | 042-99203037 | 04299212116 | 03040920033 |
|  | Chief Engineer, D&F Zone, Lahore |  | 042-99733551 | 042-99230602 | 04299230731 | 0303-7713303 |
| Zulfiqar Ali. | Chief Engineer, Irrigation DGK Zone |  | 064-9260245 | -- | 064-9260246 | 0313-9190684 |
| Zeeshan Ali | S.E., Project Circle D.G. Khan. |  | 064-9260248 | -- | 0649260426 | 0323-4443607 |
| Muhammad Munir Ahmad Abbasi | Superintending Engineer, Derajat Circle. |  | 064-9260247 | -- | -- | 0300-6920400 |
| Amir Taimor | XEN (OP) Irrigation Zone, D.G. Khan. |  | 064-9260251 | -- | 0429260252 | 0332-6206393 |
| Nouman Baig (Additional Charge) | Executive Engineer, D.G.Khan Canal Div: I |  | 064-9260249 | -- | -- | 0300-6306140 |
| Mohsin Shabir | Executive Engineer, D.G.Khan Canal Div: II |  | -- | -- | -- | 0311-7861909 |
| Mujahid Kalim | XEN, Dera Const; Division D.G. Khan |  | -- | -- | -- | 0304-2616750 |
| Muhammad Munawer Iqbal | XEN, Jampur Const: Division, Rajanpur |  | 0604-690690 | -- | -- | 0300-0425111 |
| Umer Abdul Rehman | XEN, Rajanpur Canal Division Rajanpur |  | 0604-689064 | -- | 064689064 | 0333-1871816 |
| Nouman Baig | XEN, Taunsa Barrage Division, Kot Adu. |  | 066-2242796 | -- | 0662240296 | 0300-6306140 |
| -- | Flood Emergency Cell, Lahore |  | 042-99212116 | 042-5900049 | 0429212132 | 042-99212166 |
| -- | Flood Emergency Cell, Lahore |  | 042-7572091-93 | -- | 0427572092 | 042-99212166 |
|  | Director Flood, Lahore |  | 042-99231614 | 042-6653943 | -- | 0323-4864378 |
| Abid Hussain | Telegraph Office, DGK |  | 064-9260253 | -- | -- | 0336-6615223 |
| Muhammad Nasrullah | SDO-Mechanical |  | -- | -- | -- | 0333-3625555 |
| Muhammad Abdul Mueed | SDO-HT-I |  |  | -- | -- | 0335-0502269 |
| Qazi Faiz-ul-Hassan | SDO-HT-II |  |  | -- | -- | 0333-6431973 |
| Abdul Ghafoor Allah Bakhsh | Signaler |  | 0604-690690 | -- | -- | 0333-6439136 0333-6097657 |
| CIVIL ADMINISTRATION |  |  |  |  |  |  |
|  | D.C.Rajanpur | 0604-920001 |  | -- | 0604-689131 | 0304-0920093 |
| Mhr.Sohail Mumtaz | ADCR Rajanpur | 0604-689495 |  | -- | 0604-920008 | 0300-5501673 |
| Farooq Ahmad | D.P.O. Rajanpur | 0604-689055 |  | -- | 0604-920055 | 0300-9974002 |
| Sifat Ullah | ADCG Rajanpur | 0604-689288 |  | -- | 0604-00008 | 0333-8595003 |
| Jamil Ahmad | A.C.RJP | 0604-689904 |  | -- | 0604-920020 | 0344-2109029 |
|  | A.C.Jampur | 0604-567509 |  | -- | -- | 0324-4229996 |
| Aftab Nawaz KHichi | A.C.Rojhan | 0604-610001 |  | -- | -- | 0300-6342620 |
| Nadeem Bari | C.O.Rajanpur | 0604-689320 |  | -- | 0604-920032 | 0305-7860339 |
|  | SDPO Rajanpur H/Q | 0604-689033 |  | -- | 0604-920129 | 0300-6669798 |
| ARMY AUTHORITY |  |  |  |  |  |  |
| Military Officer Head Quarter Engineers 2 Corps Multan Cantt. |  |  |  | -- | -- | 061/5313388 |
| Military Officer, Head Quarter Engineers 31-Corps Bahawalpur Cantt. |  |  |  | -- | -- | 062-89132511 |

Chapter-11.

STANDARD OPERATING PROCEDURERES (SOP)

FOR BREACHING SECTION

11.1. History of Breaching Section.

There is no breaching section in Jampur Construction Division, Rajanpur.

11.2. Location, design, quantity and verity of explosive required for detonation.

Not related.

11.3. Arrangement of explosive.

Not related.

11.4. List of the security staff along-with their training.

Not related.

11.5. Detail of Mechanical means as a stand by arrangements in case of detonation

Failure.

Not related.

11.6. Duty roaster in case of the critical situation.

Not related.

11.7. Breaching / Relief Cut committee With Their Action Plan.

Not related.

11.8. List of villages likely to be inundated in case of breach.

Not related.

11.9. Announcement and detail of evacuation arrangements.

Not related.

11.10. Detail of coordination with Civil / Army Authorities.

Not related.

11.11. Parallel Communication Arrangements.

Not related.

11.12. Index Plan.

As there is no breaching section so index plan is not required.

List of places / POINTS for Relief cuts in case of                extreme flooding

| Sr. No. | NAME OF CHANNEL | RD. | SIDE | LOCATION |
| --- | --- | --- | --- | --- |
| 1 | FCC-79 Tayyab Drain | 112+000 | R.S & L.S | Dhandla Road |
|  |  | 66+000 | R.S | Ghat Stop |
|  |  | 42+000 | R.S | Allah Abad |
| 2 | FCC-79 Tayyab Drain | 33+000 | L.S | Dajal Road D/s |
|  |  | 4+000 | L.S | Basti Ahamdani |
|  |  | 00+000 | L.S | Ghat Stop |
| 3 | Qutab Drain | 283+000 | R.S | D/s Pull Pathan |
| 4 | FCC-177 | 7+000-8+000 | L.S |  |
| 5 | FCC-145 | 39+000 | R.S |  |
| 6 | FCC-186 | 22+000 | L.S |  |
| 7 | Bukhari Drain | 54+000-55+000 | L.S | U/s Aqil Pur Road |
| 8 | Right Marginal Bund | 14+000-15+000 |  |  |
| 9 | Fakhar Flood Bund | 10+000 |  |  |
| 10 | Rojhan Flood Bund | 1+000-2+000 |  |  |

Executive Engineer

Jampur Construction Division

Jampur

|  | Superintending Engineer Project Circle (Irrigation) Dera Ghazi Khan |
| --- | --- |

Chief Engineer

D.G. Khan, Irrigation Zone

Dera Ghazi Khan

PART-B

Chapter.No.12.

Vulnerable sites on flood bund / structures.

12.1	Apprehended breaches in flood bunds / structures.

1. 	Jampur Flood Bund.

- RD: 7+500 - 18+000

- RD: 107+000 - 113+000

- RD: 139+000 - 171+000

- Hairo Flood Bund RD. 20+000 – 25+000

2. 	Fakhar Flood Bund.

- RD: 10+000 - 24+000

3. 	Rojhan Flood Bund.

- R.D: 10+000 - 25+000

12.2	Operation of Breaching Sections.

There is no breaching section exists in Jampur Construction Division, Rajanpur.

12.3	Breaches due to rising of flood water, deterioration of flood bund etc.

Nil

Chapter No.13.

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITES

Vulnerable Sites

A. 	Jampur Flood Bund.

- RD: 7+500 - 18+000

- RD: 107+000 - 113+000

- RD: 139+000 - 171+000

- Hairo Flood Bund RD. 20+000 – 25+000

13.1. 	Plan showing route of flood water coming out of the breaches supported

with level.

Marked index plan attached.

13.2. 	Detail of villages/ abadies likely to be affected and this also be shown

on the plan.

i-	Jampur Flood Bund	RD: 7+500 - 18+000

Village	:	Basti Wahid Jhaker, Basti Ditta Wali, Basti Calar Coat, Basti Sheru

Abadi:		About 11500 People

If any mishap occurs on this vulnerable reach on Jampur Flood Bund, the water will inundate the above village’s agriculture land and adjoining abadies and other infrastructures.

ii.	Jampur Flood Bund	RD: 107+000 - 113+000

Villages: 	Basti Kotla Bodla   Kotla Mughlan, Basti Thathari, Basti Sheer

Abadi:		About 157000 People

If any mishap occurs on this vulnerable reach on Jampur Flood Bund, the water will inundate the above village’s agriculture land and adjoining abadies and other infrastructures.

iii.	Jampur Flood Bund	RD: 139+000 - 171+000

Villages:	Basti Ahmadani, Basti Bule Wala, Hadi Wala, Reikh Bagh

Abadi: 		About 8000 People

If any mishap occurs on this vulnerable reach on Jampur Flood Bund, the water will inundate the above village’s agriculture land and adjoining abadies and other infrastructures.

iv.	Hairo Flood Bund   RD: 20+000 - 25+000

Villages:	Mouza Hairo and allied abadies

Abadi: 		About 9000 People

If any mishap occurs on this vulnerable reach on Hairo Flood Bund the water will inundate the above village’s agriculture land and adjoining abadies and other infrastructures.

13.3. Strategy and action taken be explained in detail.

By 1st of June, the strength of the existing establishment is doubled to clear the jungle on slopes and toe of the Embankments shanks etc. of the Spurs. Emphasis is laid to locate the holes of burrowing animals. These if found / detected or to be puddle to ensure the safety of the embankments.

In emergency the flood information is received from upper region, the Sub Divisional Officer and Sub Engineer Incharge shift on their head Quarter at site. The establishment is deployed according to flood stage and as prescribed, round the clock vigilant watching is arranged along the entire embankment to take care of eventualities. The staff is equipped with, torches, Gunny bags, kassies and any other relevant material to control seepage/ leakage in the embankment. The information is passed on quickly to the concerned quarter and simultaneously action is taken to close the same. In the event of exceptionally high flood stage the heavy earth moving machinery (Bull-Dozers) of agriculture engineering department / machinery circles also kept ready and placed on alert at vulnerable sites of Jampur Flood Bund RD: 26-54. For the safety of the public/ private infrastructures, land and abadies of District Rajanpur Tehsil Jampur, Kot Mithan Town and Rojhan Town & in emergency relief cuts ore made Gaddan Flood Bund; Musa Flood Bund erected which is the 2nd defence line of Jampur Flood Bund.

13.3.1. Arrangements.

The flood fighting material along-with watching establishment will be arranged at site as warning of high flood issued from Chasma Barrage.

13.3.2. Establishment of flood fighting camps.

Flood fighting camp will be established at Jampur Flood Bund RD: 26+000, Harrow Flood Bund RD: 17+000, Bukhari Drain RD: 75+000.

13.3.3. Duties of Officers / officials and their camp at sites.

All concerned Sub Divisional Officer & their Staff is to remain in the area round the clock. As a matter of fact further flood information is to be telecasted. As such organization of this Division has to be very vigilant throughout the flood season. Staff of this Division is set on high alert during day and night from 15 June up-to 15 September. Consequently flood watching camps are installed at vulnerable points. Adequate arrangements to fight bags the flood according to its magnitude are made by all Sub Division. Any unwanted happening is to be reported by Sub Divisional officer to the Executive Engineer, Superintending Engineer, Chief Engineer, Secretary, Irrigation & Power Department. It is also duty of the Sub Divisional Officer to deliver messages well in time to the local civil authorities in addition to military personnel.

| Camp No. | Location | Officer (Name / Designation / Contact No. | Official (Name / Designation / Contact No.) |
| --- | --- | --- | --- |
| 1 | Jampur Flood Bund RD: 26+000 | Muhammad Nasrullah SDO Mechanical Mob: 0333-3625555 | Ubaidullah, Sub Engineer Jampur Section Mob:0331-7014141 |
| 2 | Harrow Flood Bund RD: 17+000 |  | Sabir Hussain, Sub Engineer Kotla Mughlan Section Mob: 0300-3282020 |
| 3 | Bukhari Drain RD: 75+000 | Muhammad Abdul Mueed SDO Hill Torrent-I Mob: 0335-0502269 | Muhammad Azam Suleman, Sub Engineer Mithawan-II Section, Mob: 0344-5493305 |

13.3.4. Departmental machinery available.

Tractor with trolleys = 02 No.

13.3.5. Machinery available from private source.

Excavator, Bulldozer, Tractors will be hired by the Government Contractor.

| Sr. No. | Government Contractor Name | Contact No. |
| --- | --- | --- |
| 1- | M/s Sparco Construction Company | 0321-8437629 |
| 2- | M/s Rana Traders | 0300-4000662 |
| 3- | Ehsan Bari | 0333-6449205 |
| 4- | Allied Construction Company | 0336-6780110 |
| 5- | Sabir Hussain | 0333-6447489 |

13.3.6. Flood Fighting Material Required.

Required flood fighting material for Jampur Flood Bund & Harrow Flood Bund is tabulated as below:-

13.3.7. Flood Fighting Material Available.

Available flood fighting material for Jampur Flood Bund & Harrow Flood Bund is tabulated as below:-

| Sr. No. | Items | Unit | Total Required Quantity | Available Quantity | Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 5 | 0 | 5 | Balance required flood fighting Material will be procured before 30.06.2025 |
| 2. | Trangers / wire crate | No. | 75 | 0 | 75 |  |
| 3. | Gunny Bags | No. | 2000 | 0 | 2000 |  |
| 4. | Axes | No. | 20 | 1 | 19 |  |
| 5. | Kassies with handle | No. | 40 | 2 | 38 |  |
| 6. | Hammers 10 Lbs | No. | 20 | 0 | 20 |  |
| 7. | Bucket 12 Inch Size | No. | 80 | 2 | 78 |  |
| 8. | Pegs 6-8 Ft Long | No. | 2250 | 150 | 2100 |  |
| 9. | Basket Toot | No. | 80 | 5 | 75 |  |
| 10. | Sutlli | Kg | 10 | 0 | 10 |  |
| 11. | Air Cooler 12 Watt | No. | 5 | 0 | 5 |  |
| 12. | Water Cooler & W. Set | No. | 5 | 0 | 5 |  |
| 13. | Chargeable Torches | No. | 30 | 0 | 30 |  |
| 14. | Petrol & M.oil | Ltr | 800 | 0 | 800 |  |
| 15. | Generator Engine | No. | 2 | 1 | 1 |  |
| 16. | Electric Wire | Mtr | 140 | 0 | 140 |  |
| 17. | Electric Blub | Dzn | 6 | 0 | 6 |  |
| 18. | Switch | No. | 50 | 0 | 50 |  |
| 19. | Boards | No. | 15 | 0 | 15 |  |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 |  |
| 21. | Bamboos | No. | 700 | 20 | 680 |  |
| 22. | Swiss cottage / Choldari | No. | 2 | 0 | 2 |  |
| 23. | Chair Set with Table | No. | 3 | 0 | 3 |  |

13.4. Detail of other infrastructure like electric, sui gas, telephones installations, road network, other buildings, canal & drainage network.

- Canal

i. Dajal Canal, 	ii. Link No. III		iii. Kadra Canal

- Road

- Indus Road (N-55)

- Hospital

- District Head Quarter Hospital, Rajanpur

- Sui Gas

--

- Telephone installation

--

- Drainage Network

i. Tayyab Drain, ii- New Manka Drain, iii- Qutab Drain, iv- Bukhari Drain

v- Darkhast Drain, vi- FCC(s)-123, 145, 166, 177 & 186.

Chapter No.13.

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITES

Vulnerable Sites

B. 	Fakhar Flood bund.

- RD: 10+000 – 24+000

Watching & Deployment of Machinery through local Contractor stand by at vulnerable sites to handle any situation during the flood season.

13.1. 	Plan showing route of flood water coming out of the breaches supported

with level.

Marked index plan attached.

13.2. 	Detail of villages/ abadies likely to be affected and this also be shown

on the plan.

Fakahr Flood Bund RD: 10+000 - 24+000

Villages:	Basti Miani, Basti Gopang, Basti Mulan, Basti Lashari, Basti Sikhani, Basti Kotla Hassan, Kot Mithan Town

Abadi:	About 285000 People

If any mishap occurs on this vulnerable reach on Fakhar Flood Bund the water will inundate the above village’s agriculture land and adjoining abadies and other infrastructures.

13.3. Strategy and action taken be explained in detail.

By 1st of June, the strength of the existing establishment is doubled to clear the jungle on slopes and toe of the Embankments shanks etc. of the Spurs. Emphasis is laid to locate the holes of burrowing animals. These if found / detected or to be puddle to ensure the safety of the embankments.

In emergency the flood information is received from upper region, the Sub Divisional Officer and Sub Engineer Incharge shift on their head Quarter at site. The establishment is deployed according to flood stage and as prescribed, round the clock vigilant watching is arranged along the entire embankment to take care of eventualities. The staff is equipped with, torches, Gunny bags, kassies and any other relevant material to control seepage/ leakage in the embankment. The information is passed on quickly to the concerned quarter and simultaneously action is taken to close the same. In the event of exceptionally high flood stage the heavy earth moving machinery (Bull-Dozers) of agriculture engineering department / machinery circles also kept ready and placed on alert at vulnerable sites of Fakhar Flood Bund.

13.3.1. Arrangements.

The flood fighting material along-with watching establishment will be arranged at site as warning of high flood issued from Chasma Barrage.

13.3.2. Establishment of flood fighting camps.

Flood fighting camp will be established at Fakhar Flood Bund RD: 4+000.

13.3.3. Duties of Officers / officials and their camp at sites.

All concerned Sub Divisional Officer & their Staff is to remain in the area round the clock. As a matter of fact further flood information is to be telecasted. As such organization of this Division has to be very vigilant throughout the flood season. Staff of this division is set on high alert during day and night from 15 June up-to 15 September. Consequently flood watching camps are installed at vulnerable points. Adequate arrangements to fight bags the flood according to its magnitude are made by all Sub Division. Any unwanted happening is to be reported by Sub Divisional officer to the Executive Engineer, Superintending Engineer, Chief Engineer, Secretary, Irrigation & Power Department. It is also duty of the Sub Divisional Officer to deliver messages well in time to the local civil authorities in addition to military personnel.

| Camp No. | Location | Officer (Name / Designation / Contact No. | Official (Name / Designation / Contact No.) |
| --- | --- | --- | --- |
| 1 | Fakhar Flood Bund, RD: 4+000 | Qazi Faiz ul Hassan SDO Hill Torrent-II Mob: 0334-6431973 | Muhammad Rafique, Sub Engineer Chachar Section Mob: 03338822684 |

13.3.4. Departmental machinery available.

Tractor with trolleys = 02 No.

13.3.5. Machinery available from private source.

Excavator, Bulldozer, Tractors will be hired by the Government Contractor.

| Sr. No. | Government Contractor Name | Contact No. |
| --- | --- | --- |
| 1- | M/s Sparco Construction Company | 0321-8437629 |
| 2- | M/s Rana Traders | 0300-4000662 |
| 3- | Ehsan Bari | 0333-6449205 |
| 4- | Allied Construction Company | 0336-6780110 |
| 5- | Sabir Hussain | 0333-6447489 |

13.3.6. Flood Fighting Material Required.

Required flood fighting material for Fakhar Flood Bund is tabulated as below:-

13.3.7. Flood Fighting Material Available.

Available flood fighting material for Fakhar Flood Bund is tabulated as below:-

| Sr. No. | Item | Unit | Required Quantity | Available Quantity | Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 2 | 0 | 2 | Balance required flood fighting Material will be procured before 30.06.2025 |
| 2. | Trangers / wire crate | No. | 75 | 0 | 75 |  |
| 3. | Gunny Bags | No. | 200 | 25 | 175 |  |
| 4. | Axes | No. | 20 | 1 | 19 |  |
| 5. | Kassies with handle | No. | 30 | 2 | 28 |  |
| 6. | Hammers 10 Lbs | No. | 5 | 0 | 5 |  |
| 7. | Bucket 12 Inch Size | No. | 20 | 2 | 18 |  |
| 8. | Pegs 6-8 Ft Long | No. | 300 | 35 | 265 |  |
| 9. | Basket Toot | No. | 150 | 0 | 150 |  |
| 10. | Sutlli | Kg | 2 | 0 | 2 |  |
| 11. | Air Cooler 12 Watt | No. | 2 | 0 | 2 |  |
| 12. | Water Cooler & Water Set | No. | 2 | 0 | 2 |  |
| 13. | Chargeable Torches | No. | 15 | 0 | 15 |  |
| 14. | Petrol & M.oil | Ltr | 100 | 0 | 100 |  |
| 15. | Generator Engine | No. | 1 | 0 | 1 |  |
| 16. | Electric Wire | Mtr | 2 | 0 | 2 |  |
| 17. | Electric Blub | Dzn | 100 | 0 | 100 |  |
| 18. | Switch | No. | 30 | 0 | 30 |  |
| 19. | Boards | No. | 20 | 0 | 20 |  |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 |  |
| 21. | Bamboos | No. | 200 | 6 | 194 |  |
| 22. | Swiss cottage / Choldari | No. | 1 | 0 | 1 |  |
| 23. | Chair Set with Table | No. | 2 | 0 | 2 |  |

13.4. Detail of other infrastructure like electric, sui gas, telephones installations, road network, other buildings, canal & drainage network.

- Canal

i. Dajal Canal, 	ii. Link No. III		iii. Kadra Canal

- Road

- Indus Road (N-55)

- Hospital

- District Head Quarter Hospital, Rajanpur

- Sui Gas

--

- Telephone installation

--

- Drainage Network

i. Tayyab Drain, ii- New Manka Drain, iii- Qutab Drain, iv- Bukhari Drain

v- Darkhast Drain, vi- FCC(s)-123, 145, 166, 177 & 186.

Chapter No.13.

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITES

Vulnerable Sites

C. 	Rojhan Flood Bund.

- R.D: 10+000 - 25+000

Watching & Deployment of Machinery through local Contractor stand by at vulnerable sites to handle any situation during the flood season.

13.1. 	Plan showing route of flood water coming out of the breaches supported

with level.

Marked index plan attached.

13.2. 	Detail of villages/ abadies likely to be affected and this also be shown

on the plan.

Rojhan Flood Bund RD: 10+000 - 25+000

Villages:	Basti Galiani, Basti Punnar Mazari, Rojhan City.

Abadi :		About 43500 People

If any mishap occurs on this vulnerable reach on Rojhan Flood Bund the water will inundate the above village’s agriculture land and adjoining abadies and other infrastructures.

13.3. Strategy and action taken be explained in detail.

By 1st of June, the strength of the existing establishment is doubled to clear the jungle on slopes and toe of the Embankments shanks etc. of the Spurs. Emphasis is laid to locate the holes of burrowing animals. These if found / detected or to be puddle to ensure the safety of the embankments.

In emergency the flood information is received from upper region, the Sub Divisional Officer and Sub Engineer Incharge shift on their head Quarter at site. The establishment is deployed according to flood stage and as prescribed, round the clock vigilant watching is arranged along the entire embankment to take care of eventualities. The staff is equipped with, torches, Gunny bags, kassies and any other relevant material to control seepage/ leakage in the embankment. The information is passed on quickly to the concerned quarter and simultaneously action is taken to close the same. In the event of exceptionally high flood stage the heavy earth moving machinery (Bull-Dozers) of agriculture engineering department / machinery circles also kept ready and placed on alert at vulnerable sites of Rojhan Flood Bund RD: 12+000-14+000.

13.3.1. Arrangements.

The flood fighting material along-with watching establishment will be arranged at site as warning of high flood issued from Chasma Barrage.

13.3.2. Establishment of flood fighting camps.

Flood fighting camp will be established at Rojhan Flood Bund RD: 14+000 and Umer Kot Flood Bund RD: 13+000.

13.3.3. Duties of Officers / officials and their camp at sites.

All concerned Sub Divisional Officer & their Staff is to remain in the area round the clock. As a matter of fact further flood information is to be telecasted. As such organization of this Division has to be very vigilant throughout the flood season. Staff of this division is set on high alert during day and night from 15 June up-to 15 September. Consequently flood watching camps are installed at vulnerable points. Adequate arrangements to fight bags the flood according to its magnitude are made by all Sub Division. Any unwanted happening is to be reported by Sub Divisional officer to the Executive Engineer, Superintending Engineer, Chief Engineer, Secretary, Irrigation & Power Department. It is also duty of the Sub Divisional Officer to deliver messages well in time to the local civil authorities in addition to military personnel.

| Camp No. | Location | Officer (Name / Designation / Contact No. | Official (Name / Designation / Contact No.) |
| --- | --- | --- | --- |
| 1 | Rojhan Flood Bund RD: 14+000 | Qazi Faiz ul Hassan SDO Hill Torrent-II Mob: 0334-6431973 | Muhammad Adeeb, Sub Engineer Pitok Section Mob: 0334-6940819 |
| 2 | Umer Kot Flood Bund RD: 13+000 |  |  |

13.3.4. Departmental machinery available.

Tractor with trolleys = 02 No.

13.3.5. Machinery available from private source.

Excavator, Bulldozer, Tractors will be hired by the Government Contractor.

| Sr. No. | Government Contractor Name | Contact No. |
| --- | --- | --- |
| 1- | M/s Sparco Construction Company | 0321-8437629 |
| 2- | M/s Rana Traders | 0300-4000662 |
| 3- | Ehsan Bari | 0333-6449205 |
| 4- | Allied Construction Company | 0336-6780110 |
| 5- | Sabir Hussain | 0333-6447489 |

13.3.6. Flood Fighting Material Required.

Required flood fighting material for Rojhan Flood Bund and Umer Kot Flood Bund is tabulated as below:-

13.3.7. Flood Fighting Material Available.

Available flood fighting material for Rojhan Flood Bund and Umer Kot Flood Bund is tabulated as below:-

| Sr. No. | Item | Unit | Required Quantity | Available Quantity | Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Search Lights | No. | 2 | 0 | 2 | Balance required flood fighting Material will be procured before 30.06.2025 |
| 2. | Trangers / wire crate | No. | 70 | 0 | 70 |  |
| 3. | Gunny Bags | No. | 1000 | 35 | 965 |  |
| 4. | Axes | No. | 30 | 2 | 28 |  |
| 5. | Kassies with handle | No. | 60 | 2 | 58 |  |
| 6. | Hammers 10 Lbs | No. | 10 | 0 | 10 |  |
| 7. | Bucket 12 Inch Size | No. | 20 | 0 | 20 |  |
| 8. | Pegs 6-8 Ft Long | No. | 60 | 10 | 50 |  |
| 9. | Basket Toot | No. | 200 | 0 | 200 |  |
| 10. | Sutlli | Kg | 5 | 0 | 5 |  |
| 11. | Air Cooler 12 Watt | No. | 2 | 0 | 2 |  |
| 12. | Water Cooler & Water Set | No. | 2 | 0 | 2 |  |
| 13. | Chargeable Torches | No. | 25 | 0 | 25 |  |
| 14. | Petrol & M.oil | Ltr | 100 | 0 | 100 |  |
| 15. | Generator Engine | No. | 1 | 0 | 1 |  |
| 16. | Electric Wire | Mtr | 2 | 1 | 1 |  |
| 17. | Electric Blub | Dzn | 100 | 0 | 100 |  |
| 18. | Switch | No. | 30 | 0 | 30 |  |
| 19. | Boards | No. | 20 | 0 | 20 |  |
| 20. | Water Proof Camps | No. | 1 | 0 | 1 |  |
| 21. | Bamboos | No. | 200 | 7 | 193 |  |
| 22. | Swiss cottage / Choldari | No. | 1 | 0 | 1 |  |
| 23. | Chair Set with Table | No. | 2 | 0 | 2 |  |

13.4. Detail of other infrastructure like electric, sui gas, telephones installations, road network, other buildings, canal & drainage network.

- Canal

i. Dajal Canal, 	ii. Link No. III		iii. Kadra Canal

- Road

Indus Road (N-55)

- Hospital

District Head Quarter Hospital, Rajanpur

- Sui Gas

--

- Telephone installation

--

- Drainage Network

i. Tayyab Drain, ii- New Manka Drain, iii- Qutab Drain, iv- Bukhari Drain

v- Darkhast Drain, vi- FCC(s)-123, 145, 166, 177 & 186.

Chapter-14

ACTION PLAN

14.1	Re-shuffling/recouping plan of reserve stone departmentally.

Limited Reserve Stock of Stone is available at site where necessary. Reshuffling / recouping of reserve stock of stone will be completed before coming flood season 2025. Demand of funds has been submitted to Administrative Department for early arrangement of requisite funds. On receipt of funds, the balance quantity of reserve stock of stone will be procured.

  - Detail of inlet / outlet crossing along with closing methodology

Inlet and outlet / water course X-ings will be plugged well in time by sand filled plastic bags by work charge establishment. The detail is as under:-

| Sr.No. | Name of Channel | Water Course X-ing RD: |
| --- | --- | --- |
| 1 | Main Qutab Drain | 198+000 |
|  |  | 200+000 |
|  |  | 205+000 |
|  |  | 210+000 |
|  |  | 264+700 |
|  |  | 268+900 |
|  |  | 271+500 |
|  |  | 272+000 |
|  |  | 275+400 |
|  |  | 278+000 |
|  |  | 282+430 |
|  |  | 285+300 |
|  |  | 288+000 |
|  |  | 289+000 |
| 2 | FCC-145 Dajal Branch | 41+700 |
|  |  | 43+900 |
|  |  | 50+200 |
|  |  | 56+500 |
| 3 | FCC-166 Dajal Branch | 4+200 |
|  |  | 10+200 |
| 4 | FCC-177 Dajal Branch | 5+700 |
|  |  | 9+850 |
|  |  | 18+650 |
|  |  | 20+850 |
| 5 | FCC-186 Dajal Branch | 13+100 |
|  |  | 15+200 |
|  |  | 30+000 |
|  |  | 37+700 |
|  |  | 39+800 |

| Sr.No. | Name of Flood Bund / Embankment | Reach RD: | No. of Water Course X-ing. |
| --- | --- | --- | --- |
| 1 | Rojhan Flood Bund | 1+000-2+000 | 1 |
|  |  | 6+000-7+000 | 1 |
|  |  | 7+000-8+000 | 1 |
|  |  | 8+000-9+000 | 2 |
|  |  | 9+000-10+000 | 1 |
|  |  | 12+000-13+000 | 2 |
|  |  | 13+000-14+000 | 2 |
|  |  | 15+000-16+000 | 2 |
|  |  | 16+000-17+000 | 2 |
|  |  | 19+000-20+000 | 2 |
|  |  | 21+000-22+000 | 3 |
|  |  | 22+000-23+000 | 1 |
|  |  | 23+000-24+000 | 2 |
|  |  | 28+000-29+000 | 3 |
|  |  | 30+000-31+000 | 1 |
|  |  | 31+000-32+000 | 1 |
|  |  | 33+000-34+000 | 1 |
|  |  | 37+000-38+000 | 1 |
|  |  | 41+000-42+000 | 1 |
|  |  | 42+000-43+000 | 2 |
|  |  | 43+000-44+000 | 3 |
| 2 | Umer Kot Flood Bund | 1+000-2+000 | 1 |
|  |  | 6+000-7+000 | 1 |
|  |  | 7+000-8+000 | 1 |
|  |  | 8+000-9+000 | 2 |
|  |  | 9+000-10+000 | 1 |
|  |  | 12+000-13+000 | 2 |
|  |  | 13+000-14+000 | 2 |
|  |  | 15+000-16+000 | 2 |
|  |  | 16+000-17+000 | 2 |
|  |  | 18+000-19+000 | 2 |
|  |  | 19+000-20+000 | 3 |
|  |  | 21+000-22+000 | 1 |
|  |  | 22+000-23+000 | 2 |
|  |  | 23+000-24+000 | 3 |
|  |  | 28+000-29+000 | 1 |
|  |  | 30+000-31+000 | 1 |
|  |  | 31+000-32+000 | 1 |
|  |  | 35+000-36+000 | 1 |
|  |  | 39+000-40+000 | 1 |
|  |  | 40+000-41+000 | 2 |
|  |  | 41+000-42+000 | 3 |

  - Deployment Machinery (medium to high flood)

| Sub Division | Detail of Structure | Excavator | Dozer | Trolley / Dumper | Tractor with front Blade | Labour WC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Beldar + Mate |
| Hill Torrent-I | - Bukhari Drain - Tayyab Drain with system | 1 | 1 | 6 | 8 | 12+1 |
| Hill Torrent- II | i. Rojhan Flood Bund ii. Umer Kot Flood Bund iii. Fakhar Flood Bund iv. Qutab Drain with system | 2 | - | 6 | 8 | 12+1 |
| Mechanical | i. Jampur Flood Bund ii. Harrow Flood Bund | 1 | 2 | 6 | 8 | 12+1 |
|  | Total:- | 4 | 3 | 18 | 24 | 36+3 |

Note: -

Payment for Machinery and work charge establishment will be subject to verification by the TPM / Consultants.

  - Deployment of Machinery (High to very high flood)

| Sub Division | Detail of Structure | Excavator | Dozer | Trolley / Dumper | Tractor with front Blade | Labour WC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Beldar + Mate |
| Hill Torrent-I | - Bukhari Drain - Tayyab Drain with system | 2 | 2 | 12 | 10 | 24+2 |
| Hill Torrent-II | i. Rojhan Flood Bund ii. Umer Kot Flood Bund - Fakhar Flood Bund - Qutab Drain with system | 2 | 1 | 12 | 10 | 24+2 |
| Mechanical | i. Jampur Flood Bund ii. Harrow Flood Bund | 2 | 4 | 12 | 10 | 24+2 |
|  | Total:- | 6 | 7 | 36 | 30 | 72+6 |

Note: -

Payment for Machinery and work charge establishment will be subject to verification by the TPM / Consultants.

  - Deployment of Machinery (high to exceptionally high flood)

| Sub Division | Detail of Structure | Excavator | Dozer | Trolley / Dumper | Tractor with front Blade | Labour WC |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Beldar + Mate |
| Hill Torrent- I | - Bukhari Drain - Tayyab Drain with system | 10 | 2 | 20 | 15 | 84+7 |
| Hill Torrent- II | i. Rojhan Flood Bund ii. Umer Kot Flood Bund - Fakhar Flood Bund - Qutab Drain with system | 5 | 2 | 20 | 10 | 84+7 |
| Mechanical | i. Jampur Flood Bund ii. Harrow Flood Bund | 10 | 5 | 20 | 14 | 84+7 |
|  | Total:- | 25 | 9 | 60 | 39 | 252+21 |
| Note: - Payment for Machinery and work charge establishment will be subject to verification by the TPM / Consultants. |  |  |  |  |  |  |

  - Police deployment plan

- Breaching Section

| Sr. No. | Site breaching section | Concerned Canal Division | Concerned Police Station & District | Police Personal to be deployed |  | Remarks (if Any) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Inspector / SI / ASI | Constables |  |
| 1 | No Breaching Section | - | - | - | - | - |

- Vulnerable Sits

| Sr. No. | Vulnerable sites used for illegal cuts during floods | Concerned Canal Division | Concerned Police Station and District | Police Personnel to be deployed |  |  | Remarks (If any) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sub Inspector | A.S.I. | Constable |  |
| 1 | Pull Pathan | Jampur Construction Division, Rajanpur | PS Rajanpur | 1 | 1 | 5 | - |
| 2 | Fakhar Flood Bund |  | PS Kotmithan | 1 | 1 | 5 | - |

Note: 	Police help will be taken on emergent biases, when and where needed time to time.

  - Details of synthetic bags with capacity of 500 Kg and 1000 Kg

Not required, as heavy machinery is required to operate synthetic bags with capacity of 500 Kg and 1000 Kg.

  - Details of polythene sheets of black color to protect upstream slope against wave action and to control seepage through embankments

Not required

Chapter-15

Backup Divisions (In case of breach)

Rajanpur Canal Division will be the ‘Backup Division’ with fully support and backup to pass the flood safely.

Executive Engineer

Jampur Construction Division

Jampur

|  | Superintending Engineer Project Circle (Irrigation) Dera Ghazi Khan |
| --- | --- |

Chief Engineer

D.G. Khan, Irrigation Zone

Dera Ghazi Khan

ACTION PLAN DURING FLOOD

(EXCEPTIONALLY HIGH STAGE) IN JAMPUR CONSTRUCTION DIVISION

| Sr. No. | Name of Site | Length in Mile | Vulnerable Reach | Camp Location | Site Incharge by Name & Cell No. | Machinery Deployed |  |  |  |  | Availability of Stone (Lac Cft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolley / Dumper | Tractor with front Blade | Labour Beldar + Mate |  |
| 1 | Jampur Flood Bund | 4.0 | RD: 4-26 | RD: 26 | Muhammad Nasrullah SDO Mechanical 0333-3625555 | 3 | 1 | 6 | 6 | 24+2 | -- |
| 2 | Harrow Flood Bund | 7 | RD: 12-22 | RD: 17 |  | 2 | 1 | 6 | 6 | 24+2 | 0.08 |
| 3 | Fakhar Flood Bund | 2.0 | RD: 10-20 | RD: 4 |  | 2 | 1 | 6 | 8 | 24+2 | 1.50 |
| 4 | Bukhari Drain | 1.0 | RD: 75-22 | RD: 75 | Muhammad Abdul Mueed SDO Hill Torrent-I Mob: 0335-0502269 | 1 | 1 | 4 | 4 | 12+1 | -- |
| 5 | Main Qutab Drain | 5.0 | RD:270-270 | RD:230-240 |  | 2 | 2 | 6 | 6 | 24+2 |  |
| 6 | Qutab Drain | 1.2 2.0 | FCC-177 FCC-186 | RD:19-25 RD:25-35 |  | 2 | 2 | 8 | 8 | 24+2 | -- |
| 7 | Tayyab Drainage System | 5 | RD:80-130 | Tayyab Drain FCC-95 FCC-109 |  | 2 | 2 | 6 | 4 | 30+2 |  |
| 8 | Rojhan Flood Bund | 5.0 | RD: 0-30 | RD: 14 | Qazi Faiz ul Hassan SDO Hill Torrent-II 0333-6431973 | 1 | 1 | 6 | 6 | 12+1 | 1.261 |
| 9 | Umer Kot Flood Bund | 2.0 | RD: 0-13 | RD: 13 |  | 1 | 1 | 6 | 6 | 12+1 | 0.90 |