GOVERNMENT OF THE PUNJAB

FLOOD FIGHTING PLAN 2025

MUZAFFARGARH CANAL DIVISION

MUZAFFARGARH

Muzaffargarh Canal Circle  	           D.G. Khan Irrigation Zone,

Multan				Dera Ghazi Khan

2025

TABLE OF CONTENTS.

| Sr. No. |  | Description | Page No |
| --- | --- | --- | --- |
| PART – A - |  |  |  |
| Chapter 1 |  | Salient feature of the concerned Division. |  |
|  | 1.1 | Location | 1 |
|  | 1.2 | General Description | 1 |
|  | 1.3 | Administration | 2 |
| Chapter 2 |  | Flood Protection and River Training works. |  |
|  | 2.1 | Design parameter of Training works | 3-4 |
|  | 2.2 | Location Map | 5 |
| Chapter 3 |  | Brief History of past flood events | 6-8 |
| Chapter 4 |  | Design data, Historic peak flood data and previous five year flood data of Head Works /Barrages and other control points |  |
|  | 4.1 | Flood Limits | 9 |
|  | 4.2 | Time Lags of Floods | 9 |
|  | 4.3 | Highest Floods | 9 |
|  | 4.4 | Peak discharge | 10 |
| Chapter 5 |  | Flood fighting Strategy | 11-12 |
| Chapter 6 |  | Flood damages restoration works | 13 |
| Chapter 7 |  | Flood fighting watching arrangements |  |
|  | 7.1 | Pre-Flood Arrangements | 14 |
|  |  | Detail of Reserve Stock of Stone | 14-15 |
|  | 7.2 | Watching Establishment | 15 |
|  | 7.3 | Arrangement at Sensitive sites | 16 |
|  | 7.4 | Watching material | 16-17 |
|  | 7.5 | Arrangement for sounding & probing | 18 |
|  | 7.6 | Lighting arrangement | 18 |
|  | 7.7 | Ration arrangement | 18 |
|  | 7.8 | P.O.L arrangement for vehicle | 18 |
|  | 7.9 | Transportation | 18 |
|  | 7.10 | Law and Order | 18 |
|  | 7.11 | Medical arrangement for Labour | 19 |
|  | 7.12 | Liaison with other Department | 19 |
|  | 7.13 | Role of the Army | 19 |
|  | 7.14 | Duties of Telephone Attendant | 19 |
|  | 7.15 | Wireless arrangements | 19 |
| Chapter 8 |  | Detail of encroachment | 20 |
| Chapter 9 |  | Duty roaster /flood fighting program | 21 |
| Chapter 10 |  | Emergency Telephone Nos. | 22-23 |
| Chapter 11 |  | Standard operating procedure (SOP) for breaching sections. |  |
|  | 11.1 | History of the breaching section | 24 |
|  | 11.2 | Location, design, quantity and variety of the explosive required for detonation. | 24 |
|  | 11.3 | Arrangement of explosive and security of explosive stores | 24 |
|  | 11.4 | List of the security staff alongwith detail of their training etc | 24 |
|  | 11.5 | Detail of mechanical means as a standby arrangements in case of detonation failure. | 24 |
|  | 11.6 | Duty roaster in case of critical situation | 24 |
|  | 11.7 | Breaching committee with their action plan | 24 |
|  | 11.8 | List of the villages likely to be inundated in case of breach | 24 |
|  | 11.9 | Announcement and detail of evacuation arrangements | 24 |
|  | 11.10 | Details of coordination with Civil /Army authorities | 24 |
|  | 11.11 | Parallel communication arrangements | 24 |
|  | 11.12 | Index Plan | 24 |
| Chapter 12 |  | Vulnerable sites on Flood Bunds /Structures: |  |
|  | 12.1 | Apprehended breaches in flood Bunds /Structures | 25 |
|  | 12.2 | Operation of Breaching sections | 25 |
|  | 12.3 | Breaches due to rise of flood water, deterioration of flood bunds etc. | 25 |
| Chapter 13 |  | Emergency Contingency Plan for vulnerable site (sites) |  |
|  |  | SITE-1, 2 & 3 |  |
|  | 13.1 | Plan showing route of flood water coming out of the breach supported with levels. | 26 |
|  | 13.2 | Detail of villages abadies likely to be affected and this should also be shown on the plan | 26 |
|  | 13.3 | Strategy and action taken be explained in detail. This may include. | 26 |
|  | 13.3.1 | Arrangements | 27 |
|  | 13.3.2 | Establishment of flood fighting camps | 27 |
|  | 13.3.3 | Duties of Officers/Officials and their camp sites | 27 |
|  | 13.3.4 | Departmental Machinery available | 27 |
|  | 13.3.5 | Machinery available from private source | 27 |
|  | 13.3.6 | Flood fighting material required | 28 |
|  | 13.3.7 | Flood Fighting material available | 29 |
|  |  | SITE-4 |  |
|  | 13.1 | Plan showing route of flood water coming out of the breach supported with levels. | 30 |
|  | 13.2 | Detail of villages abadies likely to be affected and this should also be shown on the plan | 30 |
|  | 13.3 | Strategy and action taken be explained in detail. This may include. | 30 |
|  | 13.3.1 | Arrangements | 31 |
|  | 13.3.2 | Establishment of flood fighting camps | 31 |
|  | 13.3.3 | Duties of Officers/Officials and their camp sites | 31 |
|  | 13.3.4 | Departmental Machinery available | 31 |
|  | 13.3.5 | Machinery available from private source | 31 |
|  | 13.3.6 | Flood fighting material required | 32 |
|  | 13.3.7 | Flood Fighting material available | 33 |
|  |  | SITE-5 & 6 |  |
|  | 13.1 | Plan showing route of flood water coming out of the breach supported with levels. | 34 |
|  | 13.2 | Detail of villages abadies likely to be affected and this should also be shown on the plan | 34 |
|  | 13.3 | Strategy and action taken be explained in detail. This may include. | 35 |
|  | 13.3.1 | Arrangements | 35 |
|  | 13.3.2 | Establishment of flood fighting camps | 35 |
|  | 13.3.3 | Duties of Officers/Officials and their camp sites | 35 |
|  | 13.3.4 | Departmental Machinery available | 35 |
|  | 13.3.5 | Machinery available from private source | 35 |
|  | 13.3.6 | Flood fighting material required | 36 |
|  | 13.3.7 | Flood Fighting material available | 37 |
|  |  | Main Campus of offices of the sub divisional officers Jatoi Canal Sub Division | 38 |
|  | 13.4 | Detail of other infrastructure like Electric, Sui Gas, Telephone installations, Road network, other buildings, Canals and drainage network. | 38 |
| Chapter 14 |  | Action Plan |  |
|  | 14.1 | Re-Shuffling / Recouping plan of reserve stone departmentally. | 39 |
|  | 14.2 | Detail of inlet / outlet crossing along with closing methodology. | 39 |
|  | 14.3 | Deployment Machinery (Medium to high Flood) | 40 |
|  | 14.4 | Deployment Machinery (High to Very high Flood) | 40 |
|  | 14.5 | Deployment Machinery (High to exceptionally high Flood) | 40 |
|  | 14.6 | Police deployment plan | 41 |
|  | 14.7 | Details of Synthetic bags with capacity of 500 kg and 1000kg. | 41 |
|  | 14.8 | Detail of polythene sheet of black colour to protect upstream slope against wave action and to control seepage through embankments. | 41 |
|  |  | Follow up actions on flood preparedness activities for the year 2025 statement showing the progress achieved up to | 42 |
| Chapter. 15 |  | Back up division (In case of Breach). | 43 |

P A R T.

A

CHAPTER.

1

SALIENT FEATURE

1.1	LOCATION

This Division is situated on left side of Indus River. The Muzaffargarh Canal Division has Main Muzaffargarh Canal from RD: 206+700 to 370+700 (tail) with its system is on left side of Indus.

1.2	GENERAL DESCRIPTION

Muzaffargarh Canal Division has a mighty River of the Country, i.e. River Indus, on Western side. This River has confluence point at village Sarki of Tehsil Ali Pur with River Chenab. In order to protect the Irrigation network and other infrastructures, from the spills and erosive actions of the River, a series of flood protection works and No of River Training and protection works have been constructed along the river. There is 32.087 miles length of the flood embankments together with 68 No. J-Head Spurs, Stone Studs, Guide Head Spurs, Hockey Spurs and Concavo Convex Guide Wall.

The detail of flood embankment and River Training works is as under

STRUCTURES				      		LENGTH (MILES)

No.

- Flood Embankments.				  3			19.64

- J-Head Spurs					 10			 7.94

- Stone studs					 37			 1.87

- Guide Head Spurs				  1			 0.95

- Hockey Spur					  4			 1.99

- Concavo Convex Guide Wall		  1			 0.21

Total		56 No.			32.60 Miles

These flood works protect a population of 26 Lac inhabitants in Muzaffargarh District. Moreover, due to installation of Thermal Power Plant, Thal Jute Mills limited and Textile units, this district has been transformed into industrial area and its importance /sensitiveness is increasing day by day on the National scenario. Since the Muzaffargarh District remains in danger due to two Rivers, having different, catchment area, Flood characteristics, and timings, hence Flood Fighting Plan has been prepared to combat any emergent situation during flood season 2025.

  - ADMINISTRATIVE SETUP.

ADMINISTRATIVE STEP OF MUZAFFARGARH CANAL DIVISION.

Executive Engineer

Muzaffargarh Canal Division

Muzaffargarh

SDO

Muzaffargarh

at Muzaffargarh

SDO

Store & Workshop

at Muzaffargarh

SDO

Jatoi

at Jatoi

This Division also maintains major Irrigation system, namely, Muzaffargarh Canal System with cultural able command area of 0.278 Million Acre of Muzaffargarh District.

Detail of area as under:-

| Sr. No. | Name of Channel System | GCA in Acres | CCA in Acres |
| --- | --- | --- | --- |
| 1 | Muzaffargarh Canal System | 280692 | 255777 |

CHAPTER.

2

FLOOD PROTECTION AND RIVER TRAINING WORKS

  - DESIGN PARAMETER OF TRAINING WORKS

| Sr # | Name Of Work | Length in Mile | Designed free Board (feet) | Designed top Width (feet) | Designed side Slope |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | River Side | Country Side |
| 1 | Flood Bund along Soharu & Suleman Disty 0+000 to 12+000 | 2.40 | 7 | 25 | 3:1 | 2:1 |
| 2 | Khan Wah Flood RD 0+000 to 78+500 | 15.70 | 7 | 25 | 3:1 | 2:1 |
| 3 | Flood Bund along 1-AR Minor RD: 26+300 to 34+000. | 1.54 | 7 | 25 | 3:1 | 2:1 |
|  | Total | 19.64 |  |  |  |  |
|  | J-HEAD SPURS |  |  |  |  |  |
| 1 | J-Head Spur No. 1.-A at RD: 127+000 of Jatoi Branch | 0.56 | 7 | 30 | 3:1 | 2:1 |
| 2 | J-Head Spur No. 1 at RD: 135+000 of Jatoi Branch | 0.66 | 7 | 30 | 3:1 | 2:1 |
| 3 | J-Head Spur No. 2. at RD: 139+000 of Jatoi Branch | 0.56 | 7 | 30 | 3:1 | 2:1 |
| 4 | J-Head Spur No. 3. At RD: 2+000 Suleman Flood Bund | 1.70 | 7 | 30 | 3:1 | 2:1 |
| 5 | J-Head Spur No. 4. at RD: 8+400 Suleman Flood Bund | 0.58 | 7 | 30 | 3:1 | 2:1 |
| 6 | J-Head Spur No. 4-A at RD: 9+500 Suleman Flood Bund | 1.52 | 7 | 30 | 3:1 | 2:1 |
| 7 | J-Head Spur No. 5. RD: 7+500 Khan Wah Flood Bund | 0.80 | 7 | 30 | 3:1 | 2:1 |
| 8 | J-Head Spur No. 6. RD: 9+000 Khan Wah Flood Bund | 0.50 | 7 | 30 | 3:1 | 2:1 |
| 9 | J-Head Spur at RD: 34+500 of 1-AR Minor | 0.50 | 7 | 30 | 3:1 | 2:1 |
| 10 | J-Head Spur at RD: 40+300 1-AR Minor | 0.56 | 7 | 30 | 3:1 | 2:1 |
|  | Total | 7.94 |  |  |  |  |
|  | STONE STUDS |  |  |  |  |  |
| 1 | Stone stud RD 12+000 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 2 | Stone stud RD 13+000 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 3 | Stone Stud RD 15+000 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 4 | Sloping stone stud at RD: 15+300 Khanwah Flood Bund. | 0.02 | 7 | 20 | 2:1 | 2:1 |
| 5 | Stone stud RD 16+300 Khan Wah Flood Bund | 0.02 | 7 | 20 | 2:1 | 2:1 |
| 6 | Stone Stud RD 18+000 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 7 | Stone Stud RD 21+000 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 8 | Stone Stud RD 24+000 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 9 | Stone Stud RD 28+000 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 10 | Stone Stud RD 31+500 Khan Wah Flood Bund | 0.14 | 7 | 20 | 2:1 | 2:1 |
| 11 | Stone Stud RD 42+000 of 1-AR Minor | 0.11 | 7 | 20 | 2:1 | 2:1 |
| 12-31 | 20 No. Stone Studs along Khanwah Flood Bund between RD: 73+000 to 77+000. | 0.30 | 2 | 20 | 2:1 | 2:1 |
| 32-37 | 06 No. Stone Studs along Khanwah Flood Bund between RD: 46+000 to 48+000. | 0.30 | 0 | 15 | 2:1 | 2:1 |
|  | Total | 1.87 |  |  |  |  |
| 1 | Guide Spur No. 1 of 1 -AR Minor at RD:53+000 | 0.95 | 7 | 30 | 3:1 | 2:1 |
| 2 | Inverted Hockey Spur of 1-AR Minor at RD: 61+500. | 1.37 | 7 | 30 | 3:1 | 2:1 |
| 3 | Inverted Hockey Spur at RD: 38+000 1-AR Minor | 0.48 | 7 | 30 | 3:1 | 2:1 |
| 4 | Inverted Hockey Spur at RD: 15+650 Khanwah Flood Bund | 0.06 | 7 | 20 | 3:1 | 2:1 |
| 5 | Inverted Hockey Spur at RD: 71+750 Khanwah Flood Bund | 0.08 | 7 | 20 | 3:1 | 2:1 |
| 6 | Concavo convex Guide Wall at RD: 26+300 of 1-AR Minor | 0.21 | 7 | 25 | 3:1 | 2:1 |
|  | Total | 3.15 |  |  |  |  |

2.2 LOCATION MAP

CHAPTER.

3

Chapter. 3

BRIEF HISTORY OF PAST FLOOD EVENTS

INDUS RIVER

Unprecedented floods of 1988 & 1992 have brought morphological changes and the river is continuously drifting towards left. It has already swallowed hundreds acres of agricultural land besides shattering the irrigation network of Jatoi Brach System and the Jatoi branch has been re-aligned twice. A series of training works were constructed in the past to contain / train the river within its meandering belt. As a result of drifting, the river has further encroached the lands on the left side and is now threatening village Abadies. During the year 1999 and 2001, when the river abruptly receded, it changed its route toward extreme left and during 2001, punctured the Khan Wah Flood Bund at reaches RD. 54-57 and RD. 39-42 respectively, river remained within the deep channel otherwise there would have been colossal losses to crops and Abadies located in the area.

After conducting detail model study, the report of Irrigation Research Institute was received vide Chief Engineer Research letter No. 2764/836-C dated 16-08-2004 in which the following structures were recommended to be constructed for the long term protection of the eroded area, existing river training works, and Muzaffargarh Canal System of Mauza Nobramda, Bait Daryai, Mauza Lundi Pittafi and Mauza Kundai from the erosion of mighty river Indus.

Basically the Khanwah flood bund was constructed to protect the important badies and City Jatoi, Ali Pur and other congested badies, very fertile agriculture lands and orchards from the spills only. Due to exceptionally high flood in the past and meandering behavior, during July 2009 and, August 2010, Indus River started advancing towards left bank by its erosive action and resultantly developed a River loop between J-Head Spur No. 6 at RD: 9000 Khanwah Flood Bund and started at RD: 13000, resultantly U/S apron of Stone Stud RD: 13000 was damaged. Severe erosive action started at sloping Spur RD: 13000 Khan Wah Flood Bund and inspire of all efforts, the structure was washed at a length of 700 ft.

The situation was so alarming that Khanwah Flood Bund was in danger. In case of breach to Khan Wah Flood Bund in this reach, the badies of Gabber Arrain and Basti Hardal Wala were dislocated and heavy irreparable losses were to be occurred. In view of the gravity of situation, it was allowed to take up the work under Para 2.89 PWD code. Day and night successful efforts were made to save the Stud from further damage and subsequently Khan Wah flood bund was saved and secured. During flood season 2010, due to serious situation, so many Mauza’s of fertile land i/c the considerable segment of road was eroded at Bait Daryai and came under the River hit. The irrigation infrastructure i.e. 1-R & 1-AR Minor of Soni Disty have also come alarmingly close to the River edge. Due to heavy and constant monsoon rainfall in the upper area of the Province Khyber Pukhtun Khah, Suleman Range on the west and in the catchments area, Indus River has been in exceptionally high Floods at all Barrages as well as Taunsa Barrage with the Discharge D/S 959117 Cusecs was recorded during August 2010. The duration of flood remained for the period of about 22 days. The heavy pressure of flood was experienced on left side, breached Left Marginal Bund of Taunsa Barrage. The flood water entered in Muzaffargarh Canal with uncontrolled excessive discharge. Muzaffargarh Canal and its system breached at various places. The accumulative flood water rushed on the country slope and inundated the vast area by damaging the abadies of Kot Adu, Sanawan, Mehmood Kot, Gurmani, Qasba Gujrat and Ghazi Ghat Town. A relief cut at RD: 125800/L Muzaffargarh Canal was made to release and divert of Flood water into Chenab River. The pressure of flood water was so severe that Muzaffargarh Canal was breached at 12 various places, Downstream Head Regulator Kalu at RD: 206700 in the jurisdiction of Muzaffargarh Canal Division 3 No breaches occurred at RD: 232-233/R, 232-233/L and 239-240\L.

Flood water after breaching U/S Kalu Regulator at RD: 194000/L & R damaging the low level area of Basira, Shah Jamal, Khangarh and Rohillan Wali, Shehr Sultan Town entered into Kot Adu Main Drain. The Flood Water was disposed off into Chenab River by making relief cut at RD: 55-56 Shehr Sultan Flood Bund and RD: 56-57 and 96-97 Chandar Bhan Flood Bund.

During floods 08/2010, Indus River started severe action on Stone Stud RD: 13000, RD: 15000 and RD: 18000 and damaged their solid stone portion, apron of J-Head Spur No. 6 Khanwah Flood Bund was also damaged. The parallel flow of Indus River also damaged Khanwah Flood Bund RD: 15000 to RD: 16000. Khanwah Flood Bund was saved by hectic flood fighting during floods and the area of Gabbar Arain behind Khanwah Flood Bund was saved.

Keeping in view in the past experience during floods 2010, the relief cuts will be made at suitable places and all arrangements will be made at site.

FLOOD 2019

During flood 2019 the behavior of River Indus has become active and hit the various structures i.e. apron of Hockey spur at RD: 61+500 1-AR Minor at different locations, at RD:47+000 to 57+000 of Khanwah Flood Bund, at RD:15+650 and at J-Head Spur No.6 of Khanwah Flood Bund are partially damaged.

FLOOD 2020

During flood 2020 the behavior of River Indus has become active and hit the various structures i.e. Shank or Inverted Hockey (Guide Head Spur No.2) at RD: 5+000 to 5+500 at Tail of 1-R Minor RD: 61+500 and River Side Slope of Khanwah Flood Bund RD:7+500 to 11+000 are partially damaged.

FLOOD 2021

During Flood 2021, there was again low flood of discharge 304121 Cusecs dated: 04.08.2021 in River Indus and due to this low flood everything went good. In 2021 flood D/S Taunsa Headworks was natural and not touches the limit of medium flood even. The flood water passed to M.Garh District without any major damage and the District remained safe.

FLOOD 2022

During Flood 2022 the behavior of River Indus has becomes more active towards left, Resultantly the stone pitching and stone studs in between RD. 74+800 to 76+500 of Khanwah Flood Bund were damaged. Protective measures to ensure the safety of flood bund, the work was taken up in hands on emergency basis. The work was executed on war footing basis to meet with the critical situation. Also the work was executed under the resident supervision of M/S NESPAK Consultants and the guidelines of Officers of the Irrigation Department from time to time. Alhamdulillah high flood was passed successfully. Furthermore, the restoration work will be executed during the current year i.e. 2022-23. Pre-Flood inspection team of Drainage & Flood Irrigation Zone and the team constituted by the Secretary Irrigation, Punjab have also visited the site.

FLOOD 2023

During Flood 2023, there was medium flood of discharge 419895 Cusecs dated: 28.07.2023 in River Indus and due to this medium flood, behavior of River Indus become active and it hits various structures like Concavo convex Guide wall at RD: 26+300 of 1-AR Minor, J-Head Spur RD: 34+500 of 1-AR Minor, Stone Studs RD: 15+000 of Khanwah Flood Bund, Stone Studs RD: 15+300 of Khanwah Flood Bund and Stone  Studs and apron between RD: 73+000 to 77+500 of Khanwah Flood Bund. Partially damages occurred at these structures. Furthermore, the restoration works will be completed before commencement of flood season 2024. Due to falling of discharge in River Indus, the river drifted towards left flank and started the erosive action along Khanwah Flood Bund from RD: 46+000 to 48+000. The erosive action with developing loop in between RD: 46+000 to 48+000 continued with its pace. The river action approached the top width and damaged partially the top portion as well as slopes of embankment. As there are thickly populated abadies of Basti Jhalar, Kundai, Laarki, Amano, Mouza Hassan Pur Buha, Police station Kundai, Govt. High Schools, Roads and other infrastructures belonging to Govt. as well as private has become in risk from river erosive action.

FLOOD 2024

During Flood 2024, there was medium flood of discharge 439179 Cusecs dated: 09.08.2024 in River Indus. Due to falling trend of flood water in River Indus emergency has been occurred at various structures like Concavo convex Guide wall at RD: 26+300 of 1-AR Minor, J-Head Spur RD: 34+500 of 1-AR Minor, Reach RD: 49+000 to 51+000 of Khanwah Flood Bund and Partially damages occurred at these structures. Furthermore, the restoration works are completed at site and now the situation is much better. Due to non implementation of recommendations of Model Study report IRR-1369 conducted during 2019, these damages are continously occuring every year. As a permanent solution and for the safety of the area, a request for inclusion in Annual Development Program (ADP) on emergent basis for the scheme “Providing River Training Works along Khanwah Flood Bund from RD: 0+000 to 78+500 to Save Abadies and Other Infrastructures in District Muzaffargarh” has also moved to Administrative Department. After construction of these structures, the area will hopefully be saved permanently.

CHAPTER.

4

Chapter. 4

DESIGN DATA, HISTORIC PEAK FLOOD DATA AND PREVIOUS FIVE YEAR FLOOD DATA OF HEAD WORKS /BARRAGES AND OTHER CONTROL POINTS

  - FLOOD LIMITS

INDUS RIVER.

For Taunsa Barrage the various flood stages of River Indus are given below.

Sr.	DISCHARGES.					FLOOD STAGES.

No.	Lac (Cusecs)

1.	below 2.50						Normal

2.	above 2.50 but below 3.75				Low

3.	above 3.75 but below 5.0				Medium

4.	above 5.0 but below 6.50				High

5.	Above 6.50 but below 8.0				Very High

6.	8.00 or above						Exceptionally High

  - TIME LAGS OF FLOODS

INDUS RIVER.

Sr. No	Name of Head Work				Distance in			Time in

K.M.				Hours

1.	Attack to Kala Bagh Head works			106				36

2.	Kala Bagh H/works to Chashma Barrage 		61				24

3.	Chashma Barrage to Taunsa Barrage			152				48

4.	Taunsa Barrage to Ghazi Ghat			40				12

5.	Ghazi Ghat to Mithan Kot				89				28

4.	Mithan Kot to Guddu Barrage			59				24

  - HIGHEST FLOODS

INDUS RIVER

The maximum discharge received down stream

Taunsa Barrage on 2-8-2010 					=  9,59,117 Cusecs

  - PEAK DISCHARGE.

INDUS RIVER  D/S TAUNSA BARRAGE

| YEAR | DATE | Discharge (in Cs) |
| --- | --- | --- |
| 2011 | 01-09-2011 | 227689 |
| 2012 | 10-08-2012 | 235389 |
| 2013 | 17-08-2013 | 468048 |
| 2014 | 18-07-2014 | 247412 |
| 2015 | 27-07-2015 | 604772 |
| 2016 | 08-07-2016 | 343024 |
| 2017 | 05-08-2017 | 413888 |
| 2018 | 17-08-2018 | 266123 |
| 2019 | 17-08-2019 | 373000 |
| 2020 | 06-09-2020 | 464560 |
| 2021 | 04-08-2021 | 304121 |
| 2022 | 30-08-2022 | 622095 |
| 2023 | 28-07-2023 | 419895 |
| 2024 | 09-08-2024 | 439179 |

CHAPTER.

5

CHAPTER. 5

FLOOD FIGHTING STRATEGY

During flood alert, all the vulnerable points will be kept under keen watching. If any damage occurs to any structure, matter will be brought to the notice of high ups of the department by the concerned S.D.O. In case of any damage, the Standard Operating Procedure (S.O.P’s) defined by the Administrative Department.

RIVER INDUS

STAGE-1

When the discharge of River Indus is bellow than 2.5 Lac Cs which is considered as Normal Flood, The Sub Divisional Officer, Jatoi Canal Sub Division will be In-charge of overall Flood operation and the Sub Engineer In-charge of their Sections will perform watching duties in their respective jurisdictions. No special watching is required.

STAGE-2

When the discharge of River Indus is between 2.5 Lac Cs to 3.75 Lac Cs which is considered as Low Flood, The Sub Divisional Officer, Jatoi Canal Sub Division will be In-charge of overall Flood operation and the Sub Engineer In-charge of their Sections will perform watching duties in their respective jurisdictions.

STAGE-3

When the discharge of River Indus is between 3.75 Lac Cs to 5.00 Lac Cs which is considered as Medium Flood, The Executive Engineer, Muzaffargarh Canal Division and the Sub Divisional Officer, Jatoi Canal Sub Division will be In-charge of overall Flood operation and the Sub Engineer In-charge of their Sections will perform watching duties in their respective jurisdictions.

STAGE-4

When discharge of River Indus is between 5.00 Lac Cs to 6.50 Lac Cs which is considered as High Flood, The Executive Engineer, Muzaffargarh Canal Division will be overall In-charge of Flood operation. In addition the Sub Divisional Officers, will be In-charge of watching operations of Flood Bunds along with supervisory staff and Sub Engineer concerned, along with their watching teams as listed in duty roster.

STAGE-5

When discharge of River Indus is between 6.50 Lac Cs to 8.00 Lac Cs which is considered as Very High Flood, The Executive Engineer, Muzaffargarh Canal Division will be overall In-charge of Flood operation. In addition the Sub Divisional Officers, will be In-charge of watching operations of Flood Bunds along with supervisory staff and Sub Engineer concerned, along with their watching teams as listed in duty roster.

STAGE-6

When the discharge is above 8.0 Lac Cs which is considered as Exceptionally High Flood, the Executive Engineer, Muzaffargarh Canal Division will be overall In-charge of Flood operation. The Sub Divisional officer Jatoi Canal Sub Division, along with supervisory staff of all the concerned Sub Engineers will also be responsible for the Flood operation.

Responsibility of Staff

(1) 	Executive Engineer, of Muzaffargarh Canals Division will be responsible for watch and ward of Muzaffargarh Canal Division. The Executive Engineer, will have an overall control over the flood preparedness and should have clarity in mind how to control and avert any grievous situation developing at any point around Muzaffargarh Canal Division. He should also apprise himself and alert other concerned about weather report and flood forecast.

(2)	Sub Divisional Officer’s along with their supervisory staff of all the concerned Sub Engineers will also be responsible for watch and ward of Flood operation.

LIAISON WITH OTHER DEPARTMENT

Executive Engineer Muzaffargarh Canal Division and Sub Divisional Officer will keep liaison with their counter parts in other departments. Higher authorities of other department will be approached through Superintending Engineer Muzaffargarh Canal Circle Multan and Chief Engineer Irrigation Zone D.G. Khan if any need arises. At site, Sub Engineers will conduct inspections with the visiting officers of Civil and Army if no higher officer is available on spot.

CHAPTER.

6

CHAPTER. 6

FLOOD DAMAGES RESTORATION WORKS

Damages to the following structures were occurred during flood 2023 & 2024 and detail of restoration work is as under:-

| Sr. No. | Name of Work | Status |
| --- | --- | --- |
| 1 | Restoration of J-Head Spur RD: 35+000 of 1-AR Minor damaged during flood 2024. | Estimate for restoration of structure has been prepared and submitted to NESPAK Consultants for checking. |
| 2 | Restoration of 03 No. studs at RD: 15+000, 15+300 & 15+650 of Khanwah Flood Bund damaged during flood 2024. | Baseline data has been observed and preparation of estimate is in progress. |
| 3 | Restoration of stone studs and apron between RD: 74+400 to 77+600 of Khanwah Flood Bund damaged during flood 2024. | Baseline data has been observed and preparation of estimate is in progress. |

CHAPTER.

7

CHAPTER. 7

FLOOD FIGHTING WATCHING ARRANGEMENTS

  - PRE-FLOOD ARRANGEMENTS

a.	Flood Warning: it is important that as soon as the Officer Incharge of canal head works knows about any probable rise in the River, he will initiate the necessary watch over embankment and regulate the weir and canal according to the rules sanctioned for each head works.

b.	During High Flood, when the River over flows the banks, expensive area along the River banks is liable to be inundated. It is therefore, necessary to warn civil officer to be on the alert and to be ready to evacuate villages likely to be inundated.

c.	For the reasons stated in paragraph above, flood warning are to be sent by higher officer to the Civil administration & other Irrigation officers. Flood warning will be issued by telegraph office to concerned officers.

d.	Between the dated October 01 and March 31 warning will be issued immediately when a River rises to “Low” flood level. The Executive Engineer Muzaffargarh Canal Division and Executive Engineer Dera Ghazi Khan will be intimated whenever the gauge at Attock is 10’ ft or above.

e.	For the period April 01 to September 30 warning will be issued only when a river reaches the “Medium” stage. No warning is issued for low flood during flood season.

SPECIAL FLOOD WARNING TO THE CHIEF ENGINEER

In addition to issuing of warning as described above, when the river rises to the high flood stage, warning by postal \ wireless will also be issued to Chief Engineer from Tarbella Dam, Chashma Barrage, Kala Bagh, Mangla, Marala, Trimmu and Taunsa Barrage.

ADDITIONAL INFORMATION

LATEST POSITION OF RESERVE STOCK OF STONE

| Sr # | Name of Structure | Reserve stock Limit Lac Cft | Minimum requirement 70% | Available at Site (In Lac Cft) | Balance Required (In Lac Cft) |
| --- | --- | --- | --- | --- | --- |
| 1 | J-Head Spur No. 1.-A at RD:127+000 of Jatoi Branch | 1.500 | 1.050 | 1.050 | 0.000 |
| 2 | J-Head Spur No. 1 at RD:135+000 of Jatoi Branch | 1.500 | 1.050 | 0.550 | 0.500 |
| 3 | J-Head Spur No. 2. at RD:139+000 of Jatoi Branch | 1.500 | 1.050 | 0.800 | 0.250 |
| 4 | J-Head Spur No. 3. At RD:2+000 Suleman Flood Bund | 1.500 | 1.050 | 1.050 | 0.000 |
| 5 | J-Head Spur No. 4. at RD: 8+400 Suleman Flood Bund | 1.500 | 1.050 | 1.050 | 0.000 |
| 6 | J-Head Spur No. 4-A at RD:9+500 Suleman Flood Bund | 1.500 | 1.050 | 1.050 | 0.000 |
| 7 | Flood Bund along right side of Jatoi Branch | 0.200 | 0.140 | 0.140 | 0.000 |
| 8 | Khanwah Flood Bund RD: 57+000 - 58+000 | 0.500 | 0.350 | 0.000 | 0.350 |
| 9 | Guide Spur near Mouza Murali | 1.050 | 0.735 | 0.544 | 0.191 |
| 10 | Stone Stud Basti Mustafa Abad | 1.050 | 0.735 | 0.000 | 0.735 |
| 11 | Stone Stud Warid Tower | 0.490 | 0.343 | 0.506 | 0.000 |
| 12 | Flood Bund along 1-AR Minor RD: 29+500 - 34+500 | 0.500 | 0.350 | 0.000 | 0.350 |
| 13 | Khanwah Flood Bund RD: 75+000 to 76+000 | 0.700 | 0.490 | 0.000 | 0.490 |
| 14 | Khanwah Flood Bund RD: 79+000 to 80+000 | 0.500 | 0.350 | 0.077 | 0.273 |
| 15 | J-Head Spur No. 5. RD: 7+500 Khan Wah Flood Bund | 1.500 | 1.050 | 1.049 | 0.001 |
| 16 | J-Head Spur No. 6. RD: 9+000 Khan Wah Flood Bund | 1.500 | 1.050 | 0.287 | 0.763 |
| 17 | J-Head Spur at RD: 34+500 of 1-AR Minor | 1.500 | 1.050 | 0.000 | 1.050 |
| 18 | Stone stud RD 12+000 Khan Wah Flood Bund | 0.500 | 0.350 | 0.343 | 0.007 |
| 19 | Stone stud RD 13+000 Khan Wah Flood Bund | 0.500 | 0.350 | 0.347 | 0.003 |
| 20 | Stone Stud RD 31+500 Khan Wah Flood Bund | 0.500 | 0.350 | 0.000 | 0.350 |
| 21 | Guide Spur No. 1 of 1-R Minor at RD:53+000 | 1.500 | 1.050 | 0.150 | 0.900 |
| 22 | Concavo convex Guide Wall at RD: 26+300 of 1-AR Minor | 1.500 | 1.050 | 0.068 | 0.982 |
| 23 | Khanwah Flood Bund RD: 15+500 to 18+000 | 0.530 | 0.371 | 0.000 | 0.371 |
| 24 | Khanwah Flood Bund RD: 18+000 to 21+000 | 0.180 | 0.126 | 0.000 | 0.126 |
| 25 | J-Head Spur at RD: 13+000 of Khanwah Flood Bund | 1.500 | 1.050 | 0.000 | 1.050 |
| 26 | Khanwah Flood Bund between RD: 48+000 to 49+000 | 1.500 | 1.050 | 0.000 | 1.050 |
|  | G. Total:- | 26.700 | 18.690 | 9.061 | 9.792 |

STATUS OF RESERVE STOCK STONE

| Total Sanctioned reserve stock limit. | Minimum requirement 70% | All ready Available Quantity. | Balance Quantity. | Remarks |
| --- | --- | --- | --- | --- |
| 26.700 | 18.690 | 9.061 | 9.792 | Tenders for procurement of Reserve Stock of stone for the quantity of 885571 Cft has been floated and approval of tender is in process. Quantity of stone will be procured on availability of funds before flood season 2025. |

  - WATCHING ESTABLISHMENT

| Site | Low Flood |  | Medium Flood |  | High Flood |  | Very High Flood |  | Exceptionally High Flood |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Men | Shift | Men | Shift | Men | Shift | Men | Shift | Men | Shift |
| Flood Bund along Soharu & Suleman Disty RD 0-12+000 | 0 | 0 | 5 | 3-Shift | 10 | 3-Shift | 15 | 3-Shift | 20 | 3-Shift |
| Khan Wah Flood RD 0-80+000 | 0 | 0 | 35 | 3-Shift | 50 | 3-Shift | 70 | 3-Shift | 100 | 3-Shift |
| Total:- |  |  | 40 |  | 60 |  | 85 |  | 120 |  |

Note:-	Payment of machinery and work charge employees subject to verification by Consultants.

The establishment for maintaining flood embankments will be employed                   on 01-06-2025 as 2 men per mile. Watching establishment will be employed keeping in view the depth of water above general N.S.L as described below.

- For 1 to 2ft depth of water along bunds 2 men per shift in three shifts of 8 hours durations.

- For 2 to 3ft and above depth of water along bund 3 men per mile per shift in three shifts of 8 hours durations.

  - ARRANGEMENT AT SENSITIVE SITES

- Khanwah Flood Bund reach RD: 14+000 to 18+000, Khanwah Flood Bund RD:49+000 to 53+000, Khanwah Flood Bund in reach RD: 70+000 to 78+500, Inverted Hockey Spur at RD:61+500 of 1-R Minor, Concavo convex Guide wall at RD: 26+300 of 1-AR Minor & J-Head Spur at RD: 35+000 of 1-AR Minor.

Emergency gang of 5 men per mile will be employed when the river rises beyond low flood mark and will be maintained till the end of the season. This strength will be sufficient for medium and high flood stages.  In case of very high and exceptionally high flood, this scale may be increased where-ever necessary by the Executive Engineer Muzaffargarh Division in consultation with Superintending Engineer Muzaffargarh Canal Circle Multan.

In case, where it is difficult to get Labour at the time of high discharge, it will be arranged through the civil authorities or contractors. A group of tractors with blade or dozers must be there as a standby arrangement for use in emergency during very high floods.

7.4	WATCHING MATERIAL

Flood watching material will be provided in ample quantity and to be readily available at every dangerous site.

The quantity of material required, depends upon the importance of the bund and the distance of the bund from the nearest store from which they can be collected. The following scale of material given here under is nearly an indication of relative quantities usually provided / needed. Material will be stored in each camp. Detail of material in each camp as under:-

| Sr. No. | Name of Item | Total Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Axes | 3 | 1 | 2 | Will be Purchased before coming of Flood Season 2025. |
| 2 | Bamboos 8' to 10' (for Lighting arrangements) | 12 | - | 12 |  |
| 3 | Baskets | 30 | 2 | 28 |  |
| 4 | Buckets | 10 | 2 | 8 |  |
| 5 | Cell for Torches | 30 | - | 30 |  |
| 6 | Chouldaries | 1 | - | 1 |  |
| 7 | E.G. Bags | 95 | 10 | 85 |  |
| 8 | Energy Savers | 2 & 5% extra | - | 2 & 5% extra |  |
| 9 | Folding Chair | 1 | - | 1 |  |
| 10 | Folding Table | 1 | - | 1 |  |
| 11 | Funnels | 1 | - | 1 |  |
| 12 | G. I. Buckets | 6 | 3 | 3 |  |
| 13 | Generator | 1 | - | 1 |  |
| 14 | Hand Saws | 1 | - | 1 | Will be Purchased before coming of Flood Season 2025. |
| 15 | Handle for Kassi/Axes | 6 | 2 | 4 |  |
| 16 | Kassi with Handles | 10 | 3 | 7 |  |
| 17 | Life Belt | 2 | -. | 2 |  |
| 18 | M. Oil | 3 | -. | 3 |  |
| 19 | Manila Rope | 50 | - | 50 |  |
| 20 | Munj/Patha/Trungers 9 Cft | 3500 | 1100 | 2400 |  |
| 21 | Munj/Patha/Trungers 3 Cft | 1600 | 600 | 1000 |  |
| 22 | Needles | 2 | - | 2 |  |
| 23 | Petrol | 1 | - | 1 |  |
| 24 | Rain Coats | 10 for each Generator | - | 10 for every Generato |  |
| 25 | Steel Charpoy | 3 | - | 3 |  |
| 26 | Sutli | 4 | - | 4 |  |
| 27 | Switch / Plugs | 1 | - | 1 |  |
| 28 | Torches | 5 | - | 5 |  |
| 29 | Umbrella | 1 for every Beldar and 10 % spare | - | 1 for every Beldar and 10 % spare |  |
| 30 | Wires 7/29 | 1 | - | 1 |  |
| 31 | Wooden Hammers | 1 | - | 1 |  |
| 32 | Steel Buckets | 2 | - | 2 |  |
| 33 | Water cooler | 3 | 1 | 2 |  |
| 34 | Cell for torches | 1 for every gang /camp | - | 1 for every gang /camp |  |
| 35 | Hammer 1 Lbs | 2 | - | 2 |  |
| 36 | Water Set Steel | 1 | - | 1 |  |
| 37 | Walky Talky | 1 for each Sub Engineer | - | 1 for each Sub Engineer |  |
| 38 | Pedestal Fan | 1 | - | 1 |  |
| 39 | Chargeable battery | 4 | - | 4 |  |
| 40 | Chargeable Torches | 6 | - | 6 |  |
| 41 | Khaji Mats | 6 | - | 6 |  |
| 42 | Flood Light | 2 | - | 2 |  |
| 43 | Wire Crates 3x3x3 = 27 Cft | 500 No | - | 500 No |  |

WAVE WASH.

- Lai / Pilchi longitudinal rolls should be used for reach effected by heavy wave wash.

NOTE

As for as possible lai / pilchi rolls should be laid through beldars.

(b)		Munj Ban			One Mound per mile.

I	Mallets wooden		5 Mallets with each Earth work Mistri /Mate

(d)		G.I Wire for Tarangar		One roll 14 gauge

LEAKAGE.

(a)		Gunny Bags	where H.F.L is lower than 6’ feet of N.S.L 50 Bags per mile.

Where H.F.L is higher than 6’ feet N.S.L 100 Bags per mile

(b)		Ban			20 Kg per mile for dangerous portions.

I		Sutli			One Lbs for 100 Bags

7.5 		ARRANGEMENT FOR SOUNDING & PROBING

For every Spur site and vulnerable serious location following arrangement will be made for probing.

S. No. 	  Item.						Period.

1.		1 Large boat with boatman		  01-06-2025 to 30-09-2025

2.		1 Small boat with boatman.			  -do-

3.		2 Helpers to boatmen.			 	  -do-

4.		2 Sounders per boat.		 		  -do-

  - LIGHTING ARRANGEMENT.

Sufficient stock of dry cell torches, lanterns and patronages etc will be kept available in the emergency store. Mobile Generating set will be arranged at vulnerable points. Sub Engineer concerned assisted by electricians will be responsible to keep these in working order.

  - RATION ARRANGEMENT

The Deputy Commissioner, Muzaffargarh and District food controller will make food arrangement. Deputy Commissioner, Muzaffargarh will be requested to keep a reserve stock of following articles at Muzaffargarh Canal Colony for supply to the labour if any flood emergency occurs.

- Atta 50 mounds. Per week.

- Gur, 20 mounds. Per week.

- Onion 10 mounds. Per week.

- Sugar 10 mounds. Per week.

- Rice 30 mounds. Per week.

  - P.O.L ARRANGEMENT FOR VEHICLE

Reserve stock of P.O.L for vehicle will be kept at Canal Colony Muzaffargarh on the following scale.

- Petrol		2500 Liters

- Diesel		1500   //

- K.Oil			1000   //

  - TRANSPORTATION

This Division has two No’s jeeps and one No. Toyota double Cabin. 03 No. jeeps and 1 No. Toyota double cabin shall be made available from Executive Engineer, Khangarh Canal Division Muzaffargarh. During extreme emergency, Tehsil Municipal officer Muzaffargarh/Jatoi /Ali Pur Deputy Superintendent Police and Deputy Commissioner Muzaffargarh will be requested to arrange vehicles as per requirements.

  - LAW AND ORDER.

Law and Order situation may arise due to occurrence of breach in any of the flood retaining embankment (God forbid). In such a situation Police will maintain the law and order situation.

  - MEDICAL ARRANGEMENT FOR LABOUR.

District health office, Muzaffargarh will set up a mobile dispensary on the different points along flood bunds with sufficient medical staff and medicines required against epidemics, snake bit, Choler etc. In case of any eventuality, a mobile dispensary will be kept on wheels at short notice. Executive Engineer Muzaffargarh Canal Division will inform to Civil Administration as well as District Health Officer in such a case.

  - LIAISON WITH OTHER DEPARTMENT

Executive Engineer Muzaffargarh Canal Division and Sub Divisional Officer’s, Jatoi Canal Sub Division, Jatoi and Muzaffargarh Canal Sub Division, Muzaffargarh will keep liaison with their counter parts in other departments. Higher authorities of other department will be approached through Superintending Engineer Muzaffargarh Canal Circle Multan and Chief Engineer Irrigation Zone D.G.Khan if any need arises. At site, Sub Engineers will conduct inspections with the visiting officers of Civil and Army if no higher officer is available on spot.

  - ROLE OF THE ARMY

In case of acute flood emergency, Executive Engineer Muzaffargarh Canal Division will issue SOS message to Deputy Commissioner, Muzaffargarh who will be requested to call the Army. The Deputy Commissioner, Muzaffargarh will be requested to call the Army for immediate help and rescue. It is also appropriate to ask for 100 Army men to be stationed at Jatoi as and when discharge in the River exceeds 8.0 Lac, Cusec.

  - DUTIES OF TELEPHONE ATTENDANT

Telephone and Telegraph office are located in office of Executive Engineer Muzaffargarh and in the Canal colony Muzaffargarh as well as in the others Canal Colonies. Telephone attendant and signalers will be available round the clock in order to collect and provide information regarding flood situation. Executive Engineer Muzaffargarh office is telephonically connected with telephone No. 066-9200223 /224.

| Name of Official | Period | Time |  |
| --- | --- | --- | --- |
|  |  | From | To |
| Qaiser Wazeer | 15-06-2025 To 15-10-2025 | 06:00 AM | 02:00 PM |
| M. Shahid |  | 02:00 PM | 10:00 PM |
| M. Waqas |  | 10:00 PM | 06:00 AM |

  - WIRELESS ARRANGEMENTS

There will be long-range wireless base set in Canal Telegraph office at Muzaffargarh during flood season. Only one mobile set is allowed by the Punjab Flood Commission for vehicle of Executive Engineer Muzaffargarh. There is a necessity of at least two more mobile sets for the vehicles of the Sub Divisional Officer Jatoi Canal Sub Division, Jatoi and Sub Divisional Officer, Muzaffargarh Canal Sub Division, Muzaffargarh. This mobile unit will help the officer while on tour to remain in touch with flood situation. Arrangement of new wireless system is required.

CHAPTER.

8

CHAPTER. 8

DETAIL OF ENCROACHMENT

The encroachment reported by the field staff in the jurisdiction of Muzaffargarh Canal Division has been submitted to the District Coordination Officer Muzaffargarh for taking necessary action against un-authorized occupation.

DETAIL OF NORMAL ENCROACHMENT

| Sr. No. | Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 160 | 30 | 130 | The Balance encroachments will be removed soon with the cooperation / Help of District Administration. |

DETAIL OF CRITICAL ENCROACHMENT

| Sr. No. | Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 32 | 12 | 20 | The Balance encroachments will be removed soon with the cooperation / Help of District Administration. |

DETAIL

| Sr. No. | Name of Bund | Nature of encroachments | Reach RD. | Status |
| --- | --- | --- | --- | --- |
| 1 | Khanwah flood bund | 85 No Houses 20 No Shops | 0+000 To 78+500 | 22 No. House removed. |
| 2 | Suleman flood bund | 35 No Houses 20 No Shops | 0+000 to 12+000 | 8 No. House removed. |
| Total |  | 160 No. |  | 30 No. |

CHAPTER.

9

CHAPTER. 9

DUTY ROASTER /FLOOD FIGHTING PROGRAM

KHAN WAH  FLOOD BUND

Vulnerable Reach  RD 14+000 – 18+000, 49+000 – 53+000 & RD 70+000 – 78+500 of Khanwah Flood Bund

Inverted Hockey Spur RD: 61+500 of 1-R Minor

Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor

J-Head Spur at RD: 35+000 of 1-AR Minor

ESTABLISHMENT.

Sectional Mistri					1. No each for Sub Engineer

Mate 						2.  No’s  per shift of 8 hours

Beldars						30. No’s per shift of 8 hours

Dozers						2. No RD 1800 Khan Wah Flood Bund

Contribution of Civil authorities.			Tehsil Municipal Officer M.Garh will arrange

Average 50. No Volunteers.

Police 	=	10 No Police men i.e 2 at each site to work for 8 hours.

Army	 =	100 Hundred Jawans will be requisitioned in extra emergency

DETAIL OF FLOOD FIGHTING/ RELIF CAMPS

| Sr. # | Name of embankment | Reach allocated to the camps | Incharge of Camp (Sub Engineer) | NAME OF SDOs |
| --- | --- | --- | --- | --- |
| 1 | Khanwah Flood Bund | 14+000 – 18+000 | Abdul Rehman 0300-7864992 | Dilshad Hussain 0300-4632019 |
| 2 | Khanwah Flood Bund | 49+000 – 53+000 | Abdul Rehman 0300-7864992 |  |
| 3 | Khanwah Flood Bund | 70+000 – 78+500 | Abdul Rehman 0300-7864992 |  |
| 4 | Inverted Hockey Spur RD: 61+500 of 1-R Minor | 61+500 | M. Bilal 0300-7122822 |  |
| 5 | Concavo Convex Guide wall at RD:26+300 of 1-AR Minor | 26+300 | Majid Rasol 0345-7527007 |  |
| 6 | J-Head Spur RD: 35+000 of 1-AR Minor | 35+000 | Majid Rasol 0345-7527007 |  |

CHAPTER.

10

CHAPTER. 10

EMERGENCY TELEPHONE No’s

| Sr. No. | Name of officer | Office | Residence/ Cell No |
| --- | --- | --- | --- |
| IRRIGATION DEPARTMENT |  |  |  |
| 1 | Secretary Irrigation Department Lahore | 042-99212117 042-99212118 Fax #. 042-99212116 |  |
| 2 | Chief Engineer Irrigation Zone D.G Khan | 064/9260245 064-9260251 Fax:064-9260252 | 064-9260539 |
| 3 | Chief Engineer D&F Lahore. | 042-99230602 042-99230368 Fax= 042-35862081 |  |
| 4 | Director Flood Lahore. | 042-99231614 |  |
| 5 | Flood Emergency Cell Lahore. | 042-99205371-72 | --- |
| 6 | Incharge Flood Relief Cell Board of Revenue Lahore | 042-99204409 | --- |
| 7 | Superintending Engineer, Muzaffargarh Canal Circle, Multan. | Phone= 061-9200090 Fax = 061-9200088 | 0307-4081390 |
| 8 | Executive Engineer, Muzaffargarh Canal Division, Muzaffargarh. | 066/9200223 Fax. 066/9200245 | 0301-3277500 |
| 9 | Emergency Flood Control Centre Executive Engineer Office, Muzaffargarh | Phone=066-9200223 Fax=066-9200245 | --- |
| CIVIL DEPARTMENT |  |  |  |
| 10 | Deputy Commissioner Office, Muzaffargarh | 066-9200264 | 0669200259 0669200260 |
| 11 | Assistant Commissioner, Sadar Muzaffargarh | --- | 0333-6114689 |
| 12 | Assistant Commissioner, Jatoi | --- | 0300-4274885 |
| 13 | Assistant Commissioner, Alipur | --- | 0346-7143133 |
| 14 | D.S.P Muzaffargarh | 066-9200322 | 0300-7518331 |
| 15 | D.S.P Jatoi | 066-2591451 | 0300-6362955 |
| 16 | D.S.P Alipur | 066-2901478 | 0307-8660000 |
| 17 | District Emergency Control Centre Muzaffargarh | 066-9200260 | 066-9200256 |
| 18 | Sub-Divisional Emergency Control Centre Muzaffargarh (T.M.A. Muzaffargarh) | 066-9200274 |  |
| 19 | Sub-Divisional Emergency Control Centre Muzaffargarh (T.M.A. Alipur) | 066-2700824 |  |
| 20 | Sub-Divisional Emergency Control Centre Muzaffargarh (T.M.A. Jatoi) | 066-2391277 |  |
| 21 | Emergency Flood Control Centre Deputy Commissioner Office Muzaffargarh | 066-9200251 066-9200252-253 | 9200254 |
| 22 | D.P.O District Police Office | 066-9200311 |  |
| 23 | Wireless Control Center S.P Office Muzaffargarh | 066-9200321 |  |
| 24 | District Health Officer Muzaffargarh. | 066-9200239 |  |
| 25 | Medical Superintendent Muzaffargarh D.H.Q Hospital Muzaffargarh. | 066-9200226 |  |
| 26 | Executive Engineer, Provincial Highway Muzaffargarh | 066-2422459 |  |
|  | ARMY |  |  |
| 27 | Military Officer Head Quarter Engineers 2 Corps Multan Cantt. | 061-5313388 061-5313389 |  |
| 28 | Military Officer, Head Quarter Engineers 31-Corps Bahawalpur Cantt. | 0621/891/2353/2360 |  |

CHAPTER.

11

CHAPTER. 11

STANDARD OPERATING PROCEDURE (SOP) FOR BREACHING SECTIONS

(There is no any breaching section in the jurisdiction of Muzaffargarh Canal Division, Muzaffargarh).

  - HISTORY OF THE BREACHING SECTION

Nil

  - LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION.

Nil

QUANTITY AND VARITY OF THE EXPLOSION

STATUS OF EXPLOSIVE / ACCESSRIES

Nil

  - ARRANGEMENT OF EXPLOSIVE AND SECURITY OF EXPLOSIVE STORES

Nil

  - LIST OF THE SECURITY STAFF ALONGWITH DETAIL OF THEIR TRAINING ETC

Nil

  - DETAIL OF MACHINERY MEANS AS STANDBY ARRANGEMENTS IN CASE OF DETONATION FAILURE.

Nil

  - DUTY ROASTER IN CASE OF CRITICAL SITUATION

Nil

  - BREACHING COMMITTEE WITH THEIR ACTION PLAN

Nil

  - LIST OF THE VILLAGES LIKELY TO BE INUNDATED IN CASE OF BREACH

Nil

  - ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENTS

Nil

  - DETAILS OF COORDINATION WITH CIVIL /ARMY AUTHORITIES.

Nil

  - PARALLEL COMMUNICATION ARRANGEMENTS

Nil

  - INDEX PLAN

Nil

P A R T.

B

CHAPTER.

12

Chapter. 12

VULNERABLE SITES ON FLOOD BUNDS /STRUCTURES

  - APPREHENDED BREACHES IN FLOOD BUNDS /STRUCTURES

| Sr # | Name of Vulnerable site | Reach RD: | Length Ft. | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Khanwah Flood Bund | 14+000 – 18+000 | 4000 | Extra ordinary vigilance / watching will be done on all the vulnerable reaches from low flood limits. |
| 2 | Khanwah Flood Bund | 49+000 – 53+000 | 4000 |  |
| 3 | Khanwah Flood Bund | 70+000 – 78+500 | 8500 |  |
| 4 | Inverted Hockey Sour RD: 61+500 of 1-R Minor | 61+500 | 1000 |  |
| 5 | Concavo Convex Guide wall at RD:26+300 of 1-AR Minor | 26+300 | 1000 |  |
| 6 | J-Head Spur RD: 35+000 of 1-AR Minor | 35+000 | 1000 |  |

  - OPERATION OF BREACHING SECTION

No Breaching section exists in Jurisdiction of Muzaffargarh Canal Division, Muzaffargarh.

  - BREACHES DUE TO RISE OF FLOOD WATER, DETERIORATION OF FLOOD BUNDS.

Nil

CHAPTER.

13

# EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITE

SITE.1, 2 & 3

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan showing rout of flood water coming out of the breach from reach start

Khanwah Flood Bund

RD: 49+000 – 53+000

Basti Jhalan, Laarki

Khanwah Flood Bund

RD: 14+000 – 18+000

Basti Gabbar Arain

Khanwah Flood Bund

RD: 70+000 – 78+500

Basti Sarki, Draishak

Inverted Hockey Spur at RD: 15+650 of Khanwah Flood Bund.

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN

Khanwah Flood Bund between RD: 14+000 – 18+000

Khanwah Flood Bund between RD: 49+000 – 53+000

Khanwah Flood Bund between RD: 70+000 – 78+500

Basti Gabbar Arain, Jhalan, Laarki, Sarki & Basti Draishak

If any mishap occurs at this vulnerable, the flood water will inundate the above abadies / structures and the water will come back into the River near Khanwah Flood Bund in the area of Basti Gabbar Arain, Jhalan, Laarki, Sarki & Basti Draishak and the above abadies / structures, the flooded water will go to the confluence point of River Indus and Chenab (Plan attached herewith).

| Sr # | Name of Flood Bund | Reach RD | Length (ft) | Villages to be affected |
| --- | --- | --- | --- | --- |
| 1 | Khanwah Flood Bund | 14+000 – 18+000 | 4000 | Basti Gabbar Arain |
| 2 | Khanwah Flood Bund | 49+000 – 53+000 | 4000 | Basti Jhalan, Basti, Laarki |
| 3 | Khanwah Flood Bund | 70+000 – 78+500 | 8500 | Basti Sarki, Basti Draishak |

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

Quick strategy will be adopted according to the flood situation.

13.3.1	ARRANGEMENT.

In case of any emergency

1.	Aatta 10 Mounds per week of emergency period

2.	Ghee 05 Mounds per week of emergency period

3.	Onions 05 Mounds per week of emergency period

4.	Sugar 05 Mounds per week of emergency period

5.	Rice 15 Mounds per week of emergency period

EMERGENCY GANG

At important and dangers sites, emergency gang in strength of 20 Beldars with one or 2 mates will be maintained from the time the river rises above the prescribed gauges till it finally drops below that gauge. These gang will remain deployed on bunds and will shift to portions share lakes slips or wave action giving trouble for emergent action as deemed necessary In addition to the above, the Executive Engineer will employ additional labours as and when required in emergency under intimation to the superiors.

The Deputy Commissioner Muzaffargarh will be requested to enlistable bodies persons living in the adjoining areas for theirs engagement in case of increased requirements.

13.3.2      ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

FLOOD CAMP

Khanwah Flood Bund between RD: 14+000 – 18+000.

Khanwah Flood Bund between RD: 49+000 – 53+000

Khanwah Flood Bund between RD: 70+000 – 78+500

13.3.3 		DUTIES OF OFFICERS/OFFICIALS AND THEIR CAMPS

| Section | Location | Officer | Official |
| --- | --- | --- | --- |
| Bara Section | 14+000 – 18+000, 49+000 – 53+000 & 70+000 – 78+500 Khanwah Flood Bund | Dilshad Hussain Sub Divisional Canal Officer Jatoi Canal Sub Division. Mob # 0300-4632019 | Abdul Rehman (Sub Engineer) 0300-7864992 3 No. Mans / Shift of 8 Hours. |

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

Nill

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCES

In case of emergency Private Machinery can be made available locally through Government Contractor.

- Excavator

- Dozer

- Tractor with trolley

- Tractor with Blade with Hall

- Truck / Dalla

- Vehicles.

    - FLOOD FIGHTING MATERIAL REQUIRED

| Sr. No | Item | Unit | Total Required Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Axes | No. | 3 | Will be Purchased as per Requirement in Flood Season 2025. |
| 2 | Bamboos 8’ to 10’ (for Lighting arrangements) | No. | 12 |  |
| 3 | Baskets | No. | 30 |  |
| 4 | Buckets | No. | 10 |  |
| 5 | Cell for Torches | No. | 30 |  |
| 6 | Chouldaries | No. | 1 |  |
| 7 | E.G. Bags | No. | 95 |  |
| 8 | Energy Savers | No. | 2 & 5% extra |  |
| 9 | Folding Chair | Set | 1 |  |
| 10 | Folding Table | No. | 1 |  |
| 11 | Funnels | No. | 1 |  |
| 12 | G. I. Buckets | No. | 6 |  |
| 13 | Generator | No. | 1 |  |
| 14 | Hand Saws | No. | 1 |  |
| 15 | Handle for Kassi/Axes | No. | 6 |  |
| 16 | Kassi with Handles | No. | 10 |  |
| 17 | Life Belt | No. | 2 |  |
| 18 | M. Oil | Liter / Day | 3 |  |
| 19 | Manila Rope | Feet | 50 |  |
| 20 | Munj/Patha/Trungers 9 Cft | No. | 3500 |  |
| 21 | Munj/Patha/Trungers 3 Cft | No. | 1600 |  |
| 22 | Needles | No. | 2 |  |
| 23 | Patromax | No. | 1 |  |
| 24 | Petrol | Liter / Day | 10 for each Generator |  |
| 25 | Rain Coats | No. | 3 |  |
| 26 | Steel Charpoy | No. | 4 |  |
| 27 | Sutli | Lbs | 1 |  |
| 28 | Switch / Plugs | No. | 5 |  |
| 29 | Torches | No. | 1 for every Beldar and 10 % spare |  |
| 30 | Umbrella | No. | 1 |  |
| 31 | Wires 7/29 | Coil | 1 |  |
| 32 | Wooden Hammers | No. | 2 |  |
| 33 | Steel Buckets | No. | 3 |  |
| 34 | Water cooler | No. | 1 for every gang /camp |  |
| 35 | Hammer 1 Lbs | No. | 2 |  |
| 36 | Water Set Steel | Set | 1 |  |
| 37 | Walky Talky | No. | 1 for each Sub Engineer |  |
| 38 | Pedestal Fan | No. | 1 |  |
| 39 | Chargeable battery | No. | 4 |  |
| 40 | Chargeable Torches | No. | 6 |  |
| 41 | Khaji Mats | No. | 6 |  |
| 42 | Flood Light | No. | 2 |  |
| 43 | Wire Crates 3x3x3 = 27 Cft | No. | 500 |  |

13.3.7 FLOOD FIGHTING MATERIAL AVAILABLE

The following flood fighting material is available for pre-flood arrangements    and the required short items will be purchased before coming flood 2025.

| Sr. No | Name of Item | Unit | Total Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes | No. | 3 | 1 | 2 | Will be Purchased as per Requirement in Flood Season 2025. |
| 2 | Bamboos 8’ to 10’ | No. | 12 | - | 12 |  |
| 3 | Baskets | No. | 30 | 2 | 28 |  |
| 4 | Buckets | No. | 10 | 2 | 8 |  |
| 5 | Cell for Torches | No. | 30 | - | 30 |  |
| 6 | Chouldaries | No. | 1 | - | 1 |  |
| 7 | E.G. Bags | No. | 95 | 10 | 85 |  |
| 8 | Energy Savers | No. | 2 & 5% extra | - | 2 & 5% extra |  |
| 9 | Folding Chair | Set | 1 | - | 1 |  |
| 10 | Folding Table | No. | 1 | - | 1 |  |
| 11 | Funnels | No. | 1 | - | 1 |  |
| 12 | G. I. Buckets | No. | 6 | 2 | 4 |  |
| 13 | Generator | No. | 1 | - | 1 |  |
| 14 | Hand Saws | No. | 1 | - | 1 |  |
| 15 | Handle for Kassi/Axes | No. | 6 | 1 | 5 |  |
| 16 | Kassi with Handles | No. | 10 | 2 | 8 |  |
| 17 | Life Belt | No. | 2 | -. | 2 |  |
| 18 | M. Oil | Liter / Day | 3 | -. | 3 |  |
| 19 | Manila Rope | Feet | 50 | - | 50 |  |
| 20 | Munj/Patha/Trungers 9 Cft | No. | 3500 | 500 | 3000 |  |
| 21 | Munj/Patha/Trungers 3 Cft | No. | 1600 | 300 | 1300 |  |
| 22 | Needles | No. | 2 | - | 2 |  |
| 23 | Patromax | No. | 1 | - | 1 |  |
| 24 | Petrol | Liter / Day | 10 for each Generator | - | 10 for every Generato |  |
| 25 | Rain Coats | No. | 3 | - | 3 |  |
| 26 | Steel Charpoy | No. | 4 | 1 | 3 |  |
| 27 | Sutli | Lbs | 1 | - | 1 |  |
| 28 | Switch / Plugs | No. | 5 | - | 5 |  |
| 29 | Torches | No. | 1 for every Beldar and 10 % spare | - | 1 for every Beldar and 10 % spare |  |
| 30 | Umbrella | No. | 1 | - | 1 |  |
| 31 | Wires 7/29 | Coil | 1 | - | 1 |  |
| 32 | Wooden Hammers | No. | 2 | - | 2 |  |
| 33 | Steel Buckets | No. | 3 | - | 3 |  |
| 34 | Water cooler | No. | 1 for every gang /camp | - | 1 for every gang /camp |  |
| 35 | Hammer 1 Lbs | No. | 2 | - | 2 |  |
| 36 | Water Set Steel | Set | 1 | - | 1 |  |
| 37 | Walky Talky | No. | 1 for each Sub Engineer | - | 1 for each Sub Engineer |  |
| 38 | Pedestal Fan | No. | 1 | - | 1 |  |
| 39 | Chargeable battery | No. | 4 | 1 | 3 |  |
| 40 | Chargeable Torches | No. | 6 | 1 | 5 |  |
| 41 | Khaji Mats | No. | 6 | 1 | 5 |  |
| 42 | Flood Light | No. | 2 | - | 2 |  |
| 43 | Wire Crates 3x3x3 = 27 Cft | No. | 500 | - | 500 |  |

SITE.4

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan showing rout of flood water coming out of the breach from reach start

Inverted Hockey Spur at RD: 61+500 of 1-R Minor.

Inverted Hockey Spur RD:61+500 of 1-R Minor.

Mouza & Basti  Rampur-II

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN

Inverted Hockey Spur at RD:61+500 of 1-R Minor

Mouza & Basti Rampur-II and other allied abadies.

If any mishap occurs at this vulnerable, the flood water will inundate the above abadies / structures and the water will come back into the River near Inverted Hockey Spur at RD:61+500 of 1-R Minor in the area of Basti Ram Pur and the above abadies / structures, the flooded water will go to the confluence point of Sarki and enter into the River Chenab (Plan attached herewith).

| Sr # | Name of Flood Bund | Reach RD | Length (ft) | Villages to be affected |
| --- | --- | --- | --- | --- |
| 1 | Inverted Hockey Spur at RD: 61+500 of 1-R Minor | 61+500 | 1000 | Mouza & Basti Rampur-II |

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

Quick strategy will be adopted according to the flood situation.

13.3.1	ARRANGEMENT.

In case of any emergency

1.	Aatta 10 Mounds per week of emergency period

2.	Ghee 05 Mounds per week of emergency period

3.	Onions 05 Mounds per week of emergency period

4.	Sugar 05 Mounds per week of emergency period

5.	Rice 15 Mounds per week of emergency period

EMERGENCY GANGE

At important and dangers sites, emergency gang in strength of 20 Beldars with one or 2 mates will be maintained from the time the river rises above the prescribed gauges till it finally drops below that gauge. These gang will remain deployed on bunds and will shift to portions share lakes slips or wave action giving trouble for emergent action as deemed necessary In addition to the above, the Executive Engineer will employ additional labours as and when required in emergency under intimation to the superiors.

The Deputy Commissioner Muzaffargarh will be requested to enlistable bodies persons living in the adjoining areas for theirs engagement in case of increased requirements.

13.3.2      ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

FLOOD CAMP

Inverted Hockey Spur at RD: 61+500 of 1-R Minor

    - DUTIES OF OFFICERS/OFFICIALS AND THEIR CAMPS

| Section | Location | Officer | Official |
| --- | --- | --- | --- |
| Jatoi Section | Inverted Hockey Spur at RD:61+500 of 1-R Minor. | Dilshad Hussain Sub Divisional Canal Officer Jatoi Canal Sub Division. Mob # 0300-4632019 | M. Bilal (Sub Engineer), 0300-7122822 3 No. Mans / Shift of 8 Hours. |

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

Nill

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCES

In case of emergency Private Machinery can be made available locally through Government Contractor.

    - Excavator

    - Dozer

    - Tractor with trolley

    - Tractor with Blade with Hall

    - Truck / Dalla

    - Vehicles.

    - FLOOD FIGHTING MATERIAL REQUIRED

| Sr. No | Item | Unit | Total Required Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Axes | No. | 3 | Will be Purchased as per Requirement in Flood Season 2025. |
| 2 | Bamboos 8’ to 10’ (for Lighting arrangements) | No. | 12 |  |
| 3 | Baskets | No. | 30 |  |
| 4 | Buckets | No. | 10 |  |
| 5 | Cell for Torches | No. | 30 |  |
| 6 | Chouldaries | No. | 1 |  |
| 7 | E.G. Bags | No. | 95 |  |
| 8 | Energy Savers | No. | 2 & 5% extra |  |
| 9 | Folding Chair | Set | 1 |  |
| 10 | Folding Table | No. | 1 |  |
| 11 | Funnels | No. | 1 |  |
| 12 | G. I. Buckets | No. | 6 |  |
| 13 | Generator | No. | 1 |  |
| 14 | Hand Saws | No. | 1 |  |
| 15 | Handle for Kassi/Axes | No. | 6 |  |
| 16 | Kassi with Handles | No. | 10 |  |
| 17 | Life Belt | No. | 2 |  |
| 18 | M. Oil | Liter / Day | 3 |  |
| 19 | Manila Rope | Feet | 50 |  |
| 20 | Munj/Patha/Trungers 9 Cft | No. | 3500 |  |
| 21 | Munj/Patha/Trungers 3 Cft | No. | 1600 |  |
| 22 | Needles | No. | 2 |  |
| 23 | Patromax | No. | 1 |  |
| 24 | Petrol | Liter / Day | 10 for each Generator |  |
| 25 | Rain Coats | No. | 3 |  |
| 26 | Steel Charpoy | No. | 4 |  |
| 27 | Sutli | Lbs | 1 |  |
| 28 | Switch / Plugs | No. | 5 |  |
| 29 | Torches | No. | 1 for every Beldar and 10 % spare |  |
| 30 | Umbrella | No. | 1 |  |
| 31 | Wires 7/29 | Coil | 1 |  |
| 32 | Wooden Hammers | No. | 2 |  |
| 33 | Steel Buckets | No. | 3 |  |
| 34 | Water cooler | No. | 1 for every gang /camp |  |
| 35 | Hammer 1 Lbs | No. | 2 |  |
| 36 | Water Set Steel | Set | 1 |  |
| 37 | Walky Talky | No. | 1 for each Sub Engineer |  |
| 38 | Pedestal Fan | No. | 1 |  |
| 39 | Chargeable battery | No. | 4 |  |
| 40 | Chargeable Torches | No. | 6 |  |
| 41 | Khaji Mats | No. | 6 |  |
| 42 | Flood Light | No. | 2 |  |
| 43 | Wire Crates 3x3x3 = 27 Cft | No. | 500 |  |

13.3.7 FLOOD FIGHTING MATERIAL AVAILABLE

The following flood fighting material is available for pre-flood arrangements    and the required short items will be purchased before coming flood 2025.

| Sr. No | Name of Item | Unit | Total Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes | No. | 3 | 1 | 2 | Will be Purchased as per Requirement in Flood Season 2025. |
| 2 | Bamboos 8’ to 10’ | No. | 12 | - | 12 |  |
| 3 | Baskets | No. | 30 | 2 | 28 |  |
| 4 | Buckets | No. | 10 | 2 | 8 |  |
| 5 | Cell for Torches | No. | 30 | - | 30 |  |
| 6 | Chouldaries | No. | 1 | - | 1 |  |
| 7 | E.G. Bags | No. | 95 | 10 | 85 |  |
| 8 | Energy Savers | No. | 2 & 5% extra | - | 2 & 5% extra |  |
| 9 | Folding Chair | Set | 1 | - | 1 |  |
| 10 | Folding Table | No. | 1 | - | 1 |  |
| 11 | Funnels | No. | 1 | - | 1 |  |
| 12 | G. I. Buckets | No. | 6 | 3 | 3 |  |
| 13 | Generator | No. | 1 | - | 1 |  |
| 14 | Hand Saws | No. | 1 | - | 1 |  |
| 15 | Handle for Kassi/Axes | No. | 6 | 2 | 4 |  |
| 16 | Kassi with Handles | No. | 10 | 3 | 7 |  |
| 17 | Life Belt | No. | 2 | -. | 2 |  |
| 18 | M. Oil | Liter / Day | 3 | -. | 3 |  |
| 19 | Manila Rope | Feet | 50 | - | 50 |  |
| 20 | Munj/Patha/Trungers 9 Cft | No. | 3500 | 1100 | 2400 |  |
| 21 | Munj/Patha/Trungers 3 Cft | No. | 1600 | 600 | 1000 |  |
| 22 | Needles | No. | 2 | - | 2 |  |
| 23 | Patromax | No. | 1 | - | 1 |  |
| 24 | Petrol | Liter / Day | 10 for each Generator | - | 10 for every Generato |  |
| 25 | Rain Coats | No. | 3 | - | 3 |  |
| 26 | Steel Charpoy | No. | 4 | - | 4 |  |
| 27 | Sutli | Lbs | 1 | - | 1 |  |
| 28 | Switch / Plugs | No. | 5 | - | 5 |  |
| 29 | Torches | No. | 1 for every Beldar and 10 % spare | - | 1 for every Beldar and 10 % spare |  |
| 30 | Umbrella | No. | 1 | - | 1 |  |
| 31 | Wires 7/29 | Coil | 1 | - | 1 |  |
| 32 | Wooden Hammers | No. | 2 | - | 2 |  |
| 33 | Steel Buckets | No. | 3 | 1 | 2 |  |
| 34 | Water cooler | No. | 1 for every gang /camp | - | 1 for every gang /camp |  |
| 35 | Hammer 1 Lbs | No. | 2 | - | 2 |  |
| 36 | Water Set Steel | Set | 1 | - | 1 |  |
| 37 | Walky Talky | No. | 1 for each Sub Engineer | - | 1 for each Sub Engineer |  |
| 38 | Pedestal Fan | No. | 1 | - | 1 |  |
| 39 | Chargeable battery | No. | 4 | - | 4 |  |
| 40 | Chargeable Torches | No. | 6 | - | 6 |  |
| 41 | Khaji Mats | No. | 6 | - | 6 |  |
| 42 | Flood Light | No. | 2 | - | 2 |  |
| 43 | Wire Crates 3x3x3 = 27 Cft | No. | 500 | - | 500 |  |

SITE.5 & 6

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan showing rout of flood water coming out of the breach from reach start

Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor

J-Head Spur at RD: 35+000 of 1-AR Minor

1-AR Minor between RD:26+000 to 42+000.

Basti Baloach, Najwani, Tufail, Laskani etc

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN

Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor

J-Head Spur at RD: 35+000 of 1-AR Minor

Basti Baloach, Najwani, Tufail, Laskani etc

If any mishap occurs at this vulnerable, the flood water will inundate the above abadies / structures and the water will come back into the River near 1-AR Minor between RD:26+000 to 42+000 in the area of Basti Baloach, Najwani, Tufail, Laskani etc and the above abadies / structures, the flooded water will go to the confluence point of Sarki and enter into the River Chenab (Plan attached herewith).

| Sr # | Name of Flood Bund | Reach RD | Length (ft) | Villages to be affected |
| --- | --- | --- | --- | --- |
| 1 | Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor | 26+300 | 1000 | Basti Baloach, Najwani etc |
| 2 | J-Head Spur at RD: 35+000 of 1-AR Minor | 35+000 | 1000 | Basti Tufail, Laskani etc |

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE.

Quick strategy will be adopted according to the flood situation.

13.3.1	ARRANGEMENT.

In case of any emergency

1.	Aatta 10 Mounds per week of emergency period

2.	Ghee 05 Mounds per week of emergency period

3.	Onions 05 Mounds per week of emergency period

4.	Sugar 05 Mounds per week of emergency period

5.	Rice 15 Mounds per week of emergency period

EMERGENCY GANGE

At important and dangers sites, emergency gang in strength of 20 Beldars with one or 2 mates will be maintained from the time the river rises above the prescribed gauges till it finally drops below that gauge. These gang will remain deployed on bunds and will shift to portions share lakes slips or wave action giving trouble for emergent action as deemed necessary In addition to the above, the Executive Engineer will employ additional labours as and when required in emergency under intimation to the superiors.

The Deputy Commissioner Muzaffargarh will be requested to enlistable bodies persons living in the adjoining areas for theirs engagement in case of increased requirements.

13.3.2      ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

FLOOD CAMP

J-Head Spur RD: 34+500 of 1-AR Minor

    - DUTIES OF OFFICERS/OFFICIALS AND THEIR CAMPS

| Section | Location | Officer | Official |
| --- | --- | --- | --- |
| Bakaini Section | 1-AR Minor between RD: 26+000 – 42+000 | Dilshad Hussain Sub Divisional Canal Officer Jatoi Canal Sub Division. Mob # 0300-4632019 | Majid Rasool (Sub Engineer) 0345-7527007 3 No. Mans / Shift of 8 Hours. |

13.3.4 DEPARTMENTAL MACHINERY AVAILABLE

Nill

13.3.5 MACHINERY AVAILABLE FROM PRIVATE SOURCES

In case of emergency Private Machinery can be made available locally through Government Contractor.

- Excavator

- Dozer

- Tractor with trolley

- Tractor with Blade with Hall

- Truck / Dalla

- Vehicles.

    - FLOOD FIGHTING MATERIAL REQUIRED

| Sr. No | Item | Unit | Total Required Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Axes | No. | 3 | Will be Purchased as per Requirement in Flood Season 2025. |
| 2 | Bamboos 8’ to 10’ (for Lighting arrangements) | No. | 12 |  |
| 3 | Baskets | No. | 30 |  |
| 4 | Buckets | No. | 10 |  |
| 5 | Cell for Torches | No. | 30 |  |
| 6 | Chouldaries | No. | 1 |  |
| 7 | E.G. Bags | No. | 95 |  |
| 8 | Energy Savers | No. | 2 & 5% extra |  |
| 9 | Folding Chair | Set | 1 |  |
| 10 | Folding Table | No. | 1 |  |
| 11 | Funnels | No. | 1 |  |
| 12 | G. I. Buckets | No. | 6 |  |
| 13 | Generator | No. | 1 |  |
| 14 | Hand Saws | No. | 1 |  |
| 15 | Handle for Kassi/Axes | No. | 6 |  |
| 16 | Kassi with Handles | No. | 10 |  |
| 17 | Life Belt | No. | 2 |  |
| 18 | M. Oil | Liter / Day | 3 |  |
| 19 | Manila Rope | Feet | 50 |  |
| 20 | Munj/Patha/Trungers 9 Cft | No. | 3500 |  |
| 21 | Munj/Patha/Trungers 3 Cft | No. | 1600 |  |
| 22 | Needles | No. | 2 |  |
| 23 | Patromax | No. | 1 |  |
| 24 | Petrol | Liter / Day | 10 for each Generator |  |
| 25 | Rain Coats | No. | 3 |  |
| 26 | Steel Charpoy | No. | 4 |  |
| 27 | Sutli | Lbs | 1 |  |
| 28 | Switch / Plugs | No. | 5 |  |
| 29 | Torches | No. | 1 for every Beldar and 10 % spare |  |
| 30 | Umbrella | No. | 1 |  |
| 31 | Wires 7/29 | Coil | 1 |  |
| 32 | Wooden Hammers | No. | 2 |  |
| 33 | Steel Buckets | No. | 3 |  |
| 34 | Water cooler | No. | 1 for every gang /camp |  |
| 35 | Hammer 1 Lbs | No. | 2 |  |
| 36 | Water Set Steel | Set | 1 |  |
| 37 | Walky Talky | No. | 1 for each Sub Engineer |  |
| 38 | Pedestal Fan | No. | 1 |  |
| 39 | Chargeable battery | No. | 4 |  |
| 40 | Chargeable Torches | No. | 6 |  |
| 41 | Khaji Mats | No. | 6 |  |
| 42 | Flood Light | No. | 2 |  |
| 43 | Wire Crates 3x3x3 = 27 Cft | No. | 500 |  |

13.3.7 FLOOD FIGHTING MATERIAL AVAILABLE

The following flood fighting material is available for pre-flood arrangements    and the required short items will be purchased before coming flood 2025.

| Sr. No | Name of Item | Unit | Total Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes | No. | 3 | - | 3 | Will be Purchased as per Requirement in Flood Season 2025. |
| 2 | Bamboos 8’ to 10’ | No. | 12 | 2 | 10 |  |
| 3 | Baskets | No. | 30 | - | 30 |  |
| 4 | Buckets | No. | 10 | - | 10 |  |
| 5 | Cell for Torches | No. | 30 | 5 | 25 |  |
| 6 | Chouldaries | No. | 1 | - | 1 |  |
| 7 | E.G. Bags | No. | 95 | 15 | 80 |  |
| 8 | Energy Savers | No. | 2 & 5% extra | - | 2 & 5% extra |  |
| 9 | Folding Chair | Set | 1 | - | 1 |  |
| 10 | Folding Table | No. | 1 | - | 1 |  |
| 11 | Funnels | No. | 1 | - | 1 |  |
| 12 | G. I. Buckets | No. | 6 | - | 6 |  |
| 13 | Generator | No. | 1 | - | 1 |  |
| 14 | Hand Saws | No. | 1 | - | 1 |  |
| 15 | Handle for Kassi/Axes | No. | 6 | - | 6 |  |
| 16 | Kassi with Handles | No. | 10 | 2 | 8 |  |
| 17 | Life Belt | No. | 2 | -. | 2 |  |
| 18 | M. Oil | Liter / Day | 3 | -. | 3 |  |
| 19 | Manila Rope | Feet | 50 | - | 50 |  |
| 20 | Munj/Patha/Trungers 9 Cft | No. | 3500 | 1100 | 2400 |  |
| 21 | Munj/Patha/Trungers 3 Cft | No. | 1600 | 600 | 1000 |  |
| 22 | Needles | No. | 2 | - | 2 |  |
| 23 | Patromax | No. | 1 | - | 1 |  |
| 24 | Petrol | Liter / Day | 10 for each Generator | - | 10 for every Generator |  |
| 25 | Rain Coats | No. | 3 | - | 3 |  |
| 26 | Steel Charpoy | No. | 4 | - | 4 |  |
| 27 | Sutli | Lbs | 1 | - | 1 |  |
| 28 | Switch / Plugs | No. | 5 | 1 | 4 |  |
| 29 | Torches | No. | 1 for every Beldar and 10 % spare | - | 1 for every Beldar and 10 % spare |  |
| 30 | Umbrella | No. | 1 | - | 1 |  |
| 31 | Wires 7/29 | Coil | 1 | - | 1 |  |
| 32 | Wooden Hammers | No. | 2 | 1 | 1 |  |
| 33 | Steel Buckets | No. | 3 | - | 3 |  |
| 34 | Water cooler | No. | 1 for every gang /camp | - | 1 for every gang /camp |  |
| 35 | Hammer 1 Lbs | No. | 2 | - | 2 |  |
| 36 | Water Set Steel | Set | 1 | - | 1 |  |
| 37 | Walky Talky | No. | 1 for each Sub Engineer | - | 1 for each Sub Engineer |  |
| 38 | Pedestal Fan | No. | 1 | - | 1 |  |
| 39 | Chargeable battery | No. | 4 | - | 4 |  |
| 40 | Chargeable Torches | No. | 6 | - | 6 |  |
| 41 | Khaji Mats | No. | 6 | - | 6 |  |
| 42 | Flood Light | No. | 2 | - | 2 |  |
| 43 | Wire Crates 3x3x3 = 27 Cft | No. | 500 | 20 | 480 |  |

DETAIL

Main Camps at offices of the Sub Divisional Officer Jatoi Canal Sub Division.

| Sr. No | Site of Camp | Section | Establishment per shift of 8 hours |  |
| --- | --- | --- | --- | --- |
|  |  |  | (Man) |  |
| JATOI CANAL SUB DIVISION |  |  |  |  |
| 1 | Khanwah Flood Bund RD: 14+000 to 18+000 | Bara | 3 |  |
| 2 | Khanwah Flood Bund RD: 49+000 to 53+000 | Bara | 3 |  |
| 3 | Khanwah Flood Bund RD: 70+000 to 78+500 | Bara | 3 |  |
| 4 | Inverted Hockey Spur RD: 61+500 of 1-R Minor | Jatoi | 3 |  |
| 5 | J-Head Spur RD: 35+000 of 1-AR Minor | Bakaini | 3 |  |
| 6 | Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor | Bakaini | 3 |  |

DUTIES OF OFFICERS/OFFICIALS AND THEIR CAMPS

| S.# | Name of Flood Bund/Structure | Sub Divisional Officer | Sub Engineer | Mate |
| --- | --- | --- | --- | --- |
| a. | Khanwah Flood Bund RD: 14+000 to 18+000 | Dilshad Hussain | Abdul Rehman | Abdul Sattar |
| b. | Khanwah Flood Bund RD: 49+000 to 53+000 |  | Abdul Rehman | Abdul Sattar |
| c. | Khanwah Flood Bund RD: 70+000 to 78+500 |  | Abdul Rehman | M. Saleem |
| d. | Inverted Hockey Spur RD: 61+500 of 1-R Minor |  | M. Bilal | M. Ahsan |
| e. | J-Head Spur RD: 35+000 of 1-AR Minor |  | Majid Rasool | M. Aazam |
| f. | Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor |  | Majid Rasool | M. Tahir |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDINGS, CANALS AND DRAINAGE NETWORK.

Electric line Gas pipe line, Telephone Line and Roads are passing, other building i.e Petrol Pumps, Cotton Factories and Rice Factories are also present in surrounding localities.

Muzaffargarh Canal Jatoi Branch and its system located along left side of River Indus.

CANALS

a.	1-R Minor, 2-R Minor, Sony Disty and Jatoi Branch.

b.	Nabi Pur Minor, Mithan Wali Minor, Yare Wali Minor Ghiri Minor

c.	Nabi Pur Minor Sultan Pur Minor

ROADS

a.	Shah Jamal to Jatoi Road.

b.	Ali Pur to Khair Pur, Khair Pur to Gabar Arrain, Ali Pur to Seet Pur.

c.	Seet Pur to Sultan Pur, Sultan Pur to Nabi Pur, Seet Pur to Khangarh Doma

d	Khangarh Doma to Sarki Road.

BACK UP DIVISION

River Diversion Division at Muzaffargarh & Khangarh Canal Division at Muzaffargarh will act as back up Division in case of emergency.

# CHAPTER

# 14

# ACTION PLAN

14.1	RE-SHUFFLING/RECOUPING PLAN OF RESERVE STONE DEPARTMENTALLY

By deploying Private Contractor for re-shuffling/recouping plan of Reserve Stone in case of any emergency.

14.2	DETAIL OF INLET/OUTLET CROSSING ALONG WITH CLOSING METHODOLOGY.

| Sr. No. | Name of Embankment / Bund | Reach | Length (ft) | Category (A/B/C) | Outlet crossing RD's |
| --- | --- | --- | --- | --- | --- |
| RIVER INDUS |  |  |  |  |  |
| 1 | Right Side of Jatoi Branch | RD:0-140000 | 140000 | B | RD:127-128, 132-133 (Total = 2 No.) |
| 2 | Suleman Flood Bund | RD:0-12000 | 12000 | B | N I L. |
| 3 | Khan Wah Flood Bund | RD 0-78+500 | 78500 | A | RD: 20-21, 26-27, 28-29, 29-30, 30-31, 33-34, 36-37, 37-38, 39-40, 41-42, 44-45, 45-46, 48-49, 50-51 (Total = 14 No.) |

14.3	DEVELOPMENT MACHINERY (MEDIUM TO HIGH FLOOD)

14.4     DEVELOPMENT MACHINERY (HIGH TO VERY HIGH FLOOD)

14.5     DEVELOPMENT MACHINERY (HIGH TO EXCEPTIONALLY HIGH FLOOD)

DEPLOYMENT PLAN OF MACHINERY AT FLOOD EMBANKMENTS DURING FLOOD 2025.

| Sr.# | Name of Embankment | Length/ Width |  | Kilo Meter | Requipment of Machinery Medium to High Flood Limit |  |  |  |  | Requipment of Machinery High to Very High Flood Limit |  |  |  | Requipment of Machinery Very High to Exceptionally Flood Limit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Length (ft) | Width (ft) |  | Excavator | Front Blade Tractor | Dozers | Dump Trucks | Tractor Trolleys | Excavator | Dozers | Dump Trucks | Tractor Trolleys | Excavator | Dozers | Dump Trucks | Tractor Trolleys |
| 1 | Khanwah Flood Bund RD: 14+000 to 18+000 | 4000 | 25 | 1.22 | 1 | 1 | 0 | 2 | 4 | 1 | 1 | 2 | 6 | 3 | 1 | 4 | 10 |
| 2 | Khanwah Flood Bund RD: 49+000 to 53+000 | 4000 | 25 | 1.22 | 1 | 1 | 0 | 2 | 4 | 1 | 1 | 2 | 6 | 3 | 1 | 4 | 10 |
| 3 | Khanwah Flood Bund RD: 70+000 to 78+500 | 8500 | 25 | 2.59 | 1 | 1 | 0 | 2 | 4 | 1 | 1 | 2 | 6 | 3 | 1 | 4 | 10 |
| 4 | Inverted Hockey Spur RD: 61+500 of 1-R Minor | 1000 | 30 | 0.30 | 1 | 1 | 0 | 2 | 4 | 1 | 1 | 2 | 6 | 3 | 1 | 4 | 10 |
| 5 | Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor | 1000 | 25 | 0.30 | 1 | 1 | 0 | 2 | 4 | 1 | 1 | 2 | 6 | 3 | 1 | 4 | 10 |
| 6 | J-Head Spur RD: 35+000 of 1-AR Minor | 1000 | 30 | 0.30 | 1 | 1 | 0 | 2 | 4 | 1 | 1 | 2 | 6 | 3 | 1 | 4 | 10 |
|  | Total |  |  |  | 6 | 6 | 0 | 12 | 24 | 6 | 6 | 12 | 36 | 18 | 6 | 24 | 60 |

Note:	 To be updated as per critical site situation. Payment for machinery and work charge employees subject to verification by TPM / Consultants.

14.6	POLICE DEPLOYMENT PLAN

| Sr No | Vulnerable sites used for illegal cuts during floods | Concerned Canal Division | Concerned Police station | Police persons deployed |  | Remarks (if any) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Inspector /SI/ ASI | Constable |  |
| 1 | Khanwah Flood Bund RD: 14+000 – 18+000 | Muzaffargarh Canal Division | Seet Pur | 1 No | 3 No |  |
| 2 | Khanwah Flood Bund RD: 49+000 – 53+000 | Muzaffargarh Canal Division | Kundai | 1 No | 3 No |  |
| 3 | Khanwah Flood Bund RD: 70+000 – 78+500 | Muzaffargarh Canal Division | Kundai | 1 No | 3 No |  |
| 4 | Inverted Hockey Sour RD: 61+500 of 1-R Minor | Muzaffargarh Canal Division | Jatoi | 1 No | 3 No |  |
| 5 | J-Head Spur RD: 35+000 of 1-AR Minor | Muzaffargarh Canal Division | Bait Meer Hazar | 1 No | 3 No |  |
| 6 | Concavo Convex Guide Wall at RD: 26+300 of 1-AR Minor | Muzaffargarh Canal Division | Bait Meer Hazar | 1 No | 3 No |  |

14.7	DETAILS OF SYNTHETIC BAGS WITH CAPACITY OF 500KG AND 1000KG.

Funds are awaited on receipt of funds synthetic bags with capacity of 50 kg and 100 kg will be recouped.

| Sr. No. | Capacity | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 50 Kg | 500 No. | - | 500 No. | Will be procured before upcoming flood season 2025. |
| 2 | 100 Kg | 500 No. | - | 500 No. |  |

14.8	DETAILS OF POLYTHENE SHEETS OF BLACK COLOR TO PROTECT UPSTREAM SLOPE AGAINST WAVE ACTION AND TO CONTROL SEEPAGE THROUGH EMBANKMENTS.

Funds are awaited on receipt of funds polythene sheets of black color to protect upstream slope against wave action and to control seepage through embankments.

| Sr. No. | Required Quantity | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 5000 Square Feet | - | 5000 Square Feet | Will be procured before upcoming flood season 2025. |

CHAPTER

15

BACKUP DIVISIONS

In case of emergency River Diversion Division & Khangarh Canal Division at Muzaffargarh will be back-up division.

Superintending Engineer

Muzaffargarh Canal Circle

Multan

Chief Engineer

D.G. Khan Irrigation Zone

Dera Ghazi khan

Executive Engineer

Muzaffargarh Canal Division

Muzaffargarh

FOLLOW UP ACTIONS ON FLOOD PREPAREDNESS ACTIVITES FOR THE YEAR 2025

| Sr. No. | Irrigation Zone | Preparation of Flood Fighting Plan | Field Inspection (Certificates) | Removal of Encroachments | Installation/ repair/ painting of Gauges, RD Marks, Data Boards | Reserve Stock of Stone | Inspection Notes | Status of Breaching Sections | Status of Explosive Stores | Security of Explosive Stores | Notification of Flood Emergency Officer | Purchase of Flood Fighting Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 1 | D.G. Khan | Flood Fighting Plan submitted to Chief Engineer D&F for vetting vide XEN M.Garh No.354/68-W dated:07.02.2025. | Certified that I have been inspecting the Flood Bund and found satisfactory. Necessary repair is in progress. | The list of encroachments has already been submitted to the DCO/ DPO, Muzaffargarh for removal. | The work of Repairing and painting of gauges, RD Marks and Data Boards will be completed upto 30.06.2025. | Reserve Stock Limit submitted to Chief Office for Approval. | I have been inspected the flood bunds, necessary repair is in progress. | No breaching section existing in jurisdiction of this division. | The explosive store is available at Head Quarter Engineer 21 Battalion Core Abdul Hakeem. | Responsibility of 2 Core Abdul Hakeem. | Dilshad Hussain SDO Jatoi is notified as flood emergency Officer / Focal Person | Sufficient Flood fighting material is available as per provision in the flood fighting plan short item will be purchase soon. |