GOVERNMENT OF THE PUNJAB

IRRIGATION DEPARTMENT

FLOOD FIGHTING PLAN

FOR THE YEAR 2025

CHAKBANDI DIVISION LAHORE

LINK CIRCLE 						LAHORE IRRIGATION LAHORE						 	ZONE, LAHORE

FLOOD FIGHTING PLANS-2025

I N D E X

PART –A

| CHAPTER. # | Description | Page # |
| --- | --- | --- |
| 1 | Salient Feature of the concerned Division. 1.1 Location 1.2 General Description - Administrative setup | 4-5 6 |
| 2 | Flood Protection and River Training Works 2.1 Design parameter of Training Works 2.2 Location Map | 7-8 9 |
| 3 | Brief History of past flood events | 10 |
| 4 | Design Data, Historic Peak flood data and pervious five years flood data of Head works/ Barrages and or other control points. 4.1 Flood Limit 4.2 Time lags of Floods 4.3 Highest Floods 4.4 Peak Discharge. | 11 12 |
| 5 | Flood Fighting Strategy | 13 |
| 6 | Flood Damages Restoration Works | 14 |
| 7 | Flood fighting &Watching Arrangement. 7.1 Pre-Flood Arrangement 7.2 Watching Establishment 7.3 Arrangement at sensitive sites 7.4 Watching material. 7.5 Arrangement for sounding & Probing 7.6 Lighting Arrangement 7.7 Ration Arrangement 7.8 P.O.L Arrangement for Vehicle. 7.9 Transportation 7.10 Law and order. 7.11 Medical Arrangement for Labour. 7.12Liaison with other Department 7.13 Role of the Army 7.14 Duties of Telephone Attendant. 7.15 Wireless Arrangements. | 15 16 17-18 19 |
| 8 | Detail of Encroachment | 20 |
| 9 | Duty roaster/Flood Fighting Program | 21-22 |
| 10 | Emergency Telephone Nos. | 23-24 |
| 11 | Standard operating procedure (SOP) for Breaching Sections. 11.1 History of the breaching Section 11.2 Location, Design, Quantity and variety of the explosive required for detonation. 11.3 Arrangement of explosives and security of explosive stores 11.4 List of Security staff along with detail of their training etc . 11.5 Detail of mechanical means as a standby arrangements in case of detonation failure 11.6 Duty Roster in case of critical situation 11.7 Breaching Committee with their action plan 11.8 List of the villages likely to be inundated in case of breach . 11.9 Announcement and detail of evacuation arrangements. 11.10 Detail of coordination with Civil /Army Authorities. 11.11 Parallel communication arrangements. 11.12 Index plan | 25-26 27 |

PART –B

| Chapter. # | Description | Page # |
| --- | --- | --- |
|  | EMERGENCY CONTINGENCY PLAN (PART-B OF FLOOD FIGHTNG PLAN) | 28-29 |
| 12 | FLOOD PREPAREDNESS ACTIVITIES FOR MOONSOON SEASON 2025 OF CHAKBANDI DIVISION LAHORE. 12.1 Apprehended breaches in Flood bunds/Structures. 12.2 Removal Of Critical Encroachment. 12.3 Protection Infrastructures , Embankments Etc For Flood Season 2025. 12.4 Clearance Of Waterways /Bridges / Culverts / Road Crossing On The Rivers Nullah And Drains. 12.5 Repairing damages and paintings of guage and RD marks. 12.6 operation of breaching sections. 12.7 breaches due to rising of flood water, deterioration of flood bunds etc. 12.8 certificate regarding health of flood protection infrastructure & river training works. 12.9 certificate regarding flood preparedness 2025. chakbandi division lahore 12.10 certificate regarding removal of encroachment from water ways and flood plain of chakbandi division lahore. 1 | 30-33 |
| 13 | Emergency Contingency Plan for the vulnerable site (Sites) listed above .Emergency contingency Plan must include:- 13.1 Plan showing route of flood water coming out of the breach Support with levels. 13.2 Detail of villages abides likely to be effected and this should also be shown on the plan. 13.3. Strategy and action taken be explained in detail. This may include: Detail of villages abides likely to be affected and this should also be shown on the plan. - Arrangements. - Establishment of flood fighting camps. 13.3.3 Duties of officers/Officials and their camp sites. - Departmental Machinery available. - Machinery available from Private source. 13.3.6 Flood Fighting Material required. 13.3.7 Flood fighting Material available. 13.4 Detail of other infrastructure like electric, Sui Gas, telephone installations, road network, other building, canals and drainage networks. | 34 35 36 37-38 39-40 |
| 14 | Action Plan 14.1- Re-Shuffling / Recouping Plan of Reserve Stock Stone Departmentally. 14.2 Detail of inlet/outlet crosses along with closing methodology. 14.3 Deployment Machinery (Medium to High Flood). 14.4 Deployment Machinery ( High to very High Flood). 14.5 Deployment Machinery ( Very High to Exceptionally High Flood). 14.6 Police Deployment Plan. 14.7 Detail of synthetic Bags with Capacity of 500 ). 14.8 Detail of synthetic Bags with Capacity of 1000 kg). 14.9 Detail of Polythene Sheets of Black color to Protect U/S Slopes against wave action and to control seepage through embankments. | 40-54 40 41-42 43-44 45-46 47 48 49 |
| 15 | Back Up Division( In Case of breach) | 50 |

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION, LAHORE

CHAPTER – 1

SALIENT FEATURES OF THE CHAKBANDI DIVISION.

  - LOCATION:

Chakbandi Division, Lahore of Link Canal Circle, Lahore has its head quarter at Canal Bank Road Dharampura, Lahore. It is located in Irrigation Department Building Complex situated along left bank of Lahore Branch Canal between RD 261+000 to 265+000, Tehsil Cantt., and District Lahore.

  - GENERAL DESCRIPTION:

Bambanwala Ravi Bedian Depalpur (B.R.B.D) Link canal off takes from Upper Chenab Canal near Bombanwala Canal Rest House 4 miles south west of Daska town. The upper reach of this canal from RD 0 to 178+000 falls in the jurisdiction of Pasrur Link Division Sialkot. The reach below RD 178+000 to 434+000 is in the jurisdiction of Chakbandi Division Lahore and 435+000 to RD. 537+546 (tail) falls in the jurisdiction of Kasur Division.

Previously it was Rayya branch, which was meant to provide irrigation facility to the lands of Sialkot, Gujranwala and Lahore Districts. After independence, it was decided to convert Rayya branch into B.R.B.D Link so that lands across Ravi river may not get dry due to possible stoppage of canal water from upper Bari Doab Canal System (an off take of Madhopur Head works on River Ravi) and Upper Depalpur Canal system (an off take of Ferozepur Head works on River Sutlej).

At the time of planning of this canal, in addition to irrigation facility, another target aimed at was to utilize this canal as first defense line along Indo Pak border. Keeping in view multifold purpose, the alignment of this canal had to be adopted against the natural country slope. As a result, this canal crosses Ravi River through a Syphon. Resultantly, this canal in reach RD. 205+000 to 283+000 is flowing parallel to Ravi River and left bank of canal remains prone to river threats during floods. Left Bank of BRBD Link serves as flood bund from RD. 178+000 to 298+000 i.e. a length of 23 canal miles.

In order to save the canal from River meandering action and flood attacks, a battery of 8 number spurs at R.D 204+805, 215+000, 224+500, 232+000, 241+000, 250+100, 259+000 and

265+000 on left side of B.R.B.D Link have been constructed to keep the River spills away from canal. Similarly a Guide wall spur at R.D 288+500 left of B.R.B.D Link was also constructed for the same purpose. The position of this net work is marked on the plan as Annex-1.

The most important structure on this canal from operation and defense point of view is the Ravi Syphon where B.R.B.D Link crosses Ravi River through this structure which comprises of 5 # R.C.C. barrels of 10.25ˈ x 10.25ˈ each. This huge structure can be considered as the “life line of BRBD canal” and is of vital importance. The position of allied works of this structure is also marked on the plan, training works attached.

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

  - ADMINISTRATIVE SETUP

Administrative setup of Chakbandi Division Lahore is described below.

Chakrali Section

RD. 178+000 to 204+805

Executive Engineer

Chakbandi Division

Lahore

Sub Divisional Officer,

Link Sub Division

At Lahore

RD. 178+000 to 260+000

Sub Divisional Officer,

Ravi Syphon Sub Division

At Batapur, Lahore

RD. 260+000 to 337+144

Sub Divisional Officer,

Bedian Sub Division

At Bedian

RD. 337+144 to 435+000

Longwala Section

RD. 230+000 to 260+000

Pakhiala Section

RD. 260+000 to 282+750

Mechanical Section

RD. 300+750 to 338+000

Malikpur Section

RD. 337+144 to 373+000

Barki Section

RD. 373+000 to 401+000

Bedian Section

RD. 401+000 to 435+000

Ravi Syphon Section

RD. 282+750 to 300+750

Mandiali Section

RD. 204+805 to 230+000

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

Chapter No 2

FLOOD PROTECTION AND RIVER TRAINING WORKS

2.1  DESIGN PARAMETERS OF RIVER TRAINING WORKS.

| S.# | RD Of Spur/ Bund | Length | Top R.L | Free Board | Top Width | Side Slope | Depth of Stone | Depth of Spawl | Width of Apron | Depth of Apron | Year of Construction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Mole Head Spur at RD 205+000 | 3000’ | 741.52 | 2.0’ | 30.0’ | 3:1 | 1.30’ | 0.70 | 43.30’ | 5.0’ | 1980 |
| 2 | J Head Spur at RD 215+000 | 4150’ | 743.27 | 6.0’ | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1980 |
| 3 | J Head Spur at RD 224+500 | 6450’ | 740.83 | 6.0 | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1980 |
| 4 | Mole Head Spur at RD 232+000 | 2800’ | 738.50 | 6.0’ | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1980 |
| 5 | Mole Head at RD241+000 | 3550’ | 736.09 | 6.0’ | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1980 |
| 6 | J Head Spur at R.D250+000 | 4200’ | 737 | 8.2 | 30.0 | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1975 |
| 7 | Mole Head Spur at RD 259+000 | 3700’ | 736.49 | 10.0 | 30.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1980 |
| 8 | J Head Spur at RD:265+000 | 7500’ | 733.49 | 8.85’ | 30.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1975 |
| 9 | R.D 215+000/L Extension of spur | 3920’ | 744.25 | 6.0’ | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1994 |
| 10 | Prong R.D 4450/R of spur R.D 224+500/L | 4500’ | 740.83’ | 6.0’ | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1980 |
| 11 | Prong R.D 2000/R of spur R.D 250+100/L | 1000’ | 742.15’ | 6.0’ | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1980 |
| 12 | Prong R.D 6200/R of spur R.D 265+100/L | 1250’ | 741.39’ | 6.0’ | 25.0’ | 3:1 | 1.30’ | 0.70’ | 45.0’ | 5.0’ | 1975 |
| 13 | R/G bund of Ravi Syphon RD 283+000 | 2900’ | 727.0’ | 5.0’ | 30/60 | 2:1/3:1 | 1.30’ | 1.0’ | 85/46’ | 6.0’ | 1952 |

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

| S.# | RD Of Spur/ Bund | Length | Top R.L | Free Board | Top Width | Side Slope | Depth of Stone | Depth of Spawl | Width of Apron | Depth of Apron | Year of Construction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | L/G bund of Ravi Syphon RD 283+000 | 2900’ | 727.0’ | 5.0’ | 30/60 | 2:1/ 3:1 | 1.30’ | 1.0’ | 85/46’ | 6.0’ | 1952 |
| 15 | Shank bund of Ravi Syphon R.D 279+500/L | 5500’ | 727.0 | 5.0 | 30.0’ | 3:1 | 1.30’ | 1.0’ | 20.0’ | 6.0’ | 1952 |
| 16 | Escape Bund R.D 279+500/R | 5625’ | 722.0 | 6.5 | 20.0’ | 2:1 | 1.30’ | 1.0’ | 20.0’ | - | 1952 |
| 17 | J-spur RD 288+500/L | 1350’ | 727.0 | 5.0’ | 25.0’ | 3:1 | 1.30’ | 1.0’ | 45.0’ | 5.5’ | 1988 |
| 18 | Guide Wall Spur at R.d 1400 of J Head Spur R.D 288+500/L | 3422’ | 727.0’ | 5.0’ | 25.0’ | 3:1 | 1.00’ | 1.00’ | 50.0’ | 5.00’ | 1998 |
| 19 | Mole Head Diversion Bund opposite RD 288+500 up stream Ravi Syphon | 1350’ | 716.50 | 3.0’ | 25.0’ | 3:1 /2:1 | 1.00’ | 1.00’ | 25.0’ | 5.00’ | 1973 |
| 20 | Mole Head Extension Bund at RD 150 of Diversion Bund at Ravi post. | 2360’ | 716.50 | 3.0’ | 25.0’ | 3:1 /2:1 | 1.00’ | 1.00’ | 25.0’ | 5.00’ | 1973 |

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

Chapter-3

BRIEF HISTORY OF PAST FLOOD EVENTS

River Ravi’s discharge in Chakbandi Division is observed at Ravi Syphon RD. 283-285 of BRBD Link Canal which was constructed in 1952.

Highest discharge of 600000 Cs. was observed at this site in the year 1955. Other major floods occurred in the Ravi during years 1988, 1993, 1995, 1996, 1997 at discharges of 325000, 128188 Cs, 257000 Cs, 196080 Cs and 159200 Cs respectively. BRBD Link Canal was breached on numerous times at different locations. Besides, J-Head Spur RD. 224+500/L of BRBD Link Canal washed during flood 1995. During flood season 2013, River action also caused damage to flood infrastructure at Ravi Observation Post opposite RD 288+500/L of BRBD Link Canal. Besides, River morphology has abruptly changed in its reach from RD 200+000 to RD 203+000/L to Ravi Syphon RD 283-285 of BRBD Link Canal. River Ravi is drifting rightwards and approaching towards BRBD Link Canal. As a result, flood protection works are to be added opposite RD 188+000/L, RD 215+000/L, RD 288+500/L. Besides during flood 2013, River water flowed parallel to BRBD Link canal from RD 291+000 to 300+500/L. Outer slope of BRBD Link canal from RD 288+000/L to RD 291+000 has already been stone pitched. This pitching is required to be extended up to RD. 300+000.

There are no major damages occurred previous five year of history of flood under the jurisdiction of this Division, however during last year flood water damaged the Ravi Post near Ravi Syphon. The flood water by passed this structure from upstream side of the mole head, over toped stud No1 & 2, formed sever vortex /eddies and substantially damaging the structure of Ravi observation Post. The bund was saved from the bypassing and the branching through hectic flood fighting.

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

CHAPTER-4

DESIGN DATA, HISTORIC PEAK FLOOD DATA AND PERVIOUS FIVE YEAR.FLOOD DATA OF HEAD WORKS / BARRAGES AND OR OTHER CONTROL POINTS.

  - FLOOD LIMITS

The Flood limits of Ravi River at Ravi Syphon are explained below:

RAVI AT RAVI SYPHON

| LOW | MEDIUM | HIGH | VERY HIGH | EXCEPTIONAL HIGH |
| --- | --- | --- | --- | --- |
| 40000 Cs To 65000 Cs | 65000 Cs To 90000 Cs | 90000 Cs To 135000 Cs | 135000 Cs To 180000 Cs | 180000 Cs and Above |

4.2 	TIME LAGS OF FLOODS

| River | Site |  | Distance in Km | Time Lag Hours |
| --- | --- | --- | --- | --- |
|  | From | To |  |  |
| RAVI | Hajipur Gujran | Jassar | 94 | 12 |
|  | Jassar | Ravi Syphon | 73 | 18 |
|  | Ravi Syphon | Shahdara | 24 | 8 |

Sub Divisional Officer 							Sub Divisional Officer

Link Sub Division 							Ravi Syphon Sub Division

Lahore 									Lahore

Executive Engineer

Chakbandi Division

Lahore.

  - HIGHEST FLOODS

Highest Flood in Ravi River.

| Sr. # | Highest Floods | Discharge at Ravi Syphon |
| --- | --- | --- |
| 1. | 1955 | 600000 Cs |

4.4	PEAK DISCHARGES

The Peak Flood discharges of Ravi River during the last 25 years at the Ravi Syphon gauging

Point / station are described below.

| RAVI SYPHON |  |  |  |
| --- | --- | --- | --- |
| YEAR | DATED | GAUGE | DISCHARGE |
| 1950 | 25-08.1950 | 716.00 | 305000 |
| 1955 | 28-07-1955 | 722.00 | 600000 |
| 1973 | 05-09-1973 | 713.00 | 201500 |
| 1975 | 18-09-1975 | 711.90 | 107200 |
| 1976 | 22-08-1976 | 710.70 | 108200 |
| 1980 | 28-09-1980 | 707.60 | 105468 |
| 1988 | 27-09-1988 | 715.70 | 325040 |
| 1993 | 13-07-1993 | 706.70 | 128188 |
| 1994 | 22-07-1994 | 706.20 | 101791 |
| 1995 | 06-09-1995 | 709.80 | 257000 |
| 1996 | 25-08-1996 | 708.40 | 196080 |
| 1997 | 30-08-1997 | 707.70 | 159200 |
| 2007 | 30-06-2007 | 703.40 | 35200 |
| 2011 | 10-08-2011 | 702.20 | 39800 |
| 2012 | 22-08-2012 | 702.30 | 40112 |
| 2013 | 18-08-2013 | 705.20 | 73551 |
| 2014 | 08-09-2014 | 693.60 | 93250 |
| 2015 | 24-09-2015 | 700.90 | 39214 |
| 2016 | 29-07-2016 | 701.45 | 45081 |
| 2017 | 02-08-2017 | 701.60 | 37297 |
| 2018 | 15-08-2018 | 700.70 | 37936 |
| 2019 | 19-08-2019 | 701.20 | 37936 |
| 2020 | 28-08-2020 | 700.70 | 34531 |
| 2021 | 14-07-2021 | 701.10 | 37228 |
| 2022 | 17-08-2022 | 700.00 | 29890 |
| 2023 | 29-07-2023 | 698.70 | 38800 |

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

CHAPTER – 5

FLOOD FIGHTING STRATEGY

- LOW FLOOD LEVEL STRATEGY:- (40000 Cs to 65000 Cs)

In Case of Low Flood, Flood Camp will be established and watching of Flood Embankments and its allied structures will be started.

- MEDIUM FLOOD LEVEL STRATEGY:- (65000 Cs to 90000 Cs)

In case of medium Flood, if erosion on any structure occurs Flood fighting will be started with the help of field establishment.

- HEIGH FLOOD LEVEL STRATEGY:- (90000 Cs to 135000 Cs)

In case of high flood, action will be taken according to instructions of high-ups and also it will be timely reported to “Provincial Disaster Management Authority, C&W Department, and Civil Administration Department for taking necessary action.

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

CHAPTER – 6

FLOOD DAMAGES / RESTORATION WORKS

The following Flood Restoration Works are in pipeline under FPSP-III.

- Rehabilitation of escape channel into River Ravi off-taking RD 279+500/R of BRBD Link Canal from RD 0+000 to RD 5+800.

- Protection from erosive action along mole head extension bund & Diversion Bund upstream Ravi Syphon and Raising of Mole Head diversion Bund, Mole Head extension Bund and Tie Bund Highest Flood Level.

- Protection of infrastructure , Abadies and agriculture land of Village Jajogill and Baryar from erosion cause by River Ravi Downstream outfall RD 313+000 M.R Link Canal.

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

CHAPTER – 7

FLOOD FIGHTING / WATCHING ARRANGEMENT

7.1	PRE-FLOOD ARRANAGEMENT

The detail of watching materials required at various flood camps along BRBD has been explained below. The deficiency of material has also been worked out. Efforts are being made to fill the gap between requisite quantity and short quantity.

STATUS OF RESERVE STOCK OF STONE.

| S.No | Type of Structure with Location. | Sanctioned Reserve Stock of Stone (Lac Cft) | Minimum Required (Lac Cft) (70%) | Available Quantity (Lac Cft) | Balance Quantity Requirement @ 100% (Lac Cft) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Flood Bund Escape Channel RD 204+805 BRBD | 1.00 | 0.70 | 0.840 | 0.160 | - |
| 2 | J-Head Spur RD 205+000 BRBD. | 1.00 | 0.70 | 0.155 | 0.845 |  |
| 3 | J-Head Spur at RD 215+500 B.R.B.D. | 1.50 | 1.05 | 1.075 | 0.425 |  |
| 4 | J- Head Spur at RD 224+500 B.R.B.D. | 3.00 | 2.10 | 0.376 | 2.624 |  |
| 5 | Mole Head Spur at RD 232+000 B.R.B.D . | 1.00 | 0.70 | 0.000 | 1.000 |  |
| 6 | Mole Head Spur At RD 241+000 BRBD. | 1.00 | 0.70 | 0.482 | 0.518 |  |
| 7 | J-Head Spur at RD 250+000 B.R.B.D. | 1.50 | 1.05 | 0.641 | 0.859 |  |
| 8 | J-Head Spur at RD 259+000 B.R.B.D. | 1.00 | 0.70 | 0.106 | 0.894 |  |
| 9 | J-Head Spur at RD 265+000/L of BRBD Link Canal | 1.500 | 1.05 | 1.221 | 0.279 |  |
| 10 | Shank Bund at RD 279+500 BRBD. | 1.500 | 1.05 | 0.370 | 1.130 |  |
| 11 | Escape Bund at RD 279+500 B.R.B.D. | 0.500 | 0.35 | 0.400 | 0.100 |  |
| 12 | Right Guide Bund at Ravi Syphon B.R.B.D. | 2.50 | 1.75 | 0.470 | 2.03 |  |
| 13 | Left Guide Bund at Ravi Syphon B.R.B.D. | 2.50 | 1.75 | 0.650 | 1.850 |  |
| 14 | Guide Wall Spur at RD 288+500 BRBD. | 2.50 | 1.75 | 0.891 | 1.609 |  |
| 15 | Diversion Bund Mole Head Spur Ravi observation Post | 0.350 | 0.245 | 0.199 | 0.151 |  |
| Sub Total: (D). |  | 22.350 | 15.645 | 7.876 | 14.474 |  |

Sub Divisional Officer 				Sub Divisional Officer		Executive Engineer

Link Sub Division				Ravi Syphon Sub Division		Chakbandi Division

Lahore	.					Lahore	.				Lahore

7.2	WATCHING ESTABLISHMENT

| S.# | Particulars | Coolies |  |  |  | Mates |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | No of beats | No of Beldars /Collies per beat | No of shifts | Total No of Beldars/ Collies | No of mates per beat | No of shift | Total No of mates |
| 1 | Medium Flood Level (65000 Cs To 90000 Cs) | 2 | 20 | 1 | 40 | 1 | 1 | 1 |
| 2 | Medium Flood Level (65000 Cs To 90000 Cs) | 2 | 25 | 1 | 42 | 2 | 2 | 2 |
| 3 | High Flood Level (90000 Cs To 135000 Cs) | 2 | 31 | 1 | 62 | 3 | 2 | 3 |

ADDITIONAL ESTABLISHMENT

- Chowkidar day and night

2) 	Gauge reader in 3 Shifts

3)	Telephone attendant

NOTE:-

Additional establishment will be engaged on three shift basis in case of high flood or beyond.

Sub Divisional Officer 						Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore 								Lahore

Executive Engineer

Chakbandi Division

Lahore.

7.3	ARRANGEMENT AT SENSITIVE SITES

Spur RD 215+000 and 224+000, Diversion Bund near Ravi Post and the Ravi Syphon are the sensitive sites. Flood watching / fighting camp is provided over there with all logistics. The watching material from the flood camps described below will be shifted as and when required at the above stated sites.

7.4	WATCHING MATERIAL

| S.# | Name of article | Units | Camp Head Quarter RD 205 on BRBD | Camp Head Quarter RD 260 on BRBD | Camp Head Quarter RD 282760 on BRBD | Camp Head Quarter RD 285 on BRBD | Total Required | Available | Balance required | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Small Bag | No | 1000 | 1000 | 2500 | 2500 | 7000 | - | 7000 | Balance Qty will be procured before flood season 2024. |
| 2. | Toot Basket | No | 25 | 100 | 49 | 26 | 400 | 200 | 200 |  |
| 3. | Kassi | No | 190 | - | - | - | 400 | 190 | 210 |  |
| 4. | Axe | No | 10 | 10 | 10 | 10 | 40 | - | 40 |  |
| 5. | Pick Axe | No | 10 | 10 | 10 | 10 | 40 | - | 40 |  |
| 6. | Handle For Kassi | N0 | 80 | 100 | 100 | 100 | 400 | 80 | 320 |  |
| 7 | G.I Buckets | No | 50 | 50 | 50 | 50 | 200 | - | 200 |  |
| 8 | Torches Chargeable | No | 25 | 25 | 25 | 6 | 100 | 6 | 94 |  |
| 9 | Lanterns | No | 15 | 15 | 15 | 15 | 60 | - | 60 |  |
| 11 | Gas Lamps | No | 10 | 10 | 10 | 10 | 40 | - | 40 |  |
| 12 | Life Jacket | Doz | 10 | 10 | 10 | 5 | 40 | 5 | 35 |  |
| 17 | Steel Wire Rope ¼” | Lft | 50 | 240 | 1000 | 1000 | 4000 | 290 | 3710 |  |
| 18 | Steel Wire rope 3/8” | Lft | 1000 | 260 | 1000 | 1000 | 4000 | - | 3740 |  |
| 19 | Manila rope 1” dia | Kg | 50 | 50 | 50 | 100 | 250 | - | 250 |  |
| 20 | G.I wire 8 SWG | Kg | 100 | 100 | 100 | 100 | 400 | - | 400 |  |
| 1 | Wire Tarngers SWG 10. | No | 2000 | 2000 | 3000 | 3000 | 10000 | - | 10000 |  |
|  | Munj Tatanger | No | 500 | 500 | 300 | 500 | 2000 | 300 | 1700 |  |
| 22 | Wooden Ballies 10’ Long | No | 200 | 200 | - | 70 | 800 | - | 800 |  |
| 23 | Shishum Wooden Balies | No | 500 | 500 | - | 500 | 2000 | - | 2000 |  |
| 24 | Wood Mallets | No | 3 | 3 | - | 3 | 3 | 12 | - |  |
| 25 | G.I.Wire Creats 10 SWG 6” Mash 2’ x 2’ | No | 1000 | 1000 | - | 2500 | 7000 | - | 7000 |  |
| 26 | Bamboo 18’-20’ Long | No | 500 | 30 | - | 85 | 4000 | - | 4000 |  |
| 29 | Ors ( chappu) | No | - | - | - | 8 | 8 | - | 7 |  |
| 30 | Bulbs | Doz | 3 | 3 | 3 | 3 | 12 | - | 12 |  |
| 31 | Mercury Lights / Flood light Complete | No | 10 | 10 | 10 | 10 | 40 | - | 40 |  |
| 32 | Nyloon Bags | No | 500 | 500 | 155 | 500 | 2000 | 155 | 1845 |  |
| 33 | Emergency Siren | No. | 1 | 1 | 1 | 1 | 4 | 1 | 3 |  |
| 34 | Shouldare | No. | 1 | 5 | 5 | 5 | 20 | 1 | 19 |  |

Sub Divisional Officer 			Sub Divisional Officer			Executive Engineer

Link Sub Division			Ravi Syphon Sub Division			Chakbandi Division

Lahore	.				Lahore	.				Lahore

| 35 | Umbrella | No. | 3 | 10 | 10 | 10 | 40 | 3 | 37 | Balance Qty will be procured before flood season 2024. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | Long shoes | Pair | 10 | 10 | 10 | 10 | 40 | - | 40 |  |
| 37 | Nyloon rope 1” dia | Rft | 200 | 200 | 200 | 200 | 800 | - | 800 |  |

7.5         ARRANGEMENT FOR SOUNDING & PROBING

Sounding / Probing rads and boats are available at Ravi Syphon RD. 285+000 of

BRBD Link Canal.

7.6	LIGHTING ARRANGEMENT

Two No generators are available in case of non-availability of electricity or failure.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division				Chakbandi Division

Lahore	.			Lahore	.					Lahore.

7.7	RATION ARRANGEMENT

Ration would be made available at site.

7.8	POL ARRANGEMENT FOR VEHICLES

Normal “POL” charges will be utilized for government vehicles, extra demand would be raised for the protection of flood affected population, if required.

7.9	TRANSPORTATION

All Government vehicles would be deputed on flood duty round the clock.

7.10	LAW AND ORDER

It is the responsibility of local police which would be around during flood period.

7.11	MEDICAL ARRANGEMENT FOR LABOUR

1st Aid / medicine boxes would be arranged at site. Health Department would also help.

7.12	LIAISON WITH OTHER DEPARTMENTS

It would be ensured telephonically through official land line and private mobile numbers.

  - ROLE OF THE ARMY

Besides arrangements of watching staff by Irrigation Department, additional 100 volunteers will be requisitioned from the army on special request. They will also take position for the rescue of the persons and live stock of flood affected areas on request of the Department.

  - DUTIES OF TELEPHONE ATTENDANT

He will record daily discharges at various control points and convey important information and messages etc.

  - WIRELESS ARRANGEMENTS

Base station at Shahdara camp and mobile wireless in Government vehicles would be installed during flood season.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division				Chakbandi Division

Lahore	.			Lahore	.					Lahore.

CHAPTER – 8

DETAIL OF ENCROACHMENTS

There is no encroachment on any of the Flood embankments /spurs in this Division except J-Head Spur at RD. 259+000/L BRBD Link Canal where public has constructed colony after the flood of 1988. However, there is no hurdle to access structure, evacuation can only be made if suitable alternative is provided to nearly 50 families living at this site.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division				Chakbandi Division

Lahore	.			Lahore	.					Lahore.

CHAPTER – 9

DUTY ROSTER / FLOOD FIGHTING PROGRAM

The details of duty roster/battle station of key personnel during high flow have been given on next page attached.

B E A T No. 1.

Name of  SDO : Arshad Badal  Link Sub Division   Cell No (0300-4911904)

B.R.B.D Link R.D 178 - 205.

Incharge of beat: Muhammad Akmal Nadeem  Sub Engineer ( Mob: No 0303-5208909)

Camp Head Quarter: R.D 204+805 B.R.B.D Link.

- Regulation Baldar 	2 No

- Baldar		  	4 No

- Head Jamadar	 	1 No

B E A T No. 2.

Name of  SDO :- Arshad Badal  Link Sub Division Cell No (0300-4911904)

i.	B.R.B.D Link R.D 205-230

ii.	Spur R.D 205/L of B.R.B.D Link. Spur R.D 215/L of B.R.B.D Link

iii.	Spur R.D 224+500/L of B.R.B.D Link

In charge of beat :Muhammad Fawad Sub Engineer ( Mob: No.0300-4911904)

Camp Head Quarter R.D 204+805 B.R.B.D Link.

- Regulation Baldar 	3 No

- Baldar		  	4 No

- Earthwork Mistary 	1 No

B E A T No. 3.

Name of  SDO :- Arshad Badal Link Sub Division Cell No (0342-1688585)

- B.R.B.D Link R.D 230-260

- Spur R.D 232/L B.R.B.D Link.

- Spur R.D 241/L B.R.B.D Link.

- Spur R.D 250100/L B.R.B.D Link.

- Spur R.D 259/L B.R.B.D Link.

In Charge of Beat: Malik Naveed  Sub Engineer ( Mob: No.0300-4911904)

Camp Head Quarter R.D 260+000 B.R.B.D Link.

- Regulation Baldar 	2 No

- Baldar		  	4 No

- Earthwork Mistary 	1 No

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division				Chakbandi Division

Lahore	.			Lahore	.					Lahore.

B E A T No. 4.

Name of SDO:- Muhammad Tahir Ravi Syphon  Sub Division   (0346-4523466)

i. B.R.B.D Link R.D 260-283.

ii. Spur R.D 265/L B.R.B.D link with Prong.

iii. Shank bund RD 0-32000 with Spur at RD 1900, 2500 and 3200

iv. Right Guide bund.

v. Escape bund 279+500/R B.R.B.D Link.

In Charge of Beat:  Muhammad Azam

Pakhiala Section  Sub Engineer.

Camp Head Quarter at R.D 282+760 B.R.B.D Link

- Regulation Baldar 	4 No

- Baldar		  	4 No

- Earthwork Mistary 	1 No

B E A T No.5.

Name of SDO:- Muhammad Tahir   Ravi Syphon  Sub Division   (0346-4523466)

BRBD Link Canal RD 285+000 to 301+000

- Left Guide Bund of Ravi Syphon.

- J-Spur at RD 288+500/L B.R.B.D Link.

- Guide Wall Spur. RD 288+500 BRBD Link Canal.

- Solid Stone Studs and circular portion at Ravi Post.

In Charge of Beat: Ghulam Muhammad Ghous  Sub Engineer (Mob: No  0300-4577915)

Ravi Syphon Section Sub Engineer.

Camp Head Quarter at RD 285+000 D/S of Ravi Syphon

- Regulation Baldar 	5 No

- Baldar		  	4 No

- Earthwork Mistary 	1 No

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division				Chakbandi Division

Lahore	.			Lahore	.					Lahore.

CHAPTER – 10

EMERGENCY TELEPHONE NUMBERS

TELEPHONE NUMBERS OF FLOOD CONCERNED OFFICER / OFFICIALS

DISTRICT LAHORE AREA CODE “042”. S.NO; NAME; OFFICE NUMBER; PERSONAL NUMBER. 1; Mr. Rashid Minhas Chief Engineer, Irrigation Lahore Zone, Lahore; 99212085 99212086 99212086 Fax; -. 2; Chief Engineer Drainage & Flood; 992030602; -. 3; Mr.Muddaser Rehan Superintending Engineer, Link Circle, Lahore; 99250310 99250308; 0322-5543633. 4; Mr.Hafiz Abdur Rehman Executive Engineer Operation, Lahore.; 99212089; 0300-4810150. 5; Mr. Muhammad Numan Executive Engineer Chakbandi Division, Lahore.; 99250312 99250313; 0300-4503037. 6; Director Flood/ Secretary Flood Commission, Lahore.; 99231614; -. 7; Flood Warning Center; 99205371 99205372 Fax; -. 8; Programme Monitoring Implementation Unit (PMIU) Lahore.; 99213595-97; -. 9; PMO Barrages; 99250351; -. 10; D.C.O Lahore.; 99211003 99211004; -. 11; District Police Officer; 99200897; -. 12; Executive Engineer Wapda Shalimar Division, Lahore.; 99250046 99250047 Fax; -. DISTRICT SHEIKHUPURA AREA CODE “056”. 13; Xen Wapda (Muridke) Sheikhupura; 3406053; -. 14; Xen Telephone, Sheikhupura; 9200285; -. SUB DIVISIONAL OFFICERS / SUB ENGINEERS IN CHAKBANDI DIVISION, LAHORE.. 17; Mr. Arshad Badal Sub Divisional Officer Link Sub Division, Lahore.; 99250312 99250313; 03004911904. 18; Muhammad Akmal Naeem Sub Engineer Chakrali Section; 99250312 99250313; 0346-6705321. 19; Muhammad Fawad Sub Engineer Mandiali Section.; 99250312 99250313; 0342688585. 20; Naveed Shafie Sub Engineer. Longwala Section.; 99250312 99250313; 0345-3263012. 21; Muhammad Tahir Sub Divisional Officer Ravi Syphon Sub Division, Lahore.; 99250312 99250313; 0303-4523466 0303-5661343. 22; Muhammad Azam Sub Engineer Pakhiala Section; 99250312 99250313; 0322-7218487. 23; Ghulam Muhammad Ghous Sub Engineer Ravi Syphon Section; -99250312 99250313; 0300-4577915.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division				Chakbandi Division

Lahore	.			Lahore	.					Lahore.

CHAPTER – 11

STANDARD OPERATING PROCEDURE(SOP) FOR

BREACHING SECTIONS

There is no breaching section in Chakbandi Division.

11.1	HISTORY OF THE BREACHING SECTION.

Not Applicable.

11.2	LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION.

Not Applicable.

11.3	ARRANGEMENT OF EXPLOSIVES AND SECURITY OF EXPLOSIVE SORES.

Not Applicable

11.4	LIST OF THE SECURITY STAFF ALONGWITH DETAIL OF THEIR TRAINING ETC.

Not Applicable

11.5	DETAIL OF MECHANICAL MEANS AS A STANDBY ARRANAGMENTS IN CASE OF DETONATION FAILURE.

Not Applicable

11.6	DUTY ROSTER IN CASE OF CRITICAL SITUATION.

Not Applicable

11.7	BREACHING COMMITTEE WITH THEIR ACTION PLAN.

Not Applicable.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division				Chakbandi Division

Lahore	.			Lahore	.					Lahore.

11.8 LIST OF THE VILLAGES LIKELY TO BE INUNDATED DURING VARIOUS FLOOD LIMITS

Jandiala Kalsan

Baryar Kuhna.

| Baryar Nau. LEGEND |  |  |
| --- | --- | --- |
|  | Exceptionally High Flood |  |
|  | High Flood |  |
|  | Medium Flood |  |
|  | Low Flood |  |

Kotli Virk Baryar.

Mandiali

Longwala.

Kotli Piran.

Miani.

Rajpura.

Satrayan.

Passianeala.

Chak Dhedho.

Aulia Piran.

Nangal Waris Khan.

Meerowal.

Shattab Garh.

Kajlay.

11.9	ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENT

All announcements regarding evacuation will be made through District Government by electronic media radio, local Mosque and Loud speaker etc.

11.10 DETAIL OF COORDINATION WITH CIVIL / ARMY AUTHORITIES

Not Applicable.

11.11 PARALLEL COMMUNICATION ARRANGEMENTS

All communication will be made on wireless, special messengers and telephones mobile phones etc.

Sub Divisional Officer 		Sub Divisional Officer			Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.				Lahore.

11.12 INDEX PLAN

ATTACHED.

FLOOD FIGHTING PLAN FOR THE YEAR 2025

PART-B.

Executive Engineer	 				Superintending Engineer

Chakbandi Division	 				Link Circle, Lahore

Lahore

Chief Engineer

Irrigation Lahore Zone

Lahore

GOVERNMENT OF THE PUNJAB

IRRIGATION  DEPARTMENT

EMERGENCY CONTINGENCY PLAN

(PART – B OF FLOOD FIGHTING PLAN)

OF

CHAKBANDI  DIVISION

FOR

THE YEAR 2025

LINK CIRCLE,					LAHORE IRRIGATION

LHORE						ZONE, LAHORE

CHAPTER-12

FLOOD PREPAREDNESS ACTIVITIES FOR MOONSOON SEASON 2025 OF CHAKBANDI DIVISION LAHORE.

12.1 	VULNERABLE SITES  IN FLOOD BUNDS / STRUCTURES.

There are no vulnerable sites in Flood Bunds /structures.

12.2	REMOVAL OF CRITICAL ENCROACHMENT.

It is submitted that there are no critical encroachments from Flood Plains / Waterways in this Division.

12.3	PROTECTION INFRASTRUCTURES , EMBANKMENTS ETC FOR FLOOD SEASON 2025.

It is submitted that all infrastructures, embankments under the jurisdiction of this Division are safe and satisfactory.

12.4	CLEARANCE OF WATERWAYS /BRIDGES / CULVERTS / ROAD CROSSING ON THE RIVERS NULLAH AND DRAINS.

It is submitted that all the structure such as bridges Road crossings and railway bridges on river Nullahs, Flood Carrying channels main canal etc are clear and satisfactory.

12.5	REPAIRING DAMAGES AND PAINTINGS OF GUAGE AND RD MARKS.

It is submitted that all gauge and RD Marks are satisfactory and painted.

12.6.	OPERATION OF BREACHING SECTIONS.

There is no breaching Section in this Division.

12.7 	BREACHES DUE TO RISING OF FLOOD WATER, DETERIORATION OF FLOOD BUNDS ETC.

Not exist.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

12.8	CERTIFICATE REGARDING HEALTH OF FLOOD PROTECTION INFRASTRUCTURE & RIVER TRAINING WORKS.

This is to certify that the undersigned inspected all the Flood Bunds and River Training Works in my Jurisdiction and found in satisfactory and safe condition.

Sub Divisional Officer 					Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore								Lahore

Executive Engineer						Superintending Engineer

Chakbandi Division 						Link Circle Lahore

Lahore

Chief Engineer

Irrigation Lahore Zone

Lahore

12.9	CERTIFICATE REGARDING FLOOD PREPARENESS 2025.

CHAKBANDI DIVISION LAHORE

This is to certify that:-

- All the flood bunds, embankments etc falling in the jurisdiction of this division  have  physically been inspected  by the undersigned.

- All the short comings as pointed out by the Departmental / Army teams have been rectified.

- The repairing / painting / lettering of RD marks data boards and gauges indicating HFL  (Highest Flood Level ) etc have been completed.

- The flood fighting material at vulnerable sites has been made available.

- The reserve stock of stone where required is made available.

Sub Divisional Officer 					Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore								Lahore

Executive Engineer 						Superintending Engineer

Chakbandi Division 						Link Circle Lahore

Lahore

Chief Engineer

Irrigation Lahore Zone

Lahore

12.10       CERTIFICATE REGARDING REMOVAL OF ENCROACHMENT FROM WATER WAYS AND FLOOD PLAIN OF CHAKBANDI DIVISION LAHORE.

This is to certify that no critical encroachment exist along the water ways and in Flood Plains in jurisdiction .Further water ways of bridges and culverts in the jurisdiction are

Cleared for smooth flow of water.

Sub Divisional Officer 					Sub Divisional Officer

Link Sub Division 						Ravi Syphon Sub Division

Lahore								Lahore

Executive Engineer 						Superintending Engineer

Chakbandi Division 						Link Circle Lahore

Lahore

Chief Engineer

Irrigation Lahore Zone

Lahore

CHAPTER-13

EMERGENCY CONTINGENCY PLAN FOR THE VULNERABLE SITE (SITES) LISTED ABOVE EMERGENCY CONTINGENCY PLAN MUST INCLUDE.

Nil

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORT WITH LEVELS.

Plan showing route of flood water is attached Annexure “A”

  - DETAIL OF VILLAGES ABADIES LIKELY TO BE EFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

The villages and infrastructures likely to be affected.

VILLAGES.

Sidhanwalai, Sharif Pura, Pakhiala, Dogharanwal, Janua, Labanwala, Babkwal, Kalakhatai, Adhian, shoal, Sehol, Sham-ka-Bhattian and Rana Town

13.3. STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL. THIS MAY INCLUDE:

13.3.1	 ARRANGEMENTS.

The Flood bunds are inspected by a Committee constituted by the Secretary to Government of the Punjab, Irrigation & Power Department in the month of May every year. Inspection of all bunds and spurs by the Team of Army Authorities is also carried out before the flood season to ensure that the bunds are safe against the coming flood season.

To provide swift acquisition and disposal of any information related to flood situation in order to intimate prompt action sector in charge are present at RD 204+805 of BRBD Link Canal and at RD 255+257 BRBD Link Canal.

Camps are established on all vulnerable sites to monitor the situation during flood season. Cell Phones are provided and generator sets are installed at important / vulnerable reaches. The flood watching material is procured and shifted to the camp sites before the start of Flood season.

Keeping in view the possibility of receiving floods, precautionary measures have to be taken from 1st July to 15th October. Watching Establishment is also employed for the flood season fixed for various magnitudes of Floods in River Ravi.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

13.3.2		ESTABLISHMENT OF FLOOD FIGHTING CAMPS

The 5 No camps will be established as per detail given at Page no 15, 2 No camps at

RD 204+805, 1 No at RD 215+000, 1 No camp at RD 282+760, 1 No camp at RD 285+000 D/S of  Ravi Syphon from 15th of June 2024 to 15th of October 2024.

    - DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMPS SITES.

| Sr.no | Name of Officer /official with Designation | Camp Head | Contact No |
| --- | --- | --- | --- |
| 1 | Muhammad Arshad Badal SDO Link Sub Division | Camp Head 204+805 Camp Head 215+000 | 0300-491104 |
| i. | Muhammad Fawad Sub Engineer Mandiali Section |  | 03424688585 |
| ii | Muhammad Akmal Sub Engineer Chakrali Section |  | 0346-6705321 |
| iii | Naveed Sahfie Sub Engineer Longwala Section |  | 0343-3263012 |
| 2 | Muhammad Tahir SDO Ravi Syphon Sub Division | Camp Head 282+760 Camp Head 285+000 | 0346-4523466 |
| ii | Muhammad Azam Sub Engineer Pakhiala Section |  | 0322-7218487 |
|  | Ghulam Muhammad Ghous Sub Engineer Ravi Syphon Section |  | 0300-4577915 |

As above.

13.3.4		DEPARTMENTAL MACHINERY AVAILABLE.

Not Available

13.3.5		 MACHINERY AVAILABLE FROM PRIVATE SOURCE.

Name of owner:  		Aman-ullah-Khan

Contact No. 			 0342 – 5706773

Location.			Ghazi Kakka, Ravi Syphon.

Excavator Machine		= 06 No.

Tractor with Front Blade	= 03 No.

Tractor with Trolleys		= 12 No.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

13.3.6 & 13.3.7	:-FLOOD FIGHTING MATERIAL REQUIRED AND AVAILABLE     IN ADDITION TO THE REQUIRED MATERIAL AS PER PAGE 16-17.

| SR.NO | ITEM | LOCATION WHERE MATERIAL AVAILABLE | UNIT | Quantitative available | Quantities Required | Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Small Bag | Ravi Syphon | No | 2200 | - | - | Balance Qty will be procured before Flood Season 2025 |
|  | Tooth Basket | - | No | - | 175 | 175 |  |
|  | Axe | Ravi Syphon | No | 13 | 10 | 3 |  |
|  | GI Bucket | Ravi Syphon | No | 45 | - | - |  |
|  | Torches Chargeable | Ravi Syphon | No | 6 | 18 | 12 |  |
|  | Steel wire Rope ¼ “ | Ravi Syphon | No | 240 | 2000 | 1760 |  |
|  | Manger Tranger SWG 10 | Ravi Syphon | No | 300 | - | - |  |
|  | ORS (Chappu) | Ravi Syphon | No | - | 3 | 3 |  |
|  | Nyloon Bags | Ravi Syphon | No | 850 | 3900 | 3050 |  |
|  | Pitching Stone | Ravi Syphon | Lac Cft | 7.876 | 14.30 | 6.424 |  |
|  | Kassie | - | No | - | 140 | 140 |  |
|  | Laterns / Tourch/ search Light | - | No | - | 44 | 44 |  |
|  | Gas Lamp | - | No | - | 10 | 10 |  |
|  | Live Jacket | - | No | - | 20 | 20 |  |
|  | Steel Wire Rope 3/8” | - | Lft | - | 5000 | 5000 |  |
|  | Manila Rope 1” Dia /Nyloon rope | - | KG | - | 250 | 250 |  |
|  | GI Wire 8 SWG | - | KG | - | 250 | 250 |  |
|  | Wire Trangers SWG 10 | - | No | - | 5000 | 5000 |  |
|  | Wooden Ballies 10’ Long | - | No | - | 800 | 800 |  |
|  | Shesham Wooden BAllies | - | No | - | 2000 | 2000 |  |
|  | Wood Mallets | - | No | - | 12 | 12 |  |
|  | GI Wire Creats 10 SWG 6” Mash 2”x2” | - | No | - | 7000 | 7000 | Balance Qty will be procured before Flood Season 2025 |
|  | Bamboos 18’x20’ Long | - | No | - | 4000 | 4000 |  |
|  | Bulb | - | Dozen | - | 12 | 12 |  |
|  | Mercury Light | - | No | - | 36 | 36 |  |
|  | Emergency Siren | - | No | - | 05 | 05 |  |
|  | Shouldare | - | No | - | 08 | 08 |  |
|  | Umbrella | - | No | - | 30 | 30 |  |
|  | Long Shoes | - | No | - | 20 | 20 |  |
|  | Generator | - | No | - | 03 | 03 |  |
|  | Petrol for Generator | - | Litter | - | 1000 | 1000 |  |
|  | Electric Wire Arrangement | - | L/S | - | 2400 RFT at 04 No Camps at vulnerable sits. | 2400 RFT at 04 No Camps at vulnerable sits |  |
|  | Rain Coat | - | No | - | 25 | 25 |  |
|  | Water Cooler | - | No | - | 08 | 08 |  |
|  | Water Sets | - | No | - | 04 | 04 |  |
|  | Char Pie | - | No | - | 15 | 15 |  |
|  | Table With Chair | - | Sets | - | 04 | 04 |  |
|  | Boat | - | No | - | 01 | 01 |  |
|  | Ganny Bags | - | No | - | 2000 | 2000 |  |

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDING, CANALS AND DRAINAGE NETWORK

INFRASTRUCTURES

HIGHWAYS

G.T. Road Lahore Gujranwala From Shahdara to Kala Shah Kaku.

Kalakhatai Road Full.

Adhian Sidhanwalai Road Full.

Adhian Rana Town Road Full.

Narang Chakrali Road

Narang Mandiali Road

SCHOOLS / COLLEGES TEHSIL FEROZEWALA DISTRICT SHIEKUPURA

Government  Boys and girls High School Sidhanwalai

Government Primary School Sharifpura.

Government Primary School Sehol

Government Middle School Janua

Government  Primary School Lubanwala.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE Government High School Babakwal.

Ranger Public School Kalakhati Station

Government High School Kalakhati Station

HEALTH CENTER

R.H.C. Babakwal

R.H.C Adhian

R.H.C Kalakhtai

R.H.C Sham-ka-Bhattian

R.H.C Sehol

R.H.C Rana Town.

POLICE STATION

Police Post Lubanwala.

Narang Mandi

Police Post Chak No. 42.

RAILWAY

Lahore Narowal Railway Track From Shahdara to Kala Khatai.

Lahore Gujranwala Railway Track From Shahdara to Kala Shaha Kaku.

TELEPHONE EXCHANGES.

Babakwal

Kalakhatai

Sidhanwalai

Rana Town

Adhian

ARMY INFRASTRUCTURES

Ranger Head Quarter Kalakhatai

Company Headquarter Sharifpura

Rattian Cantt: (Army)

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE GRID STATION.

Kala Shah Kaku

OIL DEPOT

Kala Shah Kaku

RICE RESEARCH CENTER

Kala Shah Kaku

BACKUP SUB DIVISION.

Bedian Sub Division will act as Backup Sub Division in case of emergency.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE CHAPTER-14

Action Plan.

14.1-Re-Shuffling / Recouping Plan of Reserve Stock Stone Departmentally.

Re-shuffling = Nil.

| S.No | Name of Structure | Quantity available In Lac Cft | Quantity to be Procured in Lac Cft |
| --- | --- | --- | --- |
| 1 | Flood protection bund along with left bank of escape channel RD 204+805. | 0.840 | 0.160 |
| 2 | J-Head Spur RD 205+000 BRBD Link Canal. | 0.155 | 0.845 |
| 3 | J-Head Spur at RD 215+000/L of BRBD Link Canal . | 1.075 | 0.425 |
| 4 | J-Head Spur 224+000/L Bank of BRBD Link Canal. | 0.376 | 2.624 |
| 5 | Mole Head Spur at RD 232+000 B.R.B.D . | 0.00 | 1.000 |
| 6 | Mole Head Spur at RD 241+000/L of BRBD Link Canal. | 0.482 | 0.518 |
| 7 | J Head Spur 250+000/ L BRBD LINK Canal. | 0.641 | 0.859 |
| 8 | J Head Spur 259+000/ L BRBD Link Canal. | 0.106 | 0.894 |
| 9 | J Head Spur 265+000/ L BRBD Link Canal. | 1.221 | 0.279 |
| 10 | Shank Bund at RD 279+500/L OF BRBD Link Canal. | 0.370 | 1.130 |
| 11 | Escape Bund at RD 279+500/R B.R.B.D Canal. | 0.400 | 0.100 |
| 12 | Right Guide Bund at Ravi Syphon B.R.B.D Link Canal. | 0.470 | 2.030 |
| 13 | Left Guide Bund at Ravi Syphon B.R.B.D Link Canal. | 0.650 | 1.850 |
| 14 | Guide Wall Spur at RD 288+500 of BRBD Link Canal. | 0.891 | 1.609 |
| 15 | Diversion Bund Mole Head Spur at Ravi Observation Post. | 0.199 | 0.151 |
|  | Total:- | 7.876 | 14.474 |

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

14.2  Detail of inlet/outlet crosses along with closing methodology.

- No inlet exists in flood Reach from RD 178+000 to 288+500 of BRBD Link Canal.

- Following are the outlet which is operate-able when required.

| S.No | Location | Size | Type |
| --- | --- | --- | --- |
| 1 | R.D 216+000/L of BRBD Link Canal | 6” Dia | Steel Pipe 1 No outlet |
| 2 | R.D 221+000/L of BRBD Link Canal | 6” Dia | Steel Pipe 1 No outlet |
| 3 | R.D 233+500/L of BRBD Link Canal | 6” Dia | Steel Pipe 2 No outlet |
| 4 | R.D 238+000/L of BRBD Link Canal | 6” Dia | Steel Pipe 1 No outlet |
| 5 | R.D 241+500/L of BRBD Link Canal | 18” Dia | Steel Pipe 1 No outlet |
| 6 | R.D 247+500/L of BRBD Link Canal | 18” Dia | Steel Pipe 1 No outlet |
| 7 | R.D 264+500/L of BRBD Link Canal | 18” Dia | Steel Pipe 1 No outlet |
| 8 | R.D 279+500/L of BRBD Link Canal | 18” Dia | Steel Pipe 1 No outlet |
| 9 | R.D 254+500/L of BRBD Link Canal | 2’ Dia | 2 No Steel Pipe Permanent Masnory structure |

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

14.3 DEPLOYMENT MACHINERY (MEDIUM TO HIGH FLOOD).

| S.NO | Name of structure / bund | Length in Mile | Vulnerable Reach | Camp Location | Site in charge by name cell NO. | Machinery Deployment. |  |  |  |  | Availability of Stone In Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Dumper | Tractor | Labour Beldar |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Flood protection bund a long with left bank of escape channel RD 204+805. | 3000’ | - | RD 205+000 of BRBD Link Canal | Arshad Badal Sub Divisional Officer Link Sub Division . 0300-4911904 | 1 | 1 | 1 | 1 | 4 | 0.840 |
| 2 | J Head Spur at RD 205+000/L Bank of BRBD Link Canal . | 4150’ | - |  |  | - | - | - | - | 2 | 0.155 |
| 3 | J Head Spur 215+000/L of BRBD Link Canal . | 4150’ | - |  |  | - | - | - | - | 2 | 1.075 |
| 4 | J Head Spur 224+000/L Bank of BRBD Link Canal . | 6450’ | - |  |  | - | - | - | - | 2 | 0.376 |
| 5 | Mole head Spur 232+000/L BRBD Link Canal . | 2800’ | - |  |  | - | - | - | - | 2 | 0.00 |
| 6 | Mole Head at RD241+000. | 3550’ |  |  |  | - | - | - | - | 2 | 0.482 |
| 7 | J Head Spur at RD 250+000/L BRBD Link Canal | 4200’ | 256+000- |  |  | - | - | - | - | 2 | 0.641 |
| 8 | J Head Spur at RD 259+000/L BRBD Link Canal | 3700’ | - | RD 260+000 of BRBD Link |  | 1 | 1 | 1 | 1 | 4 | 0.106 |

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

| 9 | J Head Spur 265+000/L BRBD Link Canal | 7500’ | - |  | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. Cell No .0303-5661343 | - | - | - | - | 2 | 1.221 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Shank Bund at RD 279+500/L of BRBD Link Canal | 5500’ | - |  |  | - | - | - | - | 2 | 0.370 |
| 11 | Escape Bund at RD 279+500/R Bank of BRBD Link Canal | 5625’ | - |  |  | - | - | - | - | 2 | 0.400 |
| 12 | Right Bund of BRBD Link Canal at RD 282+760. | 2900’ | - | RD 282+760 on BRBD Link Canal |  | 1 | 1 | 1 | 1 | 4 | 0.470 |
| 13 | Left Bund of BRBD Link Canal at RD 285+000 | 2900’ | - | RD 285+000 on BRBD Link Canal |  | 1 | 1 | 1 | 1 | 4 | 0.650 |
| 14 | Guide Wall Spur at RD 288+500/L of BRBD Link Canal and J-Head Spur. | 3422’ | - |  |  | - | - | - | - | 2 | 0.891 |
| 15 | Diversion Bund of Mole Head spur Ravi Observation Post | 1350’ | - |  |  | - | - | - | - | 2 | 0.199 |
| Total:- |  |  |  |  |  | 4 | 4 | 4 | 4 | 40 | 7.875 |

Note:-Payment for machinery and work charge employer subject to verification by TPM.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

14.4   DEPLOYMENT MACHINERY (HIGH to Very High FLOOD).

| S.NO | Name of structure / bund | Length in Mile | Vulnerable Reach | Camp Location | Site in charge by name cell NO. | Machinery Deployment. |  |  |  |  | Availability of Stone In Lac Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Dumper | Tractor | Labour Beldar |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Flood protection bund a long with left bank of escape channel RD 204+805. | 3000’ | - | RD 205+000 of BRBD Link Canal | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. Cell No .0303-5661343 | 1 | 1 | 1 | 2 | 5 | 0.840 |
| 2 | J Head Spur at RD 205+000/L Bank of BRBD Link Canal . | 4150’ | - |  |  | - | - | - | - | 2 | 0.155 |
| 3 | J Head Spur 215+000/L of BRBD Link Canal . | 4150’ | - |  |  | - | - | - | - | 2 | 1.075 |
| 4 | J Head Spur 224+000/L Bank of BRBD Link Canal . | 6450’ | - |  |  | - | - | - | - | 2 | 0.375 |
| 5 | Mole head Spur 232+000/L BRBD Link Canal . | 2800’ | - |  |  | - | - | - | - | 2 | 0.00 |
| 6 | Mole Head at RD241+000. | 3550’ |  |  |  | - | - | - | - | 2 | 0.482 |
| 7 | J Head Spur at RD 250+000/L BRBD Link Canal | 4200’ | 256+000- |  |  | - | - | - | - | 2 | 0.641 |
| 8 | J Head Spur at RD 259+000/L BRBD Link Canal | 3700’ | - | RD 260+000 of BRBD Link |  | 1 | 1 | 1 | 2 | 5 | 0.106 |

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

| 9 | J Head Spur 265+000/L BRBD Link Canal | 7500’ | - |  | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. Cell No .0303-5661343 | - | - | - | - | 2 | 1.221 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Shank Bund at RD 279+500/L of BRBD Link Canal | 5500’ | - |  |  | - | - | - | - | 2 | 0.370 |
| 11 | Escape Bund at RD 279+500/R Bank of BRBD Link Canal | 5625’ | - |  |  | - | - | - | - | 2 | 0.400 |
| 12 | Right Bund of BRBD Link Canal at RD 282+760. | 2900’ | - | RD 282+760 on BRBD Link Canal |  | 1 | 1 | 1 | 2 | 5 | 0.470 |
| 13 | Left Bund of BRBD Link Canal at RD 285+000 | 2900’ | - | RD 285+000 on BRBD Link Canal |  | 1 | 1 | 1 | 2 | 5 | 0.650 |
| 14 | Guide Wall Spur at RD 288+500/L of BRBD Link Canal and J-Head Spur. | 3422’ | - |  |  | - | - | - | - | 2 | 0.891 |
| 15 | Diversion Bund of Mole Head spur Ravi Observation Post | 1350’ | - |  |  | - | - | - | - | 2 | 0.199 |
| Total:- |  |  |  |  |  | 4 | 4 | 4 | 8 | 42 | 7.875 |

Note:-Payment for machinery and work charge employer subject to verification by TPM.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

14.5 DEPLOYMENT MACHINERY (High to Exceptionally High Flood).

| S.NO | Name of structure / bund | Length in Mile | Vulnerable Reach | Camp Location | Site in charge by name cell NO. | Machinery Deployment. |  |  |  |  | Availability of Stone In Lac Cft |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Dumper | Tractor | Labour Beldar |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |
| 1 | Flood protection bund a long with left bank of escape channel RD 204+805. | 3000’ | - | RD 205+000 of BRBD Link Canal | Arshad Badal Sub Divisional Officer Link Sub Division 0300-4911904 | 2 | 2 | 2 | 4 | 10 | 0.840 |  |
| 2 | J Head Spur at RD 205+000/L Bank of BRBD Link Canal. | 4150’ | - |  |  | - | - | - | - | 2 | 0.155 |  |
| 3 | J Head Spur 215+000/L of BRBD Link Canal . | 4150’ | - |  |  | - | - | - | - | 2 | 1.075 |  |
| 4 | J Head Spur 224+000/L Bank of BRBD Link Canal. | 6450’ | - |  |  | - | - | - | - | 2 | 0.375 |  |
| 5 | Mole head Spur 232+000/L BRBD Link Canal. | 2800’ | - |  |  | - | - | - | - | 2 | 0.00 |  |
| 6 | Mole Head at RD241+000. | 3550’ |  |  |  | - | - | - | - | 2 | 0.482 |  |
| 7 | J Head Spur at RD 250+000/L BRBD Link Canal | 4200’ | 256+000- |  |  | - | - | - | - | 2 |  | 0.641 |
| 8 | J Head Spur at RD 259+000/L BRBD Link Canal | 3700’ | - | RD 260+000 of BRBD Link |  | 2 | 2 | 2 | 4 | 10 |  | 0.106 |

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

| 9 | J Head Spur 265+000/L BRBD Link Canal | 7500’ | - |  | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. Cell No .0303-5661343 | - | - | - | - | 2 | 1.221 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Shank Bund at RD 279+500/L of BRBD Link Canal | 5500’ | - |  |  | - | - | - | - | 2 | 0.370 |
| 11 | Escape Bund at RD 279+500/R Bank of BRBD Link Canal | 5625’ | - |  |  | - | - | - | - | 2 | 0.400 |
| 12 | Right Bund of BRBD Link Canal at RD 282+760. | 2900’ | - | RD 282+760 on BRBD Link Canal |  | 2 | 2 | 2 | 4 | 10 | 0.470 |
| 13 | Left Bund of BRBD Link Canal at RD 285+000 | 2900’ | - | RD 285+000 on BRBD Link Canal |  | 2 | 2 | 2 | 4 | 10 | 0.650 |
| 14 | Guide Wall Spur at RD 288+500/L of BRBD Link Canal and J-Head Spur. | 3422’ | - |  |  | - | - | - | - | 2 | 0.891 |
| 15 | Diversion Bund of Mole Head spur Ravi Observation Post | 1350’ | - |  |  | - | - | - | - | 2 | 0.199 |
| Total:- |  |  |  |  |  | 8 | 8 | 8 | 16 | 62 | 7.875 |

Note:-  Payment for machinery and work charge employer subject to verification by TPM.

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

14.6 Police Deployment Plan

| S.NO | NAME OF STRUCTURE | CAMP LOCATION | SITE INCHARGE BY NAME / CELL NO | DETAIL OF POLICE JAWANS |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| 1 | Mole Head Spur at RD 205+000 | RD 205+000 of BRBD Link Canal. | Arshad Badal Sub Divisional Officer Link Sub Division 0300-4911904 | SHO Farozwala ASI 1 No Constable 3 No. |
| 2 | J-Head Spur at RD 250+000. | RD 260+000 of BRBD Link. | Arshad Badal Sub Divisional Officer Link Sub Division. 0300-4911904 | Constable 01 No. |
| 3 | Right Guide Bund of Ravi Syphon at RD 283+000 | RD 282+760 on BRBD Link Canal. | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. 0303-5661343 | ASI 1 No Constable 3 No |
| 4 | Left Guide Bund of Ravi Syphon at RD 285+000 | RD 285+000 on BRBD Link Canal. | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. 0303-5661343 | ASI 1 No Constable 3 No. |

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

14.7          Detail of Synthetic Bags With Capacity of 500 kg & 1000 Kg

Synthetic Bags With Capacity of 500 kg.

| S.NO | NAME OF STRUCTURE / BUND | LENGTH IN feet | Vulnerable Reach | CAMP LOCATION | SITE INCHARGE BY NAME / CELL NO | REQUIRED DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG. | AVAILABLE DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | Mole Head Spur at RD 205+000. | 3000 ft | - | RD 205+000 of BRBD Link Canal. | Arshad Badal Sub Divisional Officer Link Sub Division. Cell No. 0300-4911904 | 500 Nos | 25 |
| 2 | J-Head Spur at RD 250+000. | 42000 ft | - | RD 260+000 of BRBD Link. |  | 500 Nos | 30 |
| 3 | Right Guide Bund of Ravi Syphon RD 283+000 | 2900 ft | - | RD 282+760 on BRBD Link Canal. | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. (0303-5661343) | 500 Nos | 50 |
| 4 | Left Guide Bund of Ravi Syphon at RD 285+000 | 2900 ft | - | RD 285+000 on BRBD Link Canal. | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. (0303-5661343) | 500 Nos | 50 |

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

Detail of Synthetic Bags with Capacity of 1000kg.

| S.NO | NAME OF STRUCTURE / BUND | LENGTH IN feet | Vulnerable Reach | CAMP LOCATION | SITE INCHARGE BY NAME / CELL NO | REQUIRED DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 1000 KG. | AVAILABLE DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 1000 KG. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | Mole Head Spur at RD 205+000. | 3000 ft | - | RD 205+000 of BRBD Link Canal. | Arshad Badal Sub Divisional Officer Link Sub Division. Cell No 0300-4911904 | 50 Nos | 15 |
| 2 | J-Head Spur at RD 250+000. | 42000 ft | -. | RD 260+000 of BRBD Link. |  | 50 Nos | 15 |
| 3 | Right Guide Bund of Ravi Syphon RD 283+000 | 2900 ft | - | RD 282+760 on BRBD Link Canal. | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. (0303-5661343) | 50 Nos | 20 |
| 4 | Left Guide Bund of Ravi Syphon RD 285+000 | 2900 ft | - | RD 285+000 on BRBD Link Canal. |  | 50 Nos | 20 |

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

14.8 	Detail of Polythene Sheets of Black Color to Protect U/S Slopes against wave action and to control seepage through embankments.

| S.No | Name of Structure / Bund | Length in Mile | Vulnerable Reach | Camp Location | Site In charge by Name Cell No | Required Detail of Polythene Sheet required. (300 Gauge) | Available Detail of Polythene Sheet required. (300 Gauge) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | Mole Head Spur at RD 205+000. | 3000 ft | - | RD 205+000 of BRBD Link Canal. | Arshad Badal Sub Divisional Officer Link Sub Division. Cell No. 03004911904 | 2000 Kg | NIL |
| 2 | J-Head Spur at RD 250+000. | 4200 ft | -. | RD 260+000 of BRBD Link. |  | 2000 Kg | NIL |
| 3 | Right Guide Bund of Ravi Syphon RD 283+000 | 2900 ft | - | RD 282+760 on BRBD Link Canal. | Muhammad Tahir Sub Divisional Officer. Ravi Syphon Sub Division. 0303-4661343 | 2000 Kg | NIL |
| 4 | Left Guide Bund of Ravi Syphon RD 285+000 | 2900 ft | - | RD 285+000 on BRBD Link Canal. |  | 2000 Kg | NIL |

Sub Divisional Officer 		Sub Divisional Officer				Executive Engineer

Link Sub Division		Ravi Syphon Sub Division			Chakbandi Division

Lahore	.			Lahore	.					Lahore.

FLOOD FIGHTING PLAN 2025 OF CHAKBANDI DIVISION LAHORE

CHAPTER-15

BACK UP DIVISION (IN CASE OF BREACH).

No breaching section exist in this Division, however in case of breach all the sister  Administrative Department ( Provincial Disaster Management Authority , C&W and Civil Administration) has informed that they will give the volunteer services  for this Division.

Executive Engineer,		Superintending Engineer,   	Chief Engineer,

Chakbandi Division,	  	   Link Circle, Lahore.	       	irrigation, Lahore Zone

Lahore.							        		Lahore.