CHAPTER-1

SALIENT FEATURES OF

DRAINAGE DIVISION MANDI BAHAUDDIN.

1.1 LOCATION

The jurisdiction of the Division is enclosed by Lower Jhelum Canal, River Jhelum in North, Upper Jhelum Canal & Pubby hills in East, Northern / Southern branches of L.J.C in west & River Chenab in south.

During the Flood season, the Drainage System is affected by heavy rains in pubby hills range and the catchments area of other main drains. The runoff due to these heavy rains results in spillover of the Drainage System and adjoining abadies are also effected High flood in River Jhelum & River Chenab also contributes to this by back flow and spill over in outfall reaches. The objective to formulate this plan is to reduce the effects of flooding up to the extent possible with in available resources.

The Division was created during 1976 covering five Districts namely Gujrat, Mandi Bahauddin, Jhelum, Sargodha & Mirpur (Azad Kashmir). This Division consists of three Sub Division with the Head Quarter at Mandi Bahauddin detail of which is given below:

| Sr.No. | Name of Drainage Sub Divisions | District | Tehsil | Drains Length (In Miles) |
| --- | --- | --- | --- | --- |
| 1 | Bahauddin Drainage Sub Division | Mandi Bahauddin/ Sargodha | Mandi Bahauddin, Malakwal, Bhalwal | 259.96 |
| 2 | Phalia Drainage Sub Division | Mandi Bahauddin/Gujrat | Phalia, Gujrat | 206.65 |
| 3 | Rasul Drainage Sub Division | Mandi Bahauddin, Gujrat, Jhelum & Mirpur | Mandi Bahauddin, Kharian, Sarai Alamgir, Mirpur | 176.19 |
| 4 | Phalia Tube Well Sub Division | Mandi Bahauddin, Gujrat, | Mandi Bahauddin, Gujrat, | --- |
| Total= |  |  |  | 642.80 |

1.2	GENERAL DESCRIPTION

The Drainage Net work consisting of Main Drains, Branch Drains and Tributary Drains having length of 642.80 Miles exist in the Division. There are 11 No’s major Drainage Systems existing in the Division which are capable to cater for a total discharge of about 7609 Cs: resulting from seepage and surface run-off from their catchments areas, out failing into River Chenab and River Jhelum. The details are as follows: -

CHENAB BASIN.

| Sr. No. | Drainage System. | Discharge. |
| --- | --- | --- |
| 1 | Miana Gondal Drainage System. | 560 Cs |
| 2 | Bhekho Drainage System | 830 Cs |
| 3 | Bhudhi Nallah Draiange System including P.R.K. Drainage System. | 4200 Cs |
| 4 | Halki Nallah Drainage System. | 100 Cs |
| 5 | D-1, D-2 & D-3 Drainage System | 70 Cs |
| 6 | Dhunni Drain, Majra , Chakrian and Mojianwal Drain (independent Drainage System) | 307 Cs |
| Total |  | 6067 Cs |

JHELUM BASIN.

| Sr.No. | Drainage System. | Discharge. |
| --- | --- | --- |
| 1 | Mona Bhera Drainage system | 750 Cs |
| 2 | Haria Drainage System | 400 Cs |
| 3 | Bahauddin Kot Baloach Drainage System. | 350 Cs |
| 4 | Malakwal Drainage System (UJC) parallel Drainage System U/S and D/S Khokhra Heaed Works. | 42 Cs |
| 5 | Parallel Drains along UJC | N/A |
| Total |  | 1542Cs |
| Grand Total |  | 7609 Cs |

Presently there are 11 Nos. Drainage System in Drainage Division

Mandi Bahauddin:-

| Sr.No. | Name of System. | Length (In Miles) | Capacity (In Cs). |
| --- | --- | --- | --- |
| 1 | Bhudhi Nallah Draiange System including P.R.K. Drainage System. | 264.73 | 4200 |
| 2 | Bhekho Drainage System | 98.48 | 830 |
| 3 | Mona Bhera Drainage system | 68.69 | 750 |
| 4 | Miana Gondal Drainage System | 40.56 | 560 |
| 5 | Malakwal Drainage System | 6.40 | 42 |
| 6 | Bahauddin Kot Baloach Drainage System. | 22.11 | 350 |
| 7 | Haria Drainage System. | 22.92 | 400 |
| 8 | Halki Nallah Drainage System. | 9.86 | 100 |
| 9 | D-1, D-2 & D-3 Drainage System | 5.28 | 70 |
| 10 | Independent Drains. | 27.91 | 307 |
| 11 | Parallel Drains along UJC | 60.00 | NA |

  - ADMINISTRATIVE SETUP.

| Sr.No. | Name of Drainage Sub Divisions | District | Tehsil | Drains Length (In Miles) |
| --- | --- | --- | --- | --- |
| 1 | Bahauddin Drainage Sub Division | Mandi Bahauddin/ Sargodha | Mandi Bahauddin, Malakwal, Bhalwal | 259.96 |
| 2 | Phalia Drainage Sub Division | Mandi Bahauddin/Gujrat | Phalia, Gujrat | 206.65 |
| 3 | Rasul Drainage Sub Division | Mandi Bahauddin, Gujrat, Jhelum & Mirpur | Mandi Bahauddin, Kharian, Sarai Alamgir, Mirpur | 176.19 |
| 4 | Phalia Tube Well Sub Division | Mandi Bahauddin, Gujrat, | Mandi Bahauddin, Gujrat, | --- |
| Total= |  |  |  | 642.80 |

ORGANIZATION CHART                                                                                                                                                                                               DRAINAGE DIVISION MANDI BAHAUDDIN.

Executive Engineer

Drainage Division

Mandi Bahauddin.

Sub Divisional Officer Bahauddin

Drainage Sub Division.

Sub Divisional Officer Phalia

Drainage Sub Division.

Sub Divisional Officer Rasul

Drainage Sub Division.

Bahauddin Section

Rukkan Section

Machiana  Section

Rasul            Section.

Malakwal    Section

Kollian         Section

Mamdana    Section

Hellan           Section

Budhi Nallah    Section

TOD Phalia

CHAPTER-2

FLOOD PROTECTION AND RIVER TRAINING WORKS

2.1	DESIGN PARAMETERS OF RIVER TRAINING WORKS

Apart from Maintenance of Drainage System this Division is responsible for maintenance of 3 flood bunds with detail as under:

| Sr.No | Name of Bund | Length of Bund | Top width | Side slopes | Free Board from H.F.L | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Flood Embankment along Mona Bhera Drain. | 35000 ft | 20 ft | 2:1 on Either side | 5 ft | _ |
| 2 | Malakwal Flood Bund. | 3500 ft | 12 ft | 2:1 on Either side | 5 ft | _ |
| 3 | Flood Bund along Budhi Nallah Drain RD.0-18500. | 18500 ft | 25 ft | 2:1 on Either side | 5 ft | _ |

2.2	LOCATION MAP

LOCATION MAP

CHAPTER-3

BREIF HISTORY OF PAST FLOOD EVENTS

The brief description of each major system affected by the flood is under: -

  - P.R.K DRAINAGE SYSTEM.

The P.R.K Drainage system comprises of 16 No. Tributary Drains along with P.R.K Main drain. The P.R.K main drain starts from pubby hills and has discharge at outfall as 2750 Cs and Bed width 100 feet. In addition to its out fall structure, there is an intake structure on R.Q.Link with intake capacity of 16000 Cs to meet with flood situations

MONA BHERA DRAINAGE SYSTEM.

Mona Bhera Drainage system consists of 8 No. tributary drains along with left Bund Mona Bhera main drain with discharge capacity of 750 Cs at out fall. Bed width is 28 feet at out fall and 4.00 feet at source. RD. 0-19000, in addition to carrying storm water also acts as flood protection bund to protect Bhera Town. For this purpose N.H.A constructed flood bund during construction of Motorway.

  - MALAKWAL DRAIN.

Left Bank of Malakwal drain in reach RD. 0-3500 is maintained as flood

Embankment to save Malakwal Town. The length of flood embankment is 3500 feet having top width 12 feet with side slop 2:1 on either sides.

BUDHI NALLAH DRAINAGE SYSTEM.

Budhi Nallah Drainage system was remodeled by WAPDA during 1977. It covers a lot of catchment area right from U.J.C up to Qadirabad Town, being outfall for P.R.K Drainage System at RD. 99000 of R.Q.Link through a Syphon. As independent system it consist of 19 No. drains in addition to main drain with discharge capacity of 4200 Cs and Bed Width of 130, at outfall. This drain consist of heavy structures and protects huge area against flood during Monsoon. During flood 1995 two breaches occurred and Budhi Nallah Flood Bund Damaged between  RD. 3+000-4+000, 6+000-7+000 & 13+000-18+500 during Flood Season 2014.

CHAPTER-4

DESIGN DATA HISTORIC PEAK FLOOD DATA AND

PREVIOUS 5 YEARS FLOOD DATA OF HEAD WORKS/ BARRAGES AND OR OTHER CONTROL POINTS.

4.1	FLOOD LIMITS FIXED BY FLOOD COMMISSION .

| FLOOD LIMITS | RIVER JHELUM |  | RIVER CHENAB |
| --- | --- | --- | --- |
|  | Discharge Ranges (Cusecs) |  |  |
| Low Flood | 75,000-1,10,000 | 1,10,000-1,50,000 |  |
| Medium Flood | 1,10,000-1,50,000 | 1,50,000-2,25,000 |  |
| High Flood | 1,50,000-2,25,000 | 2,25,000-3,00,000 |  |
| Very High Flood | 2,25,000-3,00,000 | 4,00,000-6,00,000 |  |
| Exceptionally High Flood | Above 3,00,000 | Above 6,00,000 |  |

4.2	TIME LAGS OF FLOODS

The flood from Mangla to Railway Bridge Jhelum reaches in a seven hours after traversing a distance of 20 miles whereas from Railway Bridge to Rasul Barrage is also seven hours.

4.3	HIGHEST FLOODS (River Jhelum at Rasul)

| Date | Discharges (Cs) | Pond Level (ft.) |
| --- | --- | --- |
| 02-07-1959 | 8,70,000 | 719.00 |
| 11-07-1960 | 8,60,000 | 721.80 |
| 10-09-1992 | 9,52,170 | 721.70 |
| 27-08-1997 | 5,49,598 | 716.00 |
| 06-09-2014 | 5,16,387 | 720.40 |

4.4	PEAK DISCHARGES

| Date | Discharges (Cs) | Pond Level (ft.) |
| --- | --- | --- |
| 11-07-1960 | 8,60,000 | 721.80 |
| 10-07-1972 | 1,05,150 | 716.00 |
| 09-08-1973 | 2,69,330 | 716.80 |
| 23-08-1975 | 1,23,034 | 715.00 |
| 04-08-1976 | 2,69,330 | 715.00 |
| 12-08-1978 | 93,382 | 719.00 |
| 19-08-1980 | 73,583 | 719.00 |
| 26-07-1981 | 1,04,424 | 716.00 |
| 06-08-1983 | 97,251 | 719.00 |
| 15-08-1984 | 95,905 | 719.00 |
| 08-08-1986 | 1,13,528 | 717.00 |
| 12-06-1987 | 1,27,950 | 717.00 |
| 22-07-1988 | 1,54,389 | 718.00 |
| 01-08-1989 | 1,53,174 | 718.00 |
| 18-08-1990 | 75,259 | 719.00 |
| 11-06-1991 | 97,704 | 719.00 |
| 10-09-1992 | 9,52,170 | 721.70 |
| 14-07-1993 | 1,00,998 | 718.00 |
| 23-08-1995 | 1,29,392 | 717.00 |
| 28-7-1994 | 2,85,071 | 715.00 |
| 27-06-1996 | 1,36,712 | 716.00 |
| 27-08-1997 | 5,49,598 | 716.00 |
| 18-05-1998 | 73,597 | 719.00 |
| 17-07-2005 | 92,246 | 719.00 |
| 13-07-2006 | 1,65,947 | 720.85 |
| 15-08-2007 | 23,608 | 719.00 |
| 03-05-2008 | 27,515 | 719.00 |
| 16-08-2009 | 69,952 | 719.10 |
| 30-07-2010 | 2,63,795 | 718.00 |
| 17-09-2011 | 96,864 | 718.00 |
| 05-08-2012 | 43,135 | 719.00 |
| 24-08-2013 | 35,325 | 719.00 |
| 06-09-2014 | 5,16,387 | 720.40 |
| 27-07-2015 | 99,103 | 718.20 |
| 30-08-2016 | 40,425 | 718.10 |
| 02-05-2017 | 60,723 | 719.20 |
| 02-10-2018 | 43,135 | 719.20 |
| 18.06.2019 | 91,904 | 719.20 |
| 26.03.2020 | 91,904 | 720.40 |
| 02.06.2021 | 31,420 | 716.00 |
| 16.06.2022 | 23610 | 719.10 |
| 08-09-2023 | 35325 | 719.00 |
| 30-08-2024 | 66960 | 719.00 |

CHAPTER-5

FLOOD FIGHTING STRATEGY

- P.R.K Drain.

Upper reach of this drain falls in the control of two Sub Engineer i.e. Rasul and Kolian section of Rasul Drainage Sub Division, while lower reach in Hellan section of Phalia Drainage  Sub Division.

The structure of P.R.K in Kolian Drainage Section at RD.31 (U/S) particularly suffered damage, due to rain/flood during the year 2011-12. This reach will be watched particularly during high flood. The existing establishment will watch the structure of P.R.K Drain Extra requirement for men and material is explained in the next chapter.

- MONA BHERA DRAINAGE SYSTEM.

Flood camps will be set up at RD.6-8 and RD.19 Mona Bhera Drain during coming flood season to control any emergent situation. Sub Engineer Malakwal section whose Headquarter is at Malakwal embankment will control the situation also with the help of his sectional establishment and work charge establishment (in case of emergency) in coming flood season

- MALAKWAL DRAIN.

Flood camps will be set up at RD. 3500 Malakwal Drain during coming flood season to control any emergent situation. Malakwal drain will be looked after by Sub Engineer Malakwal section, whose Headquarter is at Malakwal and he will also supervise embankment along drain with the help of his sectional establishment and work charge establishment ( in case of emergency ) in coming flood season.

- BUDHI NALLAH DRAIN.

In view of last experience, it is essential that watching be carried out in case there is spill of River Chenab. Flood camp will be set up at RD-0 of Budhi Nallah and the Sub Engineer Budhi Nallah Section will supervise the operation during high flood, with the help of regular / work charge establishment.

- BHEKHO OUTFALL LOWER DRAIN.

As experienced during the previous years, the bank of Bhekho outfall lower drain in reach RD. 0-10000 remain under the direct hit of spill of the River Chenab. This will also be required to be kept under constant watch and Sub Engineer Mamdana will take care of the activities pertaining to flood in case of emergency (Very High Flood).

CHAPTER-6

FLOOD DAMAGES RESTORATION WORKS

Flood flows passed during different years in both the rivers have affected the following systems. These systems were repeatedly damaged / over toped since the year 1976. The damages occurred to system were restored under M&R grant and also flood damages restoration project. No major flood restoration work has been executed during last year at any Flood Bund in Drainage Division Mandi Bahauddin.

  - P.R.K Drainage System.

  - Mona Bhera Drainage System.

  - Budhi Nallah Drainage System.

  - Bhekho Drainage System.

  - Malakwal Drainage System.

The details regarding damages & flood restoration project since 1992 summarized as under: -

FLOOD OF 1992.

- MONA BHERA DRAIN.

Extra ordinary high flood of 1992 effected banks from RD. 0-48 with Pacca structure i.e. V.R. Bridge RD. 6214, 8658 & 5 no. Inlets were damaged which were repaired under flood restoration project 1992.

- MALAKWAL DRAIN / FLOOD BUND.

This Bund was affected from RD.0-3500 due to exceptionally high flood 1992, which was repaired under Flood Restoration Project.

- BAHAUDDIN KOT BALOACH DRAINAGE SYSTEM.

Bahauddin Outfall Drain RD. 0-8300 and Kot Baloach Drain RD. 0-60000 were seriously damaged and were repaired under flood restoration damages project 1992.

- BHEIKHO DRAINAGE SYSTEM

Extra ordinary flood from River Chenab during 1992 damaged the banks of Bhekho outfall Drain RD. 0-25. Beside this, stone pitching at outfall part was also damaged, which was under taken in flood restoration works.

- BUDHI NALLAH DRAIN.

The bank of Budhi Nallah outfall drain RD. 0-18500 was damaged. The reach RD. 0-10 was also affected, which was restored under flood restoration 1992.

- P.R.K DRAIN.

The extra ordinary rains in the catchments and the back water of R.Q.Link damaged the bank from RD. 110-111 & also V.R. Bridge RD 63-72 along with some inlets. These works were taken under the flood restoration 1992.

FLOOD 1995.

- MONA BHERA DRAIN.

There was extra ordinary rain water and back water in River Jhelum. Caused breach at RD. 7-8 left Mona Bhera Drain, which effected the Bhera Town and also adjoining abadies bank at RD. 0-19. This work was taken under Para 2.89 P.W.D code and was completed accordingly.

- BHIKHO DRAINAGE SYSTEM.

Due to supper flood in River Chenab the Bank of Bhekho outfall lower drain RD. 0-10 was damaged besides the sloping spur. Mole head & pitching were also damaged. Some huge cuts in flood embankment were also occurred at the outfall drain, which were restored under Para 2.89 P.W.D code accordingly.

FLOOD SEASONS 2018.

No flood damages restoration work was carried out during the year 2018. Only 43135Cs water has been passed Down Stream of Rasul Barrage during Flood season.  However, normal repair works such as rain cuts and gharas have been carried out partially.

FLOOD SEASONS 2019.

No flood damages restoration work was carried out during the year 2019. Only 91904Cs water has been passed Down Stream of Rasul Barrage during Flood season.

FLOOD SEASONS 2020.

No flood damages restoration work was carried out during the year 2020. Only 91904Cs water has been passed Down Stream of Rasul Barrage during Flood season.

FLOOD SEASONS 2021.

No flood damages restoration work was carried out during the year 2021. Only 31420 Cs water has been passed Down Stream of Rasul Barrage during Flood season.

FLOOD SEASONS 2022.

No flood damages restoration work was carried out during the year 2022. Only 23610 Cs water has been passed Down Stream of Rasul Barrage during Flood season 2022.

FLOOD SEASONS 2023.

No flood damages restoration work was carried out during the year 2023. Only 35325 Cs water has been passed Down Stream of Rasul Barrage during Flood season 2023.

FLOOD SEASONS 2024.

No flood damages restoration work was carried out during the year 2024. Only 66960 Cs water has been passed Down Stream of Rasul Barrage during Flood season 2024.

CHAPTER-7

FLOOD FIGHTING WATCHING ARRANGEMENT

7.1	PRE FLOOD ARRANGEMENT

The necessary repairs of the flood bund works will be carried out before start of flood season. Moreover, arrangements to establish flood camps will be ensured and deficient quantity of flood fighting material and equipment will be procured before time for watching during flood season.

The flood fighting material will be kept in stores which are located in the premises of Canal Colony at Mandi Bahauddin under the control/ custody of the Sub Engineers concerned.

7.2	WATCHING ESTABLISHMENT

Following watching establishment will be engaged during the flood season for watching and general maintenance of flood works on monthly basis. However, in case of very high flood or in real emergency, the strength of watching establishment will be increased with the express permission of Executive Engineer, Drainage Division Mandi Bahauddin.

| Site | LENGTH | Flood Limits |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal Flood Stage |  | Low Flood Stage |  | Medium Flood Stage |  | High Flood Stage |  | Very High Flood Stage |  | Exceptionally High Flood Stage |  |
|  |  | Man | Shift | Man | Shift | Man | Shift | Man | Shift | Man | Shift | Man | Shift |
| Mona Bhera Flood Bund. | 6.628 (Mile) | No Extra Labour will be employed |  | The Flood water flows witin River bank No extra labour will be employed |  | 21 | 3 Shift | 28 | 3 Shift | 42 | 3 Shift | 63 | 3 Shift |
| Malakwal Flood Bund. | 0.662 (Mile) |  |  |  |  | 3 | 3 Shift | 4 | 3 Shift | 6 | 3 Shift | 9 | 3 Shift |
| Budhi Nallah Flood Bund. | 3.503 (Mile) |  |  |  |  | 12 | 3 Shift | 16 | 3 Shift | 24 | 3 Shift | 36 | 3 Shift |
| Total= |  |  |  |  |  | 36 |  | 48 |  | 72 |  | 108 |  |

| Sr. #. | Name of drain. | Reach Likely to spill. | Name of village likely to be effected. | Drainage Sub Division | Civil Sub Division. | Battle Station. |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Mona Bhrea drain. | RD.0-19/L & R.& 38-48/L & R | Bhera Town, Hajka, Pindi Hattar& Rakh Bher pur. | Bahauddin Drainage Sub Division. | Bhalwal & Mandi Bahauddin. | RD.5-6 |
| 2 | Malakwal drain. | RD.0-35 /L & R | Malakwal | Bahauddin Drainage Sub Division | Malakwal. | RD.-1 |
| 3 | Budhi Nallah Drainage System. | RD.0-65000 | Qadirabad, Saida & Jaggo | Phalia Drainage Sub Division | Phalia | RD.18500 |
| 4 | Main Phalia Drain. | RD.16000-17500/L & R. | Phalia Town | -do- | -do- | RD.16500 |
| 5 | Bheikho Drainage System | RD.0-25000 | Miana Hazara & Takhat Hazar etc. | -do- | -do- | RD.0 |
| 6 | P.R.K drain | RD.0-10 & RD.40-60 & RD.78-88 | Addosarwani, Sheikh pur, Haslanwala, Chak Lashkari, Wariachanwala. | Rasul & Phalia Drainage Sub Division | i.Phalia & Kharian. | RD.88 |

7.3	ARRANGEMENT AT SENSATIVE SITE

Necessary flood fighting material will be kept ready at vulnerable site as per item 7.4.

7.4	WATCHING MATERIAL.

| Sr. No. | Item | Required Qty | Available Qty. | Balance | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Gunny bags / F.C.Bags | 1000 No. | 400 No. | 600 No. |  |
| 2 | Sutli | 10 Kg | 03 Kg | 07 Kg |  |
| 3 | Axes with Handles | 30 No. | 10 No. | 20 No. |  |
| 4 | Sewing needles | 50 No. | ---- | 50 No. |  |
| 5 | Kassies with Handles | 75 No. | 20 No. | 55 No. |  |
| 6 | Killas | 100 No. | 30 No. | 70 No. |  |
| 7 | LED Torches | 10 No. | 02 No. | 08 No. |  |
| 8 | Umbrella | 20 No. | 6 No. | 14 No. |  |
| 9 | Polythene Bags (50Kg) | 2000 No. | 200 | 1800 |  |
| 10 | Ballies 10 ft long | 100 No. | ----. | 100 No. |  |
| 11 | Baskets | 150 No. | 25 No. | 125 No. |  |
| 12 | Manila rope | 200 lft | ------ | 200 lft |  |
| 13 | G.I Wire | 150 kg | 50 kg | 100 kg |  |
| 14 | Wooden Mallets | 10 No | 02 No. | 08 No. |  |
| 15 | Rubber Gum Shoes | 30Pairs | -- | 30Pairs |  |
| 16 | Rain Coats | 30 No. | 6 No. | 24 No. |  |
| 17 | Mosquito Nests | 20 No. | -- | 20 No. |  |
| 18 | Bamboo | 100 No. | 20 No. | 80 No. |  |
| 19 | Life belts / Jackets | 20No. | 05 No. | 15 No. |  |
| 20 | Water Glasses | 50 No. | 10 No. | 40 No. |  |
| 21 | Water Cooler Large) | 05 No. | 1 No | 04 No. |  |
| 22 | Tea Sets | 3 No. | -- | 3 No. |  |
| 23 | Search Lights (100 W) | 03 No. | -- | 03 No. |  |
| 24 | Portable generators | 2 No. | --- | 02 No. |  |
| 25 | Plastic Chair with Table | 03 Set |  | 03 Set |  |
| 26 | Tents | 3 No. | 1 No. | 2 No. |  |
| 27 | Super Petrol for running Generator | 200 Litre | -- | 200 Litre |  |
| 28 | Polythene Bags (500Kg) | 200 No. | -- | 200 No. |  |
| 29 | Polythene Bags (1000Kg) | 150 No. | -- | 150 No. |  |
| 30 | Black Polythene Sheet | 15000 Sft | -- | 15000 Sft |  |
| 31 | Lanterns | 10 No. | 02 No. | 08 No. |  |
| 32 | Kerosene oil | 20 ltre | --- | 20 ltre |  |
| 33 | Match Boxes | 05 Dozen | 01 Dozen | 01 Dozen |  |
| 34 | Patha Ban | 50 kg |  | 50 kg |  |

7.5	ARRANGEMENT FOR SOUNDING AND PROBING

This item relates to barrage / Head works & Canal Divisions.

7.6	LIGHTING ARRANGEMENT

Sufficient number of emergency lights, lanterns and patromaz lamps has already been provided. Petrol Generators has been arranged and made available for lighting arrangements.

7.7	RATION ARRANGEMENT

It is the responsibility of District Administrator Department to provide the ration arrangements.

7.8	POL ARRANGEMENT FOR VEHICLE

Respective Deputy District Office (Revenue) will be requested by the Sub Divisional Officer incharge to provide help for reserve at the nearest Petrol Pumps to keep the vehicles on. Any obstruction found there, will be pointed out by the Sub Divisional Officer.

7.9	TRANSPORTATION

Already allocated vehicles to the Division will be used during flood season. In case of exceptionally high flood in River Jhelum, District Coordination Officer, will be requested to supply additional vehicles for the Supervisory staff and for transportation of labour as well as flood fighting material to the sites of work.

7.10	LAW AND ORDER

There is no likehood of any interruption by any one except cutting of banks of channels to reduce heading up. In case of necessity, District Coordination Officer Mandi Bahauddin will be contacted by the Executive Engineer Drainage Division Mandi Bahauddin to take decision at site and Sub Divisional Officer Bahauddin / Phalia will seek the help of Deputy District Officer, Revenue / Assistant Commissioner Mandi Bahauddin in case of emergency.

7.11	MEDICAL ARRANGEMENT FOR LABOUR

It will be responsibility of Health Department to provide Medical Facilities in flood areas.

7.12	LIAISON WITH OTHER DEPARTMENTS

Executive Engineer Drainage Division Mandi Bahauddin will keep liaison with the Civil and Army Authorities and will keep the Superintending Engineer L.J.C Jhelum fully apprised of the flood situation in Drainage Division.

7.13	ROLE OF ARMY

Liaison with the Army Authorities will be maintained during floods and if required necessary help will be sought at appropriate level.

7.14	DUTIES OF TELEPHONE ATTENDANT

Permanent telephone attendants are already available in the office of the Executive Engineer Drainage Division Mandi Bahauddin for communication of urgent messages.

| Sr. No | Name of Telephone Attending Official | Contact No. | During |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Anwar ul Haq | 0546-506302 | 9:00 Am | 3:00 Pm |
| 2 | Abdul Rehman | 0546-506302 | 3:00 Pm | 08:00 Pm |
| 3 | Iftikhar Ahmed | 0546-506302 | 8:00 Pm | 10:00 Pm |
| 4 | Naveed Aslam | 0546-506302 | 10:00 Pm | 6:00 Am |
| 5 | Farooq Ahmad | 0546-506302 | 6:00 Am | 9:00 Am |

Permanent telephone attendants are already available in the office of the Executive Engineer Drainage Division Mandi Bahauddin for communication of urgent messages.

  - WIRELESS ARRANGEMENT

Wireless system will be installed w.e.f. 01.06.2025

in the vehicle of Executive Engineer Drainage Division Mandi Bahauddin No.SGQ-780, Sub Divisional Officer Phalia Drainage Sub Division Suzuki MIA-7362  & Sub Divisional Officer Bahauddin Drainage Sub Division Jeep No.GTB-7195.

CHAPTER-8

DETAIL OF ENCROACHMENT

| Total Encroachments | Removed Encroachments | Balance Encroachments |
| --- | --- | --- |
| NIL | N/A | NIL |

There is no encroachment on flood bunds of Drainage Division Mandi Bahauddin.

CHAPTER-9

DUTY ROSTER OF FLOOD/ FLOOD FIGHTING PROGRAM.

| Key Personal | Battle Stations | Drains & Flood Embankments | Name of Drainage Sub Division. |
| --- | --- | --- | --- |
| Sub Engineer Hellan | RD-0 | P.R.K drain | Phalia Drainage Sub Division. |
| Sub Engineer Kolian | RD.88 | P.R.K Drain | Rasul Drainage Sub Division |
| Sub Engineer Malakwal | RD.5-6 | Mona Bhera Flood Embankment | Bahauddin Drainage Sub Division |
| Sub Engineer Malkwal | RD.19 | Mona Bhera Flood Embankment | Bahauddin Drainage Sub Division. |
| Sub Engineer Mamdana | RD.0 | Bheikho Outfall Lower drain. | Bahauddin Drainage Sub Division |
| Sub Engineer Budhi Nallah. | RD.18500 | Bheikho Outfall Upper drain. | Phalia Drainage Sub Division |
| Sub Engineer Budhi Nallah | RD-0. | Bheikho Outfall Upper drain | Phalia Drainage Sub Division |
| Sub Engineer Budhi Nallah | RD.16500 | Main Phalia Drain. | Phalia Drainage Sub Division |

CHAPTER-10                                                                                    EMERGENCY TELEPHONE NUMBERS.

| Sr. No. | Designation | Office | Residence / Mobile |
| --- | --- | --- | --- |
| 1. | Minister Irrigation. | 042-99212809-10 | 042-99212809-10 |
| 2. | Secretary, Irrigation. | 042-99212117 042-99212118 | 042-99201053 |
| 3. | Additional Secretary (Tech :) | 042-99212128 | 042-99212128 |
| 4. | Additional Secretary (OP) | 042-99212123 | 042-99212123 |
| 5. | Deputy Secretary (Admn) | 042-99212224 | 042-99212224 |
| 6. | Deputy Secretary (OP) | 042-99212127 | 042-99212127 |
| 7. | Deputy Secretary (Dev) | 042-99212129 | 042-99212129 |
| 8. | Section Officer Flood | 042-99212134 | 042-99212134 |

SARGODHA IRRIGATION ZONE

| 1. | Chief Engineer, Irrigation Sargodha | 048-9230445 | 048-9230451 0322-4486727 |
| --- | --- | --- | --- |
| 2. | Executive Engineer (OP), Sargodha | 048-9230446 | 048-3740050 0302-9079639 |
| 3. | Superintending Engineer, Drainage Circle Sargodha | 048-9230450 | 0333-8161746 |
| 4. | Executive Engineer, Drainage Divn: M.B.Din. | 0546-506302 | 0333-4915939 |
| 5. | Executive Engineer, Rasul LJC Divn: Rasul. | 0546-553211 | 0340-7159459 |
| 6. | Sub Divisional Officer Phalia Drainage Sub Division M.B.Din | 0546-506302 | 0308-6289786 |
| 7. | Sub Divisional Officer, Bahauddin Drg: Sub Divn: | 0546-506302 | 0308-6289786 |

DISTRICT ADMINISTRATION M.B.DIN

| 1. | Deputy Commissioner M.B Din | 0546-504220 | 0546-504200 |
| --- | --- | --- | --- |
| 2. | District Police Office M.B Din | 0546-502324 | 0546-502324 |
| 3. | District Officer Revenue M.B Din | 0546-506488 | 0546-506488 |
| 4. | District Flood Control Officer M.B Din | 0546-504220 | 0546-504220 |
| 5. | Assistant Commissioner Mandi Bahauddin. | 0546-503850 | 0546-503850 |
| 6. | Assistant Commissioner Malikwal | 0546-591276 | 0546-591276 |
| 7. | Assistant Commissioner Phalia | 0546-596460 | 0546-596460 |
| 8. | Rescue 1122 Mandi Bahauddin | 1122 | 1122 |

FLOOD CONTROL CENTRE PUNJAB, LAHORE

| 1. | Flood Warning Centre 46- Jail Road Lahore | 042-7572091-93 | 042-7572092 |
| --- | --- | --- | --- |
| 2. | Flood Cell I & P Secretariat Lahore | 042-9212147 | Fax:9212116 |
| 3. | Flood Emergency Board of Revenue | 042-9204408 | 042-9210555 |
| 4. | Chief Engineer D & F Lahore | 042-99233551 042-35291304 | 042-99233551 |
| 5. | Director Flood D & F Zone Lahore | 042-99231614 | 042-99231614 |

MILITARY ENGINEER’S TELEPHONE NUMBERS

| 1. | Headquarter Engineer 30-Corps Gujranwala Cantt. | 0553-269/2353 |
| --- | --- | --- |
| 2. | 11-Engineer Battalion Sialkot Cantt. | 052-571/5186 |
| 3. | Headquarter Engineer 1-Corps Kharian Cantt | 053-59/2364 |

CHAPTER-11

STANDARD OPERATING PROCEDURE (SOP) FOR BREACHING SECTION.

11.1	HISTORY OF BREACHING SECTION

There is no breaching section in this Division.

11.2	LOCATION, DESIGN, QUALITY AND VARIETY OF THE 	EXPLOSIVE REQUIRED FOR DETONATION

Not applicable.

11.3	ARRANGEMENT OF EXPLOSIVE AND SECURITY OF 	EXPLOSIVE STORES

Not applicable.

11.4	LIST OF THE SECURITY STAFF ALONGWITH DETAIL OF 	THEIR TRAINING ETC.

Not applicable.

11.5	DETAIL OF MACHENICAL MEANS AS A STANDBY 	ARRANGEMENT IN CASE OF DETONATION FAILURE.

Not applicable.

11.6	DUTY ROSTER IN CASE OF CRITICAL SITUATION

Not applicable.

11.7	BREACHING COMMITTEE WITH THEIR ACTION PLAN

Not applicable

.

11.8	LIST OF THE VILLAGES LIKELY TO BE INUNDATED IN CASE 	OF BREACH

Not applicable.

11.9	ANNOUNCEMENT AND DETAIL OF EVACUATION 	ARRANGEMENT

Not applicable.

11.10	 DETAIL OF COORDINATION WITH CIVIL / ARMY 	AUTHORITIES.

Not applicable.

11.11 	PARALLEL COMMUNICATION ARRANGEMENTS.

Not applicable.

11.12 	INDEX PLAN.

Copy attached……

Sub Divisional Officer					Sub Divisional Officer

Bahauddin Drainage Sub Division				Phalia Drainage Sub Division:

Mandi Bahauddin.						Mandi Bahauddin.

Executive Engineer					Superintending Engineer

Drainage Division					     	Sargodha Drainage Circle

Mandi Bahauddin   						         Sargodha.

Chief Engineer

Irrigation Sargodha Zone

Sargodha.

PART-B

CHAPTER-12

VULNERABLE SITES ON FLOOD BUNDS/ STRUCTURES

  - APPREHENDED BREACH SITES IN FLOOD BUNDS/ STRUCTURES

There are no Apprehended Breach , Sites in Flood Bunds

  - OPERATION OF BREACHING SECTIONS

There are no breaching sections in drainage division Mandi Bahauddin.

  - BREACHES DUE TO RISE OF WATER, DETERRIORATION OF FLOOD BUNDS

No such occurrence.

CHAPTER-13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITES LISTED ABOVE.

EMERGENCY CONTINGENCY PLAN FOR SITE

NO.1 (MONA BHERA FLOOD BUND RD.5-8).

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan Showing route of Flood water coming out of the breach for site No.01 i.e RD.5+000 to 8+000 plan attached.

Annexure-A.

MONA BHERA FLOOD BUND.

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED (TO BE	SHOWN ON THE MAPS), THE EXTENT AND DEPTH OF ANTICIPATED INUNDATIONS, EMERGENCY PROTECTION MEASURES AND EVACUATION PLAN.

The tentative area inundated by the flood water will be 40 square KM as shown on attached plan at Annex – “A- Villages/Abadies are as under:-

Hjka, Ali Pur Syedan, Bhera, Fateh Ghar, Islam Pur Lokri, Gaga, Chant, Thai, Warah, Khrhn, Kot Ahmed Khan Qauehat, Gund Pur, Miani, Khurd Kot & Hazur Pur.

13.3	STRATEGY AND ACTION TAKEN  BE EXPLAINED IN DETAIL.THIS MAY INCLUDE

Mona Bhera Flood Bund fall in Malakwal section constructed by NHA for the protection of Bhera City along left bank of River Jhelum, which are safe against high Flood in river Jhelum.

Due to high flood in River Jhelum backwater effect / flooded water spread in the adjoining areas along Mona Bhera Flood embankment.

In case of High / exceptionally high flood in River Jhelum, mishap may occur at  Mona Bhera flood embankment  RD.5+000-8+000 and Flood water may enter and damage Bhera Town,  Fateh Ghar, Ali Pur Syedan, Hajka Sharif, Alampur, villages and inhabitants of these villages will have to face problematic situation. Therefore special attention should be paid for protection of Bund / Structures.

    - ARRANGEMENTS.

In view of the experience of the past flood history day & night watching will required to control the flood situation during rainy season.

Constant vigilance and caution will be required at all levels particularly on the spots for relief and flood fighting on drains, flood embankments.

13.3.2. ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

A tent would be at site and all the resources of manpower and material required for controlling the situation would be immediately shifted from the shelter huts to this tent which would then become the emergency camp for carrying out all the flood fighting operations the Sub Engineer nominated by the Executive Engineer.

MONA BHERA FLOOD BUND.

| Sr. #. | Key Personal | Name of drain. | Battle Station | Name of village likely to be effected. | Drainage Sub Division | Civil Sub Division. |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Sub Engineer Malakwal | Mona Bhrea drain. | RD.5-6 | Bhera Town, Hajka, Pindi Hattar& Rakh Bher pur. | Bahauddin Drainage Sub Division. | Bhalwal & Mandi Bahauddin. |

WATCHING ESTABLISHMENT.

Following watching establishment shall be engaged during the flood season for watching and general maintenance of flood works on monthly basis. However, in case of very high flood or in real emergency, the strength of watching establishment will be increased with the express permission of Executive Engineer, Drainage Division Mandi Bahauddin.

| Sr.# | Name of The Structure | For maintenance & Petrol during flood season. |  |  | In case of very high flood. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Beldar | Mate | Gauge Reader | Beldar | Mate | Gauge Reader |
| 1. | Mona Bhera Flood Bund. | 8 | 1 | - | 8 | 1 | - |

13.3.3.	DUTIES OF OFFICERS/ OFFICIALS AND                                      THEIR CAMP SITES.

- Executive Engineer Drainage Division Mandi Bahauddin.

The Flood activities in the jurisdiction of Mandi Bahauddin Drainage Division have been divided in to one sub set sectors i.e. Sub Sector No.01. The incharge of Sub Sector is Sub Divisional Officers of Bahauddin Drainage Sub Division Mandi Bahauddin as per following details.

DETAIL OF CAMP SITES.

| Sr.No. | Name of Flood Bund. | Name of Officer | Name of Official | Camp RD. |
| --- | --- | --- | --- | --- |
| 1 | Mona Bhera Flood Bund RD.0-35000. | Tahir Mehmood S.D.O Bahauddin (Site Incharge) | Gulraiz Haider Sub Engineer Malakwal Section. | 5+400. |
| 2 | Malakwal Flood Bund RD.0-3500. | Tahir Mehmood S.D.O Bahauddin (Site Incharge) | Gulraiz Haider Sub Engineer Bahauddin Section. | 1+000 |
| 3 | Budhi Nallah Flood Bund RD.0-18500. | Tahir Mehmood S.D.O Phalia | Mansha Khan Sub Engineer Budhi Nallah Section. | 9+200 |

DETAIL OF CAMPS ON FLOOD BUNDS OF DRAINAGE DIVISION M.B.DIN.

- CAMP OF MONA BHERA FLOOD BUND RD.0+35000 / CAMP AT RD.5+400.

| Sr.No | Name of Official | Designation | Contact No. |
| --- | --- | --- | --- |
| 1 | Tahir Mehmood | Sub Divisional Officer | 0308-6289786 |
| 2 | Ghulraiz Ahmed | Sub Engineer | 0316-1776059 |
| 3 | Faisal Shahzad | Beldar | 0341-4376240 |
| 4 | Ghulam Mustafa | Beldar | 0344-6274158 |
| 5 | Muhammad Bashir | Beldar | 0308-8853855 |
| 6 | Mulazam Hussain | Beldar | 0345-8632511 |
| 7 | Naseer | Beldar | 0344-7683879 |

- CAMP OF MALAKWAL FLOOD BUND RD.0+3500 / CAMP AT RD.1+000.

| Sr.No | Name of Official | Designation | Contact No. |
| --- | --- | --- | --- |
| 1 | Tahir Mehmood | Sub Divisional Officer | 0302-6796071 |
| 2 | Gulraiz Haider | Sub Engineer | 0316-1776059 |
| 3 | Sohail Safdar | Beldar | 0333-8481183 |
| 4 | Muhammad Ameer | Beldar | 0342-8198478 |
| 5 | Sana ullah | Beldar | 0345-2032954 |
| 6 | Hassnain Ahmed | Beldar | 0346-8726670 |
| 7 | Shafqat Iqbal | Beldar | 0343-4789212 |
| 8 | Nazar Shah | Beldar | 0347-6172451 |

- CAMP OF BUDHI NALLAH FLOOD BUND RD.0+18500 / CAMP AT RD.9+200.

| Sr.No | Name of Official | Designation | Contact No. |
| --- | --- | --- | --- |
| 1 | Tahir Mehmood | Sub Divisional Officer | 0308-6289786 |
| 2 | Manshah Khan | Sub Engineer | 0300-7741766 |
| 3 | Muhammad Akram | Mistary | 0346-6483066 |
| 4 | Muhammad Arif | Mate | 0334-4917014 |
| 5 | Muhammad Anwar | Beldar | 0346-6483066 |
| 6 | Nizam Din | Beldar | 0343-4684799 |
| 7 | Bashir | Beldar | 0345-5766054 |
| 8 | Zaheer Ahmed | Beldar | 042-5948594 |

2.	SUB DIVISIONAL OFFICER BAHAUDDIN DRAINAGE SUB DIVISION.

Incharge Sub Sector No.01 will monitor the Flood Fighting /watching activities in the part area of Mona Bhera Flood Bund Tehsil Bhalwal and Malakwal Flood Bund on the left side of River Jhelum. The flowing Drainage System are directly effected by the River Chenab spill the Drainage function as Flood embankments and protection of Bhekho Outfall lower& Upper drain & Budhi Nallah Flood Bund.

| Keeping in view of vulnerable location of drains & Flood Plan, it is essential to take up flood preparedness activities like strengthening weak and Ghats sides clearance of Jungle, shrub from banks and slope and making them fit to face the ensuing floods. Therefore, a gang of one man per mile employee from 15 June to 15 September in additional to existing establishment of one Beldar in six mile length of drain. The terminology of different kinds of floods i.e. low Medium, High and exceptionally high and they will be taken as such in the light of any wireless message received respective warning centers. Sr.No. | Name of Bund. | Length of Bund | Top width | Side slope | Free Board from H.F.L |
| --- | --- | --- | --- | --- | --- |
| 1. | Flood Embankment along Mona Bhera drain. | 35000 | 20 feet | 2.1 on either side. | 5 feet |

| Key Personal | Battle Stations | Drains & Flood Embankments | Name of Drainage Sub Division. |
| --- | --- | --- | --- |
| Sub Engineer Malikwal | RD.19 | Mona Bhera Flood Embankment | Bahauddin Drainage Sub Division. |

13.3.4.  	DEPARTMENTAL MACHINERY AVAILABLE.

Department has no machinery of any kind. However, in emergency, machinery will be arranged by the Municipal Committee, District Government and Agricultural Engineering Department.

13.3.5.	MACHINERY AVAILABLE FROM PRIVATE SOURCE.

Suitable arrangement with Agricultural Engineering Department in time of emergency of closing leakage or breaches on flood bunds. Moreover excavator and tractor trolleys are available with local inhabitants of Mona Bhera Flood Bund, which will be deployed for necessary flood fighting emergency.

13.3.6.	FLOOD FIGHTING MATERIAL REQUIRED FOR

MONA BHERA FLOOD BUND.

| S.No. | ITEM | Unit | Required Quantity |
| --- | --- | --- | --- |
| 1 | Kassies with Handles | No. | 5 |
| 2 | Ballies | No. | 10 |
| 3 | Generator set. | No. | 1 |
| 4 | Petrol | Liter | 50 |
| 5 | M. Oil | Liter | 2.5 |
| 6 | Life Jacket | No. | 1 |
| 7 | Plastic Bags | No. | 125 |
| 8 | Electric Service Cable | Coils. | 1 |
| 9 | Umbrella | No | 3 |
| 10 | Gunny Bags with Sutli | No. | 10 |
| 11 | Patha Baan | KG | 10 |
| 12 | Shouldari | No | 1 |
| 13 | G. Wire | KG | 50 |
| 14 | Rechargeable Torch | No | 2 |
| 15 | Wooden Karies | No | 50 |
| 16 | Plastic Chair with Table | Set | 1 |
| 17 | Rain Coat | No. | 2 |
| 18 | LED Bulb with Accessories | No. | 30 |
| 19 | Basket | No | 5 |
| 20 | Tent Medium size | No | 1 |

    - FLOOD FIGHTING METERIAL AVAILABLE SITE NO.1

| Sr. No. | ITEM | Required | Available | Balance |
| --- | --- | --- | --- | --- |
| 1 | Kassies with Handles | 5 | 5 | -- |
| 2 | Ballies | 10 | 10 | -- |
| 3 | Generator set. | 1 | 1 | -- |
| 4 | Petrol | 50 | 50 | -- |
| 5 | M. Oil | 2.5 | 2.5 | -- |
| 6 | Life Jacket | 1 | 1 | -- |
| 7 | Plastic Bags | 125 | 125 | -- |
| 8 | Electric Service Cable | 1 | 1 | -- |
| 9 | Umbrella | 3 | 3 | -- |
| 10 | Gunny Bags with Sutli | 10 | 10 | -- |
| 11 | Patha Baan | 10 | 10 | -- |
| 12 | Shouldari | 1 | 1 | -- |
| 13 | G. Wire | 50 | 50 | -- |
| 14 | Rechargeable Torch | 2 | 2 | -- |
| 15 | Wooden Karies | 50 | 50 | -- |
| 16 | Plastic Chair with Table | 1 | 1 | -- |
| 17 | Rain Coat | 2 | 2 | -- |
| 18 | LED Bulb with Accessories | 30 | 30 | -- |
| 19 | Basket | 5 | 5 | -- |
| 20 | Tent Medium size | 1 | 1 | -- |

  - DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDINGS, CANALS AND DRAINAGE NETWORK.

There area where it is likely hood of mishap is a remote area, there is no such huge infrastructure like Grid Station, Sui Gas Station exists in this area. However towns, villages, roads and Electric Lines canals and drainage network will suffer due to spill out water of the River Jhelum.

- MONA BHERA FLOOD BUND.

Due to back water effected / flooded water spread in the adjoining areas along Mona Bhera flood embankment. In case of breach / damages in Mona Bhera flood bund, flooded water directly entered and hit Bhera Town and adjoining abadies Bhera Town, Fateh Gargh, Ali Pur Syddan, Hajka & Motor Way M-2.

EMERGENCY CONTINGENCY PALN FOR SITE NO.2

(MALAKWAL FLOOD BUND AT RD.0-3+500).

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF 	THE BREACH SUPPORTED WITH LEVELS.

Plan showing route of Flood water coming out of the breach for site No.02 i.e RD.0+000 to 3+500 plan attached.

Annexure-B.

MALAKWAL FLOOD BUND.

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED (TO BE	SHOWN ON THE MAPS), THE EXTENT AND DEPTH OF ANTICIPATED	INUNDATIONS, EMERGENCY PROTECTION MEASURES AND 	EVACUATION PLAN.

The tentative area inundated by the flood water will be 40 square KM as shown on attached plan at Annex – “B” Villages / Abadies are as under: -

Klas Sharif, Raipur, Bolabala, Chak Saida Zarin, Chak Saida Bala, Chak Nizan, Midh Pragna, Islam Pura & Malakwal.

13.3	STRATEGY AND ACTION TAKEN BE EXPLAINED IN DETAIL.THIS MAY INCLUDE

Malakwal Flood Bund falls in Malakwal Section and is constructed along left bank of River Jhelum, which is safe against normal flood in River Jhelum.

In case of exceptionally high flood in River Jhelum, Malakwal flood bund is insufficient to protect Malakwal City. Malakwal flood bund should be extended upto Victoria Bridge, which has already been recommended by a Committee comprising of Superintending Engineer Malsi and Executive Engineer Faisalabad.

    - ARRANGEMENTS.

In view the experience of the past flood history day & night watching will be required to control the flood situation during rainy season.

Constant vigilance and caution will be required at all levels particularly on the spots for relief and flood fighting on drains, flood embankments.

13.3.2. ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

A tent would be at site and all the resources manpower and material required for controlling the situation would be immediately shifted from the shelter huts to this tent which would then become the emergency camp for carrying out all the flood fighting operations the Sub Engineer nominated by the Executive Engineer.

B.                                       MALAKWAL FLOOD BUND.

| Sr. #. | Key Personal | Name of drain. | Battle Station | Name of village likely to be effected. | Drainage Sub Division | Civil Sub Division. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Sub Engineer Malakwal | Malakwal drain. | RD.1-2 | Malakwal | Bahauddin Drainage Sub Division | Malakwal. |

WATCHING ESTABLISHMENT.

Following watching establishment shall be engaged during the flood season for watching and general maintenance of flood works on monthly basis. However, in case of very high flood or in real emergency, the strength of watching establishment may be increased with the express permission of Executive Engineer, Drainage Division Mandi Bahauddin.

| Name of The Structure | For maintenance & Petrol during flood season. |  |  | In case of very high flood. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Beldar | Mate | Gauge Reader | Beldar | Mate | Gauge Reader |
| Malakwal Flood Bund. | 4 | - | - | 4 | - | - |

13.3.3.	DUTIES OF OFFICERS/ OFFICIALS AND                                      THEIR CAMP SITES

Executive Engineer Drainage Division Mandi Bahauddin.

The Flood activities in the jurisdiction of Mandi Bahauddin Drainage Division have been divided in to one sub set sectors i.e. Sub Sector No.01. The incharge of Sub Sector are Sub Divisional Officers of Bahauddin Drainage Sub Division Mandi Bahauddin as per following details.

DETAIL OF CAMP SITES.

| Sr.No. | Name of Flood Bund. | Name of Officer | Name of Official | Camp RD. |
| --- | --- | --- | --- | --- |
| 1 | Mona Bhera Flood Bund RD.0-35000. | Tahir Mehmood S.D.O Bahauddin (Site Incharge) | Manshah Khan Sub Engineer Malakwal Section. | 5+400. |
| 2 | Malakwal Flood Bund RD.0-3500. | Tahir Mehmood S.D.O Bahauddin (Site Incharge) | Gulraiz Haider Sub Engineer Sub Engineer Bahauddin Section. | 1+000 |
| 3 | Budhi Nallah Flood Bund RD.0-18500. | Tahir Mehmood S.D.O Phalia | Mansha Khan Sub Engineer Budhi Nallah Section. | 9+200 |

2.	SUB DIVISIONAL OFFICER BAHAUDDIN DRAINAGE SUB DIVISION.

In-charge Sub Sector No.01 will monitor the Flood Fighting /watching activities in the part area of Mona Bhera Flood Bund Tehsil Bhalwal and Malakwal Flood Bund on the left side of River Jhelum. The flowing Drainage System are directly effected by the River Chenab spill the Drainage function as Flood embankments and protection of Bhekho Outfall lower& Upper drain & Budhi Nallah Flood Bund.

| Sr.No. | Name of Bund. | Length of Bund | Top width | Side slope | Free Board from H.F.L |
| --- | --- | --- | --- | --- | --- |
| 1. | Malakwal Flood Bund | 3500 feet | 12 feet | Side slope | 5 feet |

Keeping in view of vulnerable location of drains & Flood Plan, it is essential to take up flood preparedness activities like strengthening weak and Ghats sides clearance of Jungle, shurb from banks and slope and making them fit to face the ensuing floods. Therefore, a gang of one man per mile employee from 15 June to 15 September in additional to existing establishment of one Beldar in six mile length of drain. The terminology of different kinds of floods i.e. low Medium, High and exceptionally high and they will be taken as such in the light of any wireless message received respective warning centers.

| Key Personal | Battle Stations | Drains & Flood Embankments | Name of Drainage Sub Division. |
| --- | --- | --- | --- |
| Sub Engineer Malakwal | RD.1-2 | Malakwal Flood Embankment | Bahauddin Drainage Sub Division |

13.3.4.  	DEPARTMENTAL MACHINERY AVAILABLE.

Department has no machinery of any kind. However, in emergency, machinery will be arranged by the Municipal Committee, District Government and Agricultural Engineering Department.

13.3.5.	MACHINERY AVAILABLE FROM PRIVATE SOURCE.

Suitable arrangement with Agricultural Engineering Department in time of emergency of closing leakage or breaches on flood bunds. Moreover excavator and tractor trolleys are available with local inhabitants of Malakwal Flood Bund, which can be deployed for necessary flood fighting in emergency.

13.3.6.	FLOOD FIGHTING MATERIAL REQUIRED

MALAKWAL FLOOD BUND SITE NO.2

| S.No. | ITEM | Required Quantity |
| --- | --- | --- |
| 1. | Life Jacket | 01 No. |
| 2. | Umbrella | 5 No. |
| 3. | Chimneys for Lanterns | 1 No. |
| 4. | Gunny Bags with Sulti and sewing needles | 25 No. |
| 5. | Axes / with handle | 15 No. |
| 6. | Kassies / with Handles | 20 No. |
| 7. | Ballies | 15 No. |
| 8. | Baskets | 30 No. |
| 9 | LED Bulb with Accessories | 5 No. |
| 10. | Patha Baan | 20 Kg |
| 11. | Shouldari | 01 No. |
| 12. | G.I.Wire | 50 Kg |
| 13. | Generator set. | 01 No. |
| 14. | Service Cable | 01 Coils |
| 15. | Rain Coat | 03 No. |
| 16. | Rechargeable Torch | 2 No. |
| 17. | Plastic Chair with Table | 01 Set |
| 18. | Tent medium Size | 01 No. |
| 19. | M. Oil | 2 Liter |
| 20. | Plastic Pages | 1000 No. |

13.3.7.   FLOOD FIGHTING METERIAL AVAILABLE

| S.No. | ITEM | Required Quantity | Available | Balance |
| --- | --- | --- | --- | --- |
| 1. | Life Jacket | 01 No. | 01 No. | -- |
| 2. | Umbrella | 5 No. | 5 No. | -- |
| 3. | Chimneys for Lanterns | 1 No. | 1 No. | -- |
| 4. | Gunny Bags with Sulti and sewing needles | 25 No. | 25 No. | -- |
| 5. | Axes / with handle | 15 No. | 15 No. | -- |
| 6. | Kassies / with Handles | 20 No. | 20 No. | -- |
| 7. | Ballies | 15 No. | 15 No. | -- |
| 8. | Baskets | 30 No. | 30 No. | -- |
| 9 | LED Bulb with Accessories | 5 No. | 5 No. | -- |
| 10. | Patha Baan | 20 Kg | 20 Kg | -- |
| 11. | Shouldari | 01 No. | 01 No. | -- |
| 12. | G.I.Wire | 50 Kg | 50 Kg | -- |
| 13. | Generator set. | 01 No. | 01 No. | -- |
| 14. | Service Cable | 01 Coils | 01 Coils | -- |
| 15. | Rain Coat | 03 No. | 03 No. | -- |
| 16. | Rechargeable Torch | 2 No. | 2 No. | -- |
| 17. | Plastic Chair with Table | 01 Set | 01 Set | -- |
| 18. | Tent medium Size | 01 No. | 01 No. | -- |
| 19. | M. Oil | 2 Liter | 2 Liter | -- |
| 20. | Plastic Pages | 1000 No. | 500 No. | 500 |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, 	TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDINGS, 	CANALS AND DRAINAGE NETWORK.

There area where it is likely hood of mishap is a remote area, there is no such huge infrastructure like Grid Station, Sui Gas Station exists in this area. However towns, villages, roads and Electric Lines canals and drainage network will suffer due to spill out water of the River Jhelum.

B. MALAKWAL FLOOD BUND.

Flooded water directly entered and hit to Malakwal Town and adjoining abadies Mid Pragna, Chak Nizam, Raib Pur, Mohallah Islam Pura and Chak Syddan. Therefore extension of Malakwal Flood Bund is very much essential up to Victoria Bridge for the safety of Malakwal City and other Villages

EMERGENCY CONTINGENCY PLAN FOR SITE NO.3

C.	(BUDHI NALLAH FLOOD BUND AT RD.9+200.)

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF

THE BREACH SUPPORTED WITH LEVELS.

Plan showing route of Flood water coming out of the breach for site No.03 i.e RD. 9+200 plan attached.

Annexure-C.

C.	BUDHI NALLAH FLOOD BUND.

  - DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED (TO BE	SHOWN ON THE MAPS), THE EXTENT AND DEPTH OF ANTICIPATED INUNDATIONS, EMERGENCY PROTECTION MEASURES AND EVACUATION PLAN.

The tentative area inundated by the flood water will be 40 square KM as shown on attached plan at Annex – “C” Detail of Villages/Abadies are as under: -

Channi Mughalla, Qadirabad, Farakh Pur, Alahan, Dharam Kot, Jaggo & Syada

.

  - STRATEGY AND ACTION TAKEN  BE EXPLAINED IN DETAIL.THIS MAY INCLUDE

BUDHI NALLAH FLOOD BUND.

There was high flood in River Chenab during recent flood season in the jurisdiction of Phalia Drainage Sub Division Mandi Bahauddin. Budhi Nallah drain is meant for disposal of flood water and seepage water of Drainage Systems failing in District Gujrat & Mandi Bahauddin. 2 No another creek locally known as Halki creek exists. These creeks become active during heavy rains/high flood. During High Flood water of these creeks River Chenab causes exceptionally high Flood in Budhi Nallah outfall drain. Resultantly, pressure built up on right bank, which breached at various spots, during August 2010, heavy rains and high flood having magnitude of 3.60 Lac Cs (D/S Qadirabad) caused havoc, heavy loss of standing crops infrastructures and abadies.

The length of Budhi Nallah Flood Bund is 18000 feet, due to requirement of site condition Budhi Nallah Flood protection Bund is proposed be extended /constructed up to 8000 feet along Bheikho outfall Upper drain for the safety of the adjoining abadies and agriculture land.

    - ARRANGEMENTS.

In view the experience of the past flood history day & night watching required to control the flood situation during rainy season.

Constant vigilance and caution will be required at all levels particularly on the spots for relief and flood fighting on drains, flood embankments.

13.3.2.    ESTABLISHMENT OF FLOOD FIGHTING CAMPS.

A tent would be at site and all the resources manpower and material required for controlling the situation would be immediately shifted from the shelter huts to this tent which would then become the emergency camp for carrying out all the flood fighting operations the Sub Engineer nominated by the Executive Engineer.

BUDHI NALLAH FLOOD BUND.

| Sr. #. | Key Personal | Name of drain. | Battle Station | Name of village likely to be effected. | Drainage Sub Division | Civil Sub Division. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Sub Engineer Budhi Nallah | Budhi Nallah Drainage System. | RD.18500 | Qadirabad, Saida & Jaggo | Phalia Drainage Sub Division | Phalia |

WATCHING ESTABLISHMENT.

Following watching establishment shall be engaged during the flood season for watching and general maintenance of flood works on monthly basis. However, in case of very high flood or in real emergency, the strength of watching establishment may be increased with the express permission of Executive Engineer, Drainage Division Mandi Bahauddin.

| Name of The Structure | For maintenance & Petrol during flood season. |  |  | In case of very high flood. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Beldar | Mate | Gauge Reader | Beldar | Mate | Gauge Reader |
| Budhi Nallah Flood Bund. | 6 | 1 | 1 | 10 | 1 | - |

13.3.3.	DUTIES OF OFFICERS/ OFFICIALS AND                                      THEIR CAMP SITES

- Executive Engineer Drainage Division Mandi Bahauddin.

The Flood activities in the jurisdiction of Mandi Bahauddin Drainage Division have been divided in to one sub set sectors i.e. Sub Sector No.01. The incharge of Sub Sector are Sub Divisional Officers of Bahauddin Drainage Sub Division Mandi Bahauddin as per following details.

DETAIL OF CAMP SITES.

| Sr.No. | Name of Flood Bund. | Name of Officer | Name of Official | Camp RD. |
| --- | --- | --- | --- | --- |
| 1 | Mona Bhera Flood Bund RD.0-35000. | Tahir Mehmood S.D.O Bahauddin (Site Incharge) | Gulraiz Haider Sub Engineer Malakwal Section. | 5+400. |
| 2 | Malakwal Flood Bund RD.0-3500. | Tahir Mehmood S.D.O Bahauddin (Site Incharge) | Gulraiz Haider Sub Engineer Bahauddin Section. | 1+000 |
| 3 | Budhi Nallah Flood Bund RD.0-18500. | Tahir Mehmood S.D.O Phalia | Mansha Khan Sub Engineer Budhi Nallah Section. | 9+200 |

- SUB DIVISIONAL OFFICER BAHAUDDIN

DRAINAGE SUB DIVISION.

Incharge Sub Sector No.01 will monitor the Flood Fighting /watching activities in the part area of Mona Bhera Flood Bund Tehsil Bhalwal and Malakwal Flood Bund on the left side of River Jhelum. The flowing Drainage System are directly effected by the River Chenab spill the Drainage function as Flood embankments and protection of Bhekho Outfall lower& Upper drain & Budhi Nallah Flood Bund.

| Sr.No. | Name of Bund. | Length of Bund | Top width | Side slope | Free Board from 11.F.I |
| --- | --- | --- | --- | --- | --- |
| 1. | Flood Bund along Budhi Nallah RD.0-18500. | 18500 | 25 feet | 2:1 on either side. | 5 feet. |

Keeping in view of vulnerable location of drains & Flood Plan, it is essential to take up flood preparedness activities like strengthening weak and Ghats sides clearance of Jungle, shurb from banks and slope and making them fit to face the ensuing floods. Therefore, a gang of one man per mile employee from 15 June to 15 September in additional to existing establishment of one Beldar in six mile length of drain. The terminology of different kinds of floods i.e. low Medium, High and exceptionally high and they will be taken as such in the light of any wireless message received respective warning centers.

| Key Personal | Battle Stations | Drains & Flood Embankments | Name of Drainage Sub Division. |
| --- | --- | --- | --- |
| Sub Engineer Budhi Nallah. | RD.18500 | Bheikho Outfall Upper drain. | Phalia Drainage Sub Division |
| Sub Engineer Budhi Nallah | RD-0. | Bheikho Outfall Upper drain | Phalia Drainage Sub Division |
| Sub Engineer Budhi Nallah | RD.16500 | Main Phalia Drain. | Phalia Drainage Sub Division |

13.3.4.  	DEPARTMENTAL MACHINERY AVAILABLE

Department has no machinery of any kind. However, in emergency, machinery will be arranged by the Municipal Committee, District Government and Agricultural Engineering Department.

13.3.5.	MACHINERY AVAILABLE FROM PRIVATE SOURCE.

Suitable arrangement with Agricultural Engineering Department in time of emergency of closing leakage or breaches on flood bunds. Moreover excavator and tractor trolleys are available with local inhabitants of Budhi Nallah Flood Bund, which can be deployed for necessary flood fighting in emergency.

13.3.6.	FLOOD FIGHTING MATERIAL REQUIRED

BUDHI NALLAH FLOOD BUND SITE NO.3.

| S.No. | ITEM | Required Quantity |
| --- | --- | --- |
| 1. | Gunny Bags with Sulti and sewing needles | 300 No. |
| 2 | Life Jacket | 5 No. |
| 3. | Axes with handles | 30 No. |
| 4. | Ballies | 100 No. |
| 5. | Handles for Kassies | 20 No. |
| 6. | Patha Baan | 50 Kg |
| 7. | Shouldari | 1 No. |
| 8. | G.I.Wire | 100 Kg |
| 9. | Generator set. | 1 No. |
| 10. | Service Cable | 1 Coils |
| 11. | LED Bulb with accessories | 20 No. |
| 12 | Emergency light | 10 No. |
| 13 | Wooden Mallet | 10 No. |
| 14 | Petrol | 200 Liter |
| 15 | Plastic Bags. | 1000 No. |
| 16 | Bamboos. (15’-20’) | 500 No. |
| 17 | Wooden Baskets | 100 No. |
| 18 | Kassi with handle | 20 No. |
| 19. | Lylon Rope | 200 Feet |
| 20. | Chair plastic with Table | 2 Set |
| 21. | Umbrella | 10 No. |
| 22. | Rain Coats | 5 No. |
| 23. | M. Oil | 5 Liter |
| 24. | Measurement Tape | 5 No. |

    - FLOOD FIGHTING METERIAL AVAILABLE SITE NO.3

| S.No. | ITEM | Required Quantity | Available | Balance |
| --- | --- | --- | --- | --- |
| 1. | Gunny Bags with Sulti and sewing needles | 300 No. | 300 No. | - |
| 2 | Life Jacket | 5 No. | 5 No. | - |
| 3. | Axes with handles | 30 No. | 30 No. | - |
| 4. | Ballies | 100 No. | 100 No. | - |
| 5. | Handles for Kassies | 20 No. | 20 No. | - |
| 6. | Patha Baan | 50 Kg | 50 Kg | - |
| 7. | Shouldari | 1 No. | 1 No. | - |
| 8. | G.I.Wire | 100 Kg | 100 Kg | - |
| 9. | Generator set. | 1 No. | 1 No. | - |
| 10. | Service Cable | 1 Coils | 1 Coils | - |
| 11. | LED Bulb with accessories | 20 No. | 20 No. | - |
| 12 | Emergency light | 10 No. | 10 No. | - |
| 13 | Wooden Mallet | 10 No. | 10 No. | - |
| 14 | Petrol | 200 Liter | 200 Liter | - |
| 15. | Generator | 1 No. | 1 No. | -- |
| 15 | Plastic Bags. | 1000 No. | 1000 No. | - |
| 16 | Bamboos. (15’-20’) | 500 No. | 500 No. | - |
| 17 | Wooden Baskets | 100 No. | 100 No. | -- |
| 18 | Kassi with handle | 20 No. | 20 No. | -- |
| 19. | Lylon Rope | 200 Feet | 200 Feet | -- |
| 20. | Chair plastic with Table | 2 Set | 2 Set | -- |
| 21. | Umbrella | 10 No. | 10 No. | -- |
| 22. | Rain Coats | 5 No. | 5 No. | -- |
| 23. | M. Oil | 5 Liter | 5 Liter | -- |
| 24. | Measurement Tape | 5 No. | 5 No. | -- |

  - DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDINGS, CANALS AND DRAINAGE NETWORK.

There area where it is likely hood of mishap is remote area, there is no such huge infrastructure like Grid Station, Sui Gas Station exists in this area.  However town, villages, roads and Electric lines Canals and drainage network will suffer due to spill out water of the River Chenab.

C.

BUDHI NALLAH FLOOD BUND.

Flooded water directly entered and hit to Qadirabad Town, Jaggo, Bheikho, Thatti Shah Muhammad, Farakh Pur. Therefore existing flood embankment should be protected with stone pitching and raising of flood embankment.

Executive Engineer					Superintending Engineer

Drainage Division						Sargodha Drainage Circle

Mandi Bahauddin	   			                            Sargodha.

Chief Engineer

Irrigation Sargodha Zone

Sargodha.

CHAPTER-14

Action Plan

  - Re-shuffling/ Recouping plan of reserve stone departmentally

There is no reserve stock of stone in this division.

  - Detail od inlet/ outlet crossing along with closing methodology

There is no Inlet/Outlet crossing exist in Budhi Nallah flood bund.

  - Deployment of Machinery (Medium to high flood)

| Sr.No. | Name of Structure | Length in Mile | Vulnerable Reach | Camp Location | Site Incharge by Name & Cell No. | Machinery Deployed |  |  |  |  | Availability of Stone Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/Dumpers | Tractor with front blade | Labour/Beldar+Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Mona Bhera Flood Bund RD.0-35000. | 7.00 | RD.5-8 | RD.5+400 | Sub Engineer Malakwal Section Gulraiz Haider (0316-1776059) | 1 | _ | 5 | 1 | 8+1 | Nil |
| 2 | Malakwal Flood Bund RD.0-3500. | 0.70 | RD.0-3500 | RD.1+000 | Sub Engineer Bahauddin Section Gulraiz Haider (0316-1776059) | 1 | _ | 3 | 1 | 4+1 | Nil |
| 3 | Budhi Nallah Flood Bund RD.0-18500. | 3.70 | RD.3-4, 6-7 & 13-18+500 | RD.9+200 | Sub Engineer Budhi Nallah Mansha Khan (0300-7741766) | 1 | _ | 6 | 2 | 10+1 | Nil |
| Total |  |  |  |  |  | 3 | - | 14 | 4 | 22+3 | Nil |

Payment for Machinery & Works Charge employees subject to verification by TPM / Consultants

  - Deployment of Machinery (High to very high flood)

| Sr.No | Name of Structure | Length in Mile | Vulnerable Reach | Camp Location | Site Incharge by Name & Cell No. | Machinery Deployed |  |  |  |  | Availability of Stone Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/Dumpers | Tractor with front blade | Labor/ Beldar+Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Mona Bhera Flood Bund RD.0-35000. | 7.00 | RD.5-8 | RD.5+400 | Sub Engineer Malakwal Section Gulraiz Haider (0316-1776059) | 1 | _ | 7 | 2 | 10+1 | Nil |
| 2 | Malakwal Flood Bund RD.0-3500. | 0.70 | RD.0-3500 | RD.1+000 | Sub Engineer Bahauddin Section Gulraiz Haider (0316-1776059) | 1 | _ | 4 | 2 | 6+1 | Nil |
| 3 | Budhi Nallah Flood Bund RD.0-18500. | 3.70 | RD.3-4, 6-7 & 13-18+500 | RD.9+200 | Sub Engineer Budhi Nallah Mansha Khan (0300-7741766) | 1 | _ | 8 | 2 | 12+1 | Nil |
| Total= |  |  |  |  |  | 3 | - | 19 | 6 | 28+3 |  |

Payment for Machinery & Works Charge employees subject to verification by TPM / Consultants

  - Deployment of Machinery (Very high to exceptionally high flood)

| Sr.No | Name of Structure | Length in Mile | Vulnerable Reach | Camp Location | Site Incharge by Name & Cell No. | Machinery Deployed |  |  |  |  | Availability of Stone Cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/Dumpers | Tractor with front blade | Labour/Beldar+Mate |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Mona Bhera Flood Bund RD.0-35000. | 7.00 | RD.5-8 | RD.5+400 | Sub Engineer Malakwal Gulraiz Haider (0316-1776059) | 2 | _ | 7 | 4 | 12+2 | Nil |
| 2 | Malakwal Flood Bund RD.0-3500. | 0.70 | RD.0-3500 | RD.1+000 | Sub Engineer Bahauddin Section Gulraiz Haider (0316-1776059) | 2 | _ | 4 | 3 | 10+2 | Nil |
| 3 | Budhi Nallah Flood Bund RD.0-18500. | 3.70 | RD.3-4, 6-7 & 13-18+500 | RD.9+200 | Sub Engineer Budhi Nallah Mansha Khan (0300-7741766) | 2 | 1 | 8 | 4 | 15+2 | Nil |
| Total= |  |  |  |  |  | 6 | 1 | 19 | 11 | 37+6 | Nil |

Payment for Machinery & Works Charge employees subject to verification by TPM / Consultants

  - Police Deployment Plan

District administration of Mandi Bahauddin will be contacted by the Executive Engineer to provide police officers for security purposes in case of emergency.

| Sr. No. | Vulnerable sites used for illegal cuts during floods | Concerned Canal Division | Concerned Police Station and District | Police Persons Deployed |  | Remarks (if any) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Inspector / Sub Inspector / A.S.I | Constable |  |
| 1 | Budhi Nallah Flood Bund RD 3-4, 6-7 & 13-18+500 | Drainage Division Mandi Bahauddin | Mandi Bahauddin | A.S.I-1 | 4 |  |
| 2 | Malakwal Flood Bund RD 0-3+500 |  |  | ASI-1 | 2 |  |
| 3 | Mona Bhera Flood Bund RD 5-8 |  |  | ASI-1 | 2 |  |

14.7         Details of synthetic bags with capacity of 500 kg and 1000.kg

| S.No. | Items | Required Quantity | Available Quantity |
| --- | --- | --- | --- |
| 1 | Synthetic Bags (500Kg Capacity) | 200 No. | -- |
| 2 | Synthetic Bags (1000Kg Capacity) | 150 No. | -- |

14.8     Details of polythene sheet of black colour to protect upstream slope against wave action and to control seepage through embankments.

| S.No. | Items | Required Quantity | Available Quantity |
| --- | --- | --- | --- |
| 1 | Black Polythene Sheet | 15,000 Sft | -- |

CHAPTER-15

BACK UP DIVISIONS IN CASE OF BREACH

Gujrat Canal Division UJC Gujrat and Rasul Canal Division L.J.C Rasul will be worked as back up Division in case of emergency/Breach.

Sub Divisional Officer				Sub Divisional Officer

Bahauddin Drainage Sub Division		              Phalia Drainage Sub Division

Mandi Bahauddin					Mandi Bahauddin.

Executive Engineer

Drainage Division

Mandi Bahauddin.