PART-A

MANDI BAHAUDDIN CANAL DIVISION

CHAPTER - 1

SALIENT FEATURES OF THE MANDI BAHAUDDIN CANAL DIVISION.

  - LOCATION

Mandi Bahauddin Canal Division was established in July, 2021 after imposing the revamping of Punjab Irrigation Department. The Divisional office is located in Canal Colony Mandi Bahauddin on right side of Mandi Bahauddin, Rasul Road.

1.2	GENERAL DESCRIPTION

Mandi Bahauddin Canal Division is responsible for the maintenance of River training works constructed along the right side of Rivers Chenab following in jurisdiction of Mandi Bahauddin Districts. Total length of Bunds and Spurs in Mandi Bahauddin Canal Division is 30.30 miles along right bank and 497.73 Miles of canals which exist in Mandi Bahauddin/Sargodha Districts. The purpose of these infrastructures is to save tens of thousands of inhabitants dwelling along River Chenab from ravages of the river which can wreak havocs to their lives and property during flood season. A brief description of River / Nullahs which bring flood in District Mandi Bahauddin and detail of flood protection works constructed or being constructed to mitigate flood damages have been explained in this flood fighting plan. An index plan of Mandi Bahauddin Canal Division showing all flood protection works constructed along River Chenab is placed at page No. 6-A.

        - 1.3 	ADMINISTRATIVE SETUP

- Mandi Bahauddin Canal Division Mandi Bahauddin

- Sub Divisional Officer Phalia Canal Sub Division.

- Sub Divisional Officer Sohawa Canal Sub Division M.B. Din.

- Sub Divisional Officer Busal Canal Sub Division M.B. Din.

- 11 No. Sub Engineers.

CHAPTER – 2

FLOOD PROTECTION AND RIVER TRAINING WORKS

  - DESIGN PARAMETER OF TRAINING WORKS

RIVER CHENAB

Chenab River originates from the hills of occupied Jammu and Kashmir and after entering in Pakistan, it is controlled at Marala Barrage in District Sialkot. Other important structures on this River D/S Marala Barrage are Alexandra Railway Bridge, G.T Road Bridges near Gujrat city, Khanki and Qadirabad Barrage.

Prior to 1983, this River used to flow along its left bank but it started to drift towards right side in the subsequent flood seasons. The trend of right wards drift kept on increasing and the situation became critical to such an extent that Irrigation Department had to construct a No. of structures along right bank of this River in the subject reach of about 25 miles length to check erosion/embayment along right bank and to arrest the chances of Alexandra Bridge from being by-passed by the river.

The area of responsibility of Mandi Bahauddin Canal Division on river Chenab along its right bank starting from D/S Qadirabad Head Works upto the escape of Lower Jhelum Canal in District Sargodha.

Detail of works in the above mentioned river reaches is as under.

QADIRABAD BARRAGE TO OUTFALL LJC ESCAPE (RIVER REACH)

| Sr.No. | Name of spur | Location | Length of spur (in ft.) | Length of shank | Length of head | Top width (in ft.) | Side slope |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Guide wall spur village Farrukhpur. | Distt: MBDin | 2240 | -- | -- | 25 | 2:1 |
| 2 | Hockey Spur No.1 at Bhutta Marran | Distt: MBDin | 2385 | 1650 | 735 | 25 | 2:1 & 3:1 |
| 3 | Hockey Spur No.2 at Bhutta Marran | Distt: MBDin | 685 | -- | -- | 20 | 1:1 |
| 4 | Solid Stone Stud No. 4,5,6 & 7 in front of Bhutta Marran | Distt: MBDin | 220 | -- | -- | 15 | 1:1 |
| 5 | J-Head Spur Kokaray | Distt: MBDin | 4105 | 1750 | 2135 | 25 | 2:1 & 3:1 |
| 6 | J-Head Spur Haveli Manak | Distt: MBDin | 3560 | 1800 | 1760 | 25 | 2:1 & 3:1 |
| 7 | Guide wall spur U/S Haveli Manik. | Distt: MBDin | 1400 | -- | -- | 25 | 2:1 |
| 8 | Guide wall spur village Noorpur. | Distt: MBDin | 1581 | -- | -- | 25 | 2:1 |
| 9 | J-head spur Chakori village. | Distt: MBDin | 2650 | -- | 855 | 25 | 2:1 |
| 10 | Hockey Spur with studs near Village Bherowal. | Distt: MBDin | 600 | -- | -- | 20 | 2:1 |
| 11 | J-Head Spur Dera Gujjran | Distt: Sargodha | 3710 | 2000 | 1710 | 25 | 2:1 & 3:1 |
| 12 | Hockey Spur with Studs near village Miana Hazara | Distt: Sargodha | 684 | - | - | 25 | 2:1 |
|  | Total |  | 27925 |  |  |  |  |
|  |  |  | 5.585 Mile |  |  |  |  |

ABSTRACT

i	Qadirabad Barrage to outfall LJC escape sloping

Spurs / J-head spurs, hockey spurs, Guide wall.		=	11 No.

ii	Studs / Other Structures					=	12

TOTAL		=	23 No.

TOTAL LENGTH	= 4.92 Miles

- HUMBER FLOOD BUND

This bund has been constructed to protect Phalia disty system & agriculture land of villages Thakar Kalan, Miana Thakar, Waryam, Bharowal, Humber, Basti Makhdum & Dhunny Kalan etc. from flood waters of River Chenab. Its total length is about 16.50 miles and is quite safe to face high floods in River Chenab. However, general maintenance in terms of removal of rain cuts/gharas and making up of deficient height/slopes will be done during the current financial year.

Salient features of Humber Flood Bund is as under:-

i)	Length RD 0-82500		=	16.50 Miles

ii)	Location	=	Along right bank of River Chenab near village Dhuni kalan to tail LJC escape.

iii)	Side slopes	=	3:1 on water side & 2:1 on Country side.

iv)	Top width	=	25’

v)	Free board	=	5’ above HFL of 1973

vi)	Pitched section	=	39+000-40+000=1+000’

Total length of Humber Flood bund.	=	16.50 Miles

A	        CONSTRUCTED/COMPLETED WORK DURING 2005-06,

2006-07.				__

Following works namely “Construction of spurs/studs on right bank of River Chenab to protect village abadies and agricultural lands of Qadirabad, Farrukhpur, Chakori, Noorpur, etc. were planned to be executed under ADP 2005-06 and work has been completed on 30.6.2007. The salient features of these components are as under:-

1)	Constructing Farrukhpur flood protection bund                         RD 0+000 to 33+500.

| 1. | Length of bund | 6.80 Mile |
| --- | --- | --- |
| 2. | Top width | 25 feet |
| 3. | Side slopes | Outer side 2:1/Inner side 3:1 |
| 4. | Kind of bund | Earthen |

2)         Constructing Guide Wall spur near Farrukhpur village

| 1. | Length of guide wall | 2240 feet |
| --- | --- | --- |
| 2. | Top width | 25 feet |
| 3. | Side slopes | 2:1 |
| 4. | Thickness of pitching | 2 feet |
| 5. | High flood level | 675.0 ft. |
| 6. | Top level of guide wall. | 680.0 ft. |
| 7. | Top level of apron | 659.50 ft. |
| 8. | Free board | 5.00 ft. |

3)	Constructing J-head spur near Chakori village.

| 1. | Length of shank | 2650 feet |
| --- | --- | --- |
| 2. | Length of head | 855 ft. |
| 3. | Top width | 25 feet |
| 4. | Side slopes | 2:1 |
| 5. | Thickness of pitching | 2 feet |
| 6. | High flood level | 671.50 ft. |
| 7. | Top level of spur. | 676.50 ft. |
| 8. | Top level of apron | 656.00 ft. |
| 9. | Free board | 5.00 ft. |

4.	Constructing Guide wall spur with sloping stud near Noorpur village.

| 1. | Length of guide wall | 1581 feet |
| --- | --- | --- |
| 2.. | Top width | 25 feet |
| 3. | Side slopes | 2:1 |
| 4. | Thickness of pitching | 2 feet |
| 5. | High flood level | 659.60 ft. |
| 6. | Top level of guide wall. | 659.60 ft. |
| 7. | Top level of apron | 654.50 ft. |

XEN/M.B.Din

5.	Salient features of sloping studs.

| 1. | Length | Straight part = 100 feet Sloping part = 192 feet Total = 292 feet |
| --- | --- | --- |
| 2. | Top width | 15 feet |
| 3. | Side slopes | 1:1 |

6.	Constructing Guide wall spur with sloping stud U/S Haveli Manik.

| 1. | Length of guide wall | 1400 feet |
| --- | --- | --- |
| 2.. | Top width | 25 feet |
| 3. | Side slopes | 2:1 |
| 4. | Thickness of pitching | 2 feet |
| 5. | High flood level | 668.20 ft. |
| 6. | Top level of guide wall. | 668.20 ft. |
| 7. | Top level of apron | 653.70 ft. |

7.	Salient features of sloping stud.

| 1. | Length | Straight part = 100 feet Sloping part = 171 feet Total= 271 feet |
| --- | --- | --- |
| 2. | Top width | 15 feet |
| 3. | Side slopes | 1:1 |

2.2	LOCATION MAP

Location Map is attached

CHAPTER – 3

BRIEF HISTORY OF PAST FLOOD EVENTS

Unexpected heavy rains in the first week of September, 1992 caused historic floods in the catchment area of Twin Rivers i.e. Jhelum and Chenab at the same occasion. The discharge in river Chenab at Marala Barrage 845090 Cs. was experienced on 10.9.1992 at 18.00 hours.  Similarly the flow in River Chenab at Qadirabad Barrage at 1800 hours of 9.9.1992 was only 41681 Cs. which started rising on 10.9.1992 morning and went upto 948530 Cs. on 11.9.1992. The flood water spread on the right side of River Chenab and river edge were badly damaged.

During subsequent flood seasons, the main river channel started drifting towards right side between Marala to Alexandra Bridge reach. As a result of this development, the entire river started flow along its right bank. Consequently, serious problems such as embayment, land erosion, bank sloughing and inundation of agricultural lands, basis infrastructures and villages were experienced. Due to this expanding erosive tendency of the river, a number of villages and thousands of acres of valuable agricultural land was lost to the river. This problem, therefore, required a suitable and effective preventive measure. To evolve an affective flood management plan for this area, model studies were carried out by the Irrigation Research Institute. 11 No. spurs were constructed along Right Bank of River Chenab from D/S Marala Headworks to Alexandra Ridge in District Gujrat.

During flood season of 1997, heavy rains started in Gujrat District on 26.8.1997. Incessant heavy rains continued for about 36 hours without any break. Situation took serious turn on 27-28/8/97 when heavy discharge in River Chenab (exceptionally high flood of 775525 Cs.) along with dangerous wind storm and heavy rain made the situation precarious. Floods in Nallahs also joined hands. Joint action blow was so serious that a time came when situation was out of the control of a man and it power. In fact, even it could be termed as a calamity or bolts from the blue. Following spurs/guide banks suffered heavy damages i.e J-head spur No. 2, 3,4,6,8, Guide bank spur No. 9 & Guide bank spur No. 10 were damaged. These damages were restored during financial year 1997 to 1999.

A high flood was received D/S Qadirabad Barrage on 7.8.2010 with  discharge of 319733 Cs. Resultantly a loop was developed in between guide wall spur on right bank of River Chenab opposite village Randiali and inverted hockey spur opposite village Bahri and J-Head Spur Chakori in District M.B.Din. The sites were inspected by the Executive Engineer, Gujrat Division UJC on 16.8.2010. The condition of structures was serious. The apron from RD 0-70 of the inverted hockey spur Bahri was completely launched and pitching slipped into the scour pit in different reaches. The shank of the spur has also washed away in a length of 70 feet. The apron and solid portion of J-Head spur Chakori was also badly damaged. The model study of these structures was made by the Irrigation Research Institute. In the light of design and proposals given by the Principal Research Officer (Hydraulics) the restoration work of Inverted Hockey Spur Bahri was carried out. Simultaneously J-Head Spur near village Chakori was also replenished re-constructed and restoration completed accordingly. The both works have been completed at site during the fiscal year of 2011-2012.

As regard, the following infrastructures were badly damaged during flood season 2010 which was repaired/restored.

Frukhpur Flood Bund

- Repairing & providing pitching in damage and vulnerable reaches of Farrukhpur flood bund.

Haveli Manik Guide Wall

- Restoration damaged and by-passed sloping stud U/S Haveli Manik Guide wall.

Bherowal Guide Wall

- Protecting/Restoring by passed solid stud D/S hockey spur near village Bherowal.

During subsequent flood season 2011 and 2012, the discharge received in River Chenab was 166383 Cs. and 170210 Cs. respectively. During this medium flood no damages were occurred fortunately.

During flood season 2013, a high flood was received with discharge of 369960 Cs. downstream Murala Barrage. This heavy discharge damaged various infrastructures which were existed on the right bank of River Chenab. The following works were taken up under Para 2.89 of PWD code.

| Sr. | Name of scheme |
| --- | --- |
| 1 | Protection of Farrukhpur flood bund on erosion side at RD 11-12 & 14-15. |
| 2 | Protecting & restoring of hockey stud/spur for protection of village Behlolpur from erosion of right side of River Chenab, |
| 3 | Protection abadies of village Bhutta Piran from erosion on Right Side of River Chenab. |

A work namely “Checking erosion on right bank of River Chenab to protect Bhekho out fall drain and agriculture land of Miana Hazara”.

The above scheme was approved by the Federal Flood Commission Islamabad and work completed during the Financial year 2012-13 to 2014-15.

| Sr. No. | Name of Work |
| --- | --- |
| 1. | Checking erosion on right bank of River Chenab to protect Bhekho out fall drain and agriculture land of Miana Hazara |

During flood season 2014, a discharge of 856464 Cs. was received downstream Marala Barrage on 6.9.2014 which create a hue and cry situation in the surrounding abadies located on the Right Bank of River Chenab in District Gujrat. The hill torrents and nallahs also joined hands and enhanced the disturbance situation. The sudden increase in the discharge badly eroded the Right Bank of River Chenab as well as the infrastructures. It is also worth mentioning here that the heavy rains started in the first week of September 2014 in Gujrat District. The incessant heavy rains continued and the situation took serious turn hour to hour. Consequently an exceptional heavy flood played ahavoc role in the lives and livings of inhabitants of District Gujrat. Downstream Khanki Barrage the discharge of 947099 Cs. was received which created a hue and cry situation in the upstream locality and damaged a lot of agricultural and stock lives. The flood protection infrastructures and river training works between Marala Head Works to Khanki Head Works was also damaged.

Downstream Qadirabad Barrage 903504 Cs. Discharge passed which damaged the flood bunds, embankments; guide walls etc. existed on the right bank of River Chenab and falling in the jurisdiction of District Mandi Bahauddin. Consequently, the following infrastructures were damaged partially and to some extent.

| Sr. No. | Name of infrastructure/minor damaged |
| --- | --- |
| 1 | Restoration of damages/by pass sloping stud U/S Guide Wall Haveli Manik. |
| 2 | Restoration of cut between RD 23-24 of Farrukhpur Flood Bund. |
| 3 | Restoration of eroded bank between RD 3-4, 7-8, 10-11 and RD 21-22 of Farrukhpur Flood Bund. |
| 4 | Restoring cuts from RD 10-50100 tail of Qadirabad minor. |

The following works were taken up during the financial year 2014-15 and 2015-16.

INESCABLE WORKS

| Sr. No. | Name of infrastructure |
| --- | --- |
| 1 | Restoration of eroded bank between RD 3-4, 7-8, 10-11, 11-12, 21-22 and closing cut between RD 23-24 Furkhpur Flood Bund. |
| 2 | Restoraiton damaged bypassed sloping stud U/S Guide wall Haveli Manik. |

OTHER WORK

| Sr. No. | Name of infrastructure |
| --- | --- |
| 1. | Construction of Solid stone stud to protect pitching of village Kokary. |

WORK EXECUTED UNDER PARA 2.89 OF PWD (2015-16)

| Sr. No. | Name of infrastructure |
| --- | --- |
| 1. | Checking erosive action of Chenab River near village Bhuta Maranran. |

WORK EXECUTED UNDER PARA 2.89 OF PWD (2016-17)

| Sr. No. | Name of infrastructure |
| --- | --- |
| 1 | Constructing solid stone studs on right bank of River Chenab to protect village Bhutta Marran |
|  | After 2016 no work executed. |

CHAPTER – 4

DESIGN DATA, HISTORIC PEAK FLOOD DATA AND PREVIOUS FIVE YEARS FLOOD DATA OF HEAD WORKS/BARRAGES AND OR OTHER CONTROL POINTS

  - FLOOD LIMITS OF MARALA HEAD WORKS

1.	Normal flood			=	0.0 – 1.0 lac Cs.

- Low flood			=	1.0 – 1.5 lac Cs.

3.	Medium flood			=	1.5 – 2.0 lac Cs.

4.	High flood			=	2.0 – 4.0 lac Cs.

5.	Very high flood		=	4.0 – 6.0 lac Cs.

6.	Exceptionally high flood	=	6.0 & above.

4.2	TIME LAGS OF FLOODS

- Distance 8 Km from Border to Murala time for flood water.= 2hours

AHKNOOR TO MURALA

- LOW Flood					=	9-10 hours.

- Medium Flood					=	8-9 hours.

- High Flood					=	7-8 hours.

- Very high Flood				=	5 hours.

- Exceptionally high Flood			=	4 hours.

Flood water above 1.00 lac

Marala to Alexandra bridge 	(42KM)	=	 5-6 hours.

Alexandra bridge to Khanki	(14KM)	= 	1.5-2 hours

Khanki to Qadirabad		(28KM)	= 	3-4 hours

  - HIGHEST FLOODS

1992		-		845090 Cs. Marala Barrage

1992		-		650000 Cs. Alexandra Bridge

1992		-		910000 Cs.  Khanki Headworks

1992		-		948530 Cs. Qadirabad Barrage

2014		-		856464 Cs. Marala Barrage

2014		-		780216 Cs. Alexandra Bridge

2014		-		947099 Cs.  Khanki Headworks

2014		-		903504 Cs. Qadirabad Barrage

4.4	PEAK DISCHARGES.

i)           YEARLY PEAK DISCHARGES OF RIVER CHENAB AT MARALA

| Year | Peak Flood | Year | Peak Flood | Year | Peak Flood |
| --- | --- | --- | --- | --- | --- |
| 1925 | 139596 | 1959 | 870795 | 1993 | 409490 |
| 1926 | 202765 | 1960 | 599303 | 1994 | 472570 |
| 1927 | 237718 | 1961 | 272003 | 1995 | 439970 |
| 1928 | 686000 | 1962 | 331362 | 1996 | 766860 |
| 1929 | 718000 | 1963 | 204158 | 1997 | 775525 |
| 1930 | 254000 | 1964 | 215863 | 1998 | 213705 |
| 1931 | 260238 | 1965 | 144677 | 1999 | 214765 |
| 1932 | 332326 | 1966 | 618338 | 2000 | 247610 |
| 1933 | 366000 | 1967 | 190386 | 2001 | 160950 |
| 1934 | 228000 | 1968 | 155262 | 2002 | 224780 |
| 1935 | 253000 | 1969 | 219302 | 2003 | 313024 |
| 1936 | 177500 | 1970 | 203733 | 2003 | 147318 |
| 1937 | 145759 | 1971 | 175143 | 2004 | 93150 |
| 1938 | 261700 | 1972 | 132915 | 2005 | 345479 |
| 1939 | 203000 | 1973 | 769550 | 2006 | 332956 |
| 1940 | 167100 | 1974 | 192630 | 2007 | 252409 |
| 1941 | 297000 | 1975 | 582600 | 2007 | 148627 |
| 1942 | 301500 | 1976 | 549400 | 2008 | 174977 |
| 1943 | 256900 | 1977 | 437125 | 2009 | 94168 |
| 1944 | 150500 | 1978 | 456394 | 2010 | 282418 |
| 1945 | 207650 | 1979 | 248111 | 2011 | 166383 |
| 1946 | 263540 | 1980 | 199981 | 2012 | 169914 |
| 1947 | 253720 | 1981 | 527462 | 2013 | 369960 |
| 1948 | 432000 | 1982 | 647098 | 2014 | 856464 |
| 1949 | 251956 | 1983 | 171579 | 2015 | 153408 |
| 1950 | 540761 | 1984 | 160042 | 2016 | 412065 |
| 1951 | 193847 | 1985 | 222595 | 2017 | 187470 |
| 1952 | 209570 | 1986 | 241800 | 2018 | 168278 |
| 1953 | 316898 | 1987 | 114140 | 2019 | 211296 |
| 1954 | 820090 | 1988 | 481365 | 2020 | 298884 |
| 1955 | 344090 | 1989 | 352115 | 2021 | 171156 |
| 1956 | 272200 | 1990 | 160020 | 2022 | 210936 |
| 1957 | 1100000 | 1991 | 40560 | 2023 | 192960 |
| 1958 | 478457 | 1992 | 845090 | 2024 | 160950 |

iii)	YEARLY PEAK DISCHARGES OF RIVER CHENAB AT KHANKI.

| Year | Peak flood | Year | Peak flood | Year | Peak Flood |
| --- | --- | --- | --- | --- | --- |
| 1925 | 266809 | 1960 | 809524 | 1995 | 650000 |
| 1926 | 224640 | 1961 | 353994 | 1996 | 851000 |
| 1927 | 223796 | 1962 | 346279 | 1997 | 847000 |
| 1928 | 441497 | 1963 | 186435 | 1998 | 214954 |
| 1929 | 747000 | 1964 | 176590 | 1999 | 170815 |
| 1930 | 194376 | 1965 | 200618 | 2000 | 309253 |
| 1931 | 292098 | 1966 | 636670 | 2001 | 155648 |
| 1932 | 271769 | 1967 | 209767 | 2002 | 240389 |
| 1933 | 246163 | 1968 | 235397 | 2003 | 371888 |
| 1934 | 177813 | 1969 | 274357 | 2003 | 172593 |
| 1935 | 259730 | 1970 | 251954 | 2004 | 106934 |
| 1936 | 15946 | 1971 | 293293 | 2005 | 368078 |
| 1937 | 105835 | 1972 | 811723 | 2006 | 418740 |
| 1938 | 261685 | 1973 | 1000496 | 2007 | 300197 |
| 1939 | 175979 | 1974 | 189446 | 2008 | 202000 |
| 1940 | 128393 | 1975 | 666241 | 2009 | 74550 |
| 1941 | 168687 | 1976 | 615043 | 2010 | 327637 |
| 1942 | 210357 | 1977 | 472130 | 2011 | 131492 |
| 1943 | 238169 | 1978 | 433096 | 2012 | 186413 |
| 1944 | 164719 | 1979 | 246867 | 2013 | 410332 |
| 1945 | 180122 | 1980 | 194814 | 2014 | 947099 |
| 1946 | 149681 | 1981 | 563856 | 2015 | 163967 |
| 1947 | 236583 | 1982 | 243260 | 2016 | 418736 |
| 1948 | 438529 | 1983 | -- | 2017 | 184346 |
| 1949 | 224949 | 1984 | 134897 | 2018 | 182025 |
| 1950 | 986658 | 1985 | 148482 | 2019 | 181944 |
| 1951 | 217868 | 1986 | 341000 | 2020 | 286230 |
| 1952 | 264065 | 1987 | 109413 | 2021 | 183688 |
| 1953 | 315330 | 1988 | 844050 | 2022 | 210945 |
| 1954 | 806774 | 1989 | 512225 | 2023 | 204041 |
| 1955 | 230490 | 1990 | 270015 | 2024 | 163215 |
| 1956 | 340663 | 1991 | 314553 |  |  |
| 1957 | 1086460 | 1992 | 910000 |  |  |
| 1958 | 401682 | 1993 | 366680 |  |  |
| 1959 | 1021018 | 1994 | 509113 |  |  |

iv.     YEARLY PEAK DISCHARGES OF RIVER CHENAB AT

QADIRABAD

| Year | Peak discharge | Year | Peak discharge |
| --- | --- | --- | --- |
| 1968 | 221607 | 2004 | 90043 |
| 1969 | 360038 | 2005 | 379572 |
| 1970 | 240401 | 2006 | 443206 |
| 1971 | 312098 | 2007 | 311382 |
| 1972 | 249259 | 2008 | 194038 |
| 1973 | 854341 | 2009 | 55535 |
| 1974 | 198228 | 2010 | 319733 |
| 1975 | 669819 | 2011 | 164604 |
| 1976 | 628741 | 2012 | 180789 |
| 1977 | 482871 | 2013 | 403403 |
| 1978 | 428009 | 2014 | 903504 |
| 1979 | 240785 | 2015 | 175054 |
| 1980 | 178582 | 2016 | 416492 |
| 1981 | 575440 | 2017 | 164660 |
| 1982 | 245030 | 2018 | 172031 |
| 1983 | 303120 | 2019 | 159544 |
| 1984 | 108476 | 2020 | 258954 |
| 1985 | 321917 | 2021 | 167812 |
| 1986 | 318117 | 2022 | 202149 |
| 1987 | 342758 | 2023 | 185749 |
| 1988 | 892266 | 2024 | 145219 |
| 1989 | 497649 |  |  |
| 1990 | 290417 |  |  |
| 1991 | ----- |  |  |
| 1992 | 948530 |  |  |
| 1993 | 434754 |  |  |
| 1994 | 425567 |  |  |
| 1995 | 644397 |  |  |
| 1996 | 853231 |  |  |
| 1997 | 873442 |  |  |
| 1998 | 259713 |  |  |
| 1999 | 630500 |  |  |
| 2000 | 291349 |  |  |
| 2001 | 136575 |  |  |
| 2002 | 226398 |  |  |
| 2003 | 367408 |  |  |
| 2003 | 159296 |  |  |

CHAPTER - 5

FLOOD FIGHTING STRATEGY

In order to combat the flood, the following plan is to be put in operation for each River or Nallah and protective works on it.

i)		HUMBER PROTECTION BUND

Sub Engineer Bhikho and Sub Divisional Officer Phalia are incharge of this bund. Emergency flood fighting material as per list attached will also be available at huts, which have already been constructed for this purpose at vulnerable sites i.e. RD 20000,   RD 33000 and at RD 47000.

There is no likelihood of the flood water touching the bund upto 3.50 lac cs. discharge below Qadirabad Barrage. At 5.00 lac of discharge below Qadirabad Barrage, the flood water touches the bund in reach RD 10000 to RD 47000 and in this situation there is no likelihood of any attack on the bund. However when the discharge below Qadirabad exceeds 5.00 cusecs, the Sub Engineer Phalia will also shift his headquarter from Duggal Rest House to RD 55000 of Humber Protection Bund to watch the said bund from RD 40000 to 82500 and Sub Engineer Bhikho shall be Incharge of reach RD 0-40000 with temporary headquarter at RD 33000. The gang huts on these points have already been constructed.

When the discharge below Qadirabad is more than 5.50 lac cusecs Sub Divisional Officer Phalia will also shift his camp to Humber flood bund and will be overall incharge of watching Humber Protection Bund. In case of acute emergency District Coordination, Mandi Bahauddin and District Police Officer, Mandi Bahauddin will be approached to assist the Department.

ii).	FARRUKHPUR FLOOD BUND

The said bund falls on the right edge of Budi Nallah having a length of 6.80 Miles. The condition of this bund at RD 13-17, 21-22 and 31-32 is very much critical. During the spilling of flood water from the River Chenab and entring in the Budi Nallah and running parallel in the toe of bund which may damage the Farrukhpur Flood Bund and adjoining abadies i.e. Qadirabad city, Alan, Thathi Shah Muhammad, Noshera, Farrukhpur, Melu and Watto and then enters again in the River Chenab opposite village Farrukhpur.

CHAPTER – 6

FLOOD DAMAGES RESTORATION WORKS

- The following site was damaged during flood season 2010 and work completed during 2010-11 which are as under:-

| Sr.# | Name of Scheme |
| --- | --- |
| 1. | Protecting & Restoring J-Head Spur Chakori |

B)	The following sites were damaged during flood season 2013 and works have been taken up under para 2.89 of PWD Code.

| Sr. # | Name of scheme |
| --- | --- |
| 1 | Protecting abadies of village Bhutta Piran from erosion of right side of River Chenab. |
| 2 | Protection of Farrukhpur flood bund on erosion side at RD 11-12 & 14-15. |

CHAPTER – 7

FLOOD FIGHTING WATCHING ARRANGEMENT

7.1	PRE-FLOOD ARRANGEMENT

Necessary flood fighting material is available on stock. Structure wise status of Reserve Stock of stone is as under:-

| SR. NO. | NAME OF DIVISION /NAME OF INTERVENTION | SANCTIONED LIMIT OF RESERVE STOCK (Lac CFT) | STONE AVAILABLE AT SITE (LAC CFT) | BALANCE QUANTITY REQUIRED |
| --- | --- | --- | --- | --- |
| 1 | Humber flood bund. | 0.50 | 0 | 0.50 |
| 2 | Guide wall spur near Farrukhpur village. | 1.00 | 0.03 | 0.97 |
| 3 | J-head spur near Chakori village. | 1.00 | 0.45 | 0.55 |
| 4 | Guide wall spur near Noorpur Village. | 1.00 | 0 | 1.00 |
| 5 | Guide wall spur U/S Haveli Manik. | 1.00 | 0 | 1.00 |
| 6 | Farrukhpur Flood Bund | 0.50 | 0 | 0.50 |
|  | Total | 5.00 | 0.48 | 4.52 |

The balance quantity of stone will be procured after receipt of funds.

  - WATCHING ESTABLISHMENT

Following watching establishment will be engaged during the flood season for watching & general maintenance of flood works on monthly basis. However in case of very high flood or in real emergency the strength of watching establishment will be increased with the express permission of Executive Engineer Mandi Bahauddin Canal Division. Day & night watching will be done in the months of July, August & September as the chances of flood are maximum during these months.

| Sr#. | Site Description | Flood Limits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Low Flood |  | Medium Flood |  | High Flood |  | Very High Flood |  | Exceptionally High Flood |  |
|  |  | Man | Shift | Man | Shift | Man | Shift | Man | Shift | Man | Shift |
| 1 | Humber flood bund RD. 0-82+500 (16.50 Miles) | No Extra Labour will be employed |  | 3 | 8 Hrs | 4 | 8 Hrs | 6 | 8 Hrs | 8 | 8 Hrs |
| 2 | J-head spur chakori 0-3+500 (0.70 Miles) |  |  | 2 | 8 Hrs | 3 | 8 Hrs | 4 | 8 Hrs | 6 | 8 Hrs |
| 3 | Farrukhpur flood bund RD. 0-33+500 (6.70 Miles) |  |  | 3 | 8 Hrs | 4 | 8 Hrs | 6 | 8 Hrs | 8 | 8 Hrs |

* Note: Payment of Work charge Labor is Subject to verified by TPM / Competent authority

7.3	ARRANGEMENT AT SENSITIVE SITES

Necessary Flood Fighting Camps will be established at sensitive sites on 15th June 2024. The detail as under.

4)	RIVER CHENAB

- 06 No. Sites

On every spur necessary temporary camp sites shall be equipped with flood fighting material to meet with any emergency.

HUMBER FLOOD PROTECTION BUND.          =      RD 20000,33000,47000

  - WATCHING MATERIAL

| Sr. No. | Item | Required Quantity | Available | Balance required | Remarks / status |
| --- | --- | --- | --- | --- | --- |
| 1 | K. Oil | 1 Tin | -- | 1 Tin | The required material will be purchased on receipt of requisite funds before flood season 2025. Subject to approval by competent authority |
| 2 | Chimneys for lanterns | 5 No. | -- | 5 No. |  |
| 3 | Gunny bags with suit & sewing needles | 50 No. | -- | 50 No. |  |
|  | Ballies | 50 No. | -- | 50 No. |  |
|  | Service cable | 4 Coil | -- | 4 Coil |  |
| 6 | Wicks | 24 No. | -- | 24 No. |  |
| 7 | Sprit | 2 Bottles | -- | 2 Bottles |  |
| 8 | Match box | 1 Dozen | -- | 1 Dozen |  |
| 9 | Petrol | 500 Ltrs. | -- | 500 Ltrs. |  |
| 10 | M. Oil | 100 Ltrs. | -- | 100 Ltrs. |  |
| 11 | Wireless set Walky Talky VX-300 | 1 No. | -- | 1 No. |  |
| 12 | Spare chargeable cells AA size | 6 No. | -- | 6 No. |  |
| 13 | Gas lamp/Stove with 2 KG gas filled cylinder | 1 No. | -- | 1 No. |  |
| 14 | Cooking utensils 6 persons set | 1 No. | -- | 1 No. |  |
| 15 | Base set | 2 No. | -- | 2 No. |  |

WATCHING MATERIAL(TOTAL AVAILABLE

| Sr. No. | Item | Location | Total Quantity | Available Quantity | Balance Quantity | Remarks / status |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Lantern | Canal Rest House Bhikho | 140 No. | 140 No. | -- |  |
| 2. | Chimneys for lanterns |  | 70 No. | 70 No. | -- |  |
| 3. | Torches with cells |  | 42 No. | 42 No. | -- |  |
| 4. | Axes |  | 70 No. | 70 No. | -- |  |
| 5. | Kassies with handles |  | 100 No. | 100 No. | -- |  |
| 6. | Baskets |  | 110 No. | 110 No. | -- |  |
| 7. | Handles for kassies |  | 140 No. | 140 No. | -- |  |
| 8. | Gas lamps |  | 28 No. | 28 No. | -- |  |
| 9. | G.I. wire |  | 1400 kg. | 1400 kg. | -- |  |
| 10 | Generator Set large |  | 1 No. | 1 No. | -- |  |
| 11 | Generator set small |  | 1 No. | 1 No. | -- |  |
| 12 | Service cable (for lightning and charging arrangements) |  | 28 ft. | 28 ft. | -- |  |
| 13 | Wicks |  | 336 No. | 336 No. | -- |  |
| 14 | Sprit |  | 28 Bottles | 28 Bottles | -- |  |
| 15 | Match box |  | 14 dozen | 14 dozen | -- |  |
| 16 | Wooden mallet |  | 24 No. | 24 No. | -- |  |
| 17 | Search light |  | 10 No. | 10 No. | -- |  |
| 18 | Tents |  | 8 No. | 8 No. | -- |  |
| 19 | Wireless set Walky Talky VX-300 |  | 1 Set | 1 Set | -- |  |
| 20 | Spare chargeable cells AA size |  | 14 No. | 14 No. | -- |  |
| 21 | Gas lamp/Stove with 2 KG gas filled cylinder |  | 14 No. | 14 No. | -- |  |

| Sr. No. | Item | Quantity | Balance | Required |
| --- | --- | --- | --- | --- |
| 22 | Pedestal fan | 14 No. | 14 No. | -- |
| 23 | Cooking utensils 6 persons set | 14 No. | 14 No. | -- |
| 24 | Patha ban | 10 K.G | 10 K.G | -- |
| 25 | Shouldari | 7 No. | 7 No. | -- |
| 26 | Table with six Chairs | 2 Set | 2 Set | -- |
| 27 | Camping Chairs | 2 Dozen | 2 Dozen | -- |
| 28 | Charpai | 6 No. | 6 No. | -- |
| 29 | Pillow | 6 No. | 6 No. | -- |

7.5	ARRANGEMENT FOR SOUNDING & PROBING

There is no proper arrangements for sounding & probing in this Division.

7.6	LIGHTING ARRANGEMENT

During peak flood hours, flood fighting operations will be carried out  round the clock. In case of windstorm and rain, lantern and gas lamps fail to work, therefore, to overcome the problem, a reliable generator set with all accessories will be kept on each flood bund, which will facilitate successful operation. For this purpose, sufficient number of generators have already been arranged by this division.

7.7	RATION ARRANGEMENT

Relates to District Administration.

7.8	P.O.L ARRANGEMENT FOR VEHICLE

In case the petrol, oil and lubricants are in short supply on account of disruption of communication elsewhere in the country, the Executive Engineer Incharge will make timely arrangements for having adequate P.O.L reserve at convenient places i.e. petrol pumps at Phalia, Mandi Bahauddin and Qadirabad in consultation with and through an order by the concerned District Coordination Officer.

7.9	TRANSPORTATION

Presently 4 No. 4 X 4 vehicles are available in Mandi Bahaudin Canal Division. Additional vehicles, if required, will be arranged from other Departments.

EARTH MOVING MACHINERY

Agriculture department Mandi Bahauddin will made 2 No. Dozers: for for District M.B.Din).  At the time of emergency for closing leakage or breaches on flood bunds/spurs.

7.10	LAW AND ORDER

Experience shows that whenever any flood emergency arises, inhabitants of the area try to cut the protection bund at upstream to divert the flood threat from their locality and a law order situation being arised.

In such a situation, it would be necessary for District Administration to provide:

a)        Adequate day & night patrolling to guard against any attempt to stealthily cutting a bund

b)      Adequate provision of police force for any open and defiant attempt of breaching the bund at any place.

In these situations, the District Coordination Officer and the District Police Officer of the respective Districts would be expected to promptly provide adequate police force accompanied with a Magistrate on being apprised of the situation by the officers of Irrigation.

7.11	MEDICAL ARRANGEMENT FOR LABOUR

Relates to District Administration.

7.12	LIAISON WITH OTHER DEPARTMENT

The District Coordination Officer & District Police Officer will be contacted in advance and a list of volunteers will be prepared with whom contact will be made during emergency. The list containing the names of volunteers and the village lumberdars made responsible by the Civil Authorities will also be maintained.

7.13	ROLE OF THE ARMY

Pak Army Representative has to inspect the flood infrastructure before flood season. The irrigation department will remove the deficiency pointed out by the army authority before coming flood season. Any emergent situation will be reported by the Irrigation Department to the Pak Army.

7.14   DUTY OF TELEPHONE ATTENDANT

Telephone Attendant will be available round the clock at Divisional Flood Emergency Camp. The detail is as under:-

| Shift No.. | Name of Official | Cell No. | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Safdar Iqbal Signaler | 0349-7271963 | 7.00 AM | 3.00 PM |
| 2 | Muhammad Abbas Signaler | 0345-5863332 | 3.00 PM | 11.00 PM |
| 3 | Riaz Ahmad Signaler | 0341-8422068 | 11.00 PM | 7.00 AM |

Telephone Number of Telegraph Office at canal colony Mandi Bahauddin 0546-510480.

7.15 WIRELESS ARRANGEMENTS

Speedy means of communications will be adopted through departmental Wireless sets, Canal Telegraph, Messengers & running the jeep drivers to keep a close liaison beginning from that of watchman to the Superintending Engineer. No important event should take more than one hour to be conveyed to the Executive Engineer and two hours to the Superintending Engineer. Mobile wireless sets have already been purchased and given to the Executive Engineer Mandi Bahauddin (Code ‘AMJAD MBDIN’) and all the four S.D.Os (Code of S.D.O Mandi Bahauddin is ‘SADIQ MBDIN’) & Sub engineers (Code is on their names) and also applied for wireless frequency. At present this office is using frequency of Police. Base sets have also been purchased out of which one has been installed at Canal Telegraph Office Mandi Bahauddin and Phalia. During flood season, main camp will be established at Spur No. 8 (Right Bank of the River Chenab) for District Mandi Bahauddin and at RD 40000 of Humber Flood bund for District Mandi Bahauddin as these points are centrally located and are, therefore, most suitable for exercising comprehensive check at all the vulnerable points. The concept of establishment of main camps have already been acceded by the Army Monitors and District Administration of District Mandi Bahauddin during flood season, 2000.

Similarly wireless communication has been established in Jhelum Division UJC. Good interaction is available through intercommunication network. The entire “CHAJ DOAB AREA” has been covered with this communication network.

Direct communication with the Flood Information Center Lahore is required to maintain for collection of necessary flood and rainfall data in upper catchments, so that timely intimation be conveyed to the concerned quarters. Similarly one Base station has been installed with the co-ordination of Executive Engineer Murala to receive current information on wireless.

CALLING CODE DATA OF IRRIGATION OFFICERS ON WIRELESS

| Sr. No. | Officer/official with designation | Calling Code | Police control/frequency |
| --- | --- | --- | --- |
| 1. | Chief Engineer, Sargodha | Farzand Sargodha | Sargodha |
| 2. | Superintending Engineer, Mandi Bahauddin Canal Circle | Amjad-1 MBDIN | Mohsin Control/162.975 |
| 3. | Executive Engineer, Mandi Bahauddin | Amjad M.B.Din | Azam Control/162.800 |
| 4. | Executive Engineer, Jhelum | Amjad Jhelum | Mohsin Control/162.975 |
| 5. | Executive Engineer, Murala | Amjad Murala | Nisar Sialkot/162.975 |
| 6. | Sub Divisional Officer, Phalia | Sadiq M.B.Din | Azam Control/162.800 |
| 7. | Sub Divisional Officer, Phalia | Sadiq Phalia | Salah-ud-Din/163.850 |
| 8. | Sub Divisional Officer, Sohawa | Sadiq Sohawa | Salah-ud-Din/163.850 |
| 9. | Sub Divisional Officer, Busal | Sadiq Busal | Salah-ud-Din/163.850 |
| 10. | Sub Engineer, Headquarter | Qaiser Mehmood | Azam Control/162.800 |
| 11. | Sub Engineer, Bhikho | Muhammad Rizwan | -- |

FREQUENCY FOR THE POLICE CHANNELS

| Channel No.1 | 157.975 | Lahore |
| --- | --- | --- |
| Channel No.2 | 162.800 | Azam Control M.B.Din |
| Channel No.3 | 162.850 | Zulfiqar, Gujranwala |
| Channel No.4 | 160.775 | Atta Control Chakwal |
| Channel No.5 | 162.900 | Shaheen, Murree |
| Channel No.6 | 162.925 | --- |
| Channel No.7 | 162.950 | -- |
| Channel No.8 | 162.975 | Mohsin, Jhelum |
| Channel No.9 | 160.025 | --- |
| Channel No.10 | 161.400 | Nisar,Sialkot |
| Channel No.11 | 163.850 | Salah-ud-Din, M.B.Din |
| Channel No.12 | 163.875 | Hassan, P.C Rawat |
| Channel No.13 | 163.900 | --- |
| Channel No.14 | 163.925 | --- |
| Channel No.15 | 163.950 | --- |
| Channel No.16 | 163.975 | Faisalabad |

CHAPTER – 8

DETAIL OF ENCROACHMENT

There is no encroachment on any flood protection work in Mandi Bahauddin Canal Division Some private properties have been constructed by the original owners on the flood banks particularly in Phalia Canal Sub Division. Department has not acquired the land, hence these constructions does not fall in the category of encroachments.

| Total Encroachment | Removal | Balance | Action to be Taken |
| --- | --- | --- | --- |
| --------------------------Nil-------------------------------- |  |  |  |

CHAPTER – 9

DUTY ROSTER/FLOOD FIGHTING PROGRAM

Executive Engineer, Mandi Bahauddin Canal Division.

Executive Engineer will be overall in charge of flood activities in the jurisdiction of Mandi Bahauddin Canal Division. The flood protection works falling in the areas of District Mandi Bahauddin are in the administrative control of Executive Engineer. He is responsible for maintenance and supervision of flood protection / watching arrangement during flood season.  He will also keep liaison with the civil / army authorities and Superintending Engineer Mandi Bahauddin Canal Circle Mandi Bahauddin Jhelum regarding the situation.

In case of emergency, on the request of Sub Divisional Officers, Phalia Sub Division and Sohawa Sub Division, the Executive Engineer Mandi Bhauddin will order to deploy Sub Divisional Officer Sohawa and Busal along with their subordinate staff to assist/coordinate Sub Divisional Officer Phalia to cope with the emergency.  Their duty roster will be as under:-

Sub Divisional Officer, Phalia Canal Sub Division.

In case of emergency and high flood, he will be responsible for the watch and ward of Humber Flood Bund & Flood bunds along Right side of River Chenab falling in Phalia Sub Division. He will be mainly responsible for the protection of flood infrastructure & coordination with the Army/District Civil Administration/Executive Engineer Mandi Bahauddin Canal Division and also camp at Humber Flood bund at RD 40000 with base station on police frequency Mandi Bahauddin.

Sub Engineer, Chillianwala.

Sub Engineer, Challianwala will be attached at Headquarter Phalia to attend the emergency if any.

Sub Engineer, Chhimon.

Sub Engineer, Chhimon  will be incharge of Humber Bund, watch all flood activities and remain present round the clock with a gang of labour to look after flood infrastructures.

- Sub Engineer, Wasu.

Sub Engineer, Wasu will be incharge of Farrukhpur Flood Bund, watch all flood activities and remain present round the clock with a gang of labour to look after flood infrastructures.

Sub Engineer, Dinga.

Sub Engineer, Dinga will be incharge of Humber Flood Bund and watch all flood activities and remain present round the clock with a gang of labour to look after flood infrastructures.

- Sub Engineer, Phalia.

Sub Engineer, Phalia Section will be incharge of Guide wall spur Bherowal, Haveli Manik & Noorpur to watch all flood activities and remain present round the clock with a gang of labour to look after flood infrastructures.

- Sub Engineer, Bhikho

Sub Engineer, Bhikho Section will be incharge of Farrukhpur Flood Bund, J-head spur Chakori and Guide wall to watch all flood activities and remain present round the clock with a gang of labour to look after flood infrastructures.

CHAPTER – 10

EMERGENCY TELEPHONE NUMBERS

| Sr. # | Designation |  | Office | Residence |
| --- | --- | --- | --- | --- |
| i) | IRRIGATION DEPARTMENT IRRIGATION & POWER SECRETARIAT LAHORE |  |  |  |
| 1. | Minister, Irrigation | 042-99212809-10 |  |  |
| 2. | Secretary, I&P Department Lahore | 042-99212117-18 |  | 042-35731116 |
| 3. | Addl. Secretary (Admn.) Lahore. | 042-99212119 |  | -- |
| 4. | Addl. Secretary (OP) Lahore. | 042-99212129 |  | 042-5224359 |
| 5. | Addl. Secretary (Technical) Lahore. | 042-99212123 |  | 042-6818219 |
| 6. | Deputy Secretary (Admn.) Lahore. | 042-99212126 |  | 042-570392 |
| 7. | Deputy Secretary (OP) Lahore. | 042-99212127 |  | 042-7848174 |
| 8. | Deputy Secretary (Development) Lahore. | 042-99212128 |  | 042-7586597 |
| 9. | Section Officer (Flood) Lahore. | 042-99212134 |  | 042-7581015 |
| ii) | SARGODHA IRRIGATION ZONE |  |  |  |
| 1. | Chief Engineer, Irrigation Sargodha | 048-9230445 |  | 048-9230451 |
| 2. | Executive Engineer (OP) Sargodha. | 048-9230446 |  | 048-729067 |
| 3. | Superintending Engineer, Mandi Bahauddin | 0546-510410 |  | 0546-510410 |
| 4. | Telegraph Office Jhelum. | 0544-9270330 |  | -- |
| 5. | Executive Engineer, Mandi Bahauddin Canal Division. | 0546-510480 |  | 0546-510480 |
| 6. | Executive Engineer, Gujrat Canal Division. | 053-9260398 Fax -9260396 |  | 053-9260398 |
| 7. | Sub Divisional Officer Gujrat. | 053-9260395 |  | 053-9260395 |
| 8. | Sub Divisional Officer Sohawa M.B.Din. | 0546-506562 |  | 0546-506562 |
| 9. | Sub Divisional Officer Busal M.B.Din. | 0546-506561 |  | 0546-506561 |
| 10. | Sub Divisional Officer Phalia. | 0546-596811 |  | 0546-596811 |
| 11. | Sub Divisional Officer S&I Gujrat | 053-9260397 |  | 053-9260397 |
| 12. | Telegraph office Gujrat (Flood Emergency Office) | 053-9260395 |  |  |

| iii) | HEADWORKS DIVISIONS |  |  |  |
| --- | --- | --- | --- | --- |
| 1. | Executive Engineer, Murala. | 052-3502102 |  | -- |
| 2. | Executive Engineer, Khanki. | 055-6605255 |  | -- |
| 3. | Executive Engineer, Qadirabad. | 054-7550198 |  | 054-7424272 |
| iv) | DISTRICT ADMINISTRATION GUJRAT |  |  |  |
| 1. | District Administrator, Gujrat. | 053-9260071-72 |  | 053-511109 |
| 2. | District Coordination Officer, Gujrat. | 053-9260010 |  | 053-9260011 |
| 3. | District Police Officer, Gujrat. | 053/9260026-30-31 |  | 053-9260027 |
| 4. | E.D.O (Revenue), Gujrat. | 053-9260066 |  | 053-9230077 |
| 5. | District Officer (Revenue), Gujrat. | 053-9260061 |  | 053-9260062 |
| 6. | District Flood Control Officer, Gujrat. | 053-9260066 |  | -- |
| 7. | D.D.O (Revenue) Gujrat. | 053-9260063 |  | 053-9260064 |
| 8. | D.D.O (Revenue) Kharian. | 053-37532200 |  | -- |
| 9. | D.D.O (Revenue) Sarai-Alamgir. | 053-37651812 |  | 0544-614703 |
| 10. | S.D.P.O Sadar Gujrat. | 053-9260046 |  | -- |
| 11. | D.S.P Kharian. | 053-37511950 |  | -- |
| 12. | D.S.P Sara-i-Alamgir. | 053-37511950 |  | -- |
| v) | DISTRICT ADMINISTRATION MANDI BAHAUDDIN. |  |  |  |
| 1. | District Administrator, Mandi Bahauddin. |  | 0546-520900 | 0546-299009/ 599002 |
| 2. | District Coordination Officer, M.B.Din. |  | 0546-504220 | 0546-504200 |
| 3. | District Police Officer, M.B.Din. |  | 0546-502324 | -- |
| 4. | E.D.O (Revenue) M.B.Din. |  | 0546-501825 | 0546-506340 |
| 5. | D.D.O (Revenue) Phalia. |  | 0546-596460 | 0546-596690 |
| 6. | D.D.O (Revenue) M.B.Din. |  | 0546-506488 | 0546-505145 |
| 7. | D.D.O (Revenue) Malakwal. |  | 0546-591276 | 0546-591368 |

PAKISTAN BROAD CASTING CORPORATION

NEWS ORGANIZATION LAHORE

| Sr. No. | Designation | Office | Residence |
| --- | --- | --- | --- |
| 1. | Deputy Controller Incharge News Editor | 042-99200674 | 042-5862694 |
| 2. | Deputy Controller News | 042-99200681-4 | Ext. |
| 3. | Radio F.M 105 Gujrat. | 053-3520320 & 3520785 |  |
| 4. | Radio F.M 98 M.B.Din | 0546-509798 |  |

PAKISTAN TELEVISION CORPORATION LAHORE

| Sr. No. | Designation | Office | Residence |
| --- | --- | --- | --- |
| 1. | News Room | Direct= 042-99200617 042-99200618 | -- |
|  |  | Exchange= 042-99200651-59 | -- |
|  |  | Fax = 042-99200620 |  |

FLOOD CONTROL CENTER PUNJAB

| Sr. No. | Designation | Office | Residence |
| --- | --- | --- | --- |
| 1. | Flood warning Center, 46-Jail Road Lahore. | 042-99205371 | Fax 042-99205372 |
| 2. | Flood Cell, I&P Secretariat Lahore. | 042-99212134 | Fax 042-99212116 |
| 3. | Police Wireless Control, Jail Road Lahore. | 042-7572090 | -- |
| 4. | Flood Emergency Board of Rev. Punjab. | 042-99204408 | Fax-042-99204405 |
| 5. | National Flood Forecasting Bureau Lahore. | 042-7586479 | 042-7562874 |
| 6. | Senior Meteorologist | 042-99240402 | -- |
| 7. | Chief Engineer (D&F) Zone Lahore | 042/99233551 | -- |
| 8. | Director Flood (D&F) Zone Lahore | 042/99233552 | -- |
| 9. | Flood Cell (I) Secretariat Lahore | 042/99212134 |  |
| 10. | National flood forecasting Bureau Lahore | 042/99200208 | 042/7583747 |

MILITARY ENGINEERS TELEPHONE Nos.

| 1. | Headquarters Engineers, 30-Corps Gujranwala Cantonment. | 0432-269/2353 | -- |
| --- | --- | --- | --- |
| 2. | 24-Engineers Battalion, Sialkot Cantonment Sialkot. | 0432-571/5186 | -- |
| 3. | Headquarters Engineers 1-Corps, Kharian Cantonment. | 05771-095/2364 | -- |
| 4. | 15-Div. Sialkot | 0432-571/3223 & 3225 |  |
| 5. | 10-Brigade Sialkot | 0432-571/4202 |  |

CHAPTER – 11

STANDARD OPERATING PROCEDURE (SOP) FOR BREACHING SECTIONS

11.1	HISTORY OF THE BREACING SECTION

There is no breaching section on any flood protection work in Mandi Bahauddin Canal Division.

  - LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION

---------- NIL ----------

  - ARRANGEMENT OF EXPLOSIVES AND SECURITY OF EXPLOSIVE STORES.						__

---------- NIL ----------

  - LIST OF THE SECURITY STAFF ALONGWITH DETAIL OF THEIR TRAINING ETC.						__

---------- NIL ----------

  - DETAIL OF MECHANICAL MEANS AS A STANDBY ARRANGEMENTS IN CASE OF DETONATION FAILURE.	__

---------- NIL ----------

  - DUTY ROSTER IN CASE OF CRITICAL SITUATION.

---------- NIL ----------

  - BREACHING COMMITTEE WITH THEIR ACTION PLAN

---------- NIL ----------

  - LIST OF THE VILLAGES LIKELY TO BE INUNDATED IN CASE OF BREACH.

---------- NIL ----------

  - ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENTS							__

---------- NIL ----------

  - DETAILS OF COORDINATION WITH CIVIL/ARMY AUTHORITIES

---------- NIL ----------

  - PARALLEL COMMUNICATION ARRANGEMENTS

---------- NIL ----------

  - INDEX PLAN      -----------NIL------------

Superintending Engineer				      Executive Engineer

Mandi Bahauddin Canal Circle			Mandi Bahauddin Canal Division

Mandi Bahauddin					Mandi Bahauddin

Chief Engineer

Irrigation Sargodha Zone

Sargodha

PART – B

CHAPTER – 12

VULNERABLE SITES ON FLOOD BUNDS/STRUCTURES

12.1	APPREHENDED BRACHES IN FLOOD BUNDS/STRUCTURES

There is no vulnerable site at current situation.

12.2	Operation of Breaching Sections.

No Breaching section exists under the jurisdiction of Mandi Bahauddin Canal Division.

12.3	Breaches due to rising of flood water, deterioration of flood bunds etc.

No such occurrence.

CHAPTER – 13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITES

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT/INUNDATED AREA ALONG RIVER & BUNDS OF THE BREACH SUPPORTED WITH LEVELS.

NIL

  - DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED AND THIS SHOULD ALSO BE SHOWN ON THE PLAN.

NIL

13.3   Strategic & Action Taken be Explain in Details this may include.

13.3.1 -   Arrangements

13.3.2 – Establishment of Flood Fighting Camps

Farrukhpur flood bund		=	RD 14000, RD 21000 & 						   	RD 31000

WATCHING ESTABLISHMENT

| Sr. No. | Name of flood bunds | Camp RD | Beldar | Mate | Gauge Reader | Chowkidar |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Farrukhpur flood bund | 14000 | 5 | 1 | -- | 1 |
|  | Farrukhpur flood bund | 21000 | 5 | 1 | -- | 1 |
|  | Farrukhpur flood bund | 31000 | 5 | 1 | -- | 1 |
|  | Total |  | 15 | 3 | -- | 3 |

13.3.3 – Duties of Officers/Officials and their camp sites.

| CAMP No. | OFFICERS (Name, Designation& Mobile No.) | OFFICIALS (Name, Designation& Mobile No.) |
| --- | --- | --- |
| 1 | Ch. Ahmed Raza Cheema Sub Divisional Officer Phalia (0348-0971455) | Muhammad Rizwan (Sub Engineer Bheikho Section 0321-6244014) |

Executive Engineer, Mandi Bahauddin Canal Division.

Executive Engineer will be overall incharge of flood activities in the jurisdiction of Mandi Bahauddin Canal Division. The flood protection works falling in the  areas of District Mandi Bahauddin are under the administrative control of Executive Engineer. He is responsible for maintenance and supervision of flood protection / watching arrangement during flood season.  He will also keep liaison with the civil / army authorities and Superintending Engineer Mandi Bahauddin Canal Circle Jhelum regarding the situation.

Sub Divisional Officer, Phalia Canal Sub Division.

In case of emergency and high flood, he will be responsible for the watch and ward of Humber Flood Bund, Farrukhpur Flood bund and river training works along Right side of River Chenab falls in Phalia Sub Division. He will be mainly responsible for the protection of flood infrastructure & coordination with the Army/District Civil Administration/Executive Engineer Mandi Bahauddin Canal Division and also camp at Humber Flood bund at RD 40000 with base station on police frequency Mandi Bahauddin.

Sub Engineer, Kadher

Sub Engineer, kadher will be Incharge of Farrukhpur Flood bund to watch all activities and remain present round the clock with a gang of labour to look after flood infrastructures.

13.3.4 – Departmental machinery available.

No departmental machinery is available in this Division.

13.3.5 – Machinery available from Private Source

Following machinery can be hired in case of emergency from private sources. List of machinery is under:-

- Dozer

- Excavator Machine

- Dumper

- Tractor with trolley

13.3.6 – Flood Fighting Material required

The detail is prescribed at page No. 39.

13.3.7- Flood Fighting Material available.

The detail is prescribed as under.

13.3.8	FLOOD FIGHTING MATERIAL AVAILABLE IN HEADQUARTERS SUB DIVISION

| Sr. No. | Item | Required Quantity | Available Quantity | Balance Quantity | Location | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Lantern | 140 No. | 140 No. | -- | Canal Colony Duggal | Nil |
| 2. | Chimneys for lanterns | 70 No. | 70 No. | -- |  |  |
| 3. | Torches with cells | 42 No. | 42 No. | -- |  |  |
| 4. | Axes | 70 No. | 70 No. | -- |  |  |
| 5. | Kassies with handles | 100 No. | 100 No. | -- |  |  |
| 6. | Baskets | 110 No. | 110 No. | -- |  |  |
| 7. | Handles for kassies | 140 No. | 140 No. | -- |  |  |
| 8. | Gas lamps | 28 No. | 28 No. | -- |  |  |
| 9. | G.I. wire | 1400 kg. | 1400 kg. | -- |  |  |
| 10. | Generator Set Small | 1 No. | 1 No. | -- |  |  |
| 11 | Generator set Large | 1 No. | 1 No. | -- |  |  |
| 12 | Service cable | 28 ft. | 28 ft. | -- |  |  |
| 13 | Wicks | 336 No. | 336 No. | -- |  |  |
| 14 | Sprit | 28 Bottles | 28 Bottles | -- |  |  |
| 15 | Match box | 14 dozen | 14 dozen | -- |  |  |
| 16 | Wooden mallet | 24 No. | 24 No. | -- |  |  |
| 17 | Search light | 10 No. | 10 No. | -- |  |  |
| 18 | Tents | 8 No. | 8 No. | -- |  |  |
| 19 | Wireless set Walky Talky VX-300 | 1 Set | 1 Set | -- |  |  |
| 20 | Spare chargeable cells AA size | 14 No. | 14 No. | -- |  |  |
| 21 | Gas lamp/Stove with 2 KG gas filled cylinder | 14 No. | 14 No. | -- |  |  |
| 22 | Pedestal fan | 14 No. | 14 No. | -- |  |  |
| 23 | Cooking utensils 6 persons set | 14 No. | 14 No. | -- |  |  |
| 24 | Patha ban | 10 K.G | 10 K.G | -- |  |  |
| 25 | Shouldari | 7 No. | 7 No. | -- |  |  |
| 26 | Table with six Chairs | 2 Set | 2 Set | -- |  |  |
| 27 | Camping Chairs | 2 Dozen | 2 Dozen | -- |  |  |
| 28 | Charpai | 6 No. | 6 No. | -- |  |  |
| 29 | Pillow | 6 No. | 6 No. | -- |  |  |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS, TELEPHONE INSTALLATION, ROAD NETWORK, OTHER BUILDINGS, CANALS AND DRAINAGE NETWORK.

There exist Electric line, Sui gas, telephone installation, road, building canals and drain in the area

13.4.1	Detail of other infrastructure like electric, Sui Gas, Telephone installations, road network, other buildings, Canals and drainage network.

There exist Electric line, Sui gas, telephone installation, road, building canals and drain in the area.

FLOOD FIGHTING MATERIAL REQUIRED / AVAILABLE

| Sr. No. | Item | Required | Available | Balance | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | K. Oil | 1 Tin | -- | 1 Tin | The balance Flood Fighting material will be purchased before flood season 2025. Subject to approval of competent authority |
| 2 | Chimneys for lanterns | 5 No. | -- | 5 No. |  |
| 3 | Gunny bags with suit & sewing needles | 50 No. | -- | 50 No. |  |
| 4 | Ballies | 50 No. | -- | 50 No. |  |
| 5 | Service cable | 1 Coil |  | 1 Coil |  |
| 6 | Wicks | 6 No |  | 6 No. |  |
| 7 | Sprit | 1 Bottles | -- | 1 Bottles |  |
| 8 | Mach boxes | 1 Packet | -- | 1 Packet |  |
| 9 | Petrol | 50 Ltrs. | -- | 50 Ltrs. |  |
| 10 | M. Oil | 10 Ltrs. | -- | 10 Ltrs. |  |
| 11 | Wireless set Walky Talky VX-300 | 1 No. | -- | 1 No. |  |
| 12 | Spare chargeable cells AA size | 2 No. |  | 2 No. |  |
| 13 | Gas lamp/Stove with 2 KG gas filled cylinder | 1 No. | -- | 1 No. |  |
| 14 | Lantern | 10 No. | 10 No. | -- | Available in Canal Rest House Bhikho |
| 15 | Chimneys for lanterns | 7 No. | 7 No. | -- |  |
| 16 | Torches with cells | 6 No. | 6 No. | -- |  |
| 17 | Axes | 5No. | 5No. | -- |  |
| 18 | Kassies with handles | 10 No. | 10 No. | -- |  |
| 19 | Baskets | 15 No. | 15 No. | -- |  |
| 20 | Handles for kassies | 5 No. | 5 No. | -- |  |
| 21 | Gas lamps | 1 No. | 1 No. | -- |  |
| 22 | G.I. wire | 100 kg. | 100 kg. | -- |  |
| 23 | Generator set | 1 No. | 1 No. | -- |  |
| 24 | Service cable | 28 ft. | 28 ft. | -- |  |
| 25 | Wicks | 30 No. | 30 No. | -- |  |
| 26 | Sprit | 2 Bottles | 2 Bottles | -- |  |
| 27 | Match box | 1 dozen | 1 dozen | -- |  |
| 28 | Wooden mallet | 2 No. | 2 No. | -- |  |
| 29 | Search light | 1 No. | 1 No. | -- |  |
| 30 | Tents | 1 No. | 1 No. | -- |  |
| 31 | Wireless set Walky Talky VX-300 | 1 Set | 1 Set | -- |  |
| 32 | Spare chargeable cells AA size | 2 No. | 2 No. | -- |  |
| 33 | Gas lamp/Stove with 2 KG gas filled cylinder | 1 No. | 1 No. | -- |  |

.

Superintending Engineer					Executive Engineer

Mandi Bahauddin Canal Circle			Mandi Bahauddin Canal Division

Mandi Bahauddin					Mandi Bahauddin

Chief Engineer

Irrigation Sargodha Zone

Sargodha

(Chapter 14)

ACTION PLAN

14.1	Re-shuffling / Recouping plan of reserve stock stone Departmentally.

| Sr. No. | Name of Flood Infrastructures having reserve stock stone. | Available Quantity of Stone (Lac Cft) | Sites where stone can be used during Flood emergency |
| --- | --- | --- | --- |
| 1 | J-head spur near Chakori on right bank of River Chenab. | 1.03 | 1. J-Head Spur Chakori Village. 2. J-Head Spur Haveli Manak. 3. Farrukhpur Flood Bund on right bank of River Chenab. |

Reserve stock of stone will be reshuffled through Govt.Contractors

14.2	Detail of inlet . Outlet crossing along with closing methodology.

--------Nil------

14.3		Deployment Machinery (Medium to high flood)

Attached at Page 46

14.4		Deployment Machinery (High to very high flood)

Attached at Page 47

14.5	Deployment Machinery ( Very High to exceptionally high flood)

Attached at Page 48

14.6		Police Deployment plan

Attached at page 49

14.7	Details of synthetic bags with capacity of 500 kg and 1000 kg

| Synthetic Bags Capacity | Required | Available | Balance |
| --- | --- | --- | --- |
| 500 Kg. | 200 No. | Nil | 200 No. |
| 1000 Kg | 250 No. | Nil | 250 No. |

14.8          Details of polythene sheet of black color to protect upstream slope

against wave action and to control seepage through embankments

| Polythene Sheet | Required | Available |
| --- | --- | --- |
| Nil | Nil | Nil |

ACTION PLAN DURING FLOODS ON RIVER CHENAB

14.3	(DEPOLYMENT MACHINARY MEDIUM TO HIGH FLOOD)

| Sr. No. | Name of structure | Length in mile | Vulnerable reach | Camp location | Site incharge by name & Cell No. | Machinery deployed |  |  |  |  | Availability of stone Lac cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/ Dumpers | Tractor with front blade | Labour (Beldar+ Mate) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Humber flood bund | 16.500 | RD 31,47,72 | RD 31,47,72 | Muhammad Rizwan, (Sub Engineer) 0321-6244014) | 1 | 0 | 5 Tractor Trolleys | 0 | 10 Beldar + 1 Mate | 0 |
| 2 | Farrukhpur flood bund | 6.800 | RD 14-17, 21-22 & 31-32 | RD 14,21 & 31 | Muhammad Nabeel Ahmed Zafar, (Sub Engineer) (0344-6541108) | 1 | 0 | 5 Tractor Trolleys | 0 | 10 Beldar + 1 Mate | 0 |
| TOTAL |  | 23.30 |  |  | 2 | 2 |  | 10 Tractor Trolleys |  |  |  |

Note: - Payment for machinery and work charge employee subject to verification by TPM.

ACTION PLAN DURING FLOODS ON RIVER CHENAB

14.4		(DEPOLYMENT MACHINARY HIGH TO VERY HIGH FLOOD)

| Sr. No. | Name of structure | Length in mile | Vulnerable reach | Camp location | Site incharge by name & Cell No. | Machinery deployed |  |  |  |  | Availability of stone Lac cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/ Dumpers | Tractor with front blade | Labour (Beldar+ Mate) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 1 | Humber flood bund | 16.500 | RD 31,47,72 | RD 31,47,72 | Muhammad Rizwan, (Sub Engineer 0321-6244014) | 1 | 0 | 5 Tractor Trolleys | 0 | 10 Beldar + 1 Mate | 0 |
| 2 | Farrukhpur flood bund | 6.800 | RD 14-17, 21-22 & 31-32 | RD 14,21 & 31 | Muhammad Nabeel Ahmed Zafar, (Sub Engineer) (0344-6541108) | 1 | 0 | 5 Tractor Trolleys | 0 | 10 Beldar + 1 Mate | 0 |
| TOTAL |  | 23.30 |  |  | 2 | 2 | 0 | 10 | 0 |  |  |

Note: - Payment for machinery and work charge employee subject to verification by TPM.

PLAN DURING FLOODS ON RIVER CHENAB

14.5	  (DEPOLYMENT MACHINARY HIGH TO EXCEPTIONALLY HIGH FLOOD)

| Sr. No. | Name of structure | Length in mile | Vulnerable reach | Camp location | Site incharge by name & Cell No. | Machinery deployed |  |  |  |  | Availability of stone Lac cft |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Excavator | Dozer | Trolleys/ Dumpers | Tractor with front blade | Labour (Beldar+ Mate) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 15 | Humber flood bund | 16.500 | RD 31,47,72 | RD 31,47,72 | Muhammad Rizwan, (Sub Engineer 0321-6244014) | 1 | 0 | 5 Tractor Trolleys | 0 | 10 Beldar + 1 Mate | 0 |
| 16 | Farrukhpur flood bund | 6.800 | RD 14-17, 21-22 & 31-32 | RD 14,21 & 31 | Muhammad Nabeel Ahmed Zafar, (Sub Engineer) (0344-6541108) | 1 | 0 | 5 Tractor Trolleys | 0 | 10 Beldar + 1 Mate | 0 |
|  | TOTAL | 23.30 |  |  | 2 | 2 | 0 | 10 Tractor Trolleys | 0 |  |  |

Note: - Payment for machinery and work charge employee subject to verification by TPM.

14.6 POLICE DEPLOYMENT PLAN

| S.No. | Vulnerable sites | Concerned Canal Division | Concerned Police station & District | Police persons to be deployed |  | Remarks (if any) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Inspector/ SI/ASI | Constable |  |
| 1 | Farrukhpur flood bund along left bank of Budi Nallah Drain RD 14000-17000, 21000-22000 & 31000-32000 | Mandi Bahauddin | Qadirabad Mandi Bahauddin | Sub Inspector = 03 Nos. | 12 Nos |  |

(Chapter 15)

BACK UP DIVISIONS (IN CASE OF BREACH)

It is certified that no breaching section exists in the jurisdiction of this Division. The backup Division is Drainage Division Mandi Bahauddin.

Executive Engineer

Mandi Bahauddin Canal

Mandi Bahauddin