FLOOD FIGHTING PLAN 2025

PART-A

CHAPTER – 1

SALIENT FEATURE OF THE SARGODHA DRAINAGE DIVISION SARGODHA.

INTRODUCTION

1.1 LOCATION

Sargodha District is surrounded by District Mandi Bhauddin, Chackwal,Khushab,Jhang and Hafiz Abad. District Sargodha lies in between two main rivers which are called as River chanab on east side  and River jhelum on North west side, which means the District is part of Chaj Doab. Chaj Doab  is combination of two wards. chaj stands for Chanab and Jhelun , where as Doab means two rivers/ waters. The main populations inhabitated near River Jhelum are Malakwal, Chak Saida,Miani, Hazoor Pur, Bhera,Chak Mubarak Abad,Jhawervian,Shah pur Sahiwal, Nehang, Seyal Sharif Havaily Nathoka and Havely Majhokah. On the other Hand the populations of Taheli Adda, Halal Pur, Takhat Hazara, Midh Ranjha, Talib Wala,and Jallah Makhdoom are existed on the right bank of river Chanab. At its down stream Districts Chiniot and District Jhang are situated.

1.2 General Description

TOPOGRAPHY.

The area of District Sargodha is generally leveled but divided in three small patches or bowls. Mainly it is a lot river terrace influenced by Chanab river deposit. Natural elevation ranges from 550 Ft above sea level near to the confluences of River Chanab and Jhelum and 650 Ft near the boundary of Mandi Bahauddin. However the area is divided into pockets if to be seen on contour sheet, around the Sargodha city. The area of Northern Branch can be separated as one, the area of Tehsil Bhalwal and North Western area can be divided into three non proper Drainage ranges/ pockets. The areas situated near the banks of the rivers have also changed their natural topography which is quite clear from the Difference of old and new  survey work.

WEATHER

The climate of the Sargodha and its allied Districts is characterized by large seasonal factual of both temperature and precipitation except the Mandi Bahauddin which is to be considered as to have an effect of environmental impact of climatic conditions analogical to the situation of Jhelum and Azad Kashmir, having the joint climatic boundary.

The temperature in winter ranges between 60 to 80 Degree F During the day whereas it varies from 35 to 45 during night. The annual average rain fall in Sargodha District ranges from 47mm to 60mm. the Major rain fall occur in Moon Soon i.e.70%

SOIL

The geological condition of the soil in general in almost all the entity is to be seen as alluvial sediment. The particle size varies from medium sand to clay. The meandering braiding and flooding activities of the rivers resulted in strong and stratification of sediments which is more or less uniform within the limits of land farm.

The ground water table also ranges from surface to 30Ft below the ground level with seasonal fluctuation and flows from north to south with a general gradient of 2.0 Ft per Mile. in drought seasons water table becomes lower but it increases during Moon Soon. More over both the Rivers along with their irrigation system in the area causes to maintain this level and also plays a pivotal roll not only to maintain it but also a reason of its purity and cleanliness.

DRAINAGE FICILITIES IN SARGODHA DISTRICT.

Most of the head works of inundation canals net work in the area were constructed / completed during late 19th century or early 20th century. The impact of regular assured supply in channels of the area( Sargodha)became visible as early as 1935, when ponds of the area started spreading water over the surface of the areas of the district.

It was after 1940, when a proper attention was given to the construction of regular Drainage system in the canal constructed areas. A scarp system was later on developed to overcome the salinity problem. The drainage system in Sargodha District of Chaj Doab was the first link of the chain which was constructed and completed just after independence.

Since then the present Sargodha Drainage Division Sargodha. Remained attach under the control of different circles during different periods. Prior to 31.05.1974, it was attached with loyal purr drainage circle, But on creation new Sargodha irrigation zone at Sargodha w.e.f. 01.06.1974.

Owing to acute Drainage problems, to establish an independent Drainage circle at Sargodha became necessitated. Hence Drainage circle Sargodha was opened at Sargodha and present Sargodha Drainage Division Sargodha was attached with the circle on 01.06.1976.

METHODOLOGY OF DRAINAGE

There are two recognized methods of drainage

    - First one is vertical drainage.

    - Second one is surface Drainage.

Sargodha Drainage Division Sargodha has been assigned the task of surface Drainage.

The Faqirian Sillanwali, L.B.N.,Jhang Abad and Mona drainage system were constructed as pilot  projects to save the home of citruses. Other Drains in Sargodha Districts were constructed by WAPDA During 1978 to 1983.

Total 12 systems were constructed in the district and are functioning well.

FLOOD PROTECTION WORKS IN DRAINAGE DIVISION SARGODHA.

Some of Major drainage systems fall directly in both of the river side’s, The River Chanab and River Jhelum detailed as under.

- F.S Outfall  Drainage system

- Raniwah Drainage system

- Sulki Drainage system

- Sobhaga Drainage system

- Terkhanawala Drainage system

- Sahiwal Drainage system

- Auttuana  Drainage system

- Shahjewana Drainage system

- Hadda Drainage system

Each outfall drains has some of its portion which lies under direct influence of flood in both the rivers. So these reaches which are effected from the direct thrust of flood water are declared as flood embankments and flood fighting arrangements are required to be maintained at site for the best interest of public and are discussed in detail as under:-

FLOOD EMBANKMENTS

- F.S OUTFALL DRAINAGE SYSTEM.

The Faqrian Sillanwali Outfall Drainage System start from RD: 0+000 of F.S Drain downstream of Sillanwali city. It outfalls in river Chenab just upstream of village kull. The flood water of river Chenab directly hits the left bank of above noted drain and destroys not only the populated areas but also effects badly the animals, gardens, green fodder and cash crops. Sometimes the flood water enters in the drain and destabilized the banks of drain or sometime over tops the banks in shape of sheet flow in the reach RD: 0+000 to RD:25+715. The Field staff will be deployed during flood season to control the situation and flood camp is also maintained at site to protect the adjacent cultivated lands, abadies and Dera jats. Even some times it affects the local earthen and mettle roads.

- RANIWAH DRAINAGE SYSTEM

The Rani wah Drainage System start from RD: 0+000 to 266+000, this system falls in to two different Sub-Divisions I,e; from RD0+000 to 176 falls in Rani wah Sub Division whereas the remaining portion from RD176+000 to RD 266+000 lies with Shahjewena Drainage Sub-Division. The drain directly falls in River Jhelum. During Flood season most of the water of river Jhelum splits up from its banks and spreads in the area and dose not only effects the cultivated lands, crops and live stocks in large scale but at the same times it also effects the population of Chhatta Bukhsha,Kirn Mazari, Denar, Doaabah and kot Issa Shah. The Field staff has always been deployed during flood season to control the situation and flood camp is also maintained at site to protect the adjacent cultivated lands, abadies and Dera jats. Even some times it affects the local earthen and matt led roads.

- SAHIWAL DRAIN.

Sahiwal drain starts from a point located near Sargodha Jhang road towards river Jhelum. The excess from Sargodha Jhang road to Sial Shareef crosses the drain at RD 21+000, Whereas the other main abadies of Havaily Majowka, Havaily Nathowka , Adda Bagga Balocha, Polah, Muhib Pur & Bora Ladial including roads, kacha tracks lies in the area which not only provide linkage between these small territories but also work as a supply chain for the inhabitants of the area. These are also only source for linkage between farm and market due to which farmers are able to have an excess to main road towards market. Children of the area also uses these tracks for schools and colleges.

On the other hand lush green fields covering an area near about 90sq km is lying in influence of high level water table.

During the era of sixties  a  net work of seepage drains was laid in the area under Chaj Doab drainage project ,launched, taken over and completed by WAPDA. Whole the activities from surveying, designing and execution were performed by the WAPDA at that time. But it is pertinent to mention here that a free board of 0.70 ft was provided at L-Section. Whereas side slopes of the mostly drains were taken as 2:1. Due to very high water table, swear seepage took place and caused of huge sloughing.

TERKHANWALA AND 1-L TERKHANWALA DRAIN

The Drain starts from the adjoining areas of adda boori at Sargodha Jhang road. The 13th RDs of the drain coming from left side of road are bank less. The water from the drain comes out in nearby field and becomes disastrous for the inhabitants of the area.  Moreover vehicle cannot move along the drain either in dry or vet season.

- SHAHJEWENA DRAIN

Shahjewena Drain starts from adjoining areas of villages Ratta, Punj Grian, dosera and Wadden.after passing nearby Shahjewena Mandi, it enters in Shahjewena city and falls out directly in River Chanab in south west of Shahjewena city.

It crosses Sargodha-jhang road at RD 8+200, and then it enters in flood range of River Chanab. River chanab has washed it's 3 RDs, which doesn't exist now.

During flood season, even at low level flood in river Chanab, back water effect reaches up to RD 8+200 and a flood protection situation has been created there.

- HADDA OUTFALL.

Hadda Outfall Drain having length 0 to 20 having discharge capacity of 1100 Cusics. It passes through the villages of Hajji Usman, Waddda Shah and Tahro Sambhal falls in River Chenab. During Flood season most of the water of river Chenab splits up from its banks and spreads in the area and dose not only effects the cultivated lands, crops and live stocks in large scale but at the same times it also effects the population of Hajji Usman, Waddda Shah and Tahro Sambhal and other adjoining areas. The Field staff has always been deployed during flood season to control the situation and flood camp is also maintained at site to protect the adjacent cultivated lands, abadies and Dera jats. Even some times it affects the local earthen and matt led roads.

1.3 Administrative Set-up

Administratively, setup plan of Drainage Division is as under.

Executive Engineer,

Drainage Division Sargodha

Sub Divisional Officer,

Shahjewana Sub Division

Sub Divisional Officer,

Raniwah Sub Division

Sub Divisional Officer,

Mona Sub Division

Sub Engineer

Sobhaga Section

Sub Engineer

Sahiwal Secton

Sub Engineer

Sillanwali Secton

Sub Engineer

Hussain Shah Section

Sub Engineer

Nurawala Section

Sub Engineer

Farooqa Section

Sub Engineer

Vij Section

Sub Engineer

Mona Section

Sub Engineer

Mari Section

Sub Engineer

Kalara Section

Sub Engineer

Hadda Section

Sub Engineer

Chawa Section

XEN DRG SGD

CHAPTER – 2

FLOOD PROTECTION AND RIVER TRAINING WORKS.

2.1 Design Parameter of Training Works.

The Sargodha Drainage Division is affected by two Rivers, the Chenab and the Jhelum Rivers.

The Chenab runs on its East while the Jhelum flows along its West. The kacha area along the sides of the two rivers is usually in the principal hit of the floods. In addition to flood water, the drains during rainy season are also prone to over flow and cause much damage to life and property, as is the case for river flood. Thus constant vigilance and supervision over Drainage systems becomes necessary during rainy season. The catchment areas located near out-fall reaches of the drain is badly affected due to backwater flows and spills during flood in the rivers.

The Division is responsible for maintenance of Drainage Systems, Flood Protection Works and River Training Works.

DESIGN PARAMETERS OF FLOOD EMBANKMENTS & RIVER TRAINING WORKS.

| Sr. No. | Description | Total length Miles | Discharge Cs | Top width (Feet) | Top R.L (Feet) |  | Side Slop |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | River Side |  | Country Side |
| 01 | Flood Embankments along F.S Outfall Drain RD 0+000 to 25+715. | 5.14 | 2200 | 25’.0" | 550.20 | 1.5:1 |  | 1:5:1 |  |
| 02 | Flood Embankments along Hadda Outfall Drain RD:0+000 to 20+000. | 4.00 | 1100 | 25’.0" | 573.43 | 2:1 |  | 2:1 |  |
| 03 | Flood Embankments of Shahjewana Drain RD 0+000 to 8+125 | 1.63 | 200 | 20'.0" | 530.90 | 2:1 |  | 2:1 |  |
| 04 | Flood Embankments of Sahiwal Drain RD 0+000 to 25+000 | 5.0 | 425 | 20’.0" | 553.18 | 2:1 |  | 2:1 |  |
| 05 | Flood Embankments of Tarkhanwala Drain RD 0+000 to 21+000. | 4.20 | 55 | 20’.0" | 542.31 | 2:1 |  | 2:1 |  |
| 06 | Flood Embankment of Raniwah drain from RD 245+000 to 266+175 | 4.24 | 450 | 20’.0" | 543.20 | 2:1 |  | 2:1 |  |
| Total Miles |  | 28.21 |  |  |  |  |  |  |  |

XEN DRG SG

DRAINAGE SYSTEMS

| Sr. No. | Name of Drainage System | Drains likely to be over spilled | Total mile length | Discharge Cs | Out falling points |
| --- | --- | --- | --- | --- | --- |
| 1 | F.S Drainage system | F.S. Drain RD 0-42 & RD 100-150 | 18.40 | 1040 | F.S. Out fall Drain |
| 2 | F.S. Outfall Drain | F.S. Outfall Drain RD 0-76 | 15.20 | 2200 | River Chenab |
| 3 | Lower Raniwah Drainage System | Lower Raniwah Drain RD 0-60 & RD 176-266 | 30 | 450 | River Jhelum |
| 4 | Mona Drainage system | Mona Drain RD 120-124 | 0.80 | 1547 | Sulki escape |
| 5 | Sulki Drainage System | Sulki Drain / RD 0-10 | 2.00 | 1554 | Sulki Creek |
|  |  | Additional Outfall Mona Drain RD 19-20 | 0.20 | 2.52 | RD 19- Sulki escape |
| 6 | Sobhaga Drainage system | Sobhaga Drain RD 0-34 | 6.80 | 760 | River Jhelum |
| 7 | Terkhanawala Drainage System | Terkhanawala Drain RD 0-34 | 6.80 | 250 | River Jhelum Creek |
| 8 | Sahiwal Drain | Sahiwal Drain RD 0-21 | 4.20 | 425 | River Jhelum |
| 9 | Hadda Drainage System | Hadda Drain RD 30-45 | 3.00 | 720 | River Chenab |

  - 2.2 LOCATION MAP.

Map has been attached for Sargodha Drainage Division Sargodha.

XEN DRG SGD

CHAPTER – 3

BRIEF HISTORY OF PAST FLOOD EVENTS

Flood causes enormous economic losses to both urban & rural communities throughout the Sargodha Drainage Division. In 1997, torrential rains and flood in the River affected the whole Drainage Network, Flood Embankments and River Training Works. Bhalwal, Shahpur, Sahiwal and Sargodha Tehsils were badly affected by floods in the two rivers and heavy rainfall during monsoon season. Bhalwal Town was cut off from east of the District by hit of flood and rain water. A large number of villages were inundated and remained in deplorable condition for over six months till depletion of the accumulated water as result of which colossal damage was caused to livestock. Houses, infrastructures and other properties. Means of communication were disrupted and roads were badly damaged.

The rain water accumulated in the area during 1997, and made its flow into drains. The drains were flooded. Four main Drainage system i.e Mona, F.S (Faquirian-Sillanwali), Hadda and Raniwah posed immense threat due to rise in water levels in drains, causing damage to systems as well as to the areas of their influence as a whole.

The outfall reaches of F.S. Mona, Raniwah, Sahiwal, Sulki and L.B.N drains were badly affected due to backwater effect.

In 1997, the river Chenab was at its peak on 28-08-1997 and due to its effect, flood embankments along L.B.N Drain (RD 0-20), Guide Wall Spur and Tie Bund remained under threat of spill from River Chenab.

However, the strenuous and zealous efforts of the department reduced the losses to the flood hit area.

In 2002, a discharge of 2.26 lac Cs was recorded downstream Qadirabad Barrage in River Chenab in 08/2002. a freshet of 3.22 lac Cs was also experienced downstream Qadirabad Barrage in River Chenab during 02/2003, whereas river Jhelum remained clam as usual. No measurable erosion/ damage was detected this year along the bunds or around the training woks.

In 2003, a maximum discharge of 1.58 lac Cs was registered down side Qadirabad Barrage.

In 2004, a maximum discharge of 90043 Cs was recorded D/S Qadirabad Barrage.

In 2005, maximum discharge recorded D/S Qadirabad Barrage on 08-07-2005, was 369847 Cs. Only 1.00 ft free board was left on Flood Embankment along L.B.N Drain and all existing bridges were submerged.

During 2006 maximum discharge was recorded D/S Quadirabad Barrage as 443206 Cs. At the same duration two number deliberate cuts were made by the inhabitants of villages situated on the country side in between RD 7000 to 8000 & 13000 to 14000 of Flood Embankment along LBN drain, which were repaired just after the passing of freshet.

In 2007 maximum discharge recorded D/S Quadirabad Barrage was 311382 & there was no mentionable damage.

In 2008 maximum discharge 31,419 Cs and 190,438 Cs were passed D/S Rasul Barrage and D/S Quadar Abad Barrage respectively and as no reportable damages were occurred there. No abnormal situation was observed during flood season 2008, as almost all drains of Sargodha Drainage Division, have flowed with very normal   discharge or remained dry.

In 2009, no significant event took place due to lesser rainfall in the country.

In 2010 maximum discharge recorded D/S Quadirabad Barrage was   3, 19, 733 Cs and D/S Rasul Barrage was 2, 63, 795 Cs during July and August 2010 due to the heavy rain fall and flood the rain water accumulated in the area and made its flow into the Drains. The drains were flooded. Five main Drainage System i.e. Mona Drainage System. F.S. Drainage System, Hadda, Raniwah and Bola Drainage System due to rise in water levels in drains caused damage to the system.

In 2011, a maximum discharge of D/S Qadirabad Barrage was 164604 Cs and D/S Rasul Barrage 96864 & there was no flood damage.

In 2012, a maximum discharge of D/S Qadirabad Barrage was 180789 Cs and D/S Rasul Barrage 27515 Cs & there was no flood damage.

In 2013, a maximum discharge of D/S Qadirabad Barrage was 403403 Cs and D/S Rasul Barrage 98449 Cs & there was no flood damage.

In 2014, a maximum discharge of D/S Qadirabad Barrage was 903504 Cs and D/S Rasul Barrage 516387 Cs during September 2014 exceptionally high flood passed in river Chenab, Guide Wall J-Head Spur at Madokana, Tie Flood Bund and LBN Flood Bund were directly under threat of Flood. The flood water rose up to top of flood bund in many reaches. Horizontal creeks were developed on the top of bund all along its length. The drain over flowed between RD 20-50, due to back water in drain.

In 2015 a maximum discharge of D/S Qadirabad Barrage was 161054 Cs and D/S Rasul Barrage 99103 Cs and there were no flood damages.

In 2016 a maximum discharge of D/S Qadirabad Barrage was 405542 Cs and D/S Rasul Barrage 43135 Cs and there were no flood damages.

In 2017 a maximum discharge of D/S Qadirabad Barrage was 154420 Cs and D/S Rasul Barrage 39230 Cs and there were no flood damages.

In 2018 a maximum discharge of D/S Qadirabad Barrage was 172031 Cs and D/S Rasul Barrage 43135 Cs and there were no flood damages.

In 2019 a maximum discharge of D/S Qadirabad Barrage was 159544 Cs and D/S Rasul Barrage 91904 Cs and there were no flood damages.

In 2020 a maximum discharge of D/S Qadirabad Barrage was 258954 Cs and D/S Rasul Barrage 126915 Cs and there were no flood damages.

In 2021 a maximum discharge of D/S Qadirabad Barrage was 167812 Cs and D/S Rasul Barrage 31420 Cs and there were no flood damages.

In 2022 a maximum discharge of D/S Qadirabad Barrage was 154420 Cs and D/S Rasul Barrage 21050 Cs and there were no flood damages.

In 2023 a maximum discharge of D/S Qadirabad Barrage was approx 185749 Cs and D/S Rasul Barrage 23090 Cs However No serious flood damages were there except sheet flow at different RD, s of Shahjewena Drain.

In 2024 a maximum discharge of D/S Qadirabad Barrage was approx 145219 Cs and D/S Rasul Barrage 66960 Cs However No serious flood damages were there except sheet flow at different RD, s of Shahjewena Drain.

XEN DRG SGD

CHAPTER - 4

DESIGN DATA, HISTORIC PEAK FLOOD DATA AND PAST FIVE YEARS   DATA OF HEAD WORKS / BARRAGES AND OTHER CONTROL POINTS.

The terminology of different kinds of floods i.e. Low, Medium, High and exceptionally high and they will be taken as such in the light of any wireless message received from respective warning centers.

4.1	FLOOD LIMITS (ABOVE) CUSECS.

| River | Control Point | Low (Lac) | Medium (Lac) | High (Lac) | Very High (Lac) | Exceptionally High (Lac) |
| --- | --- | --- | --- | --- | --- | --- |
| Jhelum | Mangla | 0.75 to 1.1 | 1.1 to 1.5 | 1.5 to 2.25 | 2.25 to 3.0 | 3.0 & above |
|  | Rasul | 0.75 to 1.1 | 1.1 to 1.5 | 1.5 to 2.25 | 2.25 to 3.0 | 3.0 & above |
| Chenab | Khanki | 1.0 to 1.5 | 1.5 to 2.0 | 2.0 to 4.0 | 4.0 to 6.0 | 6.0 & above |
|  | Qadirabad | 1.0 to 1.5 | 1.5 to 2.0 | 2.0 to 4.0 | 4.0 to 6.0 | 6.0 to 8.0 |

  - TIME LAGS OF FLOODS

As per statement the following are the time lags for average flood flows in the two Rivers.

| River | From | To | Distance (in Miles) | Time lag (Hours) |
| --- | --- | --- | --- | --- |
| Jhelum | Mangala | Rasul | 45 | 12 |
|  | Rasul | Demontomorencey Bridge (Khushab) | 80 | 22 |
| Chenab | Marala | Qaidrabad | 58 | 14 |
|  | Qadirabad | Miana Hazara | 40 | 12 |
|  |  | Madokana Spur | 70 | 20 |

  - HIGHEST FLOOD.

The following are the important highest floods discharges recorded downstream Rasul and Qadirabad during last five years.

| Year | D/S Rasul Barrage (in Cs) | D/S Qadirabad Barrage (in Cs) |
| --- | --- | --- |
| 2019 | 91532 | 159544 |
| 2020 | 126951 | 258954 |
| 2021 | 31420 | 167812 |
| 2022 | 21050 | 202149 |
| 2023 | 35325 | 185749 |
| 2024 | 66960 | 148323 |

XEN DRG SGD

4.4 PEAK DISCHARGES (HISTORICAL PEAK FLOOD DATA)

| Date | Discharge | Pond Level |
| --- | --- | --- |
| DOWN STREAM RASUL BARRAGE (RIVER JHELUM) |  |  |
| 22.07.2000 | 39222 | 718.00 |
| 27.07.2001 | 30270 | 718.00 |
| 14.08.2002 | 49170 | 718.00 |
| 05.09.2003 | 49170 | 718.00 |
| 30.04.2004 | 31422 | 719.00 |
| 17.07.2005 | 92246 | 719.00 |
| 13.07.2006 | 165947 | 720.00 |
| 07.07.2007 | 34098 | 719.00 |
| 14.11.2008 | 20678 | 719.00 |
| 17.08.2009 | 56794 | 719.00 |
| 01-02/08/2010 | 263000 | 719.00 |
| 17-09-2011 | 96864 | 719.00 |
| 04-07-2012 | 27515 | 719.00 |
| 31-07-2013 | 98449 | 719.60 |
| 06-09-2014 | 516387 | 714.00 |
| 27-7-2015 | 99103 | 607.90 |
| 27-7-2016 | 43135 | 701.30 |
| 23-7-2017 | 39230 | 719.00 |
| 02-10-2018 | 43135 | 718.00 |
| 22-06-2019 | 91532 | 705.40 |
| 28-8-2020 | 126951 | 717.00 |
| 30-06-2021 | 31420 | 700.30 |
| 22-06-2022 | 23210 | 590.50 |
| 08.09.2023 | 35325 | 699.30 P |
| 30.08.2024 | 66960 | 719.0 |
| DOWN STREAM QADIRABAD BARRAGE (RIVER CHENAB) |  |  |
| 23.07.2000 | 291349 | 693.70 |
| 08.08.2001 | 136575 | 697.70 |
| 14.08.2002 | 226398 | 694.60 |
| 19.02.2003 | 367408 | 696.20 |
| 18.08.2004 | 90043 | 691.50 |
| 06.07.2005 | 369847 | 697.00 |
| 04.09.2006 | 443206 | 699.10 |
| 14.03.2007 | 289002 | 697.00 |
| 01.08.2008 | 190438 | 697.00 |
| 17.08.2009 | 55535 | 697.00 |
| 07.08.2010 | 298961 | 697.00 |
| 17-09-2011 | 164604 | 691.85 |
| 5-7-2012 | 180789 | 691.97 |
| 15-8-2013 | 403403 | 697.30 |
| 07-09-2014 | 903504 | 701.70 |
| 13-7-2015 | 161054 | 694.50 |
| 08-08-2016 | 405542 | 696.95 |
| 03-08-2017 | 154420 | 694.00 |
| 14.08.2018 | 172031 | 697.00 |
| 01-08-2019 | 159544 | 684.70 |
| 29.08.2020 | 258954 | 694.00 |
| 29-07-2021 | 167812 | 688.25 |
| 12.08.2022 | 202149 | 694.00 |
| 20.07.2023 | 185749 | 692.70 |
| 16.08.2024 | 145219 | 692.0 |

XEN DRG SGD

CHAPTER – 5

FLOOD FIGHTING STRATEGY

In view of the experience of the past flood history, constant watch is required to meet the on slaughts of floods in rainy season.

Constant vigilance and caution will be required at all levels particularly on spots for relief and flood fighting on Drains, Flood Embankments and River Training works.

BATTLE STATIONS

The department proposes to install the following battle stations for protection of irrigation structures, flood fighting and watching to minimize the losses.

THE PROPOSED CAMPS SITES ON DRAINS.

(Low Flood)

| Drains | Reach | Length in Mils | Battle Station |
| --- | --- | --- | --- |
| F.S. Outfall | 0 to 25+715 | 5.14 | RD- 11 |
| Sahiwal Drain | 0 to 21+000 | 4.20 | RD- 15 |
| Tarkhanwala Drain | 0 to 21 | 4.20 | RD-12 |
| Shahjewana Drain | 0-8 | 1.625 | RD-8 |
| Hadda outfall Drain | 0 to 20 | 4.00 | RD- 18 |
| Raniwah Drain | 245 to 266 | 4.20 | RD - 248 |

(Medium Flood)

| Drains | Reach | Length in Mils | Battle Station |
| --- | --- | --- | --- |
| F.S. Outfall | 0 to 25+715 | 5.14 | RD- 11 |
| Sahiwal Drain | 0 to 21+000 | 4.20 | RD- 15 |
| Tarkhanwala Drain | 0 to 21 | 4.20 | RD-12 |
| Shahjewana Drain | 0-8 | 1.625 | RD-8 |
| Hadda outfall Drain | 0 to 20 | 4.00 | RD- 18 |
| Raniwah Drain | 245 to 266 | 4.20 | RD - 248 |

XEN DRG SGD

THE PROPOSED CAMPS SITES ON DRAINS.

(High Flood)

| Drains | Reach | Length in Mils | Battle Station |
| --- | --- | --- | --- |
| F.S. Outfall | 0 to 25+715 | 5.14 | RD- 11 |
| Sahiwal Drain | 0 to 21+000 | 4.20 | RD- 15 |
| Tarkhanwala Drain | 0 to 21 | 4.20 | RD-12 |
| Shahjewana Drain | 0-8 | 1.625 | RD-8 |
| Hadda outfall Drain | 0 to 20 | 4.00 | RD- 18 |
| Raniwah Drain | 245 to 266 | 4.20 | RD - 248 |

THE PROPOSED CAMPS SITES ON DRAINS.

(Exceptionally High Flood)

| Drains | Reach | Length in Mils | Battle Station |
| --- | --- | --- | --- |
| F.S. Outfall | 0 to 25+715 | 5.14 | RD- 11 |
| Sahiwal Drain | 0 to 21+000 | 4.20 | RD- 15 |
| Tarkhanwala Drain | 0 to 21 | 4.20 | RD-12 |
| Shahjewana Drain | 0-8 | 1.625 | RD-8 |
| Hadda outfall Drain | 0 to 20 | 4.00 | RD- 18 |
| Raniwah Drain | 245 to 266 | 4.20 | RD - 248 |

The above mentioned camps will be installed on above sites for controlling the protection activities during emergency. For all spots, sufficient quantity of flood fighting material with manpower will be made available in all these camps.

Engagement of work charge establishment to supplement the efforts through regular establishment will be made in accordance with the demand of the situation.

At the stage of very high flood, the strength of watching establishment will be enhanced. Executive Engineer, will be authorized to engage labour and purchase material up to any requirement in consultation with the Superintending Engineer.

The following Sub Divisional Officers will supervise all the operations in their Respective Sub Divisions.

| Designation of Officer | Sub Division |
| --- | --- |
| Sub Divisional Officer | Shahjewana Drainage Sub Division |
| Sub Divisional Officer | Mona Drainage Sub Division |
| Sub Divisional Officer | Raniwah Drainage Sub Division |

XEN DRG SGD

The Executive Engineer is fully competent to assign and change duty of the Officer/ Officials working under him.

A flood emergency Sub Office will be established within the Divisional Office, which will become operative with effect from 15-06-2024, and remain functional through the flood season till 15-10-2024. A duty clerk along with a messenger would be available and deliver messages about flood warning emergency and any eventuality, which may arise.

- LOW FLOOD DISCHARGE 100000 – 150000 Cs.

No extra labour will be employed. The labour arranged for preparedness will continue as such.

- MEDIUM FLOOD DISCHARGE 150000 – 200000 Cs

The flood water flows within river bank. No extra labour will be employed. The labour arranged for preparedness will continue as such.

- HIGH FLOOD DISCHARGES 200000 – 400000 Cs.

The flood water spills out and inundates the low lying areas and touches the flood bund. The watching is necessary to be arranged. Therefore, four men for day & night for two mile are employed, including the existing gang, for watching purposes.

iv         VERY HIGH FLOOD DISCHARGES 400000 – 600000 Cs.

All activities will remain as such. The watching establishment will be raised to two men per mile on Drains and three men per mile per shift for three shifts on Flood Bund.

- EXCEPTIONALLY HIGH FLOOD ABOVE 600000 Cs.

This stage warrants alarming situation. The watching labour will be raised to three men per mile per shift on Drains and five men per mile per shift for three shifts on bund. On receipt of exceptionally high flood warning, Sub Divisional Officer, Shahjewena along with concerned Sub Engineer will shift their camp at Suitable location which will be intimated to all concerned at once.

XEN DRG SGD

CHAPTER – 6

| Flood Damages Restoration works for the year 2024-25 |  |  |
| --- | --- | --- |
|  | No Flood damages occurred during the flood season 2024-25 in Sargodha Drainage Division Sargodha. |  |
|  | XEN DRG SGD |  |

CHAPTER – 7

FLOOD FIGHTING WATCHING ARRANGEMENTS

  - Pre-Flood Arrangements.

| Sr.# | Activity | Target Dates | Compliance Report |
| --- | --- | --- | --- |
| I. | Preparation of Flood Fighting Plans |  |  |
|  | a) Submission of Draft Divisional Plans by Zonal Chief to Chief Engineer Drainage And Flood (After scrutiny by SE/CE). | 01-03-2025 | within target date |
|  | b) Review by Chief Engineer D&F. | 31-03-2025 | Reviewed |
|  | c) Final submission of Flood Fighting Plan. | 15-04-2025 | within target date |
| II. | Inspection of Flood Works. |  |  |
|  | a) By XEN's |  | - |
|  | b) Checking / Inspection by SE's/CE) |  | - |
|  | c) Inspection by Departmental & Army Team. |  | - |
| III. | Identification of encroachment’s on bunds and spurs to the Civil Administration. | 31-03-2025 | No encroachment on flood bund / spur |
| IV. | Painting Gauges, RD Marks, Data Boards and repair to Flood Bunds etc. | 31-05-2025 | within target date |
| V. | Replenishment of Reserve Stock of Stone and Flood fighting materials. |  |  |
|  | a) Identification of needs. | 28-02-2025 | - |
|  | b) Funding/Procurement Arrangements. | 15-03-2025 | Sanction of estimate is under progress. |
|  | c) Procurement, Stacking, Prescribed field Checks. | 31-05-2025 | It will be completed within target date |
| VI. | Installation of wireless sets | 15-06-2025 | It will be completed within target date |
| VII. | Opening of Zonal Flood Cells |  |  |
|  | a) Duty Rosters. | 01-06-2025 | It will be opened on target date |
|  | b) Operative. | 15-06-2025 | It will be operated on target date |

XEN DRG SGD

  - WATCHING ESTABLISHMENT

Following are the main spots where watching Huts will be established on temporary basis. During medium floods for embankments and high floods for adjoining abadies, the tents/shouldaries will be erected and Flood .watching material will be shifted in the huts. One Chowkidar for each Hut will be employed from the report of receipt of medium Flood and will remain employed till end of flood season. The scale / engagement of work charge labour on flood bunds and Drains will be required on work charge basis.

In addition to the existing field establishment, extra manpower will be engaged during the flood season on work charge basis for watching and general maintenance of banks of drains, flood embankments, and River Training works in whole of the Division. The engagement of labour would however will be subject to the intensity of rains / flood as the situation could warrant.

| Flood Limits |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site | Low |  | Medium |  | High |  | Very High |  | Exceptionally High |  |
|  | Men | Shift | Men | Shift | Men | Shift | Men | Shift | Men | Shift |
| F.S out fall Drain 0-25 | No extra labour will be required at the stage. |  | 5 | 02 | 10 | 03 | 15 | 03 | 15 | 03 |
| Sahiwal Drain RD 0-21 |  |  | 5 | 02 | 10 | 03 | 15 | 03 | 15 | 03 |
| L.R Drain. RD 245-266 |  |  | 5 | 02 | 10 | 03 | 30 | 03 | 15 | 03 |
| Terkhan wala DrainRD 0-21 |  |  | 5 | 02 | 10 | 03 | 06 | 03 | 15 | 03 |
| Shahjewana Drain RD 0-8 |  |  | 3 | 02 | 06 | 03 | 06 | 03 | 10 | 03 |
| Hadda Out Fall Drain RD 0-20 |  |  | 5 | 02 | 10 | 03 | 12 | 03 | 15 | 03 |

XEN DRG SGD

7.3	Arrangement at Sensitive Sites.

The total 116 RDs Flood effecting banks exist in Sargodha Drainage Division, Sargodha for which watching lengths and location of site cam/ flood fighting camps have been mentioned on page 15 above in tabular form.

The above mentioned camps will be installed on above sites for controlling the protection activities during emergency. For all spots, sufficient quantity of flood fighting material with manpower will be made available in all these camps.  Engagement of work charge establishment to supplement the efforts through regular establishment will be made in accordance with the demand of the situation. At the stage of very high flood, the strength of watching establishment may be enhanced. Executive Engineer, will be authorized to engage labour and purchase material upto any requirement in consultation with the Superintending Engineer.

7.4	WATCHING MATERIAL

There will be two No. watching centers on flood embankment along all Drains at mentioned RDs or where suitable as per site situation which will be shared to all concerned at once in case of any change. Flood material will be arranged and stored at Shahjewena Flood fighting Hut. The material will be shifted to flood fighting camp as soon as the intimation of medium / high flood limit is received.

| Sr # | Items | Unit | Total Required Qty | Available Qty | Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Plastic Bages | No | 1500 | 0 | 1500 | Material will be purchased before flood season Subject to verification by Competent Authority |
| 02 | Chouldari | No | 6 | 0 | 6 |  |
| 03 | Kassies | No | 100 | 0 | 100 |  |
| 04 | Wheel barrow | No | 12 | 0 | 12 |  |
| 05 | Bamboos/ Killas | No | 1500 | 0 | 1500 |  |
| 06 | Torche | No | 24 | 0 | 24 |  |
| 07 | Cell of torches (Dozen) | Dozen | 10 | 0 | 10 |  |
| 08 | Folding Chairs | No | 36 | 0 | 36 |  |
| 09 | Folding Table | No | 06 | 0 | 06 |  |
| 10 | 18 S.W.G. | Kg | 100 | 0 | 100 |  |
| 11 | G.I.Buckets | No | 10 | 0 | 10 |  |
| 12 | Hand Saw | No | 06 | 0 | 06 |  |
| 13 | Killas | No | 100 | 0 | 100 |  |
| 14 | Life Jecket | No | 30 | 0 | 30 |  |
| 15 | M.Oil | Ltr | 25 | 0 | 25 |  |
| 16 | Manale Rope | No | 10 | 0 | 10 |  |
| 17 | Axes | No | 05 | 0 | 05 |  |
| 18 | Petrol | Ltr | 300 | 0 | 300 |  |
| 19 | Umbrella | No | 12 | 0 | 12 |  |
| 20 | Wooden Hammer | No | 08 | 0 | 08 |  |
| 21 | Steel Hammer | No | 08 | 0 | 08 |  |
| 22 | Polythene Sheet of Black Color | Sq ft | 450 | 0 | 450 |  |

7.5 	Arrangement for Sounding & Probing.

This item relates to barrage / Head works Canal Divisions.

7.6 	Lighting Arrangement.

Sufficient numbers of emergencies lights, lanterns and have been provisioned.

7.7 	Ration Arrangement.

The establishment and supervisory staff will make their own food arrangements. However in the event of dearth of commodities, the Civil Authorities will be approached to store sufficient ration.

7.8 	P.O.L. Arrangement for Vehicle.

Respective Deputy District Officer (Revenue) will be requested by the Sub Divisional Officer in-charge to provide help for reserve at the nearest petrol pumps to keep the vehicles on. Any obstruction found there will be pointed out by the Sub Divisional Officer.

7.9 	Transportation.

Vehicles will available for transportation during flood season.

7.10 	Law and order.

There is no likelihood of any interruption by anyone except cutting of banks of F.S out fall drain between RD 0-11 near Ahmad Walla village. In case of necessity District Coordination Officer Chiniot will be contacted by the Executive Engineer, Drainage Division Sargodha to take decision at site.

7.11 	Medical Arrangement for Labour.

It will be responsibility of District Government to provide Medical Facilities in flood areas where will required.

7.12 	Liaison with other Department.

Executive Engineer, Sargodha Drainage  Division Sargodha will keep liaison with the Civil and Army Authorities and will keep the Superintending Engineer Drainage Circle Sargodha fully apprised of the flood situation in Sargodha Drainage Division.

7.13 	Role of the Army.

Role of the Army is very important in case of any emergent situation arising due to high floods. A continuous liaison with the Army Authorities will be maintained during floods and if required necessary help will be sought.

7.14 	Duties of Telephone Attendant.

| Sr # | Name of Office / Officials | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 01 | Sub Divisional Officer Shahjewana Drainage Sub Division (M Zaheer Ahmad) | 15.06.2025 to 15.10.2025 | 8:00 AM | 4:00 Pm |
| 02 | Sub Engineer Sillanwali Section (Muhammad Saeed). |  | 4:00 PM | 12:00 AM |
| 03 | Sargodha Drainage Division Sargodha (M Saeed ) SDC |  | 12:00 AM | 8:00 AM |
| 04 | Telegraph Office Sargodha. |  | 8:00 AM | 8:00 AM |

  - Wireless Arrangements.

Wireless system will be installed w.e.f 01.06.2025 in the vehicle of         XEN Drainage Sargodha No. GBA 526, SDO Shahjewana Drainage Sub Division Jeep No. MBB 4383, SDO Raniwah Sub- Division Jeep No 5335.

XEN DRG SGD

CHAPTER – 8

DETAIL OF ENCROACHMENT

DETAILS OF ENCROACHMENTS ON FLOOD WORKS

Presently there is no encroachment exists on any flood works of Sargodha Drainage Division, Sargodha.

- Detail of Encroachment.

| Sr #. | Total Encroachment | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| ------NIL------ |  |  |  |  |

- Detail of Critical Encroachment.

| Sr #. | Total Encroachment | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| ------NIL------ |  |  |  |  |

XEN DRG SGD

CHAPTER – 9

1.	DUTY ROSTER / FLOOD FIGHTING PROGRAMME

| Key personal | Battle Station | Drains | Sub Division |
| --- | --- | --- | --- |
| Sub Engineer, Hadda | RD- 11 | F.S Outfall | Shahjewana Sub Division |
| Sub Engineer, Sahiwal | RD- 15 | Sahiwal Drain | Shahjewana Sub Division |
| Sub Engineer, Farooka | RD-12 | Terkhanwala Drain | Shahjewana Sub Division |
| Sub Engineer, Sobhaga | RD-8 | Shah Jewana | Shahjewana Sub Division |
| Sub Engineer, Sillanwali | RD- 18 | Hadda Outfall | Shahjewana Sub Division |
| Sub Engineer, Noorwala | RD - 248 | Raniwah Drain | Shahjewana Sub Division |

XEN DRG SGD

# Duty Roster/Flood Fighting Program

| DUTY ROSTER OF KEY PERSONAL DURING FLOOD SEASON 2025 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SITE | LENGTH In Miles | STAGE-I DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-II DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-III DISCHARGE | LABOUR REQUIRED | ESTABLISHMENT TO BE DEPUTED (PER SHIEFT) |  |  |  | STAGE-IV DISCHARGE EXCEEDING 8 LAC Cs. |
|  |  |  |  | MATE | BELDAR |  |  | MATE | BELDAR |  |  | SDO | SUB ENGINEER | MATE | BAILDAR |  |
| F.S out fall Drain 0-25 | 5 | 2.0 lac Cs to 4.0 lac Cs. | Four men for day & night for two mile | 1 | 5 | 4.0 lac Cs to 6.0 lac Cs. | Three men per mile per shift for Three shifts on Flood Bund. | 1 | 10 | 6.0 lac Cs. To 8.0 lac Cs. | One men per mile per shift on Embankment. On receipt of exceptionally high flood warning, Sub Divisional Officer, Shahjewena alongwith concerned Sub Engineer will shift their camp. | Zaheer Ahad Sub Divisional Officer 0306-0561786 | Muhammad Saeed Sub Engineer, 0347-2006000 | 2 | 15 | Establishment will be deputed as per stage-III |
| Sahiwal Drain RD 0-21 | 4.20 |  |  | 1 | 5 |  |  | 1 | 10 |  |  |  | Muhammad Rahim Sub Engineer, 0306-5201213 | 2 | 15 |  |
| LR Drain RD 245-266 | 4.2 |  |  | 1 | 5 |  |  | 1 | 10 |  |  |  | Ghulam Raza, 0302-8146505 | 2 | 15 |  |

XEN DRG SGD

# Duty Roster/Flood Fighting Program

| DUTY ROSTER OF KEY PERSONAL DURING FLOOD SEASON 2025 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SITE | LENGTH In Miles | STAGE-I DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-II DISCHARGE | LABOUR REQUIRED | LABOUR FOR WATCHING BE DEPUTED (PER SHIFT) |  | STAGE-III DISCHARGE | LABOUR REQUIRED | ESTABLISHMENT TO BE DEPUTED (PER SHIEFT) |  |  |  | STAGE-IV DISCHARGE EXCEEDING 8 LAC Cs. |
|  |  |  |  | MATE | BELDAR |  |  | MATE | BELDAR |  |  | SDO | SUB ENGINEER | MATE | BAILDAR |  |
| Terkhanwala Drain BankRD 0-21 | 4.80 | 2.0 lac Cs to 4.0 lac Cs. | Four men for day & night for two mile | 1 | 5 | 4.0 lac Cs to 6.0 lac Cs. | Three men per mile per shift for Three shifts on Flood Bund. | 1 | 10 | 6.0 lac Cs. To 8.0 lac Cs. | One men per mile per shift on Embankment. On receipt of exceptionally high flood warning, Sub Divisional Officer, Shahjewena alongwith concerned Sub Engineer will shift their camp.. | Zaheer Ahad Sub Divisional Officer 0306-0561786 | Khawar Azam, 0307-4948043 | 2 | 15 | Establishment will be deputed as per stage-III |
| Shahjewena Drain RD 0- 8+125 | 1.62 |  |  | 1 | 3 |  |  | 1 | 6 |  |  |  | Javed Iqbal Sub Engineer, | 1 | 10 |  |
| Hadda Out Fall Drain RD 0-18 | 3.25 |  |  | 1 | 5 |  |  | 1 | 5 |  |  |  | Muhammad Saeed, 0347-2006000 | 1 | 15 |  |

XEN DRG SGD

CHAPTER – 10

Emergency Telephone Nos.

| Sr. No. | Designation | Telephone / Cell. No. |  |
| --- | --- | --- | --- |
|  |  | Office | Residence |
| 1 | Minister of Irrigation Punjab Lahore. | 042-99212809 042-99211761 |  |
|  |  | Fax 042-99212810 | - |
| 2 | Secretary Irrigation Department, Punjab Lahore. | 042-99212117 | 042-99200286 |
|  |  | 042-99212118 |  |
|  |  | Fax 042-99212116 |  |
| 3 | Chief Engineer, Irrigation Sargodha Zone. | 048-9230445 | 03336941690 |
|  |  | Fax 048-9230414 | 048-9230451 |
| 4 | Flood Emergency Officer Sargodha Irrigation Zone Sargodha. | 048-9230446 | 03006363739 |
|  |  | Fax 048-9230414 |  |
| 5 | Chief Engineer Drainage and Flood Zone Lahore. | 042-99233551 | 0300-6999490 |
|  |  | Fax 042-35291304 |  |
| 6 | Director, Flood Drainage and Flood Zone Lahore. | 042-99231614 Fax-042-35862081 | 0323-4864373 |
| 7 | Deputy Director, Drainage and Flood Lahore. | 042-99230282 | 0300+6362154 |
| 8 | Flood Warning Center Jail Road Lahore. | 042-99230282 | - |
| 9 | Superintending Engineer Lower Jhelum Canal Circle Sargodha. | 048-9230413 | 048-9230463 |
|  |  | Fax 048-9230832 | 0333-6752753 |
| 10 | Superintending Engineer Drainage Circle Sargodha. | 048-9230450 |  |
| 11 | Executive Engineer, Sargodha Division L.J.C, Sargodha. | 048-9230409 | 03000147499 |
|  |  | Fax 048-9230918 |  |
| 12 | Executive Engineer, Kirana Division L.J.C, Sargodha. | 048-9230399 | 0302-4844342 |
|  |  | Fax 048-9230649 |  |
| 13 | Executive Engineer, Shahpur Division L.J.C, Sargodha. | 048-9230447 | 0344-4132393 |
|  |  | Fax 048-9230836 |  |
| 14 | Executive Engineer, Bhalwal Division L.J.C, | 0489230406 | 03006363739 |
| 15 | District Coordination Officer Sargodha. | 048-9230025 | 0300-8199708 |
|  |  | 048-9230026 |  |
| 16 | District Coordination Officer Chiniot. | 0476-330071-2 | - |
| 17 | District Coordination Officer Jhang | 047-9200081 | - |
| 18 | Executive Engineer, Drainage Division Sargodha. | 048-9230453 | 0300-8199708 |
| 19 | Incharge Vigilance Cell Irrigation & Power Department Punjab Lahore. | 042-9213157 | - |
| 20 | Telegraph Office Sargodha (Flood Emergency Center Sargodha). | 048-9230408 | - |
| 21 | Sub Divisional Officer, Raniwah Drainage Sub Division. | 048-9230453 | 0306-0561786 |
| 22 | Sub Divisional Officer, Mona Drainage Sub Division. | 048-9230453 | 0331-8178563 |
| 23 | Sub Divisional Officer, Shahjewana Drainage Sub Division. | 048-9230453 | 0306-0561786 |

XEN DRG SGD

CHAPTER – 11

Standard operating procedure (SOP) for Breaching Sections

11.1 	History of the breaching section.

No breaching section in Sargodha Drainage Division Sargodha.

11.2 	Location, Design, Quantity and Variety of the explosive required for detonation.

Does not relate.

11.3 	Arrangement of explosives and security of explosive stores.

Does not relate.

11.4 	List of the security staff along with detail of their training etc

Does not relate.

11.5 	Detail of mechanical means as a standby arrangements in case of detonation failure.

Does not relate.

11.6 	Duty Roster in case of critical situation.

Does not relate.

11.7 	Breaching Committee with their action plan.

Does not relate.

11.8 	List of the villages likely to be inundated in case of breach.

Does not relate.

11.9 	Announcement and detail of evacuation arrangements.

Does not relate.

11.10 Details of coordination with Civil/Army Authorities

Does not relate.

11.11 Parallel communication arrangements.

Does not relate.

11.12 Index Plan.

Does not relate.

Sub Divisional Officer,

Shahjewana Drainage Sub Division,

Sargodha.

Executive Engineer,

Sargodha Drainage Division,

Sargodha.

CHAPTER - 12

Vulnerable sites on flood bunds/Structures.

12.1 	Apprehended breach sites in Flood Bunds/Structures

The vulnerable sites on flood protection works are under:-

| Sr. No. | Name of Flood Bund | Reach RD |  | No. of Sites |
| --- | --- | --- | --- | --- |
|  |  | From | To |  |
| 1 | F.S. Outfall | 0+000 | 25+750 | 1 |
| 2 | Sahiwal Drain | 0+000 | 21+000 | 1 |
| 3 | Tarkhanwala Drain | 0+000 | 21+000 | 1 |
| 4 | Shahjewana Drain | 0+000 | 8+125 | 1 |
| 5 | Hadda outfall Drain | 0+000 | 21+000 | 1 |
| 6 | Raniwah Drain | 245+000 | 266+000 | 1 |

12.2	Operation on breaching section

There is no breaching section in Drainage Division Sargodha.

12.3	Breaches due to rising of flood water deterioration of flood bund etc.

No such case.

XEN DRG SGD

CHAPTER – 13

Emergency Contingency Plan for vulnerable sites listed below.

13.1 Detail of villages abadies likely to be affected mentioned as under (to be shown on the plan).

| S.NO | NAME OF DRAIN | RD | VILLAGES |
| --- | --- | --- | --- |
| 1 | F.S Drainage system | RD 0-42 & RD 100-150 | Sargodha City, Shaheeb abad and Sillanwali |
| 2 | Sahiwal Drain | RD 0-21 | Sayal sharif, Havaily Nathoka,Havaily Majoka,Poloha , Boora Ladheyal, Meta Salika etc |
| 3 | Lower Raniwah Drainage System | RD 0-60 & RD 176-266 | Lau khokhran, Kiran Mazari, Chatta Bakhsaha, Doaba etc |
| 4 | Terkhanawala Drainage System | RD 0-34 | Nadda GHAR, Sheikh jalil, Chak qureshi, Sava Majoka, Sargana etc |
| 5 | Hadda Drainage System | RD 30-45 | Taroo sahmbel, Haji usman, Colony etc |
| 6 | Shahjewena Drain | RD 0+000 TO 8+125 | Shahjewena City, Sher Ghar etc |
| 7 | Sobhaga Drainage system | RD 0-34 | Khan Da kot, Haveli Dewana, Kulcha, Thatti Sayden Barhama and Pabarwala |
| 8 | F.S. Outfall Drain | RD 0-76 | Kul, Harianwala, Pipl Bhutha and Ahmad e wala |

XEN DRG SGD

XEN DRG SGD

FLOOD FIGHTING CAMP SITES.

| Drains | Reach | Length in Mils | Battle Station |
| --- | --- | --- | --- |
| F.S. Outfall Drain | 0-25 | 5 | RD-11 |
| Terkhan wala Drain | 0-21 | 4.20 | RD 12 |
| Sahiwal Drain | 0-21 | 4.20 | RD- 15 |
| L.R Drain | 245-266 | 4.20 | RD- 248 |
| Shahjewana Drain | 0-8 | 1.625 | RD-8 |
| Hadda out fall Drain | 0-20 | 4.00 | RD -18 |

XEN DRG SG

    - Arrangements.

The incharge of Sub Sector, Sub Divisional Officer, Shahjewana Drainage Sub Division Sargodha & Executive Engineer Drainage Division Sargodha taking the cognizance of situation will make liaison with the civil administration and police department and also arrange.

- Proclamation on loudspeakers etc for evacuation of abadies inside the route of flood water.

- Flood hut has been proposed to be established at RD 8+000 of   Shahjewena Drain high level to monitor the emergent flood situation. If any obstruction found coming out in the way of flood water, it would be removed by deploying establishment and machines such like tractors excavator machine and bulldozers as per situation prevailing at site. Efforts would be made that flood water may not stand in crops as well as abadies, to minimize the loss/ damages and ultimately fall again the same way in river Chenab and River Jhelum from the shortest possible time.

- Keep a close liaison with Civil Administration Public Representatives.  Army authorities. List of phone numbers of above authorities have been mentioned in this flood fighting plan (chapter No. 10)

    - Establishment of Flood Fighting Camps.

(Drains)

| Drains | Reach | Length in Mils | Battle Station |
| --- | --- | --- | --- |
| F.S. Outfall | 0+000 to 25+750 | 5.14 | RD-11 |
| Sahiwal Drain | 0+000 to 21+000 | 4.20 | RD 12 |
| Tarkhanwala Drain | 0+000 to 21+000 | 4.20 | RD- 15 |
| Shahjewana Drain | 0+000 to 8+125 | 1.615 | RD- 248 |
| Hadda outfall Drain | 0+000 to 21+000 | 4.80 | RD-8 |
| Raniwah Drain | 245+000 to 266+000 | 4.20 | RD -18 |

CAMP SITES ON FLOOD EMBANKMENT AND RIVER TRAINING WORKS.

As mentioned above.

XEN DRG SGD

13.3.3 Duties of Officers/Officials and their camp sites.

| Camp No. | Location | Officer’s Name , Designation & Cell # | Official’s Name, Designation & Cell # |
| --- | --- | --- | --- |
| 01 | Drainage Sub-Divisions Shahjewena & Raniwah | M Zaheer Ahmad, Sub Divisional Officer 0306-0561786 | 1.Muhammad Saeed Sub Engineer,Sillanwali & Hadda Section 0347-2006000 2.Mohammad Raheem Sub-Engineer Sahiwal Section 030665201213 3. Mohammad Javed Sub-Engineer Sobagha Section 03023167113 4. Khawar Azam Sub Engineer. Farooka Shah Section 0341-8780569 5. Arif Abbas Jafri Sub Engineer, Vijh Section, 0301-8657514 6. Muhammad Usman Sub Engineer, Hussain Shah Section 0303-5046431 7. Gulam Raza Sub Engineer, Norewala Section 0302-8146305. |

XEN DRG SGD

13.3.4 Departmental Machinery available.

There is no any kind of machinery available in the control of Sargodha Drainage Division Sargodha.

13.3.5 Machinery available from Private Source.

If machinery is needed, it will be arranged / hired from private source. (The machinery can be hired in case of emergency).

13.3.6 Flood Fighting Material Required.

13.3.7 Flood Fighting Material Available.

No any kind of flood fighting material has already been available at site or in store.

Strategy and action taken be explained in detail.

As explained at Sr. No. 13.4, there are no vital installations in this area like oil refineries, power houses and other important infrastructure

    - Arrangements.

The incharge of Sub Sector, Sub Divisional Officer, Shahjewana Drainage Sub Division Sargodha,Mona and Raniwah & Executive Engineer Drainage Division Sargodha taking the cognizance of situation will make liaison with the civil administration and police department and also arrange.

- Proclamation on loudspeakers etc for evacuation of abadies inside the route of flood water.

- Flood hut has been proposed to be established at RD 8+000 of Shahjewena Drain  at high level to monitor the emergent flood situation. If any obstruction found coming out in the way of flood water, it would be removed by deploying establishment and machines such like tractors excavator machine and bulldozers as per situation prevailing at site. Efforts would be made that flood water may not stand in crops as well as abadies, to minimize the loss/ damages and ultimately fall again the same way in river Chenab from the shortest possible time.

- Keep a close liaison with Civil Administration Public Representatives.  Army authorities. List of phone numbers of above authorities have been mentioned in this flood fighting plan.

XEN DRG SGD

    - Establishment of Flood Fighting Camps.

Drains

| Drains | Reach | Length in Mils | Battle Station |
| --- | --- | --- | --- |
| F.S. Outfall | 0+000 to 25+750 | 5.14 | RD-11 |
| Sahiwal Drain | 0+000 to 21+000 | 4.20 | RD 12 |
| Tarkhanwala Drain | 0+000 to 21+000 | 4.20 | RD- 15 |
| Shahjewana Drain | 0+000 to 8+125 | 1.615 | RD- 248 |
| Hadda outfall Drain | 0+000 to 21+000 | 4.80 | RD-8 |
| Raniwah Drain | 245+000 to 266+000 | 4.20 | RD -18 |

13.3.3 Duties of Officers/Officials and their camp sites.

| Camp No. | Location | Officer’s Name , Designation & Cell # | Official’s Name, Designation & Cell # |
| --- | --- | --- | --- |
| 01 | F.S. Outfall RD: 11+000 | M Zaheer Ahmad, Sub Divisional Officer 03060561786 | Muhammad Saeed Sub Engineer, 0347-2006000 |
| 02 | L.R Drain RD: 248+000 | M Zaheer Ahmad, Sub Divisional Officer 03008876005 | Ghulam Raza Sub Engineer 0302-8146305 |
| 05 | Sahiwal Drain RD: 15+000 | M Zaheer Ahmad, Sub Divisional Officer 03060561786 | Muhammad Raheem Sub Engineer 0306-5201213 |
| 06 | Terkhanwala Drain RD: 12+000 | M Zaheer Ahmad, Sub Divisional Officer 03060561786 | Khawar Azam Sub Engineer 0307-4939043 |
| 07 | Shahjewena Drain RD: 8+000 | M Zaheer Ahmad, Sub Divisional Officer 03060561786 | Muhammad Javed Sub Engineer 03023167113 |

13.3.4 Departmental Machinery available.

There is one no. Tractor with front lodar & Trolly available in the control of Sargodha Drainage Division Sargodha.

13.3.5 Machinery available from Private Source.

If machinery is needed, it will be arranged / hired from private source. (The machinery can be hired in case of emergency).

XEN DRG SGD

13.3.6 Flood Fighting Material Required.

| Sr # | Items | Unit | Total Required Qty | Available Qty | Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Plastic Bages | No | 1500 | 0 | 1500 | Material will be purchased before flood season |
| 02 | Chouldari | No | 6 | 0 | 6 |  |
| 03 | Kassies | No | 100 | 0 | 100 |  |
| 04 | Wheel barrow | No | 12 | 0 | 12 |  |
| 05 | Bamboos/ Killas | No | 1500 | 0 | 1500 |  |
| 06 | Torche | No | 24 | 0 | 24 |  |
| 07 | Cell of torches (Dozen) | Dozen | 10 | 0 | 10 |  |
| 08 | Folding Chairs | No | 36 | 0 | 36 |  |
| 09 | Folding Table | No | 06 | 0 | 06 |  |
| 10 | G.I.wire | Kg | 100 | 0 | 100 |  |
| 11 | G.I.Buckets | No | 10 | 0 | 10 |  |
| 12 | Hand Saw | No | 06 | 0 | 06 |  |
| 13 | Killas | No | 100 | 0 | 100 |  |
| 14 | Life Jecket | No | 30 | 0 | 30 |  |
| 15 | M.Oil | Ltr | 25 | 0 | 25 |  |
| 16 | Manale Rope | No | 10 | 0 | 10 |  |
| 17 | Axes | No | 05 | 0 | 05 |  |
| 18 | Petrol | Ltr | 300 | 0 | 300 |  |
| 19 | Umbrella | No | 12 | 0 | 12 |  |
| 20 | Wooden Hammer | No | 08 | 0 | 08 |  |
| 21 | Steel Hammer | No | 08 | 0 | 08 |  |
| 22 | Polythene Sheet of Black Color | Sq ft | 450 | 0 | 450 |  |

13.3.7 Flood Fighting Material Available.

No any kind of flood fighting material has already been available at site or in store.

XEN DRG SGD

  - Detail of other infrastructure like electric, Sui gas, Telephone Installations, Road Network, other buildings, Canals and Drainage network.

The details of infrastructure which may be affected.

- Pacca Roads from Sargodha to jhang

- Electric installation of villages.

There is no such huge infrastructure like Grid station, Sui Gas station exists in this area. It is expected that minor damages may occur.

|  | Sub Divisional Officer, Shahjewana Drainage Sub Division, Sargodha |  | Executive Engineer, Sargodha Drainage Division, Sargodha |  |
| --- | --- | --- | --- | --- |

CHAPTER – 14

14.1 Re-shuffling /recouping plan of reserve stone departmentally.

Reshuffling of stone not required as there is no reserve stock exists in premises of Sargodha Drainage Division Sargodha.

14.2 DETAIL OF INLET/OUTLET CROSSING ALONG WITH CLOSING METHODOLOGY.

_______Nil________

XEN DRG SGD

| 14.3 DEPLOYMENT OF MACHINERY. MEDIUM TO HIGH FLOOD IN SARGODHA DRAINAGE DIVISION SARGODHA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sr.No. | Name of Structure | Length in Mile | Vulnerable Reach | Camp Location |  | Site Incharge by name & Cell No. | Machinery Deployed |  |  |  |  | Labour Beldar + Mate | Availability of Stone (Lac Cft) |  |
|  |  |  |  |  |  |  | Excavator |  | Dozer | Trolleys/ Dumpers | Tractor with front blade |  |  |  |
| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |  | 8 | 9 | 10 | 11 | 12 |  |
| FLOOD BUND ALONG RIVER CHENAB IN DRAINGE DIVISION SARGODHA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | F.S Out fall Drain | 5.0 | RD: 0+000- 25+000 |  | RD: 11+000 | Muhammad Saeed Sub Engineer 0301-4995306 | 1 | - |  | 3 | 2 | 10+1 | - |  |
| 2 | Sahiwal Drain | 4.20 | 0+000 to 21+000 |  | 15+000 | Muhammad Raheem Sub Engineer 0306-5201213 | 1 | - |  | 3 | 2 | 10+1 | - |  |
| 3 | L.R. Drain | 4.20 | 246 to 266 |  | 248 | Ghulam Raza Sub Engineer 0302-8146305 | 1 | - |  | 3 | 2 | 10+1 | - |  |
| Total |  |  |  |  |  |  | 3 | 0 |  | 9 | 6 | 30+3 | 0 |  |

Payment for machinery and work charge employees subject to verification by TPM/Consultant.

SDO SHJ									XEN DRG

|  | 14.3 DEPLOYMENT OF MACHINERY. MEDIUM TO HIGH FLOOD IN SARGODHA DRAINAGE DIVISION SARGODHA. Sr.No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site Incharge by name & Cell No.; Machinery Deployed; Labour Beldar + Mate; Availability of Stone (Lac Cft). Excavator; Dozer; Trolleys/ Dumpers; Tractor with front blade. 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. FLOOD BUND ALONG RIVER CHENAB IN DRAINGE DIVISION SARGODHA. 4; Terkhanwala Drain; 4.2; RD: 0+000- 21+000; RD: 12+000; Khawar Azam Sub Engineer 0307-4939043; 1; -; 3; 2; 10+1; -. 5; Shahjewena Drain; 1.625; 0+000 to 8+125; 8+000; Muhammad Javed Sub Engineer 03023167113; 1; -; 2; 1; 10+1. 6; Hadda Out Fall Drain; 4.0; 0-20+000; 6+000; Muhammad Saeed Sub Engineer 0301-4995306; 1; -; 3; 2; 10+1; -. Total; 3; 0; 8; 5; 30+3. Payment for machinery and work charge employees subject to verification by TPM/Consultant. SDO SHJ XEN DRG SGD 14.4 DEPLOYMENT OF MACHINERY. HIGH TO VERY HIGH FLOOD IN SARGODHA DRAINAGE DIVISION SARGODHA.. Sr.No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site Incharge by name & Cell No.; Machinery Deployed; Labour Beldar + Mate; Availability of Stone (Lac Cft). Excavator; Dozer; Trolleys/ Dumpers; Tractor with front blade. 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. FLOOD BUND ALONG RIVER CHENAB IN DRAINGE DIVISION SARGODHA. 1; F.S Out fall Drain; 5.0; RD: 0+000- 25+000; RD: 11+000; Muhammad Saeed Sub Engineer 0301-4995306; 2; -; 4; 2; 15+1; -. 2; Sahiwal Drain; 4.20; 0+000 to 21+000; 15+000; Muhammad Raheem Sub Engineer 0306-5201213; 2; -; 4; 2; 15+1; -. 3; L.R. Drain; 4.20; 246 to 266; 248; Ghulam Raza Sub Engineer 0302-8146305; 2; -; 4; 2; 15+1; -. 4; Terkhanwala Drain; 4.2; RD: 0+000- 21+000; RD: 12+000; Khawar Azam Sub Engineer 0307-4939043; 2; -; 4; 2; 15+1; -. Total; 8; 0; 16; 8; 60+4; 0. Payment for machinery and work charge employees subject to verification by TPM/Consultant SDO SHJ XEN DRG SGD 14.4 DEPLOYMENT OF MACHINERY. HIGH TO VERY HIGH FLOOD IN SARGODHA DRAINAGE DIVISION SARGODHA. Sr.No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site Incharge by name & Cell No.; Machinery Deployed; Labour Beldar + Mate; Availability of Stone (Lac Cft). Excavator; Dozer; Trolleys/ Dumpers; Tractor with front blade. 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. FLOOD BUND ALONG RIVER CHENAB IN DRAINGE DIVISION SARGODHA. 5; Shahjewena Drain; 1.625; 0+000 to 8+125; 8+000; Muhammad Javed Sub Engineer 03023167113; 1; -; 2; 1; 15+1. 6; Hadda Out Fall Drain; 4.0; 0-20+000; 6+000; Muhammad Saeed Sub Engineer 0301-4995306; 2; -; 4; 2; 15+1; -. Total; 3; 0; 6; 3; 30+2. Payment for machinery and work charge employees subject to verification by TPM/Consultant SDO SHJ XEN DRG SGD 14.5 DEPLOYMENT OF MACHINERY. VERY HIGH TO EXCEPTIONAL FLOOD IN SARGODHA DRAINAGE DIVISION SARGODHA.. Sr.No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site Incharge by name & Cell No.; Machinery Deployed; Labour Beldar + Mate; Availability of Stone (Lac Cft). Excavator; Dozer; Trolleys/ Dumpers; Tractor with front blade. 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. 1; F.S Out fall Drain; 5.0; RD: 0+000- 25+000; RD: 11+000; Muhammad Saeed Sub Engineer 0301-4995306; 3; -; 5; 2; 20+1; -. 2; Sahiwal Drain; 4.20; 0+000 to 21+000; 15+000; Muhammad Raheem Sub Engineer 0306-5201213; 3; -; 5; 2; 20+1; -. 3; L.R. Drain; 4.20; 246 to 266; 248; Ghulam Raza Sub Engineer 0302-8146305; 3; -; 5; 2; 20+1; -. 4; Terkhanwala Drain; 4.2; RD: 0+000- 21+000; RD: 12+000; Khawar Azam Sub Engineer 0307-4939043; 3; -; 5; 2; 20+1; -. Total; 12; 0; 20; 8; 80+4; 0; 46+5. Payment for machinery and work charge employees subject to verification by TPM/Consultant SDO SHJ XEN DRG SGD 14.5 DEPLOYMENT OF MACHINERY. VERY HIGH TO EXCEPTIONAL FLOOD IN SARGODHA DRAINAGE DIVISION SARGODHA. Sr.No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site Incharge by name & Cell No.; Machinery Deployed; Labour Beldar + Mate; Availability of Stone (Lac Cft). Excavator; Dozer; Trolleys/ Dumpers; Tractor with front blade. 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. FLOOD BUND ALONG RIVER CHENAB IN DRAINGE DIVISION SARGODHA. 5; Shahjewena Drain; 1.625; 0+000 to 8+125; 8+000; Muhammad Javed Sub Engineer 03023167113; 2; -; 3; 1; 15+1; Shahjewena Drain. 6; Hadda Out Fall Drain; 4.0; 0-20+000; 6+000; Muhammad Saeed Sub Engineer 0301-4995306; 3; -; 4; 2; 15+1; Hadda Out Fall Drain. Total; 5; 0; 7; 3; 30+2. Payment for machinery and work charge employees subject to verification by TPM/Consultant. SDO SHJ XEN DRG SGD 14.6 POLICE DEPLOYMENT PLAN. Sr. No.; Vulnerable sites used for illegal cuts during floods; Concerned Canal Division; Concerned Police Station & Distt; Police Persons Deployed; Remarks ( if any). Inspector/ Sub Inspector/ASI; Constable. 01; F.S OUT FALL DRAIN RD0+000 TO 25; Drainage Division Sargodha; Distt Chiniot; SI = 01 ASI=01; 05; -. 02; SAHIWAL DRAIN RD 0-21; Drainage Division Sargodha; Distt Srgodha; Sub Inspector = 01; 04; -. 03; HADDA OUT FALL RD 0+000 TO 20+000; Drain age Division Sargodha; Distt Chiniot; Sub-Inspector = 01; 04; -. 04; TERKHANWALARDRAIN: 0+000 TO 121+000; Drainage Division Sargodha; Distt Jhang; Sub-Inspector = 01; 04; -. Whole the Drains falls within the administrative jurisdiction of district Sargodha, Jhang and Chaniot Proportionately, In case of observing any sort of law and order situation from the public sides; it shall immediately be contacted by the Sub Divisional Officer, concerned with the respective authority of civil administration to maintain the situation at site to avoid any mishap. SDO SHJ XEN DRG SGD 14.7 DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG AND 1000 KG. Sr.No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site Incharge by name & Cell No.; No. of Synthetic bags; Remarks. 500 Kg; 1000 Kg. 1; 2; 3; 4; 5; 6; 7; 8; 9. 1; F.S Out fall Drain; 5; RD: 0+000- 25+000; RD: 11+000; Muhammad Saeed Sub Engineer 0301-4995306 Muhammad Amir Sub Engineer 03401482022; Required; Available; Required; Available; Not available at stock and will be procured in emergency.. 125; Nil; 125; Nil. 2; Sahiwal Drain; 4.20; 0+00 to 21+000; Rd 15+000; Muhammad Raheem Sub Engineer 0306-5201213; 125; Nil; 125; Nil. 3; L.R. Drain; 4.20; 245-266; Rd 248+000; Ghulam Raza Sub Engineer 0302-8146305; 125; Nil; 125; Nil. SDO SHJ XEN DRG SGD 14.8 . DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG AND 1000 KG S No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site Incharge by name & Cell No.; Plastic Sheet(Sft; Remarks. 1; 2; 3; 4; 5; 6; 7; 8; 9. 4; Terkhanwala Drain; 4.20; 0+000 to 21+000; 12+000; Khawar Azam Sub Engineer. Farooka Shah Section 0341-8780569; Required; Available; Not available at stock and will be procured in emergency.. 125; Nil. 5; Shahjewena Drain; 1.625; 0+000 to 8+125; 8+000; Muhammad Javed Sub Engineer 03023167113; 125; Nil. 6; Hadda Out Fall Drain; 4.00; 0+000 to 20; 18+000; Muhammad Saeed Sub Engineer 0301-4995306; 125; Nil. SDO SHJ XEN DRG SGD 14.8 DETAIL OF POLYTHENE SHEETS OF BLACK COLOR TO PROTECT UPSTREAM SLOP AGAINST WAVE ACTION AND TO CONTROL SEEPAGE THROUGH EMBANKMENTS. S No.; Name of Structure; Length in Mile; Vulnerable Reach; Camp Location; Site In charge by name & Cell No.; Plastic Sheet(Sft; Remarks. 1; 2; 3; 4; 5; 6; 7; 8; 9. FLOOD BUNDS ALONG RIVER CHENAB IN DRAINGE DIVISION SARGODHA. 1; Terkhanwala Drain; 4.20; 0+000 to 21+000; 12+000; Khawar Azam Sub Engineer. Farooka Shah Section 0341-8780569; Required; Available; Not available at stock and will be procured in emergency.. 150; Nil. 2; Shahjewena Drain; 1.62; 0+000 to 8+125; 8+000; Muhammad Javed Sub Engineer 03023167113; 150; Nil. 3; Hadda Out Fall Drain; 4.00; 0+000 to 20+000; 18+000; Muhammad Saeed Sub Engineer 0301-4995306; 150; Nil. SDO SHJ XEN DRG SGD CHAPTER-15 15. BACK UP DIVISION (IN CASE OF BREACH). In case of breach or during emergency, Kirana Canal Division LJC, Sargodha and his field staff will act as sister Division. Sub Divisional Officer, Shahjewana Drainage Sub Division, Sargodha.; Executive Engineer, Sargodha Drainage Division, Sargodha. |
| --- | --- |

`