CHAPTER-1

SALIENT FEATURES OF KALABAGH HEADWORKS DIVISION   DAUDKHEL

  - LOCATION

Kalabagh Headworks Division comprises of Jinnah Barrage & Thal Canal (Main Line Upper). Jinnah Barrage previously called Kalabagh Headworks, situates on the River Indus about three miles downstream of Kalabagh Town. Its topographic location is at latitude 32.95° and longitude 71.50°. The terrain U/S of the Head works is hilly and the general topographic slope is in the North East to South West direction. River approach from Kalabagh Town to Barrage is confined between mountains of salt range on the West and high ridges on the East. The average altitude of the area of East of the Barrage is 670.00 above mean sea level.

### 1.2	GENERAL DESCRIPTION

Territorial jurisdiction of Kalabagh Headworks Division includes Jinnah Barrage and allied structures, Thal Canal (Main Line Upper) in the reach from RD: 0+000 (Head) to RD: 157+662 (Tail), off-taking channels in the uppermost reach and Kas Umar Khan Canal system, the latter being an inundation canal. The Division administers three Sub Divisions viz. Headworks Sub Division, Isakhel Canal Sub Division and Electrical Sub Division, all stationed at Jinnah Barrage Daudkhel. Executive Engineer is the technical and administrative in-charge of the Division who is responsible for reporting to the Superintending Engineer Thal Canal Circle Mianwali. He is supported by the Sub Divisional Officers, Subordinate technical and administrative staff. XEN & SDOs reside at Canal Colony Daudkhel that situates at almost 4 KM from the Barrage. The inundation channel, Kas Umar Khan Canal off takes from Kurram River near Isakhel.

In addition to above infrastructure, different Nullahs like Chichali, Broach, Rakka, Darsola and Jabba do fall under the jurisdiction of Kalabagh Headworks though not administratively controlled by the Division. These Nullahs carry flood waters (hill torrents) from nearby mountainous ranges, locally known as Koh-e-Suleiman.

Index Plan of Division is enclosed at page 52.

The infrastructure of Kalabagh Headworks Division comprises of following major components:

- Jinnah Barrage:

  - Left Guide Bund (LGB) U/S & D/S.

  - Right Guide Bund (RGB) U/S & D/S.

  - Left Marginal Bund (LMB).

  - Right Marginal Bund (RMB) including breaching section.

  - Tie Bund.

  - Subsidiary Weir.

- Thal Canal Main Line Upper RD: 0+000 to 157+662 (31.53 Miles Canal Miles).

- Daudkhel Lift Irrigation Scheme.

- Mooch Lift Irrigation Scheme.

- Kas Umar Khan Canal System (Inundation Canal).

- Right Paikhel Canal (Flood Channel).

- Bokhara Disty (Flood Channel).

- Paikhel Lift Irrigation scheme.

- Disty & Minors.			=	81.48 Miles.

- Nos. of outlets.			=	161 Nos.

- G.C.A.					=	0.068 M. Acres.

- C.C.A. 					=	0.061 M. Acres.

Total length of Disty & Minors is 81.48 Miles that irrigate 61000 acres of area (CCA) through 161 number outlets. Whereas total Gross Command area is 68000 acres.

### ADMINISTRATIVE SETUP

Organization of Kalabagh Headworks Division is shown as under: -

Executive Engineer

SDO Headworks

SDO Isakhel

Sub Engineers Electrical

Head Clerk

SDO Electrical

Divisional Staff

Sub Engineer Hydraulics

Sub Engineer Workshop

Divisional Account Officer Clerk

Divisional Head Draftsman

Sub Engineers Kas Umer Khan Canal System

Sub Engineer Headworks

Sub Engineer Canal Colony

Sub Engineer Main Line Upper

CHAPTER-2

FLOOD PROTECTION AND RIVER TRAINING WORKS

The river training works of Jinnah Barrage are as under:

- RIVER TRAINING WORKS OF JINNAH BARRAGE.

- Left Guide Bund U/S.

- Left Guide Bund D/S.

- Right Guide Bund U/S.

- Right Guide Bund D/S.

- Left Marginal Bund.

- Right Marginal Bund.

- T – Head Spur at RD: 6+500 of RMB (Length of T – Head = 500 ft.)

- Canal Colony Protection Bund.

#### OTHER RIVER TRAINING WORKS ON INDUS RIVER.

- Flood protection works of Kalabagh Town and other adjoining area on Right Flank of River Indus.

- RIVER TRAINING WORKS ON KURRAM RIVER.

- Solid stone Studs = 9 Nos. (For protection of villages of Tehsil Isa Khel on Left Bank of River Kurram).

- Flood Protection Bund RD: 10+000 to RD: 19+500 & RD: 46+000 to RD: 61+000.

- Hockey Spur at RD: 46+000.

- Stone Guide Wall Kas Umar Khan Canal.

- ONGOING FLOOD PROTECTION WORKS.

- M & R Works.

- Restoration of Loose stone downstream of floor of Jinnah Barrage and apron of divide wall.

- Replacing Chocked Liners of Breaching Sections of RMB of Jinnah Barrage.

- Restoration of Oodhy Wala flood protection bund on Adwala Nullah in Tehsil Esakhel District Mianwali.

- ADP Works.

- Channelization of Jabba Nullah Tehsil & District Mianwali.

- CPEC (Deposit Works).

- Construction of Inverted Hockey Spur at RD 19+000 downstream Jinnah Barrage.

- Construction of Left Flood Protection Bund of 6 miles (Flood bund upstream of CPEC Bridge).

- Construction of Right Flood Protection Bund of 3 miles (Flood bund upstream of CPEC Bridge).

- Construction of J-Head Spur RD: 27+000 R/S.

2.1	DESIGN PARAMETERS OF RIVER TRAINING WORKS

| Sr. | Name of Structure | Length (Ft) | Designed Top R.L. | Top Width. (Ft). | side slopes |
| --- | --- | --- | --- | --- | --- |
| 1 | Left Guide Bund U/S | 4099 | 706.00 | 0+000 – 2+028 = 30 Ft 2+028 – 2+528 = 30 – 60 ft. 2+528 – 4+099 = 60 ft. | 1:2 River side. 1:3 Country side |
| 2 | Left Guide Bund D/S | 1522 | 702.00 | 30 | 1:2 River side. 1:3 Country side |
| 3 | Right Guide Bund U/S | 1761 | 706.00 | 30 | 1:2 River side. 1:3 Country side |
| 4 | Right Guide Bund D/S | 1957 | 702.00 | 30 | 1:2 River side. 1:3 Country side |
| 5 | Left Marginal Bund | 7410 | 708.30 – 715.80 | 30 – 40 | 1:3 both sides |
| 6 | Right Marginal Bund | 9084 | 706.00 | 30 | 1:3 both sides |
| 7 | T – Head Spur at RD: 6+500 of RMB. (Length of T – Head = 500 ft.) | 4820 | 706.00 | 15 | 1:2 both sides |
| 8 | Canal Colony Protection Bund | 11730 | 715.80 – 734.20 | 30 | 1:2 River side. 1:3 Country side |
| 9 | Flood protection works of Kalabagh Town and other adjoining area on Right Flank of River Indus. | 1100 | 705.50 | 4 to 5 | 1.5:1 River side. |
| 10 | Guide Wall Wandha Kukranwala. | 1150 | 705.00 | 9 | N.A |
| 11 | Solid Stone Stud = 9 No. for Protection of Villages of Tehsil Isa Khel. | 2 No. = 115’ each, 3 No. = 145’ Each 4 No. = 40 each | 675.00 | 15 | 1:1 Both side. |
| 12 | Ganda bund near village Muqarabwala on left side of River Indus | 1500 | 680.50 | 30 | 1:3 on country 1:2 River side |
| 13 | Hokey Spur Near Village Beho Sydian. | 3585 | 686.00 | 25 | 1.5:1 Both side |
| 14 | Flood Protection Bund (i) RD:12000-19500 (ii) RD: 46000-61000 | 7500 15000 | 725.00 686.00 | 10 15 | 1.5:1 Both side 1.5:1 Both side |
| 15 | Stone Guide Wall Kas Umer Khan | 1200 | 732.50 | 12.72 | N.A |
| 16 | Sloping Stud Nullah Broach = 7 No. | 65 | - | 10 | 1:1 Both side. |
| 17 | Plug Bund Nullah Broach. | 500 | - | 32 | 2:1 on country sides |

NOTE: -	All Guide Bunds are armored with stone pitching whereas Marginal Bunds are made of local earth. Metaled Road (Mianwali – Bannu) has been constructed on Right Marginal Bund. After construction of Jinnah Hydro Power, alignment of Right Marginal Bund has been changed between RD: 0+200 to RD: 4+000. Strengthening and raising of Left Marginal Bund has been completed under Jinnah Barrage Rehabilitation & Modernization Project.

### LOCATION MAP

Location of River Training Works at the Barrage is marked on index plan of Jinnah Barrage attached.

CHAPTER-3

BRIEF HISTORY OF PAST FLOOD EVENTS

Jinnah Barrage was constructed during 1943 and commissioned in 1946. It consists of a weir, left under sluice and right under sluice.  It is designed for 9,50,000 cusec and its ultimate capacity is 11,00,000 cusec. Since its construction, the maximum discharge passed is 10,87,000 cusecs on 30-07-2010. Therein that event, about 937,453 cusec passed through the Barrage and 150,000 cusecs through the breaching section.

During Super Flood event of 2010, U/S Left Guide Bund was completely washed away. It was restored during annual closure of January, 2011 as per original design. The Breaching Section was operated for the first time in history during the Super Flood event of 2010.

### PAST FLOODS AT JINNAH BARRAGE

Histogram of past floods at Jinnah Barrage is given below:

| Year | Date | U/s gauges | D/s gauges | D/s discharge | Flood Category |
| --- | --- | --- | --- | --- | --- |
| 1947 | 09/08/1947 | 690.20 | 686.0 | 526799 | Medium |
| 1948 | 14/08/1948 | 691.45 | 690.65 | 769262 | High |
| 1949 | 20/07/1949 | 689.95 | 689.6 | 698166 | High |
| 1950 | 02/08/1950 | 691.25 | 690.75 | 743757 | High |
| 1951 | 12/08/1951 | 690.80 | 688.95 | 445973 | Medium |
| 1952 | 21/07/1952 | 691.50 | 6.88.90 | 463038 | Medium |
| 1953 | 12/07/1953 | 692.05 | 691.25 | 689661 | High |
| 1954 | 03/08/1954 | 691.50 | 686.3 | 416768 | Medium |
| 1955 | 19/08/19955 | 692.10 | 687.75 | 520622 | Medium |
| 1956 | 18/07/1956 | 691.90 | 688.15 | 566657 | Medium |
| 1957 | 20/07/1957 | 691.60 | 687.2 | 443088 | Medium |
| 1958 | 16/07/1958 | 693.30 | 690.4 | 776311 | High |
| 1959 | 04/07/1959 | 692.00 | 688.4 | 598304 | Medium |
| 1960 | 13/07/1960 | 692.00 | 689.1 | 529400 | Medium |
| 1961 | 30/07/1961 | 692.00 | 688.3 | 523144 | Medium |
| 1962 | 22/07/1962 | 692.00 | 685.55 | 422239 | Medium |
| 1963 | 31/07/1963 | 692.10 | 687.1 | 463718 | Medium |
| 1964 | 16/07/1964 | 691.60 | 690 | 555933 | Medium |
| 1965 | 27/07/1965 | 691.20 | 687.9 | 466850 | Medium |
| 1966 | 26/06/1966 | 691.50 | 689.2 | 500099 | Medium |
| 1967 | 30/07/1967 | 692.00 | 688.85 | 541835 | Medium |
| 1968 | 15/08/1968 | 692.00 | 687.05 | 476166 | Medium |
| 1969 | 13/08/1969 | 692.0 | 687.2 | 465346 | Medium |
| 1970 | 13/08/1970 | 692.50 | 685 | 408067 | Medium |
| 1971 | 29/06/1971 | 692.50 | 686.25 | 428431 | Medium |
| 1972 | 20/07/1972 | 692.50 | 686.3 | 397700 | Low |
| 1973 | 30/08/1973 | 692.50 | 689.2 | 545917 | Medium |
| 1974 | 02/08/1974 | 693.00 | 685.7 | 373854 | Low |
| 1975 | 02/08/1975 | 692.60 | 689.1 | 602541 | High |
| 1976 | 02/08/1976 | 693.00 | 690.7 | 861965 | High |
| 1977 | 17/07/1977 | 692.50 | 688.25 | 574391 | High |
| 1978 | 06/08/1978 | 692.50 | 689 | 639790 | High |
| 1979 | 10/08/1979 | 692.50 | 684.65 | 456840 | Medium |
| 1980 | 26/08/19s80 | 692.50 | 683.7 | 331197 | Low |
| 1981 | 03/07/1981 | 692.50 | 683.8 | 369944 | Low |
| 1982 | 17/08/1982 | 692.50 | 684.8 | 474231 | Medium |
| 1983 | 04/09/1983 | 692.50 | 685.2 | 499793 | Medium |
| 1984 | 15/08/1984 | 692.50 | 683.9 | 491280 | Medium |
| 1985 | 02/08/1985 | 692.50 | 682.1 | 313340 | Low |
| 1986 | 05/08/1986 | 692.50 | 684.3 | 471073 | Medium |
| 1987 | 28/07/1987 | 692.50 | 683.3 | 351268 | Low |
| 1988 | 02/08/1988 | 692.50 | 684.3 | 598303 | Medium |
| 1989 | 01/08/1989 | 692.50 | 685.65 | 557811 | Medium |
| 1990 | 10/08/1990 | 692.50 | 685.1 | 546260 | Medium |
| 1991 | 15/07/1991 | 692.50 | 682.8 | 445847 | Medium |
| 1992 | 10/09/1992 | 692.50 | 688.4 | 846040 | Exceptional High |
| 1993 | 23/07/1993 | 692.50 | 680.6 | 355569 | Low |
| 1994 | 13/07/1994 | 692.50 | 683.4 | 491375 | Medium |
| 1995 | 27/07/1995 | 692.50 | 684 | 544663 | High |
| 1996 | 16/08/1996 | 692.50 | 682 | 441410 | Medium |
| 1997 | 28/08/1997 | 692.00 | 684.9 | 654179 | Very High |
| 1998 | 14/07/1998 | 692.50 | 682.1 | 473221 | Medium |
| 1999 | 10/08/1999 | 692.50 | 682.1 | 456146 | Medium |
| 2000 | 02/08/2000 | 692.50 | 678.3 | 261141 | Low |
| 2001 | 24/07/2001 | 693.00 | 681.7 | 412238 | Medium |
| 2002 | 14/08/2002 | 693.00 | 681.3 | 395589 | Medium |
| 2003 | 03/08/2003 | 692.50 | 683 | 399352 | Medium |
| 2004 | 11/07/2004 | 693.00 | 679.7 | 245067 | Low |
| 2005 | 02/07/2005 | 693.00 | 685.8 | 515073 | High |
| 2006 | 06/08/2006 | 692.50 | 684.50 | 489606 | Medium |
| 2007 | 16/08/2007 | 693.00 | 682.10 | 359912 | Low |
| 2008 | 05/08/2008 | 693.00 | 681.00 | 336530 | Low |
| 2009 | 17/08/2009 | 693.50 | 681.20 | 348320 | Low |
| 2010 | 30/07/2010 | 695.50 | 690.10 | 937453 | Super flood |
| 2011 | 26/07/2011 | 692.50 | 668.80 | 306983 | Low |
| 2012 | 18/07/2012 | 693.00 | 679.65 | 277027 | Low |
| 2013 | 13/08/2013 | 692.50 | 682.50 | 472303 | Medium |
| 2014 | 25/07/2014 | 694.00 | 681.00 | 249992 | Normal |
| 2015 | 02/08/2015 | 692.00 | 685.40 | 529080 | High |
| 2016 | 05/07/2016 | 693.00 | 681.80 | 351490 | Low |
| 2017 | 03/08/2017 | 693.00 | 681.95 | 419460 | Medium |
| 2018 | 15/08/2018 | 693.00 | 682.80 | 311154 | Low |
| 2019 | 15/08/2019 | 693.00 | 684.00 | 364346 | Low |
| 2020 | 03/09/2020 | 693.00 | 685.20 | 457031 | Medium |
| 2021 | 01/08/2021 | 693.00 | 682.00 | 291309 | Low |
| 2022 | 28/08/2022 | 693.50 | 683.60 | 433910 | Medium |
| 2023 | 25/07/2023 | 693.50 | 683.00 | 365549 | Low |
| 2024 | 09/08/2024 | 693.50 | 683.70 | 393739 | Medium |

CHAPTER-4

DESIGNED DATA, HISTORIC PEAK FLOOD DATA AND PREVIOUS YEAR FLOOD DATA OF HEAD WORKS / BARRAGES AND OR OTHER CONTROL POINTS

### DESIGNED DATA OF JINNAH BARRAGE

A. 	GENERAL DATA OF HEADWORKS

River U/S of Weir:

- Design Discharge 							= 9,50,000 Cs

- Ultimate Capacity.					 		= 11,00,000 Cs

- Pond level to feed canal with 7500 Cs: discharge		= 692.00

- Pond level to feed canal with 9, 000 Cs: discharge		= 694.00

- Highest flood level in 1929 (Pre Barrage Construction)		= 693.00

- Highest flood level of Super flood 2010 (RD: 5+000 LMB)	= 701.30

- Assumed maximum H.F.L.						= 701.30

- Assumed lowest water level D/S 					= 674.00

- Critical Gauge location 						= RD: 5+000 LMB

- Critical Gauge Value						= 701.30

B.	WATER WAY

Length of Barrage between the abutments: -

- 55 Bays of 60 ft. each						= 3300 ft.

- 01 number Bay							= 33 ft.

- 01 number navigation lock channel				= 20 ft.

- 54 Piers of 7 ft. each 						= 378 ft.

- 02 number Divide walls and fish ladders 25 ft. each		= 50 ft.

Total	= 3781 ft.

- Water way including navigation lock channel			= 3353 ft.

C.	WEIR

- No of bays (Clear Span)						= 42 of 60 ft.

- Crest level 								= 678.00

- U/S floor level							= 673.00

- U/S curtain level of bottom					= 663.00

- D/S floor level							= 670.00

- Pile line under D/S floor bottom RL 				= 656.00

- Crest width								= 6.00 ft.

- U/S glacis								= 4H:1V

- D/S glacis								= 3H:1V

- Length of impermeable floor U/S and D/S			= 140.00 ft.

- U/S blocks protection						= 11.00 ft.

- U/S loose stone protection					= 25.00 ft.

- D/S blocks protection						= 60.58 ft.

- D/S loose stone protection					= 40.00 ft.

- Height of gates							= 19.00 ft.

D.	SUB-WEIR

- Crest level (Central Portion)					= 675.00 ft

- Crest level (Left & Right under sluices)				= 674.00 ft

- Elevation of u/s floor (ft) (Central Portion)			= 670.00 ft

- Elevation of u/s floor (ft) (Left & Right under sluices)		= 667.00 ft

- Elevation of d/s floor (ft) (Central Portion)			= 662.00 ft

- Elevation of d/s floor (ft) (Left & Right under sluices)		= 662.00 ft

- Length of U/S floor (ft)						= 20

- Thickness of u/s floor (ft)						= 2 ft

- Slope of u/s Glacis							= 2H:1V

- Slope of d/s Glacis							= 3H:1V

- Crest width (ft.)							= 4 ft

- Crest thickness (ft.)							= 3 ft

E.	UNDER SLUICES

- Left 7 bays of 60’ each

- Right 6 bays of 60’ each one bay 33 ft. and one lock channel 20 ft.

- Crest level								= 675.00

- U/S Glacis 								= 4H:1V

- D/S Glacis								= 3H:1V

- U/S Floor level							= 670.00

- U/S curtain wall, R.L of bottom					= 660.00

- D/S floor level							= 667.00

- Pile line under Toe of D/S floor Bottom R.L.			= 653.00

- Crest width								= 6.00 ft.

- Length of pacca floor 						= 150.00 ft.

- U/S loose stone protection 					= 30.00 ft.

- U/S blocks protection						= 16.50 ft.

- D/S blocks protection						= 63.50 ft.

- D/S loose stone protection 					= 50.00 ft.

- Height of gates 							= 22.00 ft.

- Road width								= 20.00 ft.

- Road R.L.								= 706.36

- Overhead Bridge, width of Decking				= 8.00 ft.

- Overhead Bridge, R.L.						= 736.30

F.	FISH LADDER

- Clear span								= 11.00 ft.

- Crest R.L.								= 684.00

- Length of pacca floor						= 237.00

- Length of divided wall						= 731.00

- Top R.L. U/S 							= 702.00

- Top R.L. D/S								= 693.00

G.   	SILT EXCLUDER

- No. of tunnels							= 8

(4 number in Bay No. 1 & 2 of Under sluice)

- Clear span								= 13.50 ft.

- Thickness of pier							= 3.00 ft

- Bottom of floor level							= 675.0

- Top of slab								= 683.00

(Level with Head Regulator crest)

- Overall width of slab							= 132.0 ft.

H.	HEAD REGULATOR OF THAL CANAL

- No. of Bays (Clear Span, water way)				= 7 No of 24 ft.

- Thickness of pier							= 5 ft.

- Crest R.L.								= 683.00

- Length of crest							= 30 ft.

- Total length of pacca floor						= 85 ft.

- D/S block protection 						= 100 ft.

- D/S stone filling							= 30 ft.

- Bottom of breast wall 						= 694.00

- Top of breast wall							= 706.00

- Clear road way							= 25 ft.

- Road R.L.								= 706.15

- SILT EJECTORS

There exist 4 number silt ejectors at RD: 0600, 0835, 1250 and 2100 of Thal Canal as detailed below:

| Sr. No | Silt Ejectors | RD-600 | R.D. 835 | R.D. 1250 | R.D. 2100 |
| --- | --- | --- | --- | --- | --- |
| i) | No of 8 feet bays | 4 | 2 | 2 | 2 |
| ii) | U/S Bed R.L. | 682.83 | 682.00 | 681.50 | 682.000 |
| iii) | D/S Bed R.L. | 682.00 | 681.50 | 681.00 | 680.65 |
| iv) | Top of slab | 682.83 | 682.00 | 681.50 | 681.00 |
| v) | Bed floor level of Tunnels | 678.00 | 677.37 | 676.87 | 676.37 |
| vi) | Height of gates | 4.0’ | 4.0’ | 4.0’ | 4.0’ |
| vii) | Size of orifice | 2x1 | 2x1 | 2x1 | 2x1 |
| viii) | Discharge. | 480 Cs. | 240 Cs. | 240 Cs. | 240 Cs. |

### 4.1-a	FLOOD LIMITS OF RIVER INDUS AT JINNAH BARRAGE

Limits of flood for different categories are fixed by the Punjab Flood Commission as under:-

Normal.					     	=	Up to 2.5 Lac Cs.

Low Flood.					     	=	2.5 to 3.75 Lac Cs.

Medium Flood.	                                      	=     	3.75 to 5.0 Lac Cs.

High Flood 				                	=     	5.0 to 6.50 Lac Cs.

Very High Flood.			                	=     	6.50 to 8.0 Lac Cs.

Exceptionally high flood 			    	=	8.00 to 9.50 Lac Cs.

Super flood 						= 	above 9.50 Lac Cs.

### 4.1-b	FLOOD LIMITS OF RIVER KURRAM AT DARRA TANG

Limits of flood for different categories are fixed by the Punjab Flood Commission as under:-

Low Flood			=	40000 Cs.

Medium flood			=	60000 Cs.

High flood			=	75000 Cs.

### 4.2	TIME LAGS OF FLOODS

| DISTANCE |  | TIME LAG |
| --- | --- | --- |
| From Tarbela to Attock | = 32 KM. | 2 Hours |
| From Attock to Jinnah Barrage | = 168 KM. | 24 Hours |
| From Jinnah Barrage to Chesham Barrage | = 59 KM. | 8 Hours |
| From Chesham Barrage to Taunsa | = 237 KM. | 60 Hours |

### NOTE: Time lag varies according to category of flood.

### 4.3	HIGHEST FLOODS AT JINNAH BARRAGE

Histogram of highest floods at Jinnah Barrage is given below:

| Year | Date | U/s gauges | Critical Gauge RD: 5+000 (LMB) | D/s gauges | D/s discharge | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1948 | 14/8/48 | 691.45 | - | 690.65 | 769262 | Very high |
| 1949 | 20/7/49 | 689.95 | - | 689.60 | 698166 | Very high |
| 1950 | 2/8/50 | 691.25 | - | 690.75 | 743757 | Very high |
| 1953 | 12/7/53 | 692.05 | - | 691.25 | 689661 | Very high |
| 1958 | 16/7/58 | 693.30 | - | 690.40 | 776311 | Very high |
| 1976 | 2/8/76 | 693.00 | - | 690.70 | 861965 | Exceptionally High Flood |
| 1992 | 10/9/92 | 692.50 | - | 688.40 | 846040 | Exceptional High Flood |
| 1997 | 28/8/97 | 692.00 | - | 684.90 | 654179 | Very High |
| 2010 | 30/7/10 | 695.50 | 701.30 | 690.10 | 937453 | Super flood |

NOTE: Critical Gauge was not defined prior to Super Flood 2010.

4.4     PEAK DISCHARGE

| Year | Date | U/s gauges | Critical Gauge RD: 5+000 (LMB) | D/s gauges | D/s discharge | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 2003 | 3/08/03 | 692.50 | - | 683 | 399352 | Medium |
| 2004 | 11/7/04 | 693.00 | - | 679.7 | 245067 | Low |
| 2005 | 02/7/05 | 693.00 | - | 685.8 | 515073 | High |
| 2006 | 06/8/06 | 692.50 | - | 684.50 | 489606 | Medium |
| 2007 | 16/8/07 | 693.00 | - | 682.10 | 359912 | Low |
| 2008 | 05/8/08 | 693.00 | - | 681.00 | 336530 | Low |
| 2009 | 17/8/09 | 693.50 | - | 681.20 | 348320 | Low |
| 2010 | 30/7/10 | 695.50 | 701.30 | 690.10 | 937453 | Super flood |
| 2011 | 26/7/11 | 692.50 |  | 668.80 | 306983 | Low |
| 2012 | 18/7/12 | 693.00 |  | 679.65 | 277027 | Low |
| 2013 | 13/8/13 | 692.50 |  | 682.50 | 472303 | Medium |
| 2014 | 25/7/14 | 694.00 |  | 681.00 | 249992 | Normal |
| 2015 | 02/8/15 | 692.00 |  | 685.40 | 529080 | High |
| 2016 | 05/7/16 | 693.00 |  | 681.80 | 351490 | Low |
| 2017 | 03/8/17 | 693.00 |  | 681.95 | 419460 | Medium |
| 2018 | 15/8/18 | 693.00 |  | 682.80 | 311154 | Low |
| 2019 | 15/9/19 | 693.00 |  | 684.00 | 364346 | Low |
| 2020 | 03/9/20 | 693.00 |  | 685.20 | 457031 | Medium |
| 2021 | 01/8/21 | 693.00 |  | 682.00 | 291309 | Low |
| 2022 | 28/8/22 | 693.50 |  | 683.60 | 433910 | Medium |
| 2023 | 25/7/23 | 693.50 |  | 683.00 | 365549 | Low |
| 2024 | 09/8/24 | 693.50 |  | 683.70 | 393739 | Medium |

NOTE: Critical Gauge was not defined prior to Super Flood 2010.

CHAPTER-5

FLOOD FIGHTING STRATEGY

  - FLOOD FIGHTING STRATEGY FOR RIVER INDUS AT JINNAH BARRAGE

STAGE – I: MEDIUM FLOOD (3.75 LAC CS. TO 5.00 LAC CS.).

When river discharge ranges between these limits, watching establishment will be deployed at the scale of one man per thousand feet.  Sub Engineer Headworks will supervise the entire watching operation of the right flank of river while Sub Engineer Hydraulic will supervise the watching operation of left flank. Sub Divisional Officer Headworks will act as the officer in charge of watching operation.

Establishment of workshop section comprising one fitter and 6 beldars shall remain present at the Headworks for emergent rectification of gates and gearing around the clock. The Sub Engineer workshop will not leave the Headquarter without prior permission of the SDO Headworks. He is authorized by the Divisional Officer to employee 6 seasonal coolies to make up deficiency caused by stationing six regular beldars / coolies at the Headworks so that normal operations do not suffer. Machinery plan attached at 14.3.

STAGE – II: HIGH FLOOD (5.00 LAC Cs. TO 6.50 LAC Cs.).

When the discharge ranges between these limits, watching establishment will be increased to three times of that needed in stage – 1. Supervisory staff shall be the same as in stage 1.  Additionally, Sub Engineer, Workshop will shift his headquarter at the Headworks and will also be available at the Barrage to assist the operation of gates. Machinery plan attached at 14.3.

STAGE – III: VERY HIGH FLOOD (6.50 LAC Cs. TO 8.00 LAC Cs.).

Watching establishment shall be increased to six times of that needed in stage – 1. All establishment of Headworks Sub Division and equal work charge establishment shall be engaged and deployed as detailed below:

Sub Divisional Officer Headworks will be the officer in-charge of entire watching operation on the left as well as on the right flank. Overall supervision of all operations will be done by the Executive Engineer. Sub Engineers of Headworks Sub Division shall supervise various reaches of the left flank as under:

Machinery plan attached at 14.4.

| Sub Engineer, Headworks Section | Regulation at Headworks with guide bunds U/S & D/S. |
| --- | --- |
| Sub Engineer, Colony Section | Left Marginal Bund and Colony protection Bund. |
| Sub Engineer, Hydraulic Section. | Right Marginal Bund in Reach RD: 0+000 to 9+084. |
| Sub Engineer, Main Line Upper Section. | Right Marginal Bund in Reach RD: 6+700 to 8+700 i.e. the Breaching Section & Tie Bund. |

STAGE – IV: EXCEPTIONALLY HIGH FLOOD AND ABOVE (EXCEEDING 8.00 LAC CS).

At this stage, volunteer labors brought by civil administration and Army will help the watching operation. Police guards will be deputed on vulnerable points such as junction points of Canal Colony Protection Bund and Left Marginal Bund to guard against any mishap/sabotage. Overall supervision of all operations will be done by the Executive Engineer. Sub Divisional Officer Headworks Sub Division will be the officer in-charge of watching operation on the left / right flanks, operation of gates, LMB, RMB (U/S & D/S) and, liaison with other departments. Sub Engineers of Headworks Sub Division shall supervise various reaches of the left/right flank. The regulation staff will be under the supervision of SDO and Sub Engineer Headwork’s. The operation will be carried out as per operation manual under the overall supervision of Executive Engineer. The other supporting staff will also be assigned duties as mentioned in chapter 9. Sub Divisional Officer Isakhel Sub Division will also be available at the barrage to assist the operations.

  - FLOOD FIGHTING STRATEGY FOR RIVER KURRAM D/S DARRA TANG BRIDGE

STAGE – I: LOW TO MEDIUM FLOOD (40000 Cs. TO 60000 Cs.).

River training works like Protection Bunds, Stone Studs and Hockey Spur have been constructed on left bank of River Kurram. Work charge establishment for watching during flood season will be required.

##### STAGE – II: MEDIUM TO HIGH FLOOD (60000 Cs. TO 75000 Cs.).

When river discharge ranges between these limits, watching establishment shall be deployed at the scale of 2 beldars for every structure for 24 hours. The Sub Divisional Officer Isakhel Sub Division and Sub Engineer Kas Umar Khan Section will handle each and every aspect of flood in River Kurram.

CHAPTER-6

FLOOD DAMAGES RESTORATION WORKS

The detail of flood damages restoration works is as under: -

| Sr. No. | Name of work | Remarks |
| --- | --- | --- |
| 1 | Replenishment of Stone Apron of Left Guide Bund of Jinnah Barrage. | Estimate submitted. |
| 2 | Restoration of Loose Stone Downstream Floor of Jinnah Barrage. | Work in progress. |
| 3 | Restoration of Oodhy wala flood protection bund on Adwala Nullah in Tehsil Esakhel District Mianwali. | Work in progress. |
| 4 | Replacing Chocked Liners of Breaching Sections of RMB of Jinnah Barrage. | Work in progress. |

CHAPTER-7

FLOOD FIGHTING / WATCHING ARRANGEMENTS

During floods, Sub Divisional officers, in the absence of the Executive Engineer will not hesitate to adopt protective measures for the safety of the Bunds and the Headworks in real emergency, under intimation to higher hierarchy, through quickest means of Communication. Similarly, Sub Engineers in the absence of the Sub Divisional Officer will act accordingly and will show no slackness to inform the factual position to the high ups simultaneously.

  - PRE-FLOOD ARRANGEMENTS

The watching establishment starts making pre-flood arrangements during the month of June for main embankments as well as for 2nd defence line. Flood watching material is arranged in ample quantities. Particular care is exercised to ensure that adequate quantity of the required material is made available at every watching hut especially at the vulnerable sites and that all the articles and materials are in good condition. The scale of watching material has been given at page 20.

All Sub Engineers carryout the inspection of all flood bunds under his respective jurisdiction by foot. They walk all along the bund on river side at the toe of the bund to identify the rat or porcupine holes on the slope. These are subsequently opened, refilled and puddled as per departmental procedures for such works. SDO Headworks inspects all vulnerable reaches of flood bunds by foot and acts in similar manner. Moreover, the Executive Engineer personally watches the whole process to ensure that no rat / porcupine hole is left unattended. SDO SCADA inspects all electrical installations along-with concerned Sub Engineers. He ensures that the maintenance and upkeeping of the overall SCADA and instrumentation system is in order and calibrated for smooth operation during the normal and particularly in flood season.

The SDO Headworks will personally check that:

- All flood fighting and watching material like torches, Kassies, baskets etc... are in good working condition and lightening arrangements at important points of the Headworks and vulnerable reaches of the bunds are satisfactory.

- Adequate arrangements are made for communication of urgent messages from any part of the flood bund or spur to the Executive Engineer in case of an emergency. If there is no telephonic, telegraphic or wireless link, special messenger is kept ready at all time for this purpose.

- Entire watching establishment is properly trained. Dry rehearsals for flood fighting by the end of June is carried out and any shortcoming noticed is immediately get rectified.

During pre-flood period, the watching staff is engaged on following works:-

- All jungle growth from the outer and inner toe of all bunds up to five feet 	      		width is cleared.

- Rats / porcupines and other burrowing animals are eradicated through fumigation and closing their hideouts as per departmental procedures.

- Repairing temporary watching huts.

CONSOLIDATED UPDATE STATUS OF RESERVE STOCK OF STONE

| Sanction Reserved stock of stone | Available Quantity | Balance Quantity | Remarks |
| --- | --- | --- | --- |
| 7.90 Lac cft | 7.52 Lac cft | 0.38 Lac cft | 95% available which is sufficient to meet with any emergency. |

STRUCTURE WISE CURRENT STATUS OF RESERVE STOCK OF STONE

Status of reserve stock of stone on every structure is as under:

| Sr.# | Name of Structure | Sanctioned Quantity | Available Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Left Guide Bund | 2.50 Lac Cft: | 2.154 Lac Cft: | 7.524 Lac Cft quantity of stone is available which is sufficient to meet with an emergent situation. |
| 2 | Right Guide Bund U/S | 2.50 Lac Cft: | 0.914 Lac Cft: |  |
| 3 | Right Guide Bund D/S | -- | 1.421 Lac Cft: |  |
| 4 | Left Marginal Bund | 2.60 Lac Cft: | 1.734 Lac Cft: |  |
| 5 | Right Marginal Bund | 0.20 Lac Cft: | - |  |
| 6 | T-Head Spur | 0.10 Lac Cft: | - |  |
| 7 | Plate Form Left Side of Jinnah Barrage. | -- | 1.301 Lac Cft: |  |
|  | Total:- | 7.90 Lac Cft. | 7.524 Lac Cft: |  |

  - WATCHING ESTABLISHMENT

Sufficient work charge establishment will be engaged for the flood season 2025. Sole purpose to hire the work charge establishment is to combat any untoward incident during the flood season. Establishment will be deployed at sensitive sites of the Headworks Division. This Division has already conducted interviews for the work charge establishment and, in any case, interview may be conducted again following the procedure under rules.

The scale of watching establishment, in addition to regular establishment is as under.

  - For different flood categories, the strength of establishment will be as per detail given below.

- During high to very high flood, strength will be increased by 2 times as needed for medium flood.

- During exceptionally high flood, strength will be increased by 3 times as needed for medium flood.

- During Super Flood strength of man power will be at the discretion of Executive Engineer, Kalabagh Headworks Division.

| Location | - Flood Category |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low (2.50 to 3.75 Lac) |  | Medium (3.75 to 5.00 Lac) |  | High (5.00 to 6.50 Lac) |  | Very High (6.50 to 8.00 Lac) |  | Exceptionally high (8.00 to 9.50 Lac) |  |
|  | Mate + Beldar | Shift | Mate + Beldar | Shift | Mate + Beldar | Shift | SDO + SEN + Mate + Beldar | Shift | SDO + SEN + Mate + Beldar | Shift |
| Left Guide Bund U/S | 0+2 | Per Shift | 0 + 4 | Per Shift | 1 + 8 | Per Shift | 1+1+2+8 | Per Shift | 1+1+2+12 | Per Shift |
| Left Guide Bund D/S | - | Per Shift | 0 + 1 | Per Shift | 0 + 2 | Per Shift | 1+1+0+2 | Per Shift | 1+1+0+3 | Per Shift |
| Right Guide Bund U/S | 0+2 | Per Shift | 0 + 4 | Per Shift | 0 + 8 | Per Shift | 1+1+2+8 | Per Shift | 1+1+2+12 | Per Shift |
| Right Guide Bund D/S | - | Per Shift | 0 + 1 | Per Shift | 0 + 2 | Per Shift | 1+1+0 +2 | Per Shift | 1+1+0 +3 | Per Shift |
| Left Marginal Bund U/S | 0+2 | Per Shift | 1 + 6 | Per Shift | 2 + 12 | Per Shift | 1+1+3+12 | Per Shift | 1+1+3+18 | Per Shift |
| Right Marginal Bund | 0+2 | Per Shift | 1 + 6 | Per Shift | 2 + 12 | Per Shift | 1+1+3+12 | Per Shift | 1+1+3+18 | Per Shift |
| T-Head Spur at RD: 6+500 of RMB | - | Per Shift | 1 + 4 | Per Shift | 1 + 8 | Per Shift | 1+1+2+8 | Per Shift | 1+1+2+12 | Per Shift |
| Canal Colony Protection Bund | 0+1 | Per Shift | 1 + 4 | Per Shift | 2 + 8 | Per Shift | 1+1+2+8 | Per Shift | 1+1+2+12 | Per Shift |

Detail of watching Establishment is as under:

Note: - Payment for labour subject to verification by Third Party Monitoring.

  - ARRANGEMENTS AT SENSITIVE SITES

Following Watching Camps will be established for different flood stages as mentioned below:

- Low to Medium Flood

    - Field office at Jinnah Barrage

    - RD: 1+000 Left Guide Bund

    - RD: 0+000 Right Guide Bund

- High to Exceptionally High Flood

    - Field office at Jinnah Barrage

    - RD: 1+000 Left Guide Bund

    - RD: 0+000 Right Guide Bund

    - RD: 5+000 Right Marginal Bund

    - RD: 6+700 to RD: 8+700 Right Marginal Bund

    - RD: 6+500 Canal Colony Protection Bund

  - WATCHING MATERIAL

Conventional articles required for flood fighting are detailed below. In addition to this, provision of stand by machinery is made in view of experience of super flood 2010.

| Sr. No. | Name of Articles | Unit | Qty: Required | Qty: Available | Qty: Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Torch Electric Charges | No. | 12 | - | 12 | Balance quantity of watching material would be purchased before the onset of flood 2025 subject to Approval of Competent Authority. |
| 2 | Three cell torches | No. | 26 | 16 | 10 |  |
| 3 | Gunny bags | No. | 450 | 250 | 200 |  |
| 4 | Plastic bags | No. | 1250 | - | 1250 |  |
| 5 | Axes | No. | 20 | 10 | 10 |  |
| 6 | Kassies with handle | No. | 130 | 100 | 30 |  |
| 7 | Wooden Killas | No. | 500 | - | 500 |  |
| 8 | Basket | No. | 200 | 35 | 165 |  |
| 9 | Manila rope | Rft | 700 | - | 700 |  |
| 10 | Portable generator with accessories (5 KVA) | No. | 3 | 2 | 1 |  |
| 11 | Shovels | No. | 80 | 40 | 40 |  |
| 12 | Shouldaries | No. | 2 | - | 2 |  |
| 13 | Handle for Kassies. | No. | 50 | - | 50 |  |
| 14 | Boat | No. | 3 | 2 | 1 |  |
| 15 | Charpai | No. | 6 | 6 | - |  |
| 16 | Plastic chairs | No. | 30 | 14 | 16 |  |
| 17 | Tables | No. | 6 | 2 | 4 |  |
| 18 | Briefing Board with stand | No. | 1 | - | 1 |  |
| 19 | Dry Cell for Torches | Doz. | 20 | - | 20 |  |
| 20 | Energy Savers | No. | 100 | - | 100 |  |
| 21 | Funnels | No. | 10 | - | 10 |  |
| 22 | Life Jacket | No. | 28 | 28 | - |  |
| 23 | Rain Coats | No. | 50 | - | 50 |  |
| 24 | Switch / Plugs | No. | 20 | - | 20 |  |
| 25 | Umbrella | No. | 10 | 6 | 4 |  |
| 26 | Wires 7/0.029” | Meter | 1500 | - | 1500 |  |
| 27 | Wooden Boards 10x12 | No. | 20 | - | 20 |  |
| 28 | Wooden Hammers | No. | 5 | 5 | - |  |
| 29 | Bulb Holder | No. | 24 | - | 24 |  |
| 30 | Solution Tape | No. | 10 | - | 10 |  |
| 31 | Gloves | No. | 50 | - | 50 |  |
| 32 | M. Oil | Liter. | 100 | - | 100 |  |
| 33 | Petrol for generator | Liter. | 500 | - | 1000 |  |
| 34 | Kerosene Oil | Liter. | 200 | - | 200 |  |

  - ARRANGEMENT FOR SOUNDING AND PROBING

For every vulnerable location, the arrangements made for sounding and probing are as follows:

| Sr. # | Items |
| --- | --- |
| 1. | One number large boat with boatman. |
| 2. | One number small boat with boatman. |
| 3. | Two number Helpers to boatman. |
| 4. | Two number sounders per boat. |

Boats with boatmen are available at Jinnah Barrage & can easily be used as & when required.

  - LIGHTING ARRANGEMENT

Sufficient stock of dry cell torches and chargeable emergency lights etc. will be made available in the emergency stores. Mobile Generating Sets shall be arranged at vulnerable points. Sub Engineer, Headworks assisted by electricians will be responsible to keep these items in working condition.

  - RATION ARRANGEMENT

The establishment and supervisory staff will make their own food arrangements. Deputy Commissioner Mianwali and District Food Controller Mianwali will be requested to keep a reserve stock of the articles at Jinnah Barrage / Canal Colony Daudkhel for supply to the labourers whenever a flood emergency occurs.

- Wheat Flour (Atta) 10 Munds, per week of emergency period.

- Gur, 10 Munds per week of emergency period.

- Onions 2 Munds per week of emergency period.

- Sugar 5 Munds per week of emergency period.

- Rice 10 Munds per week of emergency period.

  - POL ARRANGEMENTS FOR VEHICLES

2 number nearby filling stations (one at Kot Chandna on right side of river and the other at RD 10+800 of Thal Main Line Upper on left side of river) are identified. Both stations situate within a distance of three Km from the Barrage. Supply order will be placed when the flood season commences. However, POL will be bought to the site on need basis. Overall contents of POL proposed are as under:

- Petrol.		=	1000 Litres

- Diesel.		=	2000 Litres

- Kerosene Oil.		=	500 Litres

  - TRANSPORTATION ARRANGEMENTS

Kalabagh Headworks Division has Two double cabin, One Jeep, One truck, One Cran mounted, Truck and pickup number vehicles and one number Tractor with Trolley. In the event of an emergency, 3 number Pick-Ups will be hired on receipt of information of discharge above the range of low flood. In case of an acute emergency, DC Mianwali and DPO Mianwali will be requested to arrange vehicles as per requirements.

  - LAW AND ORDER

Law and order situation may arise if a breach occurs in a flood embankment (God Forbid). In such a situation, Civil Administration and Police shall provide the necessary cover.

  - MEDICAL ARRANGEMENT FOR LABOUR

District Health Officer, Mianwali will ensure sufficient medical staff and medicine in B.H.U Canal Colony, Daudkhel. He will also set up a mobile dispensary on Barrage for emergency fully equipped with medical staff and medicine required against epidemics, snake bites, cholera, Dysenteries etc. The Rescue 1122 Centre Daudkhel will respond immediately in case of emergency. Executive Engineer, Headworks Division Daudkhel will inform Civil Administration as well as District Health Officer in such case.

  - LIAISON WITH OTHER DEPARTMENTS

Executive Engineer, Kalabagh Headworks Division, and Sub Divisional Officers, will keep liaison with their counter parts in other Departments. Higher Authorities of other Departments will be approached if needed. At site, Sub Engineer Headworks and Sub Engineer Hydraulics will coordinate inspections of visiting officers of Civil and Army if no other higher officer is available at spot.

  - ROLL OF THE ARMY

In case of extreme emergency, on request by Executive Engineer, the Deputy       Commissioner, Mianwali shall be obliged to call the Army for immediate help and rescue. 100 Army men may be requested to station at the Barrage as and when discharge exceeds 6.50 Lac Cusecs.

  - DUTIES OF TELEPHONE ATTENDANT AND SIGNALLERS

Telephone & Telegraph office is located in Divisional Office, Jinnah Barrage and in Canal Colony Mianwali. Telegraph Office at canal colony Daudkhel is facilitated with PTCL Telephone No. 0459-236969. Telephone Attendants and Signallers shall be available round the clock in order to collect and provide information regarding flood situation. Detail of duties are as under: -

| Sr No. | Name of Employee | Period | Time |  |
| --- | --- | --- | --- | --- |
|  |  |  | From | To |
| 1 | Muhammad Ishtiaq, Signaller 0306 – 7810698 Muhammad Iqbal Khan, Signaller 0308-1435657 | Flood Season | 08:00 | 16:00 |
| 2 | Tariq Mahmood, Telephone Attendant, 0300 – 5191099 Muhammad Ilyas, Regulation Beldar, 0308 – 7898136 | Flood Season | 16:00 | 00:00 |
| 3 | Muhammad Rizwan, Signaller 0308-2783071 Habib Ullah, Peon 0301-6351421 | Flood Season | 00:00 | 08:00 |

  - COMMUNICATION/WIRELESS ARRANGEMENTS

All concerned officers and officials will be facilitated with cell phones. This arrangement will be ensured before onset of forth coming flood. All the official vehicles are also equipped with the mobile / wireless sets. These units will help the officers while on tour to remain in touch with flood situation.

CHAPTER-8

DETAIL OF ENCROACHMENT

There is no encroachment along flood works of Kalabagh Headworks Division Daudkhel. A certificate is added in this regard.

Detail of Encroachments:

| Sr. No | Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| NIL | NIL | NIL | NIL | NIL |

Detail of Critical Encroachments:

| Sr. No | Total Encroachments | Removed | Remaining | Remarks |
| --- | --- | --- | --- | --- |
| NIL | NIL | NIL | NIL | NIL |

### CERTIFICATE

It is certified that no encroachment exists on Flood Protection Works and River Training Works in the Jurisdiction of Kalabagh Headworks Division Daudkhel.

| ### Sub Divisional Officer ### Headworks Sub Divison ### Daudkhel, Mianwali | ### Executive Engineer Kalabagh Headworks Division Daudkhel, Mianwali |
| --- | --- |

CHAPTER-9

DUTY ROSTER / FLOOD FIGHTING PROGRAM

Duty Roster of key personal During Flood Season 2025:

| SITE | LENGTH (ft.) | LABOR REQUIRED | STAFF FOR WATCHING BE DEPUTED (PER SHIFT) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SDO | SEN | MATE | BELDAR |
| STAGE – I: LOW FLOOD (2.50 LAC CS. TO 3.75 LAC CS.). |  |  |  |  |  |  |
| Left Guide Bund U/S. | 4099 | 1 – Man Per 1000 ft | - | - | - | 2 |
| Left Guide Bund D/S. | 1522 |  | - | - | - | - |
| Right Guide Bund U/S. | 1761 |  | - | - | - | 2 |
| Right Guide Bund D/S. | 1957 |  | - | - | - | - |
| Left Marginal Bund. | 7410 |  | - | - | - | 2 |
| Right Marginal Bund. | 9084 |  | - | - | - | 2 |
| T-Head Spur at RD: 6+500 of RMB. | 4820 |  | - | - | - | - |
| Canal Colony Protection Bund. | 11730 |  | - | - | - | 1 |
| STAGE – II: MEDIUM FLOOD (3.75 LAC CS. TO 5.00 LAC CS.). |  |  |  |  |  |  |
| Left Guide Bund U/S. | 4099 | 1 – Man Per 1000 ft | - | - | - | 4 |
| Left Guide Bund D/S. | 1522 |  | - | - | - | 1 |
| Right Guide Bund U/S. | 1761 |  | - | - | - | 4 |
| Right Guide Bund D/S. | 1957 |  | - | - | - | 1 |
| Left Marginal Bund. | 7410 |  | - | - | 1 | 6 |
| Right Marginal Bund. | 9084 |  | - | - | 1 | 6 |
| T-Head Spur at RD: 6+500 of RMB. | 4820 |  | - | - | 1 | 4 |
| Canal Colony Protection Bund. | 11730 |  | - | - | 1 | 4 |
| STAGE – III: HIGH FLOOD (5.00 LAC Cs. TO 6.50 LAC Cs.). |  |  |  |  |  |  |
| Left Guide Bund U/S. | 4099 | 2 – Man Per 1000 ft | - | - | 1 | 8 |
| Left Guide Bund D/S. | 1522 |  | - | - | - | 2 |
| Right Guide Bund U/S. | 1761 |  | - | - | - | 8 |
| Right Guide Bund D/S. | 1957 |  | - | - | - | 2 |
| Left Marginal Bund. | 7410 |  | - | - | 2 | 12 |
| Right Marginal Bund. | 9084 |  | - | - | 2 | 12 |
| T-Head Spur at RD: 6+500 of RMB. | 4820 |  | - | - | 1 | 8 |
| Canal Colony Protection Bund. | 11730 |  | - | - | 2 | 8 |
| STAGE – IV: VERY HIGH FLOOD (6.50 LAC Cs. TO 8.00 LAC Cs.). |  |  |  |  |  |  |
| Left Guide Bund U/S. | 4099 | 3 – Man Per 1000 ft | SDO H/W | SEN H/W | 2 | 8 |
| Left Guide Bund D/S. | 1522 |  | SDO H/W | SEN H/W | - | 2 |
| Right Guide Bund U/S. | 1761 |  | SDO H/W | SEN H/W | 2 | 8 |
| Right Guide Bund D/S. | 1957 |  | SDO H/W | SEN H/W | - | 2 |
| Left Marginal Bund U/S. | 7410 |  | SDO H/W | SEN CC | 3 | 12 |
| Right Marginal Bund. | 9084 |  | SDO H/W | SEN MLU | 3 | 12 |
| T-Head Spur at RD: 6+500 of RMB. | 4820 |  | SDO H/W | SEN MLU | 2 | 8 |
| Canal Colony Protection Bund. | 11730 |  | SDO H/W | SEN CC | 2 | 8 |
| ##### STAGE – V: EXCEPTIONALLY HIGH FLOOD AND ABOVE (EXCEEDING 8.00 LAC CS.). |  |  |  |  |  |  |
| Left Guide Bund U/S. | 4099 | 3 – Man Per 1000 ft | SDO H/W | SEN H/W | 2 | 12 |
| Left Guide Bund D/S. | 1522 |  | SDO H/W | SEN H/W | - | 3 |
| Right Guide Bund U/S. | 1761 |  | SDO H/W | SEN H/W | 2 | 12 |
| Right Guide Bund D/S. | 1957 |  | SDO H/W | SEN H/W | - | 3 |
| Left Marginal Bund U/S. | 7410 |  | SDO H/W | SEN CC | 3 | 18 |
| Right Marginal Bund. | 9084 |  | SDO H/W | SEN MLU | 3 | 18 |
| T-Head Spur at RD: 6+500 of RMB. | 4820 |  | SDO H/W | SEN MLU | 2 | 12 |
| Canal Colony Protection Bund. | 11730 |  | SDO H/W | SEN CC | 2 | 12 |

CHAPTER-10

EMERGENCY TELEPHONE NUMBERS

| S.# | Name of post / Designation | Station / HQ. | Telephone No. |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Office | Residence | Mobile |
| 1 | Superintending Engineer, Thal Circle | Mianwali. | 0459-920181 | 0459-920183 | 0333-5901500 |
| 2 | Executive Engineer, Kalabagh Headworks | Daudkhel | 0459-236969 | 0459-236969 | 0348-8633368 |
| 3 | Sub Divisional Officer, Headworks | Daudkhel | 0459-236969 | 0459-236969 | 0301-3335402 |
| 4 | Sub Divisional Officer, Isakhel | Daudkhel | 0459-236969 | 0459-236969 | 0333-6635103 |
| 5 | Sub Divisional Officer, Kundian | Kundian | 0459-242544 | - | 0300-2836761 |
| 6 | Deputy Commissioner, | Mianwali. | 0459-234300 | 0459-232595 |  |
| 7 | Project Director Chashma Barrage | Chashma Barrage | 0459-923285 0459-241473 | 0459-923285 |  |
| 8 | Canal Telephone Officer | Mianwali. | 0459-920130 | 0459-920130 |  |
| 9 | Canal Telephone Officer | Daudkhel | 0459-388036 | 0459-388036 |  |
| 10 | Assistant Engineer Agriculture | Mianwali. | 0459-233168 | - |  |
| 11 | District Police Office | Mianwali. | 0459-232704 0459-920092 | - |  |
| 12 | Flood Emergency of D.C.O. Office | Mianwali. | 0459-230360 0459-920152 | 0459-232555 |  |
| 13 | D.H.O. Mianwali. | Mianwali. | 0459-920128 | - |  |
| 14 | Medical Superintendent. | Mianwali. | 0459-232888 | - |  |
| 15 | District Flood Controller. | Mianwali. | 0459-233641 | - |  |
| 16 | Sub Divisional officer, Telephone. | Mianwali. | 0459-232260 | 0459-920000 |  |
| SARGODHA / LAHORE. |  |  |  |  |  |
| 1 | Secretary Irrigation Department | Lahore | 042-99212117 042-99212118 | 04299200951 |  |
| 2 | Chief Engineer, Irrigation Zone, | Sargodha | 048-9230445 | 048-9230451 | 0322-4486727 |
| 3 | Flood emergency officer | Sargodha | 048-9230456 | 048-9230003 |  |
| 4 | Executive Engineer Operation Irrigation | Sargodha | 048-9230446 | 048-9230414 | 03029079639 |

| S.# | Name of post / Designation |  | Station / HQ. | Telephone No. |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Office | Residence | Mobile |
| 5 | Flood warning Center |  | Lahore | 042-99205371 042-99205372 | - |  |
| 6 | Deputy Secretary (Op) |  | Lahore | 042-99212126-042-99212127 | - |  |
| 7 | Chief Engineer, Drainage & Flood. |  | Lahore | 042-99233551 | 042-35900049 |  |
| 8 | Director Flood. |  | Lahore | 042-99233552 | 042-36653943 |  |
| 9 | Telegraph Office |  | Lahore | 042-99212166 | - |  |
| 10 | Director Regulation. |  | Lahore | 042-99212147 | - |  |
| MISCELLANEOUS |  |  |  |  |  |  |
| 1 |  | Executive Engineer, Mianwali Division | Mianwali | 0459-920137 | 0459-920154 | 0301-4585765 |
| 2 |  | Executive Engineer Bhakkar Division | Bhakkar | 0453-9200016 | - | 0306-1577172 |
| 3 |  | Executive Engineer Layyah Division | Layyah | 0606-412435 | - | 0300-6371952 |

CHAPTER-11

STANDARD OPERATING PROCEDURE FOR BREACHING SECTION

  - HISTORY OF THE BREACHING SECTION

There are 04 nominated and well established breaching section between RD: 6+700 and 8+700 of RMB. It was approved by the Technical Sub Committee of Punjab Flood Commission on 09-02-1982. The critical RL for the operation of this breaching section is 701.30. The critical gauge situates at RD: 5+000 of LMB. This breaching section was operated for a single time in the history of Jinnah Barrage on 30-07-2010 i.e. during the event of super flood of 2010. No major damage was reported due to its operation. The spilled water again joined the main river stream approximately about two miles D/S of the breaching section. However, since the main Mianwali – Bannu road exists over RMB, operation of the breaching section resulted in closure of this road and the transportation remained suspended for 30 days. Alternate route for light traffic is available through Railway Bridge at Kalabagh Town.

  - LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION.

Updated status of explosive / accessories for flood season 2025 is as under:-

| Sr. No. | Name / Nomenclature of explosive materials / Accessories | Required Quantity | Available Quantity | Deficient Quantity |
| --- | --- | --- | --- | --- |
| 1 | Wabox 80 % - 25 mm. | 9605-KG | 9605-KG | - |
| 2 | Wabocord – 10 gm / Mtr. | 3994-Mtr | 3994-Mtr | - |
| 3 | Firing cable. | 760-Mtr | 760-Mtr | - |
| 4 | Det No. 8 – plain (non electrical). | 138-Nos. | 138-Nos. | - |
| 5 | Electronic Det no. 8 3 MWL. | 19-Nos. | 19-Nos. | - |
| 6 | M.S Electric Delay Det. | 80-No. | 80-No. | - |
| 7 | Blasting Machine German. | 02-No. | 02-No. | - |
| 8 | Hands crimpers. | 08-No. | 08-No. | - |
| 9 | Pe3 A. | 4799-KG | 4799-KG | - |
| 10 | PE – 3A (450-Gram Slab). | 224.448 Kg | 224.448 Kg | - |
| 11 | Wabonite. | 1107.84 Kg | 1107.84Kg | - |
| 12 | Detonating cord / Wabocord. | 480 M | 480 M | - |
| 13 | Primer. | 48 No. | 48 No. | - |
| 14 | Elec Detonator. | 8 No. | 8 No. | - |
| 15 | Non Electric Detonator. | 8 No. | 8 No. | - |
| 16 | Detonators No. 33 MS / 25 (0.25 Sec Delay). | 12 No. | 12 No. | - |
| 17 | Detonators No. 33 MS / 50 (0.5 Sec Delay). | 12 No. | 12 No. | - |
| 18 | Safety Fuse. | 1547 No. | 1547 No. | - |

  - ARRANGEMENT OF EXPLOSIVES AND SECURITY OF EXPLOSIVE STORES

Explosive material is placed at Sargodha Cantonment under the control of Engineers Crops 1 Kharian and 3 Air Defense Division Sargodha. Security of the explosive store is the responsibility of the Army as directed by the higher authorities.

  - LIST OF THE SECURITY STAFF WITH DETAIL OF THEIR TRAINING ETC

It has been explained in preceding para.

  - DETAIL OF MECHANICAL MEANS AS A STAND – BY ARRANGEMENTS IN CASE OF DETONATION FAILURE

During operation of Breaching Section in Super Flood of 2010, it was experienced that excavator machine can also be used as an alternative arrangement in case of failure of detonation. Hiring of excavator machines has been provided in the list of Flood fighting material.

  - DUTY ROSTER IN CASE OF CRITICAL SITUATION

Arrangements as per duty roster will be ensured in case of critical situation. Duty roster of key personal during flood season 2025 is attached (Chapter – 9). In all vulnerable reaches / sites such as RD: 0+000 to 1+000 of LMB, RD: 4+000 to 6+000 and RD 10+800 to RD 11+730 of Colony Protection Bund, gang of 10 beldars and one mate will be available around the clock when river rises above low flood level. A gang of 10 beldars including 2 mates will remain vigilant at Left Guide Bund. Junction point of LMB / LGB will be focused even during low floods till the end of flood season. A reasonable quantum of machinery will be readily available at this place throughout the flood season to combat any sort of emergency.

  - BREACHING COMMITTEE WITH ITS ACTION PLAN

The breaching committee was constituted in July 2015 vide Secretary, Irrigation Department Punjab, Lahore Notification No. SO (FLOOD) VI-33/97 dated 15.07.2015. The breaching section will be operated on the order of following committee:

- Deputy Commissioner, Mianwali.					= Convener.

- Executive Engineer, Kalabagh Headworks Division.		= Member.

- Representative from C & W Department not below the

Rank of Executive Engineer.  					= Member.

- Representative from Pakistan Army not below the

Rank of Lt. Colonel.							= Member.

After receiving release information from Tarbela, only 26 hours for making necessary arrangements for operating breaching section are available. Executive Engineer Kalabagh issues warning to all concerned Departments / Agencies to become alert, vigilant and remain in contact with the committee. Before operation of Breaching Section, the explosive material placed at Sargodha would be shifted at Jinnah Barrage.  Executive Engineer, Kalabagh Headworks will also convey time for operation of the breaching section through available communication system.

  - LIST OF VILLAGES LIKELY TO BE INUNDATED IN CASE OF BREACH

No village / abadies will be affected. Only 100 to 150-acre land will be inundated.

  - ANNOUNCEMENT & DETAIL OF EVACUATION ARRANGEMENTS

In case of emergency, proper announcement through speaker etc will be made to keep potential affected well informed. The evacuation will be carried by Civil Administration with the help of Army Authorities.

  - DETAIL OF COORDINATION WITH CIVIL / ARMY AUTHORITIES

There is a definite need to ensure good coordination with Provincial Departments for combating flood menace. Respective heads of Provincial Departments and Commanders Corps Engineers will have effective coordination on all matters relating to flood mitigation and relief measures. Provincial Departments as well as Army Authorities will be informed about various flood stages. The coordination will be carried out on telephone and on wireless system as well. The list of telephone numbers is attached in Chapter – 10.

  - PARALLEL COMMUNICATION ARRANGEMENT

All concerned officers and officials will be facilitated with PTCL V mobile sets and cell phone. This arrangement will be ensured before onset of forth coming flood season. All official vehicles will also be equipped with the mobile / wireless sets. These units will help the officers while on tour to remain in touch with flood situation.

  - INDEX PLAN (BREACHING SECTION).

Index plan is attached at page 34.

| Executive Engineer Kalabagh Headworks Division Daudkhel | Superintending Engineer Thal Canal Circle Mianwali | Chief Engineer Sargodha Irrigation Zone Sargodha |
| --- | --- | --- |

PART-B

CHAPTER-12

VULNERABLE SITES ON FLOOD BUND / STRUCTURES

  - APPREHENDED BREACH SITES IN FLOOD BUNDS / STRUCTURES.

Potential breach sites on different flood embankments are presented hereunder:

      - Left Guide Bund Upstream RD: 0 – 4+099.

      - Left Marginal Bund RD: 0 – 7+410.

      - Right Marginal Bund RD: 0 – 9+084.

      - Canal Colony Protection Bund RD: 0 – 11+730.

      - Hill Torrents of Tehsil Isakhel & Mianwali.

  - OPERATION OF BREACHING SECTION

After receiving release information from Tarbela, only 26 hours for making necessary arrangements for operating breaching section are available. Executive Engineer Kalabagh issues warning to all concerned Departments / Agencies to become alert, vigilant and remain in contact with the committee. Before operation of Breaching Section, the explosive material placed at Sargodha would be shifted at Jinnah Barrage.  Executive Engineer, Kalabagh Headworks will also convey time for operation of the breaching section through available communication system.

  - BREACHES DUE TO RISING OF FLOOD WATER, DETERIORATION OF FLOOD BUNDS ETC.

Since its commissioning in 1946, maximum discharge ever passed through the barrage is 108,7000 recorded on 30-07-2010. Out of which, almost 937,000 cusecs passed through weirs and 150,000 cusecs through breaching section. Left Guide Bund at U/S of the Barrage breached during this event of Super Flood. It was restored during annual Closure of January, 2011 as per original design. The Breaching Section was also operated during this event to safeguard the Headworks. It was again damaged due to widening of Mianwali – Bannu road by Highway Department in 2017. Its restoration has now been completed.

CHAPTER-13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITES

- EMERGENCY CONTINGENCY PLAN FOR SITE NO.  1: BREACH TO LEFT GUIDE BUND UPSTREAM RD: 0 – 4+099.

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS

Route of water in case of failure of Left Guide Bund upstream is marked on the plan attached.

  - DETAIL OF VILLAGES / ABADIES LIKELY TO BE AFFECTED

- Daudkhel Town.

- Thathi.

- Pai Khel.

- Dher Umaid Ali Shah.

- Kot Belian.

- Mooch.

- Khawaja Abad Sharif.

- Dillewali.

- Tari Khel.

  - STRATEGY AND ACTION TAKEN.

In case of breach to Left Guide Bund, water will remain in the pond area and no damage to any public property, infrastructure or inundation of agriculture land will occur. Left Marginal Bund will act as 2nd defence line in this case. If Left Guide Bund comes under unbearable stress and gives away, the following instructions will be followed.

- Immediate arrangements will be made for closure of breach in following manner:-

- Watching establishment stationed at junction point of LGB and LMB will immediately move under the supervision of Sub Engineer Headworks for filling stone in trollies (stone stacked at LMB and LGB).

- Another group of establishment under the supervision of Sub Engineer Main Line Upper will start tree launching. For the purpose trees will be collected / cut from thick plantation existing between left bank of Thal Canal and LMB. The trees will be moved through tractor / trollies laying in stand by condition at junction of LMB and LGB.

- Administration of Maple Leaf Cement Factory situated 8 – Km away from Jinnah Barrage will be requested for logistic support if required i.e providing stone or boulders at site.

- At the same time nearby available Government Contractor will be engaged to enhance the resources in shape of machinery, supply of requisite material and labour at site.

- Labour of Kundian Sub Division and Other Sister Divisions of Irrigation Department will be called on phone.

- Fighting on Left Marginal Bund to protect against the severe threat by tree launching, stone dumping and strengthening will also be carried out by engaging above resources.

3.	Civil Administration and local Police will be requested to maintain law and order as well as to provide volunteer labour and other resources on request of Executive Engineer Kalabagh Headworks Division.

    - ARRANGEMENTS AT SENSITIVE SITES

In all vulnerable reaches / sites, a gang of 10 beldars and one mate will be made available around the clock when river rises above low flood level. And a gang of 10 beldars including 2 mates will remain vigilant at Left Guide Bund. Junction point of LMB/LGB will be focused even in low flood till the end of flood season. A reasonable quantum of machinery will be kept ready at this place throughout the flood season to combat any emergency.

Plan showing route of flood water coming out of the breach is attached Annex: A

    - ESTABLISHMENT OF FLOOD FIGHTING CAMPS

- Field Office at Jinnah Barrage.

- RD: 1+000 of Left Guide Bund.

    - DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

| Camp No | Location | Officer Name / Designation / Mobile No. | Official Name / Designation / Mobile No. |
| --- | --- | --- | --- |
| Camp No. 1 | Field Office Jinnah Barrage | Zia Ur Rehman / SDO Headworks / 0301-3335402 | M Sarfraz / SEN Headworks / 0300-6079014 |
| Camp No. 2 | Left Guide Bund RD: 1+000 | Zia Ur Rehman / SDO Headworks / 0301-3335402 | M Sarfraz / SEN Headworks / 0300-6079014 |

    - DEPARTMENTAL MACHINERY AVAILABLE

Kalabagh Headworks Division possesses 02 number tractors with trolley & water boozer, 01 number sand bag filling machine, 01 number small crane mounted truck and 01 number mini truck. However, contractor will be hired through competitive bidding before the flood season for hiring of machinery at the vulnerable sites to counter any untoward incident.

    - MACHINERY AVAILABLE FROM PRIVATE SOURCE

Maple Leaf Cement Factory is located at about 8 – Km from Barrage. This private source will be requested in case of emergency to provide necessary machinery to combat an emergent situation.

    - FLOOD FIGHTING MATERIAL REQUIRED

    - FLOOD FIGHTING MATERIAL AVAILABLE

| Sr. No. | Name of Articles | Unit | Qty: Required | Qty: Available | Qty: Balance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Torch Electric Charges. | No. | 12 | - | 12 | Balance quantity of watching material would be purchased before the onset of flood 2025. |
| 2 | Three cell torches. | No. | 26 | 16 | 10 |  |
| 3 | Gunny bags | No. | 450 | 250 | 200 |  |
| 4 | Plastic bags | No. | 1250 | - | 1250 |  |
| 5 | Axes. | No. | 20 | 10 | 10 |  |
| 6 | Kassies with handle. | No. | 130 | 100 | 30 |  |
| 7 | Wooden Killas. | No. | 50 | 0 | 50 |  |
| 8 | Basket. | No. | 200 | 35 | 165 |  |
| 9 | Manila rope | Rft | 700 | 0 | 700 |  |
| 10 | Portable generator with accessories. | No. | 3 | 2 | 1 |  |
| 11 | Shovels | No. | 80 | 40 | 40 |  |
| 12 | Shouldaries. | No. | 2 | - | 2 |  |
| 13 | Handle for Kassies. | No. | 50 | - | 50 |  |
| 14 | Boat. | No. | 3 | 2 | 1 |  |
| 15 | Charpai | No. | 6 | 6 | - |  |
| 16 | Plastic chairs | No. | 30 | 14 | 16 |  |
| 17 | Tables | No. | 6 | 1 | 5 |  |
| 18 | Briefing Board with stand | No. | 1 | - | 1 |  |
| 19 | Dry Cell for Torches | Doz. | 20 | - | 20 |  |
| 20 | Energy Savers | No. | 100 | - | 100 |  |
| 21 | Funnels | No. | 10 | - | 10 |  |
| 22 | Life Jacket | No. | 28 | 28 | - |  |
| 23 | Rain Coats | No. | 50 | - | 50 |  |
| 24 | Switch / Plugs | No. | 20 | - | 20 |  |
| 25 | Umbrella | No. | 10 | - | 10 |  |
| 26 | Wires 7/0.029” | Meter. | 1500 | - | 1500 |  |
| 27 | Wooden Boards 10x12 | No. | 20 | - | 20 |  |
| 28 | Wooden Hammers | No. | 5 | 5 | - |  |
| 29 | Bulb Holder | No. | 24 | - | 24 |  |
| 30 | Solution Tape | No. | 10 | - | 10 |  |
| 31 | Gloves | No. | 50 | - | 50 |  |
| 32 | M. Oil | Liter. | 100 | - | 100 |  |
| 33 | Petrol | Liter. | 1000 | - | 1000 |  |
| 34 | Diesel | Liter. | 2000 | - | 2000 |  |
| 35 | Kerosene Oil | Liter. | 200 | - | 200 |  |

- EMERGENCY CONTINGENCY PLAN FOR SITE NO. 2: BREACH

TO LEFT MARGINAL BUND RD: 0 – 7+410.

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS

Route of water in case of failure of Left Marginal Bund is marked on the plan attached.

  - DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED

            - Daudkhel Town.

            - Thathi.

            - Pai Khel.

            - Dher Umaid Ali Shah.

            - Kot Belian.

            - Mooch.

            - Khawaja Abad Sharif.

            - Delewali.

            - Tari Khel.

  - STRATEGY AND ACTION TAKEN.

Left Marginal Bund is the only defence line available to protect Thal Canal and Abadies laying on lower contour up to Mianwali. If God forbid, this embankment gives away, following instructions / steps will be followed:

- It is a matter of fact that breach to Left Marginal Bund will put unbearable stress on left bank of Thal Canal. Therefore, in case of breach to Left Marginal Bund, Thal Canal will be closed from its Head. It will be plugged at RD: 10+800 and Canal banks will be cut between RD: 4+000 to RD: 8+000 to provide passage to flood water to enter back in the main river.

- Immediate arrangements have to be made for closure of breach in following manner:

    - Labour already stationed at junction point of LGB and LMB and at RD: 7+000 of LMB will immediately move under the supervision of Sub Engineer Headworks for filling stone in trollies stone stacked at LMB and D/S LGB.

    - Another group of establishment in the supervision of Sub Engineer Main Line Upper will start tree launching. For this purpose trees will be collected / cut from thick plantation existing between left bank of Thal Canal and LMB. The trees will be moved through tractor / trollies laying in stand-by condition at junction point of LMB and LGB.

    - Administration of Maple Leaf Cement Factory stationed at just 8 – Km from Jinnah Barrage will be requested for logistic support i.e. providing stone or boulders at site.

    - At the same time, nearby available Government Contractor will be engaged to enhance the resources in shape of machinery, supply of requisite material and labour at site.

    - Labour of Kundian Sub Division and Other sister Divisions of Irrigation Department will be called on phone.

- Proclamation on vast scale will be made. Cable operators and T.V. channels will also be requested to facilitate the proclamation.

- Civil Administration and local Police will be requested to maintain law and order as well as to provide volunteer labour and other resources on request of Executive Engineer Kalabagh Headworks Division.

- Traffic on Mianwali Bannu road existing on right bank of Thal Canal between RD: 0+000 – 10+800 will be suspended and warning boards will be fixed at Kot Chandna and RD: 10+800 of Thal Canal to guide the traffic towards Kalabagh Railway Bridge.

    - ARRANGEMENTS AT SENSITIVE SITES

In all vulnerable reaches / sites such as RD: 0+000 to RD 1+000 and RD: 4+000 to 6+000 of LMB, gang of 10 beldars and one mate will be made available around the clock when river rises above low flood level. A gang of 10 beldars including 2 mates will remain vigilant at Junction point of LMB and LGB even in low flood till the end of flood season. A reasonable quantum of machinery will be kept ready at this place throughout the flood season to combat any emergency.

Plan showing route of flood water coming out of the breach is attached Annex: B

    - ESTABLISHMENT OF FLOOD FIGHTING CAMPS

            - Field Office at Jinnah Barrage.

            - RD: 5+000 of Left Marginal Bund.

    - DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

| Camp No | Location | Officer Name / Designation / Mobile No. | Official Name / Designation / Mobile No. |
| --- | --- | --- | --- |
| Camp No. 1 | Field Office Jinnah Barrage | Zia Ur Rehman / SDO Headworks / 0301-3335402 | M Sarfraz / SEN Hydraulic / 0300-6079014 |
| Camp No. 2 | RD 5+000 of Left Marginal Bund | Zia Ur Rehman / SDO Headworks / 0301-3335402 | M Sarfraz / SEN Hydraulic / 0300-6079014 |

    - DEPARTMENTAL MACHINERY AVAILABLE

Kalabagh Headworks Division possesses 02 number tractors with trolley & water boozer, 01 number sand bag filling machine, 01 number small crane mounted truck and 01 number mini truck. However, contractor will be hired through competitive bidding before the flood season for hiring of machinery at the vulnerable sites to counter any untoward incident.

    - MACHINERY AVAILABLE FROM PRIVATE SOURCE

Maple Leaf Cement Factory is located at about 8 – Km from Barrage. This private source will be requested in case of emergency to provide necessary machinery to combat an emergent situation.

    - FLOOD FIGHTING MATERIAL REQUIRED

    - FLOOD FIGHTING MATERIAL AVAILABLE

| S. No. | Name of Articles | Unit | Total Required Qty | Available Qty | Balance Qty | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Search Lights | No. | 5 | Nil | 5 | The quantity required as mentioned Herein will be purchased before onset of flood season 2025 |
| 2 | Three cell torches. | No. | 26 | Nil | 26 |  |
| 3 | Gunny bags | No. | 250 | Nil | 250 |  |
| 4 | Plastic bags | No. | 600 | Nil | 600 |  |
| 5 | Axes | No. | 10 | Nil | 10 |  |
| 6 | Kassies with handle | No. | 60 | Nil | 60 |  |
| 7 | Killas | No. | 25 | Nil | 25 |  |
| 8 | Steel Basket | No. | 20 | Nil | 20 |  |
| 9 | Manila rope | Rft. | 500 | Nil | 500 |  |
| 10 | Portable generator with accessories. | No. | 1 | Nil | - |  |
| 11 | Hammer | No. | 5 | 5 | - |  |
| 12 | Shouldary. | No. | 1 | - | 1 |  |
| 13 | Handle for Kassies. | No. | 10 | Nil | 10 |  |
| 14 | Boat. | No. | Nil |  |  |  |
| 15 | Charpai | No. | 1 | Nil | 1 |  |
| 16 | Plastic chairs | No. | 6 | Nil | 6 |  |
| 17 | Tables | No. | 1 | 1 | - |  |
| 18 | Briefing Board with stand | No. | Nil |  |  |  |
| 19 | Cell for Torches | Doz. | 20 | Nil | 20 |  |
| 20 | Energy Savers | No. | 24 | Nil | 24 |  |
| 21 | Funnels | No. | 2 | Nil | 2 |  |
| 22 | Life Jacket | No. | 16 | 16 | Nil |  |
| 23 | Rain Coats | No. | 10 | Nil | 10 |  |
| 24 | Switch / Plugs | No. | 4 | Nil | 4 |  |
| 25 | Umbrella | No. | 2 | Nil | 2 |  |
| 26 | Wires 7/29 | Meter. | 300 | Nil | 300 |  |
| 27 | Wooden Boards 10x12 | No. | 4 | Nil | 3 |  |
| 28 | Wooden Hammers | No. | 1 | Nil | 1 |  |
| 29 | Bulb Holder | No. | 24 | - | 24 |  |
| 30 | Solution Tape | No. | 10 | - | 10 |  |
| 31 | Gloves | No. | 50 | - | 50 |  |
| 32 | M. Oil | Liter. | 20 | Nil | 20 |  |
| 33 | Petrol | Liter. | 200 | Nil | 200 |  |
| 34 | Diesel | Liter. | 400 | Nil | 400 |  |
| 35 | Kerosene Oil | Liter. | 40 | Nil | 40 |  |

- EMERGENCY CONTINGENCY PLAN FOR SITE NO. 3: BREACH TO RIGHT MARGINAL BUND RD: 0 – 9+084.

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS

Route of water in case of failure of Left Marginal Bund is marked on the plan attached.

  - DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED

              - Kot Chandna.

              - Sumbal Wala.

              - Khudozai.

              - Manda Khel

  - STRATEGY AND ACTION TAKEN.

Right Marginal bund has dual function, Mianwali – Bannu road has been constructed on this marginal bund. In case of any mishap, traffic flow on Mianwali – Bannu road will be suspended. Therefore, its immediate restoration will be required and following measures will be implemented for this purpose:-

- Closing of breach by earth filling will be managed.

- Logistic support from Hydro Power Authorities will be requested.

- Inform all concerned immediately to get help:

- SHOs of Police Station Daudkhel & Kalabagh for Law and order.

- Civil Administration for law and order and support in obtaining the relevant materials that are sometimes not available due to various reasons.

- Traffic on Mianwali Bannu road existing on right bank of Thal Canal between RD: 0+000 – 10+800 will be suspended and warning boards will be fixed at Kot Chandna and RD. 10+800 of Thal Canal to guide the traffic towards Kalabagh Railway Bridge.

    - ARRANGEMENTS AT SENSITIVE SITES

In all vulnerable reaches / sites, gang of 10 beldars and one mate will be made available around the clock when river rises above low flood level. A gang of 10 beldars including 2 mates will remain vigilant at Junction point of LMB and LGB even in low flood till the end of flood season. A reasonable quantum of machinery will be kept ready at this place throughout the flood season to combat any emergency.

Plan showing route of flood water coming out of the breach is attached Annex: C.

    - ESTABLISHMENT OF FLOOD FIGHTING CAMPS

- Field Office at Jinnah Barrage.

- RD: 6+700 to 8+700 of Right Marginal Bund.

    - DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

| Camp No | Location | Officer Name / Designation / Mobile No. | Official Name / Designation / Mobile No. |
| --- | --- | --- | --- |
| Camp No. 1 | Field Office Jinnah Barrage | Zia Ur Rehman / SDO Headworks / 0301-3335402 | M Sarfraz / SEN Hydraulic / 0300-6079014 |
| Camp No. 2 | RD 6+700 to 8+700 of Right Marginal Bund | Zia Ur Rehman / SDO Headworks / 0301-3335402 | M Sarfraz / SEN Hydraulic / 0300-6079014 |

    - DEPARTMENTAL MACHINERY AVAILABLE

Kalabagh Headworks Division possesses 02 number tractors with trolley & water boozer, 01 number sand bag filling machine, 01 number small crane mounted truck and 01 number mini truck. However, contractor will be hired through competitive bidding before the flood season for hiring of machinery at the vulnerable sites to counter any untoward incident.

    - MACHINERY AVAILABLE FROM PRIVATE SOURCE

Maple Leaf Cement Factory is located at about 8 – Km from Barrage. This private source will be requested in case of emergency to provide necessary machinery to combat an emergent situation.

    - FLOOD FIGHTING MATERIAL REQUIRED

    - FLOOD FIGHTING MATERIAL AVAILABLE

| S. No. | Name of Articles | Unit | Total Required Qty | Available Qty | Balance Qty | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Search Lights | No. | 5 | Nil | 5 | The quantity required as mentioned Herein will be purchased before onset of flood season 2025 |
| 2 | Three cell torches. | No. | 10 | Nil | 10 |  |
| 3 | Gunny bags | No. | 200 | Nil | 200 |  |
| 4 | Plastic bags | No. | 600 | Nil | 600 |  |
| 5 | Axes. | No. | 10 | 5 | 5 |  |
| 6 | Kassies with handle. | No. | 15 | 10 | 5 |  |
| 7 | Killas. | No. | 25 | 15 | 10 |  |
| 8 | Steel Basket. | No. | 20 | 5 | 15 |  |
| 9 | Manila rope | Rft. | 500 | Nil | 500 |  |
| 10 | Portable generator with accessories. | No. | 1 | Nil | - |  |
| 11 | Hammer. | No. | 2 | 5 | - |  |
| 12 | Shouldary. | No. | Nil |  |  |  |
| 13 | Handle for Kassies. | No. | 10 | Nil | 10 |  |
| 14 | Boat. | No. | Nil |  |  |  |
| 15 | Tractor with trolley | No. | Nil |  |  |  |
| 16 | Excavator | No. | Nil |  |  |  |
| 17 | Charpai | No. | 1 | 1 | - |  |
| 18 | Plastic chairs | No. | 5 | 2 | 3 |  |
| 19 | Tables | No. | Nil |  |  |  |
| 20 | Briefing Board with stand | No. | Nil |  |  |  |
| 21 | Life Jacket | No. | 5 | 5 | - |  |
| 22 | Wooden Boards 10x12 | No. | 4 | 3 | 3 |  |

- EMERGENCY CONTINGENCY PLAN FOR SITE NO. 4: CANAL COLONY PROTECTION BUND RD: 0 – 11+730

  - PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS

Route of water in case of failure of Left Marginal Bund is marked on the plan attached.

  - DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED

- Canal Colony Daudkhel Town.

- Daudkhel Town.

- Thathi.

  - STRATEGY AND ACTION TAKEN.

As experienced during super flood 2010, breach to Canal Colony Protection Bund created havoc in the area. The vast area of Daudkhel town was inundated and many houses were destroyed. Moreover, Daudkhel Lift Irrigation Scheme was badly damaged. Mechanical workshop was badly hit by the breach RD 4+000 to 5+000. Keeping in view of above scenario immediate arrangements will be made for closure of breach in following manner:

- Labour already stationed at junction point of LGB and LMB and at RD: 7+000 of LMB will immediately move under the supervision of Sub Engineer Headworks for filling earth in bags and brought to site through trollies. Killa / Bamboo bushing will also be carried out.

- Another group of establishment in the supervision of Sub Engineer Main Line Upper will start tree launching. For this purpose, trees will be collected / cut from thick plantation existing between left bank of Thal Canal and LMB. The trees will be moved through tractor / trollies stationed at junction point of LMB and LGB.

- At the same time some, nearby available Government Contractor will be engaged to enhance the resources in shape of machinery, supply of requisite material and labour at site. Logistic support from Hydro Power Authorities will also be requested.

- Labour of Kundian Sub Division and Other Sister Divisions of Irrigation Department will be called on phone.

- Proclamation on vast scale will be made. Cable operators and T.V. channels will also be requested to facilitate the proclamation.

    - ARRANGEMENTS AT SENSITIVE SITES

In all vulnerable reaches / sites such as RD: 0+000 to RD 11+730 of Colony Protection Bund, gang of 10 beldars and one mate will be made available around the clock when river rises above low flood level. A gang of 10 beldars including 2 mates will remain vigilant at Junction point of LMB and LGB even in low flood till the end of flood season. A reasonable quantum of machinery will be kept ready at this place throughout the flood season to combat any emergency.

Plan showing route of flood water coming out of the breach is attached Annex: D.

    - ESTABLISHMENT OF FLOOD FIGHTING CAMPS

- Field Office at Jinnah Barrage.

- RD: 6+500 of Canal Colony Protection Bund.

    - DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

| Camp No | Location | Officer Name / Designation / Mobile No. | Official Name / Designation / Mobile No. |
| --- | --- | --- | --- |
| Camp No. 1 | Field Office Jinnah Barrage | Zia Ur Rehman / SDO Headworks / 0301-3335402 | Hassan Raza / SEN Canal Colony / 0349-6586406 |
| Camp No. 2 | RD 6+700 to 8+700 of Right Marginal Bund | Zia Ur Rehman / SDO Headworks / 0301-3335402 | Hassan Raza / SEN Canal Colony / 0349-6586406 |

    - DEPARTMENTAL MACHINERY AVAILABLE

Kalabagh Headworks Division possesses 02 number tractors with trolley & water boozer, 01 number sand bag filling machine, 01 number small crane mounted truck and 01 number mini truck. However, contractor will be hired through competitive bidding before the flood season for hiring of machinery at the vulnerable sites to counter any untoward incident.

    - MACHINERY AVAILABLE FROM PRIVATE SOURCE

Maple Leaf Cement Factory is located at about 8 – Km from Barrage. This private source will be requested in case of emergency to provide necessary machinery to combat an emergent situation.

    - FLOOD FIGHTING MATERIAL REQUIRED

    - FLOOD FIGHTING MATERIAL AVAILABLE

| S. No. | Name of Articles | Unit | Total Required Qty | Available Qty | Balance Qty | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Search Lights | No. | 2 | Nil | 2 | The quantity required as mentioned Herein will be purchased before onset of flood season 2025 |
| 2 | Three cell torches. | No. | 10 | Nil | 10 |  |
| 3 | Gunny bags | No. | 200 | 50 | 150 |  |
| 4 | Plastic bags | No. | 600 | Nil | 600 |  |
| 5 | Axes. | No. | 10 | 5 | 5 |  |
| 6 | Kassies with handle. | No. | 15 | 10 | 5 |  |
| 7 | Killas. | No. | 20 | Nil | 20 |  |
| 8 | Steel Basket. | No. | 20 | Nil | 20 |  |
| 9 | Manila rope | Rft. | 500 | Nil | 500 |  |
| 10 | Portable generator with accessories. | No. | 1 | Nil | 1 |  |
| 11 | Hammer. | No. | 2 | Nil | 2 |  |
| 12 | Shouldary. | No. | Nil |  |  |  |
| 13 | Handle for Kassies. | No. | 10 | Nil | 10 |  |
| 14 | Boat. | No. | Nil |  |  |  |
| 15 | Charpai | No. | 1 | Nil | 1 |  |
| 16 | Plastic chairs | No. | 6 | 2 | 4 |  |
| 17 | Tables | No. | 2 | Nil | 2 |  |
| 18 | Briefing Board with stand | No. | Nil |  |  |  |
| 19 | Cell for Torches | Doz. | 20 | Nil | 20 |  |
| 20 | Energy Savers | No. | 20 | Nil | 20 |  |
| 21 | Funnels | No. | 2 | Nil | 2 |  |
| 22 | Life Jacket | No. | 4 | Nil | 4 |  |
| 23 | Rain Coats | No. | 10 | Nil | 10 |  |
| 24 | Switch / Plugs | No. | 4 | Nil | 4 |  |
| 25 | Umbrella | No. | 3 | Nil | 3 |  |
| 26 | Wires 7/29 | Meter. | 300 | Nil | 300 |  |
| 27 | Wooden Boards 10x12 | No. | 4 | Nil | 4 |  |
| 28 | Wooden Hammers | No. | 1 | Nil | 1 |  |
| 29 | Bulb Holder | No. | 24 | - | 24 |  |
| 30 | Solution Tape | No. | 10 | - | 10 |  |
| 31 | Gloves | No. | 50 | - | 50 |  |
| 32 | M. Oil | Liter. | 20 | Nil | 20 |  |
| 33 | Petrol | Liter. | 200 | Nil | 200 |  |
| 34 | Diesel | Liter. | 400 | Nil | 400 |  |
| 35 | Kerosene Oil | Liter. | 40 | Nil | 40 |  |

- EMERGENCY CONTINGENCY PLAN FOR SITE NO. 5: HILL TORRENTS OF TEHSIL ISAKHEL AND MIANWALI.

13.1	PLAN SHOWING LOCATION OF HILL TORRENTS OF TEHSIL ISAKHEL AND MIANWALI.

The flood embankments constructed on hill torrents are plan to breaches in case of flash floods.

13.2	DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED

All the villages adjacent to flood embankments on hill torrents are likely to be affected.

  - STRATEGY AND ACTION TAKEN.

- There are total 07 No. Hill Torrents in Tehsil Isakhel & Mianwali.

In case of emergency 05. No. work charge Beldar will be Hired, those will work under super vision of Sub Engineer Isa Khel. Those will control the situation with help of earth fill bags and killa bushing.

- In case of serious, situation, Department machinery will be engaged. Also, machinery will be hired through contractor to fight the emergency.

- At the same time some, nearby available Government Contractor will be engaged to enhance the resources in shape of machinery, supply of requisite material and labour at site.

- Proclamation on vast scale will be made. Cable operators and T.V. channels will also be requested to facilitate the proclamation.

13.3.1	ARRANGEMENTS AT SENSITIVE SITES

In all vulnerable reaches / sites work charge establishment will be hired as per field requirement, since no regular establishment is available for hill torrents.

13.3.2	ESTABLISHMENT OF FLOOD FIGHTING CAMPS

- Field Office at Jinnah Barrage.

- Camp at Qamar Mashani near Mianwali- Bannu Road.

13.3.3	DUTIES OF OFFICERS / OFFICIALS AND THEIR CAMP SITES

| Camp No | Location | Officer Name / Designation / Mobile No. | Official Name / Designation / Mobile No. |
| --- | --- | --- | --- |
| Camp No. 1 | Field Office Jinnah Barrage | Zia Ur Rehman / SDO Headworks / 0301-3335402 | Hassan Raza / SEN Canal Colony / 0349-6586406 |
| Camp No. 2 | Camp at Qamar Mashani near Mianwali- Bannu Road | Kashif Rizwan / SDO Isakhel / 0333-6635103 | Siddique Akbar / SEN Isakhel / 0344-1788877 |

    - DEPARTMENTAL MACHINERY AVAILABLE

Kalabagh Headworks Division possesses 02 number tractors with trolley & water boozer, 01 number sand bag filling machine, 01 number small crane mounted truck and 01 number mini truck. However, contractor will be hired through competitive bidding before the flood season for hiring of machinery at the vulnerable sites to counter any untoward incident.

13.3.5	MACHINERY AVAILABLE FROM PRIVATE SOURCE

Maple Leaf Cement Factory is located at about 8 – Km from Barrage. This private source will be requested in case of emergency to provide necessary machinery to combat an emergent situation.

    - FLOOD FIGHTING MATERIAL REQUIRED

| S. No. | Name of Articles | Unit | Total Required Qty | Available Qty | Balance Qty | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Search Lights | No. | 2 | 1 | 1 | The quantity required as mentioned Herein will be purchased before onset of flood season 2025 |
| 2 | Three cell torches. | No. | 10 | Nil | 10 |  |
| 3 | Gunny bags | No. | 5000 | Nil | 5000 |  |
| 4 | Plastic bags | No. | 6000 | Nil | 6000 |  |
| 5 | Axes. | No. | 2 | Nil | 2 |  |
| 6 | Kassies with handle. | No. | 15 | 2 | 13 |  |
| 7 | Killas. | No. | 200 | Nil | 200 |  |
| 8 | Steel Basket. | No. | 20 | Nil | 20 |  |
| 9 | Manila rope | Rft. | 500 | Nil | 500 |  |
| 10 | Portable generator with accessories. | No. | 1 | Nil | 1 |  |
| 11 | Hammer. | No. | 2 | Nil | 2 |  |
| 12 | Shouldary. | No. | 2 | Nil | 2 |  |
| 13 | Boat. | No. |  |  |  |  |
| 14 | Charpai | No. | 5 | 1 | 4 |  |
| 15 | Plastic chairs | No. | 12 | 4 | 9 |  |
| 16 | Tables | No. | 2 | 1 | 1 |  |
| 17 | Briefing Board with stand | No. | Nil |  |  |  |
| 18 | Cell for Torches | Doz. | 24 | Nil | 24 |  |
| 19 | Energy Savers | No. | 12 | Nil | 12 |  |
| 20 | Funnels | No. | 1 | Nil | 1 |  |
| 21 | Life Jacket | No. | 5 | Nil | 5 |  |
| 22 | Rain Coats | No. | 10 | Nil | 10 |  |
| 23 | Switch / Plugs | No. | 4 | Nil | 4 |  |
| 24 | Umbrella | No. | 10 | 1 | 9 |  |
| 25 | Wires 7/29 | Meter. | 300 | Nil | 300 |  |
| 26 | Wooden Boards 10x12 | No. | 2 | Nil | 2 |  |
| 27 | Wooden Hammers | No. | 1 | Nil | 1 |  |
| 28 | Bulb Holder | No. | 6 | Nil | 6 |  |
| 29 | Solution Tape | No. | 10 | Nil | 10 |  |
| 30 | Gloves | No. | 50 | Nil | 50 |  |
| 31 | M. Oil | Liter. | 20 | Nil | 20 |  |
| 32 | Petrol | Liter. | 200 | Nil | 200 |  |
| 33 | Diesel | Liter. | 400 | Nil | 400 |  |
| 34 | Kerosene Oil | Liter. | 40 | Nil | 40 |  |

13.3.7	FLOOD FIGHTING MATERIAL AVAILABLE

As above

  - DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRIC, SUI GAS TELEPHONE INSTALLATIONS, ROAD NETWORK, OTHER BUILDINGS, CANALS AND DRAINAGE NETWORK

  - IN CASE OF FAILURE OF LEFT GUIDE BUND AND LEFT MARGINAL BUND FOLLOWING INFRASTRUCTURES WILL BE EFFECTED.

- Canal Colony Daudkhel.

- Mianwali - Bannu Road up to Jinnah Barrage.

- Mechanical workshop Daudkhel.

- Thal Canal Main Line Upper RD: 0+000 to 10+000.

- Daudkhel Lift Irrigation Scheme i/c Pump Houses etc.

- Bokhara Disty.

- Right Paikhel Canal.

- Rural Health Centre Thathi.

- Water Supply Scheme Daudkhel.

- Water Supply Scheme Thathi.

  - IN CASE OF OPERATION OF BREACHING SECTION ON RIGHT MARGINAL BUND FOLLOWING INFRASTRUCTURE WILL BE EFFECTED.

- Labor colony of Jinnah Hydro Power Project.

- Mianwali - Bannu road near Kot Chandna.

  - IN CASE OF FAILURE OF CANAL COLONY PROTECTION BUND FOLLOWING INFRASTRUCTURE WILL BE EFFECTED.

- Canal Colony Daudkhel.

- Mechanical workshop Daudkhel.

- Daudkhel Lift Irrigation Scheme i/c Pump Houses etc.

- Water Supply Scheme Daudkhel.

- Mianwali – Bannu road and internal roads of Daudkhel.

CHAPTER-14

ACTION PLAN

  - RESHUFFLING / RECOUPING PLAN OF RESERVE STONE DEPARTMENTALLY.

Sanction limit of reserve stone stock at Jinnah Barrage is 7.90 Lac Cft: In the financial year 2013, PMO Barrages purchased and provided the sanctioned limit of reserve stone at Jinnah Barrage. The Contractor M/S DESCON disturbed the stacks of stone so the reserve stone is not measureable. Reserve stock purchased in 2017-18 and now available quantity is 7.524 Lac Cft. Structure wise detail is given in Chapter No. 7 at 7.3. Sufficient quantity of stone is available and no shuffling of stone is required.

  - DETAIL OF INLET AND OUTLET CROSSING ALONG WITH CLOSING METHODOLOGY.

There is no such crossing in the jurisdiction of Kalabagh Headworks Division Daudkhel.

  - DEPLOYMENT OF MACHINERY (MEDIUM TO HIGH FLOOD).

| Name of Structure | Length in Mile | Vulnerable Reach | Camp Location | Site In-charge by name & Cell No. | Deployment of Machinery |  |  |  | Availability of Stone (Lac Cft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Excavator | Loader | Trolleys / Dumpers | Tractor with front blade |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| LGB RD: 0+000 to 4+099 | 0.82 | RD: 1+000 to 3+000 of LGB | RD: 1+000 of LGB | M Sarfraz Sub Engineer Headworks Section 03006079014 | 1 | -- | 2 | 1 | 2.154 Lac Cft (RD.0+800 to 4+099) |
| RGB RD: 0+000 to 3+718 | 0.70 | RD: 1+800 to 1+700 of RGB | RD: 0+000 of RGB | M Sarfraz Sub Engineer Hydraulic Section 03006079014 | 1 | -- | 2 | 1 | -- |
| LMB RD: 0+000 to 7+410 | 1.48 | RD: 1+000 to 3+000 of LMB | RD: 5+000 of LMB | M Sarfraz Sub Engineer Workshop Section 03006079014 | 1 | -- | 3 | 1 | 1.734 Lac Cft (RD.0+000 to 7+740) |
| Colony Protection Bund RD 0+000 to RD 11+730 | 2.35 | RD 6+400 to 7+000 of Colony Protection Bund | RD 6+500 | Hassan Raza Sub Engineer Canal Colony 0349-6586406 | 1 | - | 2 | 1 |  |
| Total Machinery |  |  |  |  | 4 | -- | 9 | 4 |  |

Payment for machinery and work charge employee subject to verification by TPM /Consultant.

  - DEPLOYMENT OF MACHINERY (HIGH TO VERY HIGH FLOOD).

| Name of Structure | Length in Mile | Vulnerable Reach | Camp Location | Site In-charge by name & Cell No. | Deployment of Machinery |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Excavator | Loader | Trolleys / Dumpers | Tractor with front blade | Availability of Stone (Lac Cft) |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  |  |
| LGB RD: 0+000 to 4+099 | 0.82 | RD: 1+000 to 3+000 of LGB | RD: 1+000 of LGB | M Sarfraz Sub Engineer Headworks Section 03006079014 | 1 | 1 | 2 | 1 | 2.154 Lac Cft (RD.0+800 to 4+099) |  |  |
| RGB RD: 0+000 to 3+718 | 0.70 | RD: 1+800 to 1+700 of RGB | RD: 0+000 of RGB | M Sarfraz Sub Engineer Hydraulic Section 03006079014 | 1 | -- | 2 | 1 | -- |  |  |
| LMB RD: 0+000 to 7+410 | 1.48 | RD: 1+000 to 3+000 of LMB | RD: 5+000 of LMB | M Sarfraz Sub Engineer Workshop Section 03006079014 | 1 | -- | 3 | 1 | 1.734 Lac Cft (RD.0+000 to 7+740) |  |  |
| Colony Protection Bund RD 0+000 to RD 11+730 | 2.35 | RD 6+400 to 7+000 of Colony Protection Bund | RD 6+500 | Hassan Raza Sub Engineer Canal Colony 0349-6586406 | 1 | - | 2 | 1 |  |  |  |
| Total Machinery |  |  |  |  | 4 | 1 | 9 | 4 |  |  |  |

Payment for machinery and work charge employee subject to verification by TPM /Consultant

  - DEPLOYMENT OF MACHINERY (VERY HIGH TO EXCEPTIONALLY FLOOD).

| Name of Structure | Length in Mile | Vulnerable Reach | Camp Location | Site In-charge by name & Cell No. | Deployment of Machinery |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Excavator | Loader | Trolleys / Dumpers | Tractor with front blade | Availability of Stone (Lac Cft) |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| LGB RD: 0+000 to 4+099 | 0.82 | RD: 1+000 to 3+000 of LGB | RD: 1+000 of LGB | M Sarfraz Sub Engineer Headworks Section 03006079014 | 2 | 1 | 6 | 2 | 2.154 Lac Cft (RD: 0+800 to 4+099) |
| RGB RD: 0+000 to 3+718 | 0.70 | RD: 1+800 to 1+700 of RGB | RD: 0+000 of RGB | M Sarfraz Sub Engineer Hydraulic Section 03006079014 | 1 | -- | 2 | 1 | -- |
| RMB RD: 0+000 to 9+084 | 1.82 | RD: 6+700 to 8+700 of RMB | RD: 8+000 of LMB | M Sarfraz Sub Engineer Hydraulic Section 03006079014 | 2 | -- | 6 | 2 | -- |
| LMB RD: 0+000 to 7+410 | 1.48 | RD: 1+000 to 3+000 of LMB | RD: 5+000 of LMB | M Sarfraz Sub Engineer Workshop Section 03006079014 | 1 | -- | 4 | 1 | 1.734 Lac Cft (RD: 0+000 to 7+740) |
| Colony Protection Bund RD 0+000 to RD 11+730 | 2.35 | RD 6+400 to 7+000 of Colony Protection Bund | RD 6+500 | Hassan Raza Sub Engineer Canal Colony 0349-6586406 | 1 | - | 4 | 1 |  |
| Total Machinery |  |  |  |  | 7 | 1 | 22 | 7 |  |

Payment for machinery and work charge employee subject to verification by TPM /Consultant.

  - POLICE DEPLOYMENT PLAN.

| S. No. | Name of Site. | Concerned Police Station & District. | Police personal to be deployed. |
| --- | --- | --- | --- |
| 1 | Breaching Section of Right Marginal Bund of Jinnah Barrage. | Kalabagh District Mianwali | Inspector = 1 No. SI = 1 No. ASI = 2 No. Constable = 12 No. |
| 2 | Jinnah Barrage. | Kalabagh District Mianwali | Inspector = 1 No. SI = 1 No. ASI = 2 No. Constable = 12 No. |
| 3 | Vulnerable sites used for illegal cuts during flood. | There is no such site in the jurisdiction of Kalabagh Headworks Division Daudkhel. |  |
| 4 | Traffic Police if required for smooth traffic flow at Jinnah Barrage during flood. | ASI = 1 No. Head Constable = 1 No. ASI = 2 No. Constable = 12 No. |  |

  - DETAIL OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG AND 100 KG.

| S. No. | Capacity of bags. | Required Quantity | Available Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Synthetic bags 500 Kg. | 2000 No. | - | This will be procured before ensuring flood season 2025. |
| 2 | Synthetic bags 1000 Kg. | 1500 No. | - | This will be procured before ensuring flood season 2025. |

  - DETAIL OF POLYTHENE SHEETS OF BLACK COLOR TO PROTECT UPSTREAM SLOPE AGAINST WAVE ACTION AND TO CONTROL SEEPAGE THROUGH EMBANKMENTS

| S. No. | Description. | Required Quantity | Available Quantity | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Polythene Sheets of Black Color. | 25000 Sft. | - | This will be procured before ensuring flood season 2025. |

CHAPTER-15

BACK UP DIVISIONS (IN CASE OF BREACH)

In case of an emergency, Mianwali Canal Division Mianwali of Thal Canal Circle Mianwali will be back-up Division.

| Executive Engineer Kalabagh Headworks Division Daudkhel | Superintending Engineer Thal Canal Circle Mianwali | Chief Engineer Sargodha Irrigation Zone Sargodha |
| --- | --- | --- |