PART-A

CHAPTER-1

SALIENT FEATURES OF THE KHANSAR CANAL DIVISION KALLURKOT

1.1	LOCATION.

Khansar Canal Division is located at Tehsil Kallurkot.  Divisional Office is at 25 miles Down Stream of Chashma Barrage along the left bank of River Indus. The jurisdiction of Khansar Canal Division starts from Tehsil Piplan of District Mianwali & other structured Tehsil Kallurkot, Darya khan & Bhakkar of District Bhakkar.

1.2	GENERAL DESCRIPTION.

Khansar Canal Division comprises of three Sub Divisions namely Kallurkot, Bakhat Canal Sub Division and River Survey Sub Division, Kallurkot. Executive Engineer Khansar Canal Division Kallurkot is the incharge of the Division. His primary duty is equitable distribution of canal water and safe running of irrigation channels in addition to proper watch and ward of flood structures built for safety of Kallurkot and Darya Khan City. The jurisdiction of Khansar Canal Division starts from RD 131+000 of Thal Canal Main Line Lower and extends up to RD 291+000 with its off taking distributaries and minors. The length of channels is 279.51 Miles. The gross commanded area of Khansar Canal Division is 313876 acres. The area of Thal Canal Main Line lower falling in the jurisdiction of Khansar Canal Division is spread over in two Districts as under.

| Name of District | G.C.A (Acres) | C.C.A (Acres) |
| --- | --- | --- |
| Mianwali. | 4530 | 4495 |
| Bhakkar | 313876 | 263387 |
| Total | 318401 | 267882 |

XEN KHS

1.3	ADMINISTRATIVE SETUP

XEN KHS

CHAPTER-2

FLOOD PROTECTION AND RIVER TRAINING WORKS

2.1	DESIGN PARAMETERS OF TRAINING WORKS

| Sr. No. | Name of Flood Structures | Length (Feet) | Mile | Top width | Side Slope |  | Free board | Condition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Feet | River side | Country side | Feet |  |
| 1 | J-Head spur Saggu Shumali at Kallurkot | 3491 | 0.70 | 30 | 3:1 | 2:1 | 6 | Satisfactory |
| 2 | Guide spur No.01 | 955 | 0.19 | 30 | 3:1 | 2:1 | 6 | Satisfactory |
| 3 | Stud No.1 | 380 | 0.08 | 30 | 3.:1 | 2:1 | 6 | Satisfactory |
| 4 | Stud No. 2 | 380 | 0.08 | 30 | 3:1 | 2:1 | 6 | Satisfactory |
| 5 | Guide spur No.2 | 865 | 0.17 | 30 | 3:1 | 2:1 | 6 | Satisfactory |
| 6 | Guide wall at Kallurkot | 8000 | 1.60 | 25 | 3:1 | 2:1 | 6 | Satisfactory |
| 7 | J-Head spur Village Ravi | 1983 | 0.40 | 30 | 3:1 | 2:1 | 6 | Satisfactory |
| 8 | J-Head spur Daggar Malana | 1993 | 0.40 | 30 | 3:1 | 2:1 | 6 | Satisfactory |
| 9 | Sloping stud u/s Daggar Noon Village/L River Indus | 745 | 0.15 | 30 | 3:1 | 2:1 | 6 | Satisfactory |
| 10 | Inverted hockey stud near Daggar Noon Village/L River Indus. | 1109 | 0.22 | 30 | 3:1 | 2:1 | 6 | Satisfactory |

XEN KHS

2.2	LOCATION MAP

(Attached)

CHAPTER-3

BRIEF HISTORY OF PAST FLOOD EVENTS

The Indus River originates from the Tibetan Plateau (China) and drains out into the Arabian Sea while passing through India, Afghanistan and Pakistan. It is the main river of Indus Basin with length 3180 km. It is a snow cum rain fed river. The typical pattern of flow is advent in summer season, when snow starts melting along with rain falls and discharge goes on rising In monsoon season, rain becomes the main reason for high discharges and floods After the subsidence of flows, a changed and new pattern of river bed behavior exposed which causes development of new creeks and bellas in river bed.

River Indus enters into the boundary of Bhakkar District near saggu shumali at Kallurkot. A bridge is recommend structure which link Province of Punjab with Khyber Pakhtunkhawa through tehsil Kallurkot.

The flood infrastructure of Khansar Canal Division consists of J-Head spur Saggu Shumali, Stud No.1, Guide spur No.01, Stud No. 2, Guide spur No.2, Guide wall at Kallurkot, J-Head spur Village Ravi, J-Head spur Daggar Malana, Sloping stud u/s Daggar Noon Village/L River Indus, Inverted hockey stud near Daggar Noon Village/L River Indus were constructed in 2005 and is located about 4 km downstream Kallurkot City in the vicinity of Daggar Malana and Daggar Noon to ensure the safely Villages, Abadies, Agricultural Land, Road and Railway Line. These structures performed successfully as are located right in the River creek.

River Indus D/S Chashma Barrage passed exceptionally high flood in 2010 which passed safely from the jurisdiction of Khansar Canal Division Kallurkot, However some minor damages of River Training Works in Kallurkot Sub Division were reported.

XEN KHS

CHAPTER-4

DESIGN DATA, HISTORIC PEAK FLOOD DATA AND PREVIOUS FIVE YEARS FLOOD DATA OF HEAD WORKS / BARRAGES AND OTHER CONTROL POINTS

4.1	FLOOD LIMITS

Since there is no Head` works / Barrage in the jurisdiction of Khansar Canal Division. Therefore this Division has to come active with respect to discharge downstream Chashma Barrage River Indus. The terminology of different kind of flood is low, Medium, High, Very high and exceptionally high will be taken as such in the light of message received from Chashma Barrage.

MENTIONED LIMIT TABLE

| Sr. No. | Site | Up to Low Flood | Up to Medium Flood | Up to High Flood | Up to Very High Flood | Exceptionally High Flood |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | River Indus downstream Chashma Barrage | 250000 Cs | 375000 Cs | 500000 Cs | 650000 Cs | 800000 Cs |

4.2	TIME LAG OF FLOODS

| Sr. No. | Site | Mile | Hours |
| --- | --- | --- | --- |
| 1 | Distance from Attock to Jinnah Barrage Kalabagh | 106 | 48 |
| 2 | Distance from Jinnah Barrage Kalabagh to Chashma Barrage | 41 | 18 |
| 3 | Distance from Chashma Barrage to Kallurkot | 25 | 7.30 |

4.3	HIGHEST FLOODS

| YEAR | DATE | DISCHARGE |
| --- | --- | --- |
| 1976 | 1976 | 7,87,000 Cs |
| 1998 | 17-07-1998 | 5,02,948 Cs |
| 2005 | 18-07-2005 | 5,15,581 Cs |
| 2010 | 01-08-2010 | 10,38,873Cs |
| 2013 | 15-08-2013 | 5,59,935 Cs |
| 2015 | 03-08-2016 | 6,18,626 Cs |
| 2016 | 06-07-2016 | 3,74,610 Cs |
| 2017 | 05-08-2017 | 4,46,361 Cs |
| 2019 | 11-08-2019 | 3,63,783 Cs |
| 2020 | 03-09-2020 | 4,91,184 Cs |
| 2021 | 02-08-2021 | 3,44,907 Cs |
| 2022 | 28.08.2022 | 5,23,937 Cs |
| 2023 | 25.07.2023 | 4,15,651 Cs |
| 2024 | 09.08.2024 | 4,17,763 Cs |

XENKHS

4.4	PEAK DISCHARGES (Last 12 Years)

| YEAR | DATE | DISCHARGE |
| --- | --- | --- |
| 2010 | 01-08-2010 | 10,38,873 Cs |
| 2011 | 28-07-2011 | 3,49666 Cs |
| 2012 | 06-08-2012 | 284764 Cs |
| 2013 | 15-08-2013 | 5,59,935Cs |
| 2014 | 18.08.2015 | 2,57,632Cs |
| 2015 | 03-08-2015 | 6,18,626 Cs |
| 2016 | 06-07-2016 | 3,74,610 Cs |
| 2017 | 05-08-2017 | 4,46,361 Cs |
| 2018 | 16-08-2018 | 2,86,554 Cs |
| 2019 | 11-08-2019 | 3,63,783 Cs |
| 2020 | 03.09.2020 | 4,91,184 Cs |
| 2021 | 02-08-2021 | 3,44,907 Cs |
| 2022 | 28.08.2022 | 5,23,937 Cs |
| 2023 | 25.07.2023 | 4,15,651 Cs |
| 2024 | 09.08.2024 | 4,17,763 Cs |

XEN KHS

CHAPTER-5

FLOOD FIGHTING STRATEGY

All precautionary measures will be adopted to cope with any eventuality during flood 2025, Flood fighting strategy would be adopted for the safe passage of flow.

In low flood, the temporary watching huts will be established and flood fighting material will be shifted. Watching establishment employed for this purpose will remain busy under the supervision of concerned Sub Engineer.

When river water enters in to medium stage, it will start spreading on adjoining area touching toe of Bunds. Then it becomes necessary to be vigilant, watching of Bunds and deploying of machinery will be started to avoid any leakage from bunds. Sub Engineers will shift their camps to site as per duty rosters. The Telegraph office of Khansar Canal Division Kallurkot will act as flood information center and flood situation will be conveyed to Civil Administration and flood warning centers. SDO will keep the Executive Engineer apprised about latest development regarding flood situation.

When river water enters in high flood range and above, the Sub Divisional Officers Kallurkot, River Survey Kallurkot and Bakhat shall also shift their camps at vulnerable sites as planned in the duty roaster. Executive Engineer will keep liaison with Civil Administration, Army & Public representatives.

XEN KHS

CHAPTER-6

FLOOD DAMAGES / RESTORATION WORKS

INESCAPABLE WORKS TO BE CARRIED BEFORE FLOOD 2025.

| Sr. No. | Name of Work | Amount (In Million) |
| --- | --- | --- |
| 1 | Filling Rain Cutts and Gharas of River Survey Sub Division, Kallurkot. | 1.00 |

XEN KHS

CHAPTER-7

FLOOD FIGHTING / WATCHING ARRANGEMENTS

7.1	PRE FLOOD ARRANGEMENTS

Flood season starts from 15th June every year and watching establishment as per detail will be engaged during flood season for watching operation and flood fighting activities on monthly basis from 15-06-2025 to 15-10-2025. However, in case of very high flood or in emergency, strength of watching establishment will be increased as per site condition and requirements as provided in duty roaster.

| Sr. No. | Item | # Scale |
| --- | --- | --- |
| 1 | Lamps Hurricane | 1 per beldar, 1 per mate and 20% extra of total only for the night shift |
| 2 | Wicks | 9 inch per lamp |
| 3 | Chimneys | 1 spare for each lamp |
| 4 | Burners & caps | 1 spare for each lamp |
| 5 | Torches – 3 cell | 1 for Executive Engineer, 1 for Sub Divisional Officer, 1 for Sub Engineer 1 one spare |
| 6 | Cells for Torches | 1 full and six spare sets |
| 7 | Patromix lamp | At depressions, old breach sites and dangerous sites each lamp with 2 spare chimneys, 2 nozzles, 2 funnels fitted with wire gauze |
| 8 | Fuel for lighting fire | To be collected by beldar establishment |
| 9 | Kerosine Oil | As per practice 1/3 bottle per lantern per night and one bottle per patromax per night (22 nights per month) |
| 10 | Match Boxes | 1 match box per month |
| 11 | Spirit | 1 bottle per patromax per month |
| 12 | Funnels | 2 No. per watching hut |
| 13 | Oil Extractor (Ordy) | 2 No. per watching hut |
| 14 | Spirit cans | 1 per patromax |
| 15 | Larmal 50 feet long | 50 per watching hut |
| 16 | Mallets wooden | 5 mallets per watching hut |
| 17 | Tranger wire | 1 roll of 14 BSW gauge at each bund |
| 18 | Gunny bags | 100 bags per mile |
| 19 | Axes | 2 No per mile |
| 20 | Kassies with handles | 1 for each beldar + 10% (for one shift only) |
| 21 | Hammer 2.5 lbs | 1 per mile |
| 22 | Hammer 10 lb | 2 per mile |
| 23 | Buckets (12 inch size) | 5 No. per mile |
| 24 | Sledge hammer 14 lbs | 1 per mile |
| 25 | Bhan (Cow Dung) in ECbags | 20 bags per mile for dangerous portions |
| 26 | Sand | Collection of 200 cft. Every mile for dangerous portions |
| 27 | Stacks (6-8 feet) long | 100 stacks per mile |
| 28 | Khajji mats 3’*4’ | 50 Nos per mile |
| 29 | Baskets | 1 toot/2 Pilchi baskets per beldar of shift twice a month |
| 30 | Sutli | 1 lb per 100 bags |
| 31 | Needles | 4 Nos per beldar huts |
| 32 | Kassi Handles | 50% of the no. of kassies as spare |
| 33 | Munj/Patha/Pees/ Taranger 3 cft capacity of stone. | 500 No.s for each structure. |
| 34 | Tooth basket | 100 Nos each Sub Engineer |

ADDITIONAL INFORMATION

| SR.NO | SITE | SANCTION LIMIT LAC.CFT | AVAILABLE AT SITE LAC.CFT | BALANCE REQUIRED | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | J-Head Spur Near Saggu Shumali at Kallurkot | 1.50 | 1.50 | 0.00 | 83 % Qty of reserve stock of stone is available which is sufficient to meet any emergency |
| 2 | Stud No.01 at Kallurkot | 0.00 | 0.00 | 0.00 |  |
| 3 | Guide Spur No.1 at Kallurkot | 1.00 | 1.00 | 0.00 |  |
| 4 | Stud No.2 at Kallurkot | 1.00 | 1.00 | 0.00 |  |
| 5 | Guide Spur No.2 at Kallurkot | 1.00 | 1.00 | 0.00 |  |
| 6 | Guide wall at Kallurkot | 1.50 | 1.25 | 0.25 |  |
| 7 | J-Head Spur Village Ravi at Kallurkot | 1.50 | 1.06 | 0.44 |  |
| 8 | J-Head Spur Daggar Malana at Kallurkot | 1.50 | 1.09 | 0.41 |  |
| 9 | Sloping Stud U/S Daggar Noon Village/L River Indus | 1.00 | 0.80 | 0.20 |  |
| 10 | Inverted Hockey Stud near Daggar Noon Village/L River Indus | 1.00 | 0.43 | 0.57 |  |
|  | TOTAL | 11 | 9.13 | 1.87 |  |

XEN KHS

7.2	WATCHING  ESTABLISHMENT

7:3 	ARRANGEMENTS AT SENSITIVE SITES.

In case of emergency at Sensitive sites, the arrangements for the duty roaster is already attached on page No.17-20

XEN KHS

7.4		WATCHING MATERIAL

Watching material will be provided in ample quantity as the experience shows that flood material’s deployment is very necessary. The particular care would be taken to ensure that there is adequate quantity of the required material shown as to be readily available everywhere particularly at dangerous site.

| Sr# | Description | Quantity Required | Unit | Quantity Available | Quantity Net (Balance) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Kassies with handle | 20 | No | 8 | 12 | Flood fighting material will be purchase as per necessity |
| 2 | Torches | 15 | No | 2 | 13 |  |
| 3 | Cell for Torches | 100 | No | 0 | 100 |  |
| 4 | Shouldari double fly | 4 | No | 1 | 3 |  |
| 5 | Generator 2.5 KV (Mitsubishi) China | 2 | No | 0 | 2 |  |
| 6 | Axes | 30 | No | 6 | 24 |  |
| 7 | Umbrella Large. | 4 | No | 2 | 2 |  |
| 8 | Hammer Wooden | 20 | No | 0 | 20 |  |
| 9 | Patha Ban | 200 | No | 0 | 200 |  |
| 10 | Life Jacket | 8 | No | 4 | 4 |  |
| 11 | PVC insulated twin core Copper conductor cable 7/0.036 | 150 | Metter | 0 | 150 |  |
| 12 | Electric bulb 200 watt. | 40 | No | 0 | 40 |  |
| 13 | Toot baskets. | 600 | No | 00 | 600 |  |
| 14 | Diesel. | 1500 | Litter | 0 | 1500 | For Generator |
| 15 | Gunny bags. | 400 | No | 000 | 400 | Flood fighting material will be purchase as per necessity |
| 16 | Chairs. | 24 | No | 4 | 20 |  |
| 17 | Table. | 8 | No | 1 | 7 |  |
| 18 | Munj / patha tranger | 600 | No | 0 | 600 |  |
| 19 | Steel Charpoy | 5 | No | 1 | 4 |  |
| 20 | Teak Wood Board | 5 | No | 0 | 5 |  |
| 21 | Brass Holder | 16 | No | 0 | 16 |  |
| 22 | Switch Open Type | 8 | Dozen | 0 | 8 |  |

Note: 	 Full quantum of material will be purchased before flood season on receipt of funds.

7.5	ARRANGEMENT FOR SOUNDING AND PROBING

The arrangement for sounding and probing will be taken up with the boat.

7.6	LIGHTING ARRANGEMENT

Generator along with all accessories i.e. electric insulated conductor cable and bulb etc will be kept available at site.

7.7	RATION ARRANGEMENT

Ration will be arranged by the local agencies and through NGO’s etc.

7.8	P.O.L ARRANGEMENT FOR VEHICLE.

## The arrangements of P.O.L will be made as per site requirement.

XEN KHS

## 7.9	TRANSPORTATION

# Transportation arrangements will be made with the help of Civil Administration.

7.10	LAW AND ORDER

Situation of Law & Order will be controlled with the help of law enforcing agencies, i.e.  Police

7.11	MEDICAL ARRANGEMENT FOR LABOUR

Medical facilities will be provided with the help of Health Department.

7.12	LIAISON WITH OTHER DEPARTMENT

The Executive Engineer Khansar will keep liaison with Civil Administration, Police, Health and Civil Defense Department and fully apprise the other Departments about flood situation.

7.13	ROLE OF THE ARMY

Pak Army will play his role in evacuating the Flood affected community, Livestock, Armed Forces will also help in controlling the law and order situation in case of emergency.

7.14	DUTIES OF TELEPHONE ATTENDANT

Phone and cell No of all officers / officials have been given in Flood fighting plan 2025 to make immediate contact at any emergency during flood

| Sr No. | Name of Officer / Officials | Period | Time |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | From |  | To |
| 1 | Muhammad Ramzan Cell No. 0301-6895439 | During Flood Season | 6:00 AM | 6:00 PM |  |
| 2 | Mazhar Hussain Cell No. 0300-5400345 | During Flood Season | 6:00 PM | 6:00 AM |  |

7.15	WIRELESS ARRANGEMENTS

Wireless set will be installed in the vehicles of the Executive Engineer Khansar, Sub Divisional officer River Survey Kallurkot, Sub Divisional officer Kallurkot and Flood emergency center of Khansar Canal Division Kallurkot with the help of police Department.

XEN KHS

CHAPTER-8

DETAIL OF ENCROACHMENT

The flood bunds and spurs are constructed to train the river and to ensure safe passage of the flood water. During flood season, inhabitants of river area evacuate their houses and take shelter on the flood bunds along with their animals. However no encroachments exist on the river training works / Flood bunds in Khansar Canal Division Kallurkot.

Ordinary Encroachment

| Total Encroachments | Removed | Action To Be Taken |
| --- | --- | --- |
| Nil | Nil | Nil |

XEN KHS

CHAPTER -9

DUTY ROASTER / FLOOD FIGHTING PROGRAM.

Phase-I (Pre Flood Period)

This phase covers the pre flood period starting from start of the flood season up to the time the flood water raises upto 1 foot above general Natural Surface Level (NSL) along the flood bunds.

WATCHING:

Flood watching establishment will be arranged through Government contractor on hire & fire basis according to flood scales / stages and other essential needs at site.

| Stage | Strength |
| --- | --- |
| Water level is 1 feet above NSL or below along the flood bund | 1 watchman per mile per shift in two shifts during 24 hours on Kallurkot flood bund. |
| Water level is more than 1 foot above NSL along the flood bund | 1 watchman per mile per shift in three shifts during 24 hours on Kallurkot flood bunds |

During this phase concerned Sub Divisional officers of the bunds and respective Sub Engineers of the sections of the bunds will be the controlling officers under the overall supervision of the Executive Engineer, Khansar Canal Division Kallurkot.

Phase-II (Low Flood From 250000-375000 Cs.

This phase starts when the discharge downstream Chashma barrage is between 250000 Cs to 375000 Cs. The detail of actions to be taken during this phase is as follows.

| Activity | Action |
| --- | --- |
| Shifting of watching and necessary flood material to watching huts | Respective Sub Engineers. |

| Stage | Strength |
| --- | --- |
| The discharge ranges between 2.5 to 3.75 lac cusecs | 1 person per mile per shift in three shifts during 24 hours on Kallurkot flood bund. 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

| S.No | Name of Flood Structures | Duty of Officers / Officials |
| --- | --- | --- |
| 1 | J-Head spur Saggu Shumali at Kallurkot | Incharge SDO River Kallurkot, SEN River |
| 2 | Guide spur No.01 | // |
| 3 | Stud No.1 | // |
| 4 | Stud No. 2 | // |
| 5 | Guide spur No.2 | // |
| 6 | Guide wall at Kallurkot | // |
| 7 | J-Head spur Village Ravi | // |
| 8 | J-Head spur Daggar Malana | // |
| 9 | Sloping stud u/s Daggar Noon Village/L River Indus | // |
| 10 | Inverted hockey stud near Daggar Noon Village/L River Indus. | // |

XEN KHS

Phase-III (Medium Flood From 375000 - 500000 Cs.

This phase starts when the discharge downstream Chashma crosses to flood limits 375000 - 500000 Cs.The detail of the action to be taken during this phase are as follows :-

| Stage | Strength |
| --- | --- |
| The discharge ranges between 3.75 to 5.0 Cs. | 2 persons per mile per shift in three shifts during 24 hours on Kallurkot flood bund. 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

| S.No | Name of Flood Structures | Duty of Officers / Officials |
| --- | --- | --- |
| 1 | J-Head spur Saggu Shumali at Kallurkot | Incharge SDO River Kallurkot, SEN River |
| 2 | Guide spur No.01 | // |
| 3 | Stud No.1 | // |
| 4 | Stud No. 2 | // |
| 5 | Guide spur No.2 | // |
| 6 | Guide wall at Kallurkot | // |
| 7 | J-Head spur Village Ravi | // |
| 8 | J-Head spur Daggar Malana | // |
| 9 | Sloping stud u/s Daggar Noon Village/L River Indus | // |
| 10 | Inverted hockey stud near Daggar Noon Village/L River Indus. | // |

This duty roster during this phase of the officers/officials is tabulated below. The officials under the overall control of the Sub Divisional Officers concerned will perform the duties.

Phase-IV (High Flood From 500000-650000 Cs.

This phase starts when the discharge downstream Chashma crosses to flood limits 500000 Cs to 650000 Cs. The detail of the action to be taken during this phase is as follows:-

| Stage | Strength |
| --- | --- |
| The discharge ranges between 5.0 to 6.5 Cs. | 4 persons per mile per shift in three shifts during 24 hours on Kallurkot flood bund. 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Breach Closing Squad

When the discharge will exceed 4.50 Lac cusecs, the mobile units consisting of following items are to be raised in each sub division. It will be ensured that pick up and tractor trolly are in proper working condition.

Stack bullies 16 foot long			= 140 No.

Killas						= 200 No.

Munj Ban Course				= 140 Kg

Showels					= 50 No.

Gunny bags					= 700 No.

Sutli						= 3 Kg

Needles					= 28   No.

Baskets						= 350 No.

In Kallurkot Sub Division, a unit will be kept at Guide wall. A gang of 10 persons and a mate will move along with the unit at a moment notice. At the same time the Army Authorities will also be requested to provide manpower for closing leakage/breach.

Supervisory Staff

This duty roster during this phase of the officers/officials is tabulated below. The officials under the overall control of the Sub Divisional Officers concerned will perform the duties.

| S.No | Name of Flood Structures | Duty of Officers / Officials |
| --- | --- | --- |
| 1 | J-Head spur Saggu Shumali at Kallurkot | Incharge SDO River Kallurkot, SEN River. |
| 2 | Guide spur No.01 | // |
| 3 | Stud No.1 | // |
| 4 | Stud No. 2 | // |
| 5 | Guide spur No.2 | // |
| 6 | Guide wall at Kallurkot | // |
| 7 | J-Head spur Village Ravi | // |
| 8 | J-Head spur Daggar Malana | // |
| 9 | Sloping stud u/s Daggar Noon Village/L River Indus | // |
| 10 | Inverted hockey stud near Daggar Noon Village/L River Indus. | // |

Phase-V (Very High Flood From. 650000 - 800000 Cs.

This phase starts when the discharge at the River Indus is in very high flood stage i.e. between 6.5 to 8.0 Lac cusecs. During this phase, special attention will be given to watch most vulnerable reaches of the bunds. Concerned officer In charge is responsible for proper up keeping of their reaches that these points. The details of the actions to be taken during this phase are as follows:-

| Stage | Strength |
| --- | --- |
| The discharge ranges between 5.0 to 6.50 Cs. | 8 persons per mile per shift in three shifts during 24 hours on Kallurkot flood bund. 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 foot above NSL. |

Supervisory Staff

This duty roster during this phase of the officers/officials is tabulated blew. The officials under the

Overall control of the Sub Divisional Officers concerned will perform the duties.

| S.No | Name of Flood Structures | Duty of Officers / Officials |
| --- | --- | --- |
| 1 | J-Head spur Saggu Shumali at Kallurkot | Incharge SDO River Kallurkot, SEN River |
| 2 | Guide spur No.01 | // |
| 3 | Stud No.1 | // |
| 4 | Stud No. 2 | // |
| 5 | Guide spur No.2 | // |
| 6 | Guide wall at Kallurkot | // |
| 7 | J-Head spur Village Ravi | // |
| 8 | J-Head spur Daggar Malana | // |
| 9 | Sloping stud u/s Daggar Noon Village/L River Indus | // |
| 10 | Inverted hockey stud near Daggar Noon Village/L River Indus. | // |

Phase-VI (Exceptionally High Flood i.e., more than 800000 Cs.

This phase commences when the discharge downstream Chashma Barrage crosses the very High Flood limit of 8.0 Lac Cs. The detail of actions to be taken during the phase is as follows:-

| ## Activity | Action By |
| --- | --- |
| Deputy Commissioner is to be informed to make arrangement for the evacuation of the marooned public within Bhakkar Flood Bund | Executive Engineer |
| Stage | Strength |
| The discharge ranges between 8.0 Lac Cs and above. | Six Watchmen per miles per shift in three shifts during 24 Hrs on Main embankments and other structures. 1 Chowkidar per watching hut for round the clock duty till the time watching material is taken back from the watching huts after recession of the flood to a level 1 Foot above NSL. |

Supervisory Staff

This duty roster during this phase of the officers/officials is tabulated blew. The officials under the overall control of the Sub Divisional Officers concerned will perform the duties.

| S.No | Name of Flood Structures | Duty of Officers / Officials |
| --- | --- | --- |
| 1 | J-Head spur Saggu Shumali at Kallurkot | Incharge SDO River Kallurkot, SEN River. |
| 2 | Guide spur No.01 | // |
| 3 | Stud No.1 | // |
| 4 | Stud No. 2 | // |
| 5 | Guide spur No.2 | // |
| 6 | Guide wall at Kallurkot | // |
| 7 | J-Head spur Village Ravi | // |
| 8 | J-Head spur Daggar Malana | // |
| 9 | Sloping stud u/s Daggar Noon Village/L River Indus | // |
| 10 | Inverted hockey stud near Daggar Noon Village/L River Indus. | // |

NOTE:-

- All the Sub Engineers would perform their duties assigned above alongwith their establishment with necessary T & P.

- Executive Engineers Khansar would be the overall Incharge of the whole operation during very high and exceptionally high flood.

- However, the Sub Engineers with their establishment would be called any time anywhere and at any point as desired and required by the SDO’s Incharge under intimation to Executive Engineer Khansar the Incharge of whole operation.

XEN KHS

CHAPTER-10

EMERGENCY TELEPHONE NUMBERS

| S. No | Designation | Office | Resd / Mobile |
| --- | --- | --- | --- |
| 1 | Secretary Irrigation Department Lahore. | 042-99212117 042-99212118 | 042-99200954 |
| 2 | Chief Engineer Drainage & Flood Lahore | 042-99233551 | 042-35294102 0321-8969386 |
| 3 | Director Flood Lahore | 042-99233552 | 0323-4864378 |
| 4 | Deputy Director Flood Lahore | 042-35880700 | 042-99230282 0321-9485218 |
| 5 | Flood Warning Center Lahore. | 042-99205157 | - |
| 6 | Chief Engineer Irrigation Sargodha Zone | 048-9230445 | 048-9230451 0336-7501575 |
| 7 | Executive Engineer (Operation) Sargodha Zone | 048-9230446 | 048-3740050 0302-9079639 |
| 8 | Superintending Engineer Bhakkar Canal Circle Bhakkar | 0453-9200316 | 0333-5901500 |
| 9 | Executive Engineer Khansar Canal Division Kallurkot | 0453-200905 | 03029079639 |
| 10 | Sub Divisional Officer KallurKot | 0453-200931 | 0303-8536147 |
| 11 | Sub Divisional Officer, River. | 0453.200931 | 0300-6091979 |
| 12 | Sub Engineer River. | 0453-200931 | 0335-4078500 |
| 13 | Sub Engineer River. | 0453-200931 | 0348-1282776 |
| 14 | Sub Engineer, Dala. | 0336-7567825 | 0336-7567825 |
| 1 5 | Sub Engineer, Mehmood Wala. | 0336-7567825 | 0336-7567825 |
| 16 | Sub Engineer, Hukam. | 0313-7967013 | 0313-7967013 |
| 17 | Sub Engineer Kallurkot | 0302-8136026 | 0302-8136026 |
| 18 | Sub Engineer, Bakhat Branch. | 0303-6605413 | 0303-6605413 |
| 19 | Flood Emergency Center, Telegraph Office Kallurkot. | 0453-200931 | 0301-6895439 0300-5400345 |
| 20 | Chashma Barrage Exchange at Chashma Colony Kundian | 0459-241289 0459-242858 |  |

DISTRICT ADMINISTRATION BHAKKAR.

| 1 | Deputy Commissioner Bhakkar | 0453-9200188 0453-9200288 | 0453-9200388 0304-0920077 |
| --- | --- | --- | --- |
| 2 | Additional Commissioner Revenue Bhakkar | 0453-9200396 | 0333-9995712 |
| 3 | Additional Deputy Commissioner General Bhakkar | 0453-9200307 0453-9200312 | 0333-0721412 |
| 4 | District Police Officer Bhakkar | 0453-9200350 0453-9200351 | 03004761434 |
| 5 | Assistant Commissioner Kallurkot | 0453-200909 | 0300-6001294 |
| 6 | Assistant Commissioner Darya Khan. | 0453-252974 | 0300-6861900 |
| 7 | Bhakkar Flood Control Room | 0453-9200082 | 0453-9200082 |

XEN KHS

CHAPTER-11

STANDARD OPERATION PROCEDURE (SOP) FOR BREACHING SECTIONS

  - HISTORY OF THE BREACHING SECTION.

There is no breaching section in the jurisdiction of Khansar Canal Division Kallurkot.

  - LOCATION, DESIGN, QUANTITY AND VARIETY OF THE EXPLOSIVE REQUIRED FOR DETONATION.

Not Applicable.

  - ARRANGEMENT OF EXPLOSIVES AND SECURITY OF EXPLOSIVE STORES.

Not Applicable.

  - LIST OF THE SECURITY STAFF ALONGWITH DETAIL OF THEIR TRAINING ETC.

Not Applicable.

  - DETAIL OF MECHANICAL MEANS AS A STANDBY ARRANGEMENTS IN CASE OF DETONATION FAILURE

Not Applicable.

  - DUTY ROSTER IN CASE OF CRITICAL SITUATION.

Attached in Page 17 to 20p

  - BREACHING COMMITTEE WITH THEIR ACTION PLAN.

Not Applicable.

  - LIST OF THE VILLAGES LIKELY TO BE INUNDATED IN CASE OF BREACH

Not Applicable.

  - ANNOUNCEMENT AND DETAIL OF EVACUATION ARRANGEMENTS.

Not Applicable.

  - DETAILS OF COORDINATION WITH CIVIL /ARMY AUTHORITIES.

Not Applicable.

  - PARALLEL COMMUNICATION ARRANGEMENT.

Not Applicable.

  - INDEX PLAN.

Attached

Executive Engineer				           		Superintending Engineer

Khansar Canal Division, 						      Bhakkar Canal Circle Kallurkot       			  				    Bhakkar

Chief Engineer,

Sargodha Irrigation Zone,

Sargodha.

CHAPTER-12

VULNERABLE SITES ON FLOOD BUND / STRUCTURES

12.1	APPREHENDED BREACHES IN FLOOD BUNDS / STRUCTURES

Vulnerable sites are given as below.

J-Head Spur Village Ravi River Indus at Kallurkot.

12.2  	Operation of breaching sections.

There is no such case in Khansar Canal Division, Kallurkot.

12.3  	Breaches due to rising of flood water, deterioration of flood bund.

There is no such case in Khansar Canal Division Kallurkot.

XEN KHS

CHAPTER-13

EMERGENCY CONTINGENCY PLAN FOR VULNERABLE SITES LISTED ABOVE

PLAN NO. 1

J-HEAD SPUR VILLAGE RAVI RIVER INDUS AT KALLURKOT.

13.1	PLAN SHOWING ROUTE OF FLOOD WATER COMING OUT OF THE BREACH SUPPORTED WITH LEVELS.

Plan Attached

13.2    DETAIL OF VILLAGES ABADIES LIKELY TO BE AFFECTED

| S. No | Name of Villages Abadies |
| --- | --- |
| 1 | Kallurkot City |
| 2 | Ravi |
| 3 | Malana |
| 4 | Daggar Noon |
| 5 | Saggu Shumali |

XEN KHS

13:3	STRATEGY AND ACTION TO BE TAKEN.

The arrangements will be made to watch Flood infrastructure on round the clock basis. The Incharge Sub Engineer and Sub Divisional Officer will look after all the watching activities. They will keep liaison with the Civil Administrator Public Representative, Police Department Army authorities and Executive Engineer Khansar.

In case of any emergency, the proclamation on loudspeakers will be arranged with the coordination of Civil Administration for evacuation of abadies inside the flood plan. Flood fighting activities will be carried out with the permission and guidance of Executive Engineer Khansar. The establishment will be deployed as per site requirements. The Executive Engineer Khansar will make the arrangement of hiring the machinery on emergent basis and also request will be made to the Agricultural Engineering Department for the provision of the Department machinery i.e. dozers etc.

The flood fighting activities will be performed in true letter and spirit to control the situation and to save the human beings and public property.

XEN KHS

13.3.1	ARRANGEMENTS

The effective arrangements will be made to mitigate the devastating effects of flood .

13.3.2   ESTABLISHMENT OF FLOOD FIGHTING CAMPS

The Flood Fighting Camps will be established as given under.

Guide Wall at Kallur Kot

13.3.3   DUTIES OF OFFICERS / OFFICIALS ON THEIR CAMP SITES

Sub Engineer Kallurkot Section will be incharge of the camp site. The following staff will be at the camp site.

| Camp NO. | Officer (Name Designation & Cell No. | Official |
| --- | --- | --- |
| 03 | M. Anwar ul Haq (SEN) 0348-1282776 Sajid Nawaz khan (SEN 0335-4078500 | 5 No Establishment |

13.3.4    DEPARTMENTAL MACHINERY AVAILABLE

Departmental machinery deputed at Bhakkar Flood Bund will be shifted as per requirement..

13.3.5 	MACHINERY AVAILABLE FROM PRIVATE SOURCE

The following machinery can be made available through private source i.e Government contractors.

| S.No | Name of Items Required | Medium | High | Very High | Exceptionally |
| --- | --- | --- | --- | --- | --- |
| 1 | Excavator | NIL | NIL | 1 | 1 No |
| 2 | Tractor Trolly / Dumper | NIL | 1 | 2 | 2 No |
| 3 | Tractor with front blade | NIL | 1 | 1 | 1 No |
| 4 | Tractor with water tank | 1 | 1 | 1 | 1 No |

XEN KHS

13.3.6 FLOOD FIGHTING MATERIAL AVAILABLE AT SITE NO.1

.

| S.No | Description | Location | Quantity Available |
| --- | --- | --- | --- |
| 1 | Kassies with handle | J- HEAD SPUR VILLAGE RAVI | 4 No. |
| 2 | Shouldari double fly |  | 1 No. |
| 3 | Generator 2.5 KV (Mitsubishi) China |  | 0 No. |
| 4 | Axes |  | 3 No. |
| 5 | PVC insulated twin core Copper conductor cable 7/0.036" |  | 0 Meter |
| 6 | Steel Charpoy |  | 1 No |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRICITY.

Road network, Schools, Health centers and Electric transmission lines exist all along in the Katcha area. There is no any other important installation.

XEN KHS

13.3.7 FLOOD FIGHTING MATERIAL AVAILABLE AT SITE NO.2

.

| S.No | Description | Location | Quantity Available |
| --- | --- | --- | --- |
| 1 | Kassies with handle | INVERTED HOCKEY STUD | 4 No. |
| 2 | Shouldari double fly |  | 0 No. |
| 3 | Generator 2.5 KV (Mitsubishi) China |  | 0 No. |
| 4 | Axes |  | 3 No. |
| 5 | PVC insulated twin core Copper conductor cable 7/0.036" |  | 0 Meter |
| 6 | Table & Chair |  | 1 Set |

13.4	DETAIL OF OTHER INFRASTRUCTURE LIKE ELECTRICITY.

Road network, Schools, Health centers and Electric transmission lines exist all along in the Katcha area. There is no any other important installation.

CHAPTER-14

Action Plan

14.1	RESHUFFLING/RECOUPING PLAN OF RESERVE STOCK OF STONE DEPARTMENTALLY.

Reserve stock of stone in case of emergency will be shifted to critical site through

contractor as per requirement

14.2	DETAIL OF INLET/OUTLET CROSSING ALONG WITH CLOSING METHODOLOGY

Nil

14.3	DEPLOYMENT OF MACHINERY (MEDIUM TO HIGH FLOOD)

Attached at Page No.32

14.4	DEPLOYMENT OF MACHINERY (HIGH TO VERY HIGH FLOOD)

Attached at Page No.33

14.5	DEPLOYMENT OF MACHINERY (HIGH TO EXCEPTIONALLY HIGH FLOOD)

Attached at Page No.34

14.6	POLICE DEPLOYMENT PLAN

Attached at Page No.35

14.7	DETAILS OF SYNTHETIC BAGS WITH CAPACITY OF 500 KG AND 1000 KG

Details is attached on Page No.36

14.8	DETAILS OF POLYTHENE SHEETS OF BLACK COLOR TO PROTECT UPSTREAM SLOPE WAVE ACTION AND TO CONTROL SEEPAGE THROUGH EMBANKMENTS

Details is attached on Page No.36

CHAPTER-15

BACKUP DIVISIONS (IN CASE OF BREACH)

In case of Emergency Mianwali Canal Division Mianwali will be requested for help.

Executive Engineer			 			 Superintending Engineer

Khansar Canal Division,			        		                  Bhakkar Canal Circle Kallurkot				        			   Bhakkar

Chief Engineer,

Sargodha Irrigation Zone,

Sargodha.